BS 08: Validating and identifying errors of a Frequency assignment notice.

Validate and identify the errors of an electronic notice file which was previously prepared and saved.

Solution

- To validate and identify errors in Frequency assignment notice we will use the TerRaNotices “Open file” and “Validate Notice” functionalities.

- TerRaNotices can be used to validate and identify errors in a Frequency assignment notice.

- In addition, there is a new comprehensive online validation tool, which would help in expediting the treatment of your notification. Therefore, the use of this tool is highly recommended before submitting your notices to the Bureau via WISFAT. This validation tool is accessible to all TIES users from the ITU web site at http://www.itu.int/ITU-R/terrestrial/OnlineValidation/Login.aspx. For any assistance, please contact brtpr_dp@itu.int.

The completed notices in text format.

- The next pages provide screen shots from TerRaNotices

- Open TerRaNotices and select “Open file” from the menu.

- Open the file to be validated. For this exercise, we choose a file “BS08_NoticesWithErrors.txt “ And validate the file.

Note that the file contains 3 T01 notice types.

The validation shows that  1 notice has error while 2 notices have warnings.

- In order to see the detail of errors, click on the notice with error and continue to correct.

  - Checking the notice with error, shows that the Unique identifier (t_adm_ref_id=LSO_84_2016_BP2-ex ) is repeated with in the same submission.

Solution: In this notice, the unique identification code is the same as another notice in the same submission.

The unique identification code ( t_adm_ref_id) is an identifying element and shall be unique. Thus, continue to correct this field. In this exercise t_adm_ref_id=LSO_84_2016_BP2-ex  is corrected to t_adm_ref_id=LSO_84_2016_BP2-ex2

3.2 Checking notice LSO_84_2016_BP1-ex. the Effective antenna Height at specific azimuths (every 10 degrees) are missing.

Solution:

To calculate the effective antenna height (m) at 36 different azimuths, open Tools and select the option “Calculate Effective antenna heights (eff_hgt) using the SRMT3 Terrain database. Then enter the Geographical coordinates and the height of antenna above ground level (m) and submit. The values of effective antenna height (m) at 36 different azimuths in 10 degrees interval will be filled automatically.

4- After correction, validate and save the corrected file which is ready to be submitted via wisfat.