BS 01: Modification of the GE84 Plan

Prepare an electronic notice for Addition of frequency 99.2 MHz assigned to a sound broadcasting station, for the modification of the GE84 Plan..

Solution

To prepare this notice we will use the “Wizard” functionality of TerRaNotices and we will select Liberia (LBR) as the notifying administration.

- The ”Wizard” will identify the notice type to use, once the minimum required fields (Administration, Fragment, Station class, Action and Frequency) are entered.

- The purpose of the exercise is to request the notice to be published in GE84 plan; therefore, the fragment is GE84

- The completed notice in text format.

- The next pages provide screen shots from TerRaNotices

- Open TerRaNotices and select “Wizard “

- Then select  the notifying administration and choose option  Update to one of the world regional plans.

Note: the Wizard proposes the appropriate Notice types at the bottom of the screen corresponding to the given criteria at each step.

- Then specify the action as “ADD” an assignment and continue.

Note: the Wizard identifies the appropriate Notice type at the bottom of the screen. Finish and confirm the summary to open the identified notice type T01.

- Confirm the summary to open the identified notice type T01.

- Fill in the required fields as requested.

To calculate the effective antenna height (m) at 36 different azimuths, open Tools and select the option “Calculate Effective antenna heights (eff_hgt) using the SRMT3 Terrain database.

Then enter the Geographical coordinates and the height of antenna above ground level (m) and submit. The values of effective antenna height (m) at 36 different azimuths in 10 degrees interval will be filled automatically.

- Once the required fields are completed, validate the notice and save file which is ready to be submitted via Wisfat. (Web Interface for Submission of Frequency Assignments/allotments for Terrestrial services)