FXM 01 Solution: Validating and identifying errors of a Frequency assignment notice.

Validate and identify the errors of an electronic notice file which was previously prepared and saved.

- Purpose of exercise:  To validate and identify errors in Frequency assignment notice using the online validation.

Solution

- To Validate and identify errors of a notice file, we will use the online validation tool which can be accessed from http://www.itu.int/ITU-R/terrestrial/OnlineValidation/Login.aspx

Electronic notice file:

- Log on to the online validation tool.

http://www.itu.int/ITU-R/terrestrial/OnlineValidation/Login.aspx

- Browse the file to be validated and upload to process. For this exercise, we choose a file “FXM 01_OnlineVal.txt “

- Once the process completed, we can see the following validation error messages as output;

Note that we also receive an email announcing the completion of validation.

- We can see the following error messages;

Notice 1 (Line 4) - T11/ADD

Line 4 : Notice 1 (T11 - ADD) relates to Notice 2 (T11 - ADD) as candidates for amendments. Reason: full match through identifying elements.

Notice 2 (Line 49) - T11/ADD

Line 4 : Notice 1 (T11 - ADD) relates to Notice 2 (T11 - ADD) as candidates for amendments. Reason: full match through identifying elements.

- Please check the identifying elements of two notices.

- Assigned Frequency, Class of station, Bandwidth, Emission Class, Geographical coordinates of the Transmitting site and the Operations Hours are the same

- That means that the second notice replace the first one.

- You have to submit one notice with two antennas

4.  Correction:

After corrections and the validation message will be:

Parse status: T_PARSE_NTC_COMPLETE

Total number of errors: 0

Total number of warnings: 0