BS 01: VHF analog sound broadcasting assignment

Prepare an electronic notice file of frequency 89.0 MHz assigned to a sound broadcasting station, for its recording of in the Master Register.

Solution

To prepare this notice we will use the “Wizard” functionality of TerRaNotices and we will select Vietnam (VTN) as the notifying administration.

- The ”Wizard” will identify the notice type to use, once the minimum required fields (Administration, Fragment, Station class, Action and Frequency) are entered.

- The purpose of the exercise is to record an assignment in the Master Register, the relevant fragment is: Article 11.

- In addition to the technical items, the following mandatory fields need to provide as follow:

  - Address code : Preface to the BR IFIC, Chapter IV, Section 3

  - Date of bringing into use : not earlier than 3 months

  - Operating hours : 00:00 – 23:59 (valid range for this data item)

- The completed notice in text format.

- The next pages provide screen shots from TerRaNotices

- Open TerRaNotices and select “Wizard “

- Then select  VTN as notifying administration and choose option Update of MIFR under article 11 of the RR.

Note: the Wizard proposes the appropriate Notice types at the bottom of the screen corresponding to the given criteria at each step.

- Then specify the action as “ADD”.  And continue.

Note: the Wizard narrows the selection of Notice types at the bottom of the screen corresponding to the given criteria at each step.

- Select Station class “BC” for sound broadcasting and enter Frequency to Finish.

Note: the Wizard identifies the appropriate Notice type at the bottom of the screen.

- Proceed to open the identified notice type as T01.

- Fill in the required fields as requested.

- Once the required fields are completed, validate the notice and save file which is ready to be submitted via Wisfat. (Web Interface for Submission of Frequency Assignments/allotments for Terrestrial services)