BS 02: GT1: DVB-T Assignment for recording into MIFR

Prepare an electronic notice file of frequency 474.0  MHz assigned to a digital TV broadcasting station, for its recording in the Master Register.

Solution

To prepare this notice we will use the “New File” functionality of TerRaNotices and select the administration of Angola (AGL) as the notifying administration.

- The notice type to use for the notification of a digital television broadcasting station in the GE06 plan area is: GT1

- The purpose of the exercise is to record an assignment in the Master Register, the relevant fragment is: Article 11.

- In addition to the technical items, the following mandatory fields need to be provided as follow:

  - Address code : Preface to the BR IFIC, Chapter IV, Section 3

  - Date of bringing into use : not earlier than 3 months

  - Operating hours : 00:00 – 23:59 (valid range for this data item)

- The completed notice in text format.

The next pages provide screen shots from TerRaNotices

- Open TerRaNotices and select “New File “

then select AGL as notifying administration. - Type of notice GT1 “ Digital Television broadcasting (DVB-T) assignment in the frequency and geographical scope of GE06”

- Proceed to open the GT1 notice type.

- Fill in the required fields as requested.

- To calculate the effective antenna height (m) at 36 different azimuths, open Tools and select the option “Calculate Effective antenna heights (eff_hgt) using the SRMT3 Terrain database.

- Then enter the Geographical coordinates and the height of antenna above ground level (m) and submit. The values of effective antenna height (m) at 36 different azimuths in 10 degrees interval will be filled automatically.

- Once the required fields are completed, validate the notice and save file which is ready to be submitted via Wisfat (Web Interface for Submission of Frequency Assignments/allotments for Terrestrial services)