BS 03: Validating and identifying errors of a Frequency assignment notice.

Validate and identify the errors of an electronic notice file which was previously prepared and saved.

Solution

- To validate and identify errors in Frequency assignment notice we will use the TerRaNotices “Open file” and “Validate Notice” functionalities.

- TerRaNotices can be used to validate and identify errors in a Frequency assignment notice.

- In addition, there is a new comprehensive online validation tool, which would help in expediting the treatment of your notification. Therefore, the use of this tool is highly recommended before submitting your notices to the Bureau via WISFAT. This validation tool is accessible to all TIES users from the ITU web site at http://www.itu.int/ITU-R/terrestrial/OnlineValidation/Login.aspx. For any assistance, please contact brtpr_dp@itu.int.

The completed notices in text format.

- The next pages provide screen shots from TerRaNotices

- Open TerRaNotices and select “Open file” from the menu.

- Open the file to be validated. For this exercise, we choose a file “BS 03_NoticeWithError.txt “ And validate the file.

Note that the file contains 3 T01 notice types.

The validation shows that 1 has no error while 2 notices have errors.

- In order to see the detail of errors, click on the notice with error and continue to correct.

3.1 Checking notice HNG13A091203-EX

Solution: In this notice, the effective radiated power in vertical direction is missing while effective power in horizontal direction is not required.

In checking the notice, the polarization is given as Vertical. Thus, the corresponding value can be corrected, from t_erp_h_dbw = 32 to t_erp_v_dbw = 32

Alternative solution may be changing the polarization, if we are sure that the error lies with the polarization as the case maybe.

3.2 Checking notice HNG2008MA11-EX with error.

Solution: In this notice, the unique identification code is the same as another notice in the same submission.

The unique identification code ( t_adm_ref_id) is an identifying element and shall be unique. Thus, continue to correct this field. In this exercise HNG2008MA11-EX is corrected to HNG2008MA11-EX2

4- After correction, validate and save the corrected file which is ready to be submitted via wisfat.