Manual Calculation of Break-Even Point

for the Use of Remote Subscriber Switches

Solution of Case study

Mr. H. Leijon, ITU

SOLUTION

CASE STUDY, MANUAL CALCULATION OF ECONOMICAL BREAK-EVEN POINT FOR THE USE OF REMOTE SUBSCRIBER SWITCHES

a	Traffic, Case 1 and Case 3, 500 subscribers, 0.1 E

500´0.1 E = 50 E

50 E, 0.1 % congestion => 71 circuits

Traffic, Case 2 and Case 4, 2 000 subsc., 0.1 E

2 000´0.1 E = 200 E

200 E, 0.1 % congestion => 238 circuits

b	Number of PCM systems

71 circuits => 3 PCM systems

238 circuits => 8 PCM systems

CASE 1

Check that transmission plan is not violated!

Loss B-C (maximum) 2 km, 0.5 mm cable:		2´1.21 = 2.42 dB

Remains for A-B: 8 -2.42 = 5.58 dB

if 0.4 mm cable corresponding to 	5.581.60=3.48 km

if 0.5 mm cable corresponding to	5.581.21=4.61 km

if 0.7 mm cable corresponding to	5.580.79=7.06 km

Suppose that X is in the interval 3.48 < X < 4.61 km and calculate with 0.5 mm cable (for VF) between A and B!

Suppose that Y intermediate PCM repeater points are required.

=> Cost of (RSS + PCM) alternative:

2´16 000´X+350 000+16 000´Y+3´1 500´Y+20 000+8 000+12 000/3+4 000´Y

=>	Cost of (SS + cabinet) alternative: 	5 000+113 000´X

To find the economical cross-over point, set the two costs equal.

(113 000 - 32 000) X = 350 000 + 32 000 - 5 000+ (16 000 + 4 500 + 4 000)´Y

81 000´X = 377 000 + 24 500´Y

X=37700081000+24500´Y81000=4.65+0.30Y

Y = 1 ⇒  4.95 km > 4.61 km		(PCM signal loss = 12 dB/km, max. loss = 35 dB => Y = 1)

Thus, 0.7 mm cable is necessary for determining X. Try again:

5000 + 166000 X = 2´16000 X + 350000 + 16000´Y + 3´1500 Y + 20000 + 8000 + 4000 + 4000 Y

(166000 - 32000) X = 350000 + 32000 +(16000 + 4500 + 4000) Y

134 000 X = 382 000 + 24 500 Y

X=382 000 134 000+24 500134 000Y

X = 2.85 + 0.18 Y

Attenuation 12 dB/km, maximum loss 35 dB ⇒ Y = 1 => X = 2.85 + 0.18 = 3.03 km

Conclusion

Up to 4.61 km SS with main cable, 0.5 mm is used because:

- it is cheaper than a solution with RSS

- it is permitted with regard to the transmission plan

Above 4.61 km, the RSS alternative is used. This alternative is cheaper than SSS + 0.7 mm main cable already above 3.03 km.

CASE 2

2´16000´X+350 000+16 000´Y+8´1 500´Y+20 000+7´4 000+4000+4 000´Y=20 000+300 000´X

268 000´X  = 350 000 + 52 000 - 20 000 + (16 000 + 12 000 + 4 000)´Y= 382 000 + 32 000 Y

X=382 000 268 000+32 000268 000Y=1.43+0.1Y⇒Y=0

X = 1.43 km

CASE 3

350 000 + 16 000´Y + 3´1 500´Y + 20 000 + 8 000 + 4 000 + 4 000´Y= 5 000 + 96 000´X + 96 000´X

(assume 0.4 mm cable, add 100 % for new duct)

192 000´X = 350 000 + 32 000 - 5 000 + (16 000 + 4 500 + 4 000) Y= 377 000 + 24 500´Y

X = 1.96 + 0.128 Y⇒ Y = 1

X = 2.09 km

CASE 4

350 000 + 16 000´Y + 8´1 500´Y + 20 000 + 7´4 000 + 4 000 + 4 000´Y= 20 000 + 300 000´X + 300 000´X

600 000´X = 350 000 + 52 000 - 20 000+ (16 000 + 12 000 + 4 000)´Y= 382 000 + 32 000´Y

X = 0.64 + 0.053 Y⇒ Y = 0

X = 0.64 km