Economic Period of Provisioning

Planning of Fiber Optics Cable

Case Study

by Mr. Moumoulidis, OTE, Athens

The problem

Between two large towns which are considered as “transit exchanges”, there is a demand for trunk circuits. The demand is generated by two groups of towns which are served by their transit exchanges. The layout of towns is shown in Figure 1. For the next three years, the demand growth is shown in Table 1. By assuming the linear demand growth between the long distance exchanges, we wish to determine the optimum size of the fiber optics cable which should be placed between the transit exchanges. The fiber optics cable was found to be the most economical transmission media.

We also wish to determine:

- the present worth of expenditures at optimal size cable;

- the present worth of expenditures at double size;

- the percentage variation of present worth between double and optimal size cable;

- he annual charges.

The following data are provided:

The PCM systems, which are going to be superimposed on the fiber optics cable, are of fourth order. One fully equipped system provides 1920 ch.

Cost of fiber optics cable

- Basic cost				600 MU / km

- Incremental cost				720 MU / km / pair

- Taxes on purchasing cost			20 %

- Digging cost				750 MU / km

- Placement of cable			80 MU / km

- Splicing and testing			15 MU / km / pair

- Operating plus maintenance cost		3.5 %

- Service life				18 years

- Interest rate				10 %

- Distance between transit exchanges		170 km

For references, see the relevant document.

|  | DE BROGLIE | ZEEMAN | HEISENBERG | LANDAU | BLANCK | BOHR | COMPTON | STERN | EINSTEIN |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DE BROGLIE |  |  |  |  | 50 | 20 | 25 | 26 | 25 |
| ZEEMAN |  |  |  |  | 50 | 75 | 26 | 25 | 35 |
| HEISENBERG |  |  |  |  | 72 | 25 | 25 | 36 | 25 |
| LANDAU |  |  |  |  | 95 | 27 | 75 | 60 | 80 |
| BLANCK | 45 | 25 | 75 | 92 |  |  |  |  |  |
| BOHR | 25 | 25 | 46 | 25 |  |  |  |  |  |
| COMPTON | 25 | 25 | 60 | 100 |  |  |  |  |  |
| STERN | 30 | 25 | 35 | 50 |  |  |  |  |  |
| EINSTEIN | 28 | 30 | 32 | 90 |  |  |  |  |  |

Table 1: Trunks needed for the next three years