Use of Subscriber Single Channel Carrier

to Defer Feeder Relief Project

Case Study

Mr. G. Moumoulidis, OTE

1.	The problem

The demand growth of a small village is l = 15 subs/year. The village is 4 km from the exchange. The existing cables are completely used up.

Alternatives available to add facilities are either to place a buried cable of sufficient capacity or to install a single channel carrier.

The problem is to determine the optimal policy to cater for the future demand.

2.	Input data

2.1	Cables

- Basic purchasing cost			100 MU / km

- Incremental purchasing cost			6.5 MU / pair / km

- Digging and placement cost			650 MU / km

- Service life					35 years

- Operating and maintenance cost		2.5 %

- Scrap value	0

2.2	Single channel carrier

- Purchasing cost				30 MU / piece

- Installation cost				10 MU / piece

- Service life					15 years

- Maintenance and operating cost		5 %

- Scrap value					0

- Interest rate					10 %