Measurement of

Traffic Distribution Matrix

( Solution to Exercise )

Mr. H. Leijon, ITU

MEASUREMENT OF TRAFFIC DISTRIBUTION MATRIX:

Solution to the Example

The first step in the job is to establish a form (matrix) in which the different information can be stored. See fig 5.3. It is convenient to put both routing  and traffic information in this form.

First position in first row can be filled in directly as it is a D-case. The measured value is 35 erl and the minimum value is decided to be 95% i.e. 35 erl. Note that traffic values are given as integers, normally this gives sufficient precision.

In first row there are no more D-cases, then we turn over to the H-cases. See section 2. From each 1 to 4  102 erl is measured on the H-route. The measurement shows 16.5% rejection from H-route, and from traffic route tester results the total congestion Bh⋅Bt is estimated to 5%. Thus eq. 2.2 gives

AH=102⋅1−0.051−0.165=116=102+14

which is written in the form. The minimum value is estimated as

AHm=102+0.3´14=106

Measurements show that the originating traffic from each no 1 is 292 erl of which 35 + 102 + 14 = 151 erl now have been declared. The remaining traffic 292 - 151 = 141 erl is distributed on the T-cases in accordance to calling rates found from number analysing equipment, which gives a distribution like

AT12=141⋅13864801=41  erl

AT13=141⋅22994801=68  erl

AT15=141⋅11164801=33  erl

As a consequence of this unprecise method we assume maximum error of 40% i.e. minimum values

ATm=0.6⋅AT

In the next row the three D-cases are treated as already shown and as exchange no 2 has no number analysing equipment the remaining traffic 318 - 48 - 70 - 115 - = 85 erl is distributed between exchanges 1 and 5 in accordance with their terminating traffics 221 erl and 182 erl respectively. i.e.

AT21=85⋅221221+182=47  erl

AT25=85⋅182221+182=38  erl

and minimum values 60% of this.

The following rows are calculated analogously, but the calculations and underlying measurements are omitted in this example.

| From to ® | 1 | 2 | 3 | 4 | 5 | A0 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 routing estimated traffic minimum traffic | (D) 35 33 | T 41 25 | T 68 41 | H 102 + 14 106 | T 33 20 | 292 |
| 2 routing estimated traffic minimum traffic | T 47 28 | (D) 48 46 | D 70 67 | D 115 109 | T 38 23 | 318 |
| 3 routing estimated traffic minimum traffic | H 40 + 8 42 | H 61 + 3 62 | (D) 95 90 | D 157 149 | H 50 + 3 51 | 421 |
| 4 routing estimated traffic minimum traffic | H 55 + 6 57 | H 60 + 10 63 | D 115 109 | (D) 186 177 | D 60 57 | 496 |
| 5 routing estimated traffic minimum traffic | T 36 22 | H 30 + 5 32 | H 50 + 5 52 | D 103 98 | —— | 229 |
| Terminating AI | 221 | 265 | 398 | 690 | 182 | 1756 |

Figure 5.3

Now fig. 5.3 contains 25 traffic values of varying reliability or confidence.

As explained in section 4 the minimum matrix is removed from the tentative traffic matrix. Fig 5.4 shows the result of this.

E.g. the elements of the first row are

35-33, 41-25, 68-41, 102+14-106, 33-20

and the desired sum 67 is formed as

292 - (33+25+41+106+20) = 67

Now the Kruithof method is applied on the matrix fig 5.4, thus giving the matrix shown in fig 5.5 which then is added to the minimum values shown in fig 5.3. This gives the final result shown in fig 5.6. Due to rounding-off errors the sums are not exactly right but the differences are not more than 2 erl.

| j i | 1 | 2 | 3 | 4 | 5 | A0 - Am |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 16 | 27 | 10 | 13 | 67 |
| 2 | 19 | 2 | 3 | 6 | 15 | 45 |
| 3 | 6 | 2 | 5 | 8 | 2 | 27 |
| 4 | 4 | 7 | 6 | 9 | 3 | 33 |
| 5 | 14 | 3 | 3 | 5 | 0 | 25 |
| AI - Am | 39 | 37 | 39 | 51 | 31 | 197 |

Fig 5.4

| j i | 1 | 2 | 3 | 4 | 5 | A0 - Am |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 19 | 23 | 12 | 12 | 67 |
| 2 | 17 | 3 | 3 | 8 | 15 | 45 |
| 3 | 6 | 3 | 5 | 12 | 2 | 27 |
| 4 | 4 | 9 | 6 | 12 | 3 | 33 |
| 5 | 12 | 4 | 3 | 7 | 0 | 25 |
| AI - Am | 39 | 37 | 39 | 51 | 31 | 197 |

Fig 5.5

| j i | 1 | 2 | 3 | 4 | 5 | A0 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 35 | 44 | 64 | 118 | 32 | 293 |
| 2 | 45 | 49 | 70 | 117 | 38 | 319 |
| 3 | 48 | 65 | 95 | 161 | 53 | 422 |
| 4 | 61 | 72 | 115 | 189 | 60 | 497 |
| 5 | 34 | 36 | 55 | 105 | 0 | 230 |
| AI | 223 | 266 | 399 | 690 | 183 | 1761 |

Fig 5.6