Specification of Equipment

for the Rural and Metropolitan

Case Studies

Mr. G. Moumoulidis, ITU

Switching Systems

Type:	SPC	Size:	1200 lines + 110 trunks

| Item | Purchasing Cost | Taxes | Installation | Total Investment | Replace-ment | Mainten-ance | Total Present Worth |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Subscriber Stage | 17.0 | 3.4 | 1.7 | 22.1 | 2.2 | 8.5 | 32.8 |
| Inlet | 10.8 | 2.1 | 1.1 | 14.0 | 1.4 | 5.4 | 20.8 |
| Outlet | 10.8 | 2.1 | 1.1 | 14.0 | 1.4 | 5.4 | 20.8 |
| Analog trunk | 49.3 | 9.9 | 5.0 | 64.2 | 6.4 | 24.6 | 95.2 |
| Digital trunk | 9.8 | 2.0 | 1.0 | 12.8 | 1.3 | 4.9 | 19.0 |
| Common equipment | 3288 | 658 | 329 | 4274 | 434 | 1644 | 6353 |

| (Taxes) | = 20% of purchasing cost |
| --- | --- |
| (Installation) | = 10% of purchasing cost |
| (Total investment) | = Purchasing cost + Taxes + Installation |
| (Replacement) | =(Total investment)(1+i)T−1 |
| (Maintenance) | ui ´ (Purchasing cost) |
| (Present Worth) | (Total investment) + (Replacement) + (Maintenance) |
| T = 25 years | Service life of plant |
| u = 5% | Annual operating plus maintenance cost in relation to purchasing cost |
| i = 10% | = Interest rate |

Switching Systems

Type:	SPC	Size:	15000 lines + 1900 trunks

| Item | Purchasing Cost | Taxes | Installation | Total Investment | Replace-ment | Mainten-ance | Total Present Worth |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Subscriber Stage | 17.0 | 3.4 | 1.7 | 22.1 | 2.2 | 8.5 | 32.8 |
| Inlet | 10.8 | 2.1 | 1.1 | 14.0 | 1.4 | 5.4 | 20.8 |
| Outlet | 10.8 | 2.1 | 1.1 | 14.0 | 1.4 | 5.4 | 20.8 |
| Analog trunk | 49.3 | 9.9 | 5.0 | 64.2 | 6.4 | 24.6 | 95.2 |
| Digital trunk | 9.8 | 2.0 | 1.0 | 12.8 | 1.3 | 4.9 | 19.0 |
| Common equipment | 30000 | 6000 | 3000 | 39000 | 3865 | 15000 | 57965 |

| (Taxes) | = 20% of purchasing cost |
| --- | --- |
| (Installation) | = 10% of purchasing cost |
| (Total investment) | = Purchasing cost + Taxes + Installation |
| (Replacement) | =(Total investment)(1+i)T−1 |
| (Maintenance) | ui ´ (Purchasing cost) |
| (Present Worth) | (Total investment) + (Replacement) + (Maintenance) |
| T = 25 years | Service life of plant |
| u = 5% | Annual operating plus maintenance cost in relation to purchasing cost |
| i = 10% | = Interest rate |

Switching Systems

Type:	SPC	Size:	50000 lines + 9000 trunks

| Item | Purchasing Cost | Taxes | Installation | Total Investment | Replace-ment | Mainten-ance | Total Present Worth |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Subscriber Stage | 17.0 | 3.4 | 1.7 | 22.1 | 2.2 | 8.5 | 32.8 |
| Inlet | 10.8 | 2.1 | 1.1 | 14.0 | 1.4 | 5.4 | 20.8 |
| Outlet | 10.8 | 2.1 | 1.1 | 14.0 | 1.4 | 5.4 | 20.8 |
| Analog trunk | 49.3 | 9.9 | 5.0 | 64.2 | 6.4 | 24.6 | 95.2 |
| Digital trunk | 9.8 | 2.0 | 1.0 | 12.8 | 1.3 | 4.9 | 19.0 |
| Common equipment | 42145 | 8429 | 4214 | 54788 | 5571 | 21072 | 81432 |

| (Taxes) | = 20% of purchasing cost |
| --- | --- |
| (Installation) | = 10% of purchasing cost |
| (Total investment) | = Purchasing cost + Taxes + Installation |
| (Replacement) | =(Total investment)(1+i)T−1 |
| (Maintenance) | ui ´ (Purchasing cost) |
| (Present Worth) | (Total investment) + (Replacement) + (Maintenance) |
| T = 25 years | Service life of plant |
| u = 5% | Annual operating plus maintenance cost in relation to purchasing cost |
| i = 10% | = Interest rate |

Switching Systems

Type:	Analog	Size:	10000 lines + 2000 trunks

| Item | Purchasing Cost | Taxes | Installation | Total Investment | Replace-ment | Mainten-ance | Total Present Worth |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Subscriber Stage | 13.6 | 2.7 | 2.7 | 19.0 | 1.9 | 10.9 | 31.8 |
| Inlet | 7.0 | 1.4 | 1.4 | 9.8 | 1.0 | 5.6 | 16.4 |
| Outlet | 7.0 | 1.4 | 1.4 | 9.8 | 1.0 | 5.6 | 16.4 |
| Analog trunk | 18.0 | 3.6 | 3.6 | 25.2 | 2.6 | 14.4 | 42.2 |
| Common equipment | 5860 | 1172 | 1172 | 8204 | 829 | 4688 | 13721 |

| (Taxes) | = 20% of purchasing cost |
| --- | --- |
| (Installation) | = 20% of purchasing cost |
| (Total investment) | = Purchasing cost + Taxes + Installation |
| (Replacement) | =(Total investment)(1+i)T−1 |
| (Maintenance) | ui ´ (Purchasing cost) |
| (Present Worth) | (Total investment) + (Replacement) + (Maintenance) |
| T = 25 years | Service life of plant |
| u = 8% | Annual operating plus maintenance cost in relation to purchasing cost |
| i = 10% | = Interest rate |

Switching Systems

Type:	Analog	Size:	5000 lines + 1000 trunks

| Item | Purchasing Cost | Taxes | Installation | Total Investment | Replace-ment | Mainten-ance | Total Present Worth |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Subscriber Stage | 13.6 | 2.7 | 2.7 | 19.0 | 1.9 | 10.9 | 31.8 |
| Inlet | 7.0 | 1.4 | 1.4 | 9.8 | 1.0 | 5.6 | 16.4 |
| Outlet | 7.0 | 1.4 | 1.4 | 9.8 | 1.0 | 5.6 | 16.4 |
| Analog trunk | 18.0 | 3.6 | 3.6 | 25.2 | 2.6 | 14.4 | 42.2 |
| Common equipment | 3420 | 684 | 684 | 4788 | 487 | 2736 | 8011 |

| (Taxes) | = 20% of purchasing cost |
| --- | --- |
| (Installation) | = 20% of purchasing cost |
| (Total investment) | = Purchasing cost + Taxes + Installation |
| (Replacement) | =(Total investment)(1+i)T−1 |
| (Maintenance) | ui ´ (Purchasing cost) |
| (Present Worth) | (Total investment) + (Replacement) + (Maintenance) |
| T = 25 years | Service life of plant |
| u = 8% | Annual operating plus maintenance cost in relation to purchasing cost |
| i = 10% | = Interest rate |

Remote Subscriber Unit

| Item | Purchasing Cost | Taxes | Installation | Total Investment | Replace-ment | Mainten-ance | Total Present Worth |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RSU 128 | 1200 | 240 | 120 | 1560 | 159 | 600 | 2319 |
| RSU 256 | 2050 | 410 | 205 | 2665 | 271 | 1025 | 3961 |
| RSU 1000 | 5200 | 1040 | 520 | 6760 | 687 | 2600 | 10047 |
| RSU 2000 | 8000 | 1600 | 800 | 10400 | 1057 | 4000 | 15457 |

| (Taxes) | = 20% of purchasing cost |
| --- | --- |
| (Installation) | = 10% of purchasing cost |
| (Total investment) | = Purchasing cost + Taxes + Installation |
| (Replacement) | =(Total investment)(1+i)T−1 |
| (Maintenance) | ui ´ (Purchasing cost) |
| (Present Worth) | (Total investment) + (Replacement) + (Maintenance) |
| T = 25 years | Service life of plant |
| u = 5% | Annual operating plus maintenance cost in relation to purchasing cost |
| i = 10% | = Interest rate |

Line Concentrator

Capacity:	160 lines + 28 trunks

| Item | Purchasing Cost | Taxes | Installation | Total Investment | Replace-ment | Mainten-ance | Total Present Worth |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Basic cost | 1050 | 210 | 178 | 1438 | 251 | 630 | 2319 |
| Subs unit for 16 subs | 70 | 14 | 12 | 96 | 16.8 | 42 | 155 |
| Line unit for 17 trunks | 22 | 4.4 | 3.8 | 30.2 | 5.3 | 13.2 | 48.7 |
| Interface for carrier system (12 trunks) | 700 | 140 | 120 | 960 | 168 | 420 | 1548 |

| (Taxes) | = 20% of purchasing cost |
| --- | --- |
| (Installation) | = 17% of purchasing cost |
| (Total investment) | = Purchasing cost + Taxes + Installation |
| (Replacement) | =(Total investment)(1+i)T−1 |
| (Maintenance) | ui ´ (Purchasing cost) |
| (Present Worth) | (Total investment) + (Replacement) + (Maintenance) |
| T = 20 years | Service life of plant |
| u = 6% | Annual operating plus maintenance cost in relation to purchasing cost |
| i = 10% | = Interest rate |

Cost of cables

Type:	Buried Cable			Mean size :	Æ  0.4	150 pairs

Æ  0.6	 70 pairs

Æ  0.8	 50 pairs

Æ  1.0	 30 pairs

| Item | Purchasing Cost | Digging (ducts) | Jointing Superv. | Misce-laneous | Total Investment | Replace-ment | Mainten-ance | Total Present Worth |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Æ 0.4 | 534 | 1200 | 260 | 50 | 2044 | 75 | 107 | 2226 |
| Æ 0.6 | 467 | 1200 | 300 | 85 | 2052 | 75 | 93 | 2220 |
| Æ 0.8 | 635 | 1200 | 380 | 100 | 2315 | 85 | 127 | 2527 |
| Æ 1.0 | 646 | 1200 | 300 | 100 | 2246 | 83 | 129 | 2458 |

|  | Mean size | Fill at relief | Present Worth per pair at f.a.r. |
| --- | --- | --- | --- |
| Æ 0.4 | 150 pairs | 85 % | 17.4 |
| Æ 0.6 | 70 pairs | 85 % | 37.3 |
| Æ 0.8 | 50 pairs | 90 % | 56.1 |
| Æ 1.0 | 30 pairs | 95 % | 86.2 |

| (Total investment) | = Purchasing cost + Digging + Jointing + Miscellaneous |
| --- | --- |
| (Replacement) | =(Total investment)(1+i)T−1 |
| (Maintenance) | ui ´ (Purchasing cost) |
| (Present Worth) | (Total investment) + (Replacement) + (Maintenance) |
| T = 35 years | Service life of plant |
| u = 2% | Annual operating plus maintenance cost in relation to purchasing cost |
| i = 10% | = Interest rate |

Cost of Cables

Type:	Cable in DUCT		Mean size :	Æ  0.4	300 pairs

Æ  0.6	300 pairs

Æ  0.8	750 pairs

| Item | Purchasing Cost | Digging (ducts) | Jointing Superv. | Misce-laneous | Total Investment | Replace-ment | Mainten-ance | Total Present Worth |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Æ 0.4 | 723 | 1100 | 361 | 100 | 2284 | 84 | 144 | 2513 |
| Æ 0.6 | 1340 | 1100 | 496 | 120 | 3056 | 113 | 268 | 3437 |
| Æ 0.8 | 4864 | 1100 | 972 | 486 | 7422 | 275 | 972 | 8669 |
| Æ 0.8L | 5107 | 1100 | 1000 | 500 | 7707 | 284 | 1021 | 9013 |

|  | Mean size | Fill at relief (f.a.r.) | Present Worth per pair at f.a.r. | (PW/pair at f.a.r.) = PW(Meansize)´(f.a.r.) |
| --- | --- | --- | --- | --- |
| Æ 0.4 | 300 pairs | 80 % | 10.5 |  |
| Æ 0.6 | 300 pairs | 80 % | 14.3 |  |
| Æ 0.8 | 750 pairs | 90 % | 12.8 |  |
| Æ 0.8L | 750 pairs | 90 % | 13.4 |  |

| (Total investment) | = Purchasing cost + Digging + Jointing + Miscellaneous |
| --- | --- |
| (Replacement) | =(Total investment)(1+i)T−1 |
| (Maintenance) | ui ´ (Purchasing cost) |
| (Present Worth) | (Total investment) + (Replacement) + (Maintenance) |
| T = 35 years | Service life of plant |
| u = 2% | Annual operating plus maintenance cost in relation to purchasing cost |
| i = 10% | = Interest rate |

PCM

Capacity:	30 circuits

| Item | Purchasing Cost | Taxes | Installation | Total Investment | Replace-ment (infinite) | Mainten-ance | Total Present Worth |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Terminal | 80 | 170 | 127.5 | 1147 | 361 | 425 | 1933 |
| Signalling Interface Unit | 300 | 60 | 45 | 405 | 127 | 150 | 682 |
| LTE1 | 172 | 34.4 | 26 | 232 | 73 | 86 | 391 |
| Regenerator (one way) | 15 | 3 | 2.2 | 20.2 | 6.5 | 7.5 | 34 |
| Regenerator housing (10 repeaters) | 150 | 30 | 22.5 | 202.5 | 63.6 | 75 | 340 |

| (Taxes) | = 20% of purchasing cost |
| --- | --- |
| (Installation) | = 15% of purchasing cost |
| (Total investment) | = Purchasing cost + Taxes + Installation |
| (Replacement) | =(Total investment)(1+i)T−1 |
| (Maintenance) | ui ´ (Purchasing cost) |
| (Present Worth) | (Total investment) + (Replacement) + (Maintenance) |
| T = 15 years | Service life of plant |
| u = 5% | Annual operating plus maintenance cost in relation to purchasing cost |
| i = 10% | = Interest rate |

Cost of Buildings

| New Buildings | Size in m2 | Cost |
| --- | --- | --- |
|  | 500 | 51000 |
|  | 1000 | 95000 |
|  | 2000 | 175000 |
|  | 3000 | 250000 |
| Extensions | Size in m2 | Cost |
|  | 300 | 12000 |
|  | 500 | 18500 |
|  | 700 | 25000 |
|  | 900 | 31000 |

Conversion of Subscriber Lines to Building Size

| Analogue Exchanges | Subscribers | Floor space in m2 |
| --- | --- | --- |
|  | 10000 | 550 |
|  | 15000 | 830 |
|  | 20000 | 1120 |
| SPC Exchanges | Subscribers | Floor space in m2 |
|  | 10000 | 180 |
|  | 15000 | 260 |
|  | 20000 | 325 |
|  | 30000 | 390 |
|  | 40000 | 455 |
|  | 50000 | 520 |

Parameters of Telecommunication Cables

| Unloaded | Conductor size (mm) | Capacitance (nF / km) | Loop resistance (W / km) | Loss at 800 Hz (dB / km) | Type |
| --- | --- | --- | --- | --- | --- |
|  | 0.4 | 45 | 280 | 1.55 | U, D |
|  | 0.6 | 50 | 130 | 1.1 | U, D |
|  | 0.8 | 38.5 | 72 | 0.72 | U, D |
|  | 1.0 | 38.5 | 46 | 0.57 | U, D |

An approximate formula for the calculation of the loss of the pairs providing very good results in the voice band ( 0 - 4 kHz ) is:

a=pfRC	 in Neper / km

| Loaded | Conductor size (mm) | Capacitance (nF / km) | Loop resistance (W / km) | Loss at 800 Hz (dB / km) | Type |
| --- | --- | --- | --- | --- | --- |
|  | 0.8 | 38.5 | 72 | 0.28 | U, D |

The calculation the loss in the voice band is performed throgh the formula:

a=R2SCLp	 in Neper / km

S = 1.83 km

Lp = 88mH

R = Loop resistance

C = Capacitance of the pair

f = Frequency in Hz ( 800 Hz )

U = Underground cable

D = Cable in duct

Signalling Requirements

|  | Subscriber loop resistance | Trunk loop resistance |
| --- | --- | --- |
| SPC | 2000 W | - |
| Analogue | 1200 W | 2000 W |