|  | INTERNATIONAL TELECOMMUNICATION UNION | Document SATBAG-03/01-E 29 April 2002 Original: English |
| --- | --- | --- |
|  | SATELLITE BACKLOG ACTION GROUP MEETING GENEVA, 22-24 JANUARY 2003 |  |

Director, Radiocommunication Bureau

information from working party 4A

The attached document from Working Party 4A is submitted for information and consideration by SATBAG.

Annex: 1

| Working Party 4A ANNEX |  |  |
| --- | --- | --- |
|  | INTERNATIONAL TELECOMMUNICATION UNION |  |
|  | RADIOCOMMUNICATION STUDY GROUPS | Document 4A/TEMP/225edited 15 April 2002 English only |

reply to satbag

During its last meeting in October 2001 Working Party 4A received from the satellite Backlog Group a request to make technical analysis on two aspects of Appendix S30B, as explained in paragraphs 1 and 2 of Document 4A/342.

There was insufficient time for this request to be given adequate consideration during the October meeting, and owing to an oversight the need for action was not recorded in the Chairman's report of that meeting.

Following some discussions during its meeting in April 2002, Working Party 4A would like to offer the following response to SATBAG, that may also be used by BR to develop any new Rule of Procedure or to update existing Rules of Procedure.

1)	Response on the possible application of the improved earth station antenna pattern to all part A allotments and to some system at the pre-design stage:

The improved antenna pattern referred to in Section 1.6 of Annex 1 to Appendix S30B is more stringent than Recommendation ITU-R S.580-5 for off-axis angle between 20° and 48 °. Working Party 4A proposes therefore to apply Recommendation ITU-R S.580-5 (with the formula G = 29 – 25 log ) to all Part A allotments and, subject to consultation with the responsible administrations, to apply the improved antenna pattern referred to in Section 1.6 of Annex 1 to Appendix S30B to some systems at the pre-design stage which are using the side-lobe pattern of 32-25 log θ (as proposed by SATBAG).

Working Party 4A is of the view that this proposal would not cause additional burden to BR, as this antenna pattern is already implemented in the software used to perform the necessary technical analysis under Appendix 30B.

Working Party 4A urges administrations to submit contributions to its next meeting to determine whether 7 metre antennas at 4/6 GHz and 3 metre antennas at 11/13 GHz (see Section 1.6.1 of Annex 1 to Appendix S30B) meet the improved side-lobe antenna pattern referred to in Section 1.6 of Annex 1 to Appendix 30B.

2)	Response on the possible reduction in the aggregate and single-entry protection criteria:

Working Party 4A has urged administrations participating in its work to submit contributions to its next meeting to determine whether further relaxation of these protection criteria may occur. However, WP4A believes that any change to the protection criteria would be a matter for a competent WRC.

_______________