|  | INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- | --- |
|  | SATELLITE BACKLOG ACTION GROUP MEETING GENEVA, 22-24 JANUARY 2003 | Document SATBAG-03/20-E |
|  |  | 24 January 2003 |
|  |  | Original: English |

SAT-BAG

Liaison statement to WP4A, wp6s, wp7c, wp8D: Extension of the coordination arc for identification of affected administrations

The coordination arc was first introduced for the geostationary FSS networks, in a limited range of frequency bands above 3.4 GHz, at WRC 2000. Since that date the use of the coordination arc has been extended to FSS and BSS geostationary networks, not subject to a plan, in other frequency bands, by a Rule of Procedure adopted by the RRB in response to Council Resolution 1182. In the extension of the application of the coordination arc to other frequency bands, the lower frequency limit has been maintained at 3.4 GHz as below that frequency limit, satellite networks may have to coordinate with other networks located anywhere in the visible arc. The CPM report contains an alternative approach to implementing the extension of the use of the coordination arc.

The coordination arc method for determining affected GSO networks has the following features:

- allowing the notifying administration to identify the potentially affected networks and start the coordination process at the beginning of the 5 year period rather than wait until the Bureau have completed the Appendix 8 analysis - which with processing delays may be 2 to 3 years into the 5 year time limit; and,

- substantially reducing the volume of data required under Appendix 4 as the link noise temperature data in Section D of Annex 2 is not needed for the coordination arc procedure;

In any extension of the coordination arc, the same provisions (i.e. No. 9.41) would apply to permit any network located outside the coordination arc to be included in, or networks located within the coordination arc to be excluded from, the list of networks with which coordination needs to be affected.

Hence, SAT-BAG requests that the above mentioned Working Parties complete the relevant studies into the appropriate value of orbital arc separation for each group of space services, not subject to a plan in frequency bands, above 3.4 GHz, to be used in cases of inter and intra service sharing.

_________________