|  | INTERNATIONAL TELECOMMUNICATION UNION | Document SATBAG-03/9-E 9 January 2003 Original: English |
| --- | --- | --- |
|  | SATELLITE BACKLOG ACTION GROUP MEETING GENEVA, 22-24 JANUARY 2003 |  |

United Kingdom

The satellite backlog and the complexity of 
the Radio Regulations

# 1	Introduction

In its development of an action plan for addressing the factors associated with the development of the satellite-filing backlog, one of the factors referenced by many participants is the complexity of the Radio Regulations (RR). Tackling this complexity is not a trivial exercise and requires detailed consideration of many aspects of the filing process and the RR: the effort required for this important task process deterring many administrations from starting the necessary work. However, the delay in starting this task is not without cost to administrations, industry and ITU. Hence, it is proposed that WRC-03 should initiate a study into the clarification of the satellite filing procedures. Noting that this work may take some time, in the interim, there may be benefit in introducing some clarification to the satellite network coordination and notification procedures.

# 2	Radio Regulations: complex and inconsistent

The Radio Regulations are complex, contain inconsistencies and are difficult to master: representing possibly the single largest factor contributing to the backlog. The complexity and inconsistency increase the workload and costs of administrations and operators in the submission of notices, as well as the Bureau in their treatment of satellite filings. Finding the information required for the submission of a satellite filing and following the relevant procedures can be both time-consuming and difficult. For example, specific types of information (e.g. pfd limits) are not limited to one section of the RR, but may be found in footnotes to the frequency allocation table, as well as other articles, appendices and resolutions. Also, it may be difficult to determine which procedures are applicable, and for cases where multiple procedures apply to the same satellite network, there can be inconsistencies between the procedures as well as duplication of work in applying them.

In addition, to the problems with the submission of data, administrations and operators face increased costs during the coordination process due to the effort required for resolving misunderstandings in the application of the regulatory provisions. The increased cost and the time required for correcting errors or resolving queries leads to delays in completion of the regulatory filing process and the introduction of new satellite systems, which may have a broader impact on national telecommunication infrastructures.

Proposal 1

To address the complexity of the RR, it is proposed that WRC-03 initiate a study of the relevant regulatory procedures and to systematically remove unnecessary duplication, inconsistency and complexity. A draft WRC-03 Resolution YYY containing the proposal is presented in Annex 1.

# 3	Complex regulatory procedures: contributing factors

A contributing factor to the complexity of the RR is the process by which they are revised. Individual provisions and resolutions are brought into force at a different time from the main text of the regulations, and sometimes each other. This proliferation of different dates of coming into force causes confusion for administrations and for the Bureau in their implementation. Hence, in reducing the complexity of the RR, administrations also need to consider the process by which they propose and make revisions to the regulations, in order to avoid new inconsistencies being introduced. This should include improving the process by which the Final Acts, and the RR, where necessary, are updated following a WRC, so that minor editorial errors can be resolved prior to publication. These issues assume greater importance during periods of rapid change in the RR.

The liberalization of telecommunications and the growth in private telecommunication operators (PTOs), resulting in expansion of telecommunication networks and development of the manufacturing industry, has generated pressure for a faster rate of change in regulatory provisions. In ITU, the period between WRCs has been reduced, to the extent that proposals for change to the RR are being finalized almost before the previous revision has come into force. While the current economic climate, and administrations commitment to the forthcoming RRC, has reduced the pressure for another WRC within the 2 to 3 year period that has been the standard in recent years, an extended gap following WRC-03 should only be viewed as a temporary respite. In the developed countries there is always potential for the introduction of new services, and in many developing countries the gap in telecommunications provision creates enormous potential for future development. Hence, a rise in economic confidence and an increase in growth in the market sector will undoubtedly create further demand for more rapid regulatory change.

# 4	Interim measures to clarify existing procedures

One of the RR procedures whose complexity is frequently referenced is Article 9 of the Radio Regulations. RR Article 9 contains the provisions for effecting coordination with or obtaining agreement of other administrations and includes four separate coordination processes that have been brought together in a single procedure following the simplification of the Radio Regulations. The result of this merger is a procedure where none of the individual processes is sequential and individual processes may loop back through certain provisions.

The attached revision of Article 9 attempts to show the complexity of this procedure by:

•	rearranging the provisions so that they run more sequentially;

•	inserting clarifying references into the provisions; and

•	inserting statements to clarify the application of groups of provisions.

As one of the aims of this revision has been to avoid renumbering some of the most widely referenced provisions, clarification of the procedure is necessarily limited but it does indicate the extent to which the procedure can be improved.

Proposal 2

Noting that a study of the relevant regulatory provisions will take some time to complete, and in the interim administrations and operators will continue to be adversely affected by the complexity of the RR, it is considered there may be benefits in introducing such clarifications at WRC-03.

Annex 2 contains an example of a possible clarification of RR Article 9 provision for further consideration by administrations and WRC-03.

Annex 1

DRAFT RESOLUTION YYY (WRC-03)

Rationalization and clarification of the satellite network 
coordination and notification procedures

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	Council Resolution 1182 establishing the Satellite Backlog Action Group;

b)	the continuing backlog of satellite network coordination requests pending with the Radiocommunication Bureau;

c)	that the current backlog in the ITU's satellite coordination process seriously undermines the ability of such networks to provide services and compromises the role of ITU in this process;

d)	that Council-01 noted in Council Resolution 1182, "the backlog in the Bureau's processing of satellite filings is having an adverse impact on the Radiocommunication Bureau's ability to continue providing other services to administrations";

e)	that the satellite backlog and the delays in completing the regulatory coordination process is having a negative impact on the ability of Member States to introduce new satellite services and may have a broader impact on national telecommunication infrastructures;

f)	that a major factor in the development of the satellite backlog is the complexity of the satellite network coordination and notification procedures,

resolves to invite ITU-R

1	to systematically remove unnecessary duplication, inconsistency and complexity from the satellite coordination and notification procedures;

2	to undertake, as a matter of urgency, all the relevant studies required to meet the above aim;

3	to consider the process by which proposals for change to the regulatory provisions are developed within ITU-R and adopted by a WRC,

invites administrations

to contribute actively to the relevant ITU-R studies.

Annex 2

Example of a possible clarification of the RR Article 9 procedures

The figures in square brackets indicate the original location of provisions and the symbols given in the margin indicate the changes to the texts and have the following meaning:

ADD	=	addition of a new provision

MOD	=	modification of an existing provision

(MOD)	=	editorial modification of an existing provision

SUP	=	deletion of an existing provision

SUP*	=	provision moved to another place in Article 9

ADD*	=	an existing provision moved from another place in Article 9 to the place indicated

MOD

ARTICLE  9

Procedure for effecting coordination with or 
obtaining agreement of other administrations1, 2, 3, 4, 5, 6

Section I  –  Advance publication of information on satellite
networks or satellite systems

General

|  | 9.1 Before initiating any action under this Article or under Article 11 in respect of frequency assignments for a satellite network or a satellite system, an administration, or one7 acting on behalf of a group of named administrations, shall, prior to the coordination procedure described in Section II of Article 9 below, where applicable, send to the Bureau a general description of the network or system for advance publication in the International Frequency Information Circular (BR IFIC) not earlier than five years and preferably not later than two years before the planned date of bringing into use of the network or system (see also Nos. 11.44 and 11.44B to 11.44I). The characteristics to be provided for this purpose are listed in Appendix 4. The coordination or notification information may also be communicated to the Bureau at the same time; it shall be considered as having been received by the Bureau not earlier than six months after the date of receipt of the information for advance publication where coordination is required by Section II of Article 9. Where coordination is not required by Section II, notification shall be considered as having been received by the Bureau not earlier than six months after the date of publication of the advance publication information. |
| --- | --- |
|  | 9.2 Amendments to the information sent in accordance with the provisions of No. 9.1 shall also be sent to the Bureau as soon as they become available. The use of an additional frequency band or modification of the orbital location by more than  12º for a space station using the geostationary-satellite orbit will require the application of the advance publication procedure for this band or orbital location, as appropriate. (WRC-2000) |
| MOD | 9.2A If the information sent under Nos. 9.1 or 9.2 is found to be incomplete, the Bureau shall immediately seek from the administration concerned any clarification required and information not provided. |
| Reasons | To clarify that the information referenced in No. 9.2A is that provided under both Nos. 9.1 and 9.2. |
|  | 9.2B On receipt of the complete information sent under Nos. 9.1 and 9.2, the Bureau shall publish8 it in a Special Section of its BR IFIC within three months. When the Bureau is not in a position to comply with the time limit referred to above, it shall periodically so inform the administrations, giving the reasons therefore. (WRC-2000) |

Sub-Section IA  –  Advance publication of information on satellite networks or satellite
systems that are not subject to coordination procedure under Section II

| ADD* | 9.2bis [9.5A] The procedure of Sub-Section IA shall be considered mainly for the purposes of informing all administrations of developments in the use of space radiocommunications. |
| --- | --- |
| Reasons | The text above is information relevant to the application of Sub-Section 1A and therefore is more appropriately located at the beginning of the Sub-Section than at the end of the Sub-Section. This text could be presented as a short introduction to Sub-Section 1A and may not require a specific provision number. |
|  | 9.3 If, upon receipt of the BR IFIC containing information published under No. 9.2B, any administration believes that interference which may be unacceptable may be caused to its existing or planned satellite networks or systems, it shall within four months of the date of publication of the BR IFIC communicate to the publishing administration its comments on the particulars of the anticipated interference to its existing or planned systems. A copy of these comments shall also be sent to the Bureau. Thereafter, both administrations shall endeavour to cooperate in joint efforts to resolve any difficulties, with the assistance of the Bureau, if so requested by either of the parties, and shall exchange any additional relevant information that may be available. If no such comments are received from an administration within the aforementioned period, it may be assumed that the administration concerned has no objections to the planned satellite network(s) of the system on which details have been published. |
|  | 9.4 In the case of difficulties, the administration responsible for the planned satellite network shall explore all possible means to resolve the difficulties without considering the possibility of adjustment to networks of other administrations. If no such means can be found, it may request the other administrations to explore all possible means to meet its requirements. The administrations concerned shall make every possible effort to resolve the difficulties by means of mutually acceptable adjustments to their networks. An administration on behalf of which details of planned satellite networks have been published in accordance with the provisions of No. 9.2B shall, after the period of four months, inform the Bureau of the progress made in resolving any difficulties. If necessary, a further report shall be provided prior to the submission of notices to the Bureau under Article 11. |
|  | 9.5 The Bureau shall inform all administrations of the list of administrations which have sent comments under No. 9.3 and provide a summary of the comments received. |
| SUP* |  |
| Reasons | The text above is information relevant to the application of Sub-Section 1A and therefore is more appropriately located at the beginning of the Sub-Section than at the end of the Sub-Section. This text could be presented as a short introduction to Sub-Section 1A and may not require a specific provision number. |

Sub-Section IB  –  Advance publication of information on satellite networks or satellite
systems that are subject to coordination procedure under Section II

| ADD* | 9.5bis [9.5C] The procedure of Sub-Section IB shall be considered mainly for the purposes of informing all administrations of developments in the use of space radiocommunications. |
| --- | --- |
| Reasons | The text above is information relevant to the application of Sub-Section 1B and therefore is more appropriately located at the beginning of the Sub-Section than at the end of the Sub-Section. This text could be presented as a short introduction to Sub-Section 1A and may not require a specific provision number. |
| MOD | 9.5B If, upon receipt of the BR IFIC containing information published under No. 9.2B, any administration considers its existing or planned satellite systems or networks or terrestrial stations9 to be affected, it may send its comments to the publishing administration, so that the latter may take those comments into consideration when initiating the coordination procedure in Section II of Article 9. A copy of these comments may also be sent to the Bureau. Thereafter, both administrations shall endeavour to cooperate in joint efforts to resolve any difficulties, with the assistance of the Bureau, if so requested by either of the parties, and shall exchange any additional relevant information that may be available. (WRC-2000) |
| Reasons | To clarify the coordination procedure referenced in Section II of Article 9. |
| SUP* |  |
| Reasons | The text above is information relevant to the application of Sub-Section 1B and therefore is more appropriately located at the beginning of the Sub-Section than at the end of the Sub-Section. This text could be presented as a short introduction to Sub-Section 1A and may not require a specific provision number. |
| MOD | 9.5D If under the coordination procedure in Section II of Article 9 the detailed information required under No. 9.22bis has not been received by the Bureau within a period of 24 months after the date of receipt by the Bureau of the relevant information under Nos. 9.1 and 9.2, the information published under No 9.2B shall be cancelled. The Bureau shall inform the responsible administration at least three months before the end of the 24-month period. The Bureau shall also publish the cancellation in its BR IFIC. |
| Reasons | To clarify the information require relates to the procedure in Section II of Article 9 and to clarify the Bureau shall inform the administration. Other changes consequential to renumbering. |

Section II  –  Procedure for effecting coordination10, 11

Sub-Section IIA  –  Requirement and request for coordination

| ADD | Effecting coordination |
| --- | --- |
| Reasons | Clarification of the purpose of the following group of provisions. |
| (MOD) | 9.6 Before an administration12, 13 notifies to the Bureau or brings into use a frequency assignment in any of the cases listed below, it shall effect coordination, as required, with other administrations identified under No. 9.6bis: |
| ADD* | 9.6bis [9.27] Frequency assignments to be taken into account in effecting coordination are identified using Appendix 5. |
| Reasons | The text above is information relevant to the application of Sub-Section IIA and therefore is more appropriately located at the beginning of the Sub-Section. |
| ADD | 9.6ter [9.26 & 9.28] Coordination may be effected for a satellite network under Nos. 9.7 to 9.14 and 9.21 using the information relating to the space station, including its service area, and the parameters of one or more typical earth stations located in all or part of the service area of the space station. Coordination may also be effected for terrestrial stations using the information relating to typical terrestrial stations, except for those mentioned in Nos. 11.18 to 11.23. The administration making the coordination request under Nos. 9.15 to 9.19 shall, by applying the calculation method and criteria contained in Appendix 5 to those frequency assignments, identify, to the extent possible, the administrations with which coordination is to be effected. |
| Reasons | The text above is information relevant to the application of Sub-Section IIA and therefore is more appropriately located at the beginning of the Sub-Section. This text could be presented as a short introduction to Sub-Section I1A and may not require a specific provision number. In addition, No. 9.26 is incomplete in that it doesn't refer to the criteria in Appendix 5 and No. 9.28 repeats part of No. 9.27. Although the text above currently only includes some minor editorials, combining 9.26 and 9.28 would allow the text to be further rationalized. |
| ADD | Sharing scenarios |
| Reasons | Clarification of the purpose of the following group of provisions. |

|  | 9.7 a) for a station in a satellite network using the geostationary-satellite orbit, in any space radiocommunication service, in a frequency band and in a Region where this service is not subject to a plan, in respect of any other satellite network using that orbit, in any space radiocommunication service in a frequency band and in a Region where this service is not subject to a plan, with the exception of coordination between earth stations operating in the opposite direction of transmission. |
| --- | --- |
|  | 9.7A b)14, 15 for a specific earth station in a geostationary-satellite network in the fixed-satellite service in certain frequency bands, in respect of a non-geostationary-satellite system in the fixed-satellite service; (WRC-2000) |
|  | 9.7B c)14, 15 for a non-geostationary-satellite system in the fixed-satellite service in certain frequency bands, in respect of a specific earth station in a geostationary-satellite network in the fixed-satellite service; (WRC-2000) |
|  | 9.8 (SUP - WRC-2000) |
|  | 9.9 (SUP - WRC-2000) |
|  | 9.10 Not used. |
|  | 9.11 d) for a space station in the broadcasting-satellite service in any band shared on an equal primary basis with terrestrial services and where the broadcasting-satellite service is not subject to a plan, in respect of terrestrial services; |
|  | 9.11A e) for a station for which the requirement to coordinate is included in a footnote to the Table of Frequency Allocations referring to this provision, the provisions of Nos. 9.12 to 9.16 are applicable; (WRC-2000) |
|  | 9.12 f) for a station in a satellite network using a non-geostationary-satellite orbit, for which the requirement to coordinate is included in a footnote to the Table of Frequency Allocations referring to this provision or to No. 9.11A, in respect of any other satellite network using a non-geostationary-satellite orbit, with the exception of coordination between earth stations operating in the opposite direction of transmission; (WRC-2000) |

|  | 9.12A g) for a station in a satellite network using a non-geostationary-satellite orbit, for which the requirement to coordinate is included in a footnote to the Table of Frequency Allocations referring to this provision or to No. 9.11A, in respect of any other satellite network using the geostationary-satellite orbit, with the exception of coordination between earth stations operating in the opposite direction of transmission; (WRC-2000) |
| --- | --- |
|  | 9.13 h) for a station in a satellite network using the geostationary-satellite orbit, for which the requirement to coordinate is included in a footnote to the Table of Frequency Allocations referring to this provision or to No. 9.11A, in respect of any other satellite network using a non-geostationary-satellite orbit, with the exception of coordination between earth stations operating in the opposite direction of transmission; (WRC-2000) |
|  | 9.14 i) for a space station of a satellite network for which the requirement to coordinate is included in a footnote to the Table of Frequency Allocations referring to No. 9.11A in respect of stations of terrestrial services where the threshold value is exceeded; (WRC-2000) |
|  | 9.15 j) for either a specific earth station or typical earth station of a non-geostationary satellite network for which the requirement to coordinate is included in a footnote to the Table of Frequency Allocations referring to No. 9.11A, in respect of terrestrial stations in frequency bands allocated with equal rights to space and terrestrial services and where the coordination area of the earth station includes the territory of another country; (WRC-2000) |
|  | 9.16 k) for a transmitting station of a terrestrial service for which the requirement to coordinate is included in a footnote to the Table of Frequency Allocations referring to No. 9.11A and which is located within the coordination area of an earth station in a non-geostationary-satellite network; (WRC-2000) |
|  | 9.17 l) for any specific earth station or typical mobile earth station in frequency bands above 100 MHz allocated with equal rights to space and terrestrial services, in respect of terrestrial stations, where the coordination area of the earth station includes the territory of another country, with the exception of the coordination under No. 9.15; (WRC-2000) |
|  | 9.17A m) for any specific earth station, in respect of other earth stations operating in the opposite direction of transmission, in frequency bands allocated with equal rights to space radiocommunication services in both directions of transmission and where the coordination area of the earth station includes the territory of another country or the earth station is located within the coordination area of another earth station, with the exception of the coordination under No. 9.19; (WRC-2000) |
|  | 9.18 n) for any transmitting station of a terrestrial service in the bands referred to in No. 9.17 within the coordination area of an earth station, in respect of this earth station, with the exception of the coordination under Nos. 9.16 and 9.19; (WRC-2000) |
|  | 9.19 o) for any transmitting station of a terrestrial service or any transmitting earth station in the fixed-satellite service (Earth-to-space) in a frequency band shared on an equal primary basis with the broadcasting-satellite service, with respect to typical earth stations included in the service area of a space station in the broadcasting-satellite service; (WRC-2000) |

|  | 9.20 Not used. |
| --- | --- |
|  | 9.21 p) for any station of a service for which the requirement to seek the agreement of other administrations is included in a footnote to the Table of Frequency Allocations referring to this provision. (WRC-2000) |
|  | 9.22 Not used. |
| ADD | Request for coordination |
| Reasons | Clarification of the purpose of the following group of provisions. |
| ADD* | 9.22bis [9.30] Requests for coordination made under Nos. 9.7 to 9.14 and 9.21 shall be sent by the requesting administration to the Bureau, together with the appropriate information listed in Appendix 4 to these Regulations. |
| Reasons | No. 9.30 describes the application of Nos. 9.7 to 9.14 and 9.21 and it is more appropriately located immediately after all the provisions describing the sharing scenarios. |
| (MOD) | 9.23 Whenever there is a requirement to effect more than one form of coordination in accordance with No. 9.22bis, the requests shall be appropriately identified by reference to Nos. 9.7 to 9.14 and 9.21, and they shall as far as possible be sent to the Bureau and, where appropriate, shall be published simultaneously. |
| ADD | 9.23bis [9.32] If the administration requesting coordination concludes that coordination is not required under Nos. 9.7 to 9.7B, it shall send the relevant information pursuant to Appendix 4 to the Bureau for action under No. 9.34. (WRC-2000) |
| Reasons | No. 9.32 describes the action to be taken in the event that the administration concludes that coordination of their network or station is not required under Nos. 9.7 to 9.7B and it is more appropriately located immediately after all the provisions describing how coordination requests are initially treated. In addition, the text is changed to clarify that it is the requesting administration that submits the Appendix 4 data. |
|  | 9.24 and 9.25 Not used. |
| SUP |  |
| Reasons | The text above is information relevant to the application of Sub-Section IIA and therefore is more appropriately located at the beginning of the Sub-Section - see No. 9.6ter. |
| SUP* |  |
| Reasons | The text above is information relevant to the application of Sub-Section IIA and therefore is more appropriately located at the beginning of the Sub-Section - see No. 9.6bis. |

| SUP |  |
| --- | --- |
| Reasons | The text above is information relevant to the application of Sub-Section IIA and therefore is more appropriately located at the beginning of the Sub-Section - see No. 9.6ter. |
|  | 9.29 Requests for coordination made under Nos. 9.15 to 9.19 shall be sent by the requesting administration to the identified administrations, together with the appropriate information listed in Appendix 4 to these Regulations. |
| SUP* |  |
| Reasons | No. 9.30 describes the application of Nos. 9.7 to 9.14 and 9.21 and it is more appropriately located immediately after all the provisions describing the sharing scenarios. |
|  | 9.31 The information sent under No. 9.29 shall also, in the cases covered by Nos. 9.15, 9.17 or 9.17A, include a copy of diagrams drawn to appropriate scale indicating, for both transmission and reception, the location of the earth station and its associated coordination area, or the coordination area related to the service area in which it is intended to operate the mobile earth station, and the data on which the diagrams are based. In respect of terrestrial stations, in the cases covered by Nos. 9.16, 9.18 and 9.19 the information shall include the locations of terrestrial stations within the coordination area of the relevant earth station. |
| SUP | - |
| Reasons | No. 9.32 describes the application of Nos. 9.7 to 9.7B and it is more appropriately located immediately after all the provisions describing the sharing scenarios. |
|  | 9.32A If the responsible administration, following the application of Nos. 9.15 to 9.19, concludes that coordination is not required, it may send the relevant information pursuant to Appendix 4 to the Bureau for action under Section I of Article 11. |
|  | 9.33 If for any reason an administration cannot act in accordance with No. 9.29, it shall seek the assistance of the Bureau. The Bureau shall then send the request for coordination to the administration concerned and take any necessary further action as appropriate under Nos. 9.45 and 9.46. |
| ADD | Action to be taken by the Bureau on receipt of the coordination data |
| Reasons | Clarification of the purpose of the following group of provisions. |

| (MOD) | 9.34 On receipt of the complete information sent under No. 9.22bis or No. 9.23bis the Bureau shall promptly: |
| --- | --- |
|  | 9.35 a) examine that information with respect to its conformity with No. 11.3116; (WRC-2000) |
| (MOD) | 9.36 b) identify in accordance with No. 9.6bis any administration with which coordination may need to be effected17, 18; (WRC-2000) |
|  | 9.37 c) include their names in the publication under No. 9.38; |
| MOD | 9.38 d) publish19, as appropriate, the complete information under No. 9.34 in the BR IFIC within four months. Where the Bureau is not in a position to comply with the time-limit referred to above, it shall periodically so inform the administrations, giving the reasons therefor. (WRC-2000) |
| Reasons | Clarification that the information referenced is that supplied under No. 9.34. |
|  | 9.39 Not used. |
|  | 9.40 e) inform the administrations concerned of its actions and communicate the results of its calculations, drawing attention to the relevant BR IFIC. |
| MOD | 9.40A If the information under No. 9.34 is found to be incomplete, the Bureau shall immediately seek from the administration concerned any clarification required and information not provided. |
| Reasons | Clarification that the information referenced is that supplied under No. 9.34. |
| ADD | Requests for inclusion or exclusion in the coordination procedure |
| Reasons | Clarification of the purpose of the following group of provisions. |

|  | 9.41 Following receipt of the BR IFIC referring to requests for coordination under Nos. 9.7 to 9.7B, an administration believing that it should have been included in the request or the initiating administration believing that an administration identified under No. 9.36 in accordance with the provisions of No. 9.7 (GSO/GSO) (items 1), 2) and 3) of the frequency band column), No. 9.7A (GSO earth station/non-GSO system) or No. 9.7B (non-GSO system/GSO earth station) of Table 5-1 of Appendix 5 should not have been included in the request, shall, within four months of the date of publication of the relevant BR IFIC, inform the initiating administration or the identified administration, as appropriate, and the Bureau, giving its technical reasons for doing so, and shall request that its name be included or that the name of the identified administration be excluded, as appropriate. (WRC-2000) |
| --- | --- |
| MOD | 9.42 The Bureau shall study the information, sent in accordance with No. 9.41, on the basis of the criteria in Appendix 5 and shall inform both administrations of its conclusions. Should the Bureau agree to include or exclude, as appropriate, the administration in the request, it shall publish an addendum to the publication under No. 9.38. (WRC-2000) |
| Reasons | Clarification that the information referenced is that supplied under No. 9.41 and that the reference to Appendix 5 relates to the defined criteria. |
|  | 9.43 Those administrations not responding under No. 9.41 within the time limit specified therein shall be regarded as unaffected and the provisions of Nos. 9.48 and 9.49 shall apply. |
| ADD | Additional data requirements |
| Reasons | Clarification of the purpose of the following group of provisions. |
| MOD | 9.44 The administration requesting coordination under Nos. 9.7 to 9.21 and those with which it is requested, or the Bureau when acting pursuant to No. 7.6, may request any additional information they consider necessary. |
| Reasons | That the administration referenced relates to the request under Nos. 9.7 to 9.21 rather than under No. 9.41. |

Sub-Section IIB  –  Acknowledgement of receipt of a request for coordination

|  | 9.45 An administration receiving a request for coordination under No. 9.29 shall, within 30 days from the date of the request, acknowledge receipt by telegram to the requesting administration. In the absence of an acknowledgement of receipt of its request within the 30 days, the requesting administration shall send a telegram requesting an acknowledgement. |
| --- | --- |
|  | 9.46 If there is no acknowledgement of receipt within 15 days of its second request sent under No. 9.45, the requesting administration may seek the assistance of the Bureau. In this event, the Bureau shall forthwith send a telegram to the administration which has failed to reply requesting an immediate acknowledgement. |
|  | 9.47 If there is no acknowledgement of receipt within 30 days after the Bureau's action under No. 9.46, it shall be deemed that the administration which has failed to acknowledge receipt has undertaken: |

|  | 9.48 a) that no complaint will be made in respect of any harmful interference affecting its own assignments which may be caused by the assignment for which coordination was requested; and |
| --- | --- |
|  | 9.49 b) that the use of its own assignments will not cause harmful interference to the assignment for which coordination was requested. |

Sub-Section IIC  –  Action upon a request for coordination

| ADD | Action by administrations |
| --- | --- |
| Reasons | Clarification of the purpose of the following group of provisions. |
| ADD | 9.49bis [9.53] Under Nos. 9.7 to 9.21 the administration requesting coordination and the administrations responding to the coordination request shall make every possible mutual effort to overcome any difficulties, in a manner acceptable to the parties concerned. |
| Reasons | The text above is information relevant to the application of Sub-Section IIC and therefore is more appropriately located at the beginning of the Sub-Section with the clarification it applies to Nos. 9.7 to 9.21. |
|  | 9.50 An administration having received a request for coordination under Nos. 9.7 to 9.21, or having been included in the procedure following action under No. 9.41, shall promptly examine the matter with regard to interference which may be caused to or, in certain cases, by its own assignments20, identified in accordance with Appendix 521. |
|  | 9.51 Following its action under No. 9.50, the administration with which coordination was sought under Nos. 9.7 to 9.7B shall, within four months of the date of publication of the BR IFIC under No. 9.38, either inform the requesting administration and the Bureau of its agreement or act under No. 9.52. (WRC-2000) |
|  | 9.51A Following its action under No. 9.50, the administration with which coordination was sought under Nos. 9.15 to 9.19 shall, within four months of the date of dispatch of the coordination data under No. 9.29, either inform the requesting administration of its agreement or act under No. 9.52. |
| ADD | In the case of disagreement |
| Reasons | Clarification of the purpose of the following group of provisions. |

|  | 9.52 If an administration, following its action under No. 9.50, does not agree to the request for coordination, it shall, within four months of the date of publication of the BR IFIC under No. 9.38, or of the date of dispatch of the coordination data under No. 9.29, inform the requesting administration of its disagreement and shall provide information concerning its own assignments upon which that disagreement is based. It shall also make such suggestions as it is able to offer with a view to satisfactory resolution of the matter. A copy of that information shall be sent to the Bureau. Where the information relates to terrestrial stations or earth stations operating in the opposite direction of transmission within the coordination area of an earth station, only that information relating to existing radiocommunication stations or to those to be brought into use within the next three months for terrestrial stations, or three years for earth stations, shall be treated as notifications under Nos. 11.2 or 11.9. |
| --- | --- |
|  | 9.52A In the case of coordination requested under No. 9.14, on receipt of the Special Section of the BR IFIC referred to in No. 9.38, and within the same four-month period from the publication of that Special Section, an administration in need of assistance may inform the Bureau that it has existing or planned terrestrial stations which might be affected by the planned satellite network, and may request the Bureau to determine the need for coordination by applying the Appendix 5 criteria. The Bureau shall inform the administration seeking coordination of this request, indicating the date by which it may be able to provide the results of its analysis. When these results are available, the Bureau shall inform both administrations. This request shall be considered as a disagreement, pending the results of the analysis by the Bureau of the need for coordination. |
| ADD | 9.52Abis [9.59] Under Nos. 9.7 to 9.21, if the disagreement between the administration seeking coordination and an administration with which coordination is sought concerns the level of acceptable interference, either may seek the assistance of the Bureau; in such a case, it shall provide the necessary information to enable the Bureau to endeavour to effect such coordination. |
| Reasons | The text above is more appropriate to this point in the procedure and with the clarification it applies to Nos. 9.7 to 9.21. |
|  | 9.52B When an agreement on coordination is reached, the administration responsible for the terrestrial stations or the earth station operating in the opposite direction of transmission may send to the Bureau the information concerning those stations covered by the agreement which are intended to be notified under Nos. 11.2 or 11.9. The Bureau shall consider as notifications only that information relating to existing terrestrial or earth stations operating in the opposite direction of transmission or to those to be brought into use within the next three years. |
| SUP* | - |
| Reasons | This provision relates to no response and is therefore more appropriate in Sub-Section IID, which specifically deals with that situation. |

| SUP* |  |
| --- | --- |
| Reasons | This provision follows on from No. 9.52 C which relates to no response and it is therefore also more appropriate in Sub-Section IID. |
| SUP |  |
| Reasons | The text above is information relevant to the application of Sub-Section IIC and therefore is more appropriately located at the beginning of the Sub-Section - see No. 9.49bis. |
|  | 9.53A Upon expiry of the deadline for comments in respect of a coordination request under Nos. 9.11 to 9.14 and 9.21, the Bureau shall, according to its records, publish a Special Section, indicating the list of administrations having submitted their disagreement or other comments within the regulatory deadline. (WRC-2000) |
| SUP |  |
| Reasons | Merged with No. 9.44. |
| ADD | Revision of published satellite network data |
| Reasons | Clarification of the purpose of the following group of provisions. |
|  | 9.55 All administrations may use correspondence, any appropriate means of telecommunication or meetings, as necessary, to assist in resolving the matter. The results thereof shall be communicated to the Bureau, which shall publish them in the BR IFIC, as appropriate. |
|  | 9.56 and 9.57 Not used. |
|  | 9.58 An administration which has initiated coordination, as well as any administration with which coordination is sought, shall communicate to the Bureau any modifications to the published characteristics of their respective networks that were required to reach agreement on the coordination. The Bureau shall publish this information in accordance with No. 9.38, indicating that these modifications resulted from the joint effort of the administrations concerned to reach agreement on coordination and that, for this reason, they should be given special consideration. These modifications may involve the application of Sub-Section IIA of Article 9 with respect to other administrations. |

| SUP |  |
| --- | --- |
| Reasons | The text above is more appropriately located earlier in the procedure - see No. 9.52Abis. |

Sub-Section IID  –  Action in the event of no reply, no decision or disagreement
on a request for coordination

| ADD | Time periods for response |
| --- | --- |
| Reasons | Clarification of the purpose of the following group of provisions. |
| ADD* | 9.59bis [9.52C] For coordination requests under Nos. 9.11 to 9.14 and 9.21, an administration not responding under No. 9.52 within the same four-month period shall be regarded as unaffected and, in the cases of Nos. 9.11 to 9.14, the provisions of Nos. 9.48 and 9.49 shall apply. |
| Reasons | This provision relates to no response and is therefore more appropriate in Sub-Section IID, which specifically deals with that situation. |
| ADD* | 9.59ter [9.52D] For coordination requests under Nos. 9.12 to 9.14, 45 days prior to the expiry of the same four-month period the Bureau shall dispatch a circular-telegram to all administrations, bringing the matter to their attention. Upon receipt of the aforementioned circular-telegram, an administration shall acknowledge receipt immediately by telegram. If no acknowledgement is received within 30 days, the Bureau shall dispatch a telegram requesting acknowledgement, to which the receiving administration shall reply within a further period of 15 days. |
| Reasons | This provision follows on from No. 9.52 C which relates to no response and it is therefore also more appropriate in Sub-Section IID. |
| MOD | 9.60 If, within the same four-month period specified in Nos. 9.51 or 9.51A, an administration with which coordination is sought under Nos. 9.7 to 9.7B or 9.15 to 9.19 fails to reply or to give a decision under Nos. 9.51 or 9.51A or, following its disagreement under No. 9.52 (for Nos. 9.7 to 9.21), fails to provide information concerning its own assignments on which its disagreement is based, the requesting administration may seek the assistance of the Bureau. (WRC-2000) |
| Reasons | To clarify the application of No. 9.60 to Nos. 9.15 to 9.19 is not linked to coordination under No. 9.7 to 9.7B. Also to clarify that the procedure following disagreement under No. 9.52 applies to Nos. 9.7 to 9.21. |
|  | 9.61 The Bureau, acting on a request for assistance under No. 9.60, shall forthwith request the administration concerned to give an early decision in the matter or provide the relevant information. |
|  | 9.62 If the administration concerned still fails to respond within thirty days of the Bureau's action under No. 9.61, the provisions of Nos. 9.48 and 9.49 shall apply. |

| ADD | Action in the event of continuing disagreement |
| --- | --- |
| Reasons | Clarification of the purpose of the following group of provisions. |
|  | 9.63 If there is continuing disagreement, or if any administration involved in the matter has requested the assistance of the Bureau, the Bureau shall seek any necessary information to enable it to assess the interference. It shall communicate its conclusions to the administrations involved. |
|  | 9.64 If the disagreement remains unresolved after the Bureau has communicated its conclusions to the administrations involved, the administration which requested coordination shall, having regard to the other provisions of this Section, defer the submission of its notice of frequency assignments under Article 11 to the Bureau for six months from the date of the request or the BR IFIC containing the request for coordination, as appropriate. |
|  | 9.65 If, at the date of receipt of a notice under No. 9.64 above, the Bureau has been informed of a continuing disagreement, the Bureau shall examine the notice under Nos. 11.32A or 11.3322 and shall act in accordance with No. 11.38. |

______________

1	A.9.1		For the application of the provisions of this Article with respect to stations in a space radiocommunication service using frequency bands covered by the fixed-satellite service allotment Plan, see also Appendix 30B.

2	A.9.2		These procedures may be applicable to stations on board satellite launching vehicles.

3	A.9.3		See Appendices 30 and 30A, as appropriate, for the coordination of:

a)	proposed modifications to the Appendix 30 Plans for the broadcasting-satellite service in the frequency bands 11.7-12.2 GHz (in Region 3), 11.7-12.5 GHz (in Region 1) and 12.2-12.7 GHz (in Region 2), or new or modified assignments proposed for inclusion in the Regions 1 and 3 List of additional uses, with respect to frequency assignments in the same service or in other services to which these bands are allocated;

b)	frequency assignments in other services to which the frequency bands referred to in § a) above are allocated in the same Region or in another Region, with respect to assignments in the broadcasting-satellite service in the frequency bands 11.7-12.2 GHz (in Region 3), 11.7-12.5 GHz (in Region 1) and 12.2-12.7 GHz (in Region 2);

c)	proposed modifications to the Appendix 30A Plans for feeder links to the broadcasting-satellite service in the frequency bands 17.3-17.8 GHz (in Region 2) and 14.5-14.8 GHz and 17.3-18.1 GHz (in Regions 1 and 3), or new or modified assignments proposed for inclusion in the Regions 1 and 3 Lists of additional uses, with respect to frequency assignments in the same service or in other services to which these bands are allocated;

d)	frequency assignments in other services to which the frequency bands referred to in § c) above are allocated in the same Region or in another Region, with respect to assignments in the fixed-satellite service (Earth-to-space) in the frequency bands 17.3-17.8 GHz (in Region 2) and 14.5-14.8 GHz and 17.3-18.1 GHz (in Regions 1 and 3).

For the broadcasting-satellite service and for feeder links for the broadcasting-satellite service in the fixed-satellite service in Region 2, Resolution 42 (Rev.Orb-88) is also applicable.     (WRC-2000)

4	A.9.4		Resolution 49 (Rev.WRC-2000) shall also be applied with respect to those satellite networks and satellite systems that are subject to it.     (WRC-2000)

5	A.9.5		See also Resolution 51 (Rev.WRC-2000).     (WRC-2000)

6	A.9.6		The provisions of Appendices 30, 30A and 30B do not apply to non-geostationary service-satellite systems in the fixed-satellite.     (WRC-2000)

7	9.1.1		Whenever, under this provision, an administration acts on behalf of a group of named administrations, all members of that group retain the right to respond in respect of their own networks or systems.

8	9.2B.1	If the payments are not received in accordance with the provisions of Council Decision 482, as amended, on the implementation of cost recovery for satellite network filings, the Bureau shall cancel the publication, after informing the administration concerned. The Bureau shall inform all administrations of such action, and that the network specified in the publication in question no longer has to be taken into consideration by the Bureau and other administrations. The Bureau shall send a reminder to the notifying administration not later than 60 days prior to due date of the payment if payment has not been received by that date. This provision was identified in reply to Resolution 88 (Minneapolis, 1998) of the Plenipotentiary Conference and shall enter into force at a date to be determined by the forthcoming Plenipotentiary Conference.     (WRC-2000)

9	9.5B.1	The only terrestrial stations to be taken into account are those for which the requirement to coordinate is under Nos. 9.11, 9.11A and 9.21.

1	0	A.9.II.1	These procedures are also applicable for earth stations of the Earth exploration-satellite, space research, space operation and radiodetermination-satellite services intended to be used while in motion or during halts at unspecified points.

1	1	A.9.II.2	The word "coordination" as used throughout this Article refers also to the process of seeking an agreement of other administrations when required under No. 9.21.

1	2	9.6.1		In the case of coordination of an assignment in a satellite network, an administration may act on behalf of a group of named administrations. Whenever, under this provision, an administration acts on behalf of a group of named administrations, all members of the group retain the right to respond in respect of their own services which could affect or be affected by the proposed assignment.

1	3	9.6.2		In all cases, the coordination of an earth station with terrestrial stations or other earth stations operating in the opposite direction of transmission shall remain within the authority of the administration on the territory of which this station is located.

1	4	9.7A.1 and 9.7B.1  The coordination of a specific earth station under Nos. 9.7A or 9.7B shall remain within the authority of the administration on whose territory the station is located.     (WRC-2000)

1	5	9.7A.2 and 9.7B.2  Coordination information relating to a specific earth station received by the Bureau prior to 30 June 2000 is considered as complete information under Nos. 9.7A or 9.7B from the date of receipt of complete information for the associated satellite network under No. 9.7, provided that the maximum isotropic antenna gain, the lowest total receiving system noise temperature of the earth station and the necessary bandwidth of the emission received by the earth station are equal to those of any typical earth station included in the coordination request for the geostationary-satellite network in the fixed-satellite service.     (WRC-2000)

1	6	9.35.1	The Bureau shall include the detailed results of its examination under No. 11.31 of compliance with the limits in Tables 22-1 to 22-3 of Article 22 in the publication under No. 9.38.     (WRC-2000)

1	7	9.36.1	The list of administrations identified by the Bureau under Nos. 9.11 to 9.14 and 9.21 is only for information purposes, to help administrations comply with this procedure.

1	8	9.36.2	In the case of coordination under Nos. 9.7, 9.7A and 9.7B, the Bureau shall also identify the specific satellite networks or earth stations with which coordination needs to be effected. In the case of coordination under No. 9.7 the list of the networks identified by the Bureau under No. 9.6bis is for information purposes only, to help administrations comply with this procedure.     (WRC-2000)

1	9	9.38.1	If the payments are not received in accordance with the provisions of Council Decision 482, as amended, on the implementation of cost recovery for satellite network filings, the Bureau shall cancel the publication, after informing the administration concerned. The Bureau shall inform all administrations of such action and that the network specified in the publication in question no longer has to be taken into consideration by the Bureau and other administrations. The Bureau shall send a reminder to the notifying administration not later than 60 days prior to due date of the payment if payment has not been received by that date. This provision was identified in reply to Resolution 88 (Minneapolis, 1998) of the Plenipotentiary Conference and shall enter into force at a date to be determined by the forthcoming Plenipotentiary Conference.     (WRC-2000)

2	0	9.50.1	In the absence of specific provisions in these Regulations relating to the evaluation of interference, the calculation methods and the criteria should be based on relevant ITU-R Recommendations agreed by the administrations concerned. In the event of disagreement on a Recommendation or in the absence of such a Recommendation, the methods and criteria shall be agreed between the administrations concerned. Such agreements shall be concluded without prejudice to other administrations.

2	1	9.50.2	Where Appendix 5 specifies a period for which planned assignments may be taken into account, that period may be extended by agreement between the administrations concerned.

2	2	9.65.1	A notice of a frequency assignment for which coordination was requested under No. 9.21 and in respect of which there is continuing disagreement shall not be examined under Nos. 11.32A or 11.33; it shall, however, be examined under No. 11.31.