| INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 1997 - 2000 | COM 11-R 103-E June 1998 Original: English |

Question: 12/11

E

STUDY GROUP 11 – REPORT R 103

SOURCE*:	STUDY GROUP 11

TITLE:	IMPLEMENTORS' GUIDE (05/98) FOR RECOMMENDATION Q.732.7 (07/96)

________

# 1	Introduction

## 1.1	References

[1]	ITU-T Recommendation Q.732.7 (07/96)

[2]	ITU-T Recommendation Q.115 (01/97)

[3]	ITU-T Recommendation Q.764 (09/97)

## 1.2	Background

This guide is a compilation of amendments to the 1996 edition of the ITU-T Recommendation Q.732.7 [1]. It includes all approved corrigenda and is intended to be an additional authoritative source of information for implementors to be read in conjunction with the Recommendation itself.

## 1.3	Scope of the guide

This guide records the resolutions of defects in the following categories:

•	editorial errors;

•	technical errors, such as omissions, inconsistencies, etc.; and

•	ambiguities.

## 1.4	Contacts

Editor:	Pieter Veenstra	Tel:	+31 70 3439121

KPN Telecom B.V.	Fax:	+31 70 3437303

P.O. Box 30150	Email:	p.k.veenstra@kpn-telecom.nl

2500 GD The Hague

The Netherlands

Document history

| Version | Summary |
| --- | --- |
| 05/98 | New Implementors' Guide |

# 2	Editorial errors

None.

# 3	Technical errors

## 3.1	Echo control related procedures

Synopsis

New echo control logic procedures have been specified in ITU-T Recommendation Q.115 [2] in conjunction with new echo control signalling procedures in ITU-T Recommendation Q.764 [3]. As a result the echo control related statements in the procedures for Explicit Call Transfer have become outdated. As a consequence, the echo control related text phrases in § 7.5.2.1.1.3 a) and § 7.5.2.1.1.3 b) have to be replaced by new text.

Please replace the following text phrases in ITU-T Recommendation Q.732.7 [1]:

###### 7.5.2.1.1.3	Requirements related to echo control

a)	General

The originating local exchange should have the capability to invoke echo control procedures. This is necessary in case the total propagation delay for the two legs of the transferred call is above the value where echo control is necessary (see 2.6/Q.764 [12]).

The exchange should also have the capability of storing propagation delay information received either in the Initial address message (for incoming calls) or in the Answer/Connect message (for outgoing calls) until call release. This must be done for both legs of the transferred call.

b)	Criteria to initiate echo control procedures

The originating local exchange has to sum up the propagation delay of the calls A-B and A-C in order to determine the total value of the propagation delay of the transferred call.

If echo control is necessary, the exchange shall initiate echo control procedures for the transferred call (see 2.7/Q.764 [12]).

by:

###### 7.5.2.1.1.3	Requirements related to echo control

The echo control logic as defined in ITU-T Recommendation Q.115 (01/97) shall apply. The exchange shall also have the capability of storing propagation delay information received either in the Initial Address message (for incoming calls) or in the Answer/Connect message (for outgoing calls) until call release. This shall be done for both legs of the transferred call.

NOTE - Optimal placement of the echo control devices is not guaranteed with the echo control logic as defined in ITU-T Recommendations Q.115 (01/97) and the echo control signalling procedures as defined in ITU-T Recommendation Q.764 (09/97).

# 4	Ambiguities

None.

_________________