|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T

TELECOMMUNICATION	
STANDARDIZATION  SECTOR
OF  ITU

STUDY GROUP 4

G.85x SERIES

TMN IMPLEMENTORS' GUIDE -

DEFECTS AND RESOLUTIONS

VERSION 1.1 (19/1/2001)

ABSTRACT

This document is a compilation of reported defects  in the 1996 and 1999 editions of the ITU-T G.85x series of Recommendations. It is intended to be an additional authoritative source of information for implementors to be read in conjunction with the Recommendations themselves.  Further, the Guide includes defect resolutions, that will be corrected in the next editions of the relevant Recommendations.

Preface

The purpose of this Guide is to help the implementors of the 1996 and 1999 ITU-T Recommendations on TMN network level modelling of transport networks based on ODP (Open Distributed Processing). This Guide is not part of those Recommendations, but may be used in their ongoing maintenance.

This is the first version of this guide, which was produced following the 2000 WP 4/4 meeting.

Items marked with an asterisk (*) will be added to the Guide or modified for the next Version.

Every change to the Recommendations are identified with bullets having reference numbers of the form xyz where x is a letter which corresponds to one of the ITU-T G.85x series Recommendations, y is a letter which corresponds to one of the ITU-T G.85x.y Recommendations and z is a number which identifies the particular change in the context of that Recommendation.

At the end of each bullet a defect report number (DR) is included if appropriate. This defect report number is used in Appendix A, which is  a register of defect reports raised and their current status. Note that a single bullet may relate to more than one defect report, or a single defect report may result in more than one bullet being generated.

Wide distribution of this document is expected and encouraged.  The latest version of this Guide will be available on the Word Wide Web server of the ITU (http://www.itu.ch) below the ITU-T SG 4 entry.

This Guide is published in the spirit of international communication and co-operation. However the authors assume no responsibility for the accuracy of the information it contains or for the consequences arising from its use.

TABLE OF CONTENTS

1. Introduction

1. 1. Background

1. 2. Scope of the Guide

1. 3. Contacts and Distribution of the Guide

2. Defect Report and Resolution Procedures

2. 1. Submission of defects

2. 2. Resolution of Defects

2. 3. Defect report register

3. Implementation Guidance

A Changes to Rec G.851 series

Aa Changes to Rec G.851.1 (1996)

B Changes to Rec G.852 series	11

Ba Changes to Rec G.852.1 (1996)

Bb Changes to Rec G.852.2 (1999)

Bc Changes to Rec G.852.3 (1999)

Bd Changes to Rec G.852.6 (1999)

Be Changes to Rec G.852.8 (1999)

Bf Changes to Rec G.852.10 (1999)

Bg Changes to Rec G.852.12 (1999)

C Changes to Rec G.853 series	11

Ca Changes to Rec G.853.1 (1999)

Cb Changes to Rec G.853.2 (1996)

Cc Changes to Rec G.853.3 (1999)

Cd Changes to Rec G.853.6 (1999)

Ce Changes to Rec G.853.8 (1999)

Cf Changes to Rec G.853.10 (1999)

Cg Changes to Rec G.853.12 (1999)

D Changes to Rec G.854 series	11

Da Changes to Rec G.854.1 (1996)

Db Changes to Rec G.854.3 (1999)

Dc Changes to Rec G.854.6 (1999)

Dd Changes to Rec G.854.8 (1999)

De Changes to Rec G.854.10 (1999)

Df Changes to Rec G.854.12 (1999)

E Changes to Rec G.855 series	11

Ea Changes to Rec G.855.1 (1999)

Appendix A TMN Defect Report Register	18

Appendix B TMN Defect Report Form	21

# Introduction

## Background

This Guide concerns the ITU-T G.85x series of Recommendations on Telecommunications Management Network (TMN).

This Guide is informal in nature and the Guide is not an ITU-T Recommendation. The information it contains will serve as an information source for the ITU-T SG 4, who is responsible for maintaining the G.85x series of TMN Recommendations, and other users both within and outside of the ITU-T. The changes are expected to be included into future versions of the Recommendations.

## Scope of the Guide

This guide resolves defects in (only) the following categories:

- editorial errors;

- technical errors, such as omissions or inconsistencies;

- ambiguities.

In addition the Guide may include explanatory text found necessary as a result of interpretation difficulties apparent from the defect reports.

Note:	This Guide will not address proposed additions, deletions or modifications to the Recommendations that are not strictly related to implementation difficulties in the above categories.

Initially, the Guide is limited to defects concerning information models and implementation conformance statements proformas.

## Contacts and Distribution of the Guide

This Guide will be made available at ITU-T SG4 meetings as well as meeting of ITU-T WP4/4. In addition copies of this Guide, can in general, also be made available from one's national representative for ITU-T. Copies may also be obtained from other agencies.

Contacts:

ITU-T WP 4/4 Chairman

Lakshmi G. Raman 	Tel. +1 612 946 2090

ADC Telecommunications	Fax. +1 612 946 3590

4900 West 78th Street 	Email: lakshmi@adc.com

Minneapolis, Minnesota 55435

United States

ITU-T Q 18/4 Rapporteur

Terje Henriksen 	Tel. +47 63 84 86 82

Telenor R&D	Fax. +47 63 81 00 76

PO  box 83	Email: terje-fredrik.henriksen@telenor.com

N2007 Kjeller

NORWAY

Implementors' Guide Editor

Tatsuhiko Yoshida	Tel. +81 422 59 6736

NTT Network Service Systems Laboratories	Fax. +81 422 59 4475

9-11, Midori-Cho 3-Chome	Email:	Yoshida.Tatsuhiko@lab.ntt.co.jp

Musashino-Shi, Tokyo 180-8585

Japan

ITU-T SG4 Secretariat

Ardhendu M. Ganguli					Tel. +41 22 730 58 82

Study Group 4						Fax. +41 22 730 58 53

Telecommunications Standardization Bureau			Email: ganguli@itu.ch

International Telecommunications Union

Place des Nations

CH-1211 Geneva 20

Switzerland

Defect group leaders

This list includes the names of leaders for Recommendations. Not all of the Recommendations may have defects raised against them and leaders are not identified for all Recommendations. However, whenever a defect is raised, a defect group leader will be identified for the relevant Recommendation.

| Recommendation | Defect group leader |  |
| --- | --- | --- |
| Rec. G.851.1 Management of the transport network - Application of the RM-ODP framework | Terje Henriksen Telenor R&D PO box 83 N2007 Kjeller NORWAY | Tel. +47 63 84 86 82 Fax. +47 63 81 00 76 Email: terje-fredrik.henriksen@telenor.com |
| Rec. G.852.1 Enterprise viewpoint for simple subnetwork connection management | Terje Henriksen Telenor R&D PO box 83 N2007 Kjeller NORWAY | Tel. +47 63 84 86 82 Fax. +47 63 81 00 76 Email: terje-fredrik.henriksen@telenor.com |
| Rec. G.852.2 Enterprise viewpoint description of transport network resource model | Tatsuhiko Yoshida NTT Network Service Systems Laboratories 9-11, Midori-Cho 3-Chome Musashino-Shi, Tokyo 180-8585 Japan | Tel. +81 422 59 6736 Fax. +81 422 59 4475 Email: Yoshida.Tatsuhiko@lab.ntt.co.jp |
| Rec. G.852.3 Enterprise viewpoint for topology management | Andy Walsh Telcordia Technologies 331 Newman Springs Rd. Red Bank, NJ 07701 USA | Tel. +1-732-758-5648 Fax. +1-732-758-4344 Email: awalsh@telcordia.com |
| Rec. G.852.6 Enterprise viewpoint for trail management | Terje Henriksen Telenor R&D PO box 83 N2007 Kjeller NORWAY | Tel. +47 63 84 86 82 Fax. +47 63 81 00 76 Email: terje-fredrik.henriksen@telenor.com |
| Rec. G.852.8 Enterprise viewpoint for pre-provisioned adaptation management | Terje Henriksen Telenor R&D PO box 83 N2007 Kjeller NORWAY | Tel. +47 63 84 86 82 Fax. +47 63 81 00 76 Email: terje-fredrik.henriksen@telenor.com |
| Rec. G.852.10 Enterprise viewpoint for pre-provisioned link connection management | Bernd Zeuner Deutsche Telekom T-Nova, TZ Darmstadt 64307 Darmstadt Germany | Tel. +49 61 51 83 37 09 Fax. +49 61 51 83 19 875 Email: bernd.zeuner@telekom.de |
| Rec. G.852.12 Enterprise viewpoint for pre-provisioned link management | Bernd Zeuner Deutsche Telekom T-Nova, TZ Darmstadt 64307 Darmstadt Germany | Tel. +49 61 51 83 37 09 Fax. +49 61 51 83 19 875 Email: bernd.zeuner@telekom.de |
| Rec. G.853.1 Common elements of the information viewpoint for the management of a transport network | Terje Henriksen Telenor R&D PO box 83 N2007 Kjeller NORWAY | Tel. +47 63 84 86 82 Fax. +47 63 81 00 76 Email: terje-fredrik.henriksen@telenor.com |
| Rec. G.853.2 Subnetwork connection management information viewpoint | Terje Henriksen Telenor R&D PO box 83 N2007 Kjeller NORWAY | Tel. +47 63 84 86 82 Fax. +47 63 81 00 76 Email: terje-fredrik.henriksen@telenor.com |
| Rec. G.853.3 Information viewpoint for topology management | Andy Walsh Telcordia Technologies 331 Newman Springs Rd. Red Bank, NJ 07701 USA | Tel. +1-732-758-5648 Fax. +1-732-758-4344 Email: awalsh@telcordia.com |
| Rec. G.853.6 Information viewpoint for trail management | Terje Henriksen Telenor R&D PO box 83 N2007 Kjeller NORWAY | Tel. +47 63 84 86 82 Fax. +47 63 81 00 76 Email: terje-fredrik.henriksen@telenor.com |
| Rec. G.853.8 Information viewpoint for pre-provisioned adaptation management | Terje Henriksen Telenor R&D PO box 83 N2007 Kjeller NORWAY | Tel. +47 63 84 86 82 Fax. +47 63 81 00 76 Email: terje-fredrik.henriksen@telenor.com |
| Rec. G.853.10 Information viewpoint for pre-provisioned link connection management | Bernd Zeuner Deutsche Telekom T-Nova, TZ Darmstadt 64307 Darmstadt Germany | Tel. +49 61 51 83 37 09 Fax. +49 61 51 83 19 875 Email: bernd.zeuner@telekom.de |
| Rec. G.853.12 Information viewpoint for pre-provisioned link management | Bernd Zeuner Deutsche Telekom T-Nova, TZ Darmstadt 64307 Darmstadt Germany | Tel. +49 61 51 83 37 09 Fax. +49 61 51 83 19 875 Email: bernd.zeuner@telekom.de |
| Rec. G.854.1 Computational interfaces for basic transport network model | Terje Henriksen Telenor R&D PO box 83 N2007 Kjeller NORWAY | Tel. +47 63 84 86 82 Fax. +47 63 81 00 76 Email: terje-fredrik.henriksen@telenor.com |
| Rec. G.854.3 Computational viewpoint for topology management | Andy Walsh Telcordia Technologies 331 Newman Springs Rd. Red Bank, NJ 07701 USA | Tel. +1-732-758-5648 Fax. +1-732-758-4344 Email: awalsh@telcordia.com |
| Rec. G.854.6 Computational viewpoint for trail management | Terje Henriksen Telenor R&D PO box 83 N2007 Kjeller NORWAY | Tel. +47 63 84 86 82 Fax. +47 63 81 00 76 Email: terje-fredrik.henriksen@telenor.com |
| Rec. G.854.8 Computational viewpoint for pre-provisioned adaptation management | Terje Henriksen Telenor R&D PO box 83 N2007 Kjeller NORWAY | Tel. +47 63 84 86 82 Fax. +47 63 81 00 76 Email: terje-fredrik.henriksen@telenor.com |
| Rec. G.854.10 Computational viewpoint for pre-provisioned link connection management | Bernd Zeuner Deutsche Telekom T-Nova, TZ Darmstadt 64307 Darmstadt Germany | Tel. +49 61 51 83 37 09 Fax. +49 61 51 83 19 875 Email: bernd.zeuner@telekom.de |
| Rec. G.854.12 Computational viewpoint for pre-provisioned link management | Bernd Zeuner Deutsche Telekom T-Nova, TZ Darmstadt 64307 Darmstadt Germany | Tel. +49 61 51 83 37 09 Fax. +49 61 51 83 19 875 Email: bernd.zeuner@telekom.de |
| Rec. G.855.1 GDMO engineering viewpoint for the generic network level model | Terje Henriksen Telenor R&D PO box 83 N2007 Kjeller NORWAY | Tel. +47 63 84 86 82 Fax. +47 63 81 00 76 Email: terje-fredrik.henriksen@telenor.com |

1.4	History log

| Item | Location | Reason |
| --- | --- | --- |
| 1.0 | Geneva, Februry 2000 | First version of the Guide |
| 1.1 | Geneva, Janaury 2000 | To update G.855.1 errors in order to reflect M.3120 and M.3100 discussions in August 2000 WP 4/4 and 5/4 meeting |

# Defect Report and Resolution Procedures

## Submission of defects

Any implementor of the TMN Recommendations is invited to submit TMN defect report using the form in Appendix B of the Guide. The defect should be submitted to the ITU-T SG4 Secretariat and copied to the ITU-T WP4/4 Chairman. Each form should cover a single defect. It is important that the form is completed accurately, especially the sections which relate to the base material against which the defect report is being raised.

## Resolution of Defects

A TMN defect resolution group is established for each of the Recommendations. In some cases a group covers more than one Recommendation.

Following agreement on a resolution, within the defect resolution group, the proposed resolution may require approval of ITU-T WP4/4 and ITU-T SG4.

This Guide will contain resolutions as they are agreed by the defect resolution group. The status of each will be reflected in Appendix A of the Guide and any modifications required to the resolutions themselves prior to final approval, will be reflected in Section 3 of the Guide.

Please note that individual responses can not be given to an individual submitting defect reports, and that the procedure is not intended as a consulting service.

## Defect report register

New defect reports will be included in the report of the relevant Question or Working Party.

# Implementation Guidance

Remember that this Guide is intended to be an authoritative source of information for implementors of the TMN Recommendations, however it is not itself an ITU-T Recommendation.

Items marked with an asterisk (*) were added to the Guide or modified for this version of the Guide.

Bullets have reference numbers of the form xyz where x is a letter which corresponds to one of the ITU-T G.85x series Recommendations, y is a letter which corresponds to one of the ITU-T G.85x.y Recommendations and z is a number which identifies the particular change in the context of that Recommendation.

At the end of each bullet title a defect report number (DR) is included if appropriate. This defect report number is used in Annex A, which is  a register of defect reports raised and their current status. Note that a single bullet may relate to more than one defect report, or a single defect report may result in more than one bullet being generated.  Text contained in this clause shall only be considered final when the associated changes are part of a revised Recommendation.

##### Changes to Rec G.851 series

Aa	Changes to Rec G.851.1 (1996)

None

##### Changes to Rec G.852 series

Ba	Changes to Rec G.852.1 (1996)

None

Bb	Changes to Rec G.852.2 (1999)

None

Bc	Changes to Rec G.852.3 (1999)

None

Bd	Changes to Rec G.852.6 (1999)

Bd1	DR-G85xTm-1

Section 6.4.1.  Set up point-to-point trail

It was found that the following Action Policy was missing, when ‘monitored trail management’ was being studied at the Darmstadt meeting in August 1999.     Therefore, it must be added.

PERMISSION directionality

“The caller may specify the directionality of the trail, i.e., unidirectional or bidirectional.”

Be	Changes to Rec G.852.8 (1999)

None

Bf	Changes to Rec G.852.10 (1999)

None

Bg	Changes to Rec G.852.12 (1999)

None

##### Changes to Rec G.853 series

Ca	Changes to Rec G.853.1 (1996)

Ca2	DR-G85xCiv-1

New Section  7.9 operationalState

It was found that this attribute was missing, when ‘monitored trail management’ was being studied at the Darmstadt meeting in August 1999.    The following new section should be added.

7.9 operationalState

DEFINITION

“The operationaState attribute characterises the operability of the associated resource.   The semantic of this attribute is imported from X.721: 1992 operationaState attribute.”

STATE

disabled

“The resource is totally inoperable and unable to provide service to the user(s).”

enabled

“The resource is partially or fully operable and available for use.”

Ca3	DR-G85xCiv-2

Section  8.22   linkEndHasNetworkCTPs

It was found that topologicalLinkEnd should be added to linkEndHasNetworkCTPs relationship, when ‘sub-network connection protection management’ was being studied at the Darmstadt meeting in August 1999.    The underlined  ‘topologicalLinkEnd’ should be added to containerLE as follows:

This relationship type is related to the following enterprise entity:

<COMMUNITY:tem, ROLE:link end, PROPERTY:pre-provisioned capacity>

DEFINITION

"The linkEndHasNetworkCTPs relationship class describes the relationship that exists between a linkEnd/topologicalLinkEnd and the networkCTPs that are part of it."

ROLE

containerLE

"Played by an instance of the <linkEnd> or < topologicalLinkEnd > information object type or subtype."

elementCTP

"Played by instances of  the <networkCTP> information object type or subtype."

Ca4	DR-G85xCiv-3

Section  6.9   networkCTP

The change was proposed in support of protection management requirements at the Darmstadt meeting in August 1999.       The underlined  <networkCTPIsProtected> should be added to POTENTIAL_RELATIONSHIPS as follows:

POTENTIAL_RELATIONSHIPS

<linkConnectionIsBoundTo>

<linkConnectionIsTerminatedByPointToPoint>

<linkEndHasNetworkCTPs>

<networkCTPIsBundleOfNetworkCTPs>

<networkTTPAdaptsNetworkCTP>

<subnetworkTPIsRelatedToExtremity>

<networkCTPIsProtected>

Ca5	DR-G85xCiv-3

Section  6.18   networkTTP

The change was proposed in support of protection management requirements at the Darmstadt meeting in August 1999.       The underlined  <networkTTPIsProtected> should be added to POTENTIAL_RELATIONSHIPS as follows:

POTENTIAL_RELATIONSHIPS

<accessGroupIsMadeOfNetworkTTPs>

<linkConnectionIsBoundTo>

<networkTTPAdaptsNetworkCTP>

<subnetworkTPIsRelatedToExtremity>

<topologicalLinkEndIsSupportedByNetworkTTP>

<trailIsTerminatedByPointToPoint>

<networkTTPIsProtected>

Ca6	DR-G85xCiv-3

Section  6.26   subnetworkConnection

The change was proposed in support of protection management requirements at the Darmstadt meeting in August 1999.       The underlined  <subnetworkConnectionIsProtected> should be added to POTENTIAL_RELATIONSHIPS as follows:

POTENTIAL_RELATIONSHIPS

<linkConnectionIsMadeOfTransportEntities>

<sncBidIsSupportedByUnis>

<subnetworkConnectionIsBundleOfSubnetworkConnections>

<subnetworkConnectionIsMadeOfTransportEntities>

<subnetworkConnectionIsTerminatedByPointToPoint>

<subnetworkHasSubnetworkConnections>

<tandemConnectionIsMadeOfTransportEntities>

<trailIsMadeOfTransportEntities>

<subnetworkConnectionIsProtected>

Ca7	DR-G85xCiv-3

Section  6.35   trail

The change was proposed in support of protection management requirements at the Darmstadt meeting in August 1999.       The underlined  <tailIsProtected> should be added to POTENTIAL_RELATIONSHIPS as follows:

POTENTIAL_RELATIONSHIPS

<linkConnectionIsSupportedByTrail>

<topologicalLinkIsSupportedByTrail>

<trailIsBundleOfTrails>

<trailIsMadeOfTransportEntities>

<trailIsTerminatedByPointToPoint>

<trailIsProtected>

Ca8	DR-G85xCiv-3

Section  8   Information relationship type definitions

The change was proposed in support of protection management requirements at the Darmstadt meeting in August 1999.     The following relationship types, ‘networkCTPIsProtected’, ‘networkCTPIsProtected’, ‘networkCTPIsProtected’ and ‘networkCTPIsProtected’, should  be added to sectin 8.

8.xx  networkCTPIsProtected

This relationship type is related to the following enterprise entity:

<COMMUNITY:tem, ROLE: connection termination point, PROPERTY:protection>

DEFINITION

ROLE

protectedNetworkCTP

“Played by an instance of the <networkCTP> or information object type or subtype.”

protectingNetworkCTP

“Played by an instance of the <networkCTP> information object type or subtype.”

workerNetworkCTP

“Played by an instance of the <networkCTP> information object type or subtype.”

INVARIANT

inv_protectedCTPRoleCardinality

"One and only one instance of the role protectedNetworkCTP must participate in the relationship."

inv_protectingCTPRoleCardinality

"One and only one instance of the role protectingNetworkCTP must participate in the relationship."

inv_workerCTPRoleCardinality

"One and only one instance of the role workerNetworkCTP must participate in the relationship."

inv_roles

"In an instance of the relationship, an instance can not play both roles: protectingNetworkCTP and workerNetworkCTP."

inv_signalIdentification

"In a given relationship instance of networkCTPIsProtected, the information objects playing the role protectedNetworkCTP, protectingNetworkCTP, and workerNetworkCTP must have all the same signalIdentification value."

inv_directionality

"The objects involved in the relationship must have a compatible directionality.”

8.xx  networkTTPIsProtected

This relationship type is related to the following enterprise entity:

<COMMUNITY:tem, ROLE: trail termination point, PROPERTY:protection>

DEFINITION

ROLE

protectedNetworkTTP

“Played by an instance of the <networkTTP> or information object type or subtype.”

protectingNetworkCTP

“Played by an instance of the <networkCTP> information object type or subtype.”

workerNetworkCTP

“Played by an instance of the <networkCTP> information object type or subtype.”

INVARIANT

inv_protectedTTPRoleCardinality

"One and only one instance of the role protectedNetworkTTP must participate in the relationship."

inv_protectingCTPRoleCardinality

"One and only one instance of the role protectingNetworkCTP must participate in the relationship."

inv_workerCTPRoleCardinality

"One and only one instance of the role workerNetworkCTP must participate in the relationship."

inv_roles

"In an instance of the relationship, an instance can not play both roles: protectingNetworkCTP and workerNetworkCTP."

inv_signalIdentification

"In a given relationship instance of networkTTPIsProtected, the information objects playing the role protectedNetworkTTP, protectingNetworkCTP, and workerNetworkCTP must have all the same signalIdentification value."

inv_directionality

"The objects involved in the relationship must have a compatible directionality.”

8.xx  subnetworkConnectionIsProtected

This relationship type is related to the following enterprise entity:

<COMMUNITY:tem, ROLE:sub-network connection, PROPERTY:protection>

DEFINITION

“The subnetworkConnectionIsProtected relationship class describes the relationship that exists between a protected subnetworkConection, a protecting subnetworkConnection and a worker subnetworkConnection.”

ROLE

protectedSNC

“Played by an instance of the <subnetworkConnection > information object type or subtype.”

protectingSNC

“Played by an instance of the <subnetworkConnection > information object type or subtype.”

workerSNC

“Played by an instance of the <subnetworkConnection > information object type or subtype.”

INVARIANT

inv_protectedSNCRoleCardinality

"One and only one instance of the role protectedSNC must participate in the relationship."

inv_protectingSNCRoleCardinality

"One and only one instance of the role protectingSNC must participate in the relationship."

inv_workerSNCRoleCardinality

"One and only one instance of the role workerSNC must participate in the relationship."

inv_roles

"In an instance of the relationship, an instance can not play both roles: protectingSNC and workerSNC."

inv_signalIdentification

"In a given relationship instance of subnetworkConnectionIsProtected, the information objects playing the role protectedSNC, protectingSNC, and workerSNC must have all the same signalIdentification value."

inv_directionality

"The objects involved in the relationship must have a compatible directionality.”

8.xx  trailIsProtected

This relationship type is related to the following enterprise entity:

<COMMUNITY:tem, ROLE:trail, PROPERTY:protection>

DEFINITION

“The trailIsProtected relationship class describes the relationship that exists between a protected trail, a protecting trail and a worker trail.”

ROLE

protectedTrail

“Played by an instance of the <trail> information object type or subtype.”

protectingTrail

“Played by an instance of the <trail> information object type or subtype.”

workerTrail

“Played by an instance of the <trail> information object type or subtype.”

INVARIANT

inv_protectedTrailRoleCardinality

"At least one instance of the role protectedTrail must participate in the relationship."

inv_protectingTrailRoleCardinality

"At least one instance of the role protectingTrail must participate in the relationship."

inv_workerSNCRoleCardinality

"At least one instance of the role workerSNC must participate in the relationship."

inv_roles

"In an instance of the relationship, an instance can not play both roles: protectingSNC and workerSNC."

inv_signalIdentification

"In a given relationship instance of subnetworkConnectionIsProtected, the information objects playing the role protectedSNC, protectingSNC, and workerSNC must have all the same signalIdentification value."

inv_directionality

"The objects involved in the relationship must have a compatible directionality

Cb	Changes to Rec G.853.2 (1999)

None

Cc	Changes to Rec G.853.3 (1999)

None

Cd	Changes to Rec G.853.6 (1999)

None

Ce	Changes to Rec G.853.8 (1999)

None

Cf	Changes to Rec G.853.10 (1999)

None

Cg	Changes to Rec G.853.12 (1999)

None

##### Changes to Rec G.854 series

None

##### Changes to Rec G.855 series

Ea	Changes to Rec G.855.1 (1999)

Ea1	DR-G855.1-1

Section  10.2   ASN.1 Module

The change was proposed in the work on the CORBA/IDL version of M.3100 including M.3100 Amendment 1 and selected parts from G.855.1. at the Torrance meeting in August 2000.

In the basicLayerNetworkDomain object the “establishLogicalLinkInformation” and “establishTopologicalLinkInformation” actions has layerNetworkDomain reference as an information field. It was suggested that this item is not required as the same information is captured by the managed object instance information of the action.

EstablishLogicalLinkInformation  ::= SEQUENCE {

layerNetworkDomain		ObjectInstance,

aEnd					LinkEnd,

zEnd					LinkEnd,

suppliedUserIdentifier		[1] UserIdentifier OPTIONAL,

suppliedUserLabel			[2] GraphicString OPTIONAL,

suppliedDirection			[3] LinkDirectionality OPTIONAL,

…

}

EstablishTopologicalLinkInformation::= SEQUENCE {

layerNetworkDomain		ObjectInstance,

aEnd					LinkEnd,

zEnd					LinkEnd,

suppliedUserIdentifier		[1] UserIdentifier OPTIONAL,

suppliedUserLabel			[2] GraphicString OPTIONAL,

suppliedDirection			[3] Directionality OPTIONAL,

…

}

Ea2	DR-G855.1-2

Section  10.2   ASN.1 Module

The change was proposed in the work on the CORBA/IDL version of M.3100 including M.3100 Amendment 1 and selected parts from G.855.1. at the Torrance meeting in August 2000.

Usage cost (that is part of abstractLink) is not used in the following actions:

- setupTrail

- establishLogicalLink & establishLogicalLinkAndEnds

- establishTopologicalLink & establishTopologicalLinkAndEnds

It was suggested to include usageCost in these actions.

IMPORTS

…

…

Capacity, Capacities, Count, CTPList, LinkConnectionList, LinkEnd, None, UsageCost, UserIdentifier, 	LinkDirectionality, ConnectivityEndPoint, TPList, SignalId FROM M3100ASN1TypeModule2 {itu-t 	recommendation m gnm(3100) informationModel(0) asn1Modules(2) asn1Module2(1)}

SetupTrailInformation ::= SEQUENCE {

aEnd					SET OF ConnectivityEndPoint,

zEnd					SET OF ConnectivityEndPoint,

directionality			Directionality,

additionalInformation		[2] AdditionalInformation	OPTIONAL,

qofConnectivityService 		[3] QofConnectivityService	OPTIONAL,

userId					[0] UserIdentifier			OPTIONAL,

userLabel				[1] UserLabel			OPTIONAL,

usageCost				[4] UsageCost			OPTIONAL,

…

}

EstablishLogicalLinkInformation  ::= SEQUENCE {

layerNetworkDomain		ObjectInstance,

aEnd					LinkEnd,

zEnd					LinkEnd,

suppliedUserIdentifier		[1] UserIdentifier OPTIONAL,

suppliedUserLabel			[2] GraphicString OPTIONAL,

suppliedDirection			[3] LinkDirectionality OPTIONAL,

usageCost				[4] UsageCost			OPTIONAL,

…

}

EstablishTopologicalLinkInformation::= SEQUENCE {

layerNetworkDomain		ObjectInstance,

aEnd					LinkEnd,

zEnd					LinkEnd,

suppliedUserIdentifier		[1] UserIdentifier OPTIONAL,

suppliedUserLabel			[2] GraphicString OPTIONAL,

suppliedDirection			[3] Directionality OPTIONAL,

usageCost				[4] UsageCost			OPTIONAL,

…

}

            - TMN Defect Report Register

The defects reported to date are listed below. The status of each is indicated according to the classification outlined below:

O	Open

-	The defect has been submitted, a solution may have been proposed, but the Defect Resolution Group has not yet come to an agreement.

A/U	Agreed/Unanimous

- 	Proposed solution agreed by everyone in the Defect Resolution Group

- 	Pending approval by affected Study Group

A/C	Agreed/Consensus

-	Consensus solution agreed by the Defect Resolution Group and documented in this version of the Implementors’ Guide.

-	Pending approval by affected Study Group

A	Agreed

-	Proposed solution agreed by the Defect Resolution Group

-	Pending approval by affected Study Group

Note: This status value is no longer to be used and has been replaced with the A/U and A/C status values.

C	Complete

-	Defect resolution approval by full Study Group

-	Final resolution reflected in this version of Implementor's Guide

P	Published

-	Change included in published version

R	Rejected

-	As a defect (may be misinterpretation, request for extension or have already been corrected in subsequent version of text)

W	Withdrawn

-	Defect report withdrawn by source.

The severity of each is indicated according to the classification outlined below:

m	Minor

The following defects are classified as minor:

-	Pure syntax errors in GDMO templates (e.g. missing semicolons)

-	Obvious misalignment in names of attributes in the GDMO and ASN.1

-	Missing ASN.1 productions where the text is clear as to the syntax, semantics and data type

-	Clarifications of ambiguities in the text if supported by the formal definitions

-	Typographical errors.

M	Major

-	All other defects are classified as major.

| - ITU-T Recommendation G.85x |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| - Status | - Severity | - Defect Number | - Source | - Guide Entry | - Other reference | - Subject Matter |
| O | m | DR-G85xTem-1 | Marconi Communications |  | ETSI meeting in October 1999 | Addition of protection level information (highly protected, protected, unprotected and preemptible) to LC and SNC (Note) Bb1, Bb2, Ca1 will be assigned as guide entry. The tentative texts are provided below. |
| A/C | M | DR-G85xTm-1 | Telenor | Bd1 | DAR – 7 ‘monitored trail management’ | Addition of Action Policy to ‘Section 6.4.1. Set up point-to-point trail’ of G.852.6 |
| A/C | M | DR-G85xCiv-1 | Telenor | Ca2 | DAR – 7 ‘monitored trail management’ | New section 7.9 operationalState should be added. |
| A/C | M | DR-G85xCiv-2 | Marconi Communications | Ca3 | DAR – 19 ‘sub-network connection protection management’ | Addition of topologicalLinkEnd to containerLE in “8.22 linkEndHasNetworkCTPs” |
| A/C | m | DR-G85xCiv-3 | Marconi Communications | Ca4, Ca5, Ca6, Ca7, Ca8 | DAR – 19 ‘sub-network connection protection management’ | To support protection management requirements |
| A/C | m | DR-G855.1-1 | Japan | Ea1 | WP 4/4 and 5/4 meeting in August 2000 | In the basicLayerNetworkDomain object the “establishLogicalLinkInformation” and “establishTopologicalLinkInformation” actions has layerNetworkDomain reference as an information field. It was suggested that this item is not required as the same information is captured by the managed object instance information of the action. |
| A/C | m | DR-G855.1-2 | Japan | Ea2 | WP 4/4 and 5/4 meeting in August 2000 | To include usageCost in some actions |

(Note)  The following text will be provided.

Bb1	DR-G85xTem-1

Section 6.2.7 Link connection

It was found that G.852.2 (TEM) and G.853.1 (CIV) should be modified, when ‘sub-network connection protection management’ was being studied at the ETSI meeting in October 1999.

Protection level information (highly protected, protected, unprotected and preemptible) should  be added to the properties of Link Connection as follows:

The text will be provided by Mr. Foster (Marconi Communications).

Bb2	DR-G85xTem-1

Section 6.2.12 Subnetwork connection

See Ba1 on rationale.

Protection level information (highly protected, protected, unprotected and preemptible) should  be added to the properties of Subnetwork Connection as follows:

The text will be provided by Mr. Foster (Marconi Communications).

Ca1	DR-G85xTem-1

Section 2.36 transportConnection

See Bb1 on rationale.

Protection level information (highly protected, protected, unprotected and preemptible) should  be added to the ATTRIBUTE of transportConnection as follows, because linkConnection and subnetworkConnection are the sub-type of the transportConnection information object type:

The text will be provided by Mr. Foster (Marconi Communications).

            - TMN Defect Report Form

DEFECT REPORT FORM

1.	Defect Report Number:	Recommendation code/numeric

Note: Only the recommendation is identified by the 		defect report submitter

2.	Source:	country, member etc

Note: Filled out by the defect report submitter

3.	Addressed to:	Defect editors group reference

Note: Filled out by the defect report submitter

4.	(a) ITU-T TSB:	administrative body reference

(b) ITU-T WP:

Note: Not filled out by the defect report submitter

5.	Date circulated by	date

administrative body

Note: Filled out by the TSB or WP

6.	Deadline for response from editor:	date

Note: Determined by the Defect Resolution Group

7.	Defect Report Concerning:	Recommendation number and publication date

Note: Filled out by the defect report submitter

8.	Qualifier:	e.g. error, omission, clarification required

Note: Filled out by the defect report submitter

9.	Reference in document:	clause number

Note: Filled out by the defect report submitter

10.	Nature of defect:	complete, concise explanation of the 				perceived problem

Note: Filled out by the defect report submitter

11.	Solution proposed by source:	optional

12.	Editors response:	any material proposed for processing as an erratum to, an amendment to or a commentary on the final Recommendation text. This will be included in Chapter 3 of a later version of this document.