#### Bulgaria (country code +359)

##### Communication of 26.VI.2003:

The Communications Regulation Commission (CRC), Sofia, announces that a twelve-digit international format numbering plan will be implemented for mobile operators in Bulgaria.

The new Numbering Plan (NP) will convert all existing Public Land Mobile Network (PLMN) numbers from a maximum eight-digit format to a maximum nine-digit format, introducing digit “8” as the first digit of the National (Significant) Number [N(S)N].  The existing Public Switched Telephone Network (PSTN) numbers will not be affected and will remain the same – maximum eight digits in the national format.

The need to expand the numbering capacity for mobile operators is imposed by the recent full liberalization of the telecommunication market, which has led to substantial increase in the demand for numbering resources.

At present, the mobile operators in Bulgaria use the following National Destination Codes (NDC):

“MOBILTEL”:	87, 88, 89
“COSMO BULGARIA MOBILE”:	98, 99
“RADIOTELECOMMUNICATION COMPANY”:	48

with maximum six-digit Subscriber Number (SN).

The implementation of the new Numbering Plan (NP) will be realized in two stages:

–	The first stage will be in effect from 20 July 2003 (0000 hours UTC+3). 
–	The second stage will be implemented in July 2004.

This is the final date of the implementation of changes. The details of the second stage will be presented in a later announcement.

###### Details for the first stage – Mobile telephony numbers

During the first stage only the mobile telephony numbers of “MOBILTEL” will be affected. The maximum eleven-digit numbers of “MOBILTEL” will be converted to maximum twelve digits with the addition of digit “8”.

- Existing dialling format:

+359 87 XXX XXX
+359 88 XXX XXX
+359 89 XXX XXX

- New dialling format, as from 20 July 2003:

+359 88 7XXX XXX
+359 88 8XXX XXX
+359 88 9XXX XXX

In addition, the National Destination Code (NDC) 88 will be used for access to the network of “MOBILTEL” and NDCs 87 and 89 will be released on 1 January 2004.

Description of number change for National Numbering Plan (NNP) for Country Code (CC) +359 Bulgaria, according to Recommendation E.129 (09/2002):

| (1) | (2) |  | (3) | (4) |  | (5) | (6) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Communicated time and date of change | N(S)N (National (Significant) Number) |  | Usage of E.164 Number | Parallel running |  | Operator | Proposed wording of announcement |
|  | Old number | New number |  | Begins | Ends |  |  |
| 20.VII.2003 0000 hours UTC +3 | 87XXXXXX 88XXXXXX 89XXXXXX | 88XXXXXXX | NDC – mobile network code 88 | N/A | N/A | Mobiltel EAD | This number has been changed. Please dial an additional 8 before the first 8 in the number. |

Contact:

Communications Regulation Commission (CRC)
“Technical Regulation and Electronic Signature” Directorate
6, Gurko Street
1000 SOFIA
Bulgaria
Tel:	+359 2 949 2128
Fax:	+359 2 949 2950 
E-mail:	erangelov@crc.bg
URL:	www.crc.bg