#### Spain

Communication of 19.II.1998:

New numbering plan in Spain

1	Timetable

The Ministerio de Fomento, Secretaría General de Comunicaciones, Madrid, announces that the new numbering plan for Spain (country code: 34) will enter into force on 3 April 1998 at 2300 hours UTC.

There will be a period during which both the old and new numbers may be dialled. This transitional period will end on 5 June 1998 at 2300 hours UTC.

Thereafter, when numbers are dialled which are not in accordance with the new numbering plan, advice will be provided by recorded message.

As from 18 July 1998, Spanish networks will only admit numbers dialled in accordance with the new numbering plan.

2	National number

The new numbering plan is a closed numbering plan, without abbreviated local dialling.

The national number will always contain nine significant digits. It will be structured as follows: NXYABMCDU.

On the date of entry into force of the new numbering plan, the current national prefix “9” will be abolished as such and will be incorporated in the national number. This will ease the changeover from the old to the new numbering plan.

The initial allocation of the digit N will be as follows:

N = 0	Short numbering / Prefixes
N = 1	Short numbering / Prefixes
N = 2	Available
N = 3	Available
N = 4	Available
N = 5	Available
N = 6	Mobile communications
N = 7	Personal numbering (X = 0)
N = 8	Geographical numbering / Intelligent network (X = 0)
N = 9	Geographical numbering / Intelligent network (X = 0)

3	Dialling plan

Since there is no national prefix, the full nine-digit national number NXYABMCDU will be dialled for all types of call.

The international prefix will be “00”, although the digits “07” will be used transitionally.

Services of short numbering will have the format “0XY” and “1XYZ”. Numbers in the “11X” series are reserved for services harmonized at European level.

Operator dialling codes will have the format “105XY” and “107XY”, with possible lengths from four to six digits. These codes are dialled first, followed either by the national number for national calls or by the digits "00" and the international number for calls to destinations outside the national territory.

The dialling procedure for calls dialled without going through an operator is summarized below:

|  | Before | After |  |
| --- | --- | --- | --- |
|  |  | Transitional | Final |
| Local calls | (Y)ABMCDU (*) | (Y)ABMCDU 9XYABMCDU | NXYABMCDU |
| National calls | 9XYABMCDU | 9XYABMCDU | NXYABMCDU |
| International (incoming) calls | +34 XYABMCDU (*) | +34 XYABMCDU +34 9XYABMCDU | +34 NXYABMCDU |

“Before” and “After” refer to the date of entry in force of the new numbering plan. The “Transitional” period is the period during which both the old and new numbers may be dialled. In the “Final” stage, the digit N may take on other values in addition to “9”.

(*) Numbers in the Spanish province of Zaragoza currently have nine digits. Accordingly, when calling from abroad it is necessary to dial the digit “9” after the country code for Spain (“34”). This same feature may also be available for customers of new operators, irrespective of their area of activity.

4	Geographical numbering

The only change in geographical subscriber numbers will consist in adding, to the existing number, the national prefix “9” followed by the national destination code for the province in question.

Accordingly, when calling from abroad, the digit “9” must be included as from the date of entry into force of the new numbering plan.

Subsequent expansion of the national numbering plan will also allow for the introduction of geographical subscriber numbers beginning with the digit “8”.

5	Numbering for mobile communication services

Mobile services already use nine-digit numbering, with the result that, initially, there will be no changes at the national level and the previously assigned blocks with the format “9XY” will be retained.

Nevertheless, when calling from abroad, the digit “N = 9” will have to be included as from the date of entry into force of the new numbering plan.

Subsequently, at a date to be determined, all mobile services will change over from N = 9 to N = 6.

6	Personal numbering services

At the national level, there will initially be no change in the dialling procedure, and the block “904” will be retained.

Nevertheless, when calling from abroad, the digit “N = 9” will have to be included as from the date of entry into force of the new numbering plan.

Subsequently, at a date to be determined, personal numbering services will change over from N = 9 to NX = 70.

7	Intelligent network services

The current numbering scheme will not be altered, and the blocks “90Y” (Y=0, ... , 6) are retained indefinitely. The code “900” corresponds to the automatic reverse charging service.

Numbers that can be dialled from abroad will have to include the digit “N = 9” as from the date of entry into force of the new numbering plan.

With effect from 1 September 1998, in addition to the code “900” the new code “800” will be available for the national automatic reverse charging service.