|  |  | International Telecommunication Union |  |  |
| --- | --- | --- | --- | --- |
| ITU-T | P.1202.1 |  |
| TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (10/2012) |
|  | SERIES P: TERMINALS AND SUBJECTIVE AND OBJECTIVE ASSESSMENT METHODS Models and tools for quality assessment of streamed media |  |  |  |
|  | Parametric non-intrusive bitstream assessment of video media streaming quality – Lower resolution application area |  |  |  |
|  | Recommendation ITU-T P.1202.1 |  |  |  |

ITU-T P-SERIES RECOMMENDATIONS

TERMINALS AND SUBJECTIVE AND OBJECTIVE ASSESSMENT METHODS

| Vocabulary and effects of transmission parameters on customer opinion of transmission quality | Series P.10 |
| --- | --- |
| Voice terminal characteristics | Series P.30 |
|  | P.300 |
| Reference systems | Series P.40 |
| Objective measuring apparatus | Series P.50 |
|  | P.500 |
| Objective electro-acoustical measurements | Series P.60 |
| Measurements related to speech loudness | Series P.70 |
| Methods for objective and subjective assessment of speech quality | Series P.80 |
|  | P.800 |
| Audiovisual quality in multimedia services | Series P.900 |
| Transmission performance and QoS aspects of IP end-points | Series P.1000 |
| Communications involving vehicles | Series P.1100 |
| Models and tools for quality assessment of streamed media | Series P.1200 |
| Telemeeting assessment | Series P.1300 |
| Statistical analysis, evaluation and reporting guidelines of quality measurements | Series P.1400 |

For further details, please refer to the list of ITU-T Recommendations.

Recommendation ITU-T P.1202.1 Parametric non-intrusive bitstream assessment of video media streaming quality – Lower resolution application area

Summary Recommendation ITU-T P.1202.1 specifies the algorithmic model for the lower resolution (LR) application area of ITU-T P.1202. The ITU-T P.1202 series of Recommendations specifies models for monitoring the video quality of IP-based video services based on packet-header and bitstream information. The lower resolution application area of the ITU-T P.1202.1 part of ITU-T P.1202 can be applied to the monitoring of the performance and quality of experience (QoE) of video services such as mobile TV. This Recommendation includes an electronic attachment with test vectors for testing compliance.

History Edition; Recommendation; Approval; Study Group. 1.0; ITU-T P.1202.1; 2012-10-14; 12.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications, information and communication technologies (ICTs). The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

Compliance with this Recommendation is voluntary. However, the Recommendation may contain certain mandatory provisions (to ensure, e.g., interoperability or applicability) and compliance with the Recommendation is achieved when all of these mandatory provisions are met. The words "shall" or some other obligatory language such as "must" and the negative equivalents are used to express requirements. The use of such words does not suggest that compliance with the Recommendation is required of any party.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementers are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database at http://www.itu.int/ITU-T/ipr/.

ITU 2013

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

Table of Contents

Page

1	Scope		1

2	References		1

3	Definitions		1

4	Abbreviations and acronyms		2

5	Conventions		2

6	Model description		2

6.1	Extraction of basic parameters		5

6.2	Aggregation of basic parameters into internal picture level parameters		11

6.3	Aggregation of parameters into model parameters		35

6.4	Quality estimation model		41

7	Assumptions		49

8	Model compliance		49

8.1	Test vectors for extraction of basic parameters		50

8.2	Test vectors for aggregation of basic parameters into internal picture level parameters		50

8.3	Test vectors for aggregation of parameters into model parameters		50

8.4	Test vectors for quality estimation model		51

Electronic attachment: Test vectors for testing compliance

Recommendation ITU-T P.1202.1

Parametric non-intrusive bitstream assessment of video media streaming
quality – Lower resolution application area

# 1	Scope

This Recommendation describes an algorithmic model for parametric non-intrusive bitstream assessment of video media streaming quality – lower resolution (LR) application area.  This algorithmic model can be applied to the monitoring of the  video quality of IP-based video services based on information extracted from the video bitstream encoded with the ITU-T H.264/AVC baseline video codec. The model is intended especially for the lower resolution application area, including such services as mobile TV. The ITU-T P.1202.1 model is a no-reference (i.e., non-intrusive) model which operates by analysing information in the video bitstream without fully decoding the bitstream. Further client specific information, such as concealment type and rebuffering duration, is provided to the algorithm out-of-band, for example in the form of stream-specific side information. As output, the model algorithm provides an estimate of the video quality in terms of the five-point absolute category rating (ACR) mean opinion score (MOS) scale defined in [ITU-T P.910]. This Recommendation describes the algorithm that takes an elementary stream encapsulated in RTP/UDP/IP as input, where information from RTP headers is used to identify lost pictures.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.264]		Recommendation ITU-T H.264 (2012), Advanced video coding for generic audiovisual services.

[ITU-T P.910]		Recommendation ITU-T P.910 (2008), Subjective video quality assessment methods for multimedia applications.

[ITU-T P.1202]		Recommendation ITU-T P.1202 (2012), Parametric non-intrusive bitstream assessment of video media streaming quality.

[IETF RFC 3550]	IETF RFC 3550 (2003), RTP: A Transport Protocol for Real-Time Applications.

[IETF RFC 3984]	IETF RFC 3984 (2005), RTP Payload Format for H.264 Video.

[IETF RFC 4566]	IETF RFC 4566 (2006), SDP: Session Description Protocol.

# 3	Definitions

None.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

ACR			Absolute Category Rating

AVC			Advanced Video Coding

DCT			Discrete Cosine Transform

EC MB			Error Concealed Macroblock

GOP			Group Of Pictures

IP			Internet Protocol

MB			Macroblock

MOS			Mean Opinion Score

NAL			Network Abstraction Layer

PLC			Packet Loss Concealment

QP			Quantization Parameter

RTP			Real Time Protocol

SDP			Session Description Protocol

SPS			Sequence Parameter Set

UDP			User Datagram Protocol

# 5	Conventions

None.

# 6	Model description

This clause describes the ITU-T P.1202.1 model and how it is implemented. The block diagram for the model is depicted in Figure 6-1. The model takes an ITU-T H.264/AVC baseline encoded video bitstream and side information (error concealment type, rebuffering information, etc.) as input, extracts parameters, also referred to as features, and aggregates them into model parameters which are used to calculate an estimated video quality MOS for the sequence.

Figure 6-1 – Block diagram for ITU-T P.1202.1

The model description is organized according to the flow of parameters in the model and to which degradation module the parameters belong. Clause 6.1 describes the extraction of basic parameters from the video bitstream and side information. In clause 6.2 the algorithms for aggregating the basic parameters into internal parameters at picture level are described. The words picture and frame are used interchangeably throughout this Recommendation. The aggregation of parameters into model parameters is described in clause 6.3, and finally the quality estimation model is described in clause 6.4.

Four different types of degradations are detected by the ITU-T P.1202.1 model; compression artefacts, slicing artefacts, freezing artefacts and rebuffering artefacts. Compression artefacts are introduced due to lossy compression of the encoding process. Slicing artefacts are introduced when packet losses are concealed using a packet loss concealment (PLC) scheme trying to repair erroneous frames. Freezing artefacts are introduced when the PLC scheme of the receiver replaces the erroneous frames (either due to packet loss or error propagation) with the previous error free frame until a decoded picture without errors has been received. Since the erroneous frames are not displayed, this type of artefact is also referred to as freezing with skipping. Rebuffering artefacts come from rebuffering events at the client side, which could be a result of video data arriving late. Usually, rebuffering events are indicated to the viewer, e.g., in the form of a spinning wheel. This is also referred to as freezing without skipping.

For each clause a set of input and output parameters that are used by the algorithms are defined and described. Model coefficients are also given if it is being used by the algorithms.

The flow chart in Figure 6-2 describes the flow of parameters in the model and in what clause each part can be found.

Figure 6-2 – Flow of parameters in the model

The algorithmic descriptions of the model are described using C-style pseudocode, leaving out variable declarations, row-ending semi-colons, etc., for better readability. The variables in the algorithm descriptions are prefixed according to what variable type they pertain to, e.g., i_nbr_mbs. The prefixes have the following meaning:

b 	– 	boolean

i 	– 	integer

d 	– 	double, i.e., floating point value in double precision

s 	–	string

p 	– 	struct or object holding data

The following static code words are used for the macroblock data in the pseudocode of the algorithmic descriptions in this clause:

INTER16x16_MB	The macroblock is an inter macroblock divided into macroblock partitions of size 16x16. It is the same as P_L0_16x16 in [ITU-T H.264].

INTER16x8_MB	The macroblock is an inter macroblock divided into macroblock partitions of size 16x8. It is the same as P_L0_L0_16x8 in [ITU-T H.264].

INTER8x16_MB	The macroblock is an inter macroblock divided into macroblock partitions of size 8x16. It is the same as P_L0_L0_8x16 in [ITU-T H.264].

INTER8x8_MB	The macroblock is an inter macroblock divided into macroblock partitions of size 8x8 with possibly different reference pictures for each macroblock partition. It is the same as P_8x8 in [ITU-T H.264].

INTER8x8REF0_MB	The macroblock is an inter macroblock divided into macroblock partitions of size 8x8 where all macroblock partitions uses the first reference picture in the list. It is the same as P_8x8ref0 in [ITU-T H.264].

SKIPPED_MB	The macroblock is of skip type.

INTRA_MB	The macroblock is an intra-macroblock

INTRA16x16_MB	The macroblock is an intra-macroblock with 16x16 intra prediction mode. It is the same as Intra_16x16 in [ITU-T H.264] with any setting of Intra16x16PredMode, CodedBlockPatternChroma and CodedBlockPatternLuma.

LOST			The macroblock has been lost

UNKNOWN	The type of the macroblock is unknown

## 6.1	Extraction of basic parameters

The extraction of basic parameters is done from side information; from sequence parameter set (SPS) network abstraction layer (NAL) units in the video bitstream, or from the picture level in the video bitstream. If an SPS NAL unit does not exist in the video bitstream, the output in clause 6.1.2.2 can be derived from session description protocol (SDP) data or from other side information.

### 6.1.1	Extraction of basic parameters from side information

The basic parameter extraction from side information has the following input and output parameters:

Input  = [side_information]

Output = [d_avg_frame_rate,

d_rebuffering_duration,

s_video_PLC_mode]

#### 6.1.1.1	Input parameter description

side_information		Side information containing necessary information for calculating the model score correctly.

#### 6.1.1.2	Output parameter description

d_avg_frame_rate		Average number of frames per second for the sequence. d_avg_frame_rate should be taken from side_information if available, otherwise d_avg_frame_rate should be derived from the timestamp in the RTP header.

d_rebuffering_duration

Total combined duration of rebuffering in seconds for the sequence

s_video_PLC_mode

Packet loss concealment method employed in video decoder. The value is "SLICING" or "FREEZING".

### 6.1.2	Extraction of basic parameters from SPS

The sequence parameter set (SPS) contains information necessary to properly decode a sequence. The basic parameter extraction from SPS has the following input and output parameters:

Input  = [bitstream]

Output = [i_nbr_mb_ver,

i_nbr_mbs,

b_gaps_in_frame_num_value_allowed_flag]

#### 6.1.2.1	Input parameter description

bitstream			A video bitstream encoded using the ITU-T H.264/AVC baseline profile.

#### 6.1.2.2	Output parameter description

i_nbr_mb_ver		Number of macroblocks in vertical direction for a picture.

i_nbr_mbs			Number of macroblocks in a picture calculated as i_nbr_mbs = i_nbr_mb_hor i_nbr_mb_ver, where i_nbr_mb_hor is the number of macroblocks in horizontal direction for a picture.

b_gaps_in_frame_num_value_allowed_flag

Flag indicating whether a gap in the frame_num value is allowed. If it is not allowed and a gap is present, it means that one or more pictures have been lost.

### 6.1.3 	Extraction of basic parameters from bitstream at picture level

The extraction of basic parameters from the bitstream at picture level takes part both on RTP level and at picture level in the elementary stream. The extraction of basic parameters from bitstream at picture level has the following input and output parameters:

Input  = [b_first_picture,

RTP_bitstream,

i_nbr_mbs,

p_mb_data_array[]]

struct MB_data

{

i_dct_coef_ac[][],

i_dct_coef_dc[],

i_cbp,

i_mb_type,

i_qp,

i_mvx[],

i_mvy[],

i_ref_idx,

i_sub_mb_type

}

struct MB_data p_mb_data_array[i_nbr_mbs]

Output = [i_curr_mb_type_map[],

i_curr_qp_map[],

i_curr_avg_mvx_map[],

i_curr_avg_mvy_map[],

i_max_nbr_used_ref_pics,

b_sub_mb_type_inter8x4_or_inter4x8_exists,

b_sub_mb_type_inter4x4_exists,

i_frame_num,

i_displayorder,

i_received_packets,

i_lostpackets,

i_received_bytes,

i_frame_type,

b_reflost0,

i_lostframegap]

#### 6.1.3.1	 Input parameter description

b_first_picture			Boolean indicating if the current picture is the first picture in the sequence. For the first picture this variable should be TRUE, for all other pictures FALSE.

RTP_bitstream			The video bitstream encapsulated in RTP packets.

i_nbr_mbs				Number of macroblocks in a picture.

p_mb_data_array[]	Array of structures of type "struct MB_data" containing macroblock data. The size of the array is i_nbr_mbs. The macroblock data in the structs contain the entries described below.

The following parameters of p_mb_data_array[] are obtained for each MB in a picture.

i_dct_coef_ac[][]		AC components of DCT coefficients of prediction residual. Its dimension is 16x16. First dimension corresponds to 16 4x4 sized-blocks; second dimension corresponds to 16 DCT coefficients of 4x4 DCT transform.

i_dct_coef_dc[]			DC components of DCT coefficients of prediction residual. Its dimension is 16, corresponding to Hadamard Transform coefficients of DC components of DCT transform of each 4x4 size-block.

i_cbp					It has the same meaning as the cbp defined in ITU-T H.264 syntax.

i_mb_type				Macroblock type

i_qp					QP for the macroblock

i_mvx[]					Array of horizontal motion vectors at sub-macroblock level. The size of the array is 16.

i_mvy[]					Array of vertical motion vectors at sub-macroblock level. The size of the array is 16.

i_ref_idx				Index of the reference picture for the first macroblock partition in the macroblock.

i_sub_mb_type			Sub-macroblock type for the first of the four possible sub-macroblock in the macroblock.

#### 6.1.3.2	Output parameter description

The following output parameters are obtained for each parsed picture. Parsed pictures include completely received pictures and partially received pictures.

i_curr_mb_type_map[]	Array containing macroblock types for the macroblocks in the current picture. The size of the array is i_nbr_mbs.

i_curr_qp_map[]		Array containing QP values for the macroblocks in the current picture. The size of the array is i_nbr_mbs. Same as pm_data_array[].i_qp for the current picture.

i_curr_avg_mvx_map[]	Array containing averaged horizontal motion vector values for the macroblocks in the current picture. The size of the array is i_nbr_mbs.

i_curr_avg_mvy_map[]	Array containing averaged vertical motion vector values for the macroblocks in the current picture. The size of the array is i_nbr_mbs.

i_max_nbr_used_ref_pics	The number of actually used reference pictures detected in the bitstream so far. This parameter is detected by checking for the maximum reference picture index for the first macroblock partition in each received macroblock. This parameter is updated for each picture.

b_sub_mb_type_inter8x4_or_inter4x8_exists

Boolean indicating if a sub-macroblock of type inter 8x4 or inter 4x8 exists in the bitstream so far. This parameter is updated for each picture.

b_sub_mb_type_inter4x4_exists

Boolean indicating if a sub-macroblock of type inter 4x4 exists in the bitstream so far. This parameter updated for each picture.

i_frame_num			The frame number counter in the ITU-T H.264 video bitstream as defined in [ITU-T H.264].

i_displayorder		 	Display order of a picture in the bitstream corresponding to the picture number. It is obtained from the RTP timestamp for the ITU-T H.264/AVC bitstream encoded with baseline profile, where the display order and encoding order are the same.

i_received_packets		The number of received RTP packets belonging to a picture.

i_lostpackets			The number of lost RTP packets belonging to a picture. The RTP packets lost between the start of current picture and the end of immediately previous picture are counted into current picture's i_lostpackets.

i_received_bytes		The number of received bytes belonging to a picture in the elementary stream.

i_frame_type			The frame type takes one of the following values; 0 = I_frame, 1 = P_frame, 2 = B_frame (not applicable for ITU-T H.264/AVC baseline) or 5 = unknown.

b_reflost0				Boolean indicating whether a reference picture in current pictures' reference_picture_list0 is totally lost. Default/initial value is FALSE.

i_lostframegap			The number of consecutively totally lost frames from the latest fully/partly received frame. This can be derived from the gap of RTP timestamp.

#### 6.1.3.3	Algorithms

At this stage, basic parameters are extracted and updated for the current picture. Some of the basic picture level information is obtained at the RTP level including the number of received packets for a picture, the number of lost packets for a picture, the number of received bytes belonging to a picture and the number of consecutively totally lost pictures.

The other basic parameters are obtained at picture level in the elementary stream. An array of structs holding macroblock information, p_mb_data_array[], is used for the data received from the parser for the description in this clause.

Keeping track of lost pictures can be done using timestamp information from the RTP header or be derived from the frame_num parameter in the bitstream. The most reliable and straightforward method is to use the timestamp from the RTP header. How to keep track of lost pictures from RTP timestamp and frame_num is explained below.

##### 6.1.3.3.1	Keeping track of lost packets and pictures using RTP timestamp

RTP packets having the same RTP timestamp belong to the same picture. The sequence number is defined in the RTP protocol header and it increments by one for each sent RTP packet. The number of lost RTP packets between the first and last received RTP packets belonging to frame i is counted based on the discontinuity of sequence numbers. Similarly, the lost RTP packets between the first received RTP packet of frame i and the last received RTP packet of its immediately previous frame i-1 are counted based on the discontinuity of sequence numbers and counted into the lost packets of frame i.

It can be detected if one or more consecutive pictures have been lost by checking if the difference in RTP timestamp for the preceding and following packets of where the loss occurs exceeds the expected RTP timestamp increment by a factor n, where n is an integer equal to or larger than 0. If n > 1, then the number of lost pictures for the gap is n-1. The expected RTP timestamp increment can be derived from the video frame rate and the resolution of the sample clock.

##### 6.1.3.3.2	Keeping track of lost pictures using i_frame_num

The flow chart below illustrates how lost pictures are detected using the i_frame_num parameter extracted from the bitstream. For the first picture the current frame number is the expected frame number. If the expected frame number equals the current frame number, the basic parameters are extracted as described in this clause followed by aggregating the basic parameters into internal parameters as described in clause 6.2. If the current frame is not equal to the expected frame number and the flag b_gaps_in_frame_num_value_allowed_flag is not set, a picture has been lost. The LOST state is then updated in all macroblock positions for the map holding the information about macroblock type for the current picture.

Figure 6-3 – Flow chart describing how lost pictures are detected using i_frame_num

The following pseudocode describes the procedure for keeping track of lost frames using i_frame_num:

while (!EndOfSequence) {

if (b_first_picture) {

i_expected_frame_num = i_frame_num

}

if (!b_gaps_in_frame_num_value_allowed_flag &&

i_frame_num != i_expected_frame_num) {

// Set all macro block types for current picture to LOST

for (i_mb = 0; i_mb < i_nbr_mbs; i_mb++) {

i_curr_mb_type_map[i_mb] = LOST

}

// update frame number

i_frame_num = (i_frame_num + 1) % MaxFrameNum

do aggregate basic parameters into internal parameters at picture level

} else { // The picture is not lost

i_expected_frame_num = (i_frame_num + 1) % MaxFrameNum

do extract basic parameters at picture level

do aggregate basic parameters into internal parameters at picture level

}

}

##### 6.1.3.3.3	Obtaining basic parameters from the bitstream at picture level

This clause describes how to obtain some of the needed basic parameters from the bitstream at picture level.

Macroblock maps containing macroblock type, QP value and average vertical and horizontal motion vectors for the macroblock are updated and kept in array maps for all macroblocks in the current picture. If a macroblock has been lost the corresponding value the macroblock type map for that macroblock is set to LOST.

Parameters related to the calculation of the encoder complexity level as described in clause 6.4.1 is also extracted at this stage.

The following pseudocode shows how to extract some of the basic parameters at picture level:

if (b_first_picture) {

// Initialize parameters

i_max_nbr_used_ref_pics                   = 0

b_sub_mb_type_inter8x4_or_inter4x8_exists = FALSE

b_sub_mb_type_inter4x4_exists             = FALSE

}

for (i_mb = 0; i_mb < i_nbr_mbs; i_mb++) {

if (p_mb_data_array[i_mb].mb_type == UNKNOWN) {

// Set that MB has been lost

i_curr_mb_type_map[i_mb] = LOST

} else {

// Update values

i_curr_mb_type_map[i_mb] = p_mb_data_array[i_mb].i_mb_type

i_curr_qp_map[i_mb]      = p_mb_data_array[i_mb].i_qp

// Reset mv values

i_curr_avg_mvx_map[i_mb] = 0

i_curr_avg_mvy_map[i_mb] = 0

// Traverse sub-macro blocks and set avg MVs for non-intra MBs

for (i_smb = 0; i_smb < 16; i_smb++) {

if (p_mb_data_array[i_mb].mb_type != INTRA_MB) {

i_curr_avg_mvx_map[i_mb] += p_mb_data_array[i_mb].i_mvx[i_smb]

i_curr_avg_mvy_map[i_mb] += p_mb_data_array[i_mb].i_mvy[i_smb]

}

}

i_curr_avg_mvx_map[i_mb] /= 16

i_curr_avg_mvy_map[i_mb] /= 16

// Check for parameters related to the encoder complexity

// Check for nbr of actually used reference pictures in the first macro

// block partition only

if (i_max_nbr_used_ref_pics < p_mb_data_array[i_mb].i_ref_idx + 1) {

i_max_nbr_used_ref_pics = p_mb_data_array[i_mb].i_ref_idx + 1

}

// Check for inter 8x4 or inter 4x8 sub macro block type

if (!b_sub_mb_type_inter8x4_or_inter4x8_exists &&

(p_mb_data_array[i_mb].mb_type == INTER8x8_MB ||

p_mb_data_array[i_mb].mb_type == INTER8x8REF0_MB) &&

(p_mb_data_array[i_mb].sub_mb_type == 1 ||

p_mb_data_array[i_mb].sub_mb_type == 2)) {

b_sub_mb_type_inter8x4_or_inter4x8_exists = TRUE

}

// Check for inter 4x4 sub macro block type

if (!b_sub_mb_type_inter4x4_exists &&

(p_mb_data_array[i_mb].mb_type == INTER8x8_MB ||

p_mb_data_array[i_mb].mb_type == INTER8x8REF0_MB) &&

p_mb_data_array[i_mb].sub_mb_type == 0) {

b_sub_mb_type_inter4x4_exists = TRUE

}

}

}

## 6.2	Aggregation of basic parameters into internal picture level parameters

This clause describes how internal picture level parameters are aggregated from the basic parameters extracted from the video bitstream.

### 6.2.1	Compression module parameters

The aggregation of basic parameters into internal picture level parameters used for the compression module uses the following input and output parameters.

Input  = [b_first_picture,

i_nbr_mbs,

i_curr_qp_map[],

i_curr_mb_type_map[],

i_curr_avg_mvx_map[],

i_curr_avg_mvy_map[]]

Output = [d_avg_qp_curr_pic,

d_avg_qp_prev_pic,

d_min_avg_pic_qp,

d_max_avg_pic_qp,

d_sum_avg_pic_qp,

d_sum_avg_pic_mv,

i_prev_mb_type_map[],

b_curr_pic_is_intra_like,

i_nbr_intra_pics,

i_nbr_inter_pics,

b_has_intra_pic_flicker,

i_nbr_received_pics,

i_nbr_lost_pics,

i_loss_penalty_map[],

d_sum_max_nbr_partitions]

#### 6.2.1.1	Input parameter description

b_first_picture				See clause 6.1.3.1

i_nbr_mbs					See clause 6.1.2.2

i_curr_qp_map[]			See clause 6.1.3.2

i_curr_mb_type_map[]	See clause 6.1.3.2

i_curr_avg_mvx_map[]	See clause 6.1.3.2

i_curr_avg_mvy_map[]	See clause 6.1.3.2

#### 6.2.1.2	Output parameter description

d_avg_qp_curr_pic	The average QP value for all correctly received macroblocks in the current picture.

d_avg_qp_prev_pic	The average QP value for all correctly received macroblocks in the previous picture.

d_min_avg_pic_qp		The QP value over all macroblocks are averaged for each picture. This parameter is the value for the picture with the smallest value of all pictures received so far.

d_max_avg_pic_qp	The QP value over all macroblocks are averaged for each picture. This parameter is the value for the picture with the largest value of all pictures received so far.

d_sum_avg_pic_qp		The sum of all average QP per picture for all pictures received so far.

d_sum_avg_pic_mv	Average motion per picture summed up for all received pictures so far. Used for calculating average motion for the whole sequence.

i_prev_mb_type_map[]	Array containing macroblock types for the macroblocks in the previous picture. The size of the array is i_nbr_mbs.

b_curr_pic_is_intra_like	Indicates if the current picture is considered intra-like.

i_nbr_intra_pics		The number of intra pictures received so far.

i_nbr_inter_pics		The number of inter pictures received so far.

b_has_intra_pic_flicker	Boolean indicating if there has been a detected intra picture flicker in the received bitstream up till the current picture.

i_nbr_received_pics	The number of pictures received so far.

i_nbr_lost_pics			The number of lost pictures so far in the bitstream.

i_loss_penalty_map[]	An array keeping track of if an error is propagating for each macroblock in the picture. The size of the array is i_nbr_mbs. The initial values in the array are all 0.

d_sum_max_nbr_partitions

The maximum number of macroblock partitions in a macroblock for a picture having a propagating error, summarized over all pictures received so far.

#### 6.2.1.3	Algorithms

##### 6.2.1.3.1	Aggregation of basic parameters into internal parameters at picture level

This clause describes how the basic parameters are aggregated into internal parameters at picture level. If it is the first picture, all parameters that are updated for each picture are first initialized. The following internal parameters are aggregated at picture level; average motion perception, average, min and max QP value, number of inter and intra pictures, detection of intra-picture flicker and maximum number of macroblock partitions used for the picture if an error is propagating. All of this is further described in the pseudocode below and the following clauses.

if (b_first_picture) {

// Initialize parameters before first picture

d_avg_qp_prev_pic        = 0

d_avg_qp_curr_pic        = 0

d_sum_avg_pic_mv         = 0

d_min_avg_pic_qp         = INT_MAX

d_max_avg_pic_qp         = 0

d_sum_avg_pic_qp         = 0

d_sum_max_nbr_partitions = 0

i_nbr_intra_pics         = 0

i_nbr_inter_pics         = 0

i_nbr_received_pics      = 0

i_nbr_lost_pics          = 0

b_curr_pic_is_intra_like = 0

b_has_intra_pic_flicker  = 0

}

// Initialize internal picture level counters and sums

i_qp_pic_sum               = 0

i_nbr_lost_mbs             = 0

i_nbr_intra_mb             = 0

i_motion_sum               = 0

d_nbr_mbs_that_may_have_mv = 0

d_max_nbr_partitions       = 0

for (i_mb = 0; i_mb < i_nbr_mbs; i_mb++) {

if (i_curr_mb_type_map[i_mb]==LOST) {

i_nbr_lost_mbs++

} else {

i_qp_pic_sum += i_curr_qp_map[i_mb]

}

if (i_curr_mb_type_map[i_mb] == INTRA_MB) {

i_nbr_intra_mb++

}

// Calculate average motion perception

if ((i_curr_mb_type_map[i_mb] != INTRA_MB) &&

(i_curr_mb_type_map[i_mb] != LOST)) {

i_motion_sum += (abs(i_curr_avg_mvx_map[i_mb])+

abs(i_curr_avg_mvy_map[i_mb]))/2

d_nbr_mbs_that_may_have_mv++

}

}

// Set if current and previous pictures are intra-like

b_prev_pic_is_intra_like = b_curr_pic_is_intra_like

b_curr_pic_is_intra_like =

estimateIfIntraLikePicture(i_nbr_mbs, i_curr_mb_type_map)

// Calculate average motion for the picture

if (d_nbr_mbs_that_may_have_mv && !b_curr_pic_is_intra_like) {

d_avg_motion = 1+i_motion_sum/d_nbr_mbs_that_may_have_mv

} else {

d_avg_motion = 0

}

d_sum_avg_pic_mv += d_avg_motion

// If at least one macro block was received for the picture

if (i_nbr_mbs – i_nbr_lost_mbs) {

// Set avg QP for current, previous and 2nd to previous picture

d_avg_qp_2nd_prev_pic = d_avg_qp_prev_pic

d_avg_qp_prev_pic     = d_avg_qp_curr_pic

d_avg_qp_curr_pic     =

i_qp_pic_sum/((double)( i_nbr_mbs – i_nbr_lost_mbs))

// Determine max and min QP for the sequence so far

d_min_avg_pic_qp = min(d_min_avg_pic_qp, d_avg_qp_curr_pic)

d_max_avg_pic_qp = max(d_max_avg_pic_qp, d_avg_qp_curr_pic)

// Sum up qp for later calculating sequence average

d_sum_avg_pic_qp += d_avg_qp_curr_pic

// Count intra and inter pictures

if (b_curr_pic_is_intra_like) {

i_nbr_intra_pics++

} else {

i_nbr_inter_pics++

}

// Estimate intra picture flicker and set penalty if found

if (!b_has_intra_pic_flicker) {

b_has_intra_pic_flicker =

estimateIntraPictureFlicker(b_prev_pic_is_intra_like,

d_avg_qp_curr_pic,

d_avg_qp_prev_pic,

d_avg_qp_2nd_prev_pic)

}

i_nbr_received_pics++

} else {

i_nbr_lost_pics++

}

// Determine if error is propagating

[b_error_is_propagating, i_loss_penalty_map] =

determineErrorPropagation(i_nbr_mbs,

i_curr_mb_type_map,

i_prev_mb_type_map,

i_loss_penalty_map)

// Find max number of macro block partitions in picture

d_max_nbr_partitions =

findMaxNbrMbPartitions(i_nbr_mbs,

b_error_is_propagating,

i_curr_mb_type_map)

d_sum_max_nbr_partitions += d_max_nbr_partitions

// Update mb type maps for next picture

for (i_mb = 0; i_mb < i_nbr_mbs; i_mb++) {

i_prev_mb_type_map[i_mb] = i_curr_mb_type_map[i_mb]

}

How the functions determineErrorPropagation() and findMaxNbrMbPartitions() are implemented is described in clauses 6.2.2.3.6 and 6.2.2.3.7 respectively.

##### 6.2.1.3.2	Estimate if picture is an intra-like picture

This function estimates whether the picture is an intra-picture or an intra-like picture. If more than 50% of the correctly received macroblocks in the picture are intra-macroblock, the picture is considered an intra-like picture. An intra-like picture behaves similarly in many ways to a normal intra picture containing only intra-macroblocks.

estimateIfIntraLikePicture(i_nbr_mbs, i_mb_type_map[]) {

d_nbr_intra_mbs = 0

i_nbr_lost_mbs  = 0

for (i_mb = 0; i_mb < i_nbr_mbs; i_mb++) {

if (i_mb_type_map[i_mb] == INTRA_MB) {

d_nbr_intra_mbs++

} else if (i_mb_type_map[i_mb] == LOST) {

i_nbr_lost_mbs++

}

}

return (d_nbr_intra_mbs/(i_nbr_mbs–i_nbr_lost_mbs) > 0.5)

}

##### 6.2.1.3.3	Estimate intra-picture flicker

The intra-picture flicker algorithm determines a measure of quantization parameter for each picture in the received video bitstream and identifies a quality defect when an abrupt change in the quantization parameter occurs. More specifically, after determining whether each picture of the video sequence is an intra-coded picture or an intra-like picture, intra-picture flicker is identified when an abrupt change occurs for the intra-coded picture or intra-like picture. The general process is described in the flow chart in Figure 6-4 below.

Figure 6-4 – Estimation of intra-picture flicker

If a picture is determined as an intra-like picture, an intra-picture flicker is detected if the abrupt change in the average QP value for that picture is a peak compared to the average QP values of the preceding and subsequent pictures. A peak in the average QP value for a picture is identified if the following two conditions hold:

–	the difference between the average quantization parameter for the current picture and the average quantization parameter for the preceding picture is greater than a threshold 5; and

–	the difference between the average quantization parameter for the current picture and the average quantization parameter for the subsequent picture is greater than a threshold 5.

An illustration of a peak in average quantization parameter is shown in Figure 6-5 below.

Figure 6-5 – Peak in average quantization parameter for a picture

The pseudocode for the intra-picture flicker estimation is described below:

estimateIntraPictureFlicker(b_prev_pic_is_intra_like,

d_avg_qp_curr_pic

d_avg_qp_prev_pic,

d_avg_qp_2nd_prev_pic)

if (b_prev_pic_is_intra_like) {

if (d_avg_qp_prev_pic > (d_avg_qp_curr_pic + 5) &&

d_avg_qp_prev_pic > (d_avg_qp_2nd_prev_pic + 5)) {

return 1 // Intra-picture flicker detected

}

}

return 0

}

### 6.2.2	Slicing module parameters

Input  = [i_nbr_mbs,

d_avg_frame_rate,

i_displayorder,

i_received_packets,

i_lostpackets,

i_received_bytes,

i_frame_type,

b_reflost0,

i_lostframegap,

p_mb_data_array[]]

Output = [i_processed_frames,

p_frame_parameters[]]

struct Frame_parameters

{

i_ecmbs,

i_frametype,

i_ecdist,

b_reflost0,

d_IntraRatio,

i_totalpackets,

i_lostpackets,

i_totalbytes,

i_avgbytesPerframe,

d_avgmv,

d_residual_var,

b_fade_pic,

b_scut_candidate,

b_scut_loss,

d_LoVA,

struct MB_parameters *p_mb_parameters

}

struct Frame_parameters * p_frame_parameters[MAXFRAMES]

struct MB_parameters

{

d_mvmedian,

b_ec,

b_mvexist,

b_intramode,

d_residual_energy,

d_residual_var,

d_lova

}

#### 6.2.2.1	Input parameter description

i_nbr_mbs				See clause 6.1.2.2

d_avg_frame_rate		See clause 6.1.1.2

p_mb_data_array[]	See clause 6.1.3.1

For all other input parameters see clause 6.1.3.2.

#### 6.2.2.2	Output parameter description

i_processed_frames	The number of parsed frames. This variable is increased by one when a frame is parsed by the ITU-T H.264/AVC parser using the method of keeping track of lost pictures with frame_num as described in clause 6.1.3.3.1.

p_frame_parameters[]	Array holding parameters for each parsed picture.

The following output parameters are calculated or saved from parser information for each parsed picture in p_frame_parameters[]:

i_ecmbs				The number of error concealed MBs in a frame.

i_frametype			Frame type for the picture. Default/initial value is 5, i.e., unknown frame type.

i_ecdist					An error concealment distance, defined as a distance, in display order, between the to-be-concealed picture and the concealing picture (i.e., the picture used for error concealment). It is set to the distance between the to-be-concealed picture and the immediately previous decoded reference picture. It is explained in clause 6.2.2.3 how this parameter is derived. Default/initial value is a larger value, e.g., 0xFFFF.

b_reflost0				See clause 6.1.3.2. Default/initial value is FALSE

d_IntraRatio			The ratio of the number of intra-macroblocks to all received macroblocks, i.e., i_nbr_intraMB / (i_nbr_mbs – i_ecmbs).

i_totalpackets			The total number of packets belonging to a frame. It includes the received packets and the estimate of the lost packets of the frame.

i_lostpackets			See clause 6.1.3.2.

i_totalbytes				The number of total bytes belonging to a frame. It includes the received bytes and the estimate of the bytes of lost packets.

i_avgbytesPerframe	The average number of bytes per frame calculated in a sliding window of L=10 frames. Initial value is 0.

d_avgmv				Average motion of the inter-prediction MBs in the frame. Initial value is 0.

d_residual_var			Average variance of the prediction residual signal in the frame. Initial value is 0.

b_fade_pic				Boolean indicating whether a frame have video content of gradual transition effects. The default/initial value is FALSE.

b_scut_candidate		Boolean indicating whether this frame is a scene cut candidate frame. The default/initial value is FALSE.

b_scut_loss				Boolean indicating whether a frame having packet loss is a scene cut frame. The scene cut frame can be current frame or the immediately previous reference frame that is lost completely. The initial value is FALSE.

d_LoVA				The level of visible artefacts in a frame.

p_mb_parameters[]	Pointer to an array of MB parameters. Its size is i_nbr_mbs.

The following output parameters are calculated or saved for each MB in p_mb_parameters[].

d_mvmedian			The magnitude of the median of the motion vectors of the macroblock.

b_ec					Boolean indicating whether error concealment is applied to the macroblock or not. Initial value is FALSE.

b_mvexist				If the i_mb_type is intra mode or the MB is error concealed, its value is FALSE. Otherwise, TRUE. Initial value is FALSE.

b_intramode			Boolean indicating if this MB is encoded with intra mode.

d_residual_energy		The total energy of prediction residual of the MB.

d_residual_var			Variance (i.e., AC energy) of the prediction residual signal of the MB. Initial value is 0.

d_lova					The level of visible artefacts of the MB.

#### 6.2.2.3	Algorithms

The main components are shown in Figure 6-6. First, a parsing of the sequence is made to derive some frame-level parameters in structure p_frame_parameters[] and some MB-level parameters in structure p_mb_parameters[]. Secondly, detect if a frame is a gradual transition frame and set b_fade_pic correspondingly. For each of the frames whose i_lostpackets > 0, detect if it is candidate scene cut frame and set b_scut_candidate correspondingly. If b_scut_candidate is true and the frame is not a gradual transition frame, further detect if the frame is a scene cut frame and set b_scut_loss correspondingly.

Figure 6-6 – Overview of components

The important output parameters in this stage consist of MB-level parameters, i.e., d_mvmedian, d_residual_energy, and frame-level parameters, i.e., b_fade_pic, b_scut_candidate, b_scut_loss. These parameters will be used to estimate the visible artefact level due to packet loss in each frame of the sequence. Then the frames' visible artefact levels are aggregated into a sequence-level visible artefact metric.

The important parameters are estimated below. How to use these parameters to estimate the visible artefacts due to packet loss in each frame of the sequence is described in clause 6.3.2.

##### 6.2.2.3.1	Obtain motion and residual energy of MBs

d_mvmedian is the magnitude of the median of the motion vectors of the macroblock. d_residual_energy is the energy of prediction residual of a macroblock. These two parameters are used in the next clause to estimate the level of visible artefacts in an error concealed (EC) MB. An EC MB may be a lost MB or an MB that cannot be parsed due to loss of sync of ITU-T H.264 syntax.

For a correctly received MB, these two parameters can be obtained as shown in pseudocode.

For an EC MB, the motion vector and residual information of the EC MB are not available directly from the bitstream, and thus, d_mvmedian and d_residual_energy need to be estimated.

The estimation of d_mvmedian for an EC MB depends on the frame type of the frame that the EC MB belongs to. If the EC MB is in an Intra frame, d_mvmedian of the EC MB is set to the d_mvmedian of a collocated macroblock in the closest decoded reference frame.

If an EC MB is in a non-Intra frame (e.g., a P frame), the d_mvmedian of the EC macroblock is set to that of the collocated macroblock in a previous reference frame.

For a block of an inter-predicted macroblock, its motion vectors are normalized by the distance between the current frame and its forward reference frame before used to calculate d_mvmedian.

The residual energy d_residual_energy of a non-EC MB is calculated as the sum of energy of de-quantized transform.

For an EC MB, its d_residual_energy is set to the corresponding value of the collocated MB in its previous reference frame.

The pseudocode functions for obtaining the MB-level parameters are:

void get_MB_parameters (int frmidx)

{

struct MB_parameters *currfrm_mbparameter, *prevfrm_mbparameter;

currfrm_mbparameter = p_frame_parameters[frmidx]-> p_mb_parameters;

prevfrm_mbparameter = p_frame_parameters[i_prevfrm]-> p_mb_parameters;

/*get d_mvmedian, d_residual_var, d_residual_energy of each MB*/

if (i_frame_type == I_frame)

{

for (m=0;m< i_nbr_mbs;m++)

{

currfrm_mbparameter[m].b_intramode =

(p_mb_data_array[m].i_mb_type == INTRA_MB);

currfrm_mbparameter[m].b_ec = (p_mb_data_array[m].i_mb_type ==UNKNOWN);

if (currfrm_mbparameter[m].b_ec == false)

get_DCT_energy(p_mb_data_array[m],currfrm_mbparameter[m]);

else

{

currfrm_mbparameter[m].d_residual_energy =

prevfrm_mbparameter[m].d_residual_energy;

// Variance of residual signal is used for scene cut detection, thus

// fill in with value of collocated MB in previous I-frame

int k = i_displayorder -1;

while (p_frame_parameters[k]->frametype != I_frame)

k--;

currfrm_mbparameter[m].d_residual_var =

p_frame_parameters[k]-> p_mb_parameters[m].d_residual_var;

}

currfrm_mbparameter[m].d_mvmedian =

prevfrm_mbparameter[m].d_mvmedian;

}

}

else //inter-predicted frame

{

for (m=0;m< i_nbr_mbs;m++)

{

currfrm_mbparameter[m].b_intramode =

(p_mb_data_array[m].i_mb_type == INTRA_MB);

currfrm_mbparameter[m].b_ec = (p_mb_data_array[m].i_mb_type == UNKNOWN);

if (currfrm_mbparameter[m].b_ec == false)

{

if (p_mb_data_array[m].i_mb_type != INTRA_MB)

{

currfrm_mbparameter[m].b_mvexist = true;

currfrm_mbparameter[m].d_d_mvmedian =

sqrt(pow(getMedian(p_mb_data_array[m].i_mvx),2) +

pow(getMedian(p_mb_data_array[m].i_mvy),2));

}

get_DCT_energy(p_mb_data_array[m], currfrm_mbparameter[m]);

}

else

{

currfrm_mbparameter[m].d_mvmedian =

prevfrm_mbparameter[m].d_mvmedian;

currfrm_mbparameter[m].d_residual_var =

prevfrm_mbparameter[m].d_residual_var;

currfrm_mbparameter[m].d_residual_energy =

prevfrm_mbparameter[m].d_residual_energy;

}

}

}

}

void get_DCT_energy(p_mb_data_array[m],currfrm_mbparameter[m])

{

double dc = 0;

double energy_MB = 0;

if ((i_cbp)%16 > 0 || i_mb_type == INTRA16x16_MB)

{

for (int i=0;i<16;i++)

{

for (int j=1; j<16; j++)

energy_MB += pow(i_dct_coef_ac[i][j], 2.0);

}

if (i_mb_type == INTRA16x16_MB)

{

for (int i=0;i<16;i++)

{

energy_MB += pow(i_dct_coef_dc[i], 2.0);

}

dc = i_dct_coef_dc[0] /16.0;

}

else

{

for (int i=0; i<16;i++)

{

energy_MB += pow(i_dct_coef_ac[i][0], 2.0);

dc += i_dct_coef_ac[i][0];

}

dc /= (16*4);

}

currfrm_mbparameter[m]->d_residual_energy =

energy_MB/256 * Qstepsqr(i_qp);

currfrm_mbparameter[m]->d_residual_var =

currfrm_mbparameter[m]->d_residual_energy – dc*dc*Qstepsqr(i_qp);

}

}

Qstep2QP is the square of quantization step. QP is the quantization parameter of the MB, which comes from syntax parsing. The quantization step is calculated as

QstepQP=Qstep(QP%6)∙2floor(QP/6)

Qstep0=4026=0.625

The QP takes value from 0 to 51.

##### 6.2.2.3.2	Obtain frame-level parameters

For frame frmidx having packet losses or having immediately preceding frame(s) lost, we calculate a parameter, avgpackets_perframe, by averaging i_totalpackets of the previous (non-I) frames in a sliding window of length N (here N = 10), that is, avgpackets_perframe is defined as the average (estimated) number of transmitted packets preceding the current frame.

In addition, the average number of bytes per packet, i_avgbytesPerframe, may be calculated by averaging i_totalbytes of immediately previous frames in a sliding window of N frames. For correctly received frame, i_totalbytes is set to i_received_bytes.

The pseudocode functions for calculating the frame-level parameters i_avgbytesPerframe, i_totalbytes are given below.

void get_frame_parameters(int frmidx)

{

p_frame_parameters[frmidx]->i_frametype = i_frame_type;

p_frame_parameters[frmidx]->b_reflost0 = b_reflost0;

p_frame_parameters[frmidx]->i_ecdist = frmidx – i_prevfrm;

p_frame_parameters[frmidx]->i_lostpackets = i_lostpackets;

struct MB_parameters *currfrm_mbparameter;

currfrm_mbparameter = p_frame_parameters[frmidx]-> p_mb_parameters;

acenergy = 0; cnt_goodmbs = 0;

avgmotion = 0; cnt_mv = 0;

intraMBs = 0;

for (int m=0;m<i_nbr_mbs;m++)

{

if (currfrm_mbparameter[m].b_ec == false)

{

cnt_goodmbs ++;

acenergy += currfrm_mbparameter[m].d_residual_var;

if (currfrm_mbparameter[m].b_mvexist)

{

avgmotion += currfrm_mbparameter[m].d_mvmedian;

cnt_mv ++;

}

intraMBs += (currfrm_mbparameter[m].b_intramode?1:0);

}

}

p_frame_parameters[frmidx]->i_ecmbs = i_nbr_mbs – cnt_goodmbs;

p_frame_parameters[frmidx]->d_IntraRatio = intraMBs/cnt_goodmbs;

p_frame_parameters[frmidx]->d_residual_var = acenergy/cnt_goodmbs;

if (cnt_mv > i_nbr_mbs/2)

p_frame_parameters[frmidx]->d_avgmv = avgmotion/cnt_mv;

else

p_frame_parameters[frmidx]->d_avgmv =

p_frame_parameters[i_prevfrm]->d_avgmv;

p_frame_parameters[frmidx]->i_totalpackets = i_received_packets;

p_frame_parameters[frmidx]->i_totalbytes = i_received_bytes;

// if loss occurs, calculate i_avgbytesPerframe and avgpackets_perframe in a

// sliding window of 10 frames; update i_totalpackets and i_totallbytes

if (p_frame_parameters[frmidx]->i_lostpackets > 0)

{

winLen = 10;

i_dividnum = 0;

avgbytes_perframe = 0;

avgpackets_perframe = 0;

for (int s = frmidx – i_lostframegap – winLen; s < frmidx; s++)

{

if(s < 0) break;

if(p_frame_parameters[s]->i_frametype == P_frame)

{

i_dividnum ++;

avgbytes_perframe += p_frame_parameters[s]->i_totalbytes;

avgpackets_perframe += p_frame_parameters[s]->i_totalpackets;

}

}

if (i_dividnum >0)

{

avgbytes_perframe /= i_dividnum;

avgpackets_perframe /= i_dividnum;

p_frame_parameters[frmidx]->i_avgbytesPerframe = avgbytes_perframe;

}

p_frame_parameters[frmidx]->i_totalpackets = i_received_packets  +

max(0,(i_lostpackets – avgpackets_perframe * i_lostframegap));

p_frame_parameters[frmidx]->i_totalbytes = i_received_bytes +

max(0,i_lostpackets * avgbytes_perframe/avgpackets_perframe –

avgbytes_perframe * i_lostframegap);

}

}

The pseudocode for obtaining the above MB-level and frame-level parameters in a sequence is:

void get_internal_parameters ()

{

i_processed_frames = 0;

i_prevfrm = 0;

while (get_next_pic_data is successful)

{

get_MB_parameters(i_displayorder);

get_frame_parameters(i_displayorder);

// A global variable record the previous parsed frame. It will be used in

// get_mb_parameters() function and get_frame_parameters() function

i_prevfrm = i_displayorder;

i_processed_frames ++;

}

// Count in lost frames. Assumption: the last frame of a sequence is not lost

for (i=0;i<i_processed_frames; i++)

{

if (p_frame_parameters[i]->i_frametype == UNKNOWN)

i_processed_frames ++;

}

}

##### 6.2.2.3.3	Detect scene cut candidate frames where packet loss occurs

When there is a significant scene change between two adjacent pictures and packet loss occurs in the second picture of the two adjacent pictures, the concealed second picture will have very strong visible artefacts. These artefacts are defined as scene cut artefacts in this Recommendation. Scene cut artefacts may also be detected in the first-received picture after one or more consecutive pictures have been lost completely and the first-received picture is compressed using a lost picture as reference picture.

Scene cut artefacts occur at partially received scene cut frames or at frames referring to lost scene cut frames. Thus, the frames with or surrounding packet losses may be regarded as potential scene cut artefact locations. Since a scene cut frame is usually encoded as an I-frame, a partially received I-frame may be marked as a candidate location for scene cut artefacts. A scene cut frame may also be encoded as a non-intra (for example, a P-frame). Scene cut artefacts may also occur in such a frame when it is partially received. A frame may also contain scene cut artefacts if it refers to a completely lost scene cut frame. When the total bytes of the frame (i_totalbytes) is much larger than the average bytes per frame in a sliding window near the frame (i_avgbytesPerframe), the frame may be identified as a candidate scene cut frame in the decoded video.

The flow chart for deriving b_scut_candidate is shown in Figure 6-7.

Figure 6-7 – Flow chart of detecting scene cut candidate frame

The pseudocode to derive b_scut_candidate is:

void mark_candidate_scut(int frmidx)

{

// Note: p_frame_parameters[frmidx]-> is omitted for each of the variables

// below.

double INTRA_IP = 0.8;

double scut_thrd_bytesRatio = 4.0;

if (i_lostpackets == 0)

return;

if (i_ecmbs > 0 && i_frametype == I_FRAME)

{

b_scut_candidate = true;

return;

}

else if(i_ecmbs > 0 && i_frametype == P_FRAME &&(d_IntraRatio > INTRA_IP))

{

b_scut_candidate = true;

return;

}

else

{

// i_totallbytes and i_avgbytesPerframe are calculated together with other

// frame-level parameters in function get_frame_parameters()

if (b_reflost0 > 0 && i_frametype != I_FRAME &&

i_totalbytes/i_avgbytesPerframe > scut_thrd_bytesRatio)

{

b_scut_candidate = true;

}

}

}

##### 6.2.2.3.4	Detect scene cut picture with artefacts

The flow chart for deriving b_scut_loss is shown in Figure 6-8. Frame of scene cut artefacts is detected for a candidate frame with b_scut_candidate equal to TRUE and b_fade_pic equal to FALSE.

When the candidate scene cut frame is an I-frame or P-frame with higher intra MB ratio, the prediction residual energy difference between the picture and a preceding I-frame or P-frame with higher intra MB ratio is calculated. If the difference between the energy factors is T1 times larger than the larger energy factor, the candidate I-frame is detected as a scene cut frame in the decoded video.

The prediction residual energy change or motion change around a scene change is often greater. Thus, when the residual energy difference or motion difference exceeds a threshold, the candidate frame is detected as having scene cut artefacts.

Figure 6-8 – Flow chart for detection scene cut frame

The pseudocode to derive b_scut_loss is:

void detect_scenecut (int frmidx, int currfrmid)

{

/*judge by energy difference between current and previous intra frames*/

double INTRA_IP = 0.8;

if (p_frame_parameters[frmidx]->d_IntraRatio >= INTRA_IP &&

p_frame_parameters[frmidx]->i_ecmbs <= i_nbr_mbs/3)

{

double thrd_T1      = 0.45;

double thrd_Ienergy = 36;

// Find immediately previous received frame with high intra ratio

int idx_prevIframe = i_displayorder -1;

while (idx_prevIframe >= 0)

{

if ((p_frame_parameters[idx_prevIframe]-> d_IntraRatio  >INTRA_IP) &&

p_frame_parameters[idx_prevIframe]->i_ecmbs <= i_nbr_mbs/3)

break;

idx_prevIframe --;

}

// Calculate residual energy difference

struct MB_parameters * currfrm_mbparameter, * prevfrm_mbparameter;

currfrm_mbparameter = p_frame_parameters[frmidx]->p_mb_parameters;

prevfrm_mbparameter = p_frame_parameters[idx_prevIframe]->p_mb_parameters;

mbcnt = 0;

currenergy = 0;

prevenergy =0;

for (int m=0; m< i_nbr_mbs; m++)

{

if (currfrm_mbparameter[m].b_ec == false)

{

mbcnt ++;

currenergy += currfrm_mbparameter[m].d_residual_var;

prevenergy += prevfrm_mbparameter[m].d_residual_var;

}

}

currenergy /= mbcnt;

prevenergy /= mbcnt;

if (max(currenergy,prevenergy) > thrd_Ienergy &&

abs(currenergy – prevenergy) / max(currenergy,prevenergy) > thrd_T1)

{

p_frame_parameters[i_displayorder]->b_scut_loss = true;

return;

}

}

else

{

// Detect by energy difference or motion difference around a scene change

double thrd_T2 = 0.7;

double thrd_T3 = 0.7;

double thrd_rsd_low = 4;

double thrd_motion_low = 8;

int Lwin = 10;

cnt_bef = cnt_aft = 0;

bef_rsd = aft_rsd = 0;

bef_motion = aft_motion = 0;

for (j=1; j< Lwin;j++)

{

pos = frmidx -j;

if (pos < 0)

break;

// Less likely to have two scene changes in a window of 10 frames

if (p_frame_parameters[pos]->b_scut_loss ||

p_frame_parameters[pos]->b_fade_pic)

{

cnt_bef = 0;

break;

}

if (p_frame_parameters[pos]->i_frametype == P_frame)

{

cnt_bef ++;

bef_rsd    += p_frame_parameters[pos]->d_residual_var;

bef_motion += p_frame_parameters[pos]->d_avgmv;

}

}

for (j=1; j< Lwin;j++)

{

pos = frmidx +j;

if (pos == currfrmid)

break;

// Less likely to have two scene changes in a window of 10 frames

if (p_frame_parameters[pos]->b_fade_pic)

{

cnt_bef = 0;

break;

}

if (p_frame_parameters[pos]->i_frametype == P_frame)

{

cnt_aft ++;

aft_rsd    += p_frame_parameters[pos]->d_residual_var;

aft_motion += p_frame_parameters[pos]->d_avgmv;

}

}

if (cnt_bef > 0 && cnt_aft > 0)

{

bef_rsd /= cnt_bef;

bef_motion /= cnt_bef;

aft_rsd /= cnt_aft;

aft_motion /= cnt_aft;

if (max(bef_rsd,aft_rsd) > thrd_rsd_low &&

abs(bef_rsd – aft_rsd)/max(bef_rsd,aft_rsd) > thrd_T2)

p_frame_parameters[frmidx]->b_scut_loss = true;

else if (max(bef_motion, aft_motion) > thrd_motion_low &&

abs(bef_motion – aft_motion)/max(bef_motion,aft_motion)> thrd_T3)

p_frame_parameters[frmidx]->b_scut_loss = true;

}

}

}

##### 6.2.2.3.5	Detect gradual transition picture

When one scene changes gradually to another scene, and if packet loss occurs in a gradual transition picture, the artefacts in the error concealed picture are less visible. This is quite contrary to scene cut artefacts. Even if the scene changes gradually, the energy or motion difference of these scenes may still highly probably be large. Thus, it is better to detect if a scene cut candidate frame is a gradual transition picture. If the scene cut candidate frame is a gradual transition picture, the scene cut artefacts detection algorithm in Figure 6-8 will not be applied.

The flow chart for deriving b_fade_pic is shown in Figure 6-9. The idea is that consecutive frames in a shorter period having larger intra MB ratio than their surrounding frames in a longer period are gradual transition picture positions with higher probability.

For a frame, if its intra-MB ratio is not larger than a lower threshold INTRA_THRDLOW, the frame is most probably not a gradual transition picture. Otherwise, check how many consecutive frames having intra-MB ratio larger than INTRA_THRDLOW, in the surrounding frames in a short window of 2×winshort frames' length, and increase the counter cnt_short correspondingly. These frames are candidate gradual transition frames. If cnt_short is less than a threshold THD_FADEPICS, the frame is less likely to be gradual transition picture, because gradual transition content generally takes time to be viewed as fade-in and fade-out pictures. The threshold THD_FADEPICS is set to correspond to 0.1 second. And this threshold should not be less than 2 frames; otherwise, the frame is actually a scene cut.

If cnt_short is larger than a threshold THD_FADEPICS, calculate the average intra-MB ratio of the candidate gradual transition frames, i.e., fadeintra. Calculate the average intra-MB ratio, fadeavg, of the inter-prediction frames, i.e., P-frames (excluding the candidate gradual transition frames), in the surrounding frames in a longer window of 2×winlong frames' length. If the ratio fadeintra/ fadeavg is larger than a threshold THD_FADERATIO, then the current frame is detected as a gradual transition frame.

Figure 6-9 – Flow chart of detecting a gradual transition picture

The pseudocode to derive b_fade_pic is:

void detect_transitionPicture(int frmidx, int currfrmid)

{

int winshort = d_avg_frame_rate * 0.5;

int winlong = d_avg_frame_rate * 1.5;

double INTRA_THRDLOW = 0.3;

int THD_FADEPICS = max(2,( d_avg_frame_rate * 0.1 + 0.5));

double THD_DIVFRMES =  d_avg_frame_rate *1.0;

double THD_FADERATIO = 3;

float fadeavg,fadeintra;

int cnt, pos, pos_bef, pos_aft,i;

/* consecutive frames having larger intra ratio are potential

fade-in and fade-out position */

fadeintra = 0; cnt = 0;

if (p_frame_parameters[frmidx]->d_IntraRatio > INTRA_THRDLOW)

{

fadeintra += p_frame_parameters[frmidx]->d_IntraRatio; cnt ++;

}

else

return;

for (i= 1; i< winshort; i++)

{

pos = frmidx – i;

if (pos <= 0 || p_frame_parameters[pos]->d_IntraRatio < INTRA_THRDLOW)

break;

if (p_frame_parameters[pos]->i_frametype == I_frame)

{

fadeintra += 0.6; cnt ++;

}

else if (p_frame_parameters[pos]->i_frametype == P_frame)

{

fadeintra += p_frame_parameters[pos]->d_IntraRatio; cnt ++;

}

}

pos_bef = i;

for (i= 1; i<winshort; i++)

{

pos = frmidx + i;

if (pos == currfrmid ||p_frame_parameters[pos]->d_IntraRatio <

INTRA_THRDLOW)

break;

if (p_frame_parameters [pos]->i_frametype == I_frame)

{

fadeintra += 0.6; cnt ++;

}

else if (p_frame_parameters[pos]->i_frametype == P_frame)

{

fadeintra += p_frame_parameters[pos]->d_IntraRatio; cnt ++;

}

}

pos_aft = i;

if (cnt < THD_FADEPICS)

return;

fadeintra /= cnt;

/*the average intra MB ratio of potential fading frames should be much larger than the average intra MB ratio in the long window */

fadeavg = 0; cnt = 0;

for (i= pos_bef; i<winlong; i++)

{

pos = frmidx – i;

if (pos <= 0)

break;

if (p_frame_parameters[pos]->i_frametype == P_frame)

{

fadeavg += p_frame_parameters[pos]->d_IntraRatio; cnt ++;

}

}

for (i= pos_aft; i<winlong; i++)

{

pos = frmidx + i;

if (pos == i_processed_frames)

break;

if (p_frame_parameters[pos]->i_frametype == P_frame)

{

fadeavg += p_frame_parameters[pos]->d_IntraRatio; cnt ++;

}

}

if (cnt > THD_DIVFRMES)

{

fadeavg /= cnt;

if (fadeintra > fadeavg * THD_FADERATIO)

p_frame_parameters[frmidx]->b_fade_pic = true;

}

}

##### 6.2.2.3.6	Determine error propagation

One parameter used for the quality estimation of the video bitstream is the maximum number of macroblock partitions for a picture for pictures having a propagating error. The error propagation algorithm is run for each picture in the received video bitstream. The algorithm determines an error occurrence in a picture and determines a visual degradation caused by the error. If the error is estimated to be visible, a penalty value is assigned for the macroblock. No penalty value is assigned to the error for the macroblock if the macroblock is a skip or an estimated skip. If the previous co-located macroblock, i.e., the macroblock at the same position in the previous picture, was lost and the current macroblock is of skip type, it is estimated that the co-located macroblock also was of skip type and losing it would not cause a visible artefact. The penalty value is reset upon reception of an intra-coded macroblock or an intra-coded picture frame. Finally error propagation is determined taking into account the temporal propagation of the error. The procedure is illustrated in the flow chart in Figure 6-10.

Figure 6-10 – Flow chart for the error propagation detection algorithm

The visual degradations comprising the penalty values for each partition are collected in a loss penalty map, i_loss_penalty_map[], with the same resolution as the macroblock map. In addition to the indication of error propagation, b_error_is_propagating, the updated i_loss_penalty_map[] is also returned from this function.

The pseudocode for updating the loss penalty map and determining error propagation is shown below. For input parameters to this function, see clause 6.2.1.1.

determineErrorPropagation(i_nbr_mbs,

i_curr_mb_type_map[],

i_prev_mb_type_map[],

i_loss_penalty_map[]) {

b_error_is_propagating = FALSE

b_has_loss_penalty     = FALSE

for (i_mb = 0; i_mb < i_nbr_mbs; i_mb++) {

// Reset error propagation if intra macro block

if (i_curr_mb_type_map[i_mb] == INTRA_MB) {

i_loss_penalty_map[i_mb] = 0

// If macro block has been lost set that error is propagating for pic

} else if (i_curr_mb_type_map[i_mb] == LOST) {

b_error_is_propagating = TRUE

// Set error propagation for macro block if previous macro block was

// lost and current macro block is not skipped. If the current macro block

// is skipped it is estimated that the previous one was also skipped and

// that a loss would not cause any visual artifact

} else if (i_prev_mb_type_map[i_mb] == LOST &&

i_curr_mb_type_map[i_mb] != SKIPPED_MB) {

i_loss_penalty_map[i_mb] = 1

}

// Check if the loss penalty map contains a loss penalty

if (i_loss_penalty_map[i_mb]) {

b_has_loss_penalty = TRUE

}

}

// Set that error is propagating if there is a loss penalty

if (b_has_loss_penalty) {

b_error_is_propagating = TRUE

} else {

b_error_is_propagating = FALSE

}

return [b_error_is_propagating, i_loss_penalty_map]

}

##### 6.2.2.3.7	Find max number of macroblock partitions

This algorithm extracts the maximum number of partitions, i_max_nbr_mb_partitions, in the picture if a propagating error occurs. In ITU-T H.264/AVC, each 16x16 macroblock can be further split in smaller blocks for the motion estimation process, i.e., blocks of size 16x8, 8x16, or 8x8. If the 8x8 partition size is chosen, each 8x8 partition may be further divided in sub-partitions of size 8x8, 8x4, 4x8, or 4x4 luma samples. Thus, the parameter i_max_nbr_mb_partitions is equal to the maximum number of partitions of the correctly received macroblocks in the frame where the propagating error occurs. The idea is that the more complex a scene is, the larger number of macroblock partitions will be used per macroblock. Also, the more complex a scene is, the more visible a loss will become.

The algorithm for determining the maximum number of macroblock partitions for a picture can be expressed with the following pseudocode. For input parameters to this function, see clause 6.2.1.1.

findMaxNbrMbPartitions(i_nbr_mbs,

b_error_is_propagating,

i_curr_mb_type_map)

i_max_nbr_mb_partitions = 0

if (b_error_is_propagating) {

for (i_mb = 0; i_mb < i_nbr_mbs; i_mb++) {

if (i_curr_mb_type_map[i_mb] == INTER16x16_MB) {

i_nbr_mb_partitions = 1

} else if (i_curr_mb_type_map[i_mb] == INTER16x8_MB ||

i_curr_mb_type_map[i_mb] == INTER8x16_MB) {

i_nbr_mb_partitions = 2

} else if (i_curr_mb_type_map[i_mb] == INTER8x8_MB ||

i_curr_mb_type_map[i_mb] == INTER8x8REF0_MB) {

i_nbr_mb_partitions = 4

} else {

i_nbr_mb_partitions = 0

}

if (i_nbr_mb_partitions > i_max_nbr_mb_partitions) {

i_max_nbr_mb_partitions = i_nbr_mb_partitions

}

}

}

return i_max_nbr_mb_partitions

}

### 6.2.3	Freezing module parameters

The frame-level parameter that is used by freezing module is P_frame_parameters[]->d_avgmv. It is calculated as described in clause 6.2.2.3.

## 6.3	Aggregation of parameters into model parameters

This clause describes how the aggregation of parameters into model parameters is made for each of the degradation modules.

### 6.3.1	Compression module parameters

The aggregation of parameters into compression module model parameters has the following input and output parameters.

Input  = [d_avg_frame_rate,

i_nbr_received_pics,

i_nbr_intra_pics,

i_nbr_inter_pics,

d_sum_avg_pic_qp,

d_sum_avg_pic_mv]

Output = [d_avg_avg_pic_qp,

d_avg_avg_pic_mv,

d_key_frame_rate]

#### 6.3.1.1	Input parameter description

d_avg_frame_rate		See clause 6.1.1.2

For all other input parameters see clause 6.2.1.2.

#### 6.3.1.2	Output parameter description

d_avg_avg_pic_qp		Average of the average QP of all received pictures in the sequence, where the average QP is the averaged macroblock QP per picture.

d_avg_avg_pic_mv		Average motion vector calculated as the average vertical and horizontal motion vectors averaged over all macroblocks, in the sequence, that are not intra or have been lost.

d_key_frame_rate		Estimated key frame rate calculated as the frame rate of the sequence divided by number of pictures between two intra pictures. The number of pictures between two intra pictures are calculated as the number of received inter pictures in the sequence divided by the number of received intra pictures in sequence.

#### 6.3.1.3	Algorithms

Below is the pseudocode for aggregating parameters into model parameters for the compression module.

d_avg_avg_pic_qp = d_sum_avg_pic_qp / i_nbr_received_pics

d_avg_avg_pic_mv = d_sum_avg_pic_mv / i_nbr_received_pics

d_nbr_of_pics_between_two_intra_pics =

i_nbr_inter_pics / (double) i_nbr_intra_pics

d_key_frame_rate = d_avg_frame_rate / d_nbr_of_pics_between_two_intra_pics

### 6.3.2	Slicing module parameters

Input  = [i_processed_frames,

i_nbr_mbs,

p_frame_parameters[],

d_avg_frame_rate]

Output = [d_LoVA_seq]

#### 6.3.2.1	Input parameter description

i_processed_frames	See clause 6.2.2.2

i_nbr_mbs					See clause 6.1.2.2

p_frame_parameters[]	See clause 6.2.2.2

d_avg_frame_rate 			See clause 6.1.1.2

#### 6.3.2.2	Output parameter description

d_LoVA_seq			Measurement of level of visible slicing artefacts exist in a sequence of pictures. The value is a non-negative number. A value of 0 means there is no visible slicing artefact. If s_video_PLC_mode is not "SLICING", this value should be 0.

#### 6.3.2.3	Algorithms

Generally, the goal of error concealment is to estimate EC MBs in order to minimize perceptual quality degradation. The perceived strength of artefacts produced by transmission errors depends heavily on the employed error concealment techniques. For example, if a frame far away from a current frame is used to conceal a current macroblock, the concealed macroblock is more likely to have visible artefacts. In addition, the artefact strength is also related to the video content. For example, a slow moving video is easier to be concealed. Thus, parameters, such as motion vectors and error concealment distance, can be used to assess the error concealment effectiveness and the quality of concealed video at a bitstream level.

Residual information results from motion-compensated prediction. It is observed that the energy of residual information (also referred to as residual energy) of a macroblock reflects the texture complexity and motion complexity of the macroblock.

In smooth areas of some video sequences, for example, in sky and grassland which are usually easy to be concealed, the estimated motion vectors may be large even if the movement between pictures is small. Consequently, a video quality measurement based on motion vectors may falsely estimate strong visible artefacts even though the concealed areas have good visual quality. By contrast, the residual energy in the smooth areas may be relatively small and may provide better indication about the perceived visual quality. Thus, residual energy is used as another parameter in estimating the artefact level.

Visual artefacts may still be perceived after error concealment, because error concealment may be not effective therein. Such visual artefacts caused by EC MB are denoted as initial visible artefacts. If a block having initial visible artefacts is used as a reference, for example, for intra prediction or inter prediction, the initial visible artefacts may propagate spatially or temporally to other macroblocks in the same or other frames through prediction. Such propagated artefacts are denoted as propagated visible artefacts. The overall artefacts, caused by initial and/or propagated visible artefacts, are denoted as overall visible artefacts.

For a MB indexed as (i,j) of frame n, its initial level of visible artefacts d_lova is set as

LoVAinitcurrn,i,j=0,                                                                                                                     non EC MBminf1(d_mvmediannij×ecdist/4.0),f2(d_residual_energynij), EC MB	  (1)

fx= v1,                                                                         x<T1v2-v1T2-T1×x-T1 ,                                 T1≤x≤T2v2,                                                                         x>T2   	(2)

The reason for the dividend 4.0 in the above equation is that the motion estimation accuracy in [ITU-T H.264] is ¼ sub-pixel.

For f1 the constants take the following values: v1=0, v2=100; T1=1 and T2=8 in the unit of pixel.

For f2 the constants take the following values:

v1=0, v2=100; T1=1, and T2=64.

If its reference frame is lost completely, i.e., b_reflost0 == true, additional initial artefacts caused by lost reference frame are calculated for correctly received inter-predicted MB:

LoVAinitrefn,i,j=minf1(MVmgnt×ecdist/4.0),f2(d_residual_energynij)	(3)

where the magnitude of the median of its motion vectors pointed to the lost reference frame is calculated as MVmgnt. When there is no corresponding motion vector for the macroblock, MVmgnt is set to zero. For simplicity, MVmgnt is set to the value of d_mvmedian for the correctly received inter-predicted MB in implementation. Otherwise (i.e., b_reflost0 == false), LoVAinitrefn,i,j=0.

Considering both lost macroblocks in the current frame and the loss of reference frames, the initial visible artefact level is calculated as:

LoVAinitn,i,j= max⁡{LoVAinitcurrn,i,j,LoVAinitrefn,i,j}	(4)

The overall artefacts in a MB should consider both initial artefacts and propagated artefacts. The artefact level of a pixel d_lovan,x,y, where n is the frame index, (x,y) is the pixel's coordinate in the frame, is calculated as follows:

dn,x,y=maxdn-k,x',y',   LoVAinit(n,x/16,y/16),	(5)

where dn-k,x',y' is the propagated visible artefact for pixel (n,x,y), and is the artefact level at reference pixel (n-k,x',y') of the current pixel. The initial value of d_lovan,x,y is zero, meaning no visible artefact. In summary, how the artefact level propagates may be traced through motion vectors. In this implementation, it is sufficient to use zero motion vectors instead of accurate motion vectors to roughly track the temporal propagation of visible artefacts.

In case that the frame having scene-cut artefact, i.e., b_scut_loss == true, the above method to estimate visible artefacts of an EC MB does not work well, because an assumption is made when estimate the d_mvmedian and d_residual_energy of the EC MB. The assumption is that the current picture and the previous pictures are highly correlated, thus the motion and residual in these frames are similar. When scenes change, this assumption does not hold. Generally, the scene-cut artefact is very strong. So, modify the macroblock's visible artefact level to a larger value, as:

d_lovan,i,j= max⁡(d_lovan,i,j,  100)	(6)

and set the distortion of the MB's pixels, dn,x,y, to the value of d_lova(n,i,j) correspondingly for ensuing error propagation.

The visible artefact level of a frame n is calculated as the sum of all MBs' d_lova:

d_LoVA(n)  =j=0i_nbr_mb_veri=0i_nbr_mb_hord_lovan,i,j	(7)

For the frame that is lost completely (i.e., p_frame_parameters[n]->i_frame_type == UNKNOWN), set its d_LoVA(n) to that of its previous frame.

The sequence level artefact is calculated as:

d_LoVA_seq=log10(n=1i_processed_framesd_LoVA(n))/d_avg_frame_rate+1)	(8)

This metric d_LoVA_seq will be used as slicing module output to the framework algorithm.

Note that the metric d_LoVA_seq, which measures the level of visible slicing artefacts in a segment of sequence, can be used independently. Furthermore, the parameters used in calculating the metric d_LoVA_seq, are more related to video content characteristics. They are independent of encoder configuration, e.g., prediction modes. Thus, the metric can be used in a more general scenario.

The pseudocode for deriving the initial visible artefact estimation, propagated artefacts and overall visible artefact level of a sequence, d_LoVA_seq, is:

void EC_EP_artifacts (frmidx)

{

int MV_UPTHRD = 8;

int MV_LOWTHRD = 1;

int RSD_UPTHRD = 64;

int RSD_LOWTHRD = 1;

int MAX_ARTIFACT = 100;

struct MB_parameters * currfrm_mbparameter =

p_frame_parameters[frmidx]->p_mb_parameters;

struct MB_parameters * prevfrm_mbparameter =

p_frame_parameters[frmidx-1]->p_mb_parameters;

// For completely lost frame, its artifact level equal to previous frame's

if (p_frame_parameters[frmidx]->i_frametype == UNKNOWN)

{

p_frame_parameters[frmidx]->d_LoVA = p_frame_parameters[frmidx-1]->d_LoVA;

return;

}

// Estimate initial visible artifacts for EC MB according to Eq. (1)

if (p_frame_parameters[frmidx]->i_ecmbs > 0)

{

for (int i=0;i<i_nbr_mbs;i++)

{

if (currfrm_mbparameter[i].b_ec)

{

int judgement1 = currfrm_mbparameter[i].d_mvmedian /4.0 *

p_frame_parameters[frmidx]->i_ecdist;

if (judgement1 > MV_UPTHRD)

artifacts_level = MAX_ARTIFACT;

else if (judgement1 <= MV_LOWTHRD)

artifacts_level = 0;

else

artifacts_level = (judgement1 – MV_LOWTHRD) * MAX_ARTIFACT /

(MV_UPTHRD – MV_LOWTHRD);

int judgement2 = currfrm_mbparameter[i].d_residual_energy;

if (judgement2 > RSD_UPTHRD)

artifacts_level2 = MAX_ARTIFACT;

else if (judgement2 <= RSD_LOWTHRD)

artifacts_level2 = 0;

else

artifacts_level2 = (judgement2 – RSD_LOWTHRD) * MAX_ARTIFACT

/(RSD_UPTHRD – RSD_LOWTHRD);

currfrm_mbparameter[i].d_lova = min(artifacts_level, artifacts_level2);

}

}

}

// Additional initial artifacts caused by lost reference frame per Eq. (2)

if (p_frame_parameters[frmidx]->b_reflost0)

{

for (int i=0;i<i_nbr_mbs;i++)

{

if (!currfrm_mbparameter[i].b_intramode &&

currfrm_mbparameter[i].b_ec == false)

{

int judgement1 = abs(currfrm_mbparameter[i].d_mvmedian /4.0 *

p_frame_parameters[frmidx]->i_ecdist);

if (judgement1 > MV_UPTHRD)

artifacts_level = MAX_ARTIFACT;

else if (judgement1 <= MV_LOWTHRD)

artifacts_level = 0;

else

artifacts_level = (judgement1 – MV_LOWTHRD) * MAX_ARTIFACT

/(MV_UPTHRD – MV_LOWTHRD);

int judgement2 = currfrm_mbparameter[i].d_residual_energy;

if (judgement2 > RSD_UPTHRD)

artifacts_level2 = MAX_ARTIFACT;

else if (judgement2 <= RSD_LOWTHRD)

artifacts_level2 = 0;

else

artifacts_level2 = (judgement2 – RSD_LOWTHRD) * MAX_ARTIFACT

/(RSD_UPTHRD – RSD_LOWTHRD);

artifacts_level = EC min(artifacts_level, artifacts_level2);

// Count in the effects of both types of loss according to Eq. (5)

currfrm_mbparameter[i].d_lova =

max(artifacts_level, currfrm_mbparameter[i].d_lova);

}

}

}

// In case that the frame having scene cut artifact, modify the MB's

// visible artifact level to a larger value

if (p_frame_parameters[frmidx]->b_scut_loss == true)

{

for (int i=0;i<i_nbr_mbs;i++)

{

if (currfrm_mbparameter[i].b_ec || p_frame_parameters[frmidx]->b_reflost0)

currfrm_mbparameter[i].d_lova =

max(currfrm_mbparameter[i].d_lova,MAX_ARTIFACT);

}

}

// Track visible artifact propagation by zero motion vector,

// and stop artifact propagation when an Intra MB is correctly received

for (int i=0;i<i_=nbr_mbs;i++)  {

if (currfrm_mbparameter[i].b_intramode)

currfrm_mbparameter[i].d_lova = 0;

else

currfrm_mbparameter[i].d_lova =

max(currfrm_mbparameter[i].d_lova,prevfrm_mbparameters[i].d_lova);

}

}

### 6.3.3	Freezing module parameters

Input  = [i_processed_frames,

p_frame_parameters[]]

Output = [p_frame_parameters[]->i_freezing_pics]

#### 6.3.3.1	Input parameter description

See clause 6.2.2.2 for all input parameters.

#### 6.3.3.2	Output parameter description

i_freezing_pics			Length of a freeze with skipping. For the start frame of the freeze i_freezing_pics is set to the length of the freeze. For other frames, i_freezing_pics is set to zero. Initial value is zero.

#### 6.3.3.3	Algorithms

The pseudocode for deriving the length of each group of consecutively dropped frames in a sequence is:

void get_freezing_len (i_processed_frames)

{

bool startfreeze = false;

int startfrmidx = 0;

int i;

for (i=0; i< i_processed_frames; i++)

{

if (startfreeze)

{

// Correctly received I frame, stop freezing

if (p_frame_parameters[i]->i_frametype == I_frame &&

p_frame_parameters[i]->i_ecmbs==0)

{

startfreeze = false;

p_frame_parameters[startfrmidx]->i_freezing_pics = i – startfrmidx;

}

}

else if (p_frame_parameters[i]->i_frametype == UNKNOWN ||

p_frame_parameters[i]->i_ecmbs)

{

startfreeze = true;

startfrmidx = i;

}

}

// Freezing to the end of sequence

if (startfreeze)

p_frame_parameters[startfrmidx]->i_freezing_pics = i – startfrmidx;

}

### 6.3.4	Rebuffering module parameters

The inputs and output for the calculation of the rebuffering module model parameter are:

Input  = [d_rebuffering_duration,

i_nbr_received_pics,

i_nbr_lost_pics,

d_avg_frame_rate]

Output = [d_rebuffering_ratio]

#### 6.3.4.1	 Input parameter description

d_rebuffering_duration	See clause 6.1.1.2

i_nbr_received_pics	See clause 6.2.1.1

i_nbr_lost_pics			See clause 6.2.1.1

d_avg_frame_rate			See clause 6.1.1.2

#### 6.3.4.2 	Output parameter description

d_rebuffering_ratio	Ratio between the rebuffering duration of the sequence divided by the total duration of the sequence (including the rebuffering duration).

#### 6.3.4.3	Algorithms

The formulas for calculating d_rebuffering_ratio are:

i_total_nbr_pics = i_nbr_received_pics + i_nbr_lost_pics

d_tot_duration = i_total_nbr_pics / d_avg_frame_rate + d_rebuffering_duration

d_rebuffering_ratio = d_rebuffering_duration / d_tot_duration

## 6.4	Quality estimation model

The quality estimation model consists of quality estimation modules for compression, slicing, freezing with skipping and rebuffering degradation and a framework module that is used to combine the various degradations into a total video quality score.

### 6.4.1	Compression module

The compression quality degradation estimation module has the following input and output parameters:

Input  = [i_nbr_mb_ver,

d_avg_frame_rate,

d_avg_avg_pic_qp,

d_max_avg_pic_qp,

d_min_avg_pic_qp,

b_has_intra_pic_flicker,

i_max_nbr_used_ref_pics,

b_sub_mb_type_inter8x4_or_inter4x8_exists,

b_sub_mb_type_inter4x4_exists,

d_avg_avg_pic_mv,

d_key_frame_rate]

Output = [d_compression_artifact_value]

#### 6.4.1.1	Input parameter description

i_nbr_mb_ver			See clause 6.1.2.2

d_avg_frame_rate		See clause 6.1.1.2

d_avg_avg_pic_qp		See clause 6.3.1.2

d_max_avg_pic_qp	See clause 6.2.1.1

d_min_avg_pic_qp		See clause 6.2.1.1

b_has_intra_pic_flicker	See clause 6.2.1.1

i_max_nbr_used_ref_pics	See clause 6.1.3.1

b_sub_mb_type_inter8x4_or_inter4x8_exists

See clause 6.1.3.1

b_sub_mb_type_inter4x4_exists

See clause 6.1.3.1

d_avg_avg_pic_mv		See clause 6.3.1.2

d_key_frame_rate		See clause 6.1.1.2

#### 6.4.1.2	Output parameter description

d_compression_artifact_value

Estimation of the compression quality in the range [1.0, 5.0], where 1.0 is the worst possible quality and 5.0 the best possible quality.

#### 6.4.1.3	Coefficients

d_coef_compression = [6.3675,   -8.0077,       -9.6179,      5.9917,

-5.7356,   -0.10028,      -0.0096472,   0.034306,

6.3396,   -0.12657,      -0.02691,     0.17708,

0.0085058, 0.66455,       0.33821]

#### 6.4.1.4	Algorithm

The calculation of the compression quality score is made by combining a set of compression quality related parameters. Thus the objective quality measure is based on extracted video coding dependent parameters. These compression quality related parameters each contribute in different ways to the estimation of the d_compression_artifact_value.

The quantization parameter (QP), also referred to as quantizer, is a good first indication of the compression quality as it determines the amount of quantization to be applied on the transform coefficients of the video bitstream. Three QP parameters are used when calculating the compression degradation quality score; the QP per macroblock averaged over all macroblocks in all pictures, and the difference between the maximum value when averaging macroblock level QP per picture and the minimum value when averaging macroblock level per picture.

Additional parameters used for the calculation of the compression degradation quality score is the intra-picture flicker detection indicator, described in clause 6.2.1.3.4, and the average frame rate of the sequence. A low average frame rate may result in temporal artefacts when there is motion in the video.

Another parameter used for the compression degradation quality score is the i_encoder_complexity_level, which is related to the encoding complexity used by the encoder and is derived from complexity related information extracted from the encoded bitstream. The complexity related information being indicative of the complexity of the encoding, consists of the number of used reference pictures and the existence of various macroblock partition sizes, more precisely the existence of inter 8x4 partitions, the existence of inter 4x8 partitions and the existence of inter 4x4 partitions. Figure 6-11 shows a typical example of an inter MB which has been divided into one 16x8 partition, two 8x4 partitions, one 4x8 partition and two 4x4 partitions, during encoding. Testing all possible partitions during rate distortion optimization in the encoder is very processing consuming and may therefore not be considered for low complex encoders. Since it is possible to use several reference frames in a video bitstream, also such a decision can have impact on the encoding computational power.

Figure 6-11	 – Inter macroblock divided into one 16x8 partition, 
two 8x4 partitions, one 4x8 partition and two 4x4 partitions

The extracted information is compared to a set of pre-defined complexity settings and the matching level setting is used in combination with the additional quality related parameters when calculating a bitstream quality score for the encoded bitstream.

The quality impact of the motion of the sequence is also considered and included as the product of absolute vertical and horizontal motion vectors per macroblock averaged over all macroblocks and pictures in the sequence.

The key frame rate of the video signal as a GOP-length related parameter is also used when calculating the compression degradation quality score. The key frame rate kfr is estimated as kfr = fr/d where fr is the average frame rate and d the average number of frames between two intra frames, and is integrated with the other compression quality related parameters using the following formula:

d_compression_artifact_value = (a1×kfr+a2)×exp(b×cQ)	(9)

where a1, a2 and b are coefficients and cQ the result when combining the other compression quality related parameters.

The estimated compression quality degradation score is calculated according to the following pseudocode:

// Different QP curve for QCIF resolution and below

if (i_nbr_mb_ver < 11) {

d_coef_compression0 = d_coef_compression[8]

d_coef_compression1 = d_coef_compression[2]

} else {

d_coef_compression0 = d_coef_compression[0]

d_coef_compression1 = d_coef_compression[1]

}

i_qp_max = 51

// QP and frame rate impact

d_qp_and_framerate_impact =

d_coef_compression0 +

d_coef_compression1 * pow(d_avg_avg_pic_qp/i_qp_max, d_coef_compression[3]) +

d_coef_compression[4] * 1/d_avg_frame_rate +

d_coef_compression[5] * b_has_intra_pic_flicker +

d_coef_compression[6] *(d_max_avg_pic_qp – d_min_avg_pic_qp)

// Encoder complexity impact

i_encoder_complexity_level = 1

if (b_sub_mb_type_inter8x4_or_inter4x8_exists &&

b_sub_mb_type_inter4x4_exists) {

i_encoder_complexity_level = 3

}

if (i_max_nbr_used_ref_pics < 1) {

i_max_nbr_used_ref_pics = 1

}

if (i_max_nbr_used_ref_pics > 1) {

i_encoder_complexity_level += 1

}

d_encoder_complexity_impact = d_coef_compression[9] *

i_encoder_complexity_level

// Motion impact

d_motion_impact =

d_coef_compression[10]*d_avg_avg_pic_mv*(1-d_avg_frame_rate/30)

// Key frame rate estimation impact

d_key_frame_rate_estimation_impact =

d_coef_compression[12] * d_key_frame_rate + d_coef_compression[13]

// Combine into compression score

d_compression_artifact_value =

d_key_frame_rate_estimation_impact * exp (d_coef_compression[14] *

(d_qp_and_framerate_impact +

d_encoder_complexity_impact +

d_motion_impact))

// Make sure scores are between 1 and 5

d_compression_artifact_value = min(max(d_compression_artifact_value, 1.0), 5.0)

### 6.4.2	Slicing module

The following inputs and outputs are used when calculating a slicing artefact value indicative of the visible degradation due to slicing:

Input  = [d_LoVA_seq,

d_sum_max_nbr_partitions,

i_nbr_received_pics]

Output = [d_slicing_artifact_value]

#### 6.4.2.1	Input parameter description

d_LoVA_seq			See clause 6.3.2.2

d_sum_max_nbr_partitions

See clause 6.2.1.1

i_nbr_received_pics 	See clause 6.2.1.1

#### 6.4.2.2	Output parameter description

d_slicing_artifact_value	A float value indicating the estimated level of the slicing artefact for the sequence.

#### 6.4.2.3	Coefficients

d_coef_slicing  = 0.0479

#### 6.4.2.4	Algorithm

The average maximum number of partitions is calculated and combined with the visible artefacts metric d_LoVA_seq by a linear function to produce the overall slicing artefact value according to the following pseudocode:

d_avg_max_nbr_partitions = d_sum_max_nbr_partitions / i_nbr_received_pics

d_slicing_artifact_value =

d_LoVA_seq + d_coef_slicing * d_avg_max_nbr_partitions

### 6.4.3	Freezing module

The freezing with skipping degradation module has the following input and output parameters.

Input  = [p_frame_parameters[]->i_freezing_pics,

p_frame_parameters[]->d_avgmv,

d_avg_frame_rate,

i_processed_frames]

Output = [d_freezing_artifact_value]

#### 6.4.3.1	Input parameter description

i_freezing_pics			See clause 6.3.3.2

d_avgmv				See clause 6.2.2.2

d_avg_frame_rate		See clause 6.1.1.2

i_processed_frames	See clause 6.2.2.2

#### 6.4.3.2	Output parameter description

d_freezing_artifact_value	A float value indicating the estimated level of freezing artefact of the observed sequence. If no freezing artefact is present or s_video_PLC_mode is not "FREEZING" this value should be 5.0.

#### 6.4.3.3	Coefficients

d_MMax        = 5.0

d_MMin        = 3.5

d_MVInfluence = 2.5

d_c           = 4.0

#### 6.4.3.4	Algorithms

If the video contains a pause, also referred to as a freeze, with skipping, the distorted video contains a group of consecutive frames that are dropped. Every freeze with skipping (shorted as freeze) includes one or more consecutive pictures, which are dropped and replaced by the last correctly received picture for the displayed video.

The distortion created by a freeze is mainly influenced by two parameters: the length of the freeze, and the movement at the location where the pictures are dropped. For a freezestarted at the i-th frame of the observed video, a pause term that indicates the amount of distortion associated with the duration of a freeze is defined as:

5.0-(d_MMax-(d_MMax-d_MMin)/(1.0+d_c/d_freezing_length[i]))	(10)

When d_freezing_length[i]=+∞, the above equation reaches its maximal value 5.0-d_MMin, while it reaches its minimal value 5.0-dMMax when d_freezing_length[i]=0.

Secondly, a motion term that indicates the amount of distortion associated with change in motion for a freeze is defined as:

d_MVInfluence × d_mv[i]	(11)

Therefore, higher frame movement results in higher distortion.

By adding up the pause term and motion term, the distortion estimation of the current freeze is calculated as:

5.0-(d_MMax-(d_MMax-d_MMin)/(1.0+d_c/d_freezing_length[i]))+d_MVInfluence×d_mv[i] (12)

where d_MMax, d_MMin and d_MVInfluence are constants.

Finally the estimated degradation value is averaged by the number of total pictures and brought into a square root function to get the overall freezing estimation.

The freezing artefact level is estimated according to the following pseudocode:

Void FreezingWithSkip(i_processed_frames, d_avg_frame_rate)

{

d_Total = 0;

d_freezing_length = 0;

for (i = 0; i < i_processed_frames; i++)

{

if (i == 0 || p_frame_parameters[i]->i_freezing_pics==0) continue;

d_freezing_length =

p_frame_parameters[i]->i_freezing_pics /d_avg_frame_rate;

if (d_freezing_length <= 0.04) continue;

mvIndex = i-1;

if (p_frame_parameters[mvIndex]->i_frametype == I_frame && mvIndex>0)

mvIndex--;

d_mv = p_frame_parameters[mvIndex]->d_avgmv/4.0;

/* Normalize d_mv to range [0,1] */

if (d_mv > 64.0) d_mv =1.0;

else d_mv /= 64.0;

dQ = d_MMax-(d_MMax-d_MMin)/(1.0+d_c/d_freezing_length)+

d_MVInfluence*d_mv;

if (dQ < 1.0) dQ = 1.0;

d_Total += (5.0 – dQ);

}

// Pool freezing degradations with averaging and square root operations

d_Total /= (double) i_processed_frames;

d_Total = sqrt(d_Total);

if (d_Total > 4.0) d_Total = 4.0;

if (d_Total < 0.0) d_Total = 0.0;

d_freezing_artifact_value = d_Total + 1.0;

}

### 6.4.4	Rebuffering module

The rebuffering degradation module has the following input and output parameters.

Input  = [d_rebuffering_ratio]

Output = [d_rebuffering_artifact_value]

#### 6.4.4.1	Input parameter description

d_rebuffering_ratio	See clause 6.3.4.2

#### 6.4.4.2	Output parameter description

d_rebuffering_artifact_value	Estimated buffering degradation score expressed as a double in the range [0.0, 1.0].

#### 6.4.4.3	Coefficients

d_coef_buf = 0.0737

#### 6.4.4.4	Algorithm

The estimated buffering degradation ratio is calculated according to the following formula:

d_rebuffering_artifact_value = 1 – pow (d_rebuffering_ratio, d_coef_buf)

### 6.4.5	Framework for combining modules

Input  = [d_compression_artifact_value,

d_slicing_artifact_value,

d_freezing_artifact_value,

d_rebuffering_artifact_value]

Output = [d_combined_quality_value]

#### 6.4.5.1	Input parameter description

d_compression_artifact_value	See clause 6.4.1.2

d_slicing_artifact_value		See clause 6.4.2.2

d_freezing_artifact_value		See clause 6.4.3.2

d_rebuffering_artifact_value	See clause 6.4.4.2

#### 6.4.5.2	Output parameter description

d_combined_quality_value	A float number between 1 and 5 indicating the level of video quality, considering all kinds of artefacts, i.e., estimated total video MOS.

#### 6.4.5.3	Coefficients

d_alpha	= [ 1.0833,    0.3138,   -1.7665]

d_beta2	= [ 3.3254,    5.8293]

d_beta3	= [36.0047, -175.4657]

d_beta4	= [ 2.1741]

#### 6.4.5.4	Algorithms

d_compression_artifact_value, d_slicing_artifact_value, d_freezing_artifact_value and d_rebuffering_artifact_value are provided by the above sub-clauses respectively, as the estimation of the level of coding artefact, slicing artefact, freezing artefact, and rebuffering artefact contained in the observed video. The compression artefact value corresponds to the ACR MOS scale.

The general procedure for combining the four degradation types into a predicted MOS is as follows. First, the estimated levels of distortion for different artefact types are aligned, that is, the estimated level of artefact is mapped to the same subjective artefact level, regardless of the artefact type. Second, identify the dominant visual artefacts by sorting the aligned values of d_compression_artifact_value, d_slicing_artifact_value, d_freezing_artifact_value and d_rebuffering_artifact_value in ascending order. Third, linearly combine the dominant visual artefacts into a predicted MOS.

Specifically, to make it possible to compare the different artefacts, all artefact values are aligned to the ACR MOS scale using specific alignment functions. Slicing is aligned using a negative exponential function, freezing is aligned using a linear function and rebuffering is aligned using a bounded log-function of the following form:

(13)

which outputs a number in the range [0,1] given x in the range [0,1] regardless of the value of the coefficient d. This output is scaled to the range [1, 5] according to formula 4·f(x)+1. The resulting aligned values of d_compression_artifact_value, d_slicing_artifact_value, d_freezing_artifact_value and d_rebuffering_artifact_value are sorted in ascending order, and the sorted results are stored in the array dpp[0...3]. Therefore, dpp[0] is the estimated level for the artefact with highest quality impact while dpp[3] is the estimated level for the artefact with lowest quality impact.

It is believed that human perception is not very sensitive to differences between distortion types (that is, no matter if it is a compression, slicing, freezing or rebuffering artefact). For example, suppose video V1 and V2 are affected by two kinds of distortions – compression artefacts and slicing artefacts. After alignment, denote compression artefacts by dp0, slicing artefacts by dp1. In the example the compression artefact in V1 is much more significant than the slicing artefacts (dp0(V1) > dp1(V1)). Contrarily, the slicing artefact in V2 is much more significant than the compression artefact (dp1(V2) > dp0(V2)). If dp0(V1)== dp1(V2) and dp1(V1)== dp0(V2), then according to the human visual system, human perceives nearly the same level of overall quality degradation for V1 and V2, no matter if the significant and secondary artefacts are of slicing artefact type or of compression artefact type.

Human perception will be more influenced by the most significant artefact with value dpp[0], then dpp[1]. Linear combination is applied to get the overall quality value from dpp[0] and dpp[1]:

d_combined_quality_value=α1×dpp[0]+ α2·×dpp[1]+ α3	(14)

wherein | α1| >| α2|.

The combined artefact level is then estimated according to the following pseudocode:

d[0] = d_compression_artifact_value

d[1] = d_slicing_artifact_value

d[2] = d_freezing_artifact_value

d[3] = d_rebuffering_artifact_value

// Align to MOS

dp[0] = d[0]

dp[1] = -exp(d[1]/d_beta2[0]) + d_beta2[1]

dp[2] = d_beta3[0]*d[2] + d_beta3[1]

dp[3] = 4*(1.0/d_beta4[0]*log10(1.0+d[3]*(pow(10,d_beta4[0])-1.0)))+1.0

dpp = sorting(dp)          // sorting in ascending order

// Combine the two most significant artifacts with a linear function

d_combined_quality_value = d_alpha[0]*dpp[0] + d_alpha[1]*dpp[1] + d_alpha[2]

// Make sure scores are between 1 and 5

d_combined_quality_value = min(max(d_combined_quality_value, 1.0), 5.0)

# 7	Assumptions

It is assumed that the user of this Recommendation knows how to extract an RTP packed video bitstream encapsulated in the libPcap format, as well as how to extract the video elementary stream from the RTP packed video bitstream, while keeping track of lost pictures.

It is further assumed that the user of this Recommendation has a good knowledge about the ITU-T H.264/AVC video codec defined in [ITU-T H.264], and that the user knows how to extract basic parameters from the video bitstream.

# 8	Model compliance

In order to verify compliance with the ITU-T P.1202.1 model a set of 12 test sequences with a set of corresponding test vectors are provided with this Recommendation. Test vectors are given at picture level, sequence level and model level.

The following test files have been provided with this Recommendation for verifying compliance with the ITU-T P.1202.1 model:

P1202_LR_TV01.pre

P1202_LR_TV02.pre

P1202_LR_TV03.pre

P1202_LR_TV04.pre

P1202_LR_TV05.pre

P1202_LR_TV06.pre

P1202_LR_TV07.pre

P1202_LR_TV08.pre

P1202_LR_TV09.pre

P1202_LR_TV11.pre

P1202_LR_TV01.pcap

P1202_LR_TV02.pcap

P1202_LR_TV03.pcap

P1202_LR_TV04.pcap

P1202_LR_TV05.pcap

P1202_LR_TV06.pcap

P1202_LR_TV07.pcap

P1202_LR_TV08.pcap

P1202_LR_TV09.pcap

P1202_LR_TV11.pcap

P1202_LR_TV06.buf

P1202_LR_TV09.buf

The .pcap files are ITU-T H.264/AVC video bitstream files packed in IP/UDP/RTP encapsulated in the libpcap dump format. The .pre and .buf contain side information that is used by the ITU-T P.1202.1 model. Each line in the .buf file corresponds to a rebuffering event where the first value indicates the start of the rebuffering (not used by the model) and the second value the duration in seconds of the rebuffering event.

## 8.1	Test vectors for extraction of basic parameters

Test vectors are provided for clauses 6.1.1.2, 6.1.2.2, 6.1.3.1 and 6.1.3.2. The test vectors for clauses 6.1.3.1 and 6.1.3.2 are given as excel files together with the test vector sequences.

Table 8-1 – Test vectors for extraction of basic parameters from side
information in clause 6.1.1.2

| pcap | TV01 | TV02 | TV03 | TV04 | TV05 | TV06 | TV07 | TV08 | TV09 | TV11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| d_avg_frame_ rate | 15 | 30 | 30 | 30 | 30 | 30 | 30 | 15 | 30 | 30 |
| d_rebuffering_ duration | 0 | 0 | 0 | 0 | 0 | 1.5 | 0 | 0 | 4 | 0 |
| s_video_PLC_ mode | N/A | SLICING | SLICING | FREEZING | SLICING | N/A | FREEZING | SLICING | SLICING | SLICING |

Table 8-2 – Test vectors for extraction of basic parameters from SPS in clause 6.1.2.2

| pcap | TV01 | TV02 | TV03 | TV04 | TV05 | TV06 | TV07 | TV08 | TV09 | TV11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| i_nbr_mb_ver | 9 | 9 | 9 | 9 | 15 | 15 | 15 | 15 | 20 | 20 |
| i_nbr_mbs | 99 | 99 | 99 | 99 | 300 | 300 | 300 | 300 | 600 | 600 |
| b_gaps_in_frame_num_value_allowed_flag | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

## 8.2	Test vectors for aggregation of basic parameters into internal picture level parameters

Test vectors are provided for clauses 6.2.1.1, 6.2.1.2, 6.2.2.X and 6.3.2.X in excel files together with the test vector sequences.

## 8.3	Test vectors for aggregation of parameters into model parameters

Test vectors are provided for clauses 6.3.X.1 and 6.3.X.2. Picture level test vectors can be found in excel files together with the test vector sequences.

Table 8-3 – Test vectors for clause 6.3.1.1

| pcap | TV01 | TV02 | TV03 | TV04 | TV05 | TV06 | TV07 | TV08 | TV09 | TV11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| d_avg_frame_rate | 15 | 30 | 30 | 30 | 30 | 30 | 30 | 15 | 30 | 30 |
| i_nbr_received_pics | 150 | 290 | 467 | 476 | 291 | 480 | 298 | 149 | 480 | 298 |
| i_nbr_intra_pics | 2 | 4 | 6 | 8 | 2 | 11 | 5 | 3 | 8 | 5 |
| i_nbr_inter_pics | 148 | 286 | 461 | 468 | 289 | 469 | 293 | 146 | 472 | 293 |
| d_sum_avg_pic_ qp | 4841.475 | 9282.894 | 16261.87 | 13353.11 | 10964.7 | 13952.78 | 9939.417 | 5207.17 | 13905.03 | 9153.836 |
| d_sum_avg_pic_ mv | 401.4853 | 537.6276 | 2766.81 | 1834.749 | 1662.7 | 2700.145 | 792.7342 | 1280.844 | 3302.594 | 965.5608 |

Table 8-4 – Test vectors for clause 6.3.1.2

| pcap | TV01 | TV02 | TV03 | TV04 | TV05 | TV06 | TV07 | TV08 | TV09 | TV11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| d_avg_avg_pic_ qp | 32.2765 | 32.00998 | 34.82199 | 28.05275 | 37.67937 | 29.0683 | 33.35375 | 34.94745 | 28.9688 | 30.71757 |
| d_avg_avg_pic_mv | 2.676568 | 1.853888 | 5.924646 | 3.854514 | 5.713747 | 5.625303 | 2.660182 | 8.596266 | 6.880403 | 3.240137 |
| d_key_frame_ rate | 0.202703 | 0.41958 | 0.390456 | 0.512821 | 0.207612 | 0.703625 | 0.511945 | 0.308219 | 0.508475 | 0.511945 |

Table 8-5 – Test vectors for clause 6.3.2.1

| pcap | TV01 | TV02 | TV03 | TV04 | TV05 | TV06 | TV07 | TV08 | TV09 | TV11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| i_processed_ frames | N/A | 300 | 480 | N/A | 300 | N/A | N/A | 150 | 480 | 300 |
| i_nbr_mbs | N/A | 99 | 99 | N/A | 300 | N/A | N/A | 300 | 600 | 600 |
| d_avg_frame_rate | N/A | 30 | 30 | N/A | 30 | N/A | N/A | 15 | 30 | 30 |

Table 8-6 – Test vectors for clause 6.3.2.2

| pcap | TV01 | TV02 | TV03 | TV04 | TV05 | TV06 | TV07 | TV08 | TV09 | TV11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| d_LoVA_seq | 0 | 4.15299 | 4.00047 | 0 | 4.608799 | 0 | 0 | 3.043126 | 3.435844 | 4.404755 |

Table 8-7 – Test vectors for clause 6.3.4.1

| pcap | TV01 | TV02 | TV03 | TV04 | TV05 | TV06 | TV07 | TV08 | TV09 | TV11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| d_rebuffering_ duration | 0 | 0 | 0 | 0 | 0 | 1.5 | 0 | 0 | 4 | 0 |
| i_nbr_received_pics | 150 | 290 | 467 | 476 | 291 | 480 | 298 | 149 | 480 | 298 |
| i_nbr_lost_pics | 0 | 10 | 11 | 4 | 9 | 0 | 2 | 1 | 0 | 2 |
| d_avg_frame_rate | 15 | 30 | 30 | 30 | 30 | 30 | 30 | 15 | 30 | 30 |

Table 8-8 – Test vectors for clause 6.3.4.2

| pcap | TV01 | TV02 | TV03 | TV04 | TV05 | TV06 | TV07 | TV08 | TV09 | TV11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| d_rebuffering_ratio | 0 | 0 | 0 | 0 | 0 | 0.085714 | 0 | 0 | 0.2 | 0 |

## 8.4	Test vectors for quality estimation model

Test vectors are provided for clauses 6.4.X.1 and 6.4.X.2.

Table 8-9 – Test vectors for clause 6.4.1.1

| pcap | TV01 | TV02 | TV03 | TV04 | TV05 | TV06 | TV07 | TV08 | TV09 | TV11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| i_nbr_mb_ver | 9 | 9 | 9 | 9 | 15 | 15 | 15 | 15 | 20 | 20 |
| d_avg_frame_ rate | 15 | 30 | 30 | 30 | 30 | 30 | 30 | 15 | 30 | 30 |
| d_avg_avg_pic_ qp | 32.2765 | 32.00998 | 34.82199 | 28.05275 | 37.67937 | 29.0683 | 33.35375 | 34.94745 | 28.9688 | 30.71757 |
| d_max_avg_pic_ qp | 47 | 40.74748 | 43.0101 | 43 | 44.50667 | 36.11333 | 39.62 | 50 | 39.83 | 40.18333 |
| d_min_avg_pic_ qp | 27 | 23.92929 | 25.07071 | 21.55556 | 33.26818 | 24.21667 | 24.15333 | 30.62667 | 22.63667 | 20.79167 |
| b_has_intra_pic_ flicker | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| i_max_nbr_ used_ref_pics | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| b_sub_mb_ type_inter8x4_ or_inter4x8_ exists | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| b_sub_mb_type_ inter4x4_exists | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| d_avg_avg_pic_ mv | 2.676568 | 1.853888 | 5.924646 | 3.854514 | 5.713747 | 5.625303 | 2.660182 | 8.596266 | 6.880403 | 3.240137 |
| d_key_frame_ rate | 0.202703 | 0.41958 | 0.390456 | 0.512821 | 0.207612 | 0.703625 | 0.511945 | 0.308219 | 0.508475 | 0.511945 |

Table 8-10 – Test vectors for clause 6.4.1.2

| pcap | TV01 | TV02 | TV03 | TV04 | TV05 | TV06 | TV07 | TV08 | TV09 | TV11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| d_compression_artifact_value | 3.441425 | 3.803958 | 3.323498 | 4.183827 | 3.061975 | 4.35574 | 3.811585 | 3.159155 | 4.278622 | 4.087967 |

Table 8-11 – Test vectors for clause 6.4.2.1

| pcap | TV01 | TV02 | TV03 | TV04 | TV05 | TV06 | TV07 | TV08 | TV09 | TV11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| d_LoVA_seq | 0 | 4.15299 | 4.00047 | 0 | 4.608799 | 0 | 0 | 3.043126 | 3.435844 | 4.404755 |
| d_sum_max_nbr_ partitions | 0 | 812 | 1100 | 284 | 968 | 0 | 384 | 28 | 182 | 908 |
| i_nbr_received_pics | 150 | 290 | 467 | 476 | 291 | 480 | 298 | 149 | 480 | 298 |

Table 8-12 – Test vectors for clause 6.4.2.2

| pcap | TV01 | TV02 | TV03 | TV04 | TV05 | TV06 | TV07 | TV08 | TV09 | TV11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| d_slicing_artifact_ value | 0 | 4.28711 | 4.113297 | 0.028579 | 4.768136 | 0 | 0.061723 | 3.052127 | 3.454006 | 4.550705 |

Table 8-13 – Test vectors for clause 6.4.3.1

| pcap | TV01 | TV02 | TV03 | TV04 | TV05 | TV06 | TV07 | TV08 | TV09 | TV11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| i_processed_ frames | N/A | N/A | N/A | 480 | N/A | N/A | 300 | N/A | N/A | N/A |
| d_avg_frame_rate | N/A | N/A | N/A | 30 | N/A | N/A | 30 | N/A | N/A | N/A |

Table 8-14 – Test vectors for clause 6.4.3.2

| pcap | TV01 | TV02 | TV03 | TV04 | TV05 | TV06 | TV07 | TV08 | TV09 | TV11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| d_freezing_artifact_ value | 5 | 5 | 5 | 4.961477 | 5 | 5 | 4.945679 | 5 | 5 | 5 |

Table 8-15 – Test vectors for clause 6.4.4.1

| pcap | TV01 | TV02 | TV03 | TV04 | TV05 | TV06 | TV07 | TV08 | TV09 | TV11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| d_rebuffering_ratio | 0 | 0 | 0 | 0 | 0 | 0.085714 | 0 | 0 | 0.2 | 0 |

Table 8-16 – Test vectors for clause 6.4.4.2

| pcap | TV01 | TV02 | TV03 | TV04 | TV05 | TV06 | TV07 | TV08 | TV09 | TV11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| d_rebuffering_artifact_value | 1 | 1 | 1 | 1 | 1 | 0.165616 | 1 | 1 | 0.111851 | 1 |

Table 8-17 – Test vectors for clause 6.4.5.1

| pcap | TV01 | TV02 | TV03 | TV04 | TV05 | TV06 | TV07 | TV08 | TV09 | TV11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| d_compression_artifact_value | 3.441425 | 3.803958 | 3.323498 | 4.183827 | 3.061975 | 4.35574 | 3.811585 | 3.159155 | 4.278622 | 4.087967 |
| d_slicing_artifact_value | 0 | 4.28711 | 4.113297 | 0.028579 | 4.768136 | 0 | 0.061723 | 3.052127 | 3.454006 | 4.550705 |
| d_freezing_artifact_value | 5 | 5 | 5 | 4.961477 | 5 | 5 | 4.945679 | 5 | 5 | 5 |
| d_rebuffering_artifact_value | 1 | 1 | 1 | 1 | 1 | 0.165616 | 1 | 1 | 0.111851 | 1 |

Table 8-18 – Test vectors for clause 6.4.5.2

| pcap | TV01 | TV02 | TV03 | TV04 | TV05 | TV06 | TV07 | TV08 | TV09 | TV11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| d_combined_quality_value | 3.391834 | 1.809807 | 1.859292 | 2.981295 | 1 | 3.489147 | 2.248317 | 2.699345 | 2.520283 | 1.574543 |

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | Telecommunication management, including TMN and network maintenance |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Terminals and subjective and objective assessment methods |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks, open system communications and security |
| Series Y | Global information infrastructure, Internet protocol aspects and next-generation networks |
| Series Z | Languages and general software aspects for telecommunication systems |

____________________

This Recommendation includes an electronic attachment with test vectors for testing compliance.