ITU Telecommunication Standardization Sector				Document APC-1213

Study Group 16

Q.12-14/16 Rapporteur Meeting						June 5 1997

Hertzliya, 10-13 June 1997

SOURCE:	Smith Micro Software, Inc.

TITLE    :	Allow all 16 DTMF characters in H.245 basicString

PURPOSE:	Proposal

_________________________

This contribution proposes that the H.245 UserInputIndication basicString choice indicate support for all 16 DTMF characters.

1. Introduction

There are sixteen DTMF characters, not just the twelve listed in 7.13.6/H.245. This leaves out the characters, A, B, C, and D.

# 2. Background

The touch tone system uses pairs of tones to represent the various keys. There is a "low tone" and a "high tone" associated with each button (0 through 9, plus * (star) and # (octothorpe or pound symbol). The low tones vary according to what horizontal row the tone button is in, while the high tones correspond to the vertical column of the tone button.

The tones and assignments are as follows:

1	2	3	A	:    697 Hz

4	5	6	B	:    770 Hz

(low tones)

7	8	9	C	:    852 Hz

*	0	#	D	:    941 Hz

---- 	---- 	---- 	----

1209	1336	1477	1633 Hz

(high tones)

Most phones simply do not have the fourth column of buttons.

# 3. Proposal

That the sentence in 7.13.6/H.245,

“The boolean basicString, when true, indicates that the characters 0-9, * and # are supported.”

be changed to

“The boolean basicString, when true, indicates that the characters 0-9, *, #, and A-D are supported.”

in H.245 version 3.

[end]

Contact:Paul Long	Tel: +1 503 641 1221 x12

Smith Micro Software, Inc.	Fax: +1 503 641 3344

15050 SW Koll Parkway	Email: plong@smithmicro.com

Suite 2B

Beaverton, OR 97006

USA

Explanation provided by Fritz Whittington.