ITU Telecommunication Standardization Sector

Study Group 16

Q.12-14/16 Rapporteur Meeting						June 9 1997

Hertzliya, 10-13 June 1997

SOURCE:	Smith Micro Software, Inc.

TITLE    :	A Response to APC-1234, “Support for DTMF Signaling in H.323 Systems”

PURPOSE:	Discussion

_________________________

This contribution is a response to APC-1234, “Support for DTMF Signaling in H.323 Systems.” It points out some problems with the proposed out-of-band signaling and presents an alternative, or rather a modification, to it.

1. Introduction

The author of APC-1234 has done a thorough job in describing the issues surrounding DTMF signaling. However, there are some problems.

1.1 Semantics of alphanumeric

The following proposals in the section, Changes to H.245 Version 3, are unacceptable because they would modify the current semantics of alphanumeric, which were established back in H.245 version 1. The suggestion in the section, Extended DTMF Support in H.323 Version 1, for “encoding a subset of the parameters directly in the alphanumeric stream” is similarly unacceptable for the same reason.

The allowable contents of the alphanumeric element of UserInputIndication should be clarified and extended. We recommend the following characters be permitted, with the associated meanings:

- 0,1,2,3,4,5,6,7,8,9,#,*,A,B,C,D (DTMF tones as defined in Q.23)

- “,” (comma; inserts a two-second pause)

- “!” (exclamation point; causes a hook-flash)

- “W” (wait for dial tone before continuing, with an appropriate timeout)

- “@” (wait for ringback followed by silence before continuing, with an appropriate timeout)

- Other characters should be defined as being ignored in the receiver, to allow for user flexibility in formatting of digit strings (inclusion of spaces, hyphens, etc.)

The semantics of the characters carried in alphanumeric are not defined by H.245. A vendor has therefore been free to interpret them as it sees fit. For example, some vendors of systems based on H.245 have been using UserInputIndication to transport general, non-DTMF user input, including the transport of text for a base-level chat feature and to control a video server. If adopted, the proposed meanings, including ignore “other characters,” would break these implementations. Text could, however, be added to H.245 that recommended specific character semantics. It is presumed that an endpoint looking for DTMF characters would want to use these recommendations, but there is no guarantee that it will or that the transmitter is sending them with this intent. Therefore, a transmitter should not assume any particular behavior in the receiver based on the alphanumeric choice of UserInputIndication. The transmitter can only assume that, if sent, the receiver received the characters, 0-9, *, and #.

1.2 DCE-to-DCE commands versus DTE-to-DCE commands

In addition to DTMF characters, several single-character commands are proposed for the contents of alphanumeric (see above) and digits. There is a dichotomy, though, of DCE-to-DCE commands on the one hand and DTE-to-DCE commands on the other.

The single DCE-to-DCE command, “!”, or “hook-flash,” is appropriate because it is something that is signaled from one endpoint to another; however, all of the proposed DTE-to-DCE commands, i.e., “,”, “W”, “@”, and “$”, are inappropriate in UserInputIndication because they are inherently local to the endpoint and make no sense to transmit to another endpoint. (If the remote endpoint is an H.323/GSTN gateway, one could arguably consider the gateway a DCE, in which case transmission of the DTE-to-DCE commands might make sense. However, since this is not mentioned, we assume these commands are intended to be sent to any endpoint.) For example, to pause for two seconds, an endpoint does just that—it does not transmit UserInputIndication for two seconds. To wait for a particular event in the audio signal such as dial-tone, the endpoint does the waiting. It does not tell the other endpoint that it is waiting or for the other endpoint to wait. An endpoint could, however, use characters internally

- Multilateral use of NonStandardParameter

Regarding section, Extended DTMF Support in H.323 Version 1, although using a NonStandardParameter may be the only way to retrofit version-1 H.323 systems with extended DTMF support, we are concerned about the precedent it would make, encouraging use of NonStandardParameter in various situations instead of waiting for the next standardization cycle. We would prefer keeping NonStandardParameter’s original intention, that of creating unilateral, not multilateral, extensions.

1.4 Using UserInputIndication to represent the presence of a tone

Instead of using UserInputIndication to represent that a DTMF tone has occured, why not use it like an audio frame, and indicate that a tone is occuring. That would prevent a tone from causing delay equal to its length.

Transmit a start-tone with optional duration, possibly followed by one or more continue-tones with duration, followed by an end-tone with optional duration. This approximates a continuous tone. If a message belonging to a key press in progress (i.e., continueTone or endTone) does not arrive within a certain amount of time, the receiver behaves as if an end-tone was received without a duration and then ignores all subsequent tone messages up to and including the end-tone. This prevents the receiver from misinterpreting a single tone as two or more, at the risk of misinterpreting it as being shorter than it really is. If no duration is specified between receipt of a startTone and then an endTone, the duration is “as received.”

The following is an example of ASN.1 syntax applied to the current version-3 H.245. We have no problem with it being integrated into the syntax proposed by APC-1234. We would also thow in a flash-hook indication while we are at it, rather than introduce a non-DTMF character. We also so no reason to represent multiple DTMF characters in a single UserInputIndication, although we see no reason not to, either.

Dtmf			::=IA5String SIZE(1) FROM("0123456789*#ABCD")

UserInputIndication	::=CHOICE

{

nonStandard		NonStandardParameter,

alphanumeric	GeneralString,

...,

userInputSupportIndication	CHOICE

{

nonStandard	NonStandardParameter,

basicString	NULL,

iA5String	NULL,

generalString	NULL,

...

},

telephonySignaling	CHOICE

{

startTone

{

dtmf	Dtmf,

duration	INTEGER (1..65535) OPTIONAL,

...

},

continueTone

{

duration	INTEGER (1..65535),

...

},

endTone

{

duration	INTEGER (1..65535) OPTIONAL,

...

},

flashHook	NULL,

...

}

}

[end]

Contact:Paul Long	Tel: +1 503 641 1221 x12

Smith Micro Software, Inc.	Fax: +1 503 641 3344

15050 SW Koll Parkway	Email: plong@smithmicro.com

Suite 2B

Beaverton, OR 97006

USA