ITU – Telecommunications Standardization Sector	Document TD-28

STUDY GROUP 16 Contribution

Experts Group for Video Coding and Systems in		June 10-13, 1997
ATM and Other Network Environments			Hertzliya Beach

Title:		Outline of a Profile/Level roster for H.263+ modes in H.323 systems

Source:	 	TELES AG / Technical University Berlin

Purpose: 	Proposal

Contact:	Stephan Wenger			Voice:	+49 30 31473160

Secr. FR 6-3				Fax:	+49 30 31425156

TU Berlin				Email:	stewe@cs.tu-berlin.de

Franklinstr. 28-29

D-10587 Berlin, Germany

TD-12 proposed the Liaison statement to Q15/16 (Video) to give us advise on the optional mode of H.263+. This document outlines the „subprofiles“ of the H.323 profile with their specific demands. Our idea is to leave it up to the Video group to specify both the number and the characteristics of each „subprofile“ based on their knowledge and experience of H.263+.

In the following table, bitrate means the network speed including audio and data traffic. Very low Bitrate means modem speeds below 64 kbit/s. Low bitrate is defined as single BRI ISDN speeds up to 128 kbit/s. High bitrate means everything above 128 kbit/s. Low packet loss rates are defined here as that the packet loss problem can be ignored by the Codec algorithm. High packet loss rates means, that the codec has to provide support for packet loss situations.

The following „subprofiles“ within the H.323 „main“ profile are identified by us:

- Point-to-Point, very low bitrate, low packet loss rates: two H.323 terminals connected to each other by direct modem / PPP connections.

- Point-to-Point, very low bitrate, high packet loss rates: two H.323 terminals connected via the Internet using modems for dial up

- Point-to-Point, low bitrate, low packet loss rates: Similar as (1) using ISDN

- Point-to-Point, low bitrate, high packet loss rates: Similar as (2) using ISDN

- Point-to-Point, high bitrate, low packet loss rates: two H.323 terminals connected via a guarantied QoS LAN, ATM or similar

- Point-to-Point, high bitrate, high packet loss rates: two H.323 terminals connected via non guarantied QoS LAN or the Internet

- Multipoint low bitrate, high packet loss rate: more than two H.323 terminals on a LAN/Internet at ISDN speeds

- Multipoint high bitrate, low packet loss rate: more than two H.323 terminals on a guarantied QoS LAN, ATM or similar

- Multipoint, high bitrate, high packet loss rate: more than two H.323 terminals connected on a non guarantied QoS LAN