ITU Telecommunication Standardization Sector  	                               Document TD-30

Study Group 16                                                                                                       Proposal

Q.12-14/16 Rapporteur Meeting			                       	       	  10 June 1997

Herzeliya 10-13 June1997

SOURCE:	Scott Petrack, VocalTec Communications Ltd.

Corrections to Unregistration Messages

- Need for UnregistrationRequestReason

An Unregistration Request is usually thought of as passing from an endpoint to a gatekeeper, in which case the gatekeeper can confirm (RCF) or reject (RRJ). If the gatekeeper rejects the unregistration (sends an RRJ), it includes a rejectReason. But an Unregistration Request also can pass from a gatekeeper to an Endpoint, to allow “a Gatekeeper … [to] request a terminal to consider itself unregistered.” Unfortunately there is no rejectReason field in the URQ message, so when an endpoint hears from the gatekeeper that it is no longer registered it doesn’t really know why.

We propose adding to the URQ message an UnregRequestReason field. This will be used when the URQ is sent from a gatekeeper to an endpoint to indicate why the Gatekeeper considers the endpoint unregistered.

UnregRequestReason ::= CHOICE

{

ReregistyRequired	NULL,	-- gatekeeper requires a registration

ttlExpired		NULL,	-- your time is up...

securityDenial		NULL,

undefinedReason	NULL,

...,

}

- Need to require UCF from Endpoint to Gatekeeper

When the URQ message comes from the gatekeeper to the endpoint, it is not really a request but rather an announcement. The endpoint should not be able to return an Unregistration Reject to a “request” from a gatekeeper. Further, since RAS is sent on an unreliable channel, it is vital that a UCF be returned as an acknowledgment from the endpoint that it considers itself unregistered. Otherwise, if the URQ from the gatekeeper is lost, there could be a lack of synchronization between the gatekeeper and the endpoint.

We propose that the following short text be added to section 8.10.1.

“When a URQ is sent from a gatekeeper to an Endpoint, the endpoint shall send a UCF reply to confirm receipt and to acknowledge that it knows it is no longer registered.”