|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	H.221

TELECOMMUNICATION	(Draft revision, 02/95)
STANDARDIZATION  SECTOR	Decided by SG 15 in February 1995
OF  ITU	Further edited as shown

LINE  TRANSMISSION  OF  NON-TELEPHONE
SIGNALS

FRAME  STRUCTURE  FOR  
A  64  TO  1920  kbit/s  CHANNEL
IN  AUDIOVISUAL  TELESERVICES

ITU-T  Recommendation  H.221 (draft revision)

(Previously “CCITT  Recommendation”)

# FOREWORD

The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the International Telecommunication Union. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, established the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

ITU-T Recommendation H.221 was revised by the ITU-T Study Group XV (1988-1993) and was approved by the WTSC (Helsinki, March 1-12, 1993). This document is a revision decided upon by SG15 in February 1995.

___________________

NOTES

1	As a consequence of a reform process within the International Telecommunication Union (ITU), the CCITT ceased to exist as of 28 February 1993. In its place, the ITU Telecommunication Standardization Sector (ITU-T) was created as of 1 March 1993. Similarly, in this reform process, the CCIR and the IFRB have been replaced by the Radiocommunication Sector.

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

©  ITU  1995

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Recommendation H.221     (Draft revision 02/95)

Page

µFOREWORD	2

1 Basic principle	1

1.1 Frame alignment signal (FAS)	1

1.2 Bit-rate allocation signal (BAS)	2

1.3 Encryption control signal (ECS)	2

1.4 Remaining capacity	2

2 Frame alignment	3

2.1 General	3

2.2 Multiframe structure	4

2.3 Loss and recovery of frame alignment	5

2.4 Loss and recovery of multiframe alignment	6

2.5 Procedure to recover octet timing from frame alignment	6

2.6 Description of the CRC4 procedure	7

2.7 Synchronization of multiple connections	8

3 Bit-rate allocation signal	9

3.1 Encoding of the BAS	9

3.2 Values of the BAS	10

3.3 Procedures for the use of BAS	15

A.1 Audio command values (000)	18

A.2 Transfer-rate command values (001)	18

A.3 Video, encryption, loop and other commands (010)	19

A.4 LSD/MLP commands (011)	20

A.5 Audio capabilities (100)	21

A.6 Video, MBE and encryption capabilities (101)	22

A.7 Transfer-rate capabilities (100)	22

A.8 LSD/MLP capabilities (101) and other (110)	23

A.9 Escape table values (111)	23

A.10 HSD/H-MLP/MLP capabilities (111) [10000]-(101)	23

A.11 HSD/H-MLP commands (111) [10000]-(011)	27

A.12 Au-ISO commands (111) [10000]-(001)	27

A.13 Au-ISO capabilities (111) [10000]-(100)	29

A.14 Applications within LSD/HSD channels – capabilities (111) [10010]-(101)	29

A.15 Applications within LSD/HSD/MLP/H-MLP channels – commands (111) [10010]-(011)	30

A.16 Transfer-rate capabilities and commands used in Channel Aggregation - Table A.5	30

B.1 Sub-channel arrangement	33

B.2 Operation of the 64 kbit/s terminal	33

B.3 Restriction against some communication modes	33

B.4 Audio command codes (000)	33

SUMMARY

The purpose of this Recommendation is to define a frame structure for audiovisual teleservices in single or multiple B or H0 channels or a single H11 or H12 channel which makes the best use of the characteristics and properties of the audio and video encoding algorithms, of the transmission frame structure and of the existing Recommendations. It offers several advantages:

–	It takes into account Recommendations such as G.704, X.30/I.461, etc. It may allow the use of existing hardware or software.

–	It is simple, economic and flexible. It may be implemented on a simple microprocessor using well-known hardware principles.

–	It is a synchronous procedure. The exact time of a configuration change is the same in the transmitter and the receiver. Configurations can be changed at 20 ms intervals.

–	It needs no return link for audiovisual signal transmission, since a configuration is signalled by repeatedly transmitted codewords.

–	It is very secure in case of transmission errors, since the code controlling the multiplex is protected by a double-error correcting code.

–	It allows the synchronization of multiple 64 kbit/s or 384 kbit/s connections and the control of the multiplexing of audio, video, data and other signals within the synchronized multiconnection structure in the case of multimedia services such as videoconference.

–	It can be used to derive octet synchronization in networks where this is not provided by other means.

–	It can be used in multipoint configurations, where no dialogue is needed to negotiate the use of a data channel.

–	It provides a variety of data bit-rates (from 300 bit/s up to almost 2 Mbit/s) to the user.

Recommendation H.221

Recommendation H.221     (Draft revision 02/95)

FRAME  STRUCTURE  FOR  A  64  TO  1920  kbit/s  CHANNEL
IN  AUDIOVISUAL  TELESERVICESxe "FRAME  STRUCTURE  FOR  A  64  TO  1920  kbit/s  CHANNEL
IN  AUDIOVISUAL  TELESERVICES"§1)

(revised 1990 and at Helsinki, 1993 and subsequently...)

# 1	Basic principle

This Recommendation provides for dynamically subdividing an overall transmission channel of 64 to 1920 kbit/s into lower rates suitable for audio, video, data and telematics purposes. The overall transmission channel is derived by synchronizing and ordering transmissions over from 1 to 6 B-connections, from 1 to 5 H0-connections, or an H11 or H12 connection. The first connection established is the initial connection and carries the initial channel in each direction. The additional connections carry additional channels.

The total rate of transmitted information is called the “transfer rate”; it is possible to fix the transfer rate less than the capacity of the overall transmission channel (values listed in Annex A).

A single 64 kbit/s channel is structured into octets transmitted at 8 kHz. Each bit position of the octets may be regarded as a sub-channel of 8 kbit/s (see Figure 1). The eighth sub-channel is called the service channel (SC), consisting of several parts as described in 1.1 to 1.4.

An H0, H11 or H12 channel may be regarded as consisting of a number of 64 kbit/s time-slots (TS) (see Figure 2). The lowest numbered time-slot is structured exactly as described for a single 64 kbit/s channel, while the other TS have no such structure. In the case of multiple B or H0 channels, all channels have a frame structure; that in the initial channel controls most functions across the overall transmission, while the frame structure in the additional channels is used for synchronization, channel numbering and related controls.

The term “I-channel” is applied to the initial or only B channel, to TS1 of initial or only H0 channel, and to TS1 of H11, H12 channels.

## 1.1	Frame alignment signal (FAS)

This signal structures the I-channel and other framed 64 kbit/s channels into frames of 80 octets each and multiframes (MF) of 16 frames each. Each multiframe is divided into eight 2-frame sub-multiframes (SMF). The term “frame alignment signal” (FAS) refers to bits 1–8 of the SC in each frame. In addition to framing and multiframing information, control and alarm information may be inserted in the FAS, as well as error check information to control end-to-end error performance and to check frame alignment validity. Other time-slots are aligned to the first.

The bits are transmitted to line in order, bit 1 first.

When an 8 kHz network clock is provided, FAS is transmitted and received in the least significant bit of the octet within each 125 microsecond, e.g. in an ISDN basic or primary rate interface. It should be noted that, where interworking between the audiovisual terminal and the telephone is required, transmission using the network timing is essential. In the receiver side, FAS should be sought in all bit positions. If received FAS position conflicts with the network octet timing, the FAS position is given priority. This may happen when the receiver utilizes network octet timing while the transmitter does not as in a terminal using codecs separate with ISDN terminal adaptor, or when interworking between 64 kbit/s and 56 kbit/s terminals takes place.

The FAS can be used to derive receive octet timing when it is not provided by the network. However, in the latter case, the terminal cannot transmit FAS with correct alignment into the octet timed part of the network and cannot intercommunicate with terminals which rely only on network timing for octet alignment.

FAS	Frame alignment signal

| Bit number |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 (SC) |  |  |
|  |  |  |  |  |  |  |  | 1 | Octet number |
| S | S | S | S | S | S | S | FAS | : |  |
| u | u | u | u | u | u | u |  | 8 |  |
| b | b | b | b | b | b | b |  | 9 |  |
| - | - | - | - | - | - | - | BAS | : |  |
| c | c | c | c | c | c | c |  | 16 |  |
| h | h | h | h | h | h | h |  | 17 |  |
| a | a | a | a | a | a | a | ECS | : |  |
| n | n | n | n | n | n | n |  | 24 |  |
| n | n | n | n | n | n | n |  | 25 |  |
| e | e | e | e | e | e | e |  | · |  |
| l | l | l | l | l | l | l |  | · |  |
| # | # | # | # | # | # | # | # | · |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 80 |  |

BAS	Bit-rate allocation signal

ECS	Encryption control signal

FIGURE  1/H.221

Frame structure of a single 64 kbit/s (B-channel

## 1.2	Bit-rate allocation signal (BAS)

Bits 9–16 of the SC in each frame are referred to as BAS. This signal allows the transmission of codewords to describe the capability of a terminal to structure the capacity of the channel or synchronized multiple channels in various ways, and to command a receiver to demultiplex and make use of the constituent signals in such structures. This signal is also used for controls and indications.

NOTE – For some countries having 56 kbit/s channels, the net available bit rates will be 8 kbit/s less. Interworking between a 64 kbit/s terminal and a 56 kbit/s terminal is established according to the frame structure in Annex B.

## 1.3	Encryption control signal (ECS)

Encryption capability requires a dedicated transmission channel. This is provided when required by allocating the bits 17–24 of the service channel. This reduces variable data and video transmission rates herein by 800 bit/s. The 800 bit/s is referred to as the ECS channel.

## 1.4	Remaining capacity

The remaining capacity (including the rest of the service channel), carried in bits 1-8 of each octet in the case of a single 64 kbit/s connection, may convey a variety of signals within the framework of a multimedia service, under the control of the BAS. Some examples follow:

–	voice encoded at 56 kbit/s using a truncated form of PCM of Recommendation G.711 (A-law or µ-law);

–	voice encoded at 16 kbit/s and video at 46.4 kbit/s;

–	voice encoded at 56 kbit/s with a bandwidth 50 to 7000 Hz (subband ADPCM according to Recommendation G.722); the coding algorithm is also able to work at 48 kbit/s – data can then be dynamically inserted at up to 14.4 kbit/s;

–	still pictures coded at 56 kbit/s;

–	data at 56 kbit/s inside an audiovisual session (e.g. file transfer for communicating between personal computers).

| <--- | 125 microseconds |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ---> |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | · · · |  | · · · |  | 6n–2 |  | 6n–1 |  | 6n |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | H0 n = 1 |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | H11 n = 4 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | H12 n = 5 |  |  |
|  | Audio + service channel |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | 1 |  | Octet number |  |  |  |  |  |
|  | S | S | S | S | S | S | S | FAS | : |  |  |  |  |  |  |  |
|  | u | u | u | u | u | u | u |  | 8 |  |  |  |  |  |  |  |
|  | b | b | b | b | b | b | b |  | 9 |  |  |  |  |  |  |  |
|  | - | - | - | - | - | - | - | BAS | : |  |  |  |  |  |  |  |
|  | c | c | c | c | c | c | c |  | 16 |  |  |  |  |  |  |  |
|  | h | h | h | h | h | h | h | S | 17 |  |  |  |  |  |  |  |
|  | a | a | a | a | a | a | a | u |  |  |  |  |  |  |  |  |
|  | n | n | n | n | n | n | n | b |  |  |  |  |  |  |  |  |
|  | n | n | n | n | n | n | n | - |  |  |  |  |  |  |  |  |
|  | e | e | e | e | e | e | e | c | · |  |  |  |  |  |  |  |
|  | l | l | l | l | l | l | l | h |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | a | · |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | n |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | n | · |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | e |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | l |  |  |  |  |  |  |  |  |
|  | # | # | # | # | # | # | # | # |  |  |  |  |  |  |  |  |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 80 |  |  |  |  |  |  |  |

FIGURE  2/H.221

Frame structure of higher-rate single channels
(H0, H11, H12 channels)

# 2	Frame alignment

## 2.1	General

An 80-octet frame length produces an 80-bit word in the service channel. These 80 bits are numbered 1–80. Bits 1–8 of the service channel in every frame constitute the FAS (see Figure 3), whose content is as follows:

–	multiframe structure (see 2.2);

–	Frame alignment word (FAW);

–	A-bit;

–	E- and C-bits (see 2.6).

The FAW consists of “0011011” in bits 2–8 of the FAS in even frames, complemented by an “1” in bit 2 of the succeeding odd frame.

The “A-bit” of the I-channel is set to zero whenever the receiver is in multiframe alignment, and is set to “1” otherwise (see 2.3); for additional channels, see 2.7.1.

FIGURE  3/H.221

|  | Bit number |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Successive frames | 1 | 2 |  |  | 3 | 4 | 5 | 6 | 7 | 8 |  |
| Even frames |  |  | 0 |  | 0 | 1 | 1 | 0 | 1 | 1 |  |
|  | (Note 1) |  |  | Frame alignment word (Note 2) |  |  |  |  |  |  |  |
| Odd frames |  |  | 1 |  | A | E | C1 | C2 | C3 | C4 |  |
|  | (Note 1) | (Note 2) |  |  | (Note 3) | (Note 4) |  |  |  |  |  |
| NOTES 1 See 2.2 and Figure 4. 2 The first seven bits of the frame alignment word are in the even frames. The eighth bit of the FAW in the odd frame is the complement of the first FAW bit in order to avoid simulation of FAW by a frame-repetitive pattern. 3 A-bit: loss of multiframe alignment indication (0 = alignment; 1 = loss). 4 The use of bits E and C1-C4 is described in 2.6 (0 = no error or cyclic redundancy check (CRC) not in use; 1 = error). |  |  |  |  |  |  |  |  |  |  |  |

Assignment of bits 1-8 of the service channel in each frame

## 2.2	Multiframe structure

See Figure 4.

Each multiframe contains 16 consecutive frames numbered 0 to 15 divided into eight sub-multiframes of two frames each (see Figure 4). The multiframe alignment signal is located in bit 1 of frames 1-3-5-7-9-11 and has the form 001011. Bit 1 of frame 15 remains reserved for future use. The value is fixed at 0.

Bit 1 of frames 0-2-4-6 may be used for a modulo 16 counter to number multiframes in descending order. The least significant bit is transmitted in frame 0, and the most significant bit in frame 6. The receiver uses the multiframe numbering to equalize out the differential delay of separate connections, and to synchronize the received signals.

The multiframe numbering is mandatory both in initial and additional channels for multiple B or multiple H0 communications, but it may or may not be inserted for single B or single H0 or H11/H12 for other communications where synchronization between multiple channels is not required.

Bit 1 of frame 8 is set to 1 when multiframes are numbered and is set to 0 when they are not.

Bit 1 of frames 10-12-13 shall be used to number each channel in a multiconnection structure so that the distant receiver can place the octets received in each 125 microseconds in the correct order.

Information bits in the multiframe should be validated by, for example, being received consistently for three multiframes.

1-L3		Channel number, least significant bit in L1

|  | Sub-multiframe | Frame | Bits 1 to 8 of the service channel in every frame |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (SMF) |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
|  |  | 0 | N1 | 0 | 0 | 1 | 1 | 0 | 1 | 1 |
|  | SMF1 | 1 | 0 | 1 | A | E | C1 | C2 | C3 | C4 |
|  |  | 2 | N2 | 0 | 0 | 1 | 1 | 0 | 1 | 1 |
|  | SMF2 | 3 | 0 | 1 | A | E | C1 | C2 | C3 | C4 |
|  |  | 4 | N3 | 0 | 0 | 1 | 1 | 0 | 1 | 1 |
|  | SMF3 | 5 | 1 | 1 | A | E | C1 | C2 | C3 | C4 |
|  |  | 6 | N4 | 0 | 0 | 1 | 1 | 0 | 1 | 1 |
| Multitrame | SMF4 | 7 | 0 | 1 | A | E | C1 | C2 | C3 | C4 |
|  |  | 8 | N5 | 0 | 0 | 1 | 1 | 0 | 1 | 1 |
|  | SMF5 | 9 | 1 | 1 | A | E | C1 | C2 | C3 | C4 |
|  |  | 10 | L1 | 0 | 0 | 1 | 1 | 0 | 1 | 1 |
|  | SMF6 | 11 | 1 | 1 | A | E | C1 | C2 | C3 | C4 |
|  |  | 12 | L2 | 0 | 0 | 1 | 1 | 0 | 1 | 1 |
|  | SMF7 | 13 | L3 | 1 | A | E | C1 | C2 | C3 | C4 |
|  |  | 14 | TEA | 0 | 0 | 1 | 1 | 0 | 1 | 1 |
|  | SMF8 | 15 | R | 1 | A | E | C1 | C2 | C3 | C4 |

|  | Channel | L3 | L2 | L1 |
| --- | --- | --- | --- | --- |
|  | Initial | 0 | 0 | 1 |
|  | Second | 0 | 1 | 0 |
|  | Third | 0 | 1 | 1 |
|  | · · · | · · | · · | · · |
|  | Sixth | 1 | 1 | 0 |
|  | Seventh and higher-numbered | 1 | 1 | 1 |

R		Reserved for future use set to 0.

A, E, C1-C4	As in Figure 3.

N1-N4		Used for multiframe numbering as described in 2.2; set to 0 while numbering is inactive.

|  |  | N4 | N3 | N2 | N1 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Multiframe number | 0 | 0 | 0 | 0 | 0 | (or numbering inactive) |
|  | 1 | 0 | 0 | 0 | 1 |  |
|  | 2 | 0 | 0 | 1 | 0 |  |
|  | · · | · · | · · | · · | · · |  |
|  | 15 | 1 | 1 | 1 | 1 |  |

N5		Indicates whether multiframe numbering is active (N5 = 1) or inactive (N5 = 0).

TEA		The terminal equipment alarm is set to 1 in the outgoing signal while an internal terminal equipment fault exists such that it cannot receive and act on the incoming signal. Otherwise it is set to 0.

FIGURE  4/H.221

Assignment of bits 1-8 of the service channel in each frame in a multiframe

## 2.3	Loss and recovery of frame alignmentxe "Loss and recovery of frame alignment"§

Frame alignment is defined to have been lost when three consecutive frame alignment words have been received with an error.

Frame alignment is defined to have been recovered when the following sequence is detected:

–	for the first time, the presence of the correct first seven bits of the frame alignment word;

–	the eighth bit of the frame alignment word in the following frame is detected by verifying that bit 2 is a 1;

–	for the second time, the presence of the correct first seven bits of the frame alignment word in the next frame.

If frame alignment is achieved but multiframe alignment cannot be achieved, then frame alignment shall be sought at another position.

When the frame alignment is lost, A-bit of the next odd frame is set to 1 in the transmit direction.

## 2.4	Loss and recovery of multiframe alignmentxe "Loss and recovery of multiframe alignment"§

Multiframe alignment is needed to number and synchronize two or more channels, and possibly also for encryption. Terminals such as those having only single-channel capabilities which have no use for the multiframe structure shall transmit the multiframe structure, but need not check for multiframe alignment on the incoming signal: they may transmit outgoing A = 0 when frame alignment is recovered.

NOTE – Such a terminal cannot transmit TEA (see Figure 4).

After multiframe alignment has been validated the other functions represented by bit 1 of the service channel can be used. When multiframe alignment of the distant terminal has been signalled (A = 0 received) the distant terminal is expected to have validated BAS codes and to be able to interpret BAS codes.

Multiframe alignment is defined to have been lost when three consecutive multiframe alignment signals have been received with an error. It is defined to have been recovered when the multiframe alignment signal has been received with no error in the next multiframe. When multiframe alignment is lost, even when an unframed mode is received, the A-bit of the next odd frame is set to 1 in the transmit direction. It is reset to 0 when multiframe alignment is regained. It is reset in additional channels when multiframe alignment and synchronism with the initial channel is regained.

## 2.5	Procedure to recover octet timing from frame alignmentxe "Procedure to recover octet timing from frame alignment"§

When the network does not provide octet timing, the terminal may recover octet timing in the receive direction from bit timing and from the frame alignment. The octet timing in the transmit direction may be derived from the network bit timing and an internal octet timing.

### 2.5.1	General rule

The receive octet timing is normally determined from the FAS position. But at the start of the call and before the frame alignment is gained, the receive octet timing may be taken to be the same as the internal transmit octet timing. As soon as a first frame alignment is gained, the receive octet timing is initialized at the new bit position, but it is not yet validated. It will be validated only when frame alignment is not lost during the next 16 frames.

### 2.5.2	Particular cases

a)	When, at the initiation of a call, the terminal is in a forced reception mode, or when the frame alignment has not yet been gained, the terminal may temporarily use the transmit octet timing.

b)	When frame alignment is lost after being gained, the receive octet timing shall not change until frame alignment is recovered.

c)	As soon as frame and multiframe alignment have been gained once, the octet timing is considered as valid for the rest of the call, unless frame alignment is lost and a new frame alignment is gained at another bit position.

d)	When the terminal switches from a framed mode to an unframed mode (by means of the BAS), the octet timing previously gained shall be kept.

e)	When a new frame alignment is gained on a new position, different from that previously validated, the receive octet timing is reinitialized to the new position but not yet validated and the previous bit position is stored. If no loss of frame alignment occurs in the next 16 frames, the new position is validated, otherwise the stored old bit position is reutilized.

### 2.5.3	Search for frame alignment signalxe "Search for frame alignment signal"§ (FAS)

Two methods may be used: sequential or parallel. In the sequential method, each of the eight possible bit positions for the FAS is tried. When FAS is lost after being validated, the search shall resume starting from the previously validated bit position. In the parallel method, a sliding window, shifting one bit for each bit period, may be used. In that case, when frame alignment is lost, the search shall resume starting from the bit position next to the previously validated one.

## 2.6	Description of the CRC4 procedurexe "Description of the CRC4 procedure"§

In order to provide an end-to-end quality monitoring of the connection, a 4-bit cyclic redundancy check (CRC4) procedure may be used and the four bits C1, C2, C3 and C4 computed at the source location are inserted in bit positions 5 to 8 of the odd frames. In addition, bit 4 of the odd frames, the E-bit, is used to transmit an indication as to whether the most recent CRC block, received in the incoming direction, contained errors or not.

When the CRC4 procedure is not used, bit E shall be set to 0, and bits C1, C2, C3 and C4 shall be set to 1 by the transmitter. Provisionally, the receiver may disable reporting of CRC errors after receiving eight consecutive CRCs set to all 1s, and it may enable reporting of CRC errors after receiving two consecutive CRCs each containing an 0 bit.

### 2.6.1	Computation of the CRC4 bitsxe "Computation of the CRC4 bits"§

The CRC4 bits C1, C2, C3 and C4 are computed for each B/H0/H11/H12 channel2), for a block made of two frames: one even frame (containing the first seven bits of FAW) followed by one odd frame (containing the eighth bit of FAW). The CRC4 block size is then 160/960/3840/4800 octets for a B/H0/H11/H12 channel and 320/480/640/1280/1920/2880/3680 octets for a 128/192/256/512/768/1152/1472 kbit/s channel and the computation is performed 50 times per second.

NOTE – This is still valid for the case of H0/H11 or 128/192/256/320/512/768/1152/1472 kbit/s transfer rate in restricted networks, the stuffed bits being included in the computation. For restricted B, see Annex B.

#### 2.6.1.1	Multiplication-division process

A given C1-C4 word located in block N is the remainder after multiplication by x4 and then division (modulo 2) by the generator polynomial x4 + x + 1 of the polynomial representation of block (N – 1).

When representing contents of a block as a polynomial, the first bit in the block shall be taken as being the most significant bit. Similarly C1 is defined to be the most significant bit of the remainder and C4 the least significant bit of the remainder.

This process can be realized with a four-stage register and two exclusive-ORs.

#### 2.6.1.2	Encoding procedure

i)	The CRC bit positions in the odd frame are initially set at zero, i.e. C1 = C2 = C3 = C4 = 0.

ii)	The block is the acted upon by the multiplication-division process referred to in 2.6.1.1.

iii)	The remainder resulting from the multiplication-division process is stored ready for insertion into the respective CRC locations of the next odd frame.

NOTE – These CRC bits do not affect the computation of the CRC bits of the next block, since the corresponding locations are set at zero before the computation.

#### 2.6.1.3	Decoding procedure

i)	A received block is acted upon by the multiplication-division process, referred to in 2.6.1.1, after having its CRC bits extracted and replaced by zeros.

ii)	The remainder resulting from this multiplication-division process is then stored and subsequently compared on a bit-by-bit basis with the CRC bits received in the next block.

iii)	If the decoded calculated remainder exactly corresponds to the CRC bits sent from the encoder, it is assumed that the checked block is error-free.

### 2.6.2	Consequent actions

#### 2.6.2.1	Action on bit E

Bit E of block N is set to 1 in the transmitting direction if bits C1-C4 detected in the most recent block in the opposite direction have been found in error (at least one bit in error). In the opposite case it is set to zero.

#### 2.6.2.2	Monitoring for incorrect frame alignment (see Note)

In the case of a long simulation of the FAW, the CRC4 information can be used to re-invite a search for frame alignment. For such a purpose it is possible to count the number of CRC blocks in error within two seconds (100 blocks) and to compare this number with 89. If the number of CRC blocks in error is greater than or equal to 89, a search for frame alignment shall be reinitiated.

These values 100 and 89 have been chosen in order that:

–	For a random transmission error rate of 10-3, the probability of incorrectly reinitiating a search for frame alignment, because of 89 or more blocks in error, should be less than 10-4.

–	In case of simulation of frame alignment, the probability of not reinitiating a search of frame alignment after a two-second period should be less than 2.5%.

NOTE – Values in this and the next subclause exemplify the case of a 64 kbit/s channel. For H0, H11 or H12 channels the details will differ but the principles are still applicable.

#### 2.6.2.3	Monitoring for error performance

The quality of the 64 kbit/s connection can be monitored by counting the number of CRC blocks in error within a period of one second (50 blocks). For instance, a good evaluation of the proportion of seconds without errors as defined in Recommendation G.821 can be provided.

For information purposes, Table 1 gives the proportions of CRC block in error can be computed for randomly distributed errors of error rate Pe.

By counting the received E-bits, it is possible to monitor the quality of the connection in the opposite direction.

TABLE  1/H.221

| Pe | 10-3 | 10-4 | 10-5 | 10-6 | 10-7 |
| --- | --- | --- | --- | --- | --- |
| Percentage of CRC blocks in error | 70% | 12% | 1.2% | 0.12% | 0.012% |

## 2.7	Synchronization of multiple connectionsxe "Synchronization of multiple connections"§

Some audiovisual terminals will be able to communicate over multiple B or H0 connections (see Note). In this case, a single B or H0 initial connection is established, the possibility for more connections is determined from the transfer rate capability BAS of Annex A and the additional connections are then established and synchronized by the terminal using the multiframe structure.

NOTE – A connection is an individual call between the terminals. A channel is the transmission in one direction over the connection.

### 2.7.1	Multiple B-connectionsxe "Multiple B-connections"§

FAS and BAS are transmitted in each B-channel (see Note).

NOTE – The actual bit rates allowed by this Recommendation for these audio codings within a 64 kbit/s I-channel are 64 and 56 kbit/s, commands (000) [4/5 and 18/19], respectively. Thus in a 2B audiovisual call it is not permitted to transmit framed G.711 audio in the I-channel and video in the additional channel. The two channels shall be synchronised, the audio shall be set to 56 kbit/s, and when the video is ON it shall occupy the remaining 68.8 kbit/s.

FAS operation is as follows:

–	multiframe numbering is used to determine relative transmission delay between B-channels as described in 2.2;

–	the channel numbers are transmitted in the FAS, as described in 2.2, with the channel of the initial connection being numbered 1 and there being up to twenty-three additional connections;

–	the channel numbers of the additional channels are also transmitted in the BAS  according to Table A.54/H.221

–	the outgoing A-bit is set to 1 in the additional B-channel of the same connection whenever the received additional channel is not synchronized to the initial channel;

–	when receive synchronization is achieved between the initial and additional channels by introducing delay to align their respective multiframe signals, the transmitted A-bit is set to 0;

–	the E-bit for each additional B-channel is transmitted in the additional B-channel in the same connection, because it relates to a physical condition of the transmission path.

The BAS operation in additional connections is restricted to the transmission of the additional channel number (according to Table A.5/H.221) and TIX (see Recommendation H.230) (thus the channel numbering of any additional connection shall be sent both in BAS according to Annex A and in the FAS as in 2.2), while channel numbering of the initial channel is sent only in FAS.

The distant terminal, upon receiving the A-bit set to zero with respect to sequentially numbered channels, can add their capacity to the initial connection by sending the transfer rate BAS in Annex A. The order of the bits transmitted in the channels is in accordance with the examples given in Figure 5.

### 2.7.2	Multiple H0 connectionsxe "Multiple H0 connections"§

FAS and BAS are transmitted in the first time-slot of each H0.

FAS operation is as in 2.7.1 except that the channel number is used to order the six octets received each 125 microseconds with respect to the six octet groups received in other channels.

The BAS operation in additional channels is as specified in 2.7.1.

# 3	Bit-rate allocation signalxe "Bit-rate allocation signal"§

## 3.1	Encoding of the BASxe "Encoding of the BAS"§

The bit-rate allocation signal (BAS) occupies bits 9–16 of the service channel in every frame. An eight bit BAS code (b0, b1, b2, b3, b4, b5, b6, b7) is complemented by eight error correction bits (p0, p1, p2, p3, p4, p5, p6, p7) to implement a (16,8) double error correcting code. This error correcting code is obtained by shortening the (17,9) cyclic code with generator polynomial:

g(x)  =  x8  +  x7  +  x6  +  x4  +  x2  +  x  +  1

The error correction bits are calculated as coefficients of the remainder polynomial in the following equation:

p0x7  +  p1x6  +  p2x5  +  p3x4  +  p4x3  +  p5x2  +  p6x  +  p7

= RESg(x) [b0x15  +  b1x14  +  b2x13  +  b3x12  +  b4x11  +  b5x10  +  b6x9  +  b7x8]

where RESg(x)[f (x)] represents the residue obtained by dividing f (x) by g(x).

The BAS code is sent in the even-numbered frame, while the associated error correction bits are sent in the subsequent odd-numbered frame. The bits of the BAS code or the error correction are transmitted in the order shown in Table 2 to avoid emulation of the frame alignment word.

TABLE  2/H.221

| Bit position | Even frame | Odd frame |
| --- | --- | --- |
| 09 | b0 | p2 |
| 10 | b3 | p1 |
| 11 | b2 | p0 |
| 12 | b1 | p4 |
| 13 | b5 | p3 |
| 14 | b4 | p5 |
| 15 | b6 | p6 |
| 16 | b7 | p7 |

The decoded BAS value is valid if:

–	the receiver is in frame and multiframe alignment, and

–	the FAW in the same sub-multiframe was received with two or fewer bits in error.

Otherwise the decoded BAS value is ignored.

When the receiver actually looses frame alignment, it may be advisable to undo any changes caused by the three previously decoded values as they may well have been erroneous even after correction.

## 3.2	Values of the BASxe "Values of the BAS"§

The encoding of BAS is made according to a hierarchical attribute method. This consists of attribute class (8 classes), attribute family (7 families), attribute (8 attributes) and value (32 values). The first three bits of an attribute represent its number describing the general command or capability, and the other five bits identify the “value” – the specific command or capability.

The following attributes are defined in the Class (000) and Family (000):

| Attribute | Significance |
| --- | --- |
| 000 | Audio coding command |
| 001 | Transfer rate command |
| 010 | Video and other command |
| 011 | Data command |
| 100 | Terminal capability 1 |
| 101 | Terminal capability 2 |
| 110 | Reserved |
| 111 | Escape codes |

The values of these attributes are listed and defined in Annex A. They provide for the following facilities:

–	transmission at various total rates and on single and multiple channels, on clear channels and on networks subject to restrictions to 56 kbit/s and its multiples;

–	audio transmission, digitally encoded to various recommended algorithms;

–	video transmission, digitally encoded to a recommended algorithm, with provision for future recommended improvement;

–	Low-speed data (LSD) within the I-channel, or TS1 of a higher rate initial channel;

–	High-speed data (HSD) in the highest-numbered 64 kbit/s channel or time-slots (excluding the I-channel);

–	data transmission within a standardised protocol, in a logical subchannel either in the I-channel (MLP) or in capacity other than the I-channel (H-MLP);

–	an encryption control signal;

–	loopback towards the network for maintenance purposes;

–	signalling for control and indications;

–	a message system for, inter alia, conveying information concerning equipment manufacturer and type.

The command BAS attributesxe "command BAS attributes"§ have the following significance: on receipt of a BAS command code in one (even) frame and its error-correcting code in the next (odd), the receiver prepares to accept the stated mode change beginning from the subsequent (even) frame; thus a mode change can be effected in 20 milliseconds. The command remains in force until countermanded (see 12/H.242). The bit positions occupied by combinations of BAS commands are exemplified in Figures 5a to 5g.

The capability BAS attributesxe "capability BAS attributes"§ have the following significance: they indicate the ability of a terminal to receive and properly treat the various types of signal. It follows that having received a set of capability values from the remote terminal Y, terminal X shall not transmit signals lying outside that declared range.

Values [0–7] of the attribute (111) are reserved for setting the class, and [8-14] for setting the family; the default value is (000) for both.

The next nine attribute values of the attribute (111) are temporary escape BAS codes of single byte extension (SBE). The last three bits of the temporary escape BAS form a pointer to one of eight possible escape BAS tables of 224 entries each (codes beginning with 111 are not used in the escape BAS tables). Then the next received BAS indicates the specific entry in the escape BAS table.

The value (111) [24] is the capability marker (see 2/H.242) which is followed by normal BAS codes, not by any escape values.

The last seven attribute values of the attribute (111) are of multiple byte extension (MBE) and are used to send messages as specified in A.9the Notes to Tables A.2 and A.3.

| Bit number N7mero de8bit |  |  |  | Octet number |  |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 1 |
| 2 |  |  |  |  | 2 |
| : | FAS |  |  |  | : |
| 8 |  |  |  |  | 8 |
| 9 |  |  |  |  | 9 |
| : | BAS |  |  |  | : |
| 16 |  |  |  |  | 16 |
| 17 | 18 |  |  |  | 17 |
| 19 | 20 |  |  |  | 18 |
| : | : |  |  |  | : |
| 143 | 144 |  |  |  | 80 |

FIGURE  5a/H.221

Bit numbering and position for 14.4 kbit/s LSD

| Bit number |  |  |  |  |  |  |  | Octet |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | number |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |  | 1 |
| : | : | : | : | : | : | : | FAS | 2 |
| : | : | : | : | : | : | : |  | : |
| 50 | 51 | 52 | 53 | 54 | 55 | 56 |  | 8 |
| 57 | 58 | 59 | 60 | 61 | 62 | 63 |  | 9 |
| : | : | : | : | : | : | : | BAS | : |
| : | : | : | : | : | : | : |  | : |
| 106 | 107 | 108 | 109 | 110 | 111 | 112 |  | 16 |
| 113 | 114 | 115 | 116 | 117 | 118 | 119 |  | 17 |
| 120 | 121 | 122 | 123 | 124 | 125 | 126 |  | 18 |
| : | : | : | : | : | : | : | Sub-channel 8 | : |
| : | : | : | : | : | : | : |  | : |
| 554 | 555 | 556 | 557 | 558 | 559 | 560 |  | 80 |

FIGURE  5b/H.221

56 kbit/s LSD

| Bit number |  |  |  |  |  |  |  | Octet |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | number |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |  | 1 |
| : | : | : | : | : | : | : | FAS | 2 |
| : | : | : | : | : | : | : |  | : |
| 50 | 51 | 52 | 53 | 54 | 55 | 56 |  | 8 |
| 57 | 58 | 59 | 60 | 61 | 62 | 63 |  | 9 |
| : | : | : | : | : | : | : | BAS | : |
| : | : | : | : | : | : | : |  | : |
| 106 | 107 | 108 | 109 | 110 | 111 | 112 |  | 16 |
| 113 | 114 | 115 | 116 | 117 | 118 | 119 | 120 | 17 |
| 121 | 122 | 123 | 124 | 125 | 126 | 127 | 128 | 18 |
| : | : | : | : | : | : | : | : | : |
| : | : | : | : | : | : | : | : | : |
| 617 | 618 | 619 | 620 | 621 | 622 | 623 | 624 | 80 |

FIGURE  5c/H.221

62.4 kbit/s LSD

FIGURE  5d’/H.221

| Audio bit rate | Bit number |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| Rec. G.711 | MSB | · · · | · · · | · · · | · · · | · · · | · · · | LSB |
| Rec. G.722, 64 kbit/s | H | H | L | L | L | L | L | L |
| Rec. G.722, 56 kbit/s | H | H | L | L | L | L | L | – |
| Rec. G.722, 48 kbit/s | H | H | L | L | L | L | – | – |
| Rec. G.728, 16 kbit/s | See below |  | – | – | – | – | – | – |

H	High-band audio

L	low-band audio

Bit positions for G.711 and G.722 audio

G.728 audio

The LD-CELP 2.5 ms frame consists of the following 40 numbered bits:

Codeword 0, bit 9 (MSB) to bit 0 (LSB): 09,08,07,06,05,04,03,02,01,00

Codeword 1, bit 9 (MSB) to bit 0 (LSB): 19,18,17,16,15,14,13,12,11,10

Codeword 2, bit 9 (MSB) to bit 0 (LSB): 29,28,27,26,25,24,23,22,21,20

Codeword 3, bit 9 (MSB) to bit 0 (LSB): 39,38,37,36,35,34,33,32,31,30

These are packed into two 8 kbit/s H.221 sub-channels by putting odd numbered bits in the first sub-channel and even numbered bits in the second. This structure is repeated four times in each 10 ms H.221 frame as shown below. The first codeword in each H.221 frame is then always the first codeword in the speech coder frame also. The speech coder synchronization can then be derived from H.221 FAS (frame alignment signal).

The 10 ms H.221 frame

| Bit number | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | Octet number |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 09 | 08 |  |  |  |  |  | F | 1 |
|  | 07 | 06 |  |  |  |  |  | A | 2 |
|  | 05 | 04 |  |  |  |  |  | S | 3 |
|  | 03 | 02 |  |  |  |  |  |  | " |
|  | 01 | 00 |  |  |  |  |  |  | " |
|  | 19 | 18 |  |  |  |  |  |  | " |
| Speech | 17 | 16 |  |  |  |  |  |  | " |
| coder | " | " |  |  |  |  |  |  | " |
| frame 0 | 11 | 10 |  |  |  |  |  |  | " |
|  | 29 | 28 |  |  |  |  |  |  | " |
|  | " | " |  |  |  |  |  |  | " |
|  | 21 | 20 |  |  |  |  |  |  | " |
|  | 39 | 38 |  |  |  |  |  |  | " |
|  | " | " |  |  |  |  |  |  | " |
|  | 31 | 30 |  |  |  |  |  |  | " |
|  | 09 | 08 |  |  |  |  |  |  | " |
| Speech | 07 | 06 |  |  |  |  |  |  | " |
| coder | " | " |  |  |  |  |  |  | " |
| frame 1 | 33 | 32 |  |  |  |  |  |  | " |
|  | 31 | 30 |  |  |  |  |  |  | " |
|  | 09 | 08 |  |  |  |  |  |  | " |
| Speech | 07 | 06 |  |  |  |  |  |  | " |
| coder | " | " |  |  |  |  |  |  | " |
| frame 2 | 33 | 32 |  |  |  |  |  |  | " |
|  | 31 | 30 |  |  |  |  |  |  | " |
|  | 09 | 08 |  |  |  |  |  |  | " |
| Speech | 07 | 06 |  |  |  |  |  |  | " |
| coder | " | " |  |  |  |  |  |  | " |
| frame 3 | 33 | 32 |  |  |  |  |  |  | 79 |
|  | 31 | 30 |  |  |  |  |  |  | 80 |

FIGURE  5d’’/H.221

Bit positions for G.728 audio

Note: Figure 5e/H.221 also exemplifies the bit order applicable when MLP-14.4k and H-MLP-62.4k are both in force, forming a single MLP channel.

| Initial channel |  |  |  |  |  |  |  | Additional channel |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| A1 | A2 | A3 | A4 | A5 | A6 | V1 |  | V2 | V3 | V4 | V5 | V6 | V7 | V8 |  |
| A | · · |  |  | · · | A6 | V9 | FAS | V10 |  |  |  |  |  | V16 | FAS |
| · |  |  |  |  | · |  |  |  |  |  |  |  |  |  |  |
| · |  |  |  |  | · |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | BAS |  |  |  |  |  |  |  | BAS |
|  |  |  |  |  |  | V121 |  | V122 |  |  |  |  |  | V128 |  |
|  |  |  |  |  |  | V129 | V130 | V131 |  |  |  |  |  | V137 | V138 |
|  |  |  |  |  |  | V139 |  |  |  |  |  |  |  |  | V148 |
| · |  |  |  |  | · | · |  |  |  |  |  |  |  |  | · |
| · |  |  |  |  | · | · |  |  |  |  |  |  |  |  | · |
| · |  |  |  |  | · | · |  |  |  |  |  |  |  |  | · |
| A | · · |  |  | · · | A | V759 | · · |  |  |  |  |  |  | · · | V768 |

FIGURE  5e/H.221

Bit positions for video in two B-channels

| TS1 |  |  |  |  |  |  |  | TS2 | TS3 | TS4 | TS5 | TS6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | A | A | A | A | A | A | F | V1 V8 | V9 V16 | V17 V24 | D1 D8 | D9 D16 |
|  |  |  |  |  |  |  | A | V25 |  | V48 | D17 | D32 |
|  |  |  |  |  |  |  | S |  |  |  |  |  |
|  |  |  |  |  |  |  | B |  |  |  |  |  |
|  |  |  |  |  |  |  | A |  |  |  |  |  |
|  |  |  |  |  |  |  | S | V361 |  | V384 | D241 | D256 |
|  |  |  |  |  |  |  | V | V386 |  | V409 | D257 |  |
|  |  |  |  |  |  |  | V | V411 |  |  |  |  |
|  |  |  |  |  |  |  | · | V· |  |  |  |  |
|  |  |  |  |  |  |  | · | V· |  |  |  |  |
|  |  |  |  |  |  |  | · | V· |  | ·84 | D1· | ·80 |
|  |  |  |  |  |  |  | · | V· |  | ·84 | D1· | ·80 |
|  |  |  |  |  |  |  | V | V1961 · · |  | · · V1984 | D1265 · · | · · D1280 |

FIGURE  5f/H.221

128 kbit/s HSD in H0 channel

| Initial B-channel |  |  |  |  |  |  |  | 2nd channel |  | 3rd channel |  | 4th channnel |  | 5th channel |  | 6th channel |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | A | A | A | A | A | A | F | V1 V7 | F | V8 V14 | F | V15 V21 | F | V22 V28 | F | D1 D8 |  |
|  |  |  |  |  |  |  | A | V29 | A |  | A | V42 | A | V56 | A | D9 D16 |  |
|  |  |  |  |  |  |  | S |  | S |  | S |  | S |  | S |  |  |
|  |  |  |  |  |  |  | B |  | B |  | B |  | B |  | B |  |  |
|  |  |  |  |  |  |  | A |  | A |  | A |  | A |  | A |  |  |
|  |  |  |  |  |  |  | S | V421 | S |  | S |  | S | V448 | S | D121 D128 |  |
|  |  |  |  |  |  |  | V | V450 |  |  |  |  |  | V481 |  | D129 D136 |  |
|  |  |  |  |  |  |  | V | V483 |  |  |  |  |  | V514 |  | D137 D144 |  |
|  |  |  |  |  |  |  | · | V· |  |  |  |  |  | V14 | · | D·37 D37 | · |
|  |  |  |  |  |  |  | · | V· |  |  |  |  |  | V 14 | · | D·37 D37 | · |
|  |  |  |  |  |  |  | · | · |  |  |  |  |  | 60 | · | D·37 D37 | · |
|  |  |  |  |  |  |  | · | · |  |  |  |  |  | 60 | · | D·37 D37 | · |
|  |  |  |  |  |  |  | V | V2529 · · |  |  |  |  |  | · · V2560 |  | D633 · · D640 |  |

FIGURE  5g/H.221

64 kbit/s HSD in 6 ´ 64 kbit/s channels

## 3.3	Procedures for the use of BAS

The use of BAS codes is specified in Recommendation H.242.

Annex A
Definitions and tables of BAS valuesxe "Definitions and tables of BAS values"§

(This annex forms an integral part of this Recommendation)

The definitions of BAS values are given in this Annex, and the corresponding numerical values are listed in the Tables A.1 to A.5. In the Tables the column header gives the attribute designation as bits (b0, b1, b2); the left-hand column gives the decimal value of bits [b3, b4, b5, b6, b7]; for example, “Dig-loop” has the value (010) [10100]. All unassigned values are reserved, as are values marked (R).

TABLE  A.1/H.221

BAS numerical values

|  |  | (000) | (001) | (010) | (011) | (100) | (101) | (110) | (111) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [0] |  | neutrala) | 64 | Video-off | LSD-off | neutral | var-LSD | Restrict_L | class (R) |  |
| [1] |  | capex | 2×64 | H.261 | LSD_300 | A-law | LSD_300 | Restrict_P | class (R) |  |
| [2] |  | (R) | 3×64 | vid-imp(R) | LSD_1200 | µ-law | LSD_1200 | NoRestrict | class (R) |  |
| [3] |  | (R) | 4×64 | video-ISO | LSD_4800 | G.722-64 | LSD_4800 | pref-cap | class (R) |  |
| [4] |  | A-law,0U | 5×64 |  | LSD_6400 | G.722-48 | LSD_6400 | (R) | class (R) |  |
| [5] |  | µ-law,0U | 6×64 | MLP-8k | LSD_8000 | G.728 | LSD_8000 | (R) | class (R) |  |
| [6] |  | G.722,m1a) | 384 | encryp-on | LSD_9600 | (R) | LSD_9600 | (R) | class (R) |  |
| [7] |  | Au-off,Ua) | 2×384 | encryp-off | LSD_14.4k | SM-comp | LSD_14.4k | (R) | class (R) |  |
| [8] |  | Note 1 | 3×384 | (R) | LSD_16k | 128 | LSD_16k | (R) | family(R) |  |
| [9] |  | Note 1 | 4×384 | (R) | LSD_24k | 192 | LSD_24k | (R) | family(R) |  |
| [10] |  | (R) | 5×384 | (R) | LSD_32k | 256 | LSD_32k | (R) | family(R) |  |
| [11] |  | (R) | 1536 | (R) | LSD_40k | 320 | LSD_40k | (R) | family(R) |  |
| [12] |  | (R) | 1920 | (R) | LSD_48k | 512 | LSD_48k | (R) | family(R) |  |
| [13] |  |  | 128 | (R) | LSD_56k | 768 | LSD_56k | (R) | family(R) |  |
| [14] |  |  | 192 | (R) | LSD_62.4k | Null | LSD_62.4k | (R) | family(R) |  |
| [15] |  |  | 256 | (R) | LSD_64k | 1152 | LSD_64k | (R) | Table_A.65 |  |
| [16] |  |  | 320 | freeze-pic | MLP-off | 1B | MLP-4k | (R) | Table_A.2 |  |
| [17] |  |  | loss i.c. | fast-update | MLP-4k | 2B | MLP-6.4k | (R) | H.230 |  |
| [18] |  | A-law,0Fa) | (R) | Au-loop | MLP-6.4k | 3B | var-MLP | (R) | Table_A.43 |  |
| [19] |  | µ-law,0Fa) | (R) | Vid-loop | var-MLP | 4B | MLP_Set1 | (R) | SBE numbers |  |
| [20] |  | A-law,F6a) | (R) | Dig-loop | MLP-14.4k | 5B | QCIF | (R) | SBE characters |  |
| [21] |  | µ-law,F6a) | (R) | Loop-off | MLP-22.4k | 6B | CIF | (R) | SBE (R) |  |
| [22] |  | (R) | (R) | (R) | MLP-30.4k | restrict | 1/29.97 | (R) | SBE (R) |  |
| [23] |  | (R) | 512 | SM-comp | MLP-38.4k | 6B-H0-comp | 2/29.97 | (R) | SBE (R) |  |
| [24] |  | G.722,m2a) Note 2 | 768 | not-SM-comp | MLP-46.4k | H0 | 3/29.97 | (R) | cap-mark |  |
| [25] |  | G.722,m3a) Note 2 | (R) | 6B-H0-comp | MLP-16k | 2H0 | 4/29.97 | (R) | start-MBE |  |
| [26] |  | Au-40k(R) | 1152 | not-6B-H0-comp | MLP-24k | 3H0 | V-imp(R) | (R) | (R) |  |
| [27] |  | Au-32k(R) | (R) | Restrict_Required | MLP-32k | 4H0 | video-ISO | (R) | (R) |  |
| [28] |  | Au-24k(R) | (R) | derestrict | MLP-40k | 5H0 | MLP_Set2 | (R) | (R) |  |
| [29] |  | G.728a) | 1472 | (R) | MLP-62.4k | 1472 | esc-CF(R) | (R) | (R) |  |
| [30] |  | Au<16k(R) | (R) | (R) | MLP-64k | H11 | encryp. | (R) | ns-cap |  |
| [31] |  | Au-off,Fa) | (R) | (R) | var-LSD | H12 | MBE-cap | (R) | ns-comm |  |
|  | a) Use of these codes in the 56 kbit/s environments is defined in Annex B. NOTES 1 These codes are listed in Recommendation G.725 with reference to an “application channel”; such a channel has not been defined, the concept having been superseded by that of LSD/MLP; therefore these codes should not be used. 2 These codes are listed in Recommendation G.725 with reference to “data”; however, the nature of such data (video, LSD, MLP, ECS) shall be specified by further commands (001), (010), (011). |  |  |  |  |  |  |  |  |  |

TABLE  A.2/H.221

Values reached by escape BAS (111)[16]

|  | (000) | (001) Au-ISO commands | (010) | (011) HSD/H-MLP commands | (100) Au-ISO capabilities | (101) HSD/H-MLP capabilities | (110) MLP capabilities | (111) Forbidden |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [0] |  | Au-ISO-off |  | HSD-off |  |  | MLP-14.4k |  |
| [1] |  | Au-ISO-32k |  | var-HSD | Au-ISO-1B | var-HSD | MLP-22.4k |  |
| [2] |  | Au-ISO-40k |  | H-MLP-62.4 | Au-ISO-2B | H-MLP-62.4 | MLP-30.4k |  |
| [3] |  | Au-ISO-48k |  | H-MLP-64k | Au-ISO-3B | H-MLP-64k | MLP-38.4k |  |
| [4] |  | Au-ISO-56k |  | H-MLP-128k | Au-ISO-4B | H-MLP-128k | MLP-46.4k |  |
| [5] |  | Au-ISO-62.4k |  | H-MLP-192k | Au-ISO-5B | H-MLP-192k | (R) |  |
| [6] |  | Au-ISO-64k |  | H-MLP-256k | Au-ISO-6B | H-MLP-256k | MLP-62.4k |  |
| [7] |  | Au-ISO-80k |  | H-MLP-320k |  | H-MLP-320k | MLP-8k |  |
| [8] |  | Au-ISO-96k |  | H-MLP-384k |  | H-MLP-384k | MLP-16k |  |
| [9] |  | Au-ISO-112k |  |  |  |  | MLP-24k |  |
| [10] |  | Au-ISO-2B |  |  |  |  | MLP-32k |  |
| [11] |  | Au-ISO-128k |  |  |  |  | MLP-40k |  |
| [12] |  | Au-ISO-160k |  | H-MLP-14.4k |  | H-MLP-14.4k | (R) |  |
| [13] |  | Au-ISO-3B |  | var-H-MLP |  | var-H-MLP | (R) |  |
| [14] |  | Au-ISO-192k |  | H-MLP-off |  |  | MLP-64k |  |
| [15] |  | Au-ISO-224k |  |  |  |  |  |  |
| [16] |  | Au-ISO-4B |  |  | Sample-16k |  |  |  |
| [17] |  | Au-ISO-256k |  | HSD-64k | Sample-22.05k | HSD-64k |  |  |
| [18] |  | Au-ISO-288k |  | HSD-128k | Sample 24k | HSD-128k |  |  |
| [19] |  | Au-ISO-5B |  | HSD-192k | CorrMode-1 | HSD-192k |  |  |
| [20] |  | Au-ISO-320k |  | HSD-256k | CorrMode-2 | HSD-256k |  |  |
| [21] |  | Au-ISO-352k |  | HSD-320k | CorrMode-3 | HSD-320k |  |  |
| [22] |  | Au-ISO-6B |  | HSD-384k |  | HSD-384k |  |  |
| [23] |  | Asynch |  | HSD-512k |  | HSD-512k |  |  |
| [24] |  | Synch |  | HSD-768k | AsyncMode | HSD-768k |  |  |
| [25] |  | Error-off |  | HSD-1152k | AuLayer-I | HSD-1152k |  |  |
| [26] |  | Error-1 |  | HSD-1536k | AuLayer-II | HSD-1536k |  |  |
| [27] |  | Error-2 |  |  | AuLayer-III |  |  |  |
| [28] |  | Error-3 |  |  | Sample-32k |  |  |  |
| [29] |  |  |  |  | Sample-44.1k |  |  |  |
| [30] |  |  |  |  | Sample-48k |  |  |  |
| [31] |  |  |  |  |  |  |  |  |

## A.1	Audio command valuesxe "Audio command values"§ (000)

For bit position illustrations see Figures 5d’ and 5d”. Abbreviations “G.711” and “G.722” refer to Recommendations.

Neutral	Neutralized I-channel, containing only FAS and BAS; all other bits are to be ignored at the receiver3).

Capex	transmitted by a Channel Aggregation Unit (see Rec. H.244)

Au-off, U	Switches off G.711/722/728 audio (but not Au-ISO as in Table A.2) and switches off the frame structure in the I-channel; all the I-channel is available for use under commands  other than (000)[n] (Notes 3,4).

Au-off, F	Switches off G.711/722/728 audio (but not Au-ISO as in Table A.2) ; FAS and BAS in use (mode 9); 62.4 kbit/s in the I-channel  available for use under commands  other than (000)[n].

A-law, 0U	G.711 audio at 64 kbit/s, A-law, no framing (Mode 0U)4).

A-law, 0F	G.711 audio at 56 kbit/s, A-law, truncated to 7 bits in bits 1-7, with FAS and BAS in bit 8; bit 8 is set to zero at the PCM audio decoder (Mode 0F).

µ-law, 0U	G.711 audio at 64 kbit/s, µ-law, no framing (Mode 0U)4).

µ-law, 0F	G.711 audio at 56 kbit/s, µ-law, truncated to 7 bits in bits 1-7, with FAS and BAS in bit 8; bit 8 is set to zero at the PCM audio decoder (Mode 0F).

A-law, F6	Audio according to ITU-T Recommendation G.711 [4] at 48 kbit/s, A-law truncated to 6 bits, with FAS and BAS in bit 8 (use only according to §13.4/H.242).

µ-law, F6	Audio according to ITU-T Recommendation G.711 [4] at 48 kbit/s, µ-law truncated to 6 bits, with FAS and BAS in bit 8 (use only according to §13.4/H.242).

G.722, m1	G.722 7 kHz audio at 64 kbit/s, no framing (mode 1)4).

G.722, m2	G.722 7 kHz audio at 56 kbit/s, in bits 1-7 (mode 2).

G.722, m3	G.722 7 kHz audio at 48 kbit/s, in bits 1-6 (mode 3).

Au-40k	Reserved for audio at less than 48 kbit/s (for example 40 kbit/s in bits 1-5).

Au-32k	Reserved for audio at less than 48 kbit/s (for example 32 kbit/s in bits 1-4).

Au-24k	Reserved for audio at less than 48 kbit/s (for example 24 kbit/s in bits 1-3).

G.728	Audio at 16 kbit/s to Recommendation G.728 in bits 1 and 2 (mode 7).

Au-<16k	Reserved for audio at less than 48 kbit/s (for example 8 kbit/s in bit 1).

## A.2	Transfer-rate command valuesxe "Transfer-rate command values"§ (001)

NOTE – If the transfer-rate command is less than the available connected capacity, the information occupies the lowest-numbered channel(s)/time-slot(s).

64	Signal occupies one 64 kbit/s channel.

2  ´  64	Signal occupies two 64 kbit/s channels, with FAS and BAS in each.

3 to 6  ´  64	Signal occupies three to six 64 kbit/s channels, with FAS and BAS in each.

384	Signal occupies 384 kbit/s, with FAS and BAS in the first 64 kbit/s time-slot; the effective channel may be the whole of an H0 channel or the lowest numbered time-slots of an H11 or H12 channel.

2  ´  384	Signal occupies two channels of 384 kbit/s, with FAS and BAS in each.

3 to 5  ´  384	Signal occupies three to five 384 kbit/s channels, with FAS and BAS in each.

1536	Signal occupies 1536 kbit/s, with FAS and BAS in the first 64 kbit/s time-slot. The effective channel occupies the whole of an H11 channel or the lowest numbered time-slots of an H12 channel.

1920	Signal occupies 1920 kbit/s, with FAS and BAS in the first 64 kbit/s time-slot. The effective channel occupies the whole of an H12 channel.

128/192/256/320	Signal occupies 128/192/256/320 kbit/s, with FAS and BAS in the first 64 kbit/s time-slot. The effective channel occupies the lowest numbered time-slots of a channel with corresponding or higher capacity.

512/768/1152/1472	Signal occupies 512/768/1152/1472 kbit/s, with FAS and BAS in the first 64 kbit/s time-slot. The effective channel occupies the lowest numbered time-slots of a channel with corresponding or higher capacity.

Loss-i.c.	Designated “Initial channel”, especially used following loss of the channel previously so designated (see Rec. H.242).

## A.3	Video, encryption, loop and other commandsxe "Video, encryption, loop and other commands"§ (010)

Video-off	No video; video switched off.

H.261	Video on, to Recommendation H.261: video occupies all capacity not otherwise allocated by other commands; video cannot be inserted in the I-channel when var-LSD or var-MLP is in force; examples are given in Figure 5e.

Specifically, the video rate in initial B-channel (framed) or TS1 is: 62.4 kbit/s – audio rate – {800 bit/s if ECS is ON} – {MLP rate if ON} – {LSD rate if ON} – {8kbit/s if restricted}.

Vid-imp.(R)	Reserved for video on, to improved recommended algorithm.

Video-ISO	Video on, to ISO/IEC 11172-2 (“MPEG-1”): video occupies the same capacity as stipulated above for the case of H.261 video.

Freeze-pic.	Freeze-picture request (see Recommendation H.230, VCF).

Fast-update	Fast-update request (see Recommendation H.230, VCU).

Encryp-on	ECS Channel active.

NOTE – When encryption is active, it may apply (see H.233) to all information bits in all channels of the connection, except bits 1-24 of the SC in the I-channel and the FAS and BAS positions of the other channels; use of encryption in conjunction with MLP is for further study.

Encryp-off	ECS channel off.

Au-loop	Audio loop request (see Recommendation H.230, LCA).

Vid-loop	Video loop request (see Recommendation H.230, LCV).

Dig-loop	Digital loop request (see Recommendation H.230, LCD).

Loop-off	Loop off request (see Recommendation H.230, LCO).

NOTE – Loopback requests are intended for use by maintenance staff.

SM-comp	"Single<>Multiple Channel Compatibility": to provide for compatibility between terminals connected to single-channel and multiple-64/56-channel accesses, the least significant bits of the first 16 octets of all 64 kbit/s time-slots of the single channel, except TS1, are not used; the single-channel terminal shall discard these bits from the incoming signal on receipt of this command, and shall set the same bits to "1" in the outgoing signal

Cancel-SM-comp	Negates the command SM-comp (010)[23]

6B-H0-comp	To provide for compatibility between terminals connected to single H0 channel and six B-channel accesses, the least significant bits of the first 16 octets of all time-slots of the H0 channel, except TS1, are not used; the H0 terminal shall discard these bits from the incoming signal on receipt of this code, and shall set the same bits to “1” in the outgoing signal.

Not-6B-H0	Negates the command “6B-H0-comp”.

NOTE – Used, for example, in testing.

Restrict	To provide for operation on a restricted network, and for interconnection between a terminal on restricted and unrestricted networks: on receipt of this code, a terminal shall treat the SC as being in bit 7 of the I-channel, and discard bit 8 of every other channel and/or time-slot; in the outgoing direction these bits are set to “1”.

Derestrict	On receipt of this code, a terminal shall revert to “unrestricted network” operation, treating the SC as being in bit 8 of the I-channel.

## A.4	LSD/MLP commandsxe "LSD/MLP commands"§ (011)

For bit position illustrations see Figure 5.

#	These LSD rates are not allowed if ECS channel is in use.

*	In restricted cases, the starred bit numbers are reduced by one.

LSD off	LSD switched off.

LSD_300	Low-speed data at 300 bit/s in SC, octets 38-40.

LSD_1200	Low-speed data at 1200 bit/s in SC, octets 29-40.

LSD_4800	Low-speed data at 4800 bit/s in SC, octets 33-80.

LSD_6400	Low-speed data at 6400 bit/s in SC, octets 17-80#.

LSD_8000	Low-speed data at 8000 bit/s in bit 7*.

LSD_9600	Low-speed data at 9600 bit/s in bit 7* and octets 25-40 of SC.

LSD_14.4k	Low-speed data at 14 400 bit/s in bit 7* and octets 17-80 of SC#.

LSD_16k	Low-speed data at 16 kbit/s in bit 6* and bit 7*.

LSD_24k	Low-speed data at 24 kbit/s in bits 5*, 6* and 7*.

LSD_32k	Low-speed data at 32 kbit/s in bits 4*-7*.

LSD_40k	Low-speed data at 40 kbit/s in bits 3*-7*.

LSD_48k	Low-speed data at 48 kbit/s in bits 2*-7*.

LSD_56k	Low-speed data at 56 kbit/s in bits 1-7 (no framing in restricted case).

LSD_62.4k	Low-speed data at 62.4 kbit/s in bits 1-7 and octets 17-80 of SC. If ECS channel is in use, the data rate is reduced to 61.6 kbit/s, but returns to 62.4 kbit/s if ECS channel is closed.

LSD_64k	Low-speed data at 64 kbit/s in bits 1-8, no framing.

Var-LSD	Low-speed data occupying all I-channel capacity not allocated under other fixed-rate commands; cannot be invoked when other LSD is on, or when variable-MLP is on (may also be impractical when video is on in I-channel alone).

Exact var-LSD rate: 62.4 kbit/s – audio rate – {800 bit/s if ECS is ON} – {fixed-MLP if ON} – {8000 bit/s if restricted}.

MLP-off	MLP and H-MLP off in all channels.

Var-MLP	MLP occupying all I-channel capacity not allocated under other fixed-rate commands: cannot be invoked when other MLP is on, or when variable-LSD is on (may also be impractical when video is on in I-channel alone).

Exact var-MLP rate: 62.4 kbit/s – audio rate – {800 bit/s if ECS is ON} – {fixed-LSD if ON} – {8000 bit/s if restricted}.

Other MLP commands	MLP on at the rate and bit occupancy given in Table A.3x below; where octets 17-24 of Bit 8 are shown as used, then when ECS is on it takes precedence, and the MLP rate is reduced by 800 bit/s, but is restored if the ECS channel is closed. In restricted cases, the starred bit positions are reduced by one. (MLP-4k is insufficient bandwidth for normal T.120 and H.224 applications and should be avoided).

TABLE A.3x/H.221: Bit occupancy under MLP commands

| Table A.1/H.221 reference | Rate | Bit 1 | Bit 2 | Bit 3* | Bit 4* | Bit 5* | Bit 6* | Bit 7* | Bit 8* (SC) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MLP-4k | 4 kbit/s | - | - | - | - | - | - | - | octets 41-80 |
| MLP-6.4k | 6.4 k | - | - | - | - | - | - | - | octets 17-80 |
| MLP-8k | 8k | - | - | - | - | - | - | all | - |
| MLP-14.4k | 14.4k | - | - | - | - | - | - | all | octets 17-80 |
| MLP-16k | 16k | - | - | - | - | - | all | all | - |
| MLP-22.4k | 22.4k | - | - | - | - | - | all | all | octets 17-80 |
| MLP-24k | 24k | - | - | - | - | all | all | all | - |
| MLP-30.4k | 30.4k | - | - | - | - | all | all | all | octets 17-80 |
| MLP-32k | 32k | - | - | - | all | all | all | all | - |
| MLP-38.4k | 38.4k | - | - | - | all | all | all | all | octets 17-80 |
| MLP-40k | 40k | - | - | all | all | all | all | all | - |
| MLP-46.4k | 46.4k | - | - | all | all | all | all | all | octets 17-80 |
| MLP-62.4k | 62.4k | all | all | all | all | all | all | all | octets 17-80 |
| MLP-64k | 64k | all | all | all | all | all | all | all | all |

## A.5	Audio capabilitiesxe "Audio capabilities"§ (100)

Neutral	Neutral capability: no change in the current capabilities of the terminal.

A-law	Capable of decoding audio to Recommendation G.711, A-law.

µ-law	Capable of decoding audio to Recommendation G.711, µ-law.

G.722-645-T1	Capable of decoding audio to Recommendation G.722 (mode 1) and to Recommendation G.711.

G.722-485-T2	Capable of decoding audio to Recommendation G.722 (modes 1, 2, 3) and to Recommendation G.711.

G.728	Capable of decoding audio, both to Recommendation G.728 and Recommendation G.711.

Null	Capability having no significance other than as a filler. Note: this value may occur any number of times within a capability set transmitted towards a Single-Channel Equipment - see (Channel Aggregation) Rec. H.244

## A.6	Video, MBE and encryption capabilitiesxe "Video, MBE and encryption capabilities"§ (101)

QCIF	Can decode video to QCIF picture format, but not CIF (see Recommendation H.261) – this code shall be followed by one of the four minimum picture interval (MPI) values below.

CIF	Can decode video to CIF and QCIF formats (see Recommendation H.261) – this code shall be followed by two MPI values, the first applicable to QCIF and the other to CIF format.

Minimum picture interval (MPI) codes are as follows:

1/29.97	Can decode video, having a minimum picture interval of 1/29.97 seconds, to Recommendation H.261.

2/29.97	Can decode video, having a minimum picture interval of 2/29.97 seconds, to Recommendation H.261.

3/29.97	Can decode video, having a minimum picture interval of 3/29.97 seconds, to Recommendation H.261.

4/29.97	Can decode video, having a minimum picture interval of 4/29.97 seconds, to Recommendation H.261.

Vid-imp(R)	Reserved for future improved recommended video algorithm.

Video-ISO	Can decode video to ISO/IEC 11172-2.

MBE-cap	Can handle multiple-byte extensions messages in the BAS position, those beginning with codes in the range (111) [25-31], in addition to other values.

Esc-CF(R)	Reserved for capability to accept non-zero class/family escape codes.

Encryp.	Capable of handling signals on the ECS channel.

## A.7	Transfer-rate capabilities (100)

B, H0	Can accept signals only on one 64 kbit/s channel, one 384 kbit/s channel.

2B	Can accept signals on one or two 64 kbit/s channels, and synchronize them.

. . .	. . .

6B	Can accept signals on one to six 64 kbit/s channels, and synchronize them.

2  ´  H0	Can accept signals on one or two 384 kbit/s channels, and synchronize them.

. . .	. . .

5  ´  H0	Can accept signals on one to five 384 kbit/s channels, and synchronize them.

H11/H12	Can accept signals on a 1536 kbit/s channel, a 1920 kbit/s channel.

Restrict	Can work only at p ´ 56 kbit/s, rate-adapted to p ´ 64 kbit/s by moving the SC to bit position 7 and setting bit 8 to “one” in every channel or time-slot; a constant “one”, however, may be set in bit 8 if it is known by out-of-band signalling prior to the connection that the restriction exists; this code has the effect of forcing the remote terminal to work in the p ´ 56 kbit/s mode (see Annex B).

6B-H0-comp	Capable of acting upon the corresponding command.

SM-comp	Capable of acting on the corresponding command; applies to all declared single-channel transfer rates; capable also of acting upon the commands [capex] and [AggIN]* (see H.244).

128/192/256/320	Capable of accepting the transfer rate specified by the corresponding command.

512/768/1152/1472	Capable of accepting the transfer rate specified by the corresponding command.

## A.8	LSD/MLP capabilitiesxe "LSD/MLP capabilities"§ (101) and other (110)

LSD_300 (to 64k)	Can accept LSD at 300 bit/s (to 64 kbit/s) in the bit positions specified against the corresponding commands.

Var-LSD	Can accept LSD variable rate in the bit positions specified against the corresponding command.

MLP-4k	Can accept MLP in the bit positions specified against the corresponding command

MLP-6.4k	Can accept MLP in the bit positions specified against the corresponding command.

MLP_Set1	Can accept MLP at 6.4k, 14.4k, 32k and 40k in the bit positions specified against the corresponding commands

MLP_Set2	Can accept MLP at all fixed rates up to and including 62.4k in the bit positions specified against the corresponding commands

Var-MLP	Can accept MLP in the I-channel under the corresponding command.

Restrict_P	Can receive and transmit in Restrict_P mode defined in H.242

Restrict_L	Can receive and transmit in Restrict_L mode defined in H.242

NoRestrict	Cannot receive in either Restrict_P or Restrict_L mode

Pref-cap	Can recognise and act upon Mode preference SBE symbols - see H.242

## A.9	Escape table values (111)

Table_A.65	Escape to values listed in Table A.65/H.221.

Table_A.2	Escape to values listed in Table A.2/H.221

H.230	Control and indications: see definitions in Recommendation H.230.

SBE numbers	Gives access to a table of SBE numbers - see H.230

SBE characters	Gives access to a table of SBE characters - see H.230

Start-MBE	First byte of (N + 2) octet BAS message defined in Recommendation H.230

NS-cap	First byte of non-ITU capabilities message; the message format is:

NS-cap//value of N (max = 255)//country code5)//manufacturer code*//(N – 4) bytes.

NS-comm	First byte of non-ITU command message; the message format is:

NS-comm//value of N (max = 255)//country code5)//manufacturer code*//(N – 4) bytes.

Cap-mark	Capability marker – the first item in a capability set – see 2/H.242.

Table_A.43	Applications within LSD/HSD/MLP channels– see Table A.43.

NOTES

1	The value of N is coded by its binary representation.

2	The most significant bit of each MBE message byte is transmitted as the b0 bit of BAS.

## A.10	HSD/H-MLP/MLP capabilities (Table A.2)(111) [10000]-(101)

HSD-64k to 1536k	Can accept HSD at the specified rate in the bit positions specified against the corresponding commands.

Var-HSD	Can accept HSD variable rate in the bit positions specified against the corresponding command.

H-MLP-62.4k	Can accept H-MLP at 62.4 kbit/s in the bit positions specified against the corresponding command.

H-MLP-r	Can accept H-MLP at r = 14.4/64/128/192/256/320/384 kbit/s in the bit positions specified against the corresponding command.

Var-H-MLP	Capability to accept H-MLP variable rate in the bit positions specified against the corresponding command.

MLP-14.4k/16k/22.4k/24k/30.4k/32k/38.4k/40k/46.4k/62.4k/64k

Can accept MLP in the bit positions specified against the corresponding command.

| TABLE A.43/H.221: Numerical values for applications in LSD/HSD/MLP channels - reached by escape BAS (111)[18] |  | TABLE A.54/H.221: BAS codes in additional channels |
| --- | --- | --- |

|  | Commands (010) | Commands (011) | Capabilities (101) |  |  |  |  | (001) | (010) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [0] |  | Reserved for ISO-SP on in LSD | (R) ISO-SP baseline on LSD |  |  |  | [0] |  | chan.#16 |
| [1] |  | Reserved for ISO-SP on in HSD | (R) ISO-SP baseline on HSD |  |  |  | [1] |  | chan.#17 |
| [2] |  |  | (R) ISO-SP spatial |  |  |  | [2] |  | chan.#18 |
| [3] |  |  | (R) ISO-SP progressive |  |  |  | [3] |  | chan.#19 |
| [4] |  |  | (R) ISO-SP arithmetic |  |  |  | [4] |  | chan.#20 |
| [5] |  |  |  |  |  |  | [5] |  | chan.#21 |
| [6] |  |  |  |  |  |  | [6] |  | chan.#22 |
| [7] |  |  |  |  |  |  | [7] |  | chan.#23 |
| [8] |  |  |  |  |  |  | [8] |  | chan.#24 |
| [9] |  |  | Still image (Rec. H.261) |  |  |  | [9] |  |  |
| [10] |  | Cursor data on in LSD (R) | Graphics cursor (R) |  |  |  | [10] |  |  |
| [11] |  |  |  |  |  |  | [11] |  |  |
| [12] |  |  |  |  |  |  | [12] |  |  |
| [13] |  |  |  |  |  |  | [13] |  |  |
| [14] |  |  |  |  |  |  | [14] |  |  |
| [15] |  |  |  |  |  |  | [15] |  |  |
| [16] |  | (R) Fax on in LSD | (R) Group 3 fax |  |  |  | [16] |  |  |
| [17] |  | (R) Fax on in HSD | (R) Group 4 fax |  |  |  | [17] |  |  |
| [18] |  |  |  |  |  |  | [18] | chan.#2 |  |
| [19] |  |  |  |  |  |  | [19] | chan.#3 |  |
| [20] |  | V.120 LSD | V.120 LSD |  |  |  | [20] | chan.#4 |  |
| [21] |  | V.120 HSD | V.120 HSD |  |  |  | [21] | chan.#5 |  |
| [22] |  | V.14_LSD | V.14_LSD |  |  |  | [22] | chan.#6 |  |
| [23] |  | V.14_HSD | V.14_HSD |  |  |  | [23] | chan.#7 |  |
| [24] | H.224_MLP-off | H.224_MLP-on | H.224_MLP |  |  |  | [24] | chan.#8 |  |
| [25] | H.224_LSD-off | H.224_LSD-on | H.224_LSD |  |  |  | [25] | chan.#9 |  |
| [26] | H.224_HSD-off | H.224_HSD-on | H.224_HSD |  |  |  | [26] | chan.#10 |  |
| [27] | (R) | (R) | H.224-sim |  |  |  | [27] | chan.#11 |  |
| [28] | T.120-off | T.120-on | T.120-cap |  |  |  | [28] | chan.#12 |  |
| [29] |  |  | Nil_Data |  |  |  | [29] | chan.#13 |  |
| [30] |  |  |  |  |  |  | [30] | chan.#14 |  |
| [31] |  |  |  |  |  |  | [31] | chan.#15 |  |

TABLE A.65/H.221: BAS numerical values used in Channel Aggregation - reached by escape BAS (111)[15]

|  | (000) | (001) | (010) Transfer-rate commands | (011) Transfer-rate commands | (100) Transfer-rate capabilities | (101) Transfer-rate capabilities | (110) | (111) Forbidden |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [0] |  |  |  |  |  |  |  |  |
| [1] |  |  |  |  |  |  |  |  |
| [2] |  |  |  |  |  |  |  |  |
| [3] |  |  |  |  |  |  |  |  |
| [4] |  |  |  |  |  |  |  |  |
| [5] |  |  |  |  |  |  |  |  |
| [6] |  |  |  |  |  |  |  |  |
| [7] |  |  | 7×64 | 7*64 | 7×64 | 7*64 |  |  |
| [8] |  |  | 8×64 | (R) (note 1) | 8×64 | (R) (note 1) |  |  |
| [9] |  |  | 9×64 | 9*64 | 9×64 | 9*64 |  |  |
| [10] |  |  | 10×64 | 10*64 | 10×64 | 10*64 |  |  |
| [11] |  |  | 11×64 | 11*64 | 11×64 | 11*64 |  |  |
| [12] |  |  | 12×64 | (R) (note 1) | 12×64 | (R) (note 1) |  |  |
| [13] |  |  | 13×64 | 13*64 | 13×64 | 13*64 |  |  |
| [14] |  |  | 14×64 | 14*64 | 14×64 | 14*64 |  |  |
| [15] |  |  | 15×64 | 15*64 | 15×64 | 15*64 |  |  |
| [16] |  |  | 16×64 | 16*64 | 16×64 | 16*64 |  |  |
| [17] |  |  | 17×64 | 17*64 | 17×64 | 17*64 |  |  |
| [18] |  |  | 18×64 | (R) (note 1) | 18×64 | (R) (note 1) |  |  |
| [19] |  |  | 19×64 | 19*64 | 19×64 | 19*64 |  |  |
| [20] |  |  | 20×64 | 20*64 | 20×64 | 20*64 |  |  |
| [21] |  |  | 21×64 | 21*64 | 21×64 | 21*64 |  |  |
| [22] |  |  | 22×64 | 22*64 | 22×64 | 22*64 |  |  |
| [23] |  |  | 23×64 | (R) (note 1) | 23×64 | (R) (note 1) |  |  |
| [24] |  |  | 24×64 | (R) (note 1) | 24×64 | (R) (note 1) |  |  |
| [25] |  |  |  |  |  |  |  |  |
| [26] |  |  |  |  |  |  |  |  |
| [27] |  |  |  |  |  |  |  |  |
| [28] |  |  |  |  |  |  |  |  |
| [29] |  |  |  |  |  |  |  |  |
| [30] |  |  |  |  |  |  |  |  |
| [31] |  |  |  |  |  |  |  |  |

Definitions of these codepoints, including the significance of * and ×, are contained in Recommendation H.244.

Note 1: Table A.1 contains values which otherwise would have been assigned these codes.

## A.11	HSD/H-MLP commands (Table A.2)(111) [10000]-(011)

NOTE 1: In the case of multiple channels, the term “highest-numbered time-slot” refers to the highest-numbered channel.

NOTE 2:	When the "restrict" command is in force the least significant bit of all octets covered by the HSD and H-MLP commands is set to "1", so the effective data rate is less than that indicated by the command.

HSD-off	HSD switched off; FAS and BAS restored in additional channels.

HSD-64k	HSD on, in highest numbered channel/time-slot; FAS and BAS are removed in the case of multiple B-channels.

HSD-128/192/256k	HSD on in highest-numbered time-slots of an H0 or greater channel.

HSD-320k	HSD on in highest-numbered time-slots of an H0 or greater channel.

HSD-384k	HSD on in highest-numbered H0 channel, or highest-numbered time-slots of a greater channel; FAS and BAS are removed in the case of multiple-H0 channels.

HSD-512/768/1152/1536	HSD on in highest-numbered H0 channels, or highest-numbered time-slots of a greater channel; FAS and BAS are removed in the case of multiple-H0 channels.

Var-HSD	High-speed data occupying all capacity, other than in the I-channel, not allocated under other commands: cannot be invoked when other HSD is on, or when var-H-MLP is on (may also be impractical when video is on, the latter then being confined to the I-channel).

H-MLP-off	H-MLP switched off (this does not affect I-channel MLP).

H-MLP-14.4k	H-MLP on at 14.4 kbit/s, occupying Bits 7 and 8 of B-channel #2, except FAS and BAS positions.

H-MLP-62.4k	H-MLP on at 62.4 kbit/s, occupying (additional) channel #2, except FAS and BAS positions.

| H-MLP-64k H-MLP-128k H-MLP-192k H-MLP-256k H-MLP-320k | H-MLP on at 64/128/192/256/320 kbit/s in the lowest-numbered time-slots, (other than TS1) of an H0 or greater channel, or at 124.8/187.2..... in the lowest-numbered additional channels of a multi-channel connection. |
| --- | --- |

H-MLP-384k	H-MLP on at 384 kbit/s in time-slots 2-7 of a greater channel than H0.

Var-H-MLP	H-MLP occupying all capacity, other than in the I-channel, not allocated under other commands: cannot be invoked when other H-MLP is on, or when var-HSD is on. If video is ON it is restricted to the I-channel.

## A.12 Au-ISO commands (Table A.2)(111) [10000]-(001)

For bit position illustrations see Figure 6/H.221.  Definition of "audio" and procedures for use of these codes are defined in Recommendation J.52. NB: the use of these commands in n×56 kbit/s connections and other restricted networks is for further study.

| Au-ISO-off | Audio switched off (cancellation of any of the commands (111)[10000](001)[1-22] listed in Table A.2/H.221) |
| --- | --- |
| Au-ISO-32K | 1B-mode: Audio data at 32 kbit/s in initial channel in bit 3...6 |
| Au-ISO-40K | 1B-mode: Audio data at 40 kbit/s in initial channel in bit 3...7 |
| Au-ISO-48k | 1B-mode: Audio data at 48 kbit/s in initial channel in bit 1...6 |
| Au-ISO-56k | 1B-mode: Audio data at 56 kbit/s in initial channel in bit 1...7 |
| Au-ISO-62.4k | 1B-mode: Audio data at 62.4 kbit/s in initial channel in bit 1...7 and in octets 17...80 of SC |
| Au-ISO-64k | 1B-mode: Audio data at 64 kbit/s in initial channel in bit 1...8, unframed |
| Au-ISO-80k | 2B-mode: Audio data at 80 kbit/s, occupying in the I-channel Bits 5,6 and octets 41...56 of the SC and in the lowest-numbered additional channel or TS2 all except bit 8 of octets 1-16 - see Figure 6/H.221 |
| Au-ISO-96k | 2B-mode: Audio data at 96 kbit/s, as for "Au-ISO-80k" but now including also Bits 3 and 4 of the I-channel. |
| Au-ISO-112k | 2B-mode: Audio data at 112 kbit/s, as for "Au-ISO-80k" but now including also Bits 1-4 of the I-channel. |
| Au-ISO-2B | 2B-mode: in multiple B-channels, audio data occupies all the I-channel and all the lowest-numbered additional channel except the FAS and BAS positions in both, the resultant bitrate being 124.8 kbit/s; in single high-rate channels, audio data occupies all the I-channel except the FAS and BAS positions and all of TS2, the resultant bitrate being 126.4 kbit/s. |
| Au-ISO-128k | 3B-mode: Audio data at 128 kbit/s, occupying in the I-channel only octets 41...72 of the SC and in the two lowest-numbered additional channels, or TS2 and TS3, all except bit 8 of octets 1-16 - see Figure 6/H.221 |
| Au-ISO-160k | 3B-mode: Audio data at 160 kbit/s, as for "Au-ISO-128k" but now including also Bits 3-6 of the I-channel. |
| Au-ISO-3B | 3B-mode: as for "Au-ISO-2B", now using 3 channels/time-slots; the resultant rates are 187.2 kbit/s for multiple-B and 190.4 kbit/s for single high-rate connections. |
| Au-ISO-192k | 4B-mode: Audio data at 192 kbit/s, occupying in the I-channel only octets 25...72 of the SC and in the three lowest-numbered additional channels, or TS2-4, all except bit 8 of octets 1-16 - see Figure 6/H.221 |
| Au-ISO-224k | 4B-mode: Audio data at 224 kbit/s, as for "Au-192k" but now including also Bits 3-6 of the I-channel. |
| Au-ISO-4B | 4B-mode: as for "Au-ISO-2B", now using 4 channels/time-slots; the resultant rates are 249.6 kbit/s for multiple-B and 254.4 kbit/s for single high-rate connections. |
| Au-ISO-256k | 5B-mode: Audio data at 256 kbit/s, occupying in the I-channel only octets 17-80 of the SC and in the four lowest-numbered additional channels, or TS2-5, all except bit 8 of octets 1-16 - see Figure 6/H.221 |
| Au-ISO-288k | 5B-mode: Audio data at 288 kbit/s, as for "Au-ISO-256k" but now including also Bits 3-6 of the I-channel. |
| Au-ISO-5B | 5B-mode: as for "Au-ISO-2B", now using 5 channels/time-slots; the resultant rates are 312 kbit/s for multiple-B and 318.4 kbit/s for single high-rate connections. |
| Au-ISO-320k | 6B-mode: Audio data at 320 kbit/s, occupying in the I-channel only Bit 5 and in the five lowest-numbered additional channels, or TS2-6, all except bit 8 of octets 1-16 - see Figure 6/H.221 |
| Au-ISO-352k | 6B-mode: Audio data at 352 kbit/s, as for "Au-ISO-320k" but now including also Bits 3-6 of the I-channel. |
| Au-ISO-6B | 6B-mode: as for "Au-ISO-2B", now using 6 channels/time-slots; the resultant rates are 373.4 kbit/s for multiple-B and 382.4 kbit/s for single high-rate connections. |
| Error-1/2/3/off | Error correction data of the ancillary data field of the ISO/IEC 11172-3 signal are to mode 1/2/3 or off. |
| Asynch | Asynchronous mode in use |
| Synch | Synchronous mode in use |

## A.13 Au-ISO capabilities (Table A.2)(111) [10000]-(100)

Definition of "audio" and procedures for use of these codes are defined in Recommendation J.52.

| Au-ISO-1B | Capability to operate in any of the audio modes listed in the corresponding command table, on a single B-channel** |
| --- | --- |
| Au-ISO-2B | Capability to operate in any of the audio modes listed in the corresponding command table, on one or two B-channels** (or TS1) |
| Au-ISO-3B | Capability to operate in any of the audio modes listed in the corresponding command table, on one, two or three B-channels** |
| Au-ISO-4B | Capability to operate in any of the audio modes listed in the corresponding command table, on one to four B-channels** |
| Au-ISO-5B | Capability to operate in any of the audio modes listed in the corresponding command table, on one to five B-channels** |
| Au-ISO-6B | Capability to operate in any of the audio modes listed in the corresponding command table, on one to six B-channels** |
| Asynch.mode | Can decode audio data sampled asynchronous to the network clock |
| Au-Layer-I | Capable of decoding audio to ISO/IEC 11172-3 Layer I |
| Au-Layer-II | Capable of decoding audio to ISO/IEC 11172-3 Layer II |
| Au-Layer-III | Capable of decoding audio to ISO/IEC 11172-3 Layer III |
| Sample-16k | Can decode audio sampled with 16 kHz clock frequency |
| Sample-22.05k | Can decode audio sampled with 22.05 kHz clock frequency |
| Sample-24k | Can decode audio sampled with 24 kHz clock frequency |
| Sample-32k | Can decode audio sampled with 32 kHz clock frequency |
| Sample-44.1k | Can decode audio sampled with 44.1 kHz clock frequency |
| Sample-48k | Can decode audio sampled with 48 kHz clock frequency |
| Correction - Mode 1, 2 and 3 | Can decode error correction data of the ancillary data field of the ISO/IEC 11172-3 signal, appropriate mode |

**  Or the corresponding number of an H0 or higher channel, from TS1 upwards.

## A.14	Applications within LSD/HSD channels – capabilitiesxe "Applications within LSD/HSD channels – capabilities"§ (Table A.4)(111) [10010]-(101)

ISO-SP baseline on on LSD	(Reserved) Can accept ISO-still picture (SP) baseline mode on specified LSD rate.

ISO-SP baseline on HSD	(Reserved) Can accept ISO-still picture baseline mode on specified HSD rate.

ISO-SP spatial	(Reserved) Can accept ISO-still picture baseline and spatial modes.

ISO-SP progressive	(Reserved) Can accept ISO-still picture baseline and progressive modes.

ISO-SP arithmetic	(Reserved) Can accept ISO-still picture baseline and arithmetic modes.

Still image (H.261)	Can accept still images encoded by the method defined in Annex D/H.261 (See Note)

NOTE – Administrations may use this optional procedure as a simple and inexpensive method to transmit still images. However, Recommendation T.81 (JPEG) as described in Rec. T.126 and using the T.120 protocol stack in the MLP channel is preferred.

Graphics cursor	(Reserved) Can handle graphics cursor data.

Group 3 fax	(Reserved) Can accept Group 3 fax.

Group 4 Fax	(Reserved) Can accept Group 4 fax.

V.120 LSD	Can accept V.120 terminal adaptation within an LSD channel.

V.120 HSD	Can accept V.120 terminal adaptation within an HSD channel.

V.14_LSD	Can accept V.14 terminal adaptation within an LSD channel.

V.14_HSD	Can accept V.14 terminal adaptation within an HSD channel.

H.224_MLP	Defined in Rec. H.224

H.224_LSD	Defined in Rec. H.224

H.224_HSD	Defined in Rec. H.224.

H.224-sim	Defined in Rec. H.224.

T.120-cap	Can accept the protocol defined in T.123, T.122/125 and T.124 in the MLP and/or H-MLP channel. Support for other T-series protocols is not implied

Nil_Data	No data applications available at rates specified by subsequent data capability values within the same capset; if/when data paths are opened, transmitted content is only binary Ones, and any received data will be ignored (see H.242, §9).

## A.15	Applications within LSD/HSD/MLP/H-MLP channels – commandsxe "Applications within LSD/HSD channels – commands"§ (Table A.4)(111) [10010]-(011)

ISO-SP on in LSD	(Reserved) ISO-still picture switched on in specified LSD.

ISO-SP on in HSD	(Reserved) ISO-still picture switched on in specified HSD.

Cursor data on in LSD	(Reserved) Cursor data switched on in specified LSD.

Fax on in LSD	(Reserved) Fax switched on in specified LSD.

Fax on in HSD	(Reserved) Fax switched on in specified HSD.

V.120_LSD	V.120 switched on in specified LSD.

V.120_HSD	V.120 switched on in specified HSD.

V.14_LSD	V.14 switched on in specified LSD.

V.14_HSD	V.14 switched on in specified HSD.

H.224_LSD-on/off		Defined in Rec. H.224

H.224_HSD-on/off		Defined in Rec. H.224

H.224_MLP-on/off		Defined in Rec. H.224

T.120_on/off	T.120 suite protocol On/Off in MLP and/or H-MLP channels

## A.16	Transfer-rate capabilities and commands used in Channel Aggregation (Table A.65)

n*64	n = 7 to 11, 13 to 17, 19 to 23; Commands: Signal occupies single channel of 448 kbit/s or corresponding higher multiple of 64 kbit/s, with FAS and BAS in the first 64 kbit/s time-slot. The effective channel occupies the lowest numbered time-slots of a channel with corresponding or higher capacity. Capabilities: can accept signals according to the corresponding command.

N×64	N = 7 to 24; Commands: Signal occupies the given number of 64 kbit/s channels, with FAS and BAS in each. Capabilities: can accept and synchronise signals according to the corresponding command.

FIGURE 6/H.221

Bit positions for ISO/IEC 11172-3 Audio in one or two 64 kbit/s channels

| Bit No. | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |  |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octet 1 |  |  | 1 | 2 | 3 | 4 |  |  |  | 1 |  |  | 1 | 2 | 3 | 4 | 5 |  |
| 2 |  |  | 5 | 6 | 7 | 8 |  | F |  | 2 |  |  | 6 | 7 | 8 | 9 | 10 | F |
| .... |  |  |  |  |  |  |  | A |  | .... |  |  | .... | .... | .... | .... | .... | A |
| 7 |  |  |  |  |  |  |  | S |  | 7 |  |  |  |  |  |  |  | S |
| 8 |  |  |  |  |  |  |  |  |  | 8 |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |  | 9 |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  | B |  | 10 |  |  |  |  |  |  |  | B |
| .... |  |  |  |  |  |  |  | A |  | .... |  |  |  |  |  |  |  | A |
| 15 |  |  |  |  |  |  |  | S |  | 15 |  |  |  |  |  |  |  | S |
| 16 |  |  |  |  |  |  |  |  |  | 16 |  |  |  |  |  |  |  |  |
| 17 |  |  |  |  |  |  |  |  |  | 17 |  |  |  |  |  |  |  |  |
| 18 |  |  |  |  |  |  |  |  |  | 18 |  |  |  |  |  |  |  |  |
| 19 |  |  |  |  |  |  |  |  |  | 19 |  |  |  |  |  |  |  |  |
| .... |  |  |  |  |  |  |  |  |  | .... |  |  |  |  |  |  |  |  |
| .... |  |  |  |  |  |  |  |  |  | .... |  |  |  |  |  |  |  |  |
| .... |  |  |  |  |  |  |  |  |  | .... |  |  |  |  |  |  |  |  |
| 79 |  |  |  |  |  |  |  |  |  | 79 |  |  | .... | .... | .... | .... | 395 |  |
| 80 |  |  | 317 | 318 | 319 | 320 |  |  |  | 80 |  |  | 396 | 397 | 398 | 399 | 400 |  |

| 32 kbit/s audio | 40 kbit/s audio |
| --- | --- |
| Note: Bits 1 and 2 are left free so that G.728 may be On simultaneously |  |

| Bit No. | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |  |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octet 1 | 1 | 2 | 3 | 4 | 5 | 6 |  |  |  | 1 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |  |
| 2 | 7 | 8 | 9 | 10 | 11 | 12 |  | F |  | 2 | 8 | 9 |  |  |  |  |  | F |
| .... |  |  |  |  |  |  |  | A |  | .... |  |  |  |  |  |  |  | A |
| 7 |  |  |  |  |  |  |  | S |  | 7 |  |  |  |  |  |  |  | S |
| 8 |  |  |  |  |  |  |  |  |  | 8 |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |  | 9 |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  | B |  | 10 |  |  |  |  |  |  |  | B |
| .... |  |  |  |  |  |  |  | A |  | .... |  |  |  |  |  |  |  | A |
| 15 |  |  |  |  |  |  |  | S |  | 15 |  |  |  |  |  |  |  | S |
| 16 |  |  |  |  |  |  |  |  |  | 16 |  |  |  |  |  |  |  |  |
| 17 |  |  |  |  |  |  |  |  |  | 17 |  |  |  |  |  |  |  |  |
| 18 |  |  |  |  |  |  |  |  |  | 18 |  |  |  |  |  |  |  |  |
| 19 |  |  |  |  |  |  |  |  |  | 19 |  |  |  |  |  |  |  |  |
| .... |  |  |  |  |  |  |  |  |  | .... |  |  |  |  |  |  |  |  |
| .... |  |  |  |  |  |  |  |  |  | .... |  |  |  |  |  |  |  |  |
| .... |  |  |  |  |  |  |  |  |  | .... |  |  |  |  |  |  |  |  |
| 79 |  |  |  |  |  | 474 |  |  |  | 79 |  |  |  |  |  |  | 553 |  |
| 80 |  |  |  |  | 479 | 480 |  |  |  | 80 |  |  |  |  |  | 559 | 560 |  |

| 48 kbit/s audio | 56 kbit/s audio |
| --- | --- |

| Bit No. | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |  |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octet 1 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |  |  | 1 | 1 |  |  |  |  |  |  | 8 |
| 2 |  |  |  |  |  |  | 14 | F |  | 2 |  |  |  |  |  |  |  | 16 |
| .... |  |  |  |  |  |  |  | A |  | .... |  |  |  |  |  |  |  | 24 |
| 7 |  |  |  |  |  |  |  | S |  | 7 |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  | 8 |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |  | 9 |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  | B |  | 10 |  |  |  |  |  |  |  |  |
| .... |  |  |  |  |  |  |  | A |  | .... |  |  |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  |  | S |  | 15 |  |  |  |  |  |  |  |  |
| 16 |  |  |  |  |  |  | 112 |  |  | 16 |  |  |  |  |  |  |  |  |
| 17 |  |  |  |  |  |  | 119 | 120 |  | 17 |  |  |  |  |  |  |  |  |
| 18 |  |  |  |  |  |  |  |  |  | 18 |  |  |  |  |  |  |  |  |
| 19 |  |  |  |  |  |  |  |  |  | 19 |  |  |  |  |  |  |  |  |
| .... |  |  |  |  |  |  |  |  |  | .... |  |  |  |  |  |  |  |  |
| .... |  |  |  |  |  |  |  |  |  | .... |  |  |  |  |  |  |  |  |
| .... |  |  |  |  |  |  |  |  |  | .... |  |  |  |  |  |  |  |  |
| 79 |  |  |  |  |  |  |  | 616 |  | 79 |  |  |  |  |  |  |  |  |
| 80 |  |  |  |  |  |  | 623 | 624 |  | 80 | 633 |  |  |  |  |  |  | 640 |

| 62.4 kbit/s audio | 64 kbit/s audio |
| --- | --- |

FIGURE 6/H.221 (continued)

Bit positions for ISO/IEC 11172-3 Audio in one or two 64 kbit/s channels

| Initial channel | Channel #2 |
| --- | --- |

| Bit No. | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octet 1 |  |  |  |  | 1 | 2 |  |  | 3 | 4 | 5 | 6 | 7 | 8 | 9 |  |
| 2 |  |  |  |  | 10 | 11 |  | F | 12 | 13 | 14 | 15 | 16 | 17 | 18 | F |
| .... |  |  |  |  |  |  |  | A |  |  |  |  |  |  |  | A |
| 7 |  |  |  |  |  |  |  | S |  |  |  |  |  |  |  | S |
| 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  | B |  |  |  |  |  |  |  | B |
| .... |  |  |  |  |  |  |  | A |  |  |  |  |  |  |  | A |
| 15 |  |  |  |  |  |  |  | S |  |  |  |  |  |  |  | S |
| 16 |  |  |  |  | 136 | 137 |  |  | 138 | 139 | 140 | 141 | 142 | 143 | 144 |  |
| 17 |  |  |  |  | 145 | 146 |  |  | 147 | 148 | 149 | 150 | 151 | 152 | 153 | 154 |
| 18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| .... |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 40 |  |  |  |  |  |  |  |  |  |  |  |  |  | 382 | 383 | 384 |
| 41 |  |  |  |  | 385 | 386 |  | 387 | 388 | 389 | 390 | 391 | 392 | 393 | 394 | 395 |
| .... |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 56 |  |  |  |  | 550 | 551 |  | 552 | 553 |  |  |  |  |  |  | 560 |
| 57 |  |  |  |  | 561 |  |  |  |  |  |  |  |  |  |  | 570 |
| .... |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 79 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 790 |
| 80 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 799 | 800 |

80 kbit/s audio in two 64 kbit/s channels

| Initial channel | Channel #2 |
| --- | --- |

| Bit No. | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octet 1 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |  | 8 | 9 | 10 | 11 | 12 | 13 | 14 |  |
| 2 |  |  |  |  |  |  |  | F |  |  |  |  |  |  |  | F |
| .... |  |  |  |  |  |  |  | A |  |  |  |  |  |  |  | A |
| 7 |  |  |  |  |  |  |  | S |  |  |  |  |  |  |  | S |
| 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  | B |  |  |  |  |  |  |  | B |
| .... |  |  |  |  |  |  |  | A |  |  |  |  |  |  |  | A |
| 15 |  |  |  |  |  |  |  | S |  |  |  |  |  |  |  | S |
| 16 |  |  |  |  |  |  | 217 |  | 218 |  |  |  |  |  | 224 |  |
| 17 |  |  |  |  |  |  | 231 | 232 | 233 |  |  |  |  |  | 239 | 240 |
| 18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| .... |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| .... |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| .... |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 79 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 80 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1248 |

124.8 kbit/s audio in two 64 kbit/s channels

Note: Bit positions for audio in three or more channels may be derived from the foregoing illustrations for two channels.

Annex B
Frame structure for interworking between a 64 kbit/s terminal 
and a 56 kbit/s terminal

(This annex forms an integral part of this Recommendation)

## B.1	Sub-channel arrangement

The sub-channel arrangement is given in Table B.1.

## B.2	Operation of the 64 kbit/s terminal

The transmitter fills the eighth sub-channel with “1”, while the receiver searches FAS at every sub-channel. It should be noted that at the receiver side stuffing bits “1” appear always at Bit number 8, but FAS and BAS appear at any of Bit numbers 1–7.

## B.3	Restriction against some communication modes

Since the interworking bit rate becomes 56 kbit/s, the transmission modes using more than 56 kbit/s are forbidden (receivers ignore these command BAS codes). Facilities using the original seventh sub-channel move to the sixth sub-channel.

## B.4	Audio command codes (000)

The following are applicable instead of those in Annex A.

Neutral	Neutralized I-channel, containing only FAS and BAS; all other bits are to be ignored at the receiver.

Au-off, U	No audio signal, no framing; bits 1-7 of the I-channel are available.

Au-off, F	No audio signal, FAS and BAS in use; 54.4 kbit/s available for use under other commands.

A-law, U7	G.711 audio at 56 bit/s, A-law truncated to 7 bits, no framing (Mode 0U).

A-law, F6	G.711 audio at 48 kbit/s, A-law truncated to 6 bits, with FAS and BAS in bit 7.

m-law, U7	G.711 audio at 56 kbit/s, m-law truncated to 7 bits, no framing (Mode 0U).

m-law, F6	G.711 audio at 48 kbit/s, m-law truncated to 6 bits, with FAS and BAS in bit 7.

G.722, U8	Not possible to transmit 8 bits per octet.

G.722, U7	G.722 7 kHz audio in bits 1-7, 56 kbit/s (unframed).

G.722, F6	G.722 7 kHz audio at 48 kbit/s, in bits 1-6 (mode 3).

G.728Au-16 kbit/s	Audio at 16 kbit/s to Recommendation G.728 in bits 1,2 (mode 7).

[Other]	All other values reserved.

TABLE  B.1/H.221

Transmitter of the 64 kbit/s terminal

| Bit number |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 (SC) | 8 |  |  |
|  |  |  |  |  |  |  | 1 | 1 | Octet number |
| S | S | S | S | S | S | FAS | 1 | : |  |
| u | u | u | u | u | u |  | 1 | 8 |  |
| b | b | b | b | b | b |  | 1 | 9 |  |
| - | - | - | - | - | - | BAS | 1 | : |  |
| c | c | c | c | c | c |  | 1 | 16 |  |
| h | h | h | h | h | h |  | 1 | 17 |  |
| a | a | a | a | a | a | (ECS) | 1 | : |  |
| n | n | n | n | n | n |  | 1 | 24 |  |
| n | n | n | n | n | n |  | 1 | 25 |  |
| e | e | e | e | e | e |  | 1 | · |  |
| l | l | l | l | l | l |  | 1 | · |  |
| # | # | # | # | # | # | # | 1 | · |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 1 | 80 |  |
| NOTE – C1, C2, C3 and C4 in the FAS are computed for the 160 septets, or 1120 bits. |  |  |  |  |  |  |  |  |  |

Receiver of the 64 kbit/s terminal

|  | Bit numbera) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
|  |  |  |  |  |  |  |  | 1 |
|  |  |  |  |  |  |  |  | 1 |
|  |  |  |  |  |  | s | s | 1 |
|  | s | s | s | s |  | u | u | 1 |
|  | u | u | u | u | Fb) | b | b | 1 |
|  | b | b | b | b | Ab) | - | - | 1 |
|  | - | - | - | - | Sb) | c | c | 1 |
|  | c | c | c | c |  | h | h | 1 |
| A frame | h | h | h | h |  | a | a | 1 |
| structured | a | a | a | a |  | n | n | 1 |
| by the | n | n | n | n |  | n | n | 1 |
| 56 kbit/s | n | n | n | n |  | e | e | 1 |
| terminal | e | e | e | e | Bb) | l | l | 1 |
|  | l | l | l | l | Ab) | # | # | 1 |
|  | # | # | # | # | Sb) | 1 | 2 | 1 |
|  | 3 | 4 | 5 | 6 |  |  |  | 1 |
|  |  |  |  |  |  |  |  | 1 |
|  |  |  |  |  | #7 |  |  | 1 |
|  |  |  |  |  |  |  |  | 1 |
|  |  |  |  |  |  |  |  | 1 |
|  |  |  |  |  |  |  |  | 1 |
|  |  |  |  |  |  |  |  | 1 |
|  |  |  |  |  |  |  |  | 1 |
|  |  |  |  |  |  |  |  | 1 |
|  |  |  |  |  |  |  |  | 1 |
|  | a) Synchronized with the octet timing of the network. b) FAS may appear at any of Bit number 1–7. |  |  |  |  |  |  |  |

The following (000) values are assigned maintaining the same number of audio bits per octet between the 64 kbit/s and 56 kbit/s environments:

[0]	Neutral

[6]	Not possible

[7]	Au-off, U

[18]	A-law, U7

[19]	µ-law, U7

[20]	A-law, F6

[21]	µ-law, F6

[24]	G.722, U7

[25]	G.722, F6

[29]	G.728

[31]	Au-off, F

1	)	This Recommendation completely replaces the text of Recommendations H.221 and H.222 published in Fascicle III.6 of the Blue Book.

2	)	If the transfer rate is such that a part of any H0/H11/H12 channel is unoccupied, then the computation is made only for that part covered by the transfer rate.

3	)	It is interpreted as a command to shut off all the output of the I-channel demultiplexer except FAS, BAS and ECS (if relevant). Audio is muted accordingly. Release of this shut off is activated by a fixed rate command (namely by a command other than Var-LSD, Var-MLP). Channels other than I-channel (such as additional channel for 2B communications, or the 2nd through 6th timeslot for H0 communications) remain unchanged.

If video or HSD was set on before this Neutral BAS command is issued, it continues to be on. For example, if video has been on in a 2B communication, and Neutral BAS command is issued, the video is transmitted only in the additional channel. If a fixed rate command for I-channel is then issued, the video also occupies all bit positions of I-channel other than those designated by the fixed rate command, and FAS and BAS positions. In case of 1B communication, video is completely excluded by this Neutral BAS command, but it will recover by e.g. next 16 kbit/s audio command.

It is noted that no procedures for the use of neutral BAS command have been adopted.

5	)	Country code consists of two bytes, the first being according to Recommendation T.35; the second byte and the terminal manufacturer code of two bytes are assigned nationally.