|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 676 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | November 2011 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 12/16 | 21 November – 2 December 2011 |  |  |
| STUDY GROUP 16 – CONTRIBUTION 676 |  |  |  |  |  |
| Source: |  | Waseda University |  |  |  |
| Title: |  | Updated information flow of AMS |  |  |  |

Summary

This Contribution updates the AMS communication sequence and information flow for a regular multimedia communication and an application handover reflecting the Andover meeting discussions in July 2011.

# 1.	Introduction

We have submitted several Contributions on the AMS information flows covering both a regular communication case and an application handover case to progress the AMS architecture and protocol study[1-4]. They address basic information flow among Human user, local Application, local Container, Network (represented by SN <Service Node>), remote Container and remote Application without touching upon the inside of each system entity. This Contribution updates the information flow by reflecting the discussion of AVD-4096[4] and AVD-4133[5] at the Andover Q12/16 meeting in July 2011 [6].

# 2.	Regular multimedia communication

Here the "regular communication" is meant to be basic multimedia communication between two Assemblages.

A communication scenario and corresponding information flow with associated descriptions are contained in Annex A. Remaining issues are also listed there. Compared to the previous version in AVD-4096[4], the following changes have been made:

1)	#06 message through Interface A from Container A to Container B is renamed from “Pre-invoke” to “Session Establish.” Once the Human user has shown his/her request of communication mode consisting of Applications 1, 2 and 3 to Container A, it reserves the three local Applications under its control by Pre-invoke and sends Session Establish to the remote Container B, which then reserves the corresponding Applications inside Assemblage B by Pre-invoke.

2)	The outcome of the pre-invoking Applications in Assemblage B is now sent from Container B to Container A as #07 message “Response to Session Establish” instead of “Response to Pre-invoke Applications.”

# 3.	Application handover

Here the application handover is meant to be switching from one Application to another in the same Assemblage during a multimedia communication session.

A communication scenario and corresponding information flow with associated descriptions are contained in Annex B. Remaining issues are also listed there. Compared to the previous version in AVD-4096[4], the following changes have been made:

1)	The Interface for transferring a large volume of Application data in #110 is changed from C to K. This change is based on the error-free consideration of Interface K at large[7] as well as the proposal in AVD-4132[8] even if Interface C or Interface K is suggested for this purpose[6].

2)	Indication of “successful Application handover” is added as a new step #113. This indication ensures that any failure of handover keeps the existing Application to remain in control of the Application session[6].

3)	Step #114 and subsequent steps are renumbered accordingly.

# 4.	Conclusion

This Contribution has updated the AMS information flows for discussion. It is hoped that they would assist the progress of the AMS architecture study and the work of designing necessary protocols over various interfaces.

# References

[1]	Waseda University: Information flow of AMS, Study Group 16 Contribution 543, March 2011

[2]	Waseda University: An update of the AMS information flow, AMS-0029, 20 April 2011

[3]	Waseda University: Invocation of Applications, AMS-0031, 1 June 2011

[4]	Waseda University: AMS information flow - Application handover case, AVD-4096, July 2011

[5]	Cisco Systems Inc.: AMS signalling, AVD-4133, July 2011

[6]	WP 2/16 Chairmen: Report of Questions 2, 3, 4, 5, 12, 21, 22, and 25/16 Rapp. Group meeting (Andover, MA, USA, 18-22 July 2011), Q12 report in §6, TD 606/WP2, July 2011

[7]	Waseda University: Considerations on AMS Interface K, Study Group 16 Contribution 675, November 2011

[8]	Cisco Systems: Capability Exchange within AMS, AVD-4132, July 2011

NOTE: AVD-numbered documents are available at http://ftp3.itu.int/av-arch/avc-site/2009-2012 and AMS-series documents are available at http://ftp3.itu.int/av-arch/avc-site/2009-2012/‌AMS_emeetings.

Annex A
Information flow for regular communication

## A.1	Communication scenario

A basic multimedia communication scenario is considered where three Applications (1: audio, 2: whiteboard and 3: video) are simultaneously involved.

## A.2	Information flow diagram and descriptions

An information flow corresponding to the above communication scenario is shown in Figure A.1. Some descriptions are given to the step indicated by the number with "#".

Figure A.1 – Information flow for AMS regular multimedia communication

Each information flow is numbered as #nn and detailed in the following:

#01	Application registration – Each Application registers to Container in advance of a multimedia communication session. Application description is of “higher level” such as Application type. It is assumed that the Container is always beside the human user. Container may reside in a handheld device or an Application device.

#02	Container registration – Container registers to Service Node for the use of network resource. Though Figure A.1 indicates SN (Service Node) as a single device, SN may also be a federation of multiple distributed devices.

#03	Notification of Application availability – Each Application notifies Container of its availability. In case it is involved in a certain communication, it can not be used for any other communication.

#04	Presentation of Applications – Based on the registration and status information of each Application, Container provides the human user with information on available Applications and their characteristics. It may be an Application by Application indication or pre-defined Application set (e.g. videoconferencing) indication.

#05	Communication request – The human user requests a multimedia session of his/her choice to the Container.

#06	The calling Container A pre-invokes Applications, which the human user has designated, at the local Assemblage. To the remote Container B, Container A sends “Establish Session” including requested Applications. Container B extends this Establish Session to relevant pre-invoking of the Applications under its control. Each Application returns an ACK or a NACK to the pre-invoke message.

#07	The remote Container B collects the pre-invoke responses from remote Applications and conveys them to the local Container A by the “Response to Establish Session” message.

#08	The local Container A decides a set of Applications to be invoked for the communication session.

#09	The Container A invokes that set of Applications at both local and remote Assemblages. The remote Assemblage B extends this invocation message to the remote Applications and returns corresponding Response to Container A.

#10	If some of the pre-invoked Applications become unnecessary due to unavailability of matching Applications, the local Container A sends “cancel pre-invoke” to those Applications for releasing reserved resources.

#11	Capability negotiation – Each Application negotiates its “lower level” detailed capabilities (type of codecs, picture resolution, sound bandwidth, etc) with the remote peer Application and decides operational parameters. It is noted that the Container basically relays messages between local and remote Applications.

#12	Request of network resource for media and control flows – Each Application requests network resources necessary for the media and control flows to the network (Service Node) via Container A and establishes relevant channels. SN conveys this information to Container B for user friendly alerting (see #14 below).

#13	Media and control flows – They are carried directly between local and remote Applications through the channels established by the #12 process.

#14	Alerting the remote user – Once all the media flows for the multimedia communication have become ready, the remote end user is alerted[A.1].

#15	Response – The Human user at the remote end responds to the alerting.

#16	Multimedia communication – The Human users communicate each other via the network by using a desired combination of multiple media.

## A.3	Remaining issues

#06	Some Container to Container messages like this "Establish Session" signalling may go through SN to allow policy control as in the H.323 GK routed model. This aspect is yet to be studied.

#06 - #07 The notion of “application set”, which is defined in draft H.325[A.2] and discussed in AMS-0015a[A.3], need be further studied and the information flow need be updated.

#12-1 Request of media and associated control channels assumes that the network resource is reserved for both directions. Further study is needed if each direction needs its own request.

#12-2	 In case NAT/firewall traversal is involved, more signalling information exchange would be required among AMS entities. This aspect is yet to be studied.

#14	Alerting the remote user could be by use of timer after the network resource signalling in #12, or by an explicit signalling from remote end Applications to Container B as a result of successful media flow establishment in #12. This aspect is yet to be studied in the light of distributed nature of the AMS Assemblage. It is also noted that for some Applications like “flashing lamp” to know that another Application is in an alerting state we may need “subscribe/notify” functionality, but this aspect is yet to be studied.

## References

[A.1]	Recommendation ITU-T H.460.11 (03/2004) “Delayed call establishment within H.323 systems”

[A.2]	Updated draft of new H.AMS “AMS: Advanced multimedia systems for next generation and other packet-switched networks”, AMS-0015a, December 2009

[A.3]	Cisco Systems: AMS Applications, Application Capabilities, and Capability Advertisement, AMS-0015a, March 2010

Annex B
Information flow for application handover

## B.1	Communication scenario

Based on the regular communication in Annex A, the following case is considered here:

|  |  | Assemblage A Assemblage B |
| --- | --- | --- |
|  | Initial communication: | Application 3 - Application 3 |
|  | is handed over to: | Application 4 - Application 3 |

During the regular communication as shown in Annex A is ongoing, Application 3 (video) of Assemblage A is handed over to Application 4 (video). Application 3 of Assemblage B continues video communication even if the corresponding Application of Assemblage A is switched to Application 4.

## B.2	Information flow diagram and descriptions

An information flow corresponding to the above communication scenario is shown in Figure B.1. The information flow for Application handover parallels with that of the regular communication in Annex A as much as possible. Some descriptions are given to the step indicated by the number with "#".

Figure B.1 – Information flow for AMS application handover

Each information flow is numbered as #1nn and detailed in the following:

#100	Application registration – In advance of the handover, Application 4 of Assemblage A (denoted "4/A" here) registers to the Container.

#101	Notification of Application availability – Application 4 notifies Container of its availability.

#102	Presentation of Application 4 – Based on the registration and status information of Application 4, Container provides the Human user with additional information on available Applications and their characteristics.

#103	Handover request – Human user of Assemblage A requests an Application handover from 3/A to 4/A. Here the Human user requests the Application handover. It is noted that “There might be instances where the user's Container is provisioned to automatically perform a handover (e.g. switching from mobile voice to Telepresence when coming into contact with the user's Telepresence system).”[B.1]

#104	Pre-invocation of Application 4 – Based on the Human user’s request, Container A reserves Application 4 with an indication of handover from Application 3.

#105	Application 4/A returns acknowledgment to Container A.

#106	Handover notification – Container A notifies Application 3 of handover to Application 4.

#107	Handover negotiation (local) – Application 3 negotiates with Application 4 about the Application handover possibility. The signalling for this purpose is carried via Container A.

#108	Handover negotiation (with the remote end) – Application 3/A negotiates with Application 3/B about the Application handover to Application 4/A at the local end and gets agreement from Application 3/B.

#109	Handover initiation – Application 3/A initiates the Application handover to Application 4/A. This signalling includes information on the existing communication instance between Applications 3/A and 3/B. The signalling for this purpose is carried via Container.

#110	Transfer of Application data from Application 3/A to 4/A if necessary. This transfer is carried out through Interface K.

#111	Capability negotiation  – Application 4/A negotiates its “lower level” detailed capabilities (type of codecs, picture resolution, sound bandwidth, etc) with Application 3/B and decides operational parameters.

#112	Request of network resource for media and control flows – Application 4/A requests network resources necessary for the media and control flows to the network (Service Node) via Container A and establishes relevant channels. SN conveys this information to Container B for user friendly alerting (see #115 below).

#113	Application 4/A notifies Container A of the successful Application handover.

#114	Media and control flows – They are carried directly between local Application 4/A and remote Application 3/B through the channels established by the #112 process.

#115	Alerting the remote user – Once the media flow for handed over Application has become ready, the remote end Human user is alerted.

#116	Response – The Human user at the remote end responds to the alerting. Then the video is connected between Applications 4/A and 3/B.

#117	Release of original Application 3/A – Container A releases Application 3/A at some point of time.

#118	Release of network resource – Application 3/A requests the network of releasing network resource established for its media and control flows. SN conveys this information to Container B.

#119	Closing the media and control channels – Based on the signalling in #118, all the channels relevant to Application 3/A are closed.

## B.3	Remaining issues

#110	It is assumed that there is no admission control or bandwidth management entity inside the Assemblage A and the media flow is directly established toward the destination Application. However, even if Applications are within the same Assemblage, data transfer between Applications may traverse one or more service provider networks. As an example, between an Application on a mobile phone and an Application in a home, the Application data may transfer over a 4G wireless connection to the mobile operator and then over a broadband connection back to the Application within the home[B.2]. In that scenario, a request for network resources may be necessary and signalling through Interface A and/or B is required. This aspect is yet to be studied.

#115	The issue regarding #14 in Annex A is also applicable here.

#117	An exact timing to release Application 3/A that has become unnecessary is yet to be studied.

## References

[B.1]	Q12/16 Rapporteur: Minutes of the electronic meeting on 2 November 2010,  TD 486/WP2, §5.3, March 2011

[B.2]	WP 2/16 Chairmen: Report of Questions 2, 3, 4, 5, 12, 21, 22, and 25/16 Rapp. Group meeting (Andover, MA, USA, 18-22 July 2011), Q12 report in §6.4, TD 606/WP2, July 2011

__________________