| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4252 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Brisbane, AUS, 24 - 28 September, 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.LOOPB: Loopback of TCP media traffic |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution discusses loopback of TCP media traffic.

Discussion

There are multiple options for TCP loopback (see also copied Fig. 1 from H.248.LOOPB at next page). The options are primarily related to the protocol layer considered for loopback, and secondly due to the connection-oriented nature of TCP. An H.248 TCP termination (or stream endpoint (SEP)) may be involved in TCP header processing, TCP connection state handling, etc, - dependent on the applied TCP mode of operation. How should be MG interpret loopback request for TCP.

The topic as such is within the scope of H.248.LOOPB, hence we are proposing to add a living list for collecting questions for clarification. Another option could be an assignment to work item H.248.TCP (see AVD-4296). However, if there would be generic protocol semantics of all type of connection-oriented IP transport protocols, then H.248.LOOPB might be better suited than H.248.TCP.

Figure 1/H.248.LOOPB – Protocol layer-dependent Loopback (media) path options
(Loopback path option for TCP highlighted)

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against AVD-4242, the input draft to this Q.3/16 meeting.

[Begin Proposed Correction]

Appendix x
Living List – Loopback of TCP media traffic 
or other connection-oriented IP transport protocols

{Editor's note: this item was submitted to the 2012-09 meeting.}

Discussion

The semantic of H.248 LoopBack is ambiguous in case of TCP bearer traffic, perhaps as well for other connection-oriented IP transport protocols (such as SCTP). This Appendix provides a list of items for clarification:

| Item |  | Description | Comments |  |
| --- | --- | --- | --- | --- |
| 1 |  | Fig. 1 indicates protocol layer-dependent Loopback (media) path options. Loopback of TCP media traffic could be interpreted at - L3 (IP layer), i.e., transparent loopbacking of entire TCP packets; - L4 (TCP layer), i.e., what exactly? - L7 (Application layer), i.e., if “TCP media” would be understood as “application data”; but how would a loopback packet look like exactly? Either there’s only a single protocol layer for loopback, or there are multiple options. We need in any case a description of the packet format at the loopback interface. |  |  |
| 2 | Fig. x1 illustrates a scenario with an established TCP bearer connection (Conntext C1 with two Terminations IP1 and IP2). Both Terminations are in StreamMode “SendRecv”. The MGC is requesting now a Loopback at IP1, thus changing the StreamMode settings from “SendRecv” to “LoopBack”, and back again to “SendRecv” at a later point in time. The original TCP connection shouldn’t be disrupted by such a StreamMode transitioning in our understanding. If this is not a valid network use case, then we should explicitly provide guidelines not to use LoopBack for active TCP connections. If this is a valid scenario, then we need to describe what happens with the interrupted TCP connection (besides the question of item 1). |  |
| 3 | The “StreamMode-controlled” TCP loopback (Fig. x.1) could be effectively emulated with a Context topology, see the “Topology-controlled” TCP loopback option in Fig. x.2. If the StreamMode-based Loopback would be too controversial (see items 1 and 2), then the Topology-based method might be a recommended solution? Are both options really functionally equal? Aspects of the “Topology-controlled” TCP loopback option: - Termination IP3 could use a different protocol stack than IP1 (and IP2), e.g. a tunnel based stack (similar than IETF media loopback for RTP) - the TCP connection across IP1 and IP2 could be Context-internally continued or interrupted |  |

Figure x.1 – TCP loopback – Option “StreamMode-controlled”

Figure x.2 – TCP loopback – Option “Topology-controlled”

[End Proposed Correction]

____________________