| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4271 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Brisbane, AUS, 24 - 28 September, 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.Supp.Prio: Editorial improvements for Appendix II |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes updates to Appendix II.

Discussion

There’s a pending action in revising the text from a contribution-style format towards a more general description.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 794 (WP 2/16), the output draft from last Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

Appendix II

Potential use cases with IEPS call indicator support

## II.1	Overview

The following use cases may be principally identified concerning H.248 IP-IP gateway types (no exhaustive list):

Table II.2 – Overview use cases

Potential use cases with IEPS call indicator support

Use case #A: support of multiple, different IP QoS architectures

Use case #B: IP bearer connection traversing different IP QoS architectures

Use case #C: Priority treatment with more than 16 priority levels

Use case #D: Network monitoring of signalling traffic (here: H.248)

Use case #E: National regulations which mandate the use of both, IEPS call and priority indicators

Use case #F: heterogeneous vendor landscape

others?

A crucial location of such an H.248 gateway is the edge of an administrative domain, such as the peering point between different network operators. Such an interconnection scenario needs to consider a wide spectrum of possible network architectures and technology support due to different preferences and/or network status of individual operators.

Figure 1 (in clause 5) summarizes priority traffic handling by H.248 entities. Any kind of traffic priority indication sent from the MGC to the MG leads to correspondent actions in one or multiple functional areas.

## II.2	Use case #A: support of multiple, different IP QoS architectures

This use case focuses on bearer network characteristics (so called area “5” in Figure 1) by considering support of multiple, different IP QoS architectures (see Figure II.1):

Figure II.1 – Use case #A: support of multiple, different IP QoS architectures

The assumption is that the stage 2 requirement could be satisfied by dedicated QoS support for IEPS/ETS traffic as a stage 3 solution. This implies a mapping of IEPS/ETS information at call control signalling level to appropriate “H.248 means” (see also Figure. II.1).

There are two basic solutions, - according different mapping strategies -, outlined in Figure. II.1:

- Solution A.1:

  - the MGC is responsible for all kinds of QoS mappings, thus a so-called technology-dependent decision point (TDDP), also known as bearer-dependent decision point (see clause 3.2.1);

  - the stage 2 requirement could already be supported by H.248 signalling elements for “QoS technologies”;

  - the ContextAttribute IEPS call indicator may not be needed at all;

- Solution A.2:

- the TDDP responsibility is delegated to the slave entities MG;

- the MGC would be thus acting as so-called technology-independent decision point (TIDP), also known as bearer-independent decision point (see clause 3.2.2);

- thus, a generic mapping between call control signalling indicators and H.248 ContextAttributes would be possible.

Conclusions:

- Option A.2 looks attractive from MGC perspective, however, existing profile specifications assign the TDDP to the MGC (due to the “master” attribute of a MGC, see also clause 3.2.1);

- thus, starting point for stage 3 would be the assumption of a “QoS technology aware” MGC (i.e. option A.1)

- any IEPS call indicator support isn’t straight forward to justify.

## II.3	Use case #B: IP bearer connection traversing different IP QoS architectures

The next use case is similar as scenario #A, but focuses on the interworking aspect: the end-to-end IP bearer connection for IEPS/ETS traffic traverses different IP QoS architectures (Fig. II.2). Thus, the two H.248 IP stream endpoints would then have different protocol stacks and/or different policy enforcements for IEPS/ETS support, leading to additional mapping complexity as in comparison to #A.

Figure II.2 – Use case #B: IP bearer connection traversing different IP QoS architectures

Conclusion:

- the discussion (for the indicated solutions B.1 and B.2 in Figure II.2) is fairly similar as in use case #A.

## II.4	Use case #C: Priority treatment with more than 16 priority levels

The codepoint space would be exhausted in network environments which request more than 16 priority levels (due to existing protocol design of [ITU-T H.248.1]). The “IEPS/ETS service” could not be emulated (“the option B in clause I.2”) by reserving a particular H.248 priority indicator codepoint exclusively for IEPS/ETS calls.

Figure II.3 – Use case #C: Priority treatment with more than 16 priority levels

Conclusions:

- there would be just one solution feasible (C.1) by support of the IEPS call indicator.

- further, there is also not yet any stage 2 requirement announced requesting  more than 16 priority levels.

## II.5	Use case #D: Network monitoring of signalling traffic (here: H.248)

Figure II.4 illustrates a (theoretical) network monitoring scenario, based on policy rules, - such as used by deep packet inspection (DPI), traffic detection functions (TDF) or intrusion detection systems (IDS) -, for detection of application “IEPS/ETS service” in network control plane traffic.

Figure II.4 – Use case #D: Network monitoring of signalling traffic (here: H.248)

Conclusions:

- No further consideration is required since such a hypothetical monitoring service should not place any explicit requirements on the signalling.

## II.6	Use case #E: National regulations which mandate the use of both, IEPS call and priority indicators

Regulation authorities could explicitly mandate support of all ContextAttributes for national solutions.

Conclusion:

- no further consideration here because a national (or local) H.248 profile would/could be derived from international (or global) H.248 profile specifications.

## II.7	Use case #F: heterogeneous vendor landscape

Also not any further elaboration of this use case, because this is rather a network operator topic and beyond the scope of SDOs.

## II.8	Summary and conclusions

The stage 2 requirement for IEPS/ETS support may be basically satisfied by two stage 3 solutions: with and without IEPS call indicator capability, called native (A) and emulation (B) approach in clause I.3.

A number of practical and more theoretical use cases are discussed in this Appendix. It might be concluded that t

he native approach could be beneficial in two scenarios:

- A.2: MGC as technology-independent decision point (TIDP) when IEPS/ETS support implies a dedicated QoS technology; and

- C.1: exhausted codepoint space due to unavailable priority level codepoints for additional IEPS/ETS indication.

However, the very majority of use cases could be already solved by an emulation approach.

[End Proposed Correction]

____________________