| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4309 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Brisbane, 24 - 28 September 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | NEC |  |  |  |
| Title: | F/H.TPS-Arch: A default harmonized rendering order |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes a default harmonized order when rendering views of endpoints.  The aim is to make the same “who is next” certain among endpoints. The harmonized rendering order is effective for a roll call including brief self-introduction.  The order is based on the time connected to an MCU used for the conference and the MCU informs the same order to all of endpoints.  Each endpoint renders endpoint-views from left edge to right, based on the order informed.

# Introduction

A roll call including brief self-introduction is often done in a meeting.  And the roll call is usually done by seating order.  It is no problem in the F2F meeting because “who is next” is visible.
On the other hand, it is difficult to determine “who is next” in a voice (invisible) conference.

# Discussion

Participants of video conference are visible.  However, it is difficult to see “who is next”, if rendered views of multiple remote endpoints are different endpoint-by-endpoint.

Special information of multi-screens at one remote endpoint will be distributed, but it is not enough to harmonize rendering order among multiple endpoints.

# Proposal

A proposal is to specify default rendering order for a view of continuous presence.  The order is determined by the incoming/joining order to the MCU used at the point of time. And the order is informed to all of endpoints.  Each endpoint renders based on the order, from left edge to right or upper left edge to right.

Note: After the roll call, each endpoint can arrange any order at each endpoint.

__________________