| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4317 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Brisbane, 24 - 28 September 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Applied Communication Sciences and AT&T |  |  |  |
| Title: | Proposed Modifications to Draft H.248.82 in Support of ECN and ETS Interaction |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes modifications to draft H.248.82 in support of ECN and ETS interaction.

# Introduction

Draft H.248.82 was progressed at the April 2012 Q.3/16 meeting in Geneva.  The output document is contained in TD 761 R1 (WP 2/16).  This contribution proposes modifications to draft H.248.82 to include normative text in support of ECN and ETS interaction in draft H.248.82.

# Discussion

Draft H.248.82 (ECN) Appendix II provides the interaction between ECN and ETS.  In short, it concludes that depending on whether ETS traffic is significant versus limited, there could be benefits in either enabling or disabling ECN for ETS traffic.  The following observations were made:

It is generally expected that enabling ECN for non-ETS calls, which normally constitute the biggest part of the served traffic, will have a positive service impact on ETS calls, due to the reduction of bandwidth potentially yielded by non-ETS calls.

Enabling ECN for ETS calls is subject to the following considerations:

- ECN-enabled ETS calls: This approach may have a positive service impacts on ETS calls, and thus be justified, when any of the following conditions apply:

- Amount of served ETS traffic is significant;

- ETS terminals have the capability to change to lower codec modes in response to ECN congestion indications, thus lowering their bandwidth consumption.

- ECN-disabled ETS calls: This approach may be justified (i.e., there is no obvious positive service impact on ETS calls) when any of the following conditions apply:

- Amount of served ETS traffic is much less than the amount of served non-ETS traffic;

NOTE - This condition may apply to the vast majority of the network scenarios.

- No support of sender-driven congestion control for terminals in “ETS” mode.

Given the above observations and the need to document normative text on the ECN and ETS interaction (which is currently missing in draft H.248.82), this contribution proposes text in support of ECN and ETS interaction for inclusion in draft H.248.82.

# Proposal

## Proposal 1

Under Clause 2 (References), add the following references:

[ITU-T E.107]	Recommendation ITU-T E.107 (2007), Emergency Telecommunications Service (ETS) and interconnection framework for national implementations of ETS.

[ITU-T H.248.81]	Recommendation ITU-T H.248.81 (05/11), Gateway control protocol: Guidelines on the use of the International Emergency Preference Scheme (IEPS) call indicator and priority indicator in ITU-T H.248 Profiles.

## Proposal 2

Under Clause 3.1 (Terms defined elsewhere), add the following definition:

3.1.3	Emergency Telecommunications Service (ETS): [ITU-T E.107] A national service providing priority telecommunications to the ETS authorized users in times of disaster and emergencies.

## Proposal 3

Under Clause 4 (Abbreviations and acronyms), add the following acronym:

ETS	Emergency Telecommunications Service

## Proposal 4

Include the following new Clause 7.x:

7.x	ECN and ETS Interaction

For a call/session that is ETS [ITU-T E.107] marked [ITU-T H.248.81], the MGC may or may not request the MG to enable ECN based on service provider’s local policy. If ECN is disabled, the MG when acting as an ECN endpoint shall not apply congestion control procedures as outlined in clause 7.3.3/[IETF ECN] to the ETS traffic, if congestion is experienced. Whether or not ECN and ETS can be applied to the same Context/Termination is network dependent.  For details on the possible interaction between ECN and ETS, see Appendix II.

Note: This is based on analytical analysis and a more detailed performance study is required for quantitative consideration.

__________________