| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4345 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Brisbane, 24 - 28 September 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ETRI |  |  |  |
| Title: | Additional A/V parameter for H.TPS-AV |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes an additional A/V parameter to indicate the location of active talker in a telepresence session for draft F/H.TPS-AV.

- Discussion

The draft F/H.TPS-AV specifies a number of A/V parameters to describe the characteristics of audio and video streams for the telepresence system. After analysing those parameters specified in F/H.TPS-AV, it is needed to add some additional parameters related to the capabilities of endpoints.

We proposed a parameter to describe the location of speaker. This parameter can be used to identify who is talking actively in a telepresence session. This parameter is useful when multi channels stereo sound system are used in order to choose an appropriate sound channel that it is like coming from the speaker.

- Proposal

This contribution proposes to add a new A/V parameter to indicate the location of speaker to be added in section 7.2.

========  begin =====

## 7.2.x Location of speaker

| Identity: | LocSpeaker |  |
| --- | --- | --- |
| Description: | This parameter indicates the location of active speaker |  |
| Format: | Numeric(X, Y, Z, S) |  |
| Possible Values: | -X, Y, Z indicate Cartesian coordinate of speaker, S is sound volume of speaker Where X, Y, X and S are numbers |  |
| Default: | - |  |
| Reference: | - |  |
| Interface: | - |  |
| Supplementary Information: |  | This parameter indicates the active speaker among participants to select an appropriate sound channel for the renderer. |

_________________