| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4347 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Brisbane, 24 - 28 September 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Applied Communication Sciences |  |  |  |
| Title: | Modifications to Draft H.248.82, Appendix II |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes modifications to draft H.248.82, Appendix II.

# Introduction

Draft H.248.82 was progressed at the April 2012 Q.3/16 meeting in Geneva.  The output document is contained in TD 761 R1 (WP 2/16).  This contribution proposes modifications to draft H.248.82, Appendix II.  Text changes are shown with revision marks against TD 761 R1 (WP 2/16) text.

# Proposal

It is proposed to accept the changes (marked with revision marks) provided below in the baseline document.

Appendix II

Possible interaction between ECN and ETS

(This appendix does not form an integral part of this Recommendation)

## II.1	Introduction

This interaction issue between ECN and Emergency Telecommunication Service (ETS) can be summarised by the question whether ECN could positively or negatively influence ETS traffic (“Can ECN improve or degrade ETS traffic?”). Any (individual call) enabled congestion control (for IP bearer traffic) definitely has a direct impact on the IP application/service (e.g., ETS). Whether the impact is negative or positive, from perspective of the perceived/experienced quality (i.e., QoE) by the ETS user , is firstly not straightforward. Disabled congestion control also affects the ETS quality since (temporal and/or spatial) congestion situations in a network normally decrease the grade of service (GoS) of the entire IP bearer traffic (“of a congested network area”), under the assumption of a best effort infrastructure given ETS traffic shares at least some of the same infrastructure resources as non-ETS traffic.

Some use case examples are evaluated below.

## II.2	Use case examples

The examples provide qualitative considerations. Complementary quantitative evaluations would rather be the subject of network engineering and associated performance investigations (e.g., traffic simulation studies).

### II.2.1	Example 1: Networks with "significant ETS traffic"

There is basically a mix of ETS and non-ETS traffic. In this case, it is assumed that there is significant ETS traffic.

Evaluation:

An ECN-enabled ETS call could experience a better QoE/QoS than an ECN-disabled ETS call due to a positive effect of an enabled congestion control. The positive effect is in the form of reduced load by each entity that responds to the congestion notification.

NOTE - The portion of ETS traffic, under normal load conditions, is typically limited in real networks. See e.g., [b-ETSI TS 122 153] for MPS traffic in 3GPP networks (“… MPS traffic volumes should be limited (e.g. not to exceed a regional/national specified percentage of any concentrated network resource, …), so as not to compromise this function.”).

### II.2.2	Example 2: Terminal with two-mode-multirate codec

A terminal with a generic two-mode-multirate codec provides following two operational modes:

- codec 1 "high quality mode" with 60 kbit/s transport capacity and

- codec 2 "good quality mode" with 20 kbit/s transport capacity requirement.

The codec furthermore supports a sender-driven congestion control:

- any notified network congestion would lead to a transition to the 20 kbit/s mode (“fallback to lower transport capacity”).

Consider the following example scenario:

  - A call starts in 60 kbit/s mode and experiences (in a later point in time) a mean packet loss ratio of e.g., 5 % due to network congestion

  - Given ECN is enabled (negotiated during call setup), network congestion leads to ECN marking, i.e., IP packets are marked as supporting ECN

  - Received ECN indications at sender side activate the sender-driven congestion control method

  - There is then a transition to the 20 kbit/s mode

  - This mode then experiences e.g., a mean packet loss of 0.1%, hence a lower information loss

  - There are consequently two situations with different QoE as perceived by the user, labelled as QoE60 and QoE40:

    - the QoE60 in the “high quality” codec mode could actually be worse than the QoE40 in the “low quality” codec mode under network congestion because the GoS40 would be better than the GoS60 (primarily due to relations “IPLR40 << IPLR60 and IPDV40 << IPDV60”).

Evaluation:

An ECN-enabled ETS call could experience a better QoE/QoS than an ECN-disabled ETS call due to the possible positive effect that the lower transport quality may receive a better GoS.

### II.2.3	Example 3: Network with "sparse ETS traffic"

A network situation where there is a low portion of ETS traffic, e.g., when “ETS traffic is much less than non-ETS traffic”.

Evaluation:

An ECN-disabled ETS calls could benefit from the ECN-enabled non-ETS calls due to a potential positive effect of more available transport capacity from non-ETS traffic and a reduced overall congestion level.  Congestion control could reduce the consumed bitrate by non-ETS calls in the form of reduced load by each entity that responds to the congestion notification.

### II.2.4	Example 4: Limited terminal capability

Assume a hypothetical IP terminal equipment with ECN sender-driven congestion control, but without any signalling capability of ECN information. The implemented IP protocol stack may provide ECN support at all possible protocol layers, but the application level logic (“ECN user instance”) may not be available, which does not allow to process ECN information at the IP interface.

Such a terminal limitation prevents a closed control loop (see Fig. 1), since either  the congestion notification path would be interrupted, or the feedback path, or both.

Evaluation:

Disabling ECN for ETS would not impact in such a situation since ECN marking wouldn't  provide any benefit. Therefore, ECN-disabled ETS calls might be a basic strategy.

NOTE -  Further study to evaluate the effects of ECN on the served ETS video calls where ECN could be enabled on the video stream and disabled on the voice stream is needed.

## II.3	Observations

It is generally expected that enabling ECN for non-ETS calls, which normally constitute the biggest part of the served traffic, will have a positive service impact on ETS calls, due to the reduction of bandwidth potentially yielded by non-ETS calls.

Enabling ECN for ETS calls is subject to the following considerations:

- ECN-enabled ETS calls: This approach may have a positive service impact on ETS calls, and thus be justified, when any of the following conditions apply:

- Amount of served ETS traffic is significant;

- ETS terminals have the capability to change to lower codec modes in response to ECN congestion indications, thus lowering their bandwidth consumption.

- ECN-disabled ETS calls: This approach may be justified (i.e., there is no obvious positive service impact on ETS calls), when any of the following conditions apply:

- Amount of served ETS traffic is much less than the amount of served non-ETS traffic;

NOTE -- This condition may apply to the vast majority of the network scenarios.

- No support of sender-driven congestion control for terminals in “ETS” mode.

__________________