| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | TD-27 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 2/16 | Brisbane, 24 - 28 September 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | GNU Gatekeeper Project |  |  |  |
| Title: | Proposed Amendment to H.323 |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This amendment formalizes the widely implemented call transfer mechanism in H.323 that relies upon the FACILITY message with a reason of “callForwarded”.

Recommendation ITU-T H.323

Packet-based multimedia communications systems

Amendment 1

# Changes to Clause 8.1.10

8.1.10 Call forwarding and call transfer

An endpoint wishing to forward an unestablished call or transfer an established call to another endpoint may issue a Facility message with FacilityReason callForwarded indicating the address of the new endpoint. The endpoint receiving this Facility indication should send a Release Complete and then restart the Phase A procedures with the new endpoint.

__________________