| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11 – 18 Jan. 2014 |  |  |  | Document: JCT3V-G_Notes_d5 |
| --- | --- | --- | --- | --- |
| Title: | Meeting report of the seventh meeting of the Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V), San José, US, 11 – 18 Jan. 2014 |  |  |  |
| Status: | Report Document from Chairs of JCT-3V |  |  |  |
| Purpose: | Report |  |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communications Engineering RWTH Aachen University Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 +49 241 80 27671 ohm@ient.rwth-aachen.de |  |
| Source: | Chairs |  |  |  |

_____________________________

- Summary

The Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its seventh meeting during 11-18 Jan. 2014 at the Double Tree Inn in San José, US. The JCT-3V meeting was held under the chairmanship of Dr Jens-Rainer Ohm (RWTH Aachen/Germany) and Dr Gary Sullivan (Microsoft/USA). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 1.13 of this document.

The meeting was mainly held in a “single track” fashion, with few breakout activities (as documented in this report) held in parallel. Several plenary sessions were chaired by Dr Anthony Vetro, particularly in cases when neither of the two chairs was able to attend due to commitments in other groups or joint meetings. It was usually avoided to make decisions which might cause objections in such cases.

The JCT-3V meeting sessions began at approximately 900 hours on Saturday 11 Jan. 2014. Meeting sessions were held on all days until the meeting was closed at approximately 13 hours on Friday 18 Jan. 2014. Approximately XXX people attended the JCT-3V meeting, and approximately XXX input documents were discussed. The meeting took place in a collocated fashion with a meeting of WG11 – one of the two parent bodies of the JCT-3V. The subject matter of the JCT-3V meeting activities consisted of work on 3D extensions of the Advanced Video Coding (AVC) and the High Efficiency Video Coding (HEVC) standards.

The AHG on 3D High-level Syntax (AHG7) had met the 2 days before the JCT-3V meeting (9-10 Jan.), to discuss HEVC HLS input contributions jointly with JCT-VC experts. The outcome of this joint meeting, which continued after the beginning of the JCT-3V meeting, is documented in the JCT-VC report JCTVC-P1000 (insert link).

The primary goals of the meeting were to review the work that was performed in the interim period since the sixth JCT-3V meeting in producing

- Draft Text 8 of 3D-AVC (ISO/IEC 14496-10:2012/FDAM3 – to be integrated and balloted in new edition)

- Test Model 8 of 3D-AVC

- Draft 1 of 3D-AVC reference software (ISO/IEC 14496-5:2001/PDAM 35 –for ballot)

- Draft Text 6 of MV-HEVC (Study of ISO/IEC 23008-2:200X/DAM 2)

- Test Model 6 of 3D-HEVC and  MV-HEVC, and associated software

- Draft 2 of 3D-HEVC text specification

- Draft 5 of MVC plus Depth Conformance (Study of ISO/IEC 14496-4:2004/DAM 41 – not for ballot)

- Draft 3 of MVC plus Depth Reference Software (ISO/IEC 14496-5:2001/DAM 33 –for ballot)

- Draft 3 of 3D-AVC Conformance (ISO/IEC 14496-4:2004/PDAM 43 –for ballot)

- Draft 4 of Multi-resolution Frame Compatible Stereo (ISO/IEC 14496-10:2012/FDAM5 – to be integrated and balloted in new edition)

- Draft 2 of MFC Conformance (ISO/IEC 14496-4:2004/DAM 42 – for ballot)

- Draft 2 of MFC Reference Software (ISO/IEC 14496-5:2001/DAM 34 –for ballot)

- 3D Video Subjective Quality Assessment Test plan v2

- Common Test Conditions of 3DV Core Experiments and CE Descriptions.

Furthermore, the JCT-3V reviewed the results from five interim Core Experiments (CE); reviewed technical input documents; produced updated versions of the draft texts, framework descriptions and software implementations of the items above; and planned a new set of Core Experiments (CEs) for further investigation of proposed technology.

The JCT-3V produced 13 particularly important output documents from the meeting: update

- Draft Text 8 of 3D-AVC (ISO/IEC 14496-10:2012/FDAM3 – to be integrated and balloted in new edition)

- Test Model 8 of 3D-AVC

- Draft 1 of 3D-AVC reference software (ISO/IEC 14496-5:2001/PDAM 35 –for ballot)

- Draft Text 6 of MV-HEVC (Study of ISO/IEC 23008-2:200X/DAM 2)

- Test Model 6 of 3D-HEVC and  MV-HEVC, and associated software

- Draft 2 of 3D-HEVC text specification

- Draft 5 of MVC plus Depth Conformance (Study of ISO/IEC 14496-4:2004/DAM 41 – not for ballot)

- Draft 3 of MVC plus Depth Reference Software (ISO/IEC 14496-5:2001/DAM 33 –for ballot)

- Draft 3 of 3D-AVC Conformance (ISO/IEC 14496-4:2004/PDAM 43 –for ballot)

- Draft 4 of Multi-resolution Frame Compatible Stereo (ISO/IEC 14496-10:2012/FDAM5 – to be integrated and balloted in new edition)

- Draft 2 of MFC Conformance (ISO/IEC 14496-4:2004/DAM 42 – for ballot)

- Draft 2 of MFC Reference Software (ISO/IEC 14496-5:2001/DAM 34 –for ballot)

- 3D Video Subjective Quality Assessment Test plan v2

- Common Test Conditions of 3DV Core Experiments.

Moreover, plans were established to conduct XX future CEs in the interim period until the next meeting.

For the organization and planning of its future work, the JCT-3V established XX "Ad Hoc Groups" (AHGs) to progress the work on particular subject areas. The next four JCT-3V meetings are planned for 29 March – 4 April 2014 under WG 11 auspices in Valencia, ES, 3 – 9 July 2014 under ITU-T auspices in Sapporo, JP, 18 – 24 Oct. 2014 under WG 11 auspices in Strasbourg, FR and XX – XX Feb. 2015 under ITU-T auspices in Geneva, CH.

The document distribution site http://phenix.it-sudparis.eu/jct3v/ was used for distribution of all documents.

The reflector to be used for discussions by the JCT-3V and all of its AHGs is jct-3v@lists.rwth-aachen.de.

- Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/ SC 29/ WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JCT-3V are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11.

The Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its seventh meeting during 11-18 Jan. 2014 at the Double Tree Inn in San José, US. The JCT-3V meeting was held under the chairmanship of Dr Jens-Rainer Ohm (RWTH Aachen/Germany) and Dr Gary Sullivan (Microsoft/USA). A couple of sessions were also chaired by Dr Anthony Vetro (MERL/USA).

## Meeting logistics

The JCT-3V meeting sessions began at approximately 900 hours on Saturday 11 Jan. 2014. Meeting sessions were held on all days until the meeting was closed at approximately 13 hours on Friday 18 Jan. Approximately XXX people attended the JCT-3V meeting, and approximately XXX input documents were discussed. The meeting took place in a collocated fashion with a meeting of ITU-T SG16 – one of the two parent bodies of the JCT-3V. The subject matter of the JCT-3V meeting activities consisted of work on 3D extensions of the Advanced Video Coding (AVC) and the High Efficiency Video Coding (HEVC) standards.

The AHG on 3D High-level Syntax (AHG7) had met the 2 days before the JCT-3V meeting (9-10 Jan.), to discuss HEVC HLS input contributions jointly with JCT-VC experts.

Information regarding preparation and logistics arrangements for the meeting had been provided via the email reflector jct-3v@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jct3v-site/2014_01_G_SanJose/.

## Primary goals

The primary goals of the meeting were to review the work that had been performed in the interim period since the sixth JCT-3V meeting in producing

- Draft Text 8 of 3D-AVC (ISO/IEC 14496-10:2012/FDAM3 – to be integrated and balloted in new edition)

- Test Model 8 of 3D-AVC

- Draft 1 of 3D-AVC reference software (ISO/IEC 14496-5:2001/PDAM 35 –for ballot)

- Draft Text 6 of MV-HEVC (Study of ISO/IEC 23008-2:200X/DAM 2)

- Test Model 6 of 3D-HEVC and  MV-HEVC, and associated software

- Draft 2 of 3D-HEVC text specification

- Draft 5 of MVC plus Depth Conformance (Study of ISO/IEC 14496-4:2004/DAM 41 – not for ballot)

- Draft 3 of MVC plus Depth Reference Software (ISO/IEC 14496-5:2001/DAM 33 –for ballot)

- Draft 3 of 3D-AVC Conformance (ISO/IEC 14496-4:2004/PDAM 43 – for ballot)

- Draft 4 of Multi-resolution Frame Compatible Stereo (ISO/IEC 14496-10:2012/FDAM5 – to be integrated and balloted in new edition)

- Draft 2 of MFC Conformance (ISO/IEC 14496-4:2004/DAM 42 – for ballot)

- Draft 2 of MFC Reference Software (ISO/IEC 14496-5:2001/DAM 34 –for ballot)

- 3D Video Subjective Quality Assessment Test plan v2

- Common Test Conditions of 3DV Core Experiments and CE Descriptions.

Furthermore, the JCT-3V reviewed the results from the interim Core Experiments (CE); reviewed technical input documents; produced updated versions of the aforementioned draft texts, framework descriptions, conformance and software implementations of the items above; and planned a new set of Core Experiments (CEs) for further investigation of proposed technology.

HLS (joint with JCT-VC), consideration of benefit of dedicated combined texture & depth coding tools.

## Documents and document handling considerations

### General

The documents of the JCT-3V meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jct3v/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

Document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (rather than as contribution registration and upload times) follow the local time at the meeting facility.

Highlighting of recorded decisions in this report:

- Decisions made by the group that affect the normative content of the draft standard are identified in this report by prefixing the description of the decision with the string "Decision:".

- Decisions that affect the reference software but have no normative effect on the text are marked by the string "Decision (SW):".

- Decisions that fix a bug in the specification (an error, oversight, or messiness) are marked by the string "Decision (BF):".

- Decisions regarding things that correct the text to properly reflect the design intent, add supplemental remarks to the text, or clarify the text are marked by the string "Decision (Ed.):".

- Decisions regarding … simplification or improvement of design consistency are marked by the string "Decision (Simp.):".

- Decisions regarding complexity reduction (in terms of processing cycles, memory capacity, memory bandwidth, line buffers, number of contexts, number of context-coded bins, etc.) … "Decision (Compl.):"

This meeting report is based primarily on notes taken by the chairs and projected (if possible) for real-time review by the participants during the meeting discussions. The preliminary notes were also circulated publicly by ftp (http://wftp3.itu.int/av-arch/jct3v-site/) during the meeting on a daily basis. Considering the high workload of this meeting and the large number of contributions, it should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much discussion of the contributions and discussions as is feasible in the interest of aiding study, although this approach may not result in the most polished output report.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Friday, 3 Jan. 2014. For documents on HEVC high level syntax, if submitted as joint contributions to JCT-VC and JCT-3V, the same deadline had been announced.

Non-administrative documents uploaded after 2359 hours in Paris/Geneva time Saturday 4 Jan. 2014 were considered "officially late".

Most documents in the "late" category were CE reports or cross-verification reports, which are somewhat less problematic than late proposals for new action (and especially for new normative standardization action).

The group strived to be conservative when discussing and considering the content of late documents, although no objections were raised regarding allowing some discussion in such cases.

All contribution documents with registration numbers JCT3V-G0175 and higher were registered after the "officially late" deadline (and therefore were also uploaded late). Some documents in the "G0175+" range include break-out activity reports that were generated during the meeting and are therefore considered report documents rather than late contributions.

Generally, the late document submission situation had improved relative to recent history.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly-accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with this issue was to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This was agreed to be a helpful approach to be followed at the meeting.

The following technical proposal contributions were classified as late either due to late upload or late registration:

- JCT3V-G0XXX (a technical proposal on ...) [uploaded 01-XX]

- ...]

The following other documents not proposing normative technical content were classified as late:

- JCT3V-G0XXX (an input on ...) [uploaded 01-XX]

- ...

Beyond those (approximately XX) in the “G0175+” range, the following cross-verification reports were also uploaded late: JCT3V-G0XXX, ... .

The following document registrations were later cancelled or otherwise never provided or never discussed due to lack of availability or registration errors: JCT3V-G0XXX, ...

Ad hoc group interim activity reports, CE summary results reports, break-out activity reports, and information documents containing the results of experiments requested during the meeting are not included in the above list, as these are considered administrative report documents to which the uploading deadline is not applied.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when sufficient time for studying was given after the upload. Again, an exception is applied for AHG reports, CE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there was some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload are also a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This is especially a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the "comments" field on the web site can be used to indicate what is different in a revision.

"Placeholder" contribution documents that were basically empty of content, with perhaps only a brief abstract and some expression of an intent to provide a more complete submission as a revision, were considered unacceptable and were rejected in the document management system, as has been agreed since the third meeting.

(the following cases did not happen in the current meeting)

The initial uploads of the following contribution documents were rejected as "placeholders" without any significant content and were not corrected until after the upload deadline:

- JCT3V-G0XXX (a contribution of ... , corrected ...)

- ...

A few documents had some problems relating to IPR declarations (missing or excess declarations of contributing companies), inconsistent filenames in the header etc. in the initial uploaded versions. These issues were corrected by later uploaded versions in all cases (to the extent of the awareness of the chairs). In case of JCTVC-G0XXX and ..., the first complete versions appeared only on 01-XX, such that this contribution was also flagged as “late”.

Some other errors were noticed in other initial document uploads (wrong document numbers in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Measures to facilitate the consideration of contributions

For cross-verification contributions, it was agreed that the group would ordinarily only review cross-checks for proposals that appear promising.

When considering cross-check contributions, it was agreed that, to the extent feasible, the following data should be collected:

- Subject (including document number).

- Whether common conditions were followed.

- Whether the results are complete.

- Whether the results match those reported by the contributor (within reasonable limits, such as minor compiler/platform differences).

- Whether the contributor studied the algorithm and software closely and has demonstrated adequate knowledge of the technology.

- Whether the contributor independently implemented the proposed technology feature, or at least compiled the software themselves.

- Any special comments and observations made by the cross-check contributor.

### Outputs of the preceding meeting

The report documents of the previous meeting, particularly the meeting report (JCT3V-F1000), the 3D-AVC draft text 8 (JCT3V-F1002) and test model 8 (JCT3V-F1003), the multi-view MV-HEVC draft text 6 (JCT3V-F1004), the MV-/3D-HEVC test model 6 (JCT3V-F1005) and 3D-HEVC draft text 2 (JCT3V-F1001), the draft 5 of MVC plus depth conformance spec (JCT3V-F1006), Draft 1 of 3D-AVC Reference Software (JCT3V-F1014), Draft 3 of 3D-AVC Conformance (JCT3V-F1009), Draft 3 of MVC+D Reference Software (JCT3V-F1010), Draft 2 of MFC Conformance (JCT3V-F1012), Draft 2 of MFC Reference Software (JCT3V-F1013) and the 3D Video Subjective Quality Assessment Test Plan v2 (JCT3V-F1011), which had been produced in the interim period, were approved. The ATM and HTM reference software packages produced by AHG4 and AHG5 on software development, and the software technical evaluations were also approved.

All output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

The chairs asked if there were any issues regarding potential mismatches between perceived technical content prior to adoption and later integration efforts. It was also asked whether there was adequate clarity of precise description of the technology in the associated proposal contributions.

## Attendance

The list of participants in the JCT-3V meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/ SC 29/ WG 11 (including experts who had been personally invited by the Chairs as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the Chairs.

## Agenda

The agenda for the meeting was as follows:

- IPR policy reminder and declarations

- Contribution document allocation

- Reports of ad hoc group activities

- Reports of Core Experiment activities

- Review of results of previous meeting

- Consideration of contributions and communications on 3D video coding projects guidance

- Consideration of 3D video coding technology proposal contributions

- Consideration of information contributions

- Coordination activities

- Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments, establishment of AHGs, meeting planning, refinement of expected standardization timeline, other planning issues

- Other business as appropriate for consideration

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JCT-3V and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JCT-3V as necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jct3v-site (JCT-3V contribution templates)

- http://www.itu.int/ITU-T/studygroups/com16/jct-3v/index.html (JCT-3V general information and founding charter)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/ SC 29 Procedures)

It is noted that the ITU TSB director's AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

"TSB has reported to the TSB Director’s IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur’s group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation."

The chairs invited participants to make any necessary verbal reports of previously-unreported IPR in draft standards under preparation, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that it is our understanding according to the practices of the parent bodies to make reference software available under copyright license header language which is the BSD license with preceding sentence declaring that contributor or third party rights are not granted, as e.g. recorded in N10791 of the 89th meeting of ISO/IEC JTC 1/ SC 29/ WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software header is currently used in the process of designing the new HEVC standard and for evaluating proposals for technology to be included in this design. Additionally, after development of the coding technology, the software will be published by ITU-T and ISO/IEC as an example implementation of the 3D video standard(s) and for use as the basis of products to promote adoption of the technology. This is likely to require further communication with and between the parent organizations.

To the best of our knowledge, the ATM, HTM and MFC software packages that are used in JCT-3V follow these principles. The view synthesis software used for non-normative post processing is included in the HTM package and also has the BSD header.

## Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jct3v/. Furthermore, the site http://ftp3.itu.int/av-arch/jct3v-site was used for distribution of the contribution document template and circulation of drafts of this meeting report.

Communication of JCT-3V is performed via the list jct-3v@lists.rwth-aachen.de (to subscribe or unsubscribe, go to http://mailman.rwth-aachen.de/mailman/listinfo/jct-3v).

It was emphasized that reflector subscriptions and email sent to the reflector must use their real names when subscribing and sending messages and must respond to inquiries regarding their type of interest in the work.

It was emphasized that usually discussions concerning CEs and AHGs should be performed using the reflector. CE internal discussions should primarily be concerned with organizational issues. Substantial technical issues that are not reflected by the original CE plan should be openly discussed on the reflector. Any new developments that are result of private communication cannot be considered to be the result of the CE.

For the case of CE documents and AHG reports, email addresses of participants and contributors may be obscured or absent (and will be on request), although these will be available (in human readable format – possibly with some "obscurification") for primary CE coordinators and AHG chairs.

## Terminology

Note: Acronyms should be used consistently. For example, “IV” is sometimes used for “inter-view” and sometimes for “intra-view”.

Some terminology used in this report is explained below:

- AHG: Ad hoc group.

- AMVP: Advanced motion vector prediction.

- ATM: AVC based 3D test model

- AU: Access unit.

- AUD: Access unit delimiter.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BoG: Break-out group.

- BR: Bit rate.

- B-VSP: Backward view synthesis prediction.

- CABAC: Context-adaptive binary arithmetic coding.

- CD: Committee draft – the first formal ballot stage of the approval process in ISO/IEC.

- CE: Core experiment – a coordinated experiment conducted between two subsequent JCT-3V meetings and approved to be considered a CE by the group.

- Consent: A step taken in ITU-T to formally consider a text as a candidate for final approval (the primary stage of the ITU-T "alternative approval process").

- CTC: Common test conditions.

- DC: Disparity compensation

- DIS: Draft international standard – the second formal ballot stage of the approval process in ISO/IEC.

- DF: Deblocking filter.

- DMC: Depth based motion competition.

- DMM: Depth modeling mode.

- DRPS: Depth-range parameter set.

- DRWP: Depth-range based weighted prediction.

- DT: Decoding time.

- DV: Disparity vector

- ET: Encoding time.

- HEVC: High Efficiency Video Coding – the video coding standardization initiative under way in the JCT-VC.

- HLS: High-level syntax.

- HM: HEVC Test Model – a video coding design containing selected coding tools that constitutes our draft standard design – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM).

- HTM: HEVC based 3D test model

- IC: Illumination compensation

- IDV: Implicit disparity vector

- IVMP: Inside-view motion prediction (which means motion for depth component is inherited from texture component motion)

- IVRC: Inter-view residual prediction.

- MC: Motion compensation.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/ SC 29, one of the two parent bodies of the JCT-3V).

- MPI: Motion parameter inheritance.

- MV: Motion vector.

- NAL: Network abstraction layer (HEVC/AVC).

- NBDV: Neighbored block disparity vector (used to derive unavailable depth data from reference view’s depth map) and DoNBDV = depth oriented NBDV

- NB: National body (usually used in reference to NBs of the WG 11 parent body).

- NUT: NAL unit type (HEVC/AVC).

- PDM: Predicted Depth Map

- POC: Picture order count.

- PPS: Picture parameter set (HEVC/AVC).

- QP: Quantization parameter (as in AVC, sometimes confused with quantization step size).

- QT: Quadtree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD). Often loosely associated with HE.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- RQT: Residual quadtree.

- SAO: Sample-adaptive offset.

- SEI: Supplemental enhancement information (as in AVC).

- SD: Slice data; alternatively, standard-definition.

- SDC: Segment-wise DC coding.

- SH: Slice header.

- SPS: Sequence parameter set (HEVC/AVC).

- Unit types:

  - CTB: code tree block (synonymous with LCU or TB).

  - CU: coding unit.

  - LCU: (formerly LCTU) largest coding unit (synonymous with CTB or TB).

  - PU: prediction unit, with four shape possibilities.

    - 2Nx2N: having the full width and height of the CU.

    - 2NxN: having two areas that each have the full width and half the height of the CU.

    - Nx2N: having two areas that each have half the width and the full height of the CU.

    - NxN: having four areas that each have half the width and half the height of the CU.

  - TB: tree block (synonymous with LCU – LCU seems preferred).

  - TU: transform unit.

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JCT-3V).

- VPS: Video parameter set.

- VS: View synthesis.

- VSO: View synthesis optimization (RDO tool for depth maps).

- VSP: View synthesis prediction.

- WD: Working draft – the draft HEVC standard corresponding to the HM.

- WG: Working group (usually used in reference to WG 11, a.k.a. MPEG).

## Liaison activity

The JCT-3V did not send or receive formal liaison communications at this meeting.

## Opening remarks

Work has progressed to integrate draft texts of 3D-AVC and MFC, as well as the previous MVC+D amendment into new editions of ISO/IEC 14496-10 (FDIS planned as output of this meeting) and  ITU-T Rec. H.264 (to be delivered during the next few days).

Ballot closing 2014-01-16 for MV-HEVC, too late to progress to FDAM during this meeting.

Various ballots on software and conformance for MVC+D, 3D-AVC and MFC closing between this and the next meeting.

A small decrease in number of document contributions was noted, which is however understandable due to the fact that AVC 3D extensions have come to an end.

## Contribution topic overview

Update The approximate subject categories and quantity of contributions per category for the meeting were summarized as follows.

- AHG reports (section 1.14) (13)

- Project development and status (section 3) XX)

- CE1. Sub-PU processing (section 4.1) (20)

- CE2: Motion prediction for depth coding (section 4.2) (18)

- CE3: Motion prediction for texture coding (section 4.3) (28)

- CE4: Residual prediction (section 4.4) (21)

- CE5: Depth intra modes (section 4.5) (40)

- 3DV standards development (incl. software, conformance) (section 5) (0)

- High-Level Syntax (section 5.5) (43)

- Non-CE technical contributions (section 7) (25)

- Alternative Depth Formats (section 7.2) (1)

- Non-normative Contributions (section 9) (2)

- Withdrawn, unclear allocation (section 10) (1)

NOTE – The number of contributions noted in each category, as shown in parenthesis above, may not be 100% precise.

## Scheduling planning

Scheduling: Generally meeting time was scheduled during 0900 – 2000, with coffee and lunch breaks as convenient. Ongoing refinements were announced on the group email reflector as needed.

Some particular scheduling notes are shown below, although not necessarily 100% accurate:

- Saturday, first day

  - 0900-1115: Opening and AHG report review (Siskiyou)

  - 1145-?: CE review (Siskiyou): CE1, CE2, CE3

  - 1800-?: BoG on ATM software (Carmel)

- Sunday, second day

  - 0900-1300 & 1400-1500: CE review (Siskiyou): CE3(3), CE4, CE5

  - 15-17 (MPEG FTV AHG)

  - if necessary continue CE review afterwards

- Monday, third day

  - 1400-1600: 3.4, 3.6, 9.1

  - 1600-1800: Joint meeting of parent bodies

  - Preparation of viewing

- Tuesday, fourth day:

  - 800-1100: 3.5, BoG reports, 8, 7.3

  - 1100-1300: Joint meeting with JCT-VC on HLS

  - 1400-1800: Revisits, 7.1, 7.2, 7.4 and 3D-only HLS

- Joint meetings:

  - Wed 16-17 with 3DG on commonalities 3DV/AR (Dong, Karsten, Krzysztof)

  - Wed 17-18 with parent bodies on 3D HEVC

# AHG reports (13)

The activities of ad hoc groups that had been established at the prior meeting are discussed in this section.

JCT3V-G0001 JCT-3V AHG Report: JCT-3V project management (AHG1) [J.-R. Ohm, G. J. Sullivan]

This document reports on the work of the JCT-3V ad hoc group on Project Management, including an overall status report on the project and the progress made during the interim period since the last meeting.

The work of the JCT-3V overall has proceeded well in the interim period. A large amount of discussion was carried out on the group email reflector. All documents from the preceding meeting had been made available at the document site (http://phenix.it-sudparis.eu/jct3v/) or the ITU-based JCT-3V site (http://wftp3.itu.int/av-arch/jct3v-site/2013_10_F_Geneva/), particularly including the following:

- The meeting report (JCT3V-F1000)

- Draft Text 8 of 3D-AVC (JCT3V-F1002)

- Test Model 8 of 3D-AVC (JCT3V-F1003)

- Draft 1 of 3D-AVC reference software (JCT3V-F1014)

- Draft Text 6 of MV-HEVC (JCT3V-F1004)

- Test Model 6 of 3D-HEVC and  MV-HEVC (JCT3V-F1005), and associated software

- Draft 2 of 3D-HEVC text specification (JCT3V-F1001)

- Draft 5 of MVC plus Depth Conformance (JCT3V-F1006)

- Draft 3 of MVC plus Depth Reference Software (JCT3V-F1010)

- Draft 3 of 3D-AVC Conformance (JCT3V-F1009)

- Draft 4 of Multi-resolution Frame Compatible Stereo (MFC) (JCT3V-F1008)

- Draft 2 of MFC Conformance (JCT3V-F1012)

- Draft 2 of MFC Reference Software (JCT3V-F1013)

- 3D Video Subjective Quality Assessment Test plan v2 (JCT3V-F1011)

- Common Test Conditions of 3DV Core Experiments (JCT3V-F1100)

- Finalized core experiment descriptions (JCT3V-F1101 through JCT3V-F1105).

The 13 ad hoc groups and 5 core experiments had made progress, and various reports from those activities had been submitted.

The software for ATM version 10, and HTM version 9 had been prepared and released approximately as scheduled.

A "bug tracking" systems for software and text specifications had been installed. The sites are https://hevc.hhi.fraunhofer.de/trac/3d-hevc and https://hevc.hhi.fraunhofer.de/trac/3d-avc/. The bug tracker reports were automatically forwarded to the group email reflector, where the issues could be further discussed.

More than 220 input contributions to the current meeting had been registered. A significant number of late-registered and late-uploaded contributions were noted again.

The meeting announcement had been made available from the aforementioned document site and http://wftp3.itu.int/av-arch/jct3v-site/2014_01_G_SanJose/JCT3V-G_Logistics.doc. A joint meeting of AHG7 (high level syntax) with JCT-VC on topics of HEVC high-level syntax, two days prior to the current JCT-3V meeting, was also organized.

A preliminary basis for the document subject allocation and meeting notes had been circulated to the participants as http://wftp3.itu.int/av-arch/jct3v-site/2014_01_G_SanJose/JCT3V-G_Notes_d0.doc.

The editors reported that, as of the opening of the meeting, the drafting of the AVC new edition was nearly completed (but not entirely finished), and that the corrigendum changes to bitstream extraction needed to be integrated into MVC+D and that one other small set of text needed to be integrated.

JCT3V-G0002 JCT-3V AHG report: 3D-AVC Draft and Test Model editing (AHG2) [M. M. Hannuksela, Y. Chen, T. Suzuki, J.-R. Ohm, G. Sullivan]

TBA. (AHG to be discontinued, although the test model document may need some updating)

JCT3V-G0003 JCT-3V AHG Report: MV-HEVC / 3D-HEVC Draft and Test Model editing (AHG3) [G. Tech, K. Wegner, J. Boyce, Y. Chen, T. Suzuki, S. Yea, J.-R. Ohm, G. Sullivan]

TBA.

It was noted that F0159 CE5 (Fast depth lookup table application method to intra modes for depth data, method 3) required an enabling flag at some position t.b.d. There was no decision where to add the enabling flag. A list of other minor issues is also listed in the bug tracking system

JCT3V-G0004 JCT-3V AHG Report: 3D-AVC Software Integration (AHG4) [D. Rusanovskyy, J.Y. Lee, J.-L. Lin, O.Stankiewicz]

TBA

P versus B to be clarified in revised AHG report.

It was remarked that a significant reason for enabling features is for purposes of enabling conformance testing primarily, rather than primarily to improve coding efficiency.

Some bugs were encountered, but were expected to be fixed quickly.

DAM ballot expected to be issued at the next meeting. PDAM 35 ballot closing 2014-03-02.

Recommendations from AHG

- Finalize integration of bug-fixes for ALC and prediction structure problems.

- To establish a BoG to further study the problem identified in breakout report JCT3V-F0268 and discuss possible software or/or specification text solution. Planning to align software with the text. Coordinator D. Tian.

- AhG4 recommend extending conformance bitstream set to include IBP inter-view prediction structure. Currently observed bug are not concerned of normative, decoder side and all localized in encoding side.

Initial effort was made to fix a bug in the IBP configuration. However, a mismatch is still observed for the Balloons sequence. This should be identified within a breakout activity. This configuration is needed to finalize the conformance spec, not that relevant for compression benefit in 3-view case.

New draft of software (Study of PDAM) to be issued.

JCT3V-G0005 JCT-3V AHG Report: MV-HEVC and 3D-HEVC Software Integration (AHG5) [G. Tech, L. Zhang, Y. Chang, K. Wegner]

TBA (Gerhard’s PPT)

Open issues:

- Some minor mismatches related to 3D-HEVC HLS.

- MV-HEVC / 3D-HEVC SEI messages missing.

- Some missing items related to MV-HEVC HLS (CL-RAS, POC alignment, Auxiliary pictures)

- Other minor issues are listed in the bug tracking system.

It was noted that when a proposal is adopted, we need to discuss how to enable it in the HTM software (e.g. encoder parameter / parameter set flag, or always on).

HTM8 -> HTM9.0 approx. 2% BR reduction for coded views, approx. 1.8%  for synthesized views.

HTM9.1 HLS of MV-HEVC (SEI, POC align, aux pictures, CLRAS missing)

HTM9.2 HLS of 3D-HEVC (SEI missing)

Inclusion of depth coding for MV-HEVC is still in separate branch

based on HM 12.0, should be simple to align with 13.0

For new AHG: Prepare reference software extension related to MV-HEVC (also coordinate with RExt).

JCT3V-G0006 JCT-3V AHG Report: 3D Coding Tool Testing (AHG6) [K. Müller, A. Vetro]

This document reports on the work of the JCT-3V ad hoc group on 3D Coding Tool Testing (AHG6) between the 6th JCT-3V meeting in Geneva (25 October – 1 November, 2013) and 7th JCT-3V meeting in San Jose (11 – 17 January 2014).

The CE work plans were finalized and made available via the JCT-3V web site by December 13, 2013.

For the CEs, the 3DV-HTM software version 9.0r1 was used, which includes a bug fix (related to sub PU level inter-view motion vector prediction). Accordingly, it became necessary to announce this version to be used for CEs.

Other than this, there were no other needs during this AHG period to coordinate between any of the core experiments.

All CEs used the CTC configuration. In addition,

- CE4 used a configuration with BVSP (Backward view synthesis prediction) OFF for ADVD (adaptive disparity vector derivation), and

- CE5 used the all-intra configuration.

- There were 5 CEs established at the previous meeting to further study various aspects of the HEVC-based coding designs.

- To get a sense of where the main areas of development are focused, the approximate number of CE reports and CE related contributions are noted below.

- CE5 on Depth intra modes is most active with 38 contributions, followed by CE3 on Motion prediction for texture coding with 25 contributions. The CEs on Sub-PU processing (CE1), Motion prediction for depth coding (CE2) and Residual prediction (CE4) are also active with 16-17 contributions.

The CEs and related contributions were listed in the report.

The 3D viewing equipment has been prepared and the test room Silicon Valley has been setup.

Test sequences for MV-HEVC (with depth map carriage) and 3D-HEVC had been prepared, as reported in document JCT3V-G0110, and could be viewed during the week. A few inputs on coding performance for MV-HEVC and 3D-HEVC had been received and were to be discussed during the meeting.

JCT3V-G0007 JCT-3V AHG report: 3D High level syntax (AHG7) [M. M. Hannuksela, J. Boyce, Y. Chen, A. Norkin, Y.-K. Wang]

This report summarizes the activities of the ad-hoc group of 3D high level syntax (AHG7).

JCT-VC AHG10 on multi-layer picture order count (POC) design had several phone conferences between the 6th JCT-3V meeting and the 7th JCT-3V meeting.. Invitations to the phone conferences were distributed through the JCT-3V reflector too. JCT-VC AHG10 agreed on recommendations on the multi-layer POC design.

JCT-3V AHG7 had met collocated with JCT-VC on 9–10 January, 2014. The JCT-VC meeting notes document the recommendations that were reached.

Most high-level syntax input contributions have been submitted to both JCT-VC and JCT-3V. These input contributions have been categorized in the JCT-VC meeting notes. Additionally the following two input contributions relate to 3D-HEVC high-level syntax (as correctly categorized in "JCT3V-G_Notes_d0.doc", Section 6.1):

- JCT3V-G0048 3D-HEVC HLS: On inter-component reference pictures [Y.-L. Chang, Y.-W. Chen, J.-L. Lin, Y.-P. Tsai, S. Lei (MediaTek)]

- JCT3V-G0053 3D-HEVC HLS: On ARP reference picture signalling [J.-L. Lin, Y.-W. Chen, Y.-L. Chang, Y.-W. Huang, S. Lei (MediaTek)]

There were no emails related to the mandates of this ad-hoc group over the reflector between the 6th JCT-3V meeting and the 7th JCT-3V meeting.

J-BoG on HLS (coordinated by J. Boyce) met in parallel to consider HLS issues.

JCT3V-G0008 JCT-3V AHG Report: Study of 3D-HEVC entropy coding (AHG8) [J.-L. Lin, T. Ikai]

This document reports on the work of the JCT-VC ad hoc group on study of 3D-HEVC entropy coding (AHG8) between the 6th JCT-3V meeting in Geneva (25 Oct. – 1 Nov., 2013) and the 7th JCT-3V meeting in San José (11–17 January 2014).

The contributions related to the simplifications and unifications of binarization and context models and the reduction of context coded bins for 3D-HEVC that had been submitted for consideration were listed in the report. There were four such contributions, and four accompanying cross-checks.

- JCT3V-G0060 is a proposal asserted to simplify the CABAC coding of the syntax related to the inter SDC, intra SDC and DMM modes.

- JCT3V-G0061 is a proposal asserted to simplify the CABAC context modeling for ic_flag and iv_res_pred_weight_idx.

- JCT3V-G0062 is a proposal to remove the number of context coded bins in DMM mode coding be using bypass coding.

- JCT3V-G0056 is a proposal to reduce the number of context coded bins for inter SDC residual coding.

In the discussion, it was remarked that G0108 and G0124 are also related.

It was suggested to have a BoG (coordinated by J.-L. Lin and P. Merkle) identify context coded syntax elements for consideration for simplification.

JCT3V-G0009 JCT-3V AHG Report: 3D Quality Assessment (AHG9) [T. Ebrahimi, V. Baroncini, A. Vetro]

This document reports on the work of the JCT-3V ad hoc group on 3D Quality Assessment (AHG9) between the 6th JCT-3V meeting in Geneva (25 Oct. – 1 Nov. 2013) and 7th JCT-3V meeting in San Jose (11–17 Jan. 2014).

The test material for the assessment of AVC-based codecs was prepared and made available during the previous meeting. A report of the viewing results was expected during the San Jose meeting.

Additionally, draft test material for the HEVC-based codecs was generated. One document had been submitted to this meeting describing the test material for different codec configurations:

- JCT3V-G0110: Test Material for Subjective Comparison of 3D-HEVC and MV-HEVC with depth coding [Gerhard Tech, Karsten Müller]

The test material was available for viewing during the San Jose meeting.

The following contributions related to this mandate had been submitted:

- JCT3V-G0109: Comparative Results for 3D-HEVC and MV-HEVC with Depth Coding [Ying Chen, Gerhard Tech, Karsten Müller, Anthony Vetro, Li Zhang, Shinya Shimizu]

- JCT3V-G0142: On comparison of 3D-HEVC and MV-HEVC [M. M. Hannuksela (Nokia), Y. Yan (USTC)]

- JCT3V-G0174: On MV-HEVC+D encoder parameters [A. Norkin, U. Hakeem, R. Sjöberg (Ericsson)]

The documents discuss different encoding configurations, which should be reviewed and discussed during the meeting.

The AHG does not have any recommendations at this time on changes to the evaluation methodology, but would certainly consider this based on the experiments. However, it is encouraged that CE participants consider using alternative metrics as part of their work.

The AHG on 3D Quality Assessment recommended to:

- Solicit reports on the subjective testing of AVC-based codecs

- Conduct informal viewing of the encoded test material for HEVC-based codecs

- Review the contributions related to HEVC-based encoding parameters

- Prepare a draft test plan for HEVC-based codecs, considering test conditions and schedule – e.g., for comparing 3D-HEVC to MV-HEVC.

- Re-establish the AHG to coordinate the subjective testing and further study objective quality assessment metrics

JCT3V-G0010 JCT-3V AHG Report: Complexity Assessment (AHG10) [G. G. Lee, G. Bang, T. Ikai, K. Rapaka, D. Rusanovskyy]

This document reports on the work of the JCT-3V ad hoc group (AHG) on Complexity assessment (AHG10) between the 6th JCT-3V meeting in Geneva (25 Oct. – 1 Nov. 2013) and the 7th JCT-3V meeting in San Jose (11–17 Jan. 2014).

A spreadsheet had previously been developed to discuss, identify and tabulate the coding tools or cases to be studied at different suggested data granularities. It was requested for a copy of the spreadsheet to be provided as an attachment to a revision to the AHG report. The spreadsheet provides a high-level overview of the scope of the work in this ad hoc group. Several examples had been completed by a co-chair, and proponents in the JCT-3V 6th meeting had also included this spreadsheet. The spreadsheet had not been updated since the previous meeting.

Two contributions were identified as related:

- JCT3V-G0114, AHG 10: Complexity Assessment on Illumination Compensation (IC). This document provides an informative case study for the coding tools in 3D-HEVC based on number of operations, data transfer rate, and data storage requirements.

- JCT3V-G0208, AHG 10: Complexity assessment of motion compensation process. This document provides an informative case study for the coding tools in 3D-HEVC based on number of operations and data transfer rate.

JCT3V-G0011 JCT-3V AHG Report: 3D Test Material (AHG11) [S. Shimizu, S. Yea]

This document reports on the work of the JCT-VC ad hoc group on Test Material (AHG11) between the 6th JCT-3V meeting in Geneva (25 October – 1 November, 2013) and the 7th JCT-3V meeting in San Jose (11 – 17 January, 2014).

There was no e-mail exchange. At this meeting, we had no input contribution related to this AHG.

Tried to contact contributors of test material to Vienna, but no reaction received.

JCT3V-G0012 JCT-3V AHG Report: Conformance testing development (AHG12) [T.Suzuki, D.Rusanovskyy]

The ftp site at ITU-T is used to exchange bitstreams. The ftp site for downloading bitstreams is,

http://wftp3.itu.int/av-arch/jct3v-site/bitstream_exchange/

The spreadsheet to summarize the status of bitstream exchange, conformance bitstream generation is available at this directory. It includes the list of bitstreams, codec features and settings, and status of verification.

The list of the candidate of the conformance bitstream and its volunteers are summarized in the table below.

So far, 15 bitstreams were collected for MVC+D. AhG4 and proponents provided an update to reference MVC+D software which was used to generate missing bitstream MVDRS-1. As for today, all planned bitstreams are collected

| Categories | Bitstream | Donated by | File Name | Multi-view Depth High | Level | Frame Rate (Frame/Sec) |
| --- | --- | --- | --- | --- | --- | --- |
| Depth Resolution | MVDDR-1 | Nokia | MVDDR-1 | X | 3 and higher | 30 |
|  | MVDDR-2 | Nokia | MVDDR-2 | X | 4 and higher | 25 |
|  | MVDDR-3 | Sony | MVDDR-3 | X | 3 and higher | 30 |
|  | MVDDR-4 | Sony | MVDDR-4 | X | 4 and higher | 25 |
| Views Configuration | MVDVC-1 | Sony | MVDVC-1 | X | 3 and higher | 30 |
|  | MVDVC-2 | Sony | MVDVC-2 | X | 4 and higher | 25 |
|  | MVDVC-3 | Nokia | MVDVC-3 | X | 3 and higher | 30 |
|  | MVDVC-4 | Nokia | MVDVC-4 | X | 4 and higher | 25 |
| Inter View Prediction / temporal inter prediction | MVDIV-1 | Nokia | MVDIV-1 | X | 4 and higher | 25 |
|  | MVDIV-2 | Sony | MVDIV-2 | X | 3 and higher | 30 |
|  | MVDIV-3 | MERL, Qualcomm | MVDIV-3 | X | 5.1 and higher | 25 |
|  | MVDIV-4 | MERL, Qualcomm | MVDIV-4 | X | 5.1 and higher | 30 |
| Interlaced Coding Tools | MVDIL-1 | ITRI, Sony | MVDIL-1 | X | 3.1 and higher | 30 |
|  | MVDIL-2 | ITRI | MVDIL-2 | X | 4.1 and higher | 15 |
| MVD representation and SEI | MVDRS-1 | Nokia | MVDRS-1 | X | 3 and higher | 30 |

In the last meeting, it was reported that the post dilation filter is ON in the reference software. Md5 checksum must be calculated using yuv before applying the post dilation filter. Md5 files calculated with the reconstructed picture after the post filter were replaced.

The generated bitstreams for MVC+D are available at

http://wftp3.itu.int/av-arch/jct3v-site/bitstream_exchange/under_test/MVCD/

As for 3D-AVC development, the list of recommended conformance bitstream was signficiantly updated, as per recommendation of JCT3V-F1009. The list of candidate bitstreams, their parameters and expected proponents are given in the table below.

Candidate conformance bitstreams for 3D-AVC development.

| Categories | Bitstream |  | Donated by |  | File Name |  | Multi-view Depth High |  |  |  | Level |  | Frame Rate (Frame/Sec) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Depth Resolution |  | MVDDR-1 |  | Nokia |  |  |  | X |  |  |  | 3 and higher |  | 30 |  |
|  |  | MVDDR-2 |  | Nokia |  |  |  | X |  |  |  | 4 and higher |  | 25 |  |
| Coding tools |  | MVDCT-1 |  | MERL |  |  |  | X |  |  |  | 3 and higher |  | 30 |  |
|  |  | MVDCT-2 |  | Mediatek |  |  |  | X |  |  |  | 4 and higher |  | 25 |  |
|  |  | MVDCT-3 |  | Samsung |  |  |  | X |  |  |  | 3 and higher |  | 30 |  |
|  |  | MVDCT-4 |  | Poznan |  |  |  | X |  |  |  | 4 and higher |  | 25 |  |
|  |  | MVDCT-5 |  | Qualcomm |  |  |  | X |  |  |  | 4 and higher |  | 25 |  |
|  |  | MVDCT-6 |  | Qualcomm |  |  |  | X |  |  |  | 3 and higher |  | 30 |  |
|  |  | MVDCT-7 |  | Qualcomm |  |  |  | X |  |  |  | 4 and higher |  | 25 |  |
|  |  | MVDCT-8 |  | MERL |  |  |  | X |  |  |  | 3 and higher |  | 30 |  |
|  |  | MVDCT-9 |  | Samsung |  |  |  | X |  |  |  | 4 and higher |  | 25 |  |

As for today, 7 bitstream were collected for 3D-AVC and 4 bitstreams are still missing.

The bitstream for 3D-AVC is available at

http://wftp3.itu.int/av-arch/jct3v-site/bitstream_exchange/under_test/3D-AVC/

It should be noted, that AhG4 is planning to finalize 3DV-AVC reference software, which is cable of coding MVD data with bi-directional inter-view prediction.

In the last meeting, it was suggested to upload the bitstream, short description, trace file, decoded image, md5 checksum in a zip archive. All files inside the zip archive should have the same base name. Only the extension is changed in the following way:

- .bit - bitstream

- .txt - description

- .trc - tace file

- .md5 - MD5 sum of the complete decoded file

- .yuv - decoded image

Note: Having a trace file is useful for debugging and it is recommend to include short trace file. Full trace is optional. Decoded image and MD5 checksum are optional. But one of the decoded YUV file, md5 checksum file must be provided.

MD5 must be provided for decoded YUV file. Hash must be the existed at the beginning of the file. (Text remarks by volunteers are included in some md5 files)

A description of the recommended md5sum tool on UNIX can be found at http://en.wikipedia.org/wiki/Md5sum.

It was noted that the PDAM conformance ballot closes 2014-03-01.

It was suggested to issue a "study document" to contain the additional bitstreams that had become available.

JCT3V-G0013 JCT-3V AHG Report: Mixed Resolution Coding (AHG13) [K. Wegner, S. Shimizu]

This document reports on the work of the JCT-VC ad hoc group on Mixed Resolution Coding (AHG13) between the 6th JCT-3V meeting in Geneva (25 October – 1 November, 2013) and the 7th JCT-3V meeting in San Jose (11 – 17 January, 2014).

The following contributions are related to AHG14:

JCT3V-G0151: Shinya Shimizu, Shiori Sugimoto, “AHG13: Results with quarter resolution depth map coding”

It was reported that a reduced resolution depth map coding in the context of 3D-HEVC gave no coding benefit. However, such coding scheme achieves low complexity decoding in terms of the number of operation and memory usage, and decoder throughput.

There are many efforts in order to migrate JCT3V-D0215 and JCT3V-D0216, which are based on HTMv6.x, to the latest version of HTM. Due to the limited time after the release of HTMv9.x, the migration has not been finished yet. The software used in JCT3V-G0151 supports only VSO; no 3D-HEVC coding tool is supported with the reduced resolution depth map coding. Simulation result shows about 3.38% bitrate reduction and 6.22% bitrate increase for coded and synthesized views, respectively, with saving about 30% decoder runtime.

The AHG recommended to review the related input contribution and decide whether to support mixed resolution coding in MV-HEVC and 3D-HEVC.

# Project development, status, and guidance (0)

## Communication by parent bodies (0)

## 3D video application areas (0)

## Profile/level definitions (0)

## 3D-HEVC / MV-HEVC performance assessment (4)

JCT3V-G0109 Comparative Results for 3D-HEVC and MV-HEVC with Depth Coding [Y. Chen, G. Tech, K. Müller, A. Vetro, L. Zhang, S. Shimizu]

This contribution evaluates the coding performance of the current implementations of 3D-HEVC and MV-HEVC with depth coding (based on HTM 9.0r1). The results show that 3D-HEVC provides an average bit rate reduction of 20% (measured by PSNR of synthesized views) relative to MV-HEVC. Based on this result, it is proposed that the 3D-HEVC working draft progress to Proposed Draft Amendment (PDAM) at this meeting.

It was noted that VSO was turned on for both MV-HEVC and 3D-HEVC in all tests.

Revisit: PDAM for 3D-HEVC?

JCT3V-G0110 Test Material for Subjective Comparison of 3D-HEVC and MV-HEVC with depth coding [G. Tech, K. Müller] [late]

This contribution summarizes the test material, prepared for the visual comparison between MV-HEVC (depth map carriage) and 3D-HEVC. For each coding method, all 8 test sequences with 4 rate points have been prepared with the following Texture QPs for the independent view: 45, 40, 35, and 30. For each rate point, two stereo pairs have been produced.

It is recommended that informal subjective viewing be done and that rate points for subjective testing be confirmed.

There was a question on whether interview prediction for depth is enabled in both codec configurations. It was confirmed that interview prediction for depth is enabled.

JCT3V-G0243 Viewing Report for Comparison of 3D-HEVC and MV-HEVC with depth coding [Vittorio Baroncini, Gerhard Tech, Karsten Müller]

Follow-up: Visual comparison has been done with 5 experts Mon afternoon – comparing MV-HEVC with depth coding, inter-view prediction enabled, depth at full resolution, VSO enabled, sequences with approximately same coded texture and synthesized view PSNRs were compared. The result is reported (input doc to be registered) that only minor differences (only in some cases slightly better/slightly worse) were found. The results of this informal test could be interpreted such as the above PSNRs are approximately reflecting the visual quality.

JCT3V-G0142 On comparison of 3D-HEVC and MV-HEVC [M. M. Hannuksela (Nokia), Y. Yan (USTC)]

The contribution suggests aspects to be taken into account when preparing bitstreams for comparing the performance of MV-HEVC (with depth auxiliary picture layers) and 3D-HEVC.

One suggestion is that MV-HEVC and 3D-HEVC should have same level of view synthesis optimization. This is already being done.

Another suggestion is that codecs should be used with their best configuration, e.g., considering spatial resolution of depth views, range extension coding tools for the coding of auxiliary depth pictures and SHVC coding tools such as inter-layer motion prediction.

Results are shown with different depth resolutions that demonstrate notable coding gains. However, these results were generated with VSO disabled and there were doubts whether the same gains could be obtained when VSO is enabled, and whether they would exceed the VSO enabled result with full-resolution depth. In a related contribution (G0151) it is shown that depth down-sampling does not provide any coding efficiency benefit when VSO is enabled. It was agreed to use the best codec configuration considering both VSO and depth downsampling factor, but further study is needed to determine this.

Regarding the use of RExt tools for auxiliary pictures, it was not clear what the benefit of RExt tools would provide with depth and whether this is a plausible use case scenario for MV-HEVC. There was a question on whether version 1 decoders are able to decode monochrome format.

Inter-layer motion prediction refers to prediction between texture and depth, and is shown to provide a 1% coding gain. It was remarked that inter-layer prediction is currently disabled for auxiliary layers. It was noted that this would complicate the design and the dependencies. There is no proposal to enable this, and  it was considered desirable by the group to not have these inter-layer dependencies.

JCT3V-G0174 On MV-HEVC+D encoder parameters [A. Norkin, U. Hakeem, R. Sjöberg (Ericsson)]

The contribution proposes to enable inter-view prediction between the depth components in MV-HEVC+Depth configuration of MV-HEVC reference software (HTM). The reported BD-Rate gains relative to the current coding parameters of MV-HEVC+D are −4.6% on the synthesized views under the common test conditions. It is also proposed to use the configuration of MV-HEVC+D with enabled inter-view prediction between the depth components in the subjective test between the MV-HEVC+D and 3D-HEVC.

It was remarked that inter-view prediction for depth has been enabled in the comparative results of G0109/G0110.

There was a question on the reported gains, specifically that the 4.6% average gain seemed high. It was noted that the configuration used in the reported results disabled the quadtree limitation, so the gain would be much smaller.

It was suggested to add configuration files in the SVN repository for MV-HEVC+D that enable inter-view prediction for depth.

Decision(SW): Adopt (add config files)

JCT3V-G0232 Cross-check of Ericsson’s proposal JCT3V-G0174 on MV-HEVC+D encoder parameters [P. Aflaki, M. M. Hannuksela] [late]

## MFC plus depth (2)

JCT3V-G0115 Preliminary Results of MFC plus Depth [L. Grimaldi, K. Schueuer, P. Yin, T. Lu, H. Ganapathy, T. Chen, W. Husak]

This contribution shows a comparison between MFC plus depth (MFC+D) and FC plus depth (FC+D) in multi-view rendering. All tests have been performed without compression. The study aims at providing some upper bound on the expected gain. The texture views are given by the upsampled FC left and right views for FC+D case and by the reconstructed MFC left and right views for the MFC+D case, respectively. The original full resolution depth data is used in all the tests. The simulation results show that the average PSNR gain of synthesized views is in a similar range to that of original views. In addition, subjective evaluation justifies the objective gain.

Main intention of the contribution is that the characteristics of MFC coded views are retained in synthesized views when depth data are used.

Neither texture nor depth were compressed.

The suggested approach is using MFC for texture, an using MVC coding for depth (or MVC+D with depth only).

Bring this to the attention of the parent bodies and seek advice how to proceed. Revisit after that what to do next.

JCT3V-G0117 MFC+D: Crosscheck on Dolby Proposal JCT3V-G0115 [D. Tian (MERL)]

## Complexity assessment (2)

JCT3V-G0114 AHG 10: Complexity Assessment on Illumination Compensation (IC) [G. G. Lee, B.-S. Li, C.-F. Chen, Z.-H. Yu, C.-H. Huang (NCKU)]

This proposal provides a case study of complexity assessment on illumination compensation (IC) in 3D-HEVC and complexity metrics, including number of operations, data storage requirement, and data transfer rate are assessed. The analyzed results in this proposal might be considered as anchor complexity to assist JCT3V in evaluating the proposals.

This proposal also presents the experimental results without IC coding tools. The results reveal that the coding performance will be degraded without IC in HTM-9.0r1 configuration and the coding loss is 0.6%, 0.6%, and 0.4% for video PSNR vs. video bitrate, video PSNR vs. total bitrate, and synth PSNR vs. total bitrate, respectively, at HTM-9.0r1 configuration without IC.

JCT3V-G0208 AHG10: Complexity assessment of motion compensation process [T. Ikai (Sharp)] [late]

This proposal reports a complexity assessment of motion compensation process, especially considering the worst case. 3D-HEVC enhances motion compensation process by introducing Sub-block, VSP, ARP and IC methods. This evaluation is carried out by spreadsheet in terms of number of operation and data transfer rate. It is reported that the simplification techniques for the tools, such as horizontal only motion comp in VSP, is useful to reduce the worst case complexity to be comparable to HEVC version1. It is recommended to retain these simplification techniques and keep complexity in check for further development.

It is reported that VSP is relatively low in terms of computation and bandwidth, ARP incurs a higher bandwidth relative to HEVC v1, and that MC+IC is slightly higher than 8x8 MC in HEVC v1. Notable increases are observed when certain constraints in the current design are relaxed.

## Quality assessment (0)

## Source video test material (0)

# Core experiments

## CE1: Sub-PU processing (20)

### Summary (1)

JCT3V-G0021 CE1: Summary report on Sub-PU processing [S. Shimizu (NTT)]

Sub-test a: Default motion vector for SPIVMP

All contribution is aiming to improve the parallel processing capability of sub-PUs.

In this category, default motion parameters for SPIVMP are investigated. The default motion parameters are utilized on motion hole filling as well as the pruning process of merge candidates. All contribution is aiming to improve the parallel processing capability.

In order to distinguish motion parameters for motion hole filling with those for the pruning process, the former is called as default motion parameters (DMP) and the latter is called as representative motion parameters (RMP). In addition, motion parameters on the block which corresponds to center pixel of the current PU in a reference picture is called as center motion parameters (CMP), and motion parameters with zero motion vectors and zero reference index for both L0 and L1 lists is called as zero motion parameters (ZMP).

TABLE 1: Summary of results on Test (a)

| Doc. | Availability of SPIVMP candidate | DMP | Update of DMP | RMP | BD-Rate (Video PSNR) | BD-Rate (Synth. PSNR) | Dec. time | Cross-check | WD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HTM | At least one sub-PU has available MP | Last available MP | Yes | First available MP | - | - | - | - | - |
| G0120 | Always | CMP or ZMP | No | CMP or ZMP | 0.07% | 0.08% | 103.5% | G0203 | Yes |
|  | At least one sub-PU has available MP | CMP or ZMP | No | First available MP | 0.01% | 0.02% | 99.8% | G0203 | No |
| G0084 | At least one sub-PU has available MP | ZMP | Yes | First available MP | 0.01% | 0.02% | 101.2% | G0214 | Yes |
|  | At least one sub-PU has available MP | (Do)NBDV | Yes | First available MP | 0.01% | 0.00% | 100.6% | G0214 | No |
| G0147 | CMP is available | CMP | No | CMP | 0.00% | -0.01% | 98.9% | G0219 | Yes |

Among the three investigated methods, G0147 is agreed to be the simplest solution achieving the intended parallelism. Compared to HTM, it however requires additional processing to determine the CMP.

Offline activity of proponents of G0120, G0084 and G0147 to report back what is the necessary number of additional operations, memory accesses etc. compared to reference (HTM).

Offline activity is verbally reported on 01-14 PM, but no precise numbers are given. It is reported that no additional memory accesses are necessary. It is reported that all proponents of G0120, G0084 and G0147 suggest adoption of G0147 as the simplest solution. Proponents of G0120 also confirm that the WD text is mature.

Decision: Adopt G0147.

Sub-test b: Sub-PU level deblocking/motion parameter storage

In the current WD, the disparity parameters utilized to fetch depth map are stored in the motion storage in for VSP while sub-PU based motion parameters are stored for SPIVMP. The impact by storing sub-PU based disparity parameters for VSP is investigated. In addition, the impact by applying deblocking filter for sub-PU edges is also investigated.

TABLE 2: Summary of results on Test (b)

| Doc. | Store sub-PU based DVs for VSP | Deblocking filter on sub-PU edges | Type of sub-PU edge | Deblocking edges located other than 8x8 grid | BD-Rate (Video PSNR) | BD-Rate (Synth. PSNR) | Dec. time | Cross-check | WD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HTM | No | No | n/a | No | - | - | - | - | - |
| G0118 | Yes* | Yes (SPIVMP and VSP) | TU | No | -0.29% | -0.18% | 102.8% | G0152 | Yes |
| G0148 | Yes | No | n/a | No | -0.11% | -0.08% | 96.0% | G0218 | Yes |
|  | Yes | Yes (VSP) | TU | No | -0.11% | -0.07% | 99.7% | G0218 | No |
| G0068 | Yes | Yes (SPIVMP and VSP) | PU | No | -0.31% | -0.19% | 102.9% | G0155 | Yes |

Any of the proposals require changes to core parts

- G0118 modifies the TU tree based on sub-PU structure (3 pages of text)

- G0068 modifies the de-blocking decisions such that sub-PUs are handled as PUs, and would require a modification of the deblocking specification

G0068 may be closer to hardware implementations which determine the PU boundary based de-blocking decision from stored motion vectors.

Side activity (Y. Chang) to organize informal viewing to investigate whether the additional deblocking improves the subjective quality. Revisit. A viewing session is planned for Thurs morning.

A small benefit comes due to the benefit of storing sub-PU based DV and using them for VSP (0.08% according to the separate results in G0148). It is agreed that this is coming without complexity increase. This approach is used in all three proposals of this part of the CE, but the way of specifying it is not unified - G0118 is believed to use the minimum change. In principle, it is agreed to adopt this.

Proponents of G0118, G0148 and G0068 were asked to unify the WD specification.

As a follow-up activity, G0148 has been investigated as candidate text by proponents of G0118 and G0068. In the follow-up discussion 01-14 PM, both other parties confirmed that the WD text submitted in G0148 is appropriate.

Decision: Adopt G0148 (version without the deblocking change). The WD text submitted with G0148 only implements this version and can be used “as is”.

Sub-test c: VSP with residual prediction

In the current WD, VSP merge candidates become available when iv_res_pred_weight_idx is equal to 0; no combination of VSP and ARP is allowed. Such combination of VSP and ARP is investigated in this category.

The difference between G0059 and G0076 is an interpolation filter utilized for VSP, i.e. main depth-based disparity compensated prediction, when ARP is applied. G0059 proposes to use the 7/8-tap interpolation filter in HEVC while G0076 proposed to use a bi-linear interpolation filter as used in the 3D-HEVC residual predictions. For the residual prediction process, both G0059 and G0076 propose to use a bi-linear interpolation filter.

It is reported that there are minor impact on decoding time; however, one of the major concerns is the decoder complexity since a tri-prediction is performed for each 8x4 or 4x8 block. In both proposals, the number of the referred samples is smaller than the normal 8x8 bi-prediction, which requires to access 548 samples (450 luma + 98 chroma) per 8x8 block.

TABLE 3: Summary of results on Test (c)

| Doc. | Interpolation filter for VSP when ARP is applied | # of accessed samples per sub-PU (8x4 or 4x8) | BD-Rate (Video PSNR) | BD-Rate (Synth. PSNR) | Dec. time | Crosscheck | WD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| G0059 | HEVC interpolation filter | 228 (main pred.: 108, resi. pred.: 120) | -0.08% | -0.07% | 100.4% | G0185 | Yes |
| G0076 | Bi-linear filter | 172 (main pred.:52, resi, pred.: 120) | -0.06% | -0.06% | 100.2% | G0164 | Yes |

Note: G0076 is classified under CE4

Additional complexity/memory accesses not justified by the small compression gain.

The ARP in this case would use 4x8/8x4 block size (same as VSP), whereas the existing ARP uses 8x8 at minimum.

Sub-test d: Other

JCT3V-G0069: Restriced bi-prediction for sub-PU

In HEVC, the bi-prediction for small PU-size, 8x4 or 4x8, is prohibited. This contribution proposes to disable the bi-prediction for sub-PU whose block size is smaller than 8x8 in SPIVMP or VSP in order to align with the base HEVC design. Under the CTC, such bi-prediction is utilized only for the SPIVMP on 16x4/4x16 PUs.

TABLE 4: Simulation results of JCT3V-G0069

| Doc. | BD-Rate (Video PSNR) | BD-Rate (Synth. PSNR) | Dec. time | Crosscheck | WD |
| --- | --- | --- | --- | --- | --- |
| G0069 | 0.00% | -0.02% | 100.2% | G0157 | Yes |

Bi-prediction for 4x8/8x4 can currently occur in two cases:

- sub-block partitioning with AMP

- VSP

G0069 disables bi-prediction in both cases. However, the first case can also be solved by disabling the combination of SB partitioning and AMP (as below per G0077)

Decision: Adopt G0069 second case (disallow bi prediction with VSP).

JCT3V-G0077: MCP Size and DV for Sub-PU Prediction

This contribution contains three SPIVMP-related aspects; 1) MCP size, 2) disparity vector utilized for the motion parameter derivation, and 3) clean-up of sub-PU size syntax element.

In the current WD, a block size of sub-PU can be 12x8 or 8x12 which is not allowed in the base HEVC. In order to align with HEVC, this contribution proposes two different solutions; 1) dividing 12x8(8x12) PU into 8x8 and 4x8 (8x4 and 8x8) blocks, and 2) allowing SPIVMP only to 2Nx2N partition.

This contribution also proposes to set a vertical component of disparity vector utilized in SPIVMP to 0.

The last proposal is to clean-up the sub-PU size syntax, log2_sub_pb_size_minus2, in VPS. It is proposed to change it into log2_sub_pb_size_minus3 with moving from VPS to SPS.

TABLE 5: Simulation results of JCT3V-G0077

| Doc. | SPIVMP on block whose size is not 2Nx2N | 12x8/8x12 MCP | Vertical comp. of DV for SPIVMP | BD-Rate (Video PSNR) | BD-Rate (Synth. PSNR) | Dec. time | Cross-check | WD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HTM | Yes | Yes | Any | - | - | - | - | - |
| G0077 | Yes | No | Any | -0.01% | -0.01% | 107.6% | G0153 | Yes |
|  | No | No | Any | 0.00% | -0.02% | 100.1% |  | Yes |
|  | Yes | No | 0 | -0.03% | -0.04% | 107.4% |  | Yes |
|  | No | No | 0 | -0.03% | -0.06% | 100.1% |  | Yes |

Aspect 1 (MCP size): 2 solutions: Allowing sub-block part. only for 2Nx2N, or splitting 12x8/8x12 partitions into 8x8 and 4x8/8x4. First solution is simpler, also disallowing combination of AMP and sub-block part.

Aspect 2 (disallowing vert. disp. for MV derivation) – no complexity advantage, no coding advantage – no action.

Aspect 3 (syntax/semantics change of signalling) making it dependent of CTU and CU size in the way it is suggested would no longer allow using 8x8 sub-block part. when smallest CU size is 16x16 – no action. The suggested change of syntax element name “minus3” is reasonable  (editorial).

Decision: Adopt G0077 aspect 1 solution 1 (only enabling SPIVMP with 2Nx2N PU

Decision(Ed.): Change name of syntax element “log2_sub_pb_size_minus2“ to “log2_sub_pb_size_minus3“.

### CE contributions (6)

JCT3V-G0118 CE1: De-blocking friendly sub-PU processing in 3D-HEVC [H. Liu, Y. Chen, G. V. D. Auwera (Qualcomm)]

JCT3V-G0152 CE1: Crosscheck on De-blocking friendly sub-PU processing in 3D-HEVC (JCT3V-G0118) [S. Shimizu, S. Sugimoto (NTT)]

JCT3V-G0120 CE1: Simplifications to sub-PU level inter-view motion prediction [H. Liu, Y. Chen, L. Zhang (Qualcomm)]

JCT3V-G0203 3D-CE1: Crosschecking for Qualcomm's simplification for sub-PU level inter-view motion prediction [J. An, K. Zhang (MediaTek)] [late]

JCT3V-G0148 CE1: Motion parameters stored for VSP-coded blocks [S. Shimizu, S. Sugimoto (NTT)]

JCT3V-G0218 CE1: Crosscheck on Motion parameters stored for VSP-coded blocks (JCT3V-G0148) [H. Liu (Qualcomm)] [late]

### Related contributions (13)

JCT3V-G0059 3D-CE1 related: VSP combining inter-view ARP [Y.-W. Chen, J.-L. Lin, Y.-W. Huang, S. Lei (MediaTek)]

JCT3V-G0185 3D-CE1 related: Crosscheck on VSP combining inter-view ARP (JCT3V-G0059) [S. Shimizu, S. Sugimoto (NTT)] [late]

JCT3V-G0068 3D-CE1 related: Deblocking for sub-PU edge [J. An, K. Zhang, J.-L. Lin, S. Lei (MediaTek)]

JCT3V-G0155 3D-CE1 related: Crosscheck on Deblocking for sub-PU edge (JCT3V-G0068) [S. Shimizu, S. Sugimoto (NTT)] [late]

JCT3V-G0069 3D-CE1 related: Restriced bi-prediction for sub-PU [J. An, J.-L. Lin, K. Zhang, Y.-W. Chen, S. Lei (MediaTek)]

JCT3V-G0157 3D-CE1 related: Crosscheck on Restriced bi-prediction for sub-PU (JCT3V-G0069) [S. Shimizu, S. Sugimoto (NTT)] [late]

JCT3V-G0077 3D-CE1 related: MCP Size and DV for Sub-PU Prediction [M. W. Park, J. Y. Lee, C. Kim (Samsung)]

JCT3V-G0153 3D-CE1 related: Crosscheck on MCP Size and DV for Sub-PU Prediction (JCT3V-G0077) [S. Shimizu, S. Sugimoto (NTT)]

JCT3V-G0084 CE1 related: Modifications to motion hole filling in sub-PU level inter-view motion prediction [Y. Zhang, C. Zhu (UESTC), Y. Lin, J. Zheng (HiSilicon)]

JCT3V-G0214 CE1 related: Crosscheck on Modifications to motion hole filling in sub-PU level inter-view motion prediction (JCT3V-G0084) [H. Liu (Qualcomm)] [late]

JCT3V-G0147 3D-CE1 related: Simplification of sub-PU level temporal interview motion prediction [Y.S. Heo, M.S. Lee, W.W. Gwun, Y.J. Lee, G.H. Park (KHU)]

JCT3V-G0219 CE1 related: Crosscheck on Simplification of sub-PU level temporal interview motion prediction (JCT3V-G0147) [H. Liu (Qualcomm)] [late]

## CE2: Motion prediction for depth coding (20)

### Summary (1)

JCT3V-G0022 CE2: Summary report on motion prediction for depth coding [Y.-L. Chang, Y. Chen]

The following table describes the best coding gain in each CE contribution.

| Doc. No. | Contributions | Video /total BR | Synthesized /total BR |
| --- | --- | --- | --- |
| Test 1: New merging candidates for depth coding |  |  |  |
| JCT3V-G0063 | 3D-CE2: results on additional merging candidates for depth coding (DDD Candidate only) | 0.01% | -0.20% |
|  | 3D-CE2: results on additional merging candidates for depth coding (Additional texture candidate) | -0.05% | -0.08% |
|  | 3D-CE2: results on additional merging candidates for depth coding (Both two candidates) | -0.03% | -0.27% |
| Test 2: Sub-PU based MPI |  |  |  |
| JCT3V-G0119 | CE2: Sub-PU based MPI | -0.24% | -0.23% |
| Test 3: Interview motion vector prediction for depth coding |  |  |  |
| JCT3V-G0046 | 3D-CE2: results on interview motion prediction by DoNBDV for depth coding | -0.01% | -0.03% |

G0063: DDD candidate converts a motion vector that is already available to a depth value, which requires a table lookup or an on-the-fly calculation including multiplications, comparisons and shift operations. The calculation includes parameters that would change per slice, such that it would be necessary to compute new table values at slice level for every possible MV value. It was requested to give information how large the table would be and precise number of operations at slice level, or possibly necessary on-the-fly operations.

Further information provided 01-14 PM (new analysis in PPT slide to be uploaded).

1 mul, 1 add, 1 shift and 1 clip per PU in case of on-the-fly comparison

Alternatively: approx. *256 if computed beforehand at slice level (where a slice could hypothetically as small as 1 CTU)

Several experts express support to adopt this proposal. One expert points out that the additional complexity is not critical.

Decision: Adopt G0063 (DDD candidate only)

Additional texture candidate requires an additional fetch of a texture motion vector. Not justified by the relative small gain (Note: the same approach was earlier proposed as JCT3V-E0229, but not adopted.

G0119: Does not increase worst case complexity, since MPI could also be required for PUs with same size as sub-PUs. WD text is available and confirmed to be of sufficient quality by the cross-checker (G. Tech). Some compression benefit for both coded and synth. views.

Decision: Adopt G0119.

G0046: Solves a problem of possibly unavailable reconstructed depth pixel which could cause delays in decoder pipeline. The approach uses the same approach as in texture coding, applying NBDV and DoNBDV. However, some CE related proposals (G0047, G0074, G0126) might be even simpler. Further, the results reported in G0046 are based on software with a bug, which is corrected in G0047 that is based on G0046

The CE2-related contributions are roughly classified into three categories according to their purpose: simplifcation of disparity vector derivation in depth coding, simplification of merge candidate list in depth coding, and coding efficiency. The following table shows the classfication and their coding results.

| Doc. No. | Contributions | Video /total BR | Synthesized /total BR |
| --- | --- | --- | --- |
| Simplification of disparity vector derivation in depth coding |  |  |  |
| JCT3V-G0047 | 3D-CE2 related: Simplification on interview motion prediction for depth coding | -0.02% | -0.05% |
| JCT3V-G0074 | 3D-CE2 related: Simplification of DV Derivation for Depth Coding | 0.00% | 0.04% |
| JCT3V-G0126 | CE2 related: Disparity derivation for depth coding | -0.01% | -0.02% |
| Simplification of merge candidate list in depth coding |  |  |  |
| JCT3V-G0098 | 3D-CE2 related: Performance evaluation on depth Merge mode candidate (remove DV, Shift IVMV and Shift DV candidates) | -0.01% | 0.05% |
| JCT3V-G0127 | CE2 related: Simplification of merge candidate list in depth coding (Remove shifting candidate) | 0.00% | 0.02% |
|  | CE2 related: Simplification of merge candidate list in depth coding (Remove DV and shifting candidates) | -0.01% | 0.05% |
| Coding efficiency |  |  |  |
| JCT3V-G0055 | 3D-CE2 related: A texture-partition-dependent depth partition for 3D-HEVC | 0.04% | -0.16% |

G0047 is based on G0046, replacing NBDV by 128 depth converted to disparity per CU, then used in DoNBVD.

G0074 uses 128 depth converted to disparity directly (no DoNBDV that is used in 46/47), and only uses one DV for the whole picture.

G0126 uses 0 disparity and DoNBDV.

Decision: Adopt the aspect of G0074, use 128 depth converted to disparity for the entire picture, no DoNBDV.

The other aspects of G0074 relate to the second category (simpl. of merge cand. list)

G0074 reports additional results, combining the aspect as adopted above with removal of merge candidates as follows:

#### Additional coding results of the CE-related contribution G0074

|  | Video 1 | Video 2 | Video/video BR | Video/total BR | Synthesized/total BR | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| G0074 | 0.00% | 0.00% | 0.00% | 0.00% | 0.04% | 99.8% | 100.5% |
| G0074(Removal1) | 0.00% | 0.00% | 0.00% | -0.01% | 0.04% | 100.1% | 101.5% |
| G0074(Removal2) | 0.00% | 0.00% | 0.00% | 0.00% | 0.06% | 99.9% | 102.8% |

“Removal1” removes DV candidate and shifted DV candidate

“Removal2” additionally removes shifted IVMV candidate

Both G0127 and G0098 are equivalent to “Removal2”.

Decision: Adopt G0098/G0127. Proponents were asked to unify the WD text proposals. In a follow-up discussion on 01-14 PM, the proponents of G0098 confirmed that the WD text provided with G0127 fulfills the intended purpose.

Note: By this, only one merging candidate is using a disparity vector. It should be investigated in the ongoing CE whether this is needed any more.

G0055: Modifies the restrictions in depth quad-tree predictive coding as follows. Vertical/horizontal PU partitions are allowed for a depth CU only when the collocated texture CU is coded with vertical/horizontal or NxN partition. Several experts expressed support for this. The modification is well understood, and the situation regarding dependencies of depth and texture coding is not becoming worse than it is currently. WD text is confirmed by cross-checker.

Decision: Adopt.

### CE contributions (6)

JCT3V-G0046 3D-CE2: results on interview motion prediction by DoNBDV for depth coding [Y.-L. Chang, Y.-W. Chen, J.-L. Lin, Y.-P. Tsai, S. Lei (MediaTek)]

JCT3V-G0093 3D-CE2: Cross check of interview motion prediction for depth coding (JCT3V-G0046) [M. W. Park, C. Kim (Samsung)]

JCT3V-G0063 3D-CE2: results on additional merging candidates for depth coding [K. Zhang, J. An, J.-L. Lin, Y.-L. Chang, S. Lei (MediaTek)]

JCT3V-G0195 3D-CE2: Cross check of additional merging candidates for depth coding (JCT3V-G0063) [J. Y. Lee, C. Kim (Samsung)] [late]

JCT3V-G0119 CE2: Sub-PU based MPI [Y. Chen, H. Liu, L. Zhang (Qualcomm)]

JCT3V-G0212 Cross check of: CE2: Sub-PU based MPI (JCT3V-G0119) [G. Tech (HHI)] [late]

### Related contributions (13)

JCT3V-G0047 3D-CE2 related: Simplification on interview motion prediction for depth coding [Y.-L. Chang, Y.-W. Chen, J.-L. Lin, Y.-P. Tsai, S. Lei (MediaTek)]

JCT3V-G0196 3D-CE2 related: Cross check of simplification on interview motion prediction for depth coding (JCT3V-G0047) [J. Y. Lee, C. Kim (Samsung)] [late]

JCT3V-G0234 CE2-related: Crosscheck on Simplification on interview motion prediction for depth coding (JCT3V-G0047) [S. Shimizu, S. Sugimoto (NTT)] [late]

JCT3V-G0055 3D-CE2 related: A texture-partition-dependent depth partition for 3D-HEVC [X. Zhang, K. Zhang, J. An, J.-L. Lin, S. Lei (MediaTek)]

JCT3V-G0210 3D-CE2 related: Cross-check of JCT3V-G0055: a texture-partition-dependent depth partition for 3D-HEVC [J. Jung (Orange Labs)] [late]

JCT3V-G0074 3D-CE2 related: Simplification of DV Derivation for Depth Coding [M. W. Park, J. Y. Lee, C. Kim (Samsung)]

JCT3V-G0170 3D-CE2 related: Crosscheck of Simplification of DV Derivation for Depth Coding (JCT3V-G0074) [T. Ikai (Sharp)] [late]

JCT3V-G0098 3D-CE2 related: Performance evaluation on depth Merge mode candidate [J. Y. Lee, M. W. Park, C. Kim (Samsung)]

JCT3V-G0227 3D-CE2 related: Cross-check on performance evaluation on depth Merge mode candidate (JCT3V-G0098) [Y.-L. Chang (MediaTek)] [late]

JCT3V-G0126 CE2 related: Disparity derivation for depth coding [Y. Chen, L. Zhang, H. Liu (Qualcomm)]

JCT3V-G0235 CE2 related : Cross-check on Disparity derivation for depth coding (JCT3V-G0126) [T. Kim] [late]

JCT3V-G0127 CE2 related: Simplification of merge candidate list in depth coding [H. Liu, Y. Chen (Qualcomm)]

JCT3V-G0197 3D-CE2 related: Cross check of simplification of merge candidate list in depth coding (JCT3V-G0127) [J. Y. Lee, C. Kim (Samsung)] [late]

## CE3: Motion prediction for texture coding (28)

### Summary (1)

JCT3V-G0023 CE3: Summary report on motion prediction for texture coding [J.-L. Lin, J. Heo]

Test 1: AMVP candidate list construction for DCP blocks

JCT3V-G0065 is the CE3 results for new AMVP candidate list construction.

In the contribution document, the refined disparity of neighboring blocks, which has been used for merge process, is inserted into the AMVP candidate list. This replaces the zero candidate in case when a candidate is unavailable.

#### Coding results of the CE contribution G0065

| G0065 | Video 1 | Video 2 | Video/video BR | Video/total BR | Synthesized/total BR | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| With DoNBDV | -0.21% | -0.19% | -0.08% | -0.09% | -0.07% | 99.7% | 105.7% |
| With NBDV | -0.08% | -0.05% | -0.04% | -0.04% | -0.03% | 99.5% | 103.1% |

Current AMVP is identical to base spec., this proposal would change the candidate list construction.

No simplification, though probably the worst case complexity would not be increased, the gain is marginal. No action.

Test 2: Additional depth-based DV candidate

This contribution proposes to replace the shift DV candidates with the depth-based DV (F0144) at the previous meeting. As planned in CE, this contribution evaluates the performance of not only the proposed candidate but also the shift DV candidates. The experiment illustrates that both the proposed candidate and the shift DV candidates provide the coding gain less than 0.1% in CTC and VSP off conditions, respectively, which is negligible. Therefore, it is recommended to remove the two shift DV candidates in the Merge mode list.

Note: The anchor in the table below is not CTC, but the CTC with the corresponding tools disabled. Then, the codec under comparison enables the two tools separately. However, the first row is not proposed any more, as it is believed to provide insufficient gain. Instead, it is proposed to removed the shift DV candidate which is also giving only marginal gain.

#### Coding results of the CE contribution G0100

| G0100 | Video 1 | Video 2 | Video/video BR | Video/total BR | Synthesized/total BR | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| minDoNBDV | -0.25% | -0.07% | -0.06% | -0.05% | -0.05% | 100.0% | 100.2% |
| Shift DV candidate | -0.12% | -0.05% | -0.04% | -0.04% | -0.04% | 100.8% | 100.6% |

Question: When removing the condition for checking the shift DV candidate, how many operations are saved in the merge list construction? In a follow-up discussion on 01-14, it was reported by the proponents that the worst case is reduced by 30 condition checks by removing subclause H.8.5.3.2.15.

However, another expert expresses the opinion that in a proper implementation it should be at most 4 candidates that need not to be checked.

Further offline clarification between the proponents and the doubting expert was requested. The discussion did not lead to any agreements.

It was remarked that further options to improve the design, including removal of additional candidates such as bottom-right access, should be considered more thoroughly in the CE. It was agreed to further study this in the CE.

Test 3: Depth based block partitioning

JCT3V-G0106: In the proposed method, an arbitrarily shaped block partitioning for the collocated texture block is derived based on a binary segmentation mask computed from the collocated (virtual) depth map. Each of the two partitions (resembling foreground and background) is motion compensated and afterwards merged based on the depth-based segmentation mask.

Coding results of the CE contribution G0106

| G0106 | Video 1 | Video 2 | Video/video BR | Video/total BR | Synthesized/total BR | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | -0.5% | -0.5% | -0.2% | -0.2% | -0.1% | 103.3% | 100.8% |

It was noted that the current implementation does not allow combination of DBBP and SPIVMP, but SPIVMP was turned on in the evaluation. It was observed that the highest gains are on sequences with clean and high-quality depth, which is expected, but the gains are consistent overall. The method is also reusing most of the existing modules so complexity impact is considered minimal.

Decision: Adopt

The CE3-related contributions are roughly classified into four categories according to their purpose:

- Simplifcation on DV derivation

- Merge candidate construction

- Removal of redundant candidate when enabling illumination compensation

- Others

- Simplification

#### Coding results of the CE-related contribution G0049, G0050, G0073 (CTC)

|  | Video 1 | Video 2 | Video/video BR | Video/total BR | Synthesized/total BR | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| G0049: Remove DV-MCP flag | 0.01% | 0.03% | 0.00% | 0.00% | 0.00% | 96.2% | 87.7% |
| G0050 Test1: Remove above block | 0.07% | 0.06% | 0.02% | 0.01% | 0.02% | 99.8% | 98.3% |
| G0050 Test2: Remove above block + Remove DV-MCP flag | 0.06% | 0.10% | 0.02% | 0.02% | 0.01% | 98.5% | 88.2% |
| G0050 Test3: Remove above block + 128 DV | -0.05% | 0.00% | 0.00% | -0.01% | -0.01% | 99.9% | 96.3% |
| G0073: Remove DV-MCP block + 128 DV’ | -0.01% | 0.01% | 0.01% | 0.00% | -0.01% | 100.1% | 104.3% |

Additional results are also reported in non-CTC (BVSP off) which show 0-0.1% bit rate reduction. This is however not homogeneous over sequences.

The same proposals were brought to the last meeting. An extensive analysis was performed in BoG JCT3V-F0263, and it was concluded that the benefit is not significant enough to make a change. No new evidence about the real complexity benefit is brought with the new contributions. No action.

- Merge candidate construction

JCT3V-G0034 proposes to reduce worst case complexity for merge candidate derivation process in 3D-HEVC. It is asserted that this complexity increase in the worst case causes narrower bottle neck (in software) or increased implementation cost (in hardware).

This proposal omits additional 3D-HEVC candidates in 8x4 and 4x8 PU to reduce the worst case complexity. The change is that no inter-view merge candidate, no texture candidate, no shifted inter-view candidate and no VSP candidate are used. It is however still using 6 candidates. The additional condition check is as for bi prediction, and text changes are minimal.

#### Coding results of the CE-related contribution G0034

|  | Video 1 | Video 2 | Video/video BR | Video/total BR | Synthesized/total BR | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| G0034 | -0.08% | -0.06% | -0.01% | -0.01% | -0.01% | 99.2% | 99.2% |

JCT3V-G0099

Test1: The proposed method only performs the pruning processes between IVMV-A1/B1 in the texture coding and MPI-A1/B1 in the depth coding, respectively. Hence, the total number of the pruning processes is reduced from 11 to 4.

Test2: Instead of performing the pruning processes between IVMV-A1/B1 in the texture coding and MPI-A1/B1 in the depth coding, respectively, it is proposed to perform the pruning processes between IVMV and the first two available candidates in the HEVC merge list in the texture coding and MPI the first two available candidates in the HEVC merge list in the depth coding, respectively.

#### Coding results of the CE contribution G0099

| G0099 | Video 1 | Video 2 | Video/video BR | Video/total BR | Synthesized/total BR | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test1 | -0.02% | -0.07% | 0.00% | 0.01% | 0.05% | 99.2% | 99.7% |
| Test2 | 0.09% | 0.14% | 0.06% | 0.06% | 0.14% | 98.9% | 99.3% |

JCT3V-G0104

This contribution describes the clean ups of VSP merging candidates. The number of VSP candidates in merge candidate list is constrained to one by three different methods.

Method 1 (extra VSP converted to DCP): When checking the VSP flags of spatial neighboring blocks (A1, B1, and B0), only the first candidate with its corresponding block coded by VSP mode is set to a VSP candidate.

Method 2 (VSP pruning): On top of method 1.2, extra VSP candidates are removed from MCL instead of converting to a DCP candidate.

Method 3 (fixed VSP position in MCL):

- All the spatial VSP candidate is converted to a DCP candidate;

- Default VSP candidate is not inserted to MCL;

- A VSP candidate is inserted to a fixed position in MCL (only when merge index equals to a certain value N, i.e., N = 3).

#### Coding results of the CE contribution G0104

| G0104 | Video 1 | Video 2 | Video/video BR | Video/total BR | Synthesized/total BR | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Method 1 | -0.09% | -0.08% | -0.02% | -0.03% | -0.01% | 99.26% | 99.56% |
| Method 2 | -0.08% | -0.07% | -0.03% | -0.03% | -0.01% | 102.00% | 98.87% |
| Method 3 | 0.19% | 0.17% | 0.02% | 0.01% | 0.03% | 96.59% | 97.74% |

- Removal of redundant candidate when enabling illumination compensation

JCT3V-G0078

Method 1: Adaptive Disabling Temporal Inter-view Motion Candidates

In the current 3D-HEVC, the illumination compensation is only applied to the coding unit which performs the inter-view prediction. Thus, if the illumination compensation is enabled (i.e. ic_flag is equal to 1), the temporal inter-view motion candidates is proposed to be excluded from the merge list. Although the temporal inter-view motion candidate will be disabled, motion information of the temporal inter-view motion candidate can be used for pruning process.

Method 2: Allowing non-zero vertical comp. for Disparity Candidates

Additionally, when the illumination compensation is enabled, it is also proposed to allow the non-zero vertical component of the disparity vector for the disparity inter-view predicted temporal motion candidate.

JCT3V-G0066 [MediaTek] Syntax redundancy removal for illumination compensation

In the current 3D-HEVC, there is syntax redundancy for illumination compensation (IC) in merge blocks. It is proposed to remove such redundancy without imposing parsing dependency.

In the merging candidate construction process, when an ic_flag equal to 1 is received, a candidate that does not employ DCP is considered as unavailable.

When an ic_flag equal to 0 is received, the original merging candidate construction applies. With this proposed method, there is no parsing dependency problem or syntax redundancy.

#### Coding results of G0078 Method1 and G0066

|  | Video 1 | Video 2 | Video/video BR | Video/total BR | Synthesized/total BR | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| G0078-Method1 | -0.20% | -0.13% | -0.09% | -0.09% | -0.05% | 99.63% | 100.61% |
| G0066 | -0.24% | -0.20% | -0.09% | -0.08% | -0.05% | 96.5% | 94.3% |

#### Coding results of G0078 Method1 + Method2

| G0078 | Video 1 | Video 2 | Video/video BR | Video/total BR | Synthesized/total BR | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Method1+Method2 | -0.28% | -0.21% | -0.13% | -0.13% | -0.08% | 99.7% | 98.9% |

G0078 method 1 requires 2 additional comparisons, whereas in worst case G0066 could require six additional comparisons in the merge list construction, whereas it is claimed to possibly simplify the pruning process in the case of IC.

G0078 method 2 only gives very little gain, while further increasing the complexity.

Further investigation of G0034, G0099, G0104 (simplifying merging) and G0078 method 1 and G0066 (making merging a little bit more complex with small benefit in compression) in CE. The result of the CE shall be accompanied by a decent worst case complexity analysis.

d) Others

JCT3V-G0082 proposes to limit the range of NBDV according to the possible disparity range between the current texture view and the reference view. Two different methods are proposed. Both of the two methods can limit the range NBDV into a rectangle area, which makes the possible accessed area in reference view is much smaller than current design.

#### Coding results of G0082

|  | Video 1 | Video 2 | Video/video BR | Video/total BR | Synthesized/total BR | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Method1 | -0.09% | -0.11% | -0.03% | -0.03% | -0.03% | 100.4% | 98.2% |
| Method2 | -0.14% | -0.10% | -0.03% | -0.03% | -0.04% | 102.4% | 98.0% |

It is claimed that the approach limits the maximum cache size necessary. Data are given based on the set of test sequences, which may however not cover the possible worst case, since in principle we currently do not have any limitation on the motion vector. Another possibility to achieve this could be a limitation on the allowable range of inter-view motion vectors (as bitstream constraint within a given profile/level). Further study required on these issues.

JCT3V-G0128 [Qualcomm& PKU] CE3 related: Motion storage optimization in 3D-HEVC

In current 3D-HEVC, motion compression is achieved by two steps: compression to 8x8 level after each picture is decoded, and compression to 16x16 level after the whole access unit is decoded. In this document, it is proposed that the 3D-HEVC design should be aligned with the HEVC design for inside access unit motion storage optimization and the current 8x8 motion compression should be removed.

#### Coding results of G0128

|  | Video 1 | Video 2 | Video/video BR | Video/total BR | Synthesized/total BR | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| G0128 | -0.22% | -0.16% | -0.05% | -0.04% | -0.06% | 102.2% | 99.9% |

The proposal is to use uncompressed motion parameters within the same access unit. It is claimed that the amount of memory is not increased. WD text is not available. Further information necessary.

In the discussion, it is emphasized that if the amount of memory shall be kept the same, the memory access mechanism is probably having some irregular content-dependent condition, dependent on 4x8/8x4/8x8 and uni-/bi prediction.

An update of the contribution is announced including WD text and more clarification about the memory consumption. This was further discussed 01-14 PM.

With a sophisticated implementation, it is true that no additional memory is needed to store the motion parameters. However, additional would be needed 1) for determining the meaning of the two MP sets stored per 8x8 block depending on uni/bi prediction mode 2) for the interpretation of list 0/1 signalling. Several experts expressed concern that instead of that hardware implementations might rather prefer storing uncompressed motion parameters on a 4x4 grid.

No action.

JCT3V-G0149 proposes to utilize depth map for sub-PU level inter-view motion prediction (SPIVMP). While the current SPIVMP utilizes single disparity vector to derive motion parameters for all sub-PUs in single CU, the proposed method derives disparity vector for each sub-PU from depth map as view synthesis prediction does.

#### Coding results of G0149

|  | Video 1 | Video 2 | Video/video BR | Video/total BR | Synthesized/total BR | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| G0149 | -0.18% | -0.04% | -0.04% | -0.03% | -0.01% | 99.7% | 100.1% |

Coding gain very small – does not justify additional complexity.

### CE contributions (6)

JCT3V-G0065 3D-CE3: AMVP candidate list construction for DCP blocks [S. Yoo, T. Kim, J. H. Nam, S. Yea (LGE)]

JCT3V-G0176 CE3: Cross Check of AMVP candidate list construction for DCP blocks (JCT3V-G0065) [F. Jäger (RWTH Aachen University)] [late]

JCT3V-G0100 3D-CE3: Additional depth-based DV candidate [J. Y. Lee, M. W. Park, C. Kim (Samsung)]

JCT3V-G0221 3D-CE3: Cross-check on additional depth-based DV candidate (JCT3V-G0100) [J.-L. Lin, Y.-W. Chen (MediaTek)] [late]

JCT3V-G0106 CE3: Results on Depth-based Block Partitioning (DBBP) [F. Jäger (RWTH Aachen University), J. Konieczny, G. Cordara (Huawei Technologies)]

JCT3V-G0070 3D-CE3 : Crosscheck on Depth block based partitioning from RWTH Aachen University [S. Yoo, S. Yea (LGE)]

### Related contributions (21)

JCT3V-G0034 CE3-related: Worst case complexity reduction for merge candidate construction [T. Ikai (Sharp)]

JCT3V-G0198 3D-CE3 related: Cross check of worst case complexity reduction for merge candidate construction (JCT3V-G0034) [J. Y. Lee, C. Kim (Samsung)] [late]

JCT3V-G0049 3D-CE3 related: Simplification on DV-MCP searching process [K. Zhang, J. An, J.-L. Lin, X. Zhang, Y.-L. Chang, S. Lei (MediaTek)]

JCT3V-G0186 3D-CE3 related: Crosscheck on Simplification on DV-MCP searching process (JCT3V-G0049) [S. Shimizu, S. Sugimoto (NTT)] [late]

JCT3V-G0050 3D-CE3 related: Simplification on NBDV derivation in 3D-HEVC [J.-L. Lin, Y.-W. Chen, K. Zhang, Y.-W. Huang, S. Lei (MediaTek)]

JCT3V-G0156 3D-CE3 related: Crosscheck on Simplification on NBDV derivation in 3D-HEVC (JCT3V-G0050) [S. Shimizu, S. Sugimoto (NTT)] [late]

JCT3V-G0169 3D-CE3 related: Crosscheck of Simplification on NBDV derivation in 3D-HEVC (JCT3V-G0050) [T. Ikai (Sharp)] [late]

JCT3V-G0073 3D-CE3 related: Simplification of DV Derivation and Default DV Improvement [M. W. Park, J. Y. Lee, B. Choi, Y. Cho, C. Kim (Samsung)]

JCT3V-G0228 3D-CE3 related: Cross-check of Simplification of DV Derivation and Default DV Improvement (JCT3V-G0073) [Y.-L. Chang (MediaTek)] [late]

JCT3V-G0078 3D-CE3 related: Adaptive Disabling Temporal Inter-View Candidates [M. W. Park, J. Y. Lee, C. Kim (Samsung)]

JCT3V-G0171 3D-CE3 related: Crosscheck of Adaptive Disabling Temporal Inter-View Candidates (JCT3V-G0078) [T. Ikai (Sharp)] [late]

JCT3V-G0082 CE3 related: Results on Limitation of the Range of NBDV [P. Lu, L. Yu (Zhejiang University)]

JCT3V-G0177 CE3 related: Cross-check results on limitation of the Range of NBDV (JCT3V-G0082) [M. Lu, P. Wu (ZTE)] [late]

JCT3V-G0099 3D-CE3 related: Simplification of 3D Merge list construction [J. Y. Lee, M. W. Park, C. Kim (Samsung)]

JCT3V-G0172 3D-CE3 related: Crosscheck of Simplification of 3D Merge list construction (JCT3V-G0099) [T. Ikai (Sharp)] [late]

JCT3V-G0104 CE3 related: Result on clean ups of VSP merging candidates [Y. Zhang, L. Yu (Zhejiang University)]

JCT3V-G0225 Crosscheck results on clean ups of VSP merging candidates (Method 1 & 2 in JCT3V-G0104) [M. Li, P. Wu (ZTE)] [late]

JCT3V-G0230 CE3 related: Crosscheck on ZJU's proposal "Result on clean ups of VSP merging candidates (JCT3V-G0104)" [X. Zheng (HiSilicon)] [late]

JCT3V-G0128 CE3 related: Motion storage optimization in 3D-HEVC [Y. Chen (Qualcomm), Q. Yu, S. Ma (PKU)]

JCT3V-G0194 CE3 related: Crosscheck on Qualcomm and PKU's proposal "Motion storage optimization in 3D-HEVC (JCT3V-G0128)" [X. Zheng (HiSilicon)] [late]

JCT3V-G0149 CE3-related: Depth-based motion field synthesis for sub-PU level inter-view motion prediction [S. Shimizu, S. Sugimoto (NTT)]

## CE4: Residual prediction (21)

### Summary (1)

JCT3V-G0024 CE4 Summary Report: Residual Prediction [L. Zhang, J.-L. Lin]

Complexity Reduction

JCT3V-G0158: it is claimed to simplify the inter-view ARP by utilizing the following aspects:

- The temporal motion information used in inter-view ARP is derived using the same way as shifted temporal inter-view merging candidate derivation process. This would be a unification of the MV derivation process with the existing MV candidate derivation process. It was stated that complexity increases since there would be an increase in checking of the reference picture list. Editorial improvements to the test were encouraged.

- The residual and residual predictor are calculated as temporal residuals from current view, and the reference view, respectively. It was asserted that two branches in the ARP process could be unified into a single branch, but there is some difference of opinion on this. There is also some minor loss with this proposal.

- The disparity vector from DoNBDV process is updated to the coded disparity motion vector associated with current block. Not relevant since DoNBDV is not used in ARP. This aims to improve coding efficiency and could be considered separately.

- Removal of the constraint of DV-MCP: It is proposed that in NBDV derivation process, the DV-MCP information of spatial neighbouring blocks are always checked even they are not coded as skip mode. Further study of this aspect without DoNBDV, but not needed to study as part of CE.

- Combination of inter-view and temporal ARP: The DVs from NBDV and DoNBDV processes are both updated to the disparity motion vector associated with current block, if available. The updated disparity vectors are used in ARP process. It is not clear that this is a simplification since the current ARP does not use DoNBDV.

No action on any of the above aspects.

JCT3V-G0033: This contribution proposes to disable ARP for 4x4 chroma blocks. Simulation results show that the proposed method introduces 0.1% coding loss for non-base texture views and minor coding loss for coded texture views (0.03%).

Complexity analysis shows that worst case bandwidth for ARP is 22% higher that HEVC v1 with 8x8 bi-prediction case. By disabling ARP for 4x4 chroma blocks, the worst case is then defined by 16x16 ARP; there is only a 5% relative increase in memory bandwidth for this. It was asserted by the proponent that this is a more appropriate trade-off between complexity and coding efficiency.

A detailed analysis is provided in the contribution and was reviewed. It was mentioned that there is some mismatch with earlier analysis (22% in the contribution versus 15% in an earlier analysis). The difference may be due to differences in memory access pattern assumptions. The proposed simplification was agreed, but some offline discussions will be done to clarify.

Further study in CE (together with G0121).

Coding Efficiency

JCT3V-G0064: In this contribution, two aspects are proposed to improve the coding efficiency of the advanced residual prediction. The first part was in the scope of CE:

Motion vector candidate lists for ARP (MV-Cand ARP): three motion vector candidates (disparity motion vectors for inter-view ARP and temporal motion vectors for temporal ARP) are derived from temporal/spatial neighbouring blocks. Spatial neighbouring blocks are the top and left blocks of current block while temporal neighbouring blocks are those used in NBDV or the TMVP process. For temporal ARP, the DoNBDV results of current block and spatial neighbouring blocks may be included. The best motion vector candidate index is selected according to RDO criterion and signaled in the bitstream.

It is further proposed to add two more candidates for inter-view ARP based on Aligned Temporal DV (ATDV), which considers a new temporal motion vector candidate, i.e., ATDV, is obtained from the aligned block, which is located by a scaled MV to the collocated picture. Two collocated pictures used in the NBDV derivation may be used to derive the DMV candidates. ATDV is checked before DV candidates from neighbouring blocks when it is used. Note that this item was not concluded as part of CE.

There was concern about complexity of ATDV in the previous meeting; as a result, it was decided not to study this aspect in the CE. There is no new information since the last meeting, so the earlier decision stands. No action.

JCT3V-G0121: The techniques to improve the coding efficiency of ARP in this proposal include two aspects in the CE:

1)	Block-level ARP: block-level ARP is proposed by splitting one prediction unit into several blocks to perform ARP separately. Furthermore, the residual of chroma components is not coded if the current PU is coded with ARP;

2)	Disable IC when ARP is enabled: the ic_flag is not signalled when the ARP is enabled (i.e., ARP weighting factor is unequal to 0); (same as #1 in JCT3V-G0072)

JCT3V-G0072: In this contribution, two aspects are proposed:

1)	Disable the IC when ARP is enabled, (i.e., iv_res_pred_weight_idx is greater than 0. (same as #2 in JCT3V-G0121); and

2)	Disable ARP when illumination compensation (IC) is enabled, i.e., ic_flag is equal to 1.

It was noted that coding efficiency is not the most critical aspect for this proposal. It is more important to clean up the syntax tables. A preference for the first option was expressed, i.e., to disable IC when ARP is enabled. This was agreed and supported by others.

Decision: Adopt (disable IC when ARP is enabled)

Summary of CE results compared to 3D-HTM

| Topics | Proposals |  |  |  | Simulation Results |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Video 1 |  | Video 2 |  | Video PSNR / Video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc. time | Dec. time |
| MV-Cand ARP with 3 weighting factors | JCT3V-G0064* | -0.4% |  | -0.4% |  | -0.2% | -0.2% | -0.1% | 110% | 99% |
| MV-Cand ARP with 2 weighting factors | JCT3V-G0064* | -0.5% |  | -0.5% |  | -0.2% | -0.2% | -0.1% | 103% | 100% |
| Block-level ARP | JCT3V-G0121 | -0.5% |  | -0.7% |  | -0.2% | -0.2% | -0.2% | 104% | 102% |
| disable IC when ARP is enabled | JCT3V-G0072 | -0.1% |  | -0.1% |  | -0.1% | -0.1% | 0.0% | 98% | 99% |
|  | JCT3V-G0121 | -0.1% |  | -0.1% |  | -0.1% | -0.1% | 0.0% | 98% | 100% |
| disable ARP when IC is enabled | JCT3V-G0072 | -0.1% |  | -0.1% |  | 0.0% | 0.0% | 0.0% | 98% | 99% |

Block-level ARP gives the highest gain among the proposed methods. There was a question on how much each aspect of the proposal contributes, i.e., whether or not to code the residual of chroma components if the current PU is coded with ARP.

It was noted that block-level ARP would incur the worst case for 8x8, but this is not considered to be a concern since it is used in combination with sub-PU prediction.

For first aspect of G0121 (inclusion of block level ARP in merge candidate list), additional simulation results with only block-level ARP were requested. It is reported in v2 of G0121 that the gain is still 0.2%.

One expert points out that the current implementation of this first aspect of G0121 would be in contradiction with the simplification that was proposed in G0033, and G0121 would make ARP more complex on average.

Further study of the first aspect of G0121 and G0033 in a CE.

Further study second aspect of G0121 (remove CBF of chroma component) in CE was initially suggested. In a follow-up discussion after the results about using only block-level ARP were presented, it appears that the gain by this is rather negligible, which can be interpreted such that CABAC appropriately encodes the case that non-zero chroma blocks rarely occur.

JCT3V-G0095: This related contribution proposes a disparity motion vector candidate list for temporal ARP. The list includes two candidates, one is the disparity vector from NBDV process and the other is the disparity vector from DoNBDV process. The best candidate is signalled in the bitstream. The results indicate 0.l% bit rate reduction on coded and synthesized views. No action.

### CE contributions (12)

JCT3V-G0064 3D-CE4: Results on improved advanced residual prediction [K. Zhang, J. An, J.-L. Lin, Y.-L. Chang, S. Lei (MediaTek)]

JCT3V-G0167 3D-CE4: Crosscheck of Results on improved advanced residual prediction (JCT3V-G0064) [T. Ikai (Sharp)] [late]

JCT3V-G0222 3D-CE4: Cross check of Improved advanced residual prediction (JCT3V-G0064) [L. Zhang] [late]

JCT3V-G0072 3D-CE4: Results on IC and ARP Flags Signaling [M. W. Park, J. Y. Lee, C. Kim (Samsung)]

JCT3V-G0075 Crosscheck on IC and ARP Flags Signaling from Samsung (JCT3V-G0072) [S. Yoo, S. Yea (LGE)] [late]

JCT3V-G0121 CE4: Further improvements on advanced residual prediction [L. Zhang, Y. Chen, M. Karczewicz (Qualcomm), Q. Yu, S. Ma (PKU)]

JCT3V-G0204 CE4: Crosscheck for Qualcomm's JCT3V-G0121 [K. Zhang, J. An, X. Zhang (MediaTek)] [late]

JCT3V-G0220 3D-CE4: Cross check of Further improvements on advanced residual prediction (JCT3V-G0121) [M. W. Park, C. Kim (Samsung)] [late]

JCT3V-G0226 CE4: Crosscheck for Qualcomm's JCT3V-G0121 [J. Heo, J. Nam (LGE)] [late]

JCT3V-G0158 3D-CE4: Simplification of inter-view ARP [S. Sugimoto, S. Shimizu (NTT)]

JCT3V-G0223 3D-CE4: Cross check of Simplification of inter-view ARP (JCT3V-G0158) [L. Zhang] [late]

JCT3V-G0231 3D-CE4.h: Cross-check on simplification of inter-view ARP [Y.-L. Chang, K. Zhang (MediaTek)] [late]

### Related contributions (8)

JCT3V-G0033 CE4-related: ARP simplification [T. Ikai (Sharp)]

JCT3V-G0041 CE4 related: Crosscheck of Sharp's proposal on ARP simplification (JCT3V-G0033) [P. Lu, L. Yu (Zhejiang University)]

JCT3V-G0076 3D-CE4 related: Residual Prediction for View Synthesis Prediction [M. W. Park, J. Y. Lee, C. Kim (Samsung)]

see also in CE1

JCT3V-G0154 3D-CE4 related: Crosscheck on Residual Prediction for View Synthesis Prediction (JCT3V-G0076) [S. Shimizu, S. Sugimoto (NTT)]

JCT3V-G0095 3D-CE4 related: An alternative disparity vector derivation method for ARP [J.-S. Tu, C.-C. Lin]

JCT3V-G0187 3D-CE4.h related: Crosscheck on an alternative disparity vector derivation method for ARP (JCT3V-G0095) [G. G. Lee, B.-S. Li, C.-F. Chen, Z.-H. Yu, C.-H. Huang (NCKU)] [late]

JCT3V-G0057 3D-AHG5: On complexity reduction of bi-prediction for advanced residual prediction [Y.-W. Chen, J.-L. Lin, Y.-W. Huang, S. Lei (MediaTek)]

JCT3V-G0168 3D-AHG5: Crosscheck of On complexity reduction of bi-prediction for advanced residual prediction (JCT3V-G0057) [T. Ikai (Sharp)] [late]

## CE5: Depth intra modes (40)

### Summary (1)

JCT3V-G0025 CE5 Summary Report: Depth Intra Modes [H. Liu]

Coding Efficiency

JCT3V-G0101: Segment-wise depth inter mode coding, proposes three aspects to improve the inter SDC.

- Simplification of the inter SDC:

  - Only apply inter SDC to CUs that are coded with 2Nx2N partition mode.

  - Allow DC residual in inter SDC mode to be zero.

- Improvement of the DC residual: search the best DC residual from {deltaDC – 1, deltaDC, deltaDC + 1} through view synthesis optimization process at encoder, where deltaDC refers to the DC residual generated in inter SDC mode in 3D-HTM 9.0r1.

- Improvement of the DC residual: search the best DC residual from {deltaDC – 2, deltaDC – 1, deltaDC, deltaDC + 1, deltaDC + 2} through view synthesis optimization process at encoder, where deltaDC refers to the DC residual generated in inter SDC mode in 3D-HTM 9.0r1.

Item #2 and item #3 are non-normative methods. Among these two options, it seems preferred to get the maximum gain with a larger search.

For #1b, the normative change is start the residual coding from 0 rather than 1 for inter SDC. There is minor coding loss and benefit is not clear.

Support was expressed to adopt the simplification in #1a and the non-normative increase in search space in #3.

Decision: Adopt (#1a + #3)

JCT3V-G0058: This contribution is based on JCT3V-G0101, which allows the DC residual to be equal to zero in the inter SDC mode. The main difference is that it proposes to keep the non-zero constraint of the DC residual magnitude in 3D-HEVC to remove the overlap between the skip mode and the inter SDC mode. This gives an additional 0.07% gain, which is rather small.

WD text was reviewed. An additional branch is added based on merge mode. Although the concept is simple, the syntax change is more than expected.

No action.

JCT3V-G0122: Generic SDC for all Intra modes in 3D-HEVC, this contribution proposes two aspects to improve the intra SDC coding. One item is in the scope of the CE.

- Generalization of the intra SDC: generalize intra SDC to all depth intra prediction modes.

One item of this proposal is considered as outside of the CE scope.

- Simplification of the depth mode parameter syntax table: enable DMM4 for 4x4 blocks to further simplify the depth mode parameter syntax table.

As noted in the table below, #1 gives 0.3% coding gain and there is negligible loss from the proposed simplification.

WD text was reviewed, and there was support to adopt the syntax.

It was remarked that the all-intra complexity is relatively high: 20% increase without the encoder optimization, and 47% with the encoder optimization.

Decision: Adopt (with optimization in CTC)

G0102: related to G0122, and proposes to only apply intra SDC to modes that are included in MPM list or DMM modes in order to reduce the encoder complexity. It was remarked that G0122 may be simplified using non-normative techniques and signaling proposed in the G0122 design. It was believed that this provides more flexibility and the benefits of constraining the design are unclear.

Further study of design could be considered, clarifying the benefits of a constrained design.

Summary of CE proposals on coding efficiency

| With encoder optimization | Proposals | Simulation results |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Video PSNR / Video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc. time |  | Dec. time |
| N | JCT3V-G0122 #1 | 0.00% | -0.25% | -0.30% | 101.1% |  | 101.8% |
| N | JCT3V-G0122 #1 (AI) | 0.00% | -0.35% | -0.46% | 120.7% |  | 92.8% |
| N | JCT3V-G0101 #1 | -0.01% | -0.02% | 0.06% | 98.5% |  | 99.5% |
| N | JCT3V-G0101 #1 (AI) | n/a | n/a | n/a | n/a |  | n/a |
| Y | JCT3V-G0122 wo #2 | 0.01% | -0.28% | -0.50% | 104.1% | 100.7% |  |
| Y | JCT3V-G0122 | -0.01% | -0.29% | -0.49% | 104.1% | 98.1% |  |
| Y | JCT3V-G0101 #1+#2 | -0.01% | 0.09% | -0.31% | 102.6% | 99.9% |  |
| Y | JCT3V-G0101 #1+#3 | -0.01% | 0.14% | -0.46% | 108.2% | 97.8% |  |
| Y | JCT3V-G0122 wo#2 (AI) | 0.00% | -0.40% | -0.93% | 147.5% | 89.8% |  |
| Y | JCT3V-G0122(AI) | 0.00% | -0.39% | -0.95% | 149.8% | 92.1% |  |
| Y | JCT3V-G0101 (AI) | n/a | n/a | n/a | n/a | n/a |  |

Simplifications

JCT3V-G0113: This contribution proposes two methods to remove the 64x64 intra prediction in SDC planar coding.

1)	The sub-sampled (2:1 sub-sampling) neighboring samples are used to generate a sub-sampled prediction block, which is added to the residual to generate a sub-sampled reconstruction block. Then, each pixel of the sub-sampled reconstruction block is used to fill a corresponding 2x2 block with in the 64x64 block.

2)	The first 32 samples from the left-column or the top-row neighboring samples are used to generate a sub-sampled prediction block, which is added to the residual to generate a sub-sampled reconstruction block. Then, each pixel of the sub-sampled reconstruction block is used to fill a corresponding 2x2 block with in the 64x64 block.

Meanwhile, removal of the 64x64 intra prediction is also tested, in which an option that disables SDC planar mode for 64x64 intra coded PU. This is just for information.

The proponent of G0113 states that this method gives the highest coding efficiency, but the related proposals in G0111/0123 are preferred since they are more closely aligned with HEVC v1.

No action.

JCT3V-G0111: CE5 related: Implicit split process for intra SDC. This contribution proposes to split 64x64 intra prediction in 3D-HEVC into four 32x32 intra prediction (this is for CTC, it can also splits any block size larger than (1<< log2TrafoSize) into multiple blocks with size (1<< log2TrafoSize)) in a similar way with the intra prediction process in HEVC. Meanwhile, it proposes to encode one DC residual for each 32x32 block in this case.

JCT3V-G0123: CE5 related: Simplification of 64x64 Intra SDC mode in 3D-HEVC. This difference with G0111 is that this proposal encodes one DC residual for the 64x64 block rather than encoding one DC residual for each 32x32 block.

A preference was expressed for G0123 since it has less coding loss, and in the case of split, it is identical to G0111. When SDC coding size is greater than the maximum block size, split mode is used.

Decision: Adopt G0123 with split method from G0111.

JCT3V-G0056: This contribution proposes three aspects to reduce the bins used for the inter SDC residual coding.

1)	Binarization and entropy coding of the DC residual: the DC residual is binarized using truncated unary code and only the first bin is coded with context model while the remaining bins are bypass coded.

2)	Constraint of the DC residual magnitude: constrain the DC residual magnitude to be no larger than 3.

3)	Constraint of the DC residual magnitude: constrain the DC residual magnitude to be no larger than 5.

This proposal is related AHG8.

There are currently 13 context coded bins; this proposal reduces this to 1.

It was remarked that solving this bottleneck for inter does not completely address the issue.

Additional remarks suggested that it is not desirable to constrain the DC residual magnitude.

JCT3V-G0124: CE5 related: Delta DC coding for SDC and DMM modes. This contribution proposes four binarization methods for the delta DC magnitude. Meanwhile, when encoding the delta DC magnitude, bins coded with context model are reduced to be no more than nine.

In contrast to G0056, this method applies to both intra and inter. The total context coded bins is reduced from 13 to 9, which is the same as HEVC v1.

BoG to clarify the benefits of G0056, G0108 and G0124 relative the current HEVC and 3D-HEVC designs. It would be desirable to also consider AHG related proposals. 
Coordinators: P. Merkle and J.L Lin.

DMM

JCT3V-G0112: Test result on simplification of DMM pattern generation and signaling. This contribution proposes an alternative DMM pattern generation and signaling method. A prediction block can have two types of partitions: rectangle partition and non-rectangle partition. For rectangle partition, it can be further classified to horizontal pattern and vertical pattern. For non-rectangle case, the partitions can be categorized by the partition line’s slope and orientation (left-down or right-up). Nine slope values including 1:1, 2:3, 3:2, 2:1, 1:2, 3:1, 1:3, 6:1 and 1:6 are allowed. For both rectangle partition and non-rectangle partition, an offset is encoded to indicate the distance between the partition line and central partition line. The central partition line has the same slope and orientation as the partition line, and divides the prediction block into two partitions with equal size.

The purpose of this proposal is to simplify the pattern generation of DMM. It incurs a loss of 0.02% for the CTC case and 0.16% for all-intra. It was noted that some sequences lose as much as 0.9%. There is some concern about the loss of precision, which could make the mode less useful. Even with additional slopes tested in the CE, the same losses are observed.

The proponent expresses concern about the storage size of the current DMM design, but there are alternative implementations that could overcome the storage issues, e.g., generating the patterns on the fly.

No action.

Additional Related Contributions

JCT3V-G0051: This contribution proposes to disable the boundary filter of DC mode and the gradient based boundary filter of vertical and horizontal intra prediction modes in depth picture coding.

JCT3V-G0052: This contribution proposes to disable the neighbouring samples filters including the [1, 2, 1] smoothing filter and the bi-linear interpolation filter in depth picture coding.

JCT3V-G0054: This contribution proposes to disable the intra interpolation in depth coding for all intra prediction directions. Instead, the predicted sample is set to the nearest sample of the pointed position along the intra prediction direction.

None of the above three proposals show any notable change in coding efficiency. There were remarks indicating that it would be preferred to keep the HEVC v1 process intact.

No action on G0051, G0052 and G0054.

### CE contributions (11)

JCT3V-G0056 3D-CE5: Results on bin reduction for inter SDC residual coding [Y.-W. Chen, J.-L. Lin, Y.-W. Huang, S. Lei (MediaTek)]

JCT3V-G0190 CE5: Crosscheck on MediaTek's proposal "Results on bin reduction for inter SDC residual coding (JCT3V-G0056)" [X. Zheng (HiSilicon)] [late]

JCT3V-G0101 3D-CE5: Segment-wise depth inter mode coding [J. Y. Lee, M. W. Park, C. Kim (Samsung)]

JCT3V-G0216 CE5: Crosscheck on Segment-wise depth inter mode coding (JCT3V-G0101) [H. Liu (Qualcomm)] [late]

JCT3V-G0112 CE5: Test result on simplification of DMM pattern generation and signaling [W. Li (Tsinghua Univ.), X. Zheng, J. Zheng (Hisilicon), Y. He (Tsinghua Univ.)]

JCT3V-G0199 3D-CE5: Cross check of simplification of DMM pattern generation and signaling (JCT3V-G0112) [J. Y. Lee, C. Kim (Samsung)] [late]

JCT3V-G0113 CE5: Test results on simplification on SDC planar coding [X. Zheng, Y. Lin, X. Chen, J. Zheng (Hisilicon)]

JCT3V-G0181 CE5: Crosscheck on simplification on SDC planar coding (JCT3V-G0113) [Y.-W. Chen, J.-L. Lin (MediaTek)] [late]

JCT3V-G0122 CE5: Generic SDC for all Intra modes in 3D-HEVC [H. Liu, Y. Chen (Qualcomm)]

JCT3V-G0184 CE5: Cross check of generic SDC for all Intra modes (JCT3V-G0122) [P. Merkle (HHI)] [late]

JCT3V-G0209 CE5: Crosscheck for Qualcomm's JCT3V-G0122 [X. Chen (HiSilicon)] [late]

### Related contributions (28)

JCT3V-G0051 3D-CE5 related: Removal of boundary filters for depth intra prediction [J.-L. Lin, Y.-W. Chen, X. Zhang, Y.-W. Huang, S. Lei (MediaTek)]

JCT3V-G0200 3D-CE5 related: Cross check of removal of boundary filters for depth intra prediction (JCT3V-G0051) [J. Y. Lee, C. Kim (Samsung)] [late]

JCT3V-G0052 3D-CE5 related: Removal of neighbouring sample filtering for depth intra prediction [J.-L. Lin, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek)]

JCT3V-G0188 CE5 related: Crosscheck on MediaTek's proposal "Removal of neighbouring sample filtering for depth intra prediction (JCT3V-G0052)" [X. Zheng (HiSilicon)] [late]

JCT3V-G0054 3D-CE5 related: Simplified Angular Intra Prediction for Depth Coding in 3D-HEVC [X. Zhang, K. Zhang, J. An, J.-L. Lin, S. Lei (MediaTek)]

JCT3V-G0189 CE5 related: Crosscheck on MediaTek's proposal "Simplified Intra Prediction for Depth Coding in 3D-HEVC (JCT3V-G0054)" [X. Zheng (HiSilicon)] [late]

JCT3V-G0058 3D-CE5 related: Conditional zero residual for segment-wise depth inter mode coding [Y.-W. Chen, J.-L. Lin, Y.-W. Huang, S. Lei (MediaTek), J.-Y. Lee, M.-W. Park (Samsung)]

JCT3V-G0173 3D-CE5 related: Crosscheck of Conditional zero residual for segment-wise depth inter mode coding (JCT3V-G0058) [T. Ikai (Sharp)] [late]

JCT3V-G0083 CE5 related: Results on Simplification on DLT-based Depth Intra Coding [P. Lu, L. Yu (Zhejiang University)]

This contribution proposes to skip the transform for DLT-based depth residual coding. The method incurs a minor coding loss of 0.02% for CTC and 0.05% for all-intra.

Proponents remark that they aim to reduce the worst case complexity, but there is no convincing analysis that this is true. It was suggested that this might be considered for 4x4 only. Further study may be considered including more detailed analysis of complexity.

JCT3V-G0097 CE5 related: Crosscheck on Simplification on DLT-based Depth Intra Coding from Zhejiang University (JCT3V-G0083) [J.Seo, J.Heo, S.Yea (LGE)] [late]

JCT3V-G0094 CE5 related: Advanced boundary chain (ABC) coding for depth intra coding [J.Seo, J.Nam, J.Heo, S.Yea (LGE)]

This contribution proposes an advanced boundary chain (ABC) coding method to replace DMM4 for depth intra coding. When compared with the region boundary chain (RBC) coding that was removed at the last meeting, the ABC removes two traverse types, i.e., 135 degree and -135 degree and only use five traverse types including 0, 45, -45, 90, and -90 degrees.

The proposal aims to remove inter-component correlation between texture and depth. There are four steps: find internal edges, code the edges using chain codes, convert chain codes into bitstream syntax, and calculate predictors and fill them in the block. One difference from the previous RBC method is that the chain codes are generated based on their frequency.

A coding gain of 0.25% on CTC is reported, but only 0.08% on all-intra. Typically, we would expect higher gain for the all-intra case so it is believed that there may be some inaccuracy with the results. Another explanation is that some CABAC optimizations were performed that have a broader impact on the results.

The performance of the proposed method combined with DMM4 has not been evaluated.

There were also questions on the flexibility of the proposed design, i.e., whether it would handle all types of patterns.

No action.

JCT3V-G0229 CE5 related: Crosscheck on Advanced boundary chain (ABC) coding for depth intra coding (JCT3V-G0094) [S. Shimizu, S. Sugimoto (NTT)] [late]

JCT3V-G0102 3D-CE5 related: Segment-wise depth intra mode coding [J. Y. Lee, M. W. Park, B. Choi, Y. Cho, C. Kim (Samsung)]

JCT3V-G0217 CE5 related: Crosscheck on Segment-wise depth intra mode coding (JCT3V-G0102) [H. Liu (Qualcomm)] [late]

JCT3V-G0103 3D-CE5 related: Depth quantization in intra residual domain [J. Y. Lee, M. W. Park, B. Choi, Y. Cho, C. Kim (Samsung)]

This contribution proposes to quantize depth in residual domain before the transform and the quantization, and this method applies to all modes except DC, vertical and horizontal modes. Meanwhile, it disables DLT-based residual coding for DMM modes. It is reported that the proposed method gives 0.09% bit rate reduction on synthesized views for CTC and 0.4% bit rate reduction for coded video for the all-intra case.

Several experts expressed support to study this method further. It was suggested that similar results might be achieved with a DLT-based approach, which can be studied as part of the CE.

Further study. Since this is the only proposal in this area, it was agreed to study outside of a CE. Involvement with other experts was encouraged.

JCT3V-G0182 3D-CE5 related: Crosscheck on Depth quantization in intra residual domain (JCT3V-G0103) [Y.-W. Chen, J.-L. Lin (MediaTek)] [late]

JCT3V-G0108 CE5 related: DMM1 signaling based on intra direction [P. Merkle, K. Müller (HHI)]

JCT3V-G0213 CE5 related: Crosscheck on DMM1 signaling based on intra direction (JCT3V-G0108) [H. Liu (Qualcomm)] [late]

JCT3V-G0111 CE5 related: Implicit split process for intra SDC [W. Li (Tsinghua Univ.), X. Zheng, J. Zheng (Hisilicon), Y. He (Tsinghua Univ.)]

JCT3V-G0205 CE5 Related: Crosschecking for HiSilicon's Implicit split process for intra SDC JCT3V-G0111 [X. Zhang, K. Zhang, J. An (MediaTek)] [late]

JCT3V-G0123 CE5 related: Simplification of 64x64 Intra SDC mode in 3D-HEVC [Q. Yu, H. Liu, Y. Chen (Qualcomm), S. Ma (PKU)]

JCT3V-G0193 CE5 related: Crosscheck on Qualcomm and PKU's proposal "Simplification of 64x64 Intra SDC mode in 3D-HEVC (JCT3V-G0123)" [X. Zheng (HiSilicon)] [late]

JCT3V-G0124 CE5 related: Delta DC coding for SDC and DMM modes [Q. Yu, H. Liu, Y. Chen, L. Zhang (Qualcomm), S. Ma (PKU)]

JCT3V-G0178 CE5 related: Crosscheck of Delta DC coding for SDC and DMM modes (JCT3V-G0124) [J. Nam, J. Heo, S. Yea (LGE)] [late]

JCT3V-G0130 CE5 related: Unification of delta DC coding and signaling for inter SDC and intra SDC in 3D-HEVC [H. Liu, Y. Chen (Qualcomm)]

This contribution builds on JCT3V-G0122. It proposes to unify delta DC coding and the signaling for inter SDC and intra SDC. The delta DC coding method of intra SDC is extended to be applied to inter SDC in the same way. Meanwhile, instead of implicitly signalling the intra SDC flag in the depth intra mode, the intra SDC flag is explicitly signalled in the same way as in inter SDC.

There were remarks from several experts that this unification seems quite straightforward and natural. The changes have no impact on coding efficiency.

Decision: Adopt

JCT3V-G0175 CE5-related: Cross Check of Unification of delta DC coding and signaling for inter SDC and intra SDC in 3D-HEVC (JCT3V-G0130) [F. Jäger (RWTH Aachen University)] [late]

JCT3V-G0143 3D-CE5 related: On neighbouring reference pixel selection for depth intra coding [Z. Gu (SCU), J. Zheng (HiSilicon), N. Ling ((SCU), P. Zhang (HiSilicon)]

This contribution proposes to use above right or below left neighboring pixel to predict the DC values for partitions (in DMM modes) that does not have any neighboring left or above pixel. Meanwhile, it proposes to remove the filtering process of the neighboring samples when the PU size is smaller than 16 and the PU is coded with HEVC intra modes. The results indicate a 0.1% bit rate reduction for both CTC and all-intra cases on synthesized views.

Question: how much gain comes from DC prediction? Most of the gain comes from this.

A similar proposal was considered at the last meeting and it was remarked that some additional complexity may be incurred. The proponent notes that the method is using information that is already available.

Based on prior discussions, it was suggested that no action be taken on the proposed changes to the filtering process.

There was support among several experts to adopt this method since it gives some gains and complexity impact is very minor.

Decision: Adopt

JCT3V-G0206 CE5 related: Crosschecking for HiSilicon's neighbouring reference pixel selection for depth intra coding, JCT3V-G0143 [X. Zhang, K. Zhang, J. An (MediaTek)] [late]

# 3DV standards development (incl. software, conformance) (XX)

## MVC plus depth (XX)

## 3D-AVC (XX)

## MFC (XX)

## MV-HEVC (XX)

During the joint meeting of JCT-VC and JCT-3V Sun 2 pm it was decided that for progression of RExt and MV-HEVC the documents should not be balloted separately as FDAMs in April, but rather integrate them into a new edition and ballot that edition after the July meeting. This would allow consistent alignment.

## 3D-HEVC (XX)

# High-level syntax (43)

## MV-HEVC/3D-HEVC (2)

JCT3V-G0048 3D-HEVC HLS: On inter-component reference pictures [Y.-L. Chang, Y.-W. Chen, J.-L. Lin, Y.-P. Tsai, S. Lei (MediaTek)]

In the current 3D-HEVC, the depth oriented coding tools – depth oriented neighboring block disparity vector (DoNBDV), view synthesis prediction (VSP) and motion parameter inheritance (MPI) – all need to access a picture in other component (e.g. texture picture refers to depth picture) to derive a sample array or predictors. It is assumed that the reference pictures of the other component can be found and accessed by default. However, the reference picture of the other component is implicitly used in the 3D-HTM but not clearly defined in the working draft text. The availability and location of the reference picture of the other component are not described for 3D-HEVC. It is proposed to derive an inter-component reference picture set and use the inter-component reference picture set to indicate the availability and the location of the inter-component reference picture in this contribution.

Discussion: Texture and depth are in same access unit, therefore the availability is not a problem. The suggested inter component reference picture set might only be useful when several depth maps would be associated with one texture, or several texture views would be associated with one depth map. The later case could hypothetically apply when scalable texture coding would be combined with non-scalable depth coding. This currently not under consideration, and could probably resolved when needed.

No action.

JCT3V-G0053 3D-HEVC HLS: On ARP reference picture signaling [J.-L. Lin, Y.-W. Chen, Y.-L. Chang, Y.-W. Huang, S. Lei (MediaTek)]

In advanced residual prediction (ARP), the first temporal reference picture in the reference picture list is selected as the reference picture of the corresponding block in the reference view. A change of reference list reordering between slides in a same picture could cause the reference picture of the reference view to change. Frequently changing the reference picture for ARP will break the hardware pipeline design and causes a burden for decoders due to the DRAM access limitations. This contribution simply proposes to prohibit changes to the reference picture for ARP within a picture. An additional benefit of more flexibility in the reference picture selection for ARP is also provided.

Discussion: Solution 2 would be preferred, allowing only the picture with smallest POC distance to current picture in case of ARP.

Does not take influence on CTC.

Decision: Adopt. Revisit: WD text to be confirmed by Y. Chen

JCT3V-G0129 AHG5: Comments on default NBDV view selection and ARP target reference picture [Y. Chen, L. Zhang (Qualcomm)]

This document contributes 1) a clean-up for the derivation of default reference view order index; 2) a bug fix for the derivation process of target reference index for ARP.

No agreement on first aspect.

Decision (BF): Adopt second aspect.

## Common REXT/MV-HEVC/SHVC (41)

The documents in this section were discussed in joint AHG/BoG sessions with JCT-VC experts. For a documentation about these discussions and conclusions, refer to the report JCTVC-O1000 and BoG report JCT3V-G0233. Decisions and adoptions for HLS of joint interest for MV-HEVC and SHVC were made in a joint meeting of JCT-VC and JCT-3V on Sunday 12th Jan. 2-3 PM. Adoptions are also documented in section 14 of this report.

Follow-up meeting Tue 11-13. GS reports about further suggestions of the BoG: Aspects of DPB and sub-layer DPB sharing may have impact on multi-view. This was further discussed and no concerns were raised from the JCT-3V side about the flexible approach (allowing to define either separate or sub-layer shared DPB dependent on profile/level) by JCT-VC.

JCT3V-G0233 Joint BoG report on High Level Syntax [J. Boyce]

Presented and further discussed in joint JCT-VC/3V plenary Sunday 01-12 am (JRO & GJS).

The suggested plan for publication for ISO/IEC was described as follows:

- Edited DAM or FDAM considered issued in April for RExt and MV-HEVC, but not balloted to enable preparation of new FDIS.

- Edited DAM or FDAM considered issued in July for SHVC, but not balloted.

- FDIS of new edition issued in July with all three amendments integrated, and balloted.

(Consent in July of full text new edition.)

Decision: The BoG recommended, and the JCT-VC/3V, endorsed, the following actions:

- Remove profile_ref_minus1 from the VPS extension, from JCTVC-G0040

- Move video signal information syntax structure earlier in the VPS VUI, from JCTVC-G0090

- Not signal the sps_max_num_reorder_pics[], sps_max_latency_increase_plus1[], and sps_max_dec_pic_buffering_minus1[] syntax elements in the SPS when nuh_layer_id > 0, from JCTVC-G0144.

- Add PPS extension type flags for conditional presence of syntax extensions per extension type, aligned with the SPS extension type flags, from JCTVC-P0166. Further align the SPS extension type flags syntax between RExt and MV-HEVC/SHVC.

- Modification of derivation of variable NumActiveRefLayerPics, subject to review of the revised text by interested experts, from JCTVC-G0092.

- Require that end of bitstream NAL unit shall have nuh_layer_id equal to 0, from JCTVC-G0131. Decoders shall allow an end of bitstream NAL unit with nuh_layer_id > 0 to be present, and shall ignore the NAL unit.

- Add constraint restricting pictures marked as discardable from being present in the temporal or inter-layer RPS, from JCTVC-G0131.

The BoG recommended the following activities take place:

- To further discuss JCTVC-G0116 to select between two options to enable no default output layer sets (See notes for P0110 in JCT-VC report, which recorded the decision in the follow-up joint meeting Tue 11).

Decision (Ed.): The BoG recommended, and the JCT-VC/3V endorsed, the following suggestions to the editors:

- Improve or add definitions in the MV-HEVC and SHVC specifications for layer sets, target output layers, output layer sets, and consider adding explanatory notes

- Delegate to the editors aspects raised by the following contributions: JCTVC-G0044, JCTVC-G0091, JCTVC-G0144, JCTVC-G0159, JCTVC-G0131

Further BoG activity was planned.

### Summaries of documents

### Contributions

JCT3V-G0031 MV-HEVC/SHVC HLS: On picture order count [Hendry, A. K. Ramasubramonian, Y.-K. Wang, Y. Chen (Qualcomm), M. Li, P. Wu (ZTE)]

JCT3V-G0224 MV-HEVC/SHVC HLS: Additional information on the POC design in JCTVC-P0041/JCT3V-G0031 [A. K. Ramasubramonian, Hendry, Y.-K. Wang (Qualcomm)] [late] [miss]

JCT3V-G0032 Version 1/MV-HEVC/SHVC HLS: Access unit boundary detection [M. M. Hannuksela (Nokia)]

JCT3V-G0037 MV-HEVC/SHVC HLS: On layer set definition [T. Ikai, T. Tsukuba, T. Yamamoto (Sharp)]

JCT3V-G0038 MV-HEVC/SHVC HLS: Additional layer set [T. Ikai, T. Tsukuba, T. Yamamoto (Sharp)]

JCT3V-G0039 MV-HEVC/SHVC HLS: On sub-bitstream extraction [T. Tsukuba, T. Yamamoto, T. Ikai (Sharp)]

JCT3V-G0040 MV-HEVC/SHVC HLS: Syntax clean-up of profile, tier and level information [T. Tsukuba, T. Yamamoto, T. Ikai (Sharp)]

JCT3V-G0042 MV-HEVC/SHVC HLS: Layer-tree POC [M. M. Hannuksela (Nokia)]

JCT3V-G0043 MV-HEVC/SHVC HLS: Redundant frames for SHVC/MV-HEVC/HEVC [M. Sychev, V. Stepin, V. Anisimovskiy, S. Ikonin (Huawei)]

JCT3V-G0044 MV-HEVC/SHVC HLS: VPS extension clean-up [Y. Cho, B. Choi, M. W. Park, J. Y. Lee, H.-C. Wey, C. Kim (Samsung)]

JCT3V-G0045 MV-HEVC/SHEVC HLS: Comments on POC alignment [M. Li, P. Wu, G. Shang, Y. Xie (ZTE)]

JCT3V-G0085 MV-HEVC/SHVC HLS: On parameter improvements [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)]

JCT3V-G0086 MV-HEVC/SHVC HLS: Decoded picture buffer signalling [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)]

JCT3V-G0087 MV-HEVC/SHVC HLS: On video parameter set extension [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)]

JCT3V-G0088 MV-HEVC/SHVC HLS: On auxiliary pictures [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)] [late]

TBR

(from JCT-VC / BoG notes)

Some constraints for slice header parameters of auxiliary pictures were proposed.

Proposed constraint 2:

- For all slices of auxiliary pictures, slice_sao_luma_flag and slice_sao_chroma_flag shall both be equal to 0; or

- When AuxId[ lId ] is equal to AUX_DEPTH, slice_sao_luma_flag and slice_sao_chroma_flag shall both be equal to 0

It was remarked that if such a constraint is needed, it should be part of a profile constraint and specific to particular auxiliary picture types.

As applied to depth maps, this should be considered a 3V issue.

Proposed constraint 3:

- For all slices of auxiliary pictures, slice_deblocking_filter_disabled_flag shall be equal to 0

- When AuxId[ lId ] is equal to AUX_DEPTH, slice_deblocking_filter_disabled_flag shall be equal to 0

It was remarked that if such a constraint is needed, it should be part of a profile constraint and specific to particular auxiliary picture types.

As applied to depth maps, this should be considered a 3V issue.

JCT3V-G0090 MV-HEVC/SHVC HLS: On VPS extension and VPS VUI [H. Lee, J. W. Kang, J. Lee, J. S. Choi (ETRI)]

JCT3V-G0091 MV-HEVC/SHVC HLS: On output_layer_flag [H. Lee, J. W. Kang, J. Lee, J. S. Choi (ETRI)]

JCT3V-G0092 MV-HEVC/SHVC HLS: comments on MV-HEVC WD 6 and SHVC WD 4 [H. Lee, J. W. Kang, J. Lee, J. S. Choi (ETRI)]

JCT3V-G0105 MV-HEVC/SHVC HLS: Proposal for supporting optional overlays with help of auxiliary pictures [N. Stefanoski, O. Wang, A. Smolic (DRZ), T. Szypulski (ESPN)]

JCT3V-G0116 MV-HEVC/SHVC HLS: On default output layer sets [K. Ugur, M. M. Hannuksela (Nokia)]

JCT3V-G0125 MV-HEVC/SHVC HLS: On VPS extension offset and VPS VUI offset [A.K. Ramasubramonian, Hendry, Y.-K. Wang (Qualcomm)]

JCT3V-G0131 MV-HEVC/SHVC HLS: Miscellaneous HLS topics [A.K. Ramasubramonian, Hendry, Y.-K. Wang, Y. Chen, V. Seregin (Qualcomm)]

JCT3V-G0132 MV-HEVC/SHVC HLS: Layer up-switching information SEI message [A.K. Ramasubramonian, Hendry, Y.-K. Wang (Qualcomm)]

JCT3V-G0133 MV-HEVC/SHVC HLS: On alt_output_layer_flag [A.K. Ramasubramonian, Y.-K. Wang, Hendry, Y. Chen (Qualcomm)]

JCT3V-G0134 MV-HEVC/SHVC HLS: On recovery point and region refresh SEI messages [Hendry, Y.-K Wang, A.-K. Ramasubramonian (Qualcomm)]

JCT3V-G0135 MV-HEVC/SHVC HLS: Auxiliary pictures for multiple overlays [J. Boyce, S. Wenger (Vidyo)] [miss]

JCT3V-G0136 MV-HEVC/SHVC HLS: Improvements of Video and Picture Parameter Sets [T. C. Thang (UoA), J. W. Kang, J. Lee, H. Lee, J. S. Choi (ETRI)]

JCT3V-G0137 REXT/MV-HEVC/SHVC/3D-HEVC HLS: On indication of decoding process and profile-level-tier combinations [M. M. Hannuksela (Nokia)]

JCT3V-G0138 MV-HEVC/SHVC HLS: HRD parameters for bitstreams excluding CL-RAS pictures [M. M. Hannuksela (Nokia)]

JCT3V-G0139 MV-HEVC/SHVC HLS: Header parameter set (HPS) [M. M. Hannuksela, H. Roodaki (Nokia)]

JCT3V-G0140 MV-HEVC/SHVC HLS: On non-HEVC base layer [M. M. Hannuksela (Nokia)]

JCT3V-G0141 MV-HEVC/SHVC HLS: On temporal enhancement layers [M. M. Hannuksela (Nokia)] [late]

JCT3V-G0144 MV-HEVC/SHVC HLS: On Sequence Parameter Set [S. Deshpande (Sharp)]

JCT3V-G0145 MV-HEVC/SHVC HLS: On DPB Parameters in VPS [S. Deshpande (Sharp)]

JCT3V-G0146 MV-HEVC/SHVC HLS: On Indications for Inter-layer Prediction [S. Deshpande (Sharp)]

JCT3V-G0159 MV-HEVC/SHVC HLS: On Picture Parameter Set [Y. He, Y. Ye (InterDigital)]

JCT3V-G0160 MV-HEVC/SHVC HLS: On Sub-bitstream extraction and re-writing process [Y. He, Y. Ye (InterDigital)]

JCT3V-G0161 MV-HEVC/SHVC HLS: On decoded picture buffer management [Y.-K. Wang, A.K. Ramasubramonian, Y. Chen (Qualcomm)]

JCT3V-G0162 HEVCv1/MV-HEVC/SHVC HLS: On inference of NoOutputOfPriorPicsFlag [Y.-K. Wang, Y. Chen (Qualcomm)]

JCT3V-G0165 MV-HEVC/SHVC HLS: Sub-bitstream property SEI message [Y. Chen, Y.-K. Wang (Qualcomm)]

JCT3V-G0166 RExt/MV-HEVC/SHVC: On Auxiliary Alpha Plane Pictures [K. Misra, S. Deshpande, A. Segall (Sharp)]

JCT3V-G0236 MV-HEVC/SHVC HLS: On default target output layer set [Y.-K. Wang (Qualcomm)] [late]

JCT3V-G0237 MV-HEVC/SHVC HLS: Cross-layer POC anchor picture derivation (follow-up of JCTVC-P0056/JCT3V-G0042) [M. M. Hannuksela (Nokia), Y.-K. Wang (Qualcomm)] [late]

JCT3V-G0238 MV-HEVC/SHVC HLS: On alt_output_layer_flag [M. M. Hannuksela (Nokia)] [late]

JCT3V-G0240 MV-HEVC/SHVC HLS: Side-activity on ue(v) coding of VPS extension syntax elements [A. K. Ramasubramonian (Qualcomm)]

JCT3V-G0242 MV-HEVC/SHVC HLS: An extension for separation of non-VUI and VUI data in the VPS [Y.-K. Wang (Qualcomm)] [late]

# Non-CE technical contributions (25)

## VSP (7)

JCT3V-G0035 VSP depth access simplification [T. Ikai (Sharp)]

This proposal presents a simplification for VSP. Currently VSP applies to 8x4 and 8x4 block basis and access four depth values in each block. However it is asserted such a small block doesn’t need so many accesses and there is more efficient position for coding efficiency. This contribution proposes one position instead of four corners. The experiment result shows that the BD-rate gain is 0.07 %, 0.07 % and 0.05 % in video, total video and synthesis respectively.

With the proposed change, it was asserted that the number of accesses per block is reduced from 4 to 1, but this depends on the implementation. The impact of this change on memory bandwidth and complexity relative to the entire design was not clear, e.g., considering data reuse.

The position that was selected seems rather specific and appears to be tuned to the current test set. There was some concern on whether this generalizes to other test content.

See additional notes below on G0036.

JCT3V-G0179 Crosscheck on VSP depth access simplification (JCT3V-G0035) [Y.-W. Chen, J.-L. Lin (MediaTek)] [late]

JCT3V-G0036 Simplified VSP subblock decision [T. Ikai (Sharp)]

This proposal presents a simplified VSP subblock decision. Currently subblock size of VSP is decided based on depth values except of AMP PU. This contribution proposes to further simplify the decision so that depth base decision is applied only to square PUs. With this simplification, the depth access in VSP subbock decision is the same as the DoNBDV depth access since the access point is completely the same (no additional, no less). The experiment result shows that the BD-rate gain is 0.0 %, 0.0 % and 0.0 % in video, total video and synthesis respectively.

It was asserted by the proponent that the benefit of this proposal is that the number of access is reduced from 8 to 4.

It was remarked that the corner samples that are used to determine the sub-PU division are used to determine the disparity in the current design. The proposed design removes the access to those corner samples for the deciding the division, but it still requires them to determine the disparity. G0035 proposes to reduce the number of samples to determine the disparity to just one sample per sub-PU.

Considering the above, this proposal only makes sense in combination with G0035.

Revisit: clarify benefit of combined simplification (G0035/0036) with more comprehensive analysis

JCT3V-G0180 Crosscheck on Simplified VSP subblock decision (JCT3V-G0036) [Y.-W. Chen, J.-L. Lin (MediaTek)] [late]

JCT3V-G0150 Frame-level view synthesis prediction [S. Shimizu, S. Sugimoto (NTT)]

This contribution proposes a frame-level view synthesis prediction. In the proposed scheme, a forward warping based view synthesis is utilized in order to generate a synthesized picture and a hole map, which shows whether synthesized picture is available or not for each pixel. Based on the hole map, each CU is coded by either the normal scheme or the bypassed scheme. In the bypassed scheme, only prediction residuals are coded. Experiments reportedly show bitrate increase of 2.39% on average over all test sequences, but also show significant bitrate reduction for dependent view and total bitrate reduction of around 3% for GT_Fly and Shark sequences.

Motivation: consider whether side views extrapolated from center view provide a good prediction with sufficient subjective quality.

In the results, the QP values for coding the dependent texture view are increased by 2.

One issue with the current design is that the parsing is dependent on the hole map.

There may also be other ways to achieve the similar capability, e.g., new slice type that is a virtually decodable slice, or using an SEI message such as texture and depth view packing.

Similar proposals were put forward in response to the CfP. With such an approach, PSNR is not likely a good measure of quality.

No action, but proponents encouraged to follow up with further study.

## MV/DV candidates (7)

JCT3V-G0067 Availability checking of temporal inter-view MV candidate and VSP candidate [J. An, Y.-W. Chen, K. Zhang, J.-L. Lin, S. Lei (MediaTek)]

This contribution fixes a problem of using an unavailable reference view. It is proposed to check the availability of temporal inter-view MV candidate and VSP candidate according to the existence of inter-view reference picture. When there is no inter-view picture in the current reference lists, the temporal inter-view MV candidate and VSP candidate should not be inserted into the merge candidate list.

The problem will happen when there is no inter-view reference in the merge list construction.

As an alternative, it was suggested to disable NBDV and VSP at the slice level when no inter-view reference is available, but this signalling is not currently supported.

One expert expressed support for this proposal. WD text was reviewed, and was considered straightforward.

Decision(BF): Adopt

JCT3V-G0080 Improvement on Do-NBDV derivation [X. Chen, Y. Lin, X. Zheng, J. Zheng, (Hisilicon)]

In current 3D-HEVC, Do-NBDV uses the maximum among four corners of the depth samples to extract a disparity vector from the virtual depth block. There are two contributions for Do-NBDV:

1, The proposed method uses the maximum among two or three corners of the depth samples to extract a disparity vector from the virtual depth block. The experimental results reportedly show that the proposed method (three corners) has minor coding efficiency gain or no overall coding efficiency loss (two corners), and the computational complexity and memory access of depth samples are reduced.

2, It is proposed to recalculate corners position of the depth samples from the virtual depth block and select the corner which has maximum depth value after maximum among corners of the depth samples from the virtual depth block. The experimental results reportedly show that the proposed method has 0.04 coding efficiency gain.

In the current design, the disparity vector is derived using four corners. It is suggested that two or three corners is sufficient.

It was remarked that there is no problem with the current design and it is not desirable to make small and unjustified changes. No action.

JCT3V-G0183 Crosscheck of Improvement on Do-NBDV derivation (JCT3V-G0080) [T. Ikai (Sharp)] [late]

JCT3V-G0081 Improvement on NBDV derivation [X. Chen, X. Zheng, Y. Lin, J. Zheng, (Hisilicon)]

In HTM 9.0r1, NBDV is calculated by candidates of temporal, special, MCP or Default. We build the relationship between NBDV and Do-NBDV in certain condition. The experimental results reportedly show that build relationship between NBDV and Do-NBDV can achieve 0.05% BD-rate saving for overall coded and synthesized views compared to HTM-9.0r1.

This proposal aims to improve coding efficiency, but only gives 0.05% gain which is considered negligible. No action.

JCT3V-G0215 Crosscheck on Improvement on NBDV derivation (JCT3V-G0081) [H. Liu (Qualcomm)] [late]

JCT3V-G0096 Bug-fix on merging candidate list construction [G. Bang (ETRI), Y.S. Heo, G.H.Park (KHU), G.S. Lee, N.H.Hur (ETRI)]

This report presents a bug-fix on merging candidate list construction. In current 3D-HEVC, the maximum size of merging candidate list is generally 6 (5 + NumExtraMergeCand) for dependent view or depth. Therefore, in order to prevent buffer-overflow, checking current size of the list is necessary in decision of inserting candidates whose priority are lower than IvDC. However, as shown in Table 1, current WD does not check the size of the list when inserting VSP, A0, and B2 to the list.

This contribution identifies a mismatch between the text and software. The WD text has been checked by the editor and is agreed.

Decision(BF): Adopt

## Entropy coding (8)

JCT3V-G0241 BoG report on Entropy Coding [J.-L. Lin, P. Merkle]

The purpose of this BoG activity is identifying context coded syntax elements for consideration for simplification.

The BoG meeting took place on January 13, 2014 (3.30 to 6pm) in room San Carlos. Approximately 8-10 experts attended the meeting.

The following table lists the properties of context coded syntax elements included in the 3D-HEVC extension HTM-9.0r1 as well as proposals related to simplification of the corresponding syntax elements.

Table 1: Context coded syntax elements in HTM-9.0r1

| Syntax element | Num. contexts (initType 0 / 1 / 2) | Binarization process | Num. context coded bins (dependency) | Related proposal |
| --- | --- | --- | --- | --- |
| depth_intra_mode | 5 / 5 / 5 | TU | 1~4 (blocksize) |  |
| wedge_full_tab_idx | 1 / 1 / 1 | FL | 7~11 (blocksize) | JCT3V-G0062 JCT3V-G0108 |
| depth_dc_flag | 2 / 2 / 2 | FL | 1 | JCT3V-G0060 |
| depth_dc_abs | 1 / 1 / 1 | UEG0 | 1~13 (value) | JCT3V-G0060 JCT3V-G0124 |
| iv_res_pred_weight_idx | 0 / 4 / 4 | TU | 2 | JCT3V-G0061 |
| ic_flag | 0 / 3 / 3 | FL | 1 | JCT3V-G0061 |
| inter_sdc_flag | 1 / 1 / 1 | FL | 1 | JCT3V-G0060 |
| inter_sdc_resi_abs_minus1 | 1 / 1 / 1 | UEG0 | 1~13 (value) | JCT3V-G0056 JCT3V-G0060 JCT3V-G0124 |
| inter_sdc_resi_sign_flag | 1 / 1 / 1 | FL | 1 | JCT3V-G0060 |

The first group of syntax elements considered for simplification consists of wedge_full_tab_idx and depth_dc_abs / inter_sdc_resi_abs_minus1. Currently they all require a high number of context coded bins. Reducing the number of context coded bins for these syntax elements seems desirable.

The second group of syntax elements considered for simplification consists of iv_res_pred_weight_idx and ic_flag. Currently they both require a high number of contexts. Reducing the number of contexts for these syntax elements seems desirable.

Due to the adoption of JCT3V-G0122 the number of context coded bins for depth_intra_mode is reduced to 1.

Due to adoption of JCT3V-G0130 syntax elements depth_dc_flag and inter_sdc_resi_sign_flag are combined to a single syntax element named depth_dc_flag, syntax elements depth_dc_abs and inter_sdc_resi_abs_minus1 are combined to a single syntax element named depth_dc_abs, and syntax elements depth_dc_sign_flag and inter_sdc_resi_sign_flag are combined to a single syntax element named depth_dc_sign_flag, which is not context coded.

The following table summarized the changes introduced by these two adoptions.

Table 2: Context coded syntax elements after adoption of JCT3V-G0122 and JCT3V-G0130.

| Syntax element | Num. contexts (initType 0 / 1 / 2) | Binarization process | Num. context coded bins (dependency) |
| --- | --- | --- | --- |
| depth_intra_mode | 1 / 1 / 1 | FL | 1 |
| sdc_flag | 3 / 3 / 3 | FL | 1 |
| hevc_intra_flag | 3 / 3 / 3 | FL | 1 |
| wedge_full_tab_idx | 1 / 1 / 1 | FL | 7~11 (blocksize) |
| depth_dc_flag | 2 / 2 / 2 | FL | 1 |
| depth_dc_abs | 1 / 1 / 1 | UEG0 | 1~13 (value) |
| iv_res_pred_weight_idx | 0 / 4 / 4 | TU | 2 |
| ic_flag | 0 / 3 / 3 | FL | 1 |

JCT3V-G0056, G0108, G0124 were also CE/related contributions which were recommended for further investigation in the BoG. Further discussion performed in the plenary Tue AM.

JCT3V-G0056 3D-CE5: Results on bin reduction for inter SDC residual coding [Y.-W. Chen, J.-L. Lin, Y.-W. Huang, S. Lei (MediaTek)]

Currently syntax element inter_sdc_resi_abs_minus1 is coded using UEG0, including up to 13 context coded bins.

Proposal restricts the range of values for inter_sdc_resi_abs_minus1 to [0..5], using 1 context coded plus 0-3 bypass coded bins.

No impact on coding efficiency.

One concern raised is the restriction of the range of values which may be undesirable as it restricts the flexibility of the tool which might be needed outside of the current test set

Another concern raised is that the proposal only applies the reduction only to the case of inter SDC. Due to adoption G0130 intra and inter SDC are unified, and further study (CE) is necessary how the reduction of context coded bins would perform with the unified case.

JCT3V-G0060 3D-AHG8: Simplification on CABAC contexts for the syntax related to inter SDC, intra SDC and DMM [Y.-W. Chen, Y.-L. Chang, J.-L. Lin, Y.-W. Huang, S. Lei (MediaTek)]

Proposal consists of two parts.

Part1:

Reducing the number of contexts for flag type syntax elements depth_dc_flag, inter_sdc_flag, and inter_sdc_resi_sign_flag by replacing context coding by bypass coding. Currently 1 or 2 contexts are used for these syntax elements.

No impact on coding efficiency.

It was asked, whether the statistics of these flags was analyzed?

Several experts remarked that in general decision to replace context coding by bypass coding should better be made based on such a statistical analysis. It was noted that coding results show no impact and the proponent commented that the statistic is expected to be close to 50% for the three flags.

It was noted that removing the contexts might have a negative impact (coding efficiency) on the flexibility to disable the corresponding functionality for some of the CUs within a slice.

The BoG reports back to JCT-3V that it was shown that removing up to 4 contexts does not have an impact on the coding efficiency, but it was not clear whether removing 4 contexts is beneficial or not.

Due to adoption of JCT3V-G0130, only the depth_dc_flag remains. Not clear from the results whether the benefit claimed by the proponents would still apply. No action.

Part2:

Reducing the number of context coded bins for syntax elements depth_dc_abs and inter_sdc_resi_abs_minus1 by replacing the 1~13 context coded bins by 1 context coded plus 0~12 bypass coded bins.

No impact on coding efficiency.

One expert supported part 2.

Part 2: Further study in CE

JCT3V-G0201 3D-AHG8: Cross check of simplification on CABAC contexts for the syntax related to inter SDC, intra SDC and DMM (JCT3V-G0060) [J. Y. Lee, C. Kim (Samsung)] [late]

JCT3V-G0124 CE5 related: Delta DC coding for SDC and DMM modes [Q. Yu, H. Liu, Y. Chen, L. Zhang (Qualcomm), S. Ma (PKU)]

Reducing the number of context coded bins for syntax elements depth_dc_abs and inter_sdc_resi_abs_minus1 by replacing the 1~13 context coded bins of the prefix by 1~9 context coded bins. The remaining information is coded together with the suffix using bypass coding (methods 1-3 with EG0 or EG3 or FL). Alternatively the range of values of these syntax elements can be restricted to [0..30], using EG0 with 1~9 context coded bins (method 4).

|  | HEVC | 3D-HEVC | Method 1 | Method 2 | Method 3 | Method 4 |
| --- | --- | --- | --- | --- | --- | --- |
| Maximum number of context coded bins | 9 | 13 | 9 | 9 | 9 | 9 |
| Maximum number of bins | 30 | 28 | 24 | 21 | 17 | 9 |
| Delta BD rate | n/a | 0.00% | -0.01% | -0.03% | -0.02% | 0.23% |

It was noted that the numbers for HEVC are not clear and that the maximum number of context code bins might be more than 9, e.g. for transform coefficients.

General note: The BoG recommended to consider studying the impact of different numbers of context coded bins for syntax element depth_dc_abs / inter_sdc_resi_abs_minus1 (unified by JCT3V-G0130) in a CE or as AHG mandate.

Further study in CE

JCT3V-G0062 3D-AHG8 related: A simplified method on DMM mode coding in 3D-HEVC [K. Zhang, Y.-W. Chen, J. An, X. Zhang, J.-L. Lin, S. Lei (MediaTek)]

Reducing the number of context coded bins for syntax element wedge_full_tab_idx by replacing the 7~11 context coded bins by 7~11 bypass coded bins.

No impact on coding efficiency.

Some experts supported this proposal.

JCT3V-G0192 AHG8 related: Crosscheck on MediaTek's proposal "A simplified method on DMM mode coding in 3D-HEVC (JCT3V-G0062)" [X. Zheng (HiSilicon)] [late]

JCT3V-G0108 CE5 related: DMM1 signaling based on intra direction [P. Merkle, K. Müller (HHI)]

Reducing the number of context coded bins for syntax element wedge_full_tab_idx by replacing the 7~11 context coded bins by 5~11 bypass coded bins, consisting of a fixed-length prefix (representing the direction) with 5 bypass coded bins a variable length suffix (representing a refinement index) with 0~6 bypass coded bins, based on a lookup table with 32x4 entries.

The results show 0.07% coding gain for CTC and AI, consistent for all sequences.

Some experts supported this proposal.

In the follow-up discussion in JCT-3V, it was pointed out that the length of the suffix depends on the decoding of the prefix, and this may have some implication on parallelization of parsing processes, which might require some more study, whether the benefit of replacing context coding by bypass coding is really improving the throughput in this case

Further study of G0062 and G00108 in CE.

JCT3V-G0061 3D-AHG8: Simplification on CABAC contexts for IC and ARP Flags [Y.-W. Chen, K. Zhang, J. An, J.-L. Lin, X. Zhang, S. Lei (MediaTek)]

Reducing the number of contexts for syntax elements iv_res_pred_weight_idx and ic_flag by removing/restricting the conditional context selection based on neighboring block information. Furthermore, the purpose of the proposed changes is removing the line buffer (memory requirement) for these two syntax elements.

For ic_flag the number of contexts is reduced from 3 to 1 by completely removing the dependency on left and above block information. For iv_res_pred_weight_idx the number of contexts is reduced from 4 to 3 by removing the dependency on above block information.

No impact on coding efficiency.

In the BoG, some experts had recommended adoption of this proposal. (SW and WD had been confirmed.)

Decision: Adopt G0061

JCT3V-G0191 AHG8: Crosscheck on MediaTek's proposal "Simplification on CABAC contexts for IC and ARP Flags (JCT3V-G0061)" [X. Zheng (HiSilicon)] [late]

JCT3V-G0066 Syntax redundancy removal for illumination compensation [K. Zhang, J. An, X. Zhang, J.-L. Lin, Y.-L. Chang, S. Lei (MediaTek)]

Has been discussed in context of CE3.

JCT3V-G0164 Crosscheck of MediaTek's propsoal on Syntax redundancy removal for illumination compensation (JCT3V-G066) [J. Zheng (HiSilicon)] [late]

## Other (3)

JCT3V-G0071 The removal of quad-tree limitation for depth maps [S. Yoo, J. Heo, S. Yea (LGE)]

This proposal has already been discussed in the context of CE2. No action, but proponents may study further.

JCT3V-G0202 Cross check of removal of quad-tree limitation for depth maps (JCT3V-G0071) [J. Y. Lee, C. Kim (Samsung)] [late]

JCT3V-G0151 AHG13: Results with quarter resolution depth map coding [S. Shimizu, S. Sugimoto (NTT)]

This contribution reports the result of quarter resolution depth map coding on MV-HEVC. In the simulation, VSO was activated for quarter resolution depth map coding with assuming the nearest neighbour upsampling would be performed during view synthesis. Experiments reportedly show that about 3.38% bitrate reduction and 6.22% bitrate increase for coded and synthesized views, respectively, with saving about 30% decoder runtime.

The main benefit of reduced resolution depth is to reduce the decoder run time, or other implementation factors such as memory or throughput.

It was observed that VSO gives significant improvement and uses a simple nearest neighbor upsampling. It would be desirable to use a more complex up-sampling filter to interpolate the depth values used for rendering, but this would need to be consistent with that used for VSO. It might be beneficial if VSO used a more complex up-sampling process.

The contribution also analyzes the impact of reduced resolution depth on the coding tools in 3D-HEVC. There would be substantial work involved to support reduced resolution depth with all the 3D-HEVC coding tools.

The contribution recommends supporting reduced resolution coding of depth in the software.

The MV-HEVC specification supports reduced resolution depth since auxiliary pictures can have a different spatial resolution than the primary pictures.

There were some reservations to adopt this into the software since it doesn’t provide a benefit in terms of coding efficiency. Also, there was some concern that there might be a desire to add other scaling ratios and make the software more complex.

There was some interest to integrate into the software so that this capability could be studied further.

It was suggested that the software might be integrated into a branch of the software, or simply shared with interested parties. It was noted that even the branch would need to be maintained and there were no volunteers to do this. As a result, it was agreed to simply share the software with interested parties.

It was also agreed that the mandates of the AHG13 have been completed, and there is no need to continue the activity.

# Alternative depth formats (1)

JCT3V-G0107 Technically improved texture and depth view packing SEI message in 3D-AVC [Takanori Senoh, Koki Wakunami, Yasuyuki Ichihashi, Hisayuki Sasaki, Kenji Yamamoto, Masayuki Tanimoto (NICT/NISRI), Ying Chen (Qualcomm)]

This document provides technically improved texture and depth view packing SEI message in 3D-AVC. The document is improved based on the editor’s comments at the 6th JCT-3V meeting.

The new version of the SEI message has new syntax elements for homography mapping data, expressed via camera parameters. The syntax of the camera parameters is similar to the existing camera parameter SEI message (different in the length of the exponent of the float representation).

Discussion whether the homography mapping could be expressed more compactly. However, the camera parameters are further needed to determine the pixel shifts from the depth values in the synthesis process.

It is suggested that the camera parameters should be identical in syntax and semantics to the existing SEI message

Position of the residual views in the frame packing should be changed to:

R1 at 0,0

R2 at W/2, 0

R3 at 0, H/2

R4 at W/2, H/2

If there is only one residual view, it will be at R1, if there are two, they are at R1 and R2 etc.

It does not matter where the physical views are, and how they are associated with the residual views, as the camera parameters are describing that.

Remove Figures J1 and J2, and do not associate the base view with #3.

Revisit after the changes and after communicating with parent bodies.

# Non-normative contributions (2)

## Encoder optimization (2)

JCT3V-G0079 Improvement on AMP fast mode decision [X. Chen, X. Zheng, Y. Lin, J. Zheng, (Hisilicon)]

In this document, we constrained AMP_MRG using condition and test results of asymmetric motion partition (AMP) are provided. Three methods average gain are 0.36%, 0.34% and 0.3% with coding time 3.94%, 2.57% and 1.81% increase, respectively.

The method was also proposed for HEVC single layer coding, but has additional benefit for multiview video coding. Currently, there is a fast estimation process to determine the AMP merge mode which incurs coding loss. The proposed method maintains the fast estimation, but does not lose as much.

It was considered desirable by one expert to use the same base layer encoding techniques as HM; it was suggested that the method should be proposed to JCT-VC.

JCT3V-G0207 Crosscheck for HiSilicon's JCT3V-G0079 [K. Zhang, J. An, X. Zhang (MediaTek)] [late]

JCT3V-G0163 VSD bugfix and improvement [Z. Gu (SCU), J. Zheng (HiSilicon), N. Ling (SCU), P. Zhang (HiSilicon)]

Bugfix of VSD (View Synthesis Distortion) calculation is proposed to correct the bitDepth value assignment for VSD calculation. After bugfix, 0.1% BD-bitrate saving is achieved on synthesized view under CTC. Additionally a modification of VSD calculation is proposed and 0.1% BD-bitrate saving on view 1 and view 2, and 0.3% BD-bitrate saving on synthesized view are achieved under CTC.

It was remarked that the bug fix seems like an obvious thing to do, and the proposed weighting function appears to provide a good gain. There was a question on whether the function was optimized to the test set – it was not.

The same optimization can be applied for both MV-HEVC with depth as well as 3D-HEVC.

Offline discussion has taken place to understand technique and confirm cross-check. Experts understand the proposed fix and improvement and support their inclusion in the software.

Decision(SW): Adopt

JCT3V-G0211 Crosschecking for Hisilicon's VSD bugfix and improvement, JCT3V-G0163 [X. Zhang, K. Zhang, J. An (MediaTek)] [late]

## Rate control (xx)

# Withdrawn, unclear allocation (0)

see section 1.4.

JCT3V-G0089 Withdrawn

# Plenary Discussions and BoG Reports

## Project development

## BoGs

JCT3V-G0239 JCT3V BoG Report on ATM Software Improvements [D. Tian (MERL)]

In this BoG, the problems with ATM software Ver 10.0 (SVN REV 334) are identified. A plan to improve the software is drafted for considerations by the JCT3V group. With the plan, it is expected to solve the identified software problems and release an improved version that capable of generating all 3D-AVC conforming bitstreams before Friday, March 14, 2014.

The BoG has identified various items where deviations between the specification text and the software exist. However, all items are related to the software and it is asserted that the specifiation text is correct. This assertion includes the changes made by the last meeting in the context of JCT3V-F0268. Item 8 in the table of v2 of the BoG report (related to (VSP mbPartIdx) is confirmed to be resolved during the editing process (editor Y. Chen confirms that the editorial comment was removed in the final version).

# Project planning

## General issues for CEs

A preliminary CE description is to be approved at the meeting at which the CE plan is established.

It is possible to define sub-experiments within particular CEs. As a general rule, it was agreed that each CE should be run under the same testing conditions using same software codebase, which should be based on either the ATM or HTM software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

The general agreed common conditions for experiments were described in the output document JCT3V-F1100.

A deadline of two weeks after the meeting was established for organizations to express their interest in participating in a CE to the CE coordinators and for finalization of the CE descriptions by the CE coordinator with the assistance and consensus of the CE participants.

Any change in the scope of what technology will be tested in a CE, beyond what is recorded in the meeting notes, requires discussion on the general JCT-3V reflector.

As a general rule, all CEs are expected to include software available to all participants of the CE, with software to be provided within two (calendar) weeks after the release of the relevant software basis. Exceptions must be justified, discussed on the general JCT-3V reflector, and recorded in the abstract of the summary report.

Final CEs shall clearly describe specific tests to be performed, not describe vague activities. Activities of a less specific nature are delegated to Ad Hoc Groups rather than designated as CEs.

Experiment descriptions should be written in a way such that it is understood as a JCT-3V output document (written from an objective "third party perspective", not a company proponent perspective – e.g. referring to methods as "improved", "optimized" etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions should not contain verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in referenced documents that are also available in the JCT-3V document archive.

Those who proposed technology in the respective context (by this or the previous meeting) can propose a CE or CE sub-experiment. Harmonizations of multiple such proposals and minor refinements of proposed technology may also be considered. Other subjects would not be designated as CEs.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

It is strongly recommended to plan resources carefully and not waste time on technology that may have little or no apparent benefit – it is also within the responsibility of the CE coordinator to take care of this.

A summary report written by the coordinator (with the assistance of the participants) is expected to be provided to the subsequent meeting. The review of the status of the work on the CE at the meeting is expected to rely heavily on the summary report, so it is important for that report to be well-prepared, thorough, and objective.

Non-final CE plan documents were reviewed and given tentative approval during the meeting (in some cases with guidance expressed to suggest modifications to be made in a subsequent revision).

The CE description for each planned CE is described in an associated output document JCT3V-E110x for CE n, where “n" is the CE number (n = 1, 2, etc.). Final CE plans are recorded as revisions of these documents.

It must be understood that the JCT-3V is not obliged to consider the test methodology or outcome of a CE as being adequate. Good results from a CE do not impose an obligation on the group to accept the result (e.g., if the expert judgment of the group is that further data is needed or that the test methodology was flawed).

Some agreements relating to CE activities were established as follows:

- Only qualified JCT-3V members can participate in a CE

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting.

- All software, results, documents produced in the CE should be announced and made available to all CE participants in a timely manner.

### Cross check

For cross checking the same steps as described in the section above should be carried out as soon as possible, but should not take more than five working days. Moreover it should be checked if integrated tools correspond to the adopted proposal. Results of cross check should be announced to the reflector. The result-sheet should be made available to the group.

### Procedure if cross check fails or planned delivery data cannot be held

If a planned delivery date cannot be held this should be announced to the reflector.

If the crosscheck fails or the previous integrator has not delivered the software within 3 days or result sheet within 5 days after planned delivery date

- the current integrator should integrate in the last cross-checked version

- the previous integrator falls back to the end of the integration plan

## Common Conditions for 3D Video Coding Experiments

Preferred Common Conditions for experiment testing that are intended to be appropriate for both CEs and other experiments were selected by the group and described in output document JCT3V-F1100.

Changes from the previous meeting include enabling sub-PU level inter-view motion prediction with a default setting of 8x8. It was also agreed to include the Shark sequence as part of the CTC.

## Software development (to be updated)

ATM software:

(work plan to be copied from BoG report JCT3V-G0239)

Current availability: (see CTC), to be made publicly available without password protection

Version 9.0 (including all accepted changes) should be available within 3 weeks after the meeting. Version 9.1 (if deemed necessary for further bug fixing) is planned to be available 1 week later.

It was confirmed that the software plan for ATM includes extensions to support IBP across views.

HTM software:

Starting date: November, 1st, Due to limited time to next meeting weekends are also counted as "Days".

HTM-9.0 Integration plan Track 1: VSP, ARP

Coordinator: Li Zhang

| Item | Integrator | Days |
| --- | --- | --- |
| JCT3V-F0123/JCT3V-F0108 CE4: Results on improved advanced residual prediction | Qualcomm/MediaTek | 3 |
| JCT3V-F0161 CE4: Coding of weighting factor of advanced residual prediction | LGE | 2 |
| JCT3V-F0105 CE4: ARP reference picture selection and its availability check | Sharp | 3 |
| JCT3V-F0109/JCT3V-F0120 CE1: A simplified block partitioning method for VSP | MediaTek/LGE | 3 |
| JCT3V-F0102 CE1: VSP partitioning for AMP | Sharp | 2 |

Track 2: Merge.

Coordinator: Yulin Chang

| Item | Integrator | Days |
| --- | --- | --- |
| JCT3V-F0125 CE3: Inter-view motion vector prediction for depth coding | Qualcomm | 3 |
| JCT3V-F0111 CE1: Simplified view synthesis prediction | MediaTek | 3 |
| JCT3V-F0104 CE3: Removal of redundancy on VSP, ARP and IC | LGE | 3 |
| JCT3V-F0150 CE3: MPI candidate in depth merge mode list construction | Samsung | 2 |
| JCT3V-F0093 CE3: Results on simple merge candidate list construction for 3DV | ETRI | 3 |

Track 3: Depth Intra + IC Flags

Coordinator: Gerhard Tech

| Item | Integrator | Days |
| --- | --- | --- |
| JCT3V-F0132/JCT3V-F0171 CE5: Unification of delta DC coding for depth intra modes | - Qualcomm | 3 |
| JCT3V-F0159 CE5: Fast depth LUT application method to intra modes for depth data | LGE | 2 |
| JCT3V-F0160 Non-CE: Illumination compensation flag coding | LGE | 1 |
| JCT3V-F0147 CE5: DMM simplification and signalling | Samsung | 3 |
| JCT3V-F0151 Removal of IC in depth coding and IC flag signalling in 3D-HEVC | Samsung | 2 |

Integration after merge (HHI)  of track 1 + 2

| Item | Integrator | Days |
| --- | --- | --- |
| JCT3V-F0110 CE3: Sub-PU level inter-view motion prediction | MediaTek | 4 |

HTM-9.1 Integration plan

Coordinator: Gerhard Tech

| Item | Integrator | Days |
| --- | --- | --- |
| MV-HEVC 6 HLS | HHI + x |  |

HTM-9.2 Integration plan

Coordinator: Gerhard Tech

| Item | Integrator | Days |
| --- | --- | --- |
| JCT3V-F0136 Comments on camera parameters in 3D-HEVC | Qualcomm |  |
| JCT3V-F0045 Constraints on camera parameter signaling | MediaTek |  |
| JCT3V-F0082 On slice-level camera parameter signaling | MediaTek |  |
| JCT3V-F0044 HEVC compatible slice segment header in 3D-HEVC | MediaTek |  |
| - JCT3V-F0131/JCT3V-F0139/ Depth Lookup Table Coding for 3D-HEVC | ZTE/Qualcomm/MediaTek |  |
| Move DLT signaling (DLTs for multiple layers) from VPS to PPS | ZTE |  |
| JCT3V-F0138 3D-HEVC HLS: Inter-view Updating Mechanism for Coding of DLT | RWTH |  |

Note: It would be desirable in the future to make software integration of adopted tools more in parallel

It is to the discretion of the software coordinators to set up a time line for the integration and request proponents to finish integration by a given date.

Integration Procedure & Guidelines

Integration is done in a serial way. Each integrator cross-checks the version provided by his predecessor.  The cross check for the last version is carried out by the software coordinators.

Integration Guidelines

When integrating

- software changes should be enclosed by macros switchable by defines including company and proposal number e.g.

#define MYCOMPANYS_DEPTHFILTER_JCT3V_B0555  1

#if MYCOMPANYS_DEPTHFILTER_JCT3V_B0555

// do stuff

#endif

- new tools should be made switchable in the cfg-file if reasonable

- cfg-files should be updated

Delivery of software

Before delivering the software to the next integrator it should be checked if

- the software compiles under windows and linux

- software compiles and delivers same results as previous version when integrated tools are disabled by macro or cfg-settings

- there are encoder-decoder mismatches

- there are memory leaks by measuring maximum memory consumption (or specific tools e.g. valgrind) is

- visual quality is not disturbed

Additional to the software cfg-files that reflect proposed settings and an excel sheet with coding results should be provided. Software and cfg-files should be delivered by checking it in to the corresponding (HTM or ATM) software repositories.

When software is delivered this should be announced to the reflector. Moreover, every further change on the software should be announced. If there is a delay in integration this should be communicated to the reflector.

## Software repositories

### HTM

HTM software can be checked in

https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/

Therefore for each integrator an own software branch will be created by the software coordinator containing the current anchor version or the version of the previous integrator:

e.g.  branches/0.7-mycompany

The branch of the last integrator will become the new release candidate tag.

e.g. tags/0.8rc1

This tag can be cross-check by the group for. If no problems occur the release candidate will become the new tag after 7-days:

e.g. tags/0.8

If reasonable intermediate release candidate tags can be created by the software coordinator.

### ATM

An official release of the 3DV-ATM software can be check out by mpeg3dv SVN users from the following location:

http://mpeg3dv.research.nokia.com/svn/mpeg3dv/trunk/

Following every MPEG 3DV meeting, software coordinator creates a new branch for the integration of adopted proposals:

http://mpeg3dv.research.nokia.com/svn/mpeg3dv/branches/

(Note: Would be desirable to rename “mpeg3dv” to “jct3v”)

Software integrator checks-out the software from integration branch at its turn of integration plan and integrates proposal as it is specified in Section 12.3.

Software integrator communicates a new software version over the email list to the cross-checker and to the software coordinator.

The software coordinator checks-in to the integration branch every new software integration with confirmed cross-check, as it is specified in Section 12.3.

Once the integration plan is completed, the software coordinator cross-check version available in the integration branch and checks-in a new official release of the 3DV-ATM to the http://mpeg3dv.research.nokia.com/svn/mpeg3dv/trunk/.

## List of CEs

All CEs will only be on HEVC – no need to differentiate a/h. Shall be called CEx (without h suffix)

- CE1 on Merge List Construction [J.-L. Lin, J. Heo]

  - Merge candidate list simplification (G0100, G0034, G0099, G0104)

  - Coding efficiency improvement by removal of redundant merge list candidates (G0078, G0066)

- CE2 on Residual Prediction [L. Zhang, T. Ikai]

  - Sub-block ARP (G0121)

  - Chroma ARP skip for 4x4 (G0033)

- CE3 on Entropy Coding Simplifications [Y.L. Chang, H. Liu]

  - Simplification of context coded bins (G0062, G00108, G0124, G0060, G0056)

# Establishment of ad hoc groups (to be updated)

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups will be the main JCT-3V reflector (jct-3v@lists.rwth-aachen.de).

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| JCT-3V project management (AHG1) (jct-3v@lists.rwth-aachen.de) - Coordinate overall JCT-3V interim efforts. - Report on project status to JCT-3V reflector. - Provide report to next meeting on project coordination status. | G. J. Sullivan, J.-R. Ohm (co-chairs) | N |
| 3D-AVC Draft and Test Model editing (AHG2) (jct-3v@lists.rwth-aachen.de) - Produce and finalize JCT3V-F1002 3D-AVC Draft Text 8. - Produce and finalize JCT3V-F1003 3D-AVC Test Model 8 Description. - Gather and address comments for refinement of these documents. - Coordinate with the 3D-AVC Software Integration AhG to address issues relating to mismatches between software and text. | M. Hannuksela (chair), Y. Chen, T. Suzuki, J.-R. Ohm, G. Sullivan (vice chairs) | N |
| MV-HEVC / 3D-HEVC Draft and Test Model editing (AHG3) (jct-3v@lists.rwth-aachen.de) - Produce and finalize JCT3V-F1005 3D-HEVC Test Model 6. - Produce and finalize JCT3V-F1004 MV-HEVC Draft Text 6. - Produce and finalize JCT3V-F1001 3D-HEVC Draft Text 2. - Gather and address comments for refinement of these documents. - Coordinate with the 3D-HEVC Software Integration AhG to address issues relating to mismatches between software and text. | G. Tech, K. Wegner (co-chairs), J. Boyce, Y. Chen, M. Hannuksela, T. Suzuki, S. Yea, J.-R. Ohm, G. Sullivan (vice chairs) | N |
| 3D-AVC Software Integration (AHG4) (jct-3v@lists.rwth-aachen.de) - Coordinate development of the 3DV-ATM software and its distribution to JCT-3V members - Produce documentation of software usage for distribution with the software - Prepare and deliver 3DV-ATM v10.0 software version and the reference configuration encodings according to JCT3V-E1100 based on common conditions (expected within 3 weeks after the meeting). - Enable the software to support inter-view IBP configurations. - If necessary, prepare and deliver 3DV-ATM v10.1 software version for further improvements and bug fixes. - Perform analysis and reconfirmation checks of the behavior of technical changes adopted into the draft design, and report the results of such analysis. - Coordinate with 3D-AVC Draft and Test Model editing (AHG2) to identify any mismatches between software and text. - Start an effort for software code clean up. | D. Rusanovskyy (chair), F. C. Chen, J. Y. Lee, J.-L. Lin, O. Stankiewicz, D. Tian (vice chairs) | N |
| MV-HEVC and 3D-HEVC Software Integration (AHG5) (jct-3v@lists.rwth-aachen.de) - Coordinate development of the HTM software and its distribution to JCT-3V members - Produce documentation of software usage for distribution with the software - Prepare and deliver HTM 9.0 software version and the reference configuration encodings according to JCT3V-F1100 based on common conditions suitable for use in most core experiments (expected within four weeks after the meeting). - Prepare and deliver HTM 9.1 software that include additional items not integrated into the 9.0 version (expected within one week after the 9.0 software release). - Prepare and deliver HTM 9.2 software that include 3D-HEVC HLS not integrated into the 9.1 version (expected within two weeks after the 9.1 software release). - Perform analysis and reconfirmation checks of the behaviour of technical changes adopted into the draft design, and report the results of such analysis. - Suggest configuration files for additional testing of tools. - Coordinate with MV-HEVC Draft and 3D-HEVC Test Model editing AhG to identify any mismatches between software and text. | G. Tech, L. Zhang (co-chairs), Y. Chang, K. Wegner (vice chairs) | N |
| 3D Coding Tool Testing (AHG6) (jct-3v@lists.rwth-aachen.de) - Discuss and finalize CE work plans - Study the Common Test Conditions and suggest possible changes - Coordinate between core experiments when necessary. - Study coding tool performance of non-CTC configurations - Report on status of core experiments. - Coordinate with AHG9 to prepare encodings to perform a visual comparison between MV-HEVC (with depth map carriage) and 3D-HEVC at the 7th JCT-3V meeting - Prepare viewing logistics for 7th JCT-3V meeting. | K. Müller, A. Vetro (co-chairs) | N |
| 3D High level syntax (AHG7) (jct-3v@lists.rwth-aachen.de) - Identify opportunities for common approaches for multi-view and scalable high-level extensions of HEVC. - Study NAL unit header, video parameter set, sequence parameter set, picture parameter set, and slice header syntax designs. - Study SEI messages and VUI syntax designs needed for HEVC extensions. - Study mechanisms for enabling 3D-HEVC coding tools in high-level syntax on the basis of JCT3V-F0256 and JCT3V-F0267. - Assist in software development and text drafting for the high-level syntax in the HEVC extensions designs. - Coordinate efforts with JCT-VC AHG 9 (HLS), AHG10 (POC) and AHG 20 (HRD) | M. M. Hannuksela (Chair), J. Boyce, Y. Chen, A. Norkin, Y.-K. Wang (vice chairs) | Y (Thursday and Friday before the 7th JCT-3V meeting) |
| Study of 3D-HEVC entropy coding (AHG8) (jct-3v@lists.rwth-aachen.de) - Study throughput issues of CABAC processing in 3D-HEVC. - Study the number of context coded bins for various syntax elements of 3D-HEVC and investigate ways to reduce it, especially taking into account the worst case. Study possible simplifications and unifications of binarization and context models for 3D-HEVC syntax elements. | J.-L. Lin, T. Ikai (co-chairs), | N |
| 3D Video Quality Assessment (AHG9) (jct-3v@lists.rwth-aachen.de) - Finalize verification test plan and prepare test material for systematic assessment of quality metrics and verification testing - Coordinate with AHG6 to prepare a visual comparison between MV-HEVC (with depth map carriage) and 3D-HEVC at the 7th JCT-3V meeting - Study objective quality assessment metrics that would provide an accurate evaluation of synthesized views - Recommend improvements to evaluation methodology | V. Baroncini, T. Ebrahimi, A.Vetro, (co-chairs) | N |
| Complexity assessment (AHG10) (jct-3v@lists.rwth-aachen.de) - Study intrinsic complexity measures or metrics to assess hardware and software implementations of algorithms/computational models based on number of operations, potential for parallelism, data transfer rate, and data storage requirements. - Analyze existing designs and produce complexity perspective per tool, from the viewpoint of possible slowdown, bottlenecks or implementation issues. - Develop a recommendation to JCT-3V on formalized complexity assessment in a reporting document. | C. Lee (chair), G. Bang, T. Ikai, K. Rapaka, D. Rusanovskyy (vice chairs) | N |
| 3D Test Material (AHG11) (jct-3v@lists.rwth-aachen.de) - Identify deficiencies and limitations of current 3D video test material. - Identify, collect, and make available a variety of additional 3D video sequences. - Study the characteristics of test materials and their impact on coding performance, synthesis quality etc. - Recommend appropriate test materials for use in 3D Video Coding Extension Development. - Assess the suitability of the new test material that was offered to the 5th and 6th JCT-3V meetings. - Collect and make available test materials featuring complex 3D video scenes, e.g. scenes including transparent or semi-transparent objects. | S. Shimizu, S. Yea (co-chairs) | N |
| Conformance testing development (AHG12) (jct-3v@lists.rwth-aachen.de) - Further discuss and improve the conformance drafts related to MVC plus Depth (JCT3V-F1006) and 3D-AVC (JCT3V-F1009) - Collect the conformance test streams according to JCT3V-F1006 and JCT3V-F1009 | T. Suzuki (chair), D. Rusanovskyy, D. Tian, Y. W. Chen (vice chairs) | N |

# Output documents (to be updated)

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate those responsible for document production.

(it was communicated by the chair that only the subsequent lists of adoptions is relevant, and in case of contradiction with any of meeting notes above the latter are considered invalid in first place)

JCT3V-F1000 Meeting Report of 6th JCT-3V Meeting [J.-R. Ohm, G. J. Sullivan]

(Note: Initial versions of the subsequent draft documents should be uploaded by the end of the meeting, continually updating to be performed until the final the version is released)

JCT3V-F1001 3D-HEVC Draft Text 2 [G. Tech, K. Wegner, Y. Chen, S. Yea] (WG 11 N13936) [2013-12-13]

Summary of Adoptions for 3D-HEVC

- ...

JCT3V-F1002 3D-AVC Draft Text 8 [M. Hannuksela, Y. Chen, T. Suzuki] (ISO/IEC 14496-10 FDAM3, to be integrated in new edition WG 11 N13916) [2013-11-08]

Summary of adoptions for 3D-AVC

...

JCT3V-F1003 3D-AVC Test Model 8 [D. Rusanovskyy, F. C. Chen, L. Zhang, T. Suzuki] (WG 11 N13919) [2013-11-29]

See list under JCT3V-F1002 as applicable

Summary of additional adoptions for 3D-ATM Software only

- ...

JCT3V-F1004 MV-HEVC Draft Text 6 [G. Tech, K. Wegner, Y. Chen, M. Hannuksela, J. Boyce] (Study of ISO/IEC 23008-2 DAM2, WG 11 N13934) [2013-11-29]

Summary of Adoptions for MV-HEVC

- ....

JCT3V-F1005 Test Model 6 of 3D-HEVC and MV-HEVC [L. Zhang, G. Tech, K. Wegner, S. Yea] (WG 11 N13940) [2013-12-13]

It is planned to structure this document into 2 parts (Test models for MV-HEVC and 3D-HEVC), and make a separation into two documents by the next meeting.

Summary of additional SW only adoptions and bug fixes (CTC only):

JCT3V-F0122 HTM support of depth coding in MV-HEVC

JCT3V-F1006 Draft 5 of MVC plus Depth Conformance [T. Suzuki, D. Rusanovskyy] (Study of ISO/IEC 14496-4:2004 DAM41, WG 11 N13904) [2013-11-05]

one more stream is added

JCT3V-F1007 Draft 4 of Multi-resolution Frame Compatible Stereo [P. Yin, Y.-K. Wang, G. Sullivan] (ISO/IEC 14496-10:2012 FDAM5, to be integrated in new edition WG 11 N13916) [2013-11-08]

JCT3V-E1008 MFC Test Model 2 remains valid

JCT3V-F1009 Draft 3 of 3D-AVC Conformance [D. Rusanovskyy, T. Suzuki, D. Tian, Y. W. Chen] (ISO/IEC 14496-4:2004 PDAM43, WG 11 N13908) [2013-11-29]

JCT3V-F1010 Draft 3 of MVC+D Reference Software [D. Rusanovskyy, T. Suzuki, D. Tian] (ISO/IEC 14496-5:2001 DAM33, WG 11 N13910) [2013-11-29]

JCT3V-F1011 3D Video Subjective Quality Assessment Test Plan  v2 [V. Baroncini, T. Ebrahimi, A. Vetro] (WG 11 N13920) [2013-11-01]

JCT3V-F1012 Draft 2 of MFC Conformance [P. Yin, D. Tian] (Study of ISO/IEC 14496-4:2004 DAM42, WG 11 N13906) [2013-11-29]

JCT3V-F1013 Draft 2 of MFC Reference Software [P. Yin, D. Tian] (Study of ISO/IEC 14496-5:2001 DAM34, WG 11 N13912) [2013-11-29]

JCT3V-F1014 Draft 1 of 3D-AVC Reference Software [D. Rusanovskyy, D. Tian] (Study of ISO/IEC 14496-5:2001 PDAM35, WG 11 N13914) [2013-11-29]

JCT3V-F1100 Common Test Conditions of 3DV Core Experiments [D. Rusanovskyy, K. Müller, A. Vetro]

JCT3V-F1101 Description of Core Experiment 1 (CE1) on Sub-PU Processing [S. Shimizu]

JCT3V-F1102 Description of Core Experiment 2 (CE2) on Motion Prediction for Depth Coding [Y.-L. Chang, Y. Chen]

JCT3V-F1103 Description of Core Experiment 3 (CE3) on Motion Prediction for Texture Coding [J.-L. Lin, J. Heo]

JCT3V-F1104 Description of Core Experiment 4 (CE4) on Residual Prediction [L. Zhang, J. L. Lin]

JCT3V-F1105 Description of Core Experiment 5 (CE5) on Depth Intra Modes [H. Liu]

# Future meeting plans, expressions of thanks, and closing of the meeting

The document upload deadline for the 8th meeting of the JCT-3V will be March XX, 2014, 2359 MET (Geneva/Paris time zone). Documents that are intended to be discussed in the joint meeting of JCT-VC and JCT-3V AHG7 (starting from XXX) shall be registered as joint contributions, the deadline of the 17th JCT-VC meeting is to be applied, i.e. March XX, 2014, 2359 MET (Geneva/Paris time zone).

Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (starting meetings on the Wednesday or Thursday of the first week and closing it on the Tuesday or Wednesday of the second week of the SG 16 meeting);

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (starting meetings on the Saturday prior to such meetings and closing it on the last day of the WG 11 meeting);

- In cases where JCT-3V meets during the first week of the SG16 meeting under ITU-T auspices, and co-located with an MPEG meeting at a nearby meeting place, the meeting dates could also be approximately aligned with the MPEG meeting.

Some specific future meeting plans were established as follows:

- 29 March–04 April 2014 under WG 11 auspices in Valencia, ES.

- 3-9 July 2014 under ITU-T auspices in Sapporo, JP

- 18-24 Oct. 2014 under WG 11 auspices in Strasbourg, FR

- XX-XX Feb. 2015 under ITU-T auspices in Geneva, CH

XXX was thanked for its excellent hosting of the 7th meeting of the JCT-3V. XXX was thanked for providing the 3D viewing equipment used at the 7th JCT-3V meeting. XXX was thanked for providing new 3D test material.

It was also reminded that final output documents (if also registered under a WG11 output doc number) have to be uploaded separately with a WG11 header.  To do this in a timely fashion is particularly important for standards submitted for ballot.

It was also reminded that slide presentation decks should be made available.

The JCT-3V meeting was closed at approximately XXXX hours on Friday 18 Jan. 2014.

# Annex A to JCT-3V report:
List of documents

# Annex B to JCT-3V report:
List of meeting participants

The participants of the fourth meeting of the JCT-3V, according to a sign-in sheet passed around during the meeting (approximately 107 in total), were as follows: