| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 2nd Meeting: Geneva, CH, 21-28 July, 2010 | Document: JCTVC-B101 |
| --- | --- |

| Title: | Preliminary Results on Motion Vector Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Gordon Clare, Joel Jung, Stephane Pateux 38-40 rue du Général Leclerc 9279 ISSY LES MOULINEAUX | Tel: Email: | +33 1 45 29 53 66 joelb.jung@orange-ftgroup.com |
| Source: | Orange Labs – France |  |  |

_____________________________

# Abstract

This contribution gives some preliminary results on motion vector prediction in the TMuC. It is intended to launch a Tool Experiment on the subject.

# Introduction

This contribution gives some preliminary results on motion vector prediction in the TMuC. Its goal at this stage is not to discuss tool efficiency, implementation issues, etc., but only to show the need to set up a Tool Experiment (TE) on the subject.

# Current MV prediction in the TMuC

The document JCT-VC-A205 provides the description of motion vector prediction. The current MV prediction design in the TMuC document includes the interleaved motion vector prediction (IMVP). Motion vector competition is applied on the vertical component. The set of predictors contains the median motion vector computed from neighboring motion vectors, as in AVC, and possibly the motion vector associated to the collocated block in the previously decoded frame. The configuration (set of predictors) is signaled at the slice level.

The current version of the software contains IMVP, which is enabled by default, and MV-Comp (also called AMVP), which is disabled by default. This motion vector competition uses up to 5 predictors applied on Inter and Skip blocks: 1- left neighbor, 2- top neighbor, 3- either top-right, bottom-left or top-left neighbor, 4- median, 5- scaled collocated.

# Experimental Results

Test conditions:

- Version 32 of the software was used, with configuration files based on "low complexity" variants.

- Only approximately 1 second of each sequence was encoded (25, 33, 49 or 57 frames for 24Hz, 30Hz, 50Hz or 60Hz sequences, respectively).

- SymbolMode: 2 (pipe) is used here, rather than 0 (cavlc).

Table 1 shows the results obtained for CS1 and CS2 for two variants. The reference is the TMuC v32 with default configuration. This corresponds to the define #define HHI_AMVP_OFF set to 1 in the TypeDef.h file.  Both MRG:1 and IMP:1 were set in the configuration file.  I.e. IMVP is enabled and MV-Competition is disabled.

Test 1: the first test includes both IMVP and motion vector competition. Motion vector competition is as implemented in the AMVP, and performed only on the vertical component of the motion vector. This corresponds to the define #define HHI_AMVP_OFF set to 0 in TypeDef.h file. Note that this is not exactly what is specified in the TMuC document, given that more predictors are used.

Test 2: the second test disables IMVP, and enables motion vector competition for both vector components and skip mode. This corresponds to the define #define HHI_AMVP_OFF set to 0 in the TypeDef.H file, and both MRG:0 and IMP:0 were used in the configuration file.

|  | CS1 |  |  | CS2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test | Avg | Min | Max | Avg | Min | Max |
| Test 1: AMVP1-MRG1-IMVP1 | -0.01% | -0.37% | +0.42% | -0.00% | -0.54% | +0.73% |
| Test 2: AMVP1-MRG0-IMVP0 | +1.35% | +0.10% | +2.42% | +1.29% | -0.69% | +2.77% |

Table 1: IMVP + MV-Comp and MV-Comp alone compared to IMVP alone, for CS1 and CS2.

Figures 1 and 2 present the same results, sequence by sequence.

Figure 1: IMVP + MV-Comp and MV-Comp alone compared to IMVP alone, for CS1.

Figure 2: IMVP + MV-Comp and MV-Comp alone compared to IMVP alone, for CS2.

We can observe that at this stage that neither the default TMuC configuration nor the method that is close to the one specified in the TMuC document provides the best result.

# Conclusion

According to these results, we propose to set up a Tool Experiment on this topic.

The goal of this TE would be to study the MV-Prediction tools currently listed in the TMuC document (MV-Competition (MV-Comp), and Interleaved Motion Vector Prediction (IMVP)), and check how they can be efficiently used, and to propose an adapted design for the TMuC at the next meeting in October.

# Configuration Files, Encoder Output Headers

## IPP Example Encoder Output

TMuC SW candidate: Encoder Version [0.0][Windows][VS 1400][32 bit]

Input          File          : c:/sequences/wqvga/RaceHorses_416x240_30.yuv

Bitstream      File          : str/RACEHORSES_416x240_30_IPPP_QP21.8.265

Reconstruction File          : rec/RACEHORSES_416x240_30_enc_IPPP_QP21.8.yuv

Real     Format              : 416x240 30Hz

Internal Format              : 416x240 30Hz

Frame index                  : 0 - 32 (33 frames)

Number of Ref. frames (P)    : 2

Number of Ref. frames (B_L0) : 1

Number of Ref. frames (B_L1) : 1

Number of Reference frames   : 2

CU size / depth              : 64 / 4

Transform depth (min / max)  : 0 / 1

Motion search range          : 64

Intra period                 : -1

QP                           : 22.00

GOP size                     : 1

Rate GOP size                : 1

Max physical trans. size     : 64

Bit increment                : 0

Luma interpolation           : TEN directional interpolation filter

Chroma interpolation         : TEN two-stage bi-linear filter

Entropy coder                : PIPE

TOOL CFG: ALF:0 IBD:0 HAD:1 SRD:1 RDQ:1 SQP:0 ASR:0 PAD:0 LDC:1 NRF:1 BQP:0 QBO:0 GPB:0 FEN:1 RQT:1 CIP:1 ROT:1 AIS:1 MRG:0 IMP:0

## HB7 Example Encoder Output

TMuC SW candidate: Encoder Version [0.0][Windows][VS 1400][32 bit]

Input          File          : c:/sequences/wqvga/RaceHorses_416x240_30.yuv

Bitstream      File          : str/RACEHORSES_416x240_30_HierB7_QP20.7.265

Reconstruction File          : rec/RACEHORSES_416x240_30_enc_HierB7_QP20.7.yuv

Real     Format              : 416x240 30Hz

Internal Format              : 416x240 30Hz

Frame index                  : 0 - 32 (33 frames)

Number of Ref. frames (P)    : 2

Number of Ref. frames (B_L0) : 1

Number of Ref. frames (B_L1) : 1

Number of Reference frames   : 2

CU size / depth              : 64 / 4

Transform depth (min / max)  : 0 / 1

Motion search range          : 64

Intra period                 : 32

QP                           : 21.00

GOP size                     : 8

Rate GOP size                : 8

Max physical trans. size     : 64

Bit increment                : 0

Luma interpolation           : TEN directional interpolation filter

Chroma interpolation         : TEN two-stage bi-linear filter

Entropy coder                : PIPE

TOOL CFG: ALF:0 IBD:0 HAD:1 SRD:1 RDQ:1 SQP:0 ASR:0 PAD:0 LDC:0 NRF:1 BQP:0 QBO:0 GPB:0 FEN:1 RQT:1 CIP:1 ROT:1 AIS:1 MRG:0 IMP:0

# Patent rights declaration(s)

Orange may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).