| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 2nd Meeting: Geneva, CH, 21-28 July, 2010 | Document: JCTVC-B102 |
| --- | --- |

| Title: | Mode-dependent residual reordering for Intra prediction residual |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao Institute of Computing Technology, Chinese Academy of Sciences Beijing, China Li Zhang, Siwei Ma, Wen Gao Institute of Digital Media, Peking University Beijing, China | Tel: Email: | +86-010-62753424 xzhao@jdl.ac.cn zhanglili@jdl.ac.cn swma@jdl.ac.cn |
| Source: | Peking University |  |  |

_____________________________

# Abstract

In this contribution, a mode-dependent residual reordering (MDRR) method is proposed to further simplify the mode-dependent directional transform (MDDT) for Intra coding. In the proposed MDRR, between the prediction and transform stages, a certain kind of reordering is implemented on the residual samples for each mode in spatial. The reordering is manipulated in a way that the distribution statistics of reordered residual samples present less mode-dependent characteristic. After the reordering, the nine Intra modes in I4MB and I8MB modes will be assigned into three groups, and only one transform matrix is assigned for each group. With the proposed MDRR, the number of transform matrices in MDDT is reduced from original 18, 18 and 8 to the proposed 3, 3 and 3 for I4MB, I8MB and I16MB, respectively. Experimental results show that, under the testing condition specified in the AhG report on alternative transforms with all-Intra conditions, compared with MDDT, average BD rate increase of 0.1% and BD PSNR drop of -0.005dB is observed.

# Problem Statement

For Intra coding, due to the mode-dependent distribution characteristics of residual samples, mode-dependent transforms are proposed in MDDT, and superior coding performance over conventional DCT is achieved [1]. Furthermore, a novel rate-distortion optimized transform (RDOT) which employs multiple candidate transform matrices for each Intra mode has also been proposed [2] to collaboratively work with MDDT, and further improvements on the coding performance is achieved.

Although both MDDT and RDOT further refine the transform for Intra prediction residuals by applying more transform functions, the complexity is also increased. In MDDT, non-orthogonal column and row transform matrices are employed, and total 18, 18 and 8 transform matrices are needed for I4MB, I8MB and I16MB, respectively. To simplify MDDT, an orthogonal MDDT (OMDDT) [3] method is proposed by applying orthogonal column and row transforms. With OMDDT, the number of candidate transform matrices is halved, and the matrix multiplication in transform can be reused.

To further reduce the number of transform matrices, in this contribution, a mode-dependent residual reordering method is proposed.

# Proposed method

To illustrate the idea of MDRR, comparisons of original MDDT and simplified MDDT using MDRR is shown in Fig. 1 and 2, respectively. As it is shown, for mode 0, 1 and 7 in I4MB and I8MB, the original MDDT employs 3 different pairs of transform matrices. However, in the simplified MDDT using proposed MDRR, the residuals are reordered prior to transform, and only one pair of orthogonal column and row transform matrix is used to these modes.

C0, R0

C1, R1

C7, R7

Prediction Direction

Residual Distribution

Mode-dependent Transform Matrices

Figure 1 Transform scheme for mode 0, 1, 7 of I4/8MB in MDDT

Residual Reordering

S, ST

Prediction Direction

Residual Distribution

Single transform matrix

Figure 2 Transform scheme for mode 0, 1, 7 of I4/8MB in simplified MDDT using MDRR

The residual reordering methods for different modes of I4MB are also shown in Fig. 3. For the case of I8MB, the reordering methods are similar.

Figure 3 Residual reordering methods for different modes of I4MB in MDRR

Furthermore, after the residual reordering, the nine modes are classified into three groups indicated in Table 1. For each group of prediction modes, a single transform matrix is used.

Table 1 Prediction modes grouping in different Intra modes

|  | Group 1 | Group 2 | Group 3 |
| --- | --- | --- | --- |
| I4MB modes (0~8) | 0, 1, 3, 7, 8 | 4, 5, 6 | 2(DC) |
| I8MB modes (0~8) | 0, 1, 3, 7, 8 | 4, 5, 6 | 2(DC) |
| I16MB modes (0~3) | 0, 1 | 3 | 2(DC) |

# Simulation results

The proposed MDDR has been incorporated into jm11.0kta2.6r1. The testing condition specified in the AhG report on alternative transforms with all-Intra conditions is used for running the simulation. Table 2 below shows BD-PSNR and BD-Rate [4] compared to jm11.0kta2.6r1 with MDDT off. Table 3 shows a comparison of the total number of transform matrices in MDDT and proposed MDRR.

Table 2 Coding gain of original MDDT and simplified MDDT using proposed MDRR compared to jm11.0kta2.6r1 with MDDT disabled

| Sequence | Original MDDT |  | Simplified MDDT using proposed MDRR |  |
| --- | --- | --- | --- | --- |
|  | BD-PSNR | BD-Rate | BD-PSNR | BD-Rate |
| BasketballPass_p416x240_50fps | 0.240 | -4.538 | 0.227 | -4.313 |
| BQSquare_p416x240_60fps | 0.348 | -4.264 | 0.365 | -4.469 |
| BlowingBubbles_p416x240_50fps | 0.211 | -3.985 | 0.220 | -4.152 |
| RaceHorses_p416x240_30fps | 0.231 | -4.088 | 0.218 | -3.883 |
| BasketballDrill_p832x480_50fps | 0.237 | -5.215 | 0.226 | -4.976 |
| BQMall_p832x480_60fps | 0.349 | -5.918 | 0.362 | -6.137 |
| PartyScene_p832x480_50fps | 0.267 | -3.933 | 0.274 | -4.027 |
| RaceHorses_p832x480_30fps | 0.186 | -3.016 | 0.188 | -3.052 |
| vidyo1_p1280x720_60fp | 0.438 | -7.675 | 0.381 | -6.735 |
| vidyo3_p1280x720_60fps | 0.528 | -8.239 | 0.528 | -8.231 |
| vidyo4_p1280x720_60fps | 0.359 | -7.209 | 0.338 | -6.804 |
| ParkScene_p1920x1080_24fps | 0.204 | -4.661 | 0.200 | -4.576 |
| Kimono1_p1920x1080_24fps | 0.277 | -6.742 | 0.267 | -6.500 |
| Cactus_p1920x1080_50fps | 0.221 | -5.409 | 0.213 | -5.217 |
| BasketballDrive_p1920x1080_50fps | 0.241 | -7.689 | 0.255 | -8.128 |
| BQTerrace_p1920x1080_60fps | 0.357 | -6.890 | 0.345 | -6.671 |
| Average | 0.293 | -5.592 | 0.288 | -5.492 |
| Best Case | 0.528 | -8.24 | 0.528 | -8.23 |
| Worst Case | 0.186 | -3.02 | 0.188 | -3.05 |

Table 3 Comparison on the number of transform matrices used in original MDDT and simplified MDDT with proposed MDRR

|  | Original MDDT | Simplified MDDT with MDRR |
| --- | --- | --- |
| I4MB modes (0~8) | 18 | 3 |
| I8MB modes (0~8) | 18 | 3 |
| I16MB modes (0~3) | 8 | 3 |

# References

[1]	M. Karczewicz, “Improved Intra Coding,” ITU-T Q.6/SG16 VCEG, VCEG-AF15, San Jose, USA, April 2007.

[2] Xin Zhao, Li Zhang, Siwei Ma, Wen Gao, “Rate-Distortion Optimized Transform”, ISO/IEC JTC1/SC29/WG11 document M12926, Oct. 2009.

[3] Madhukar Budagavi, Minhua Zhou, “Orthogonal MDDT and Mode Dependent DCT,” ITU-T Q.6/SG16 VCEG, VCEG-AM20_r1, Kyoto, Japan, Jan. 2010.

[4]	Gisle Bjontegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SC16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.

# Patent rights declaration(s)

Peking University may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).