| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L_Notes_d2 |
| --- | --- |

| Title: | Meeting report of the 12th meeting of the Joint Collaborative Team on Video Coding (JCT-VC), Geneva, CH, 14–23 Jan. 2013 |  |  |
| --- | --- | --- | --- |
| Status: | Report Document from Chairs of JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communications Engineering RWTH Aachen University Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Chairs |  |  |

_____________________________

- Summary

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its twelfth meeting during 14–23 Jan. 2013 at the ITU-T premises in Geneva, CH. The JCT-VC meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 1.14 of this document.

The JCT-VC meeting sessions began at approximately 1000 hours on Monday 14 Jan. 2013. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately 1400 hours on Wednesday 23 Jan. 2013. Approximately XXX people attended the JCT-VC meeting, and approximately XXX input documents were discussed. The meeting took place in a collocated fashion with a meeting of ITU-T SG16 – one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the new next-generation video coding standardization project known as High Efficiency Video Coding (HEVC) and its extensions.

The primary goals of the meeting were to review the work that was performed in the interim period since the eleventh JCT-VC meeting in producing the 9th HEVC Test Model (HM9) software and text and editing the 9th HEVC specification Draft (which was issued as a Study of ISO/IEC Draft International Standard document), review the results from one interim Core Experiment (CE) on range extensions and six Tool Experiments (TE) on scalable extensions, review technical input documents, establish the 10th draft of the HEVC specification (to be issued as ISO/IEC Final Draft International Standard document, also to be submitted for Consent in ITU-T), and the tenth version of the HEVC Test Model (HM10). Besides the finalization of version 1 of HEVC, important topics of the meeting were the review of progress made towards definitions of HEVC Scalable Video Coding extensions and range extensions into higher bit depths and non-4:2:0 colour sampling. Advancing the work on development of conformance and reference software for HEVC and its extensions is also a significant goal.

In addition to experiment plan descriptions, the JCT-VC produced XX other particularly important output documents from the meeting: the HEVC Test Model 10 (HM10), the HEVC specification draft 10 a.k.a. Final Draft International Standard (FDIS), the Disposition of Comments (DoC) on DIS, draft 2 for HEVC conformance testing, draft 2 for HEVC range extensions, a design description of high-level syntax for HEVC scalable and 3D extensions, SMuC, and two documents specifying common test conditions and software reference configurations for experiments – one for HEVC coding efficiency experiments, one for HEVC range extension experiments, and one for scalable coding experiments.

For the organization and planning of its future work, the JCT-VC established XX "ad hoc groups" (AHGs) to progress the work on particular subject areas. The next six JCT-VC meetings are planned for 17–26 Apr. 2013 under WG 11 auspices in Incheon, KR, 24 July – 2 Aug. 2013 under WG 11 auspices in Vienna, AT, [23?] 24–30 Oct. [1 Nov?] 2013 under ITU-T auspices in Geneva, CH, [9?] 8–17 Jan. 2014 under WG 11 auspices in San José, US, 26 March – 4 April 2014 in Valencia, ES, and [17?] 16–25 June 2014 in Geneva, CH.

The document distribution site http://phenix.it-sudparis.eu/jct/ was used for distribution of all documents.

The reflector to be used for discussions by the JCT-VC and all of its AHGs is the JCT-VC reflector:
jct-vc@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see
http://mailman.rwth-aachen.de/mailman/listinfo/jct-vc.

- Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Collaborative Team on Video Coding (JCT-VC) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/ SC 29/ WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JCT-VC are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11.

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its twelfth meeting during 14-23 Jan. 2013 at the ITU-T premises in Geneva, CH. The JCT-VC meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany).

## Meeting logistics

The JCT-VC meeting sessions began at approximately 1000 hours on Monday 14 Jan. 2013. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately 1400 hours on Wednesday 23 Jan. 2013. Approximately XXX people attended the JCT-VC meeting, and approximately XXX input documents were discussed. The meeting took place in a collocated fashion with a meeting of ITU-T SG16 – one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the new next-generation video coding standardization project known as High Efficiency Video Coding (HEVC) and its extensions.

Some statistics are provided below for historical reference purposes:

- 1st "A" meeting (Dresden, 2010-04):		188 people, 40 input documents

- 2nd "B" meeting (Geneva, 2010-07):		221 people, 120 input documents

- 3rd "C" meeting (Guangzhou, 2010-10):		244 people, 300 input documents

- 4th "D" meeting (Daegu, 2011-01):		248 people, 400 input documents

- 5th "E" meeting (Geneva, 2011-03):		226 people, 500 input documents

- 6th "F" meeting (Torino, 2011-07):		254 people, 700 input documents

- 7th "G" meeting (Geneva, 2011-11)		284 people, 1000 input documents

- 8th "H" meeting (San Jose, 2012-02)		255 people, 700 input documents

- 9th "I" meeting (Geneva, 2012-04/05)		241 people, 550 input documents

- 10th "J" meeting (Stockholm, 2012-07)		214 people, 550 input documents

- 11th "K" meeting (Shanghai, 2012-10)		235 people, 350 input documents

- 12th "L" meeting (Geneva, 2013-01)		XXX people, XXX input documents

Information regarding logistics arrangements for the meeting had been provided at 
http://wftp3.itu.int/av-arch/jctvc-site/2013_01_L_Geneva/

## Primary goals

The primary goals of the meeting were to review the work that was performed in the interim period since the eleventh JCT-VC meeting in producing the 9th HEVC Test Model (HM9) software and text and editing the 9th HEVC specification Draft (which was issued as a Study of ISO/IEC Draft International Standard document), review the results from one interim Core Experiment (CE) on range extensions and six Tool Experiments (TE) on scalable extensions, review technical input documents, establish the 10th draft of the HEVC specification (to be issued as ISO/IEC Final Draft International Standard document, also to be submitted for Consent in ITU-T), and the tenth version of the HEVC Test Model (HM10). Besides the finalization of version 1 of HEVC, important topics of the meeting were the review of progress made towards definitions of HEVC Scalable Video Coding extensions and range extensions into higher bit depths and non-4:2:0 colour sampling. Advancing the work on development of conformance and reference software for HEVC and its extensions is also a significant goal.

## Documents and document handling considerations

### General

The documents of the JCT-VC meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jct/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (other than as contribution registration and upload times) follow the local time at the meeting facility.

Highlighting of recorded decisions in this report:

- Decisions made by the group that affect the normative content of the draft standard are identified in this report by prefixing the description of the decision with the string "Decision:".

- Decisions that affect the reference software but have no normative effect on the text are marked by the string "Decision (SW):".

- Decisions that fix a "bug" in the specification (an error, oversight, or messiness) are marked by the string "Decision (BF):".

- Decisions regarding things that correct the text to properly reflect the design intent, add supplemental remarks to the text, or clarify the text are marked by the string "Decision (Ed.):".

- Decisions regarding simplification or improvement of design consistency are marked by the string "Decision (Simp.):".

- Decisions regarding complexity reduction (in terms of processing cycles, memory capacity, memory bandwidth, line buffers, number of entropy-coding contexts, number of context-coded bins, etc.) … "Decision (Compl.):".

This meeting report is based primarily on notes taken by the chairs and projected for real-time review by the participants during the meeting discussions. The preliminary notes were also circulated publicly by ftp during the meeting on a daily basis. Considering the high workload of this meeting and the large number of contributions, it should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much discussion of the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Monday, 7 Jan. 2013.

Non-administrative documents uploaded after 2359 hours in Paris/Geneva time Tuesday 8 Jan. 2013 were considered "officially late".

Most documents in the “late” category were CE reports or cross-verification reports, which are somewhat less problematic than late proposals for new action (and especially for new normative standardization action).

At this meeting, we again had a substantial amount of late document activity, but in general the early document deadline gave us a significantly better chance for thorough study of documents that were delivered in a timely fashion. The group strived to be conservative when discussing and considering the content of late documents, although no objections were raised regarding allowing some discussion in such cases.

All contribution documents with registration numbers JCTVC-L0364 to JCTVC-L0XXX were registered after the "officially late" deadline (and therefore were also uploaded late). However, some documents in the "L0364+" range include break-out activity reports that were generated during the meeting, and are therefore better considered as report documents rather than as late contributions.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly-accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions was to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This was agreed to be a helpful approach to be followed at the meeting.

The following other technical design proposal contributions were registered on time but were uploaded late:

- JCTVC-LXXXX (a proposal ...) [uploaded 01-XX]

- ...

The following other profile/level proposal contributions were registered on time but were uploaded late:

- JCTVC-LXXXX (a profile/level proposal) [uploaded 01-xx]

- ...

The following other documents not proposing normative technical content were registered on time but were uploaded late:

- JCTVC-LXXXX (advocating ...) [uploaded 01-XX]

The following cross-verification reports were registered on time but were uploaded late: JCTVC-LXXXX [uploaded 01-XX], ... .

The following document registrations were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn document, or were registered in error: JCTVC-LXXXX, ....

Ad hoc group interim activity reports, CE summary results reports, break-out activity reports, and information documents containing the results of experiments requested during the meeting are not included in the above list, as these are considered administrative report documents to which the uploading deadline is not applied.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when sufficient time for studying was given after the upload. Again, an exception is applied for AHG reports, CE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there was some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload are also a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This is especially a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the "comments" field on the web site can be used to indicate what is different in a revision.

"Placeholder" contribution documents that were basically empty of content, with perhaps only a brief abstract and some expression of an intent to provide a more complete submission as a revision, were considered unacceptable and were to be rejected in the document management system, as has been agreed since the third meeting.

The initial uploads of the following contribution documents were rejected as "placeholders" without any significant content and were not corrected until after the upload deadline:

- JCTVC-LXXXX (a ..., corrected by a late upload on 01-XX)

- ...

A few contributions had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). These issues were corrected by later uploaded versions in all cases (to the extent of the awareness of the chairs).

Some other errors were noticed in other initial document uploads (wrong document numbers in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Measures to facilitate the consideration of contributions

It was agreed that, due to the continuingly high workload for this meeting, the group would try to rely more extensively on summary CE reports. For other contributions, it was agreed that generally presentations should not exceed 5 minutes to achieve a basic understanding of a proposal – with further review only if requested by the group. For cross-verification contributions, it was agreed that the group would ordinarily only review cross-checks for proposals that appear promising.

When considering cross-check contributions, it was agreed that, to the extent feasible, the following data should be collected:

- Subject (including document number).

- Whether common conditions were followed.

- Whether the results are complete.

- Whether the results match those reported by the contributor (within reasonable limits, such as minor compiler/platform differences).

- Whether the contributor studied the algorithm and software closely and has demonstrated adequate knowledge of the technology.

- Whether the contributor independently implemented the proposed technology feature, or at least compiled the software themselves.

- Any special comments and observations made by a cross-check contributor.

### Outputs of the preceding meeting

The report documents of the previous meeting, particularly including the meeting report JCTVC-K1000, the HEVC Test Model (HM) JCTVC-K1002, the Draft Specification JCTVC-K1003, the Conformance Draft JCTVC-K1004, the Draft Specification of Range Extensions JCTVC-K1005 and the Report of Results of the Joint CfP on SVC extensions JCTVC-K1008 were approved. The HM reference software produced by the AHG on software development and HM software technical evaluation was also approved.

The group was asked to review the prior meeting report for finalization. The meeting report was later approved without modification.

All output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

The chairs asked if there were any issues regarding potential mismatches between perceived technical content prior to adoption and later integration efforts. It was also asked whether there was adequate clarity of precise description of the technology in the associated proposal contributions.

It was remarked that, in regard to software development efforts – for cases where "code cleanup" is a goal as well as integration of some intentional functional modification, it was emphasized that these two efforts should be conducted in separate integrations, so that it is possible to understand what is happening and to inspect the intentional functional modifications.

The need for establishing good communication with the software coordinators was also emphasized.

At previous meetings, it had been remarked that in some cases the software implementation of adopted proposals revealed that the description that had been the basis of the adoption apparently was not precise enough, so that the software unveiled details that were not known before (except possibly for CE participants who had studied the software). Also, there should be time to study combinations of different adopted tools with more detail prior to adoption.

CE descriptions need to be fully precise – this is intended as a method of enabling full study and testing of a specific technology.

Greater discipline in terms of what can be established as a CE may be an approach to helping with such issues. CEs should be more focused on testing just a few specific things, and the description should precisely define what is intended to be tested (available by the end of the meeting when the CE plan is approved).

It was noted that sometimes there is a problem of needing to look up other referenced documents, sometimes through multiple levels of linked references, to understand what technology is being discussed in a contribution – and that this often seems to happen with CE documents. It was emphasized that we need to have some reasonably understandable description, within a document, of what it is talking about.

Software study can be a useful and important element of adequate study; however, software availability is not a proper substitute for document clarity.

Software shared for CE purposes needs to be available with adequate time for study. Software of CEs should be available early, to enable close study by cross-checkers (not just provided shortly before the document upload deadline).

Issues of combinations between different features (e.g., different adopted features) also tend to sometimes arise in the work.

## Attendance

The list of participants in the JCT-VC meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/‌SC 29/‌WG 11 (including experts who had been personally invited by the Chairs as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the Chairs.

## Agenda

The agenda for the meeting was as follows:

- IPR policy reminder and declarations

- Contribution document allocation

- Reports of ad hoc group activities

- Reports of Core Experiment and Tool Experiment activities

- Review of results of previous meeting

- Consideration of contributions and communications on HEVC project guidance

- Consideration of HEVC technology proposal contributions

- Consideration of information contributions

- Coordination activities

- Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments, establishment of AHGs, meeting planning, refinement of expected standardization timeline, other planning issues

- Other business as appropriate for consideration

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JCT-VC and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JCT-VC as necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jctvc-site (JCT-VC contribution templates)

- http://www.itu.int/ITU-T/studygroups/com16/jct-vc/index.html (JCT-VC general information and founding charter)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/ SC 29 Procedures)

It is noted that the ITU TSB director's AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

"TSB has reported to the TSB Director’s IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur’s group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation."

The chairs invited participants to make any necessary verbal reports of previously-unreported IPR in draft standards under preparation, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that, as had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time, the HEVC reference software copyright license header language is the BSD license with preceding sentence declaring that contributor or third party rights are not granted, as recorded in N10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software is used in the process of designing the new HEVC standard and for evaluating proposals for technology to be included in this design. Additionally, after development of the coding technology, the software will be published by ITU-T and ISO/IEC as an example implementation of the HEVC standard and for use as the basis of products to promote adoption of the technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

## Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jct/. For the first two JCT-VC meetings, the JCT-VC documents had been made available at http://ftp3.itu.int/av-arch/jctvc-site, and documents for the first two JCT-VC meetings remain archived there as well. That site was also used for distribution of the contribution document template and circulation of drafts of this meeting report.

JCT-VC email lists are managed through the site http://mailman.rwth-aachen.de/mailman/options/jct-vc, and to send email to the reflector, the email address is jct-vc@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JCT-VC participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages and subscribers must respond to inquiries regarding the nature of their interest in the work.

It was emphasized that usually discussions concerning CEs and AHGs should be performed using the reflector. CE internal discussions should primarily be concerned with organizational issues. Substantial technical issues that are not reflected by the original CE plan should be openly discussed on the reflector. Any new developments that are result of private communication cannot be considered to be the result of the CE.

For the case of CE documents and AHG reports, email addresses of participants and contributors may be obscured or absent (and will be on request), although these will be available (in human readable format – possibly with some "obscurification") for primary CE coordinators and AHG chairs.

## Terminology

Some terminology used in this report is explained below:

- AHG: Ad hoc group.

- AI: All-intra.

- AIF: Adaptive interpolation filtering.

- ALF: Adaptive loop filter.

- AMP: Asymmetric motion partitioning.

- AMVP: Adaptive motion vector prediction.

- APS: Active parameter sets.

- AU: Access unit.

- AUD: Access unit delimiter.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BA: Block adaptive.

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BL: Base layer.

- BoG: Break-out group.

- BR: Bit rate.

- CABAC: Context-adaptive binary arithmetic coding.

- CBF: Coded block flag(s).

- CD: Committee draft – the first formal ballot stage of the approval process in ISO/IEC.

- CE: Core experiment – a coordinated experiment conducted after the 3rd or subsequent JCT-VC meeting and approved to be considered a CE by the group.

- Consent: A step taken in ITU-T to formally consider a text as a candidate for final approval (the primary stage of the ITU-T "alternative approval process").

- CTC: Common test conditions.

- CVS: Coded video sequence.

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics).

- DCTIF: DCT-derived interpolation filter.

- DIS: Draft international standard – the second formal ballot stage of the approval process in ISO/IEC.

- DF: Deblocking filter.

- DT: Decoding time.

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element).

- EL: Enhancement layer.

- ET: Encoding time.

- HE: High efficiency – a set of coding capabilities designed for enhanced compression performance (contrast with LC). Often loosely associated with RA.

- HEVC: High Efficiency Video Coding – the video coding standardization initiative under way in the JCT-VC.

- HLS: High-level syntax.

- HM: HEVC Test Model – a video coding design containing selected coding tools that constitutes our draft standard design – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM).

- IBDI: Internal bit-depth increase – a technique by which lower bit depth (8 bits per sample) source video is encoded using higher bit depth signal processing, ordinarily including higher bit depth reference picture storage (ordinarily 12 bits per sample).

- ILP: Inter-layer prediction (in scalable coding).

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC).

- JM: Joint model – the primary software codebase that has been developed for the AVC standard.

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions.

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures.

- LC: Low complexity – a set of coding capabilities designed for reduced implementation complexity (contrast with HE). Often loosely associated with LD.

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Often loosely associated with LC. Typically refers to LB, although also applies to LP.

- LM: Linear model.

- LP or LDP: Low-delay P – the variant of the LD conditions that uses P frames.

- LUT: Look-up table.

- LTRP: Long-term reference pictures

- MANE: Media-aware network elements.

- MC: Motion compensation.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JCT-VC).

- MV: Motion vector.

- NAL: Network abstraction layer (as in AVC).

- NB: National body (usually used in reference to NBs of the WG 11 parent body).

- NSQT: Non-square quadtree.

- NUH: NAL unit header.

- NUT: NAL unit type (as in AVC).

- OBMC: Overlapped block motion compensation.

- PCP: Parallelization of context processing.

- POC: Picture order count.

- PPS: Picture parameter set (as in AVC).

- QM: Quantization matrix (as in AVC).

- QP: Quantization parameter (as in AVC, sometimes confused with quantization step size).

- QT: Quadtree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD). Often loosely associated with HE.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- ROT: Rotation operation for low-frequency transform coefficients.

- RPS: Reference picture set

- RQT: Residual quadtree.

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q).

- RVM: Rate variation measure.

- SAO: Sample-adaptive offset.

- SDIP: Short-distance intra prediction.

- SEI: Supplemental enhancement information (as in AVC).

- SD: Slice data; alternatively, standard-definition.

- SH: Slice header.

- SHVC: Scalable high efficiency video coding.

- SPS: Sequence parameter set (as in AVC).

- TB: transform block.

- TE: Tool Experiment – a coordinated experiment conducted toward HEVC design between the 1st and 2nd or 2nd and 3rd JCT-VC meeting, or a coordinated experiment conducted toward SHVC design between the 11th and 12th JCT-VC meeting.

- TFD: Tagged for discard.

- Unit types:

  - CTB: code tree block (synonymous with LCU).

  - CU: coding unit.

  - LCU: (formerly LCTU) largest coding unit (synonymous with CTB).

  - PU: prediction unit, with four shape possibilities.

    - 2Nx2N: having the full width and height of the CU.

    - 2NxN: having two areas that each have the full width and half the height of the CU.

    - Nx2N: having two areas that each have half the width and the full height of the CU.

    - NxN: having four areas that each have half the width and half the height of the CU.

  - TU: transform unit.

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JCT-VC).

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy.

- WD: Working draft – the draft HEVC standard corresponding to the HM.

- WG: Working group (usually used in reference to WG 11, a.k.a. MPEG).

## Liaison activity

The JCT-VC did not send or receive formal liaison communications at this meeting.

## Opening remarks

The status of the HEVC draft text in ISO/IEC and ITU-T was noted. A DIS ballot had been issued in ISO/IEC and had closed prior to the meeting (WG doc number Mxxxxx, make it available?). In ITU-T, the text status remained as preparation for Consent (describe submission action).

It was noted that in the most-recently-established voting process in ISO/IEC, a "No" vote has a different status than it previously did for the DIS ballot stage. WG 11 NBs should make sure to be aware of the implications of their votes, and may wish to consider voting "Yes with comments" in some circumstances in which they would previously have been inclined to vote "No with comments".

## Scheduling of discussions

Scheduling: Generally meeting time was scheduled during 0800 – 2200, with coffee and lunch breaks as convenient.

First day (Mon. 14 Jan.): 1000–2000, morning plenary, Tracks A and B in afternoon.

Note: The opening plenary of SG 16 was held at 14:30 on the 1st day.

Second day (Tue. 15 Jan.): 1115–XXXX, Tracks A and B

Note: The opening plenary of WP3/16 was held at 0930 on the 2nd day.

From the third day onwards, meeting sessions generally began at 0800.

On Wed. 23 Jan., the meeting was to end by lunchtime (and did). The meeting was closed at XXXX on 23 January.

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized and categorized into "tracks" (A, B, or P) for "parallel session A", "parallel session B", or "Plenary" review, as follows. Discussions on topics categorized as "Track A" were primarily chaired by Jens-Rainer Ohm, and discussions on topic categorized as "Track B" were primarily chaired by Gary Sullivan. Some plenary sessions were chaired by both co-chairmen, and others were chaired by Gary Sullivan. (Note: allocation to tracks were subject to changes)

- AHG reports (12) Track P (section 2)

- Communication to and by parent bodies (3) Track P (section 3.1)

- Conformance testing development (2) Track P (section 3.2)

- Draft text specification of version 1 (2) Track B (section 3.3)

- Coding performance, implementation, and design analysis (9) Track P (section 3.4)

- Profile, level and constraint definitions (3) Track P (section 3.5)

- Source video test material (2) Track B (section 3.6)

- RExt CE1: Square and non-square transforms in range extensions (11) Track B (section 4.1)

- SHVC Tool Experiments (133) Track A (section 5)

  - SHVC TE1: Upsampling filter in SHVC (7) Track A (section 5.1)

  - SHVC TE2: Inter-layer texture prediction in SHVC (29) Track A (section 5.2)
	BoG [G1] (L. Guo)

  - SHVC TE3: Combined prediction in SHVC (32) Track A (section 5.3)

  - SHVC TE4: Inter-layer filtering in SHVC (19) Track A (section 5.4)

  - SHVC TE5: Inter-layer syntax prediction using HEVC base layer (44) Track A (section 5.5)

  - SHVC TE6: Inter-layer syntax prediction using AVC base layer (2) Track A (section 5.6)

- HEVC version 1 (36) Track B (sections 6.1–6.3)

  - Deblocking filter in version 1 (3) Track B (section 6.1)
	BoG [Room T072] (A. Norkin)

  - High-level syntax in version 1 (30) Track B (section 6.2)

  - Temporal MV prediction "hook" in version 1 (3) (section 6.3)

  - Other clean-up topics for version 1 (1) (section 6.4)

- RExt (29) Track B (section 6.5)
	BoG [Room K2, then H] (D. Flynn)

  - RExt: Transforms (CE1 related) (5) Track B (section 6.5.1)

  - RExt: Intra coding (9) Track B (section 6.5.2)

  - RExt: Mixed format coding (7) Track B (section 6.5.3)

  - RExt: Other (8) Track B (section 6.5.4)

- SHVC (113) Track A (section 6.6)

  - SHVC: Upsampling filter (TE1 related) (4) Track A (section 6.6.1)

  - SHVC: Inter-layer texture prediction (TE2 related) (6) Track A (section 6.6.2)

  - SHVC: Combined prediction (TE3 related) (23) Track A (section 6.6.3)

  - SHVC: Inter-layer filtering (TE4 related) (22) Track A (section 6.6.4)

  - SHVC: Inter-layer syntax prediction w. HEVC base (TE5 related) (31) Track A (section 6.6.5)

  - SHVC: Tool combinations (6) Track A (section 6.6.6)

  - SHVC: Low-complexity SNR scalability (4) Track B (section 6.6.7)

  - SHVC: Modifications of ref_idx scheme (9) Track B (section 6.6.8)

  - SHVC: Transforms (4) Track B (section 6.6.9)

  - SHVC: Other modalities (5) Track B (section 6.6.10)

- High-level syntax in SHVC and 3D extensions (30) Track B (section 6.7)

- Lossless compression (8) Track B (section 6.9)

- Interlaced scan and field-based video (2) Track B (section 6.10)

- Non-normative (4) Track B (section 6.11)

- Unclear category (0) (section 6.12)

- Plenary discussions and BoG reports (section 7)

- Outputs & planning: AHG & CE plans, Conformance, Chroma format, CTC. (section 10)

NOTE – The number of contributions noted in each category, as shown in parenthesis above, may not be 100% precise.

Overall approximate contribution allocations: Track P: 27; Track A: 225; Track B: 144.

# AHG reports

The activities of ad hoc groups (AHGs) that had been established at the prior meeting are discussed in this section.

JCTVC-L0001 JCT-VC AHG report: Project Management (AHG1) [G. J. Sullivan, J.-R. Ohm (AHG chairs)]

TBA - reviewed

JCTVC-L0002 JCT-VC AHG report: HEVC Draft and Test Model editing (AHG2) [B. Bross, K. McCann (co-chairs), W.-J. Han, I.-K. Kim, J.-R. Ohm, K. Sugimoto, G. J. Sullivan, T. Wiegand (vice-chairs)]

TBA - reviewed

JCTVC-L0003 JCT-VC AHG report: Software development and HM software technical evaluation (AHG3) [F. Bossen, D. Flynn, K. Sühring]

- Delays occurred in SW 9.1 development (mainly due missing HL syntax implementations)

- AHG recommends to keep changes to HL syntax as low as possible

TBA – reviewed

Remark: There are a number of bugs on the ticket system, and participants are invited to review them.

Remark: Note that there is also a ticket system for spec problems – the editor remarked that any non-editorial aspects of active tickets are also covered by input contributions.

JCTVC-L0004 JCT-VC AHG report: HEVC Still Picture profile (AHG4) [K. Ugur (chair), J.-R Ohm, G. J. Sullivan, A. Tabatabai (vice-chairs)]

The report noted the submission of four relevant contributions:

- L0277 proposing changes to the drafted Main Still Picture profile

- L0244 proposing to define a 10 bit still picture profile (which would seem to need parent-body review)

- L0041 and L0380 providing information on the compression capability of the Main Still Picture profile.

JCTVC-L0005 JCT-VC AHG report: HEVC conformance test development (AHG5) [T. Suzuki, W. Wan, C. Fogg]

The ftp site at ITU-T is used to exchange bitstreams. The ftp site for downloading bitstreams is,

http://wftp3.itu.int/av-arch/jctvc-site/bitstream_exchange/

A spreadsheet to summarize the status of bitstream exchange, conformance bitstream generation is available at this directory. It includes the list of bitstreams, codec features and settings, and status of verification.

The schedule to prepare conformance bitstreams is as follows.

- Design conformance bitstream features for each categories

- And provide bitstream specification text for conformance specification by the end of Dec. 2012

- Finish the first bitstream exchange based on HM9.1 syntax by Geneva meeting

- Then identify problems

- Revise the bitstreams based on the final standard after Jan. 2013.

- The revised bitstreams should be ready by the 13rd JCT-VC meeting (April, 2013)

A list of the candidate of the conformance bitstream and its volunteers are summarized in a table in the report. 99 bitstreams were generated and those are available at
http://wftp3.itu.int/av-arch/jctvc-site/bitstream_exchange/under_test/

The features and conformance point of each bitstream are summarized in an attached Excel sheet.

Some experts suggested establishing a mailing list for bitstream exchange and conformance. It is not necessary to discuss low level working issues in JCT-VC mailing list. But there was also a suggestion that everything should be discussed by JCT-VC main reflector. This should be discussed and decide how we communicate.

In the last meeting, it was suggested to upload the bitstream, short description, trace file, decoded image, md5 checksum in a zip archive. All files inside the zip archive should have the same base name. Only the extension is changed in the following way:

- .bit - bitstream

- .txt - description

- .trc - tace file

- .md5 - MD5 sum of the complete decoded file

- .yuv - decoded image

Note: Trace file is useful for debugging and it is recommend to include short trace file. Full trace is optional. Decoded image and MD5 checksum are optional. But one of the decoded YUV file, md5 checksum file, or hash SEI in bitstream must be provided.

During bitstream generation, it was suggested to mandate to provide md5 file. Bitstream files become huge and md5 can reduce the file size. Other advantage is that conformance bitstream could independent from HM software.

This should be discussed during Geneva meeting and should decide what should be provided.

During the bitstream generation, bitstreams were suggested. The followings are still missing among the suggested bitstream types.

- "lossless" (with selection of this mode on a CU-by-CU basis)

- long-term reference pictures

- all NAL unit types

- clipping operation of weighted prediction

Those bitstreams should be provided.

Two relevant contributions were noted as follows:

- JCTVC-L0157 is the editor’s proposed draft specification of conformance testing. The descriptions of detail specification of some bitstreams are still missing. Those should be reviewed and updated.

- JCTVC-L0158 is the guideline to generate bitstreams. This guideline should be updated after the discussion.

The AHG recommended

- to review related contributions during the meeting

- to identify volunteers for missing bitstreams

- to continue to collect more bitstreams

- to update all bitstreams based on the FDIS spec after Geneva meeting

- to reconsider necessity of tools if no conformance bitstream is provided

It was remarked that keeping the discussion on the current main email reflector is desirable to ensure that sufficient attention is being paid to the conformance development activity. It was therefore agreed to keep the discussion on the main email reflector.

It was also agreed to use the bug tracking system for conformance test development as well as for text and software development.

The report discussed the need for md5 checksums and hash SEI with the bitstreams, and the need for decoded YUV files. It was agreed that the md5 checksums should be provided (for entire decoded video sequences), and that hash SEI is also desirable (but not absolutely required if difficult for a contributor to provide). The YUV files seem too big to try to include (at least in most cases).

It was suggested that having md5 checksums for the coded bitstreams is also desirable, as well as for the decoded video data. This was agreed.

JCTVC-L0006 JCT-VC AHG report: HEVC in-loop filtering (AHG6) [T. Yamakage, A. Norkin]

A bug fix patch had been provided and included in HM 9.1.

The AHG report suggested to use two types of sequences for testing deblocking filtering. "Type 1" sequences are to be used to evaluate the proposed deblocking modifications on critical sequences. "Type 2" sequences are used to ensure there is no subjective quality loss on the common test set.

Relevant contributions noted included the following:

- JCTVC-L0232 [Ericsson] AHG6: On deblocking filter and parameters signaling

  - JCTVC-L0404 [Samsung] AhG6: Cross-check for deblocking filter process and parameter modifications suggested in JCTVC-L0232

- JCTVC-L0386 [Qualcomm] AHG6: On HEVC block artifact reduction

  - JCTVC-L0397 [Sony] AHG6: cross check of JCTVC-L0386 On HEVC block artifact reduction

Subjective testing was planned to occur during the meeting.

A suggestion for the subjective viewing candidates is as follows:

- Anchor: HM9.0 common test conditions

- Test 1: L0232-1 (modified config (non-normative))

- Test 2: L0386 (non-normative)

- Test 3: L0232-1 + L0232-3 (strong filter modification (normative))

- Test 4: L0232-1 + L0232-2 (TU size restriction (non-normative)) + L0232-3

- Test 5: L0232-1 + L0232-2 (if possible)

L0007 (range extensions)

TBA - reviewed

Issue noted: Potential need for version 1 SPS syntax change.

Plan BoG (D. Flynn) to begin Tuesday.

Topic noted: Lossless & screen content coding (incl. dictionary coder)

JCTVC-L0008 JCT-VC AHG report: Screen Content Coding (AHG8) [W. Gao (chair), M. Budagavi, R. Cohen, A. Duenas, T. Lin, J. Xu (vice-chairs)]

Test sequences were noted – existing and new ones.

In the 11th JCT-VC meeting, four sequences had been contributed and can be used as test sequence candidates for screen content coding. The four sequences are listed as follows;

- VenueVu_1920x1080_30 (BBC)

- cad_waveform_1920x1080_20 (Tongji Univ.)

- pcb_layout_1920x1080_20 (Tongji Univ.)

- ppt_doc_xls_1920x1080_20 (Tongji Univ.)

Note that the for the four sequences, both 8-bit RGB format and 8-bit YUV444 format are provided except for VenueVu, 10-bit YUV format is provided, in JCT-VC ftp server.  During the interim period, six more screen captured sequences are provides, listed as follows:

- video_conferencing_doc_sharing_1280x720_30 (Tongji Univ.)

- web_browsing_1280x720_30 (Tongji Univ.)

- cg_twist_tunnel_1280x720_30 (Tongji Univ.)

- wordEditng_1280x720_60 (Huawei & MERL)

- programming_1280x720_60 (Huawei & MERL)

- map_1280x720_60 (Huawei & MERL)

Note that for the six new sequences, both 8-bit RGB format and 8-bit YUV 4:4:4 format are provided and uploaded in the JCTVC ftp server. For purpose of illustration, one sample frame from each of the six new sequences is shown below.

In short, we have total 10 sequences with both RGB and YUV444 format. In addition, we also have four Class F sequences with YUV420 format. Thus total 14 sequences are available for SCC test. Since each sequence has different characteristics and is useful to evaluate coding tools for screen content coding,  we recommend using all of them in the AGH8 experiments.

Five related contributions were noted in the AHG report – L0301 and L0317 on test material, L0302 on lossless coding test results, and two (L0303, L0313) technical proposals.

It was noted that there are additional contributions on lossless coding that had been submitted (list).

The HM-range-extensions branch was suggested as the base software for AhG8-related work.

It was remarked that the inclusion of more efficient lossless coding tools may mostly be beneficial for a lossless profile, not that much for mixed operating mode

It was remarked that the potential need for additional profiles should be discussed at the parent-body level. It was noted that for many of the tested sequences, the bit rate would fit within a general-purpose profile constraint, and that mixed content could benefit from specialized coding tools.

JCTVC-L0009 JCT-VC AHG report: High-level syntax (AHG9) [Y.-K. Wang, G. J. Sullivan]

There have not been many discussions related to the mandates of this ad-hoc group over the reflector between the 11th JCT-VC meeting and the 12th JCT-VC meeting. However, a substantial number of relevant contributions had been submitted for consideration at the current meeting.

Input documents related to the mandates of this AHG are categorized to different topics as follows, wherein the numbers indicates the numbers of documents of the corresponding topic categories.

Altogether there were reportedly 39 high-level syntax input documents for HEVC Version 1, and 21 non-Version-1 high-level syntax documents.

HEVC Version 1 (39):

- General high-level syntax cleanups (see section 6.2.1) (10)

- NAL unit header (see section 6.2.2) (2)

- Parameter sets (see section 6.2.3) (9)

- Slices and slice header  (see section 6.2.4) (4)

- Parallel processing (see section 6.2.5) (2)

- HRD (see section 6.2.6) (3)

- Frame packing (see section 6.2.7) (4)

- SEI messages (see section 6.2.8) (5)

Non-Version-1 (21):

- SHVC high-level syntax (see section 6.7.1) (5)

- Generic high-level syntax for both SHVC and 3D extensions (see section 6.7.2)

  - Non-VPS (see section 6.7.2.1) (4)

  - VPS extensions (see section 6.7.2.2) (12)

- JCTVC-L0010 JCT-VC AHG report: SHVC tool experiments (AHG10) [X. Li (chair), J. Boyce, P. Onno, Y. Ye (vice-chairs)]

Presented in Track A

AHG discussed test conditions for TEs, provided an Excel sheet and anchor sequences.

Questions raised by AHG:

- Whether YUV files should be output during decoding. (in some experiments this was done, in others not, which may have implication on runtime. If however the same is done for anchor and technology under test, it should not make a big difference).

A: Should be done consistently in future experiments.

- Whether support temporal scalability in SHVC common test conditions.

A: Unclear what benefit that would have, unless a combination of temporal and other scalability would be tested.

- Whether test combined scalability (such as one spatial layer and one SNR layer)? Currently it is unknown whether >2 layers scalability works with current software.

A: Should be investigated in upcoming AHG

- JCTVC-L0011 JCT-VC AHG report: SHVC software (AHG11) [V. Seregin (chair), T. Chuang, Y. He, D. Kwon, F. Le Leannec (vice chairs)]

Activities included completing the porting K0348 software base into HM8.1, porting “reference index framework” from the K0034 and implementing upsampling filter from K0378. As a result, primary software base for SHVC development is now available.

The summary of the activities performed according to the mandates are provided below.

- Software was prepared according to the mandate, during porting and implementation activities the intermediate versions of the software were released to the involved parties for verification.

- The first version 0.1 of the software was delivered according to the schedule.

- The missed RD cost of Intra-BL flag at encoder was fixed in version 0.1.1.

- Development of the software with the regard of supporting AVC base was coordinated with TE6 participants.

As a result of above mentioned activities, current software includes the following:

- Intra-BL framework

- Reference index or HLS only framework

- AVC base layer input support

The most recent software version for the experiments is 0.1.1. Another software version 0.1.1avc contains AVC base layer support and is used in TE6 experiments.

After software release, the issue with misaligned forward and inverse transforms for 4x4 luma transform units in Intra-BL was reported. A contribution JCTVC-L0204 discusses the solution for this problem.

Recommendations of the AHG

- Continue to develop reference software based on version 0.1.1 and improve its quality.

- It is helpful if a bug tracker could be used for software development.

We need a way to avoid divergence between HM and future SHVC software.

Key progress: Only one SHVC software codebase.

However, the codebase is currently based on HM 8 – it is necessary to align with the current base spec software (on an ongoing basis).

- JCTVC-L0012 JCT-VC AHG report: SHVC upsampling and downsampling filters (AHG12) [A. Segall, E. Alshina, J. Chen, P. Topiwala, M. Zhou]

Not much activity in AHG, but many TE contributions (TE1, TE4) on upsampling filters.

# Project development, status, and guidance

## Communication to and by parent bodies

JCTVC-L0409 CANNB comments on HEVC [late]

JCTVC-L0415 UKNB Comment on Main 10 Profile of HEVC [UK National Body] [late]

NB ballot comments

## Conformance test set development

JCTVC-L0157  Editor's proposed draft HEVC conformance spec [T. Suzuki, G. Sullivan, W. Wan]

JCTVC-L0158 Guideline to generate conformance bitstreams [T. Suzuki (Sony), W. Wan (Broadcom)] [late]

## Draft text specification of version 1

See also JCTVC-L0363 regarding clarity of text relating to slice_temporal_mvp_enable_flag and request for review of editors' notes in JCTVC-L0030.

JCTVC-L0030 Proposed Editorial Improvements for High efficiency video coding (HEVC) Text Specification Draft 9 (SoDIS) [B. Bross, G. J. Sullivan, Y.-K. Wang]

Contains "[Ed" comments for review.

## HEVC coding performance, implementation demonstrations and design analysis

### Coding performance

JCTVC-L0041 AHG4: Performance evaluation of HEVC on still picture coding [K. Ugur, J. Lainema (Nokia)]

JCTVC-L0380 AhG4: Subjective evaluation of HEVC intra coding for still image compression [Philippe Hanhart, Martin Rerabek, Pavel Korsunov, Touradj Ebrahimi (EPFL)] [late]

JCTVC-L0191 Comparison of Compression Performance of HEVC Draft 9 with AVC for UHD-1 material [P. Andrivon, P. Bordes, P. Lopez]

JCTVC-L0322 Comparison of Compression Performance of HEVC Draft 9 with AVC High Profile and Performance of HM9.0 with Temporal Scalability Characteristics [B. Li (USTC), G. J. Sullivan, J. Xu (Microsoft)]

JCTVC-L0304 Crosscheck for HM9.0 with temporal scalability characteristics (JCTVC-L0322) [W. Zhang, L. Xu, Y. Han, Z. Deng, X. Cai, Y. Chiu (Intel)] [late]

### Implementation demonstrations

JCTVC-L0096 Implementation of a HEVC hardware decoder [S. Cho, H. Kim (ETRI)] [late]

JCTVC-L0098 On software complexity: decoding 4K60p content on a laptop [T. K. Tan, Y. Suzuki (NTT DOCOMO), F. Bossen (DOCOMO Innovations)]

JCTVC-L0379 HEVC Real-time Hardware Encoder for HDTV signal [A. Minezawa, H. Sakate, N. Motoyama, S. Sekiguchi (Mitsubishi), Y. Sugito, K. Iguchi, A. Ichigaya, S. Sakaida (NHK)] [late]

JCTVC-L0380 AhG4: Subjective evaluation of HEVC intra coding for still image compression [Philippe Hanhart, Martin Rerabek, Pavel Korsunov, Touradj Ebrahimi (EPFL)] [late]

    - Design analysis

JCTVC-L0332 Dynamic Range Analysis of HEVC/H.265 Inverse Transform Operations [Louis Kerofsky (Sharp), Shevach Riabtsev (CSR)]

## Profile, level, and constraint definitions (for version 1 of HEVC)

JCTVC-L0244 Main 10 Still Picture Profile [C. Auyeung, A. Tabatabai, T. Suzuki (Sony)]

JCTVC-L0277 Suggested Modifications for Still Picture Profile [K. Sato (Sony)]

JCTVC-L0292 AHG9: On LCU bit size limit [W. Zhang, L. Xu, Y. Chiu, L. Liu (Intel)]

## Source video test material

JCTVC-L0301 AHG8: New 4:4:4 screen-content sequences for HEVC extension development [H. Yu (Huawei), R. Cohen (MERL), W. Gao (Huawei), Y. Cao, J. Ye, X. Wang, A. Vetro (MERL), H. Sun]

JCTVC-L0317 AHG8: YUV444 and RGB screen content test sequences [Shuhui Wang, Mengcao Jiao, Tao Lin, Kailun Zhou (Tongji U)]

# Core experiments (in Range Extensions)

## CE1: Square and non-square transforms for 4:2:2 chroma format coding

### CE1 summary and general discussion

JCTVC-L0020 CE1: Summary report of HEVC Range Extensions Core Experiment on square and non-square transforms for 4:2:2 chroma format [J. Sole]

### CE1 primary contributions

JCTVC-L0146 CE1: Test 2 - Non 32x32 point transforms in chroma for Range Extensions [C. Rosewarne, V. Kolesnikov, M. Maeda (Canon)]

JCTVC-L0148 CE1: Test 3 - Square transforms for Range Extensions [C. Rosewarne, V. Kolesnikov, M. Maeda (Canon)]

JCTVC-L0182 CE1: Test 1 - Rectangular transform units for 4:2:2 (and AHG7 benchmarks) [K. Sharman, N. Saunders, J. Gamei (Sony)]

JCTVC-L0333 CE1: Test 3 - Square transform blocks for 4:2:2 [J. Sole, R. Joshi, M. Karczewicz (Qualcomm)]

### CE1 cross checks

JCTVC-L0147 Cross-check of CE1: Test 1 - Rectangular transforms (L0182) by Sony [C. Rosewarne, V. Kolesnikov, M. Maeda (Canon)]

JCTVC-L0159 CE1: Cross-verification of CE1 Test 3 square transforms for 4:2:2 chroma format coding [H. Nakamura (JVC Kenwood)]

JCTVC-L0320 Cross-verification results of CE1: Test 2 - Non 32x32 point transforms in chroma for Range Extensions (L0146) [X. Zheng, Y. Lin (Hisilicon)] [miss]

JCTVC-L0331 Crosscheck of CE1 test1: rectangular transforms for 4:2:2 [J. Sole (Qualcomm)] [late]

JCTVC-L0350 CE1: Cross-check of L0333 Test 3 - Square transform blocks for 4:2:2 [A. Gabriellini, M. Mrak (BBC)]

JCTVC-L0406 Cross-check of CE1: Test 3 - Square transform blocks for 4:2:2 (L0333) by Qualcomm [C. Rosewarne, M. Maeda (Canon)] [late]

# Tool experiments (in SHVC)

## TE1: Upsampling filter

JCTVC-L0021 TEA1: Summary Report Upsampling Filter [A. Segall, J. Chen, J. Dong, E. Alshina]

| Test | Proponent | Avg. | BD-rate | x2 | Avg. | BD-rate | x1.5 | Enc.Time | Dec.Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # |  | Y | U | V | Y | U | V |  |  |
| test 4.1 (L0083) | Samsung | -17.1 | -12.4 | -11.9 | -26.9 | -23.4 | -22.3 | 111 | 126 |
| test 4.2_(L0054) 6bit luma + K0378 chroma | InterDigital | -17.1 | -12.4 | -11.9 | -26.9 | -23.5 | -22.4 | 107 | 125 |
| test 4.3 (L0335) | Qualcomm | -17.1 | -12.4 | -11.9 | -26.9 | -23.4 | -22.3 | 111 | 128 |
| test 4.4 (L0083) | Samsung | -17.1 | -12.4 | -11.9 | -26.9 | -23.4 | -22.3 | 111 | 124 |

Performance-wise, no difference

Current “default” (4.1) as used in TE2+ was a design with 6 bit precision for luma and chroma (which is identical with the MC interpolation filter of HEVC in case of 2X, where chroma is using 5 bits)

For 2X scalability luma, no difference between the proposals. For chroma, 4.1-4.3 are identical, and 4.4 uses a different phase calculation for vertical upsampling (but still using one of the MC chroma interpolation filters)

For 1.5X scalability (which requires 1/3 phase position for luma and some more for vertical chroma which are not existing as MC interpolation filters), 4.1, 4.3 and 4.4 use the same luma filter, 4.2 uses a different one.

From the frequency response presented in the cross-check document JCTVC-L0088, the design of 4.1/4.3/4.4 seems to be closer to the MC interpolation filters (avoiding the “bumps” in the passband).

Decision: Adopt solution from 4.1/4.3/4.4 as default luma filter.

For 1.5X chroma where 1/3, 7/12 and 11/12 phase positions are additionally required, 4.3 uses 5 bit rounding, whereas 4.1/4.2/4.4 use 6 bit (and 4.4 uses different phase calc. with less accuracy).

5 bit rounding is more consistent with the MC interpolation filter design.

Decision: Adopt solution from 4.3 as default chroma upsampling filter.

Further study (AHG12) on accuracy of chroma phase shift for possible simplification.

    - TE1 summary and general discussion

### TE1 primary contributions

JCTVC-L0054 TE1: Results of test 4.2 on fixed upsampling filter [J. Dong, Y. Ye (InterDigital)]

JCTVC-L0083 TEA1: Fixed up-sampling filter tests by Samsung (tests 4.1 and 4.4) [E.Alshina, A.Alshin, J.H.Park (Samsung)]

JCTVC-L0335 TE-A1: Results of test 4.3 on fixed up-sampling filter [J. Chen, R. Krishna, M. Karczewicz (Qualcomm)]

### TE1 cross checks

JCTVC-L0088 TEA1: Cross-check of fixed 6 and 7 bit-depth up-sampling filters performance (test 4.2) [E.Alshina, A.Alshin, J.H.Park (Samsung)]

JCTVC-L0089 TEA1: Cross-check of low bit-depth Chroma up-sampling filter (test 4.3) [E.Alshina, A.Alshin, J.H.Park (Samsung)]

JCTVC-L0396 TEA1: Cross check for Test 4.4 on chroma up-sampling filter (JCTVC-L0083) [M. Budagavi (TI)] [late]

## TE2: Inter-layer texture prediction

### TE2 summary and general discussion

JCTVC-L0022 TE2: Inter-layer Texture Prediction Signaling in SHVC [L. Guo, Y. He, D.-K. Kwon, J. Zan, H. Lakshman, J.-W. Kang]

Current SMuC implements both Intra_BL and ref_idx for signaling of base-layer prediction, Intra_BL is “default” of SMuC download, but the TEs define individually what method to use as basis.

Three subtests:

- Intra_BL approach

- ref_idx approach

- skip enhancement layer slice (extension of ref_idx approach)

Main question: Basic signalling Intra_BL vs. ref_idx

If Intra_BL, some additional aspects were investigated:

- DCT vs. DST

- CU based vs. PU based signalling

- Deblocking BS

- Intra_BL skip

- cbf_root

No substantial difference in compression performance from the signalling method as such (variations of 0.3%).

Criteria for judging benefit:

- Advantage of using the same parser for EL

- Ability to have only HL-syntax extension

- Flexibility which in combination with other tools may give better compression performance

- Latency and memory bandwidth

- Re-using of decoder architectures

Decision on TE2 depends on the “whole picture”, i.e. whether finally SHVC should only consist of high-level syntax changes or also sub-CU level solutions.

Keep both methods in software model.

Revisit: Define which of the methods to become default in CTC.

For future experiments, a CTC anchor other than simulcast is needed (as the gap from simulcast is too big to compare different tools against each other).

This does not mean that any new experiment has to use the method from CTC as basis

BoG activity (L. Guo) to suggest from the TE2 results which should be the test model configurations of Intra_BL and ref_idx (one for each).

### TE2 primary contributions

JCTVC-L0035 TE2: Results of test 3.1.8 on PU based Intra BL signaling [J. Lainema, K. Ugur (Nokia)]

JCTVC-L0051 TE2: Results of test 3.2.1 on inter-layer reference picture placement [X. Xiu, Y. He, Y. He, Y. Ye (InterDigital)]

JCTVC-L0055 TE2: Results of test 3.3.1 Enhancement layer skipped slice [Y. He, X. Xiu, Y. He, Y. Ye (InterDigital)]

JCTVC-L0060 TE2 : Results of test 3.2.2 on inter-layer reference picture placement [H. Lee, J. W. Kang, J. Lee, J. S. Choi (ETRI)]

JCTVC-L0069 TE2-3.1.7: Inter-layer intra prediction signaling with residual skip flag [T.-D. Chuang, Y.-W. Huang, S. Lei (MediaTek), C. Kim, J. Park, B. Jeon (LG)]

JCTVC-L0110 TE2 Subtest 1: Inter-layer Texture Prediction Signalling (test 3.1.9) [T. Lee, E. Alshina (Samsung)]

JCTVC-L0165 TE2: Results of test 3.1.3 on CU based IntraBL signaling [J. Zan, X. Wei, H. Yang (Huawei)]

JCTVC-L0166 TE2: Results of test 3.1.4 on CU based IntraBL signalling with skip mode [J. Zan, X. Wei, H. Yang (Huawei)]

JCTVC-L0251 Infer Zero motion vector when referencing co-located base layer picture [K. Misra, J. Zhao, A. Segall (Sharp)]

JCTVC-L0284 TE2: Results of Test 3.1.1 on Inter-layer Texture Prediction Signaling [L. Guo, V. Seregin, J. Chen, M. Karczewicz (Qualcomm)]

JCTVC-L0285 TE A2: Signalling of Inter-layer Skip and Direct [W. Zhang, L. Xu, Y. Han, Z. Deng, X. Cai, Y. Chiu (Intel)]

### TE2 cross checks

JCTVC-L0061 TE2: Cross-check results of test 3.1.1 on CU level IntraBL Mode  [H. Lee, J. W. Kang, J. Lee (ETRI)]

JCTVC-L0062 TE2: Cross-check results of test 3.2.1 on inter-layer reference picture placement [H. Lee, J. W. Kang, J. Lee (ETRI)]

JCTVC-L0090 TEA2: Cross-check of refIdx performance (tests 3.2.1) [E.Alshina, A.Alshin, J.H.Park (Samsung)]

JCTVC-L0130 TE2: Cross-check of test 3.1.9 [K. Ugur (Nokia)]

JCTVC-L0201 TE2: Cross-verification of test 3.2.2 on inter-layer reference picture placement [X. Xiu, Y. He (InterDigital)]

JCTVC-L0214 Cross-check results of TE2 3.1.4 Inter-layer texture prediction mode with skip signaling from Huawei [Yong He (InterDigital)]

JCTVC-L0264 TE2: Cross-check report of test 3.2.1 on inter-layer reference picture placemen (JCTVC-L0051) [L. Guo (Qualcomm)] [late]

JCTVC-L0269 TE2: Cross check for test 3.1.5 on signalling of inter-layer skip and direct (JCTVC-L0285) [T. Yamamoto (Sharp)]

JCTVC-L0281 TE2: Cross-check results of test 3.1.8 on PU based Intra BL signaling [C. Kim, B. Jeon (LG)]

JCTVC-L0295 TE2: Cross-check results of test 3.1.1 on CU based IntraBL signaling [Zhuoyi Lyu, Jinwen Zan (Huawei)]

JCTVC-L0296 TE2: Cross-check results of test 3.3.1 on skipped slice in the enhancement layer [Dong Jiang, Jinwen Zan (Huawei)]

JCTVC-L0306 TE A2: Crosscheck for 3.2.2 on inter-layer reference picture placement (JCTVC-L0060) [W. Zhang, L. Xu, Y. Han, Z. Deng, X. Cai, Y. Chiu (Intel)]

JCTVC-L0340 TE2-3.1.9: Crosscheck of inter-layer intra prediction signaling in JCTVC-L0110 proposed by Samsung [Y.-W. Huang (MediaTek)] [miss]

Duplicate with L0341?

JCTVC-L0341 TE2-3.1.9: Crosscheck of inter-layer intra prediction signaling in JCTVC-L0110 proposed by Samsung [T.-D. Chuang, Y.-W. Huang (MediaTek)] [late]

JCTVC-L0353 TE2: Crosscheck for test 3.2.3 [D.-K. Kwon (TI)] [late]

JCTVC-L0382 TE 2: Cross-check results of JCTVC-L0251 on TE2 test 3.2.3 [E. Francois (Canon)] [late]

JCTVC-L0387 TE2: Cross-check of test 3.1.7 on inter-layer intra prediction signaling with residual skip flag (JCTVC-L0069) [K. Kawamura, S. Naito (KDDI)] [late]

JCTVC-L0388 TE2: Cross-check of test 3.1.3 on CU based IntraBL signaling (JCTVC-L0165) [K. Kawamura, S. Naito (KDDI)] [late]

## TE3: Combined prediction in SHVC

### TE3 summary and general discussion

JCTVC-L0023 TE3: Summary Report of Tool Experiment on Combined Prediction in SHVC [X. Li, E. François, P. Lai, D.-K. Kwon, A. Saxena]

Reference for comparison: SMuC with Intra_BL. The Intra_BL implementation had a bug that in case of 4x4 intra forward DCT was used with inverse DST. Removing this bug would improve the anchor by 0.1–0.2%.

| Test | Subtests | Proposals and short descriptions | Crosschecking documents |
| --- | --- | --- | --- |
| 4.1 Intra prediction based on reconstructed base layer | 4.1.2 | JCTVC-L0036 (Nokia) dc_delta is added to the Intra Prediction block when size >= 8x8. dc_delta = (DC of BL Pred block) - (DC of EL Pred block). | JCTVC-L0235 (Qualcomm) |
|  | 4.1.3 | JCTVC-L0099 (LG) Unavailable ref samples for Intra Prediction at EL are replaced by collocated BL samples. | JCTVC-L0270 (Sharp) |
| 4.2 Intra prediction based on differential picture | 4.2.1 | JCTVC-L0037 (Nokia) Intra horizontal and vertical prediction modes at EL are replaced by new modes which combine EL and BL layer as Pred_el = Ref_el + Rec_bl - Ref_bl. No additional Signaling. | JCTVC-L0342 (MediaTek) |
|  | 4.2.2 | JCTVC-L0222 (Qualcomm) CU level Diff domain Intra Prediction. MDIS is disabled in Diff Mode. Planar Mode is modified to set all the pixels in the bottom-right portion ((x + y) >= N-1) to zero in diff mode. | JCTVC-L0079 (Samsung) JCTVC-L0253 (Gent) |
|  | 4.2.3 | JCTVC-L0135 (LG) CU Level Diff Domain Intra Prediction. Diff signal is clipped to [-128,127] | JCTVC-L0237 (Qualcomm) |
|  | 4.2.4 | JCTVC-L0183 (Vidyo/Samsung) CU level Diff Intra Prediction. Offset 128 is added to diff signal to keep the dynamic range of [0, 255] | JCTVC-L0217 (MediaTek) |
| 4.3 Inter prediction based on reconstructed base layer | 4.3.1 | JCTVC-L0072 (MediaTek) Rec_el=Clip((Pred_el+Rec_bl)/2+Residue_el). CU level signaling. | JCTVC-L0354 (TI) |
| 4.4 Inter prediction based on differential picture | 4.4.1 | JCTVC-L0136 (LG) CU level diff-inter prediction. Diff signal is clipped to [-128, 127] | JCTVC-L0081 (Qualcomm) |
|  | 4.4.2 | JCTVC-L0184 (Vidyo/Samsung) CU level diff-inter prediction. Offset 128 was added to diff signal to keep the dynamic range of [0, 255]. Bi-linear interpolation for sub-pel diff blocks. Deblocking BS=1 for the diff/non-diff boundary. | JCTVC-L0218 (MediaTek) |
| 4.5 SVC style residual prediction | 4.5.1 | JCTVC-L0286 (Intel) SVC style residual prediction. The signaling is at CU level for intra while PU level for inter. Base residues are bi-linearly up-sampled. | JCTVC-L0343 (MediaTek) |
| 4.6 Generalized residual prediction | 4.6.1 | JCTVC-L0100 (Canon) CU level signaling. Pred_el=Ref_el+Rec_bl-Ref_bl. This method does not apply to skip mode. | JCTVC-L0095 (Samsung) |
|  | 4.6.2.1 | JCTVC-L0078 (Qualcomm) CU level signaling. Pred_el=Ref_el + w*(Rec_bl-Ref_bl), w= 0.5, 1. Interpolation directly on differential block for sub-pel positions. Fast GRP mode selection method (encoder only). Additional options: 4-tap up-sampling filter. Additional motion estimation in differential pictures (encoder only). Test 1: GRP (0.5) Test 2: GRP (0.5, 1) + Fast GRP mode selection Test 3: GRP (0.5, 1) + Fast GRP mode selection + 4-tap up-sampling filter for GRP mode Test 4 (recently released): Test 1 + additional motion estimation Test 5 (recently released): Test 2 + additional motion estimation | JCTVC-L0141 (LG) JCTVC-L0063 (ETRI) |
|  | 4.6.2.2 | JCTVC-L0206 (Qualcomm) Combined prediction of base and enhancement layer. Rec_el=Clip(Pred_el+Rec_bl+1)/2+Residue_el) | JCTVC-L0064 (ETRI) |
|  | 4.6.3 | JCTVC-L0038 (Nokia) PU level signaling. Pred_el=Ref_el + w*(Rec_bl-Ref_bl), w=0.5, 1 | JCTVC-L0082 (Qualcomm) JCTVC-L0080 (Samsung) |

Results from CE report:

| Subtests | Proposal | Test case | Results (BD-rate reduction of EL+BL) |  |  |  |  | Average of Coding Gain and Geometry Mean of Coding Time |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.1 Intra prediction based on reconstructed base layer | 4.1.2 | AI-2x | -0.2% | -0.1% | 0.0% | 105% | 99% | -0.2% | 0.0% | 0.0% | 105% | 100% |
|  |  | AI-1.5x | -0.1% | 0.0% | 0.0% | 105% | 101% |  |  |  |  |  |
|  | 4.1.3 (Inter results are not mandatory) | AI-2x | -0.2% | -0.1% | -0.1% | 102% | 101% | -0.1% | -0.1% | -0.1% | 102% | 101% |
|  |  | AI-1.5x | -0.1% | 0.0% | -0.1% | 102% | 101% |  |  |  |  |  |
|  |  | RA-2x | -0.1% | -0.1% | -0.1% | 100% | 100% | 0.0% | -0.1% | -0.1% | 100% | 100% |
|  |  | RA-1.5x | 0.0% | -0.1% | -0.1% | 100% | 100% |  |  |  |  |  |
|  |  | RA-SNR | 0.0% | 0.0% | 0.0% | 102% | 102% |  |  |  |  |  |
|  |  | LDP-2x | 0.0% | -0.1% | -0.1% | 100% | 99% |  |  |  |  |  |
|  |  | LDP-1.5x | 0.0% | -0.1% | -0.1% | 101% | 100% |  |  |  |  |  |
|  |  | LDP-SNR | 0.0% | -0.1% | -0.1% | 99% | 99% |  |  |  |  |  |
| 4.2 Intra prediction based on differential picture | 4.2.1 | AI-2x | -0.3% | -0.1% | -0.2% | 101% | 99% | -0.3% | -0.1% | -0.1% | 100% | 99% |
|  |  | AI-1.5x | -0.2% | 0.0% | -0.1% | 99% | 100% |  |  |  |  |  |
|  | 4.2.1 + 4.1.2 | AI-2x | -0.4% | -0.2% | -0.2% | 105% | 99% | -0.4% | -0.1% | -0.2% | 104% | 100% |
|  |  | AI-1.5x | -0.3% | 0.0% | -0.1% | 104% | 101% |  |  |  |  |  |
|  | 4.2.2 | AI-2x | -1.1% | -0.3% | -0.5% | 160% | 109% | -0.9% | 0.0% | -0.2% | 156% | 108% |
|  |  | AI-1.5x | -0.8% | 0.2% | 0.0% | 151% | 107% |  |  |  |  |  |
|  | 4.2.3 | AI-2x | -0.9% | -0.4% | -0.6% | 163% | 110% | -0.8% | -0.1% | -0.3% | 159% | 109% |
|  |  | AI-1.5x | -0.6% | 0.1% | 0.0% | 154% | 109% |  |  |  |  |  |
|  | 4.2.4 | AI-2x | -0.9% | -0.5% | -0.7% | 177% | 141% | -0.8% | -0.2% | -0.4% | 171% | 142% |
|  |  | AI-1.5x | -0.6% | 0.0% | -0.2% | 166% | 143% |  |  |  |  |  |
| 4.3 Inter prediction based on reconstructed base layer | 4.3.1 | RA-2x | -0.4% | -1.7% | -1.9% | 105% | 101% | -1.2% | -3.3% | -3.8% | 104% | 100% |
|  |  | RA-1.5x | -0.6% | -2.3% | -2.7% | 103% | 100% |  |  |  |  |  |
|  |  | RA-SNR | -0.8% | -3.3% | -3.9% | 103% | 100% |  |  |  |  |  |
|  |  | LDP-2x | -1.2% | -2.6% | -3.0% | 105% | 100% |  |  |  |  |  |
|  |  | LDP-1.5x | -1.5% | -4.1% | -4.7% | 103% | 99% |  |  |  |  |  |
|  |  | LDP-SNR | -2.4% | -6.0% | -7.0% | 104% | 100% |  |  |  |  |  |
| 4.4 Inter prediction based on differential picture | 4.4.1 | RA-2x | -1.8% | -3.3% | -3.3% | 165% | 104% | -2.0% | -3.5% | -3.7% | 153% | 103% |
|  |  | RA-1.5x | -2.4% | -4.3% | -4.5% | 154% | 101% |  |  |  |  |  |
|  |  | RA-SNR | -1.5% | -3.5% | -3.7% | 145% | 105% |  |  |  |  |  |
|  |  | LDP-2x | -2.0% | -2.9% | -3.0% | 161% | 106% |  |  |  |  |  |
|  |  | LDP-1.5x | -2.8% | -4.2% | -4.3% | 149% | 101% |  |  |  |  |  |
|  |  | LDP-SNR | -1.5% | -3.0% | -3.1% | 143% | 105% |  |  |  |  |  |
|  | 4.4.2 | RA-2x | -1.7% | -2.6% | -2.6% | 182% | 137% | -2.4% | -3.7% | -3.7% | 168% | 131% |
|  |  | RA-1.5x | -2.1% | -3.8% | -3.9% | 170% | 138% |  |  |  |  |  |
|  |  | RA-SNR | -1.7% | -3.7% | -4.0% | 157% | 115% |  |  |  |  |  |
|  |  | LDP-2x | -2.7% | -2.9% | -2.5% | 179% | 139% |  |  |  |  |  |
|  |  | LDP-1.5x | -3.3% | -4.5% | -4.4% | 165% | 140% |  |  |  |  |  |
|  |  | LDP-SNR | -2.7% | -4.9% | -5.2% | 155% | 118% |  |  |  |  |  |
| 4.5 SVC style residual prediction | 4.5.1 | AI-2x | -0.1% | -0.1% | -0.1% | 172% | 109% | -0.1% | 0.0% | 0.0% | 167% | 109% |
|  |  | AI-1.5x | 0.0% | 0.0% | 0.0% | 163% | 109% |  |  |  |  |  |
|  |  | RA-2x | -0.3% | -0.8% | -0.9% | 103% | 112% | -0.4% | -0.9% | -1.0% | 103% | 110% |
|  |  | RA-1.5x | -0.5% | -1.1% | -1.3% | 103% | 114% |  |  |  |  |  |
|  |  | RA-SNR | -0.3% | -0.8% | -0.9% | 103% | 100% |  |  |  |  |  |
|  |  | LDP-2x | -0.1% | -0.6% | -0.7% | 103% | 115% |  |  |  |  |  |
|  |  | LDP-1.5x | -0.6% | -1.3% | -1.4% | 103% | 117% |  |  |  |  |  |
|  |  | LDP-SNR | -0.3% | -0.7% | -0.7% | 103% | 102% |  |  |  |  |  |
| 4.6 Generalized residual prediction | 4.6.1 | RA-2x | -1.5% | -2.5% | -2.5% | 155% | 85% | -1.6% | -2.8% | -2.9% | 143% | 83% |
|  |  | RA-1.5x | -1.8% | -3.2% | -3.3% | 143% | 80% |  |  |  |  |  |
|  |  | RA-SNR | -1.1% | -2.7% | -2.9% | 132% | 80% |  |  |  |  |  |
|  |  | LDP-2x | -1.8% | -2.5% | -2.5% | 154% | 81% |  |  |  |  |  |
|  |  | LDP-1.5x | -2.3% | -3.5% | -3.5% | 142% | 87% |  |  |  |  |  |
|  |  | LDP-SNR | -1.3% | -2.4% | -2.6% | 133% | 87% |  |  |  |  |  |
|  | 4.6.2.1 Test1 | RA-2x | -1.5% | -2.9% | -3.0% | 121% | 108% | -2.4% | -3.0% | -2.8% | 118% | 107% |
|  |  | RA-1.5x | -2.3% | -3.7% | -4.0% | 116% | 107% |  |  |  |  |  |
|  |  | RA-SNR | -1.7% | -3.6% | -3.9% | 114% | 110% |  |  |  |  |  |
|  |  | LDP-2x | -2.4% | -2.2% | -1.7% | 122% | 105% |  |  |  |  |  |
|  |  | LDP-1.5x | -3.4% | -2.6% | -1.5% | 117% | 105% |  |  |  |  |  |
|  |  | LDP-SNR | -3.2% | -3.1% | -2.6% | 116% | 106% |  |  |  |  |  |
|  | 4.6.2.1 Test2 | RA-2x | -1.9% | -4.0% | -4.2% | 119% | 105% | -2.8% | -4.3% | -4.2% | 116% | 104% |
|  |  | RA-1.5x | -2.8% | -5.2% | -5.6% | 111% | 103% |  |  |  |  |  |
|  |  | RA-SNR | -2.1% | -4.7% | -5.2% | 114% | 107% |  |  |  |  |  |
|  |  | LDP-2x | -2.7% | -3.4% | -3.0% | 124% | 102% |  |  |  |  |  |
|  |  | LDP-1.5x | -3.9% | -4.3% | -3.5% | 113% | 101% |  |  |  |  |  |
|  |  | LDP-SNR | -3.4% | -4.2% | -3.8% | 118% | 105% |  |  |  |  |  |
|  | 4.6.2.1 Test3 | RA-2x | -2.3% | -3.4% | -3.6% | 119% | 125% | -3.5% | -3.7% | -3.8% | 116% | 129% |
|  |  | RA-1.5x | -3.0% | -4.1% | -4.6% | 111% | 125% |  |  |  |  |  |
|  |  | RA-SNR | -2.8% | -4.3% | -4.8% | 113% | 141% |  |  |  |  |  |
|  |  | LDP-2x | -3.4% | -3.1% | -2.7% | 123% | 124% |  |  |  |  |  |
|  |  | LDP-1.5x | -4.5% | -3.6% | -3.0% | 113% | 123% |  |  |  |  |  |
|  |  | LDP-SNR | -4.8% | -4.0% | -3.8% | 117% | 141% |  |  |  |  |  |
|  | 4.6.2.1 Test4 (recently released) | RA-2x | -2.0% | -3.6% | -3.6% | 171% | 110% | -3.1% | -3.7% | -3.4% | 159% | 109% |
|  |  | RA-1.5x | -2.7% | -4.1% | -4.4% | 160% | 109% |  |  |  |  |  |
|  |  | RA-SNR | -2.4% | -4.5% | -4.8% | 149% | 113% |  |  |  |  |  |
|  |  | LDP-2x | -3.0% | -2.9% | -2.3% | 168% | 107% |  |  |  |  |  |
|  |  | LDP-1.5x | -4.1% | -3.0% | -2.0% | 158% | 106% |  |  |  |  |  |
|  |  | LDP-SNR | -4.2% | -4.0% | -3.5% | 148% | 109% |  |  |  |  |  |
|  | 4.6.2.1 Test5 (recently released) | RA-2x | -2.4% | -4.5% | -4.5% | 156% | 105% | -3.4% | -4.8% | -4.7% | 148% | 105% |
|  |  | RA-1.5x | -3.2% | -5.5% | -5.8% | 140% | 105% |  |  |  |  |  |
|  |  | RA-SNR | -2.6% | -5.4% | -5.8% | 139% | 108% |  |  |  |  |  |
|  |  | LDP-2x | -3.4% | -4.0% | -3.5% | 162% | 103% |  |  |  |  |  |
|  |  | LDP-1.5x | -4.7% | -4.8% | -4.0% | 143% | 101% |  |  |  |  |  |
|  |  | LDP-SNR | -4.3% | -4.9% | -4.5% | 146% | 106% |  |  |  |  |  |
|  | 4.6.2.2 | RA-2x | -0.4% | -1.9% | -2.1% | 122% | 103% | -1.2% | -3.6% | -4.2% | 116% | 100% |
|  |  | RA-1.5x | -0.6% | -2.5% | -3.0% | 114% | 99% |  |  |  |  |  |
|  |  | RA-SNR | -0.8% | -3.5% | -4.1% | 113% | 102% |  |  |  |  |  |
|  |  | LDP-2x | -1.2% | -2.9% | -3.3% | 118% | 97% |  |  |  |  |  |
|  |  | LDP-1.5x | -1.5% | -4.6% | -5.3% | 113% | 98% |  |  |  |  |  |
|  |  | LDP-SNR | -2.5% | -6.4% | -7.5% | 115% | 101% |  |  |  |  |  |
|  | 4.6.3 | RA-2x | -1.7% | -3.6% | -3.8% | 122% | 103% | -2.4% | -3.8% | -4.0% | 115% | 104% |
|  |  | RA-1.5x | -2.5% | -4.5% | -4.9% | 113% | 110% |  |  |  |  |  |
|  |  | RA-SNR | -1.6% | -4.2% | -4.7% | 114% | 104% |  |  |  |  |  |
|  |  | LDP-2x | -2.3% | -2.9% | -2.8% | 117% | 103% |  |  |  |  |  |
|  |  | LDP-1.5x | -3.4% | -3.8% | -3.6% | 109% | 100% |  |  |  |  |  |
|  |  | LDP-SNR | -2.9% | -3.8% | -4.2% | 114% | 103% |  |  |  |  |  |

Remark: Test identifier numbers “4.x” should be “3.x”, as this is TE3. However, the numbering shown here was used extensively in the contributions and discussions, and was retained for this reason.

Further discussion:

Intra:

- 4.1.2 additional addition of constant DC value to each sample, additional memory access for computing the average from base layer – increase of complexity, but only small benefit.

- 4.1.3 gives only marginal gain of 0.1%

- 4.2.1 additional computation and additional memory accesses, 0.3% BR decrease

- other 4.2 category: additional computation and additional memory accesses, up to 0.9% BR decrease, non-negligible increase of encoder and decoder runtime

Reported gains may also change depending on removal of Intra_BL bug, and the decision to be made from TE2 about “best” Intra_BL configuration.

Intra: Further study, continuation of TE

Inter:

- 4.3.1/4.6.2.2 use an additional mode which uses average of EL pred. and BL pred. (i.e. could use 3 prediction references at maximum), 1.2% BR decrease, some of which may be due to enc. opt.

- 4.4.1/4.4.2 either require storage of EL-BL residual or computation on the fly, and MC is then applied on this residual (instead of the reference picture itself) – significant increase of encoder and decoder runtime, BR reduction roughly 2%

- 4.5 (simple “AVC-SVC style” upsampling of base layer residual) gives only marginal gain (one of the reasons might be that due to the better motion comp of HEVC the BL residual is close to zero, at least for the range of current test conditions)

- 4.6 “generalized residual prediction” requires additional motion comp.

Interesting gains from the different proposals, but all come with (significantly) increased memory accesses and computation.

- Gains of 4.4 and 4.6 proposals likely not additive

- Unknown whether gain of 4.3/4.6 is additive (it is said that JCTVC-L0074 report that they are)

- More thorough study on additional computations and memory accesses required, it is likely necessary to seek for further simplifications

Inter: Further study, continuation of TE

### TE3 primary contributions

JCTVC-L0036 TE3: Results of test 4.1.2 on intra DC correction [J. Lainema, K. Ugur (Nokia)]

JCTVC-L0037 TE3: Results of test 4.2.1 on gradient based intra prediction [J. Lainema, K. Ugur (Nokia)]

JCTVC-L0038 TE3: Results of test 4.6.3 on base enhanced motion compensated prediction [J. Lainema, K. Ugur (Nokia)]

JCTVC-L0072 TE3-4.3.1: Adaptive predictor compensation by using reconstructed BL texture [T.-D. Chuang, Y.-W. Huang, P. Lai, S. Liu, S. Lei (MediaTek)]

JCTVC-L0078 TE3: Results of Test 4.6.2.1 on Generalized Residual Prediction [X. Li, J. Chen, K. Rapaka, M. Karczewicz (Qualcomm)]

JCTVC-L0099 TE 3: Results of test 4.1.3 on filling unavailable reference samples in intra prediction at enhancement layer [C. Kim, B. Jeon (LG)]

JCTVC-L0100 TE3: Results of test 4.6.1 on the Generalized Residual Inter-Layer Prediction (GRILP) [C. Gisquet, F. Le Léannec, J. Taquet, E. François, G. Laroche, P. Onno (Canon)]

JCTVC-L0135 TE3-4.2.3: Intra prediction based on difference picture [J. Park, B. Jeon (LG)]

JCTVC-L0136 TE3-4.4.1: Inter prediction based on difference picture [J. Park, B. Jeon (LG)]

JCTVC-L0183 TE3: Results of test 4.2.4 on intra prediction based on difference coding [W. Jang, J. Boyce, A. Abbas (Vidyo), E. Alsina, C. Kim(Samsung)]

JCTVC-L0184 TE3: Results of test 4.2.4 on intra prediction based on difference coding [W. Jang, J. Boyce, A. Abbas (Vidyo), E. Alshina, C. Kim (Samsung)]

JCTVC-L0206 TEB3: Combined inter mode (test 4.6.2.2) [V. Seregin, J. Chen, M. Karczewicz (Qualcomm)]

JCTVC-L0222 TE3 : Results of Test 4.2.2 on Intra prediction based on differential picture [K. Rapaka, J. Chen, X. Li, M. Karczewicz (Qualcomm)]

JCTVC-L0286 TE B3: Inter-layer residual refinement [W. Zhang, L. Xu, Y. Han, Z. Deng, X. Cai, Y. Chiu (Intel)]

### TE3 cross checks

JCTVC-L0063 TE3: Cross-check results of test 4.6.2.1 on generalized residual prediction [H. Lee, J. W. Kang, J. Lee (ETRI)]

JCTVC-L0064 TE3: Cross-check results of test 4.6.2.2 on combined mode [H. Lee, J. W. Kang, J. Lee (ETRI)]

JCTVC-L0079 TE 3: Cross-Check of test 4.2.2 in TE 3 [A. Saxena, E. Alshina, F. Fernandes (Samsung)]

JCTVC-L0080 TE 3: Cross-Check of test 4.6.3 in TE 3. [A. Saxena, E. Alshina, F. Fernandes (Samsung)]

JCTVC-L0081 TE3: Cross-check results of test 4.4.1 on Difference Domain Intra Prediction by LGE/MTK [X. Li (Qualcomm)] [late]

This document contains a proposed modification of the cross-checked technology, reporting additional gain for LDP.

JCTVC-L0082 TE3: Cross-check results of test 4.6.3 on Generalized Residual Prediction by Nokia [X. Li (Qualcomm)] [late]

JCTVC-L0095 TEB3: Cross-check of generalized residual prediction from Canon (test 4.6.1) [E.Alshina, A.Alshin, J.H.Park (Samsung)]

JCTVC-L0141 TE3 : crosscheck of TE3-4.6.2.1 [J. Park, B. Jeon (LG)] [late]

JCTVC-L0217 TE3: Cross-check results of test 4.2.4 on intra differential coding [Ximin Zhang, Shan Liu (MediaTek)] [late]

JCTVC-L0218 TE3: Cross-check results of test 4.4.2 on inter differential coding	[Ximin Zhang, Shan Liu (MediaTek)] [late]

JCTVC-L0235 TE3: Cross-check results of test 4.1.2 on Intra prediction based on reconstructed base layer [K. Rapaka (Qualcomm)] [late]

JCTVC-L0237 TE3: Cross-check results of test 4.2.3 on Intra prediction based on differential picture [K. Rapaka (Qualcomm)] [late]

JCTVC-L0253 TE 3: Cross-Check of 4.2.2 Intra prediction based on differential picture [S. Van Leuven, G. Van Wallendael, J. De Cock, R. Van de Walle (Ghent University - iMinds)] [late]

JCTVC-L0270 TE3: Cross check for test 4.1.3 on filling unavailable reference samples (JCTVC-L0099) [T. Yamamoto (Sharp)]

JCTVC-L0342 TE3-4.2.1: Crosscheck of intra prediction based on differential picture in JCTVC-L0037 proposed by Nokia [T.-D. Chuang, Y.-W. Huang (MediaTek)] [late]

JCTVC-L0343 TE3-4.5.1: Crosscheck of inter-layer residual prediction in JCTVC-L0286 proposed by Intel [T.-D. Chuang, Y.-W. Huang (MediaTek)] [late]

JCTVC-L0354	TE3: Crosscheck for test 4.3.1 on adaptive predictor compensation by using reconstructed BL texture [D.-K. Kwon (TI)] [late]

## TE4: Inter-layer filtering in SHVC

### TE4 summary and general discussion

JCTVC-L0024 TE-B4: Summary report of inter-layer filtering for SHVC [J. Chen, A. Segall, E. Alshina, S. Liu, J. Dong, J. Park]

| Test | Proposal documents | Proposal Title | Cross-checking documents |
| --- | --- | --- | --- |
| TE4.1 | ## TE4.1 Switchable up-sampling filters |  |  |
| 4.1.1 | JCTVC-L0056 | TE4: Results of test 4.1.1 on selectable up-sampling filter (SUF) | JCTVC-L0339 |
| 4.1.2 | JCTVC-L0198 | TE4: Result of Test of Intra Mode Dependent Directional Filter | JCTVC-L0091 |
| TE4.2 | Adaptive up-sampling filter |  |  |
| 4.2.1 | JCTVC-L0309 | TE-B4: Results of test 4.2.1 on adaptive up-sampling filter | JCTVC-L0092 |
| 4.2.2 | withdraw |  |  |
| TE4.3 | Interlayer SAO |  |  |
| 4.3.1 | JCTVC-L0212 | TE4.3.1: Interlayer SAO | JCTVC-L0307 |
| 4.3.2 | JCTVC-L0085 | TEB4: Inter-layer SAO (test 4.3.2) | JCTVC-L0338 |
| TE4.4 | Interlayer adaptive filter |  |  |
| 4.4.1 | JCTVC-L0268 | TE4: Inter-layer adaptive filter | JCTVC-L0058 |
| 4.4.2 | JCTVC-L0075 | TE4.4: Inter-layer adaptive filter on up-sampled BL | JCTVC-L0218 |
| 4.4.3 | JCTVC-L0287 | TE B4: Inter-layer adaptive refine filtering | JCTVC-L0216 |
| TE4.5 | Interlayer de-blocking |  |  |
| 4.5.1 | JCTVC-L0133 | TE 4: Results of test on 4.5.1 Inter layer filtering of boundary between EL and BL | JCTVC-L0343 |

| Test | Avg. BD-rate spatial (2X, 1.5X) |  |  |  |  | Avg. BD-rate SNR |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc. Time | Dec. Time | Y | U | V | Enc. Time | Dec. Time |
| 4.1.1 | -0.3% | 0.0% | 0.0% | 103% | 111% | 0.0% | 0.0% | 0.0% | 102% | 106% |
|  | -0.3% | 0.0% | 0.0% | 102% | 116% | 0.0% | 0.0% | 0.0% | 101% | 104% |
| 4.1.2 | -0.1% | -0.1% | 0.0% | 101% | 102% | 0.0% | 0.0% | 0.0% | 101% | 101% |
| 4.2.1 | -0.5% | 0.1% | 0.1% | 102% | 100% | -2.5% | 0.1% | 0.3% | 102% | 126% |
| 4.3.1 | -0.2% | -0.4% | -0.4% | 100% | 103% | -0.4% | -0.8% | -0.8% | 100% | 105% |
| 4.3.2 | -0.3% | -0.1% | -0.1% | 100% | 109% | -0.4% | 0.0% | 0.1% | 100% | 111% |
| 4.4.1 | -0.4% | -0.7% | -0.5% | 106% | 188% | -2.9% | -1.4% | -0.9% | 104% | 212% |
| 4.4.2 | -0.3% | -0.5% | -0.2% | 100% | 102% | -2.7% | -1.6% | -1.1% | 100% | 104% |
| 4.4.3 | -0.2% | -0.5% | -0.4% | 108% | 98% | -2.3% | -1.3% | -1.0% | 104% | 102% |
| 4.5.1 | -0.1% | 0.0% | 0.0% | 101% | 101% | -0.1% | 0.0% | 0.0% | 101% | 102% |

4.1.1 uses non-separable filters (8 bit precision)

4.1.2 requires storage of intra coding modes (as the filter choice is derived from the mode), total of 136 sets of filter coefficients

For the 4.1 category, the gain does not justify the additional complexity that is introduced

4.2. uses alternative separable filters, but also filters the integer position (which means that more processing is necessary than with current fixed filter), and currently no filtering is applied in case of SNR scalablity. Filters for different phases signalled at picture level (APS?); note: Could also be signalled in slice header.

4.3 Picture level SAO (not LCU level), but remaining part is identical to current SAO, signalling in slice header

4.4. “ALF style” filter as second step after the upsampling filter. Signalling at picture level (currently signalled in first slice header)

4.5. should be called “boundary smoothing” rather than deblocking

In the current configuration, all proposals in TE4 introduce latency

For integer position filtering in SNR scalability, 4.2.1 and 4.4.2 have similar complexity (8 vs, 9 multiplications per sample)

In case of SNR scalability, the increase in decoder complexity is non-negligible, and 4.2.1 could also be categorized as “separable ALF”

It is also reported that for SNR scalability, the gain is higher for LD P than for RA configuration, and gain is also higher for class A

At integer pel position in spatial scalability, 4.2.1 is also filtering the integer pel position, which is the source of most of the gain. Without filtering integer positions, the gain may be more in the range of 0.1-0.3% (similar to 4.1 category)

In case of 2X scalability, the upsampling filter complexity is approximately doubled by 4.2.1

It is reported that in the AVC-SVC design, a similar phenomenon of gain by filtering integer positions in upsampling was observed (at least for 2X case), but later by introducing half-pel phase shift in downsamling the same gain was realized.

In terms of hardware complexity, fixed filters would be the first choice (may be implementation dependent); for software it likely does not make a difference.

Several experts expressed the opinion that for the case of spatial scalability the adjustment of downsampling filters is a better solution.

One expert mentions that in principle, also in SNR scalability a lowpass filter could be used before encoding the base layer to achieve a similar effect.

4.1/4.2/4.4 categories: Further study in AHG on down/upsampling filter.

4.3 (SAO) and 4.5 (boundary smoothing): Gain does not justify the additional complexity. One expert mentions that it also may harm the visual quality due interaction with the enhancement layer deblocking filter.

### TE4 primary contributions

JCTVC-L0056 TE4: Results of test 4.1.1 on selectable upsampling filter (SUF) [J. Dong, Y. He, Y. Ye (InterDigital)]

JCTVC-L0075 TE4.4: Inter-layer adaptive filter on upsampled BL [C.-Y. Chen, C.-Y. Tsai, X. Zhang, S. Liu, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-L0085 TEB4: Inter-layer SAO (test 4.3.2) [E.Alshina, A.Alshin, J.H.Park (Samsung)]

JCTVC-L0133 TE 4: Results of test on 4.5.1 Inter layer filtering of boundary between EL and BL [C. Kim, B. Jeon (LG)]

JCTVC-L0212 TE4.3.1: Inter-layer SAO [Ximin Zhang, Shan Liu, Chih-Ming Fu, Shawmin Lei (MediaTek)]

JCTVC-L0268 TE4: Inter-layer adaptive filter [T. Yamamoto, Y. Yasugi (Sharp)]

JCTVC-L0287 TE B4: Inter-layer adaptive refine filtering [W. Zhang, L. Xu, Y. Han, Z. Deng, X. Cai, Y. Chiu (Intel)]

JCTVC-L0309 TE-B4: Results of test 4.2.1 on adaptive up-sampling filter [J. Chen, X. Li, M. Karczewicz (Qualcomm)]

### TE4 cross checks

JCTVC-L0058 TE4: Cross-verification results of test 4.4.1 on inter-layer adaptive filter [J. Dong, Y. Ye (InterDigital)]

JCTVC-L0091 TEB4: Cross-check of IMDDF performance (test 4.1.2) [E.Alshina, A.Alshin, J.H.Park (Samsung)] [late]

JCTVC-L0092 TEB4: Cross-check of adaptive up-sampling filter performance (test 4.2.1) [E.Alshina, A.Alshin, J.H.Park (Samsung)] [late]

JCTVC-L0198	Description of Tool Experiment B4.1.2: Intra Mode Dependent Directional Filter [Y. He, Y. Ye, J. Dong (InterDigital)]

JCTVC-L0216 TE4: Cross-check results of test 4.4.3 on inter-layer refining filter [Ximin Zhang, Shan Liu (MediaTek)] [late]

JCTVC-L0307 TE B4: Crosscheck for 4.3.1 on inter-layer SAO (JCTVC-L0212) [W. Zhang, L. Xu, Y. Han, Z. Deng, X. Cai, Y. Chiu (Intel)] [late]

JCTVC-L0308 TE B4: Crosscheck for 4.4.2 on inter-layer adaptive filter (JCTVC-L0075) [W. Zhang, L. Xu, Y. Han, Z. Deng, X. Cai, Y. Chiu (Intel)] [late]

JCTVC-L0338 TE-B4: Cross-verification of test 4.3.2 on Inter-layer SAO (JCTVC-L0085) [J. Chen (Qualcomm)] [late]

JCTVC-L0339 TE-B4: Cross-verification of test 4.1.1 on selectable upsampling filter (JCTVC-L0056) [J. Chen (Qualcomm)] [late]

JCTVC-L0347 TEB4: Cross-check of inter-layer de-blocking filter performance (test 4.5.1) [E.Alshina, A.Alshin, J.H.Park (Samsung)] [late]

## TE5: Inter-layer syntax prediction using HEVC base layer

### TE5 summary and general discussion

JCTVC-L0025 TE5: Summary report of tool experiment on inter-layer syntax prediction using HEVC base layer [V. Seregin, P. Onno, S. Liu, T. Lee, C. Kim, H. Yang, H. Laksman]

This contribution summarizes the activities and test results performed in TE5 on inter-layer syntax prediction using HEVC base layer. The tools in TE5 were classified into four categories:

- TE5.1 Inter-layer intra mode prediction.

- TE5.2 Inter-layer motion prediction including merge and AMVP modes.

- TE5.3 Inter-layer inferred prediction mode (also known as InterBL or Base Mode). In this mode, co-located base layer block is divided into sub-blocks and motion information associated with each sub-block is used to form a prediction of the corresponding block in the enhancement layer.

- TE5.4 Motion mapping for HLS approach (also known as ”ref_idx” approach), where reconstructed upsampled base layer picture is inserted into DPB buffer and there is no low level block modifications.

TE5.1

| Test | Changes in intra mode coding |
| --- | --- |
| 5.1.1 | Luma intra mode coding: - Planar and DC modes are removed - Use BL intra mode derived from the center as an MPM, if BL intra mode is neither Planar nor DC - Total number of MPMs is one - Change in intra mode coding/parsing Chroma intra mode coding: - Planar and DC modes are removed - Three intra modes (VER, HOR and DM) instead of five (Planar, VER, HOR, DC and DM) are used - Change in intra mode coding/parsing To avoid parsing dependency, MDCS was disabled and additional results were provided. |
| 5.1.2 | Luma intra mode coding: - Replace the first MPM with the BL intra mode, if BL intra mode is neither DC nor equal to any already derived MPMs To avoid parsing dependency, MDCS was disabled and additional results were provided. |
| 5.1.3 | Luma intra mode coding: - The difference between BL intra mode or Planar mode is coded - Change in intra mode coding/parsing To avoid parsing dependency, MDCS was disabled and additional results were provided. |
| 5.1.4 | Luma intra mode coding: - Insert BL intra mode as a first MPM, if BL intra mode is distinct To avoid parsing dependency, MDCS was disabled and additional results were provided. |
| 5.1.5 | BL-MPM (test is withdrawn by the proponent) Luma intra mode coding: - Use BL intra mode as one more MPM inserted as forth MPM - BL MPM is context coded and one more context is added - Change in intra mode coding/parsing ILIPM Luma intra mode coding: - Use BL intra mode as intra mode predictor, HEVC intra mode coding is applied if EL intra mode is not equal to BL intra mode - BL intra mode is used if BL block is intra coded (**) - BL intra mode is used if EL PU size is twice of the co-located BL PU size (*) - Change in intra mode coding/parsing Additional test result was provided without condition (*). One more additional test result was provided in addition with disabled MDCS. The condition (**) doesn’t have an effect on AI test configuration. |

These methods give between 0.1% and 0.25% BR reduction (AI, on average)

The current results are not providing sufficient benefit for taking action – further improvement required.

TE5.2

| Test | Changes in Merge and AMVP modes |
| --- | --- |
| 5.2.1 | Merge modification: - Add center C3 BL MV as a first candidate - Add bottom right BL MV after second spatial candidate - Uncompressed motion field is used to derive BL MV - Pruning with BL MV candidate - Check to guarantee TMVP in the candidate list AMVP modification: - Add center C3 BL MV as first candidate - Uncompressed motion field is used to derive BL MV - Pruning with BL MV |
| 5.2.2 | Merge modification: - Add center C3 BL MV as a first candidate - Uncompressed motion field is used to derive BL MV - Pruning with BL MV candidate - Check to guarantee TMVP in the candidate list AMVP modification: - Add center C3 BL MV as a last candidate - Uncompressed motion field is used to derive BL MV Additional results without changing Merge and AMVP list generation were provided. BL MV is inserted as a first candidate into the complete candidate list. |
| 5.2.3 | Merge modification: - Add center C3 BL MV as a first candidate - Uncompressed motion field is used to derive BL MV - Pruning only between left candidate and BL MV candidate - Check to guarantee TMVP in the candidate list AMVP modification: - Add center C3 BL MV as first candidate - Uncompressed motion field is used to derive BL MV - Pruning with BL MV candidate |
| 5.2.4 | Merge modification: - Maximum number of candidates is 6 - Add bottom right BL MV as a first candidate - Pruning with BL MV candidate - Increased number of combined candidates AMVP modification: - Maximum number of candidates is 3 - Add bottom right BL MV as first candidate - Pruning with BL MV candidate |
| 5.2.5 | Merge modification: - Add center C3 BL MV as a first candidate - Uncompressed motion field is used to derive BL MV (*) - Pruning with BL MV candidate (*) - Check to guarantee TMVP in the candidate list - BL MV scaling with integer arithmetic Additional results without changes (*) were provided. |
| 5.2.6 | Modifications to the EL coding: - No motion compression for EL reference frames Merge modification: - Maximum number of candidates is 6 - TMVP derived only from the center C3 is the first candidate if the temporal collocated blocks are selected in the forward reference frame. - Pruning with TMVP candidate - Bottom left spatial candidate is added if count is less than 4 - Add bottom right or center C3 BL MV after spatial candidates - Four offset candidates are added after BL MV candidate - Candidate cannot be obtained from the block coded with Base Mode (*) AMVP modification: - Candidate cannot be obtained from the block coded with Base Mode (*) Additional results with enabled Base Mode (5.3.3) and changes (*) were provided. |
| 5.2.7 | AMVP modification: - Add center C3 BL MV as a first candidate - Temporal scaling of BL MV |
| 5.2.8 | Merge modification: - Maximum number of candidates is 6 - Add center C3 BL MV as a third candidate - BL MV derivation process - Derived based on 4x4 blocks - Unavailable motion field is copied from the neighboring blocks - MV refinement - Pruning with BL MV candidate AMVP modification: - Maximum number of candidates is 3 - Add center C3 BL MV before TMVP candidate - Similar to Merge mode BL MV derivation - Scale BL MV according to the temporal distance - Pruning with BL MV candidate |
| 5.2.9 | Merge modification: - Maximum number of candidates is 6 - Add bottom right or center C3 BL MV after TMVP candidate - Uncompressed motion field is used to derive BL MV - BL MV candidate derivation - Inter direction depends on EL slice type - Reference list can be switched according to the motion availability - If reference list switching happens check the reference index for validity - Temporal scaling following by spatial scaling for BL MV - BL MV clipping AMVP modification: - Maximum number of candidates is 3 - Add bottom right or center BL MV as a first candidate - Uncompressed motion field is used to derive BL MV - Similar to Merge mode BL MV derivation - Pruning with BL MV candidate |
| 5.2.10 | Merge modification: - Maximum number of candidates is 6 - Add center BL MV after TMVP candidate - Increased number of combined candidates AMVP modification: - Maximum number of candidates is 3 - Add center BL MV after TMVP candidate - Switch the list if the target reference picture is different from the BL reference picture |

Uncompressed MV requires more memory, with compressed this would not be necessary.

Some proposals increase number of candidates to 6/3 in merge/AMVP

One proposal (5.2.6) uses uncompressed MV in EL (for TMVP), but the proponent reports that using MV compression would not change the results

Simplest versions using compressed MV from BL, not extending the candidate list, no modification of parsing, no pruning (simpler versions from 5.2.2 and 5.2.5) give around 1% BR reduction on average

Pruning gives another 0.2%

Using uncompressed MV from baselayer gives around 0.7%

The additional effect with increased number of candidates (and change of parsing) cannot exactly be analysed from the data, but seems not to be large (0.2-0.3%)

Using a second candidate from the base layer provides additional gain around 0.4% (from 5.2.1)

As a first estimate from the results of the TE, using a modification of the MV coding at EL without increeasing complexity and memory and without change of parsing could give around 1.6% BR reduction.

Test 5.3

| Test | Base Mode implementation |
| --- | --- |
| 5.3.1 | - Inferred mode is CU based - One context is added for new mode coding - For each sub-block, prediction is formed either by motion compensation or Intra-BL. - Sub-block size can be 4x4 or 8x8. - For MC basis units, MV if available is derived from the base layer. - Base layer motion compression can be postponed for BL MVs derivation |
| 5.3.2 | - It has parsing dependency on base layer depth - CU based mode - One context is added for new mode coding - Partition structure is derived from the base layer - Prediction is formed either by motion compensation or Intra-BL - Applied only for SNR and dyadic case |
| 5.3.3 | - CU based mode - One context is added for new mode coding - Partition and motion mapping is done do get upsampled information from the base layer - The prediction in this mode is done by adding the residue between EL and BL reference pictures, obtained by MC using derived base layer MVs, to the Intra-BL prediction - Base layer MVs are derived from uncompressed motion field |
| 5.3.4 | - CU based Inter-BL mode - Three contexts are added for new mode coding - Motion information is derived from the base layer based on 8x8 units - Motion information is obtained from the previous block in z-order, if base layer co-located block is intra coded - Zero motion can be used as initialization for the very first 8x8 block - Deblock is applied for Inter-BL on 8x8 basis |
| 5.3.5 | - CU based Inter-BL mode signaled with a new flag. Three contexts are added for new mode coding. - PU based Inter-BL mode signaled with merge index equal to zero - Motion information is derived from the base layer based on 8x8 units - If co-located BL block doesn’t have motion, it is derived from the neighbors or set to zero. Up to three neighbor blocks can be checked. - Base layer MVs are derived from uncompressed motion field - Deblock is applied for Inter-BL on 8x8 basis |

Approach that is common to all proposals:

Signal this mode at CU level, which implies

- Perform sub-partitioning of this CU into 8x8 (in one case also 4x4) “PUs”

- MV(s) and ref_idx(s) for the PUs derived from base layer

- Intra_BL is used without change

Simplest version, 8x8 partitioning using compressed BL MV (5.3.1, 5.3.4) gives around 1.1% BR reduction (this version would be implementable with practically no complexity and memory increase)

Usage of uncompressed BL MV gives similar gain as in 5.2 (around 0.7%)

Usage of 4x4 gives another 0.2% but may be undesirable in terms of memory BW

Using in combination with generalized residual prediction (inferring usage of GRP whenever the mode is selected) gives another 1.5% (from 5.3.3)

Action from 5.2 and 5.3: Test combination of 5.2.5 (simplest version, 5 candidates, no pruning, compressed MV) and 5.3.1. Report back (revisit)

TE5.4

| Test | Motion mapping |
| --- | --- |
| 5.4.1 | - Up to four MVs and reference indices can be derived from the base-layer co-located blocks. - Median or average MV is assigned to the 4x4 block in the ILR picture out of up to four base-layer MVs. - The most frequently used reference index out of up to four indices is assigned to the 4x4 block in the ILR picture. - Base layer motion information can be from compressed or uncompressed motion field - In the current software implementation, the mapped ILR MVs are treated from a short term reference |

Experimental results based one the HLS-only or “reference index framework” anchor is summarized in the next table.

| Test | RA |  |  | LD-P |  |  | Average |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2x | 1.5x | SNR | 2x | 1.5x | SNR |  |
| Compressed base layer motion field | -1.10 | -1.73 | -1.96 | -0.21 | -0.46 | -0.79 | -1.04 |
| Uncompressed base layer motion field | -1.56 | -1.89 | -1.99 | -0.60 | -0.57 | -0.82 | -1.24 |

BL MV is replacing the TMVP (and can therfore appear in the merge or AMVP list of the enhancement layer).

The current implementation is not using 16x16 grid for MV and ref_idx storage in the enhancement layer, therefore it is not purely HL syntax changes (note: JCTVC-L0336 reports results on such a case).

Note: “HLS-only” means that a normative description of the upsampling filter and the process how to derive the compressed MV field and prediction mode associated with the ILR picture is required.

Further study required, several experts expressed the opinion that the “HLS only” approach is interesting.

### TE5 primary contributions

JCTVC-L0031 TE5: Results of test 5.1.3 on inter-layer intra mode prediction [D. Bugdayci, K. Ugur (Nokia)]

JCTVC-L0032 TE5: Results of 5.2.5 on inter-layer motion vector prediction [D. Bugdayci, K. Ugur (Nokia)]

JCTVC-L0052 TE5: Results on test 5.4.1 on motion field mapping [X. Xiu, Y. He, Y. He, Y. Ye(InterDigital)]

JCTVC-L0065 TE5: Results of test 5.2.3 on inter-layer motion vector prediction [J. Lee, H. Lee, J. W. Kang, J. S. Choi (ETRI)]

JCTVC-L0070 TE5-5.3.1: Inter-layer motion vector prediction [T.-D. Chuang, Y.-W. Huang, S. Lei (MediaTek), J. Park, B. Jeon (LG)]

JCTVC-L0101 TE5: Results of test 5.2.6 on the Inter-Layer Motion Vector Prediction [G. Laroche, T. Poirier, P. Onno, C. Gisquet, E. François (Canon)]

JCTVC-L0102 TE5: Results of test 5.3.3 on the Inter-Layer Prediction Mode (Base Mode) [C. Gisquet, T. Poirier, J. Taquet, F. Le Léannec, E. François, G. Laroche, P. Onno (Canon)]

JCTVC-L0113 TE5 Subtest 1: Inter-layer intra mode prediction (test 5.1.4) [T. Lee, E. Alshina (Samsung)]

JCTVC-L0115 TE5 Subtest 3: Inter-layer inferred prediction mode (test 5.3.4) [T. Lee, E. Alshina (Samsung)]

JCTVC-L0153 TE5: Test Results of TE5.1.5 [Z. Zhao, J. Ostermann (Leibniz Uni Hannover)]

JCTVC-L0164 TE5: Results of TE5 tools from Huawei [L. Li, B. Li, H. Li (USTC), H. Yang (Huawei)]

JCTVC-L0185 TE5: Results of test 5.2.10 on inter-layer motion vector prediction [J. Boyce, W. Jang (Vidyo)] [late]

JCTVC-L0205 TEC5: Using base layer MV in Merge and AMVP modes (test 5.2.2)  [V. Seregin, J. Chen, M. Karczewicz (Qualcomm)]

JCTVC-L0224 TE5 : Results of Test 5.1.2 on Inter-Layer Intra Mode Prediction [K. Rapaka, J. Chen, M. Karczewicz (Qualcomm)]

- JCTVC-L0423 Non-TE5.1.2: Cross-check of additional result in JCTVC-L0224 [D. Bugdayci, K. Ugur (Nokia)] [late]

JCTVC-L0259 TE5.1.1: Inter-layer Intra mode prediction [M. Guo, S. Liu, S. Lei (MediaTek), J. Park, J. Kim, B. Jeon (LG)]

JCTVC-L0266 TE5: Results of Test 5.3.1 on Inter-layer Inferred Prediction Mode [L. Guo, J. Chen, M.Karczewicz (Qualcomm)]

JCTVC-L0288 TE C5: Inter-layer motion prediction [W. Zhang, L. Xu, Y. Han, Z. Deng, X. Cai, Y. Chiu (Intel)]

JCTVC-L0289 TE C5: Inter-layer pattern/mode prediction [W. Zhang, L. Xu, Y. Han, Z. Deng, X. Cai, Y. Chiu (Intel)]

JCTVC-L0297 TE5: Results of test 5.2.9 on inter-layer motion prediction [K. Misra, J. Zhao, A. Segall (Sharp)]

JCTVC-L0376 TE5: Inter-layer motion-vector prediction by the base-layer MV up-scaling and refinement using HEVC base layer [K. Kawamura, T. Yoshino, S. Naito (KDDI)] [late]

### TE5 cross checks

JCTVC-L0066 TE5: Cross-check results of test 5.3.1 on inter-layer inferred prediction mode [J. Lee, H. Lee, J. W. Kang (ETRI)]

JCTVC-L0093 TEC5: Cross-check inferred base mode performance (test 5.3.1) [E.Alshina, A.Alshin, J.H.Park (Samsung)] [late]

JCTVC-L0121 TE5: Cross-check of TE5.2.2 inter-layer motion prediction [D. Bugdayci, K. Ugur (Nokia)]

JCTVC-L0125 TE5: Cross-check of TE5.1.1 inter-layer intra prediction [D. Bugdayci, K. Ugur (Nokia)]

JCTVC-L0142 TE5 : crosscheck of TE5.3.1 [J. Park, B. Jeon (LG)] [late]

JCTVC-L0160 TEC5: Cross-verification of TEC5 Test 5.1.5 inter-layer intra mode prediction [H. Nakamura (JVC Kenwood)]

JCTVC-L0168 TE5: Cross-check of 5.2.4 and 5.3.2 tools from Intel [H. Yang (Huawei)]

JCTVC-L0169 TE5: Cross-check of InterDigital’s proposal on Motion Field Mapping [D. Jiang (Huawei)]

JCTVC-L0172 TE5: Cross-check of inter-layer intra prediction mode coding tool from MediaTek [Z. Zhao (Leibniz Uni Hannover)] [late]

JCTVC-L0203 TE5: Cross-verification of test 5.2.3 inter-layer motion vector prediction [X. Xiu, Y. He (InterDigital)]

JCTVC-L0207 TE5: Cross-verification of the test 5.2.5 on base layer MV candidate [V. Seregin (Qualcomm))] [late]

JCTVC-L0238 TE5: Cross-check results of test 5.1.3 on Inter-layer intra mode prediction [K. Rapaka (Qualcomm)] [late]

JCTVC-L0242 TE5: cross check report of Samsung's proposal TE 5.1.4 (JCTVC-L0113) [J. Kim (LG)]

JCTVC-L0274 SHVC: Crosscheck of TE5.2.3 [K. Sato (Sony)]

JCTVC-L0300 TE5: Cross check report for TE5 test 5.2.8 on inter-layer motion vector prediction [K. Misra, A. Segall (Sharp)] [late]

JCTVC-L0310 TE C5: Crosscheck for 5.2.7 on EL AMVP mode (JCTVC-L0164) [W. Zhang, L. Xu, Y. Han, Z. Deng, X. Cai, Y. Chiu (Intel)] [late]

JCTVC-L0311 TE C5: Crosscheck for 5.3.5 on inter-layer motion copy (JCTVC-L0164) [W. Zhang, L. Xu, Y. Han, Z. Deng, X. Cai, Y. Chiu (Intel)] [late]

JCTVC-L0344 TE5-5.2.6: Crosscheck of inter-layer motion vector prediction in JCTVC-L0101 proposed by Canon [T.-D. Chuang, Y.-W. Huang (MediaTek)] [late]

JCTVC-L0359 TE5: Cross check report for test 5.1.2 on inter-layer intra mode prediction [M. Guo, S. Liu (MediaTek)]

JCTVC-L0360 TE5: Cross check report for test 5.1.4 on inter-layer intra mode prediction [M. Guo, S. Liu (MediaTek)] [late]

JCTVC-L0389 TE5: Cross-check of test 5.2.9 on inter-layer motion prediction (JCTVC-L0297) [K. Kawamura, S. Naito (KDDI)] [late]

JCTVC-L0394 TE5: Cross-check of TE5 section 5.2.1 results (JCTVC-L0070 from LG/Mediatek) [P. Onno (Canon)] [late]

JCTVC-L0395 TE5: Cross-check of T5 section 5.3.4 results (JCTVC-L0115 from Samsung) [P. Onno (Canon)] [late]

JCTVC-L0422 TE5: Cross-check of test 5.2.10 on inter-layer motion vector prediction [H. Lakshman, T. Hinz, H. Schwarz (Fraunhofer HHI)]

## TE6: Inter-layer syntax prediction using AVC base layer

### TE6 summary and general discussion

JCTVC-L0026 TE6: Summary report of tool experiment on inter-layer syntax prediction using AVC base layer [J. Boyce, K. Kawamura, H. Lakshman]

This contribution summarizes the activities and test results performed in TE6 on inter-layer syntax prediction using AVC base layer. The tools in TE6 were classified into two categories:

- TE6.1 Inter-layer motion prediction for AMVP

- TE6.2 Inter-layer motion prediction for Merge

The TE6 contributions were tested using the SMuC software, which was modified to support an AVC base layer (which also caused some delay in the TE6)

In the below table, the EL results of the same techniques between the AVC base layer and HEVC base layer are compared.    EL only results are considered a more appropriate comparison in this case vs. EL+BL, because the base layers are different (base layers are said to be designed for approximately same PSNR, which makes the AVC base layer rate higher (less than 2X).

The gains for the proposed techniques for AVC base layers are consistently higher than for HEVC base layers.

|  | L0377, L0376 |  |  |  | L0385, L0185 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Overall (EL) | RA Hybrid 2x | RA Hybrid 1.5x | LD-P Hybrid 2x | LD-P Hybrid 1.5x | RA Hybrid 2x | RA Hybrid 1.5x | LD-P Hybrid 2x | LD-P Hybrid 1.5x |
| AVC AMVP | -1.9% | -2.5% | -1.1% | -1.0% | -3.9% | -2.5% | -0.6% | -0.6% |
| HEVC AMVP | -1.4% | -2.6% | -0.5% | -1.0% | -1.2% | -2.4% | -0.3% | -0.6% |
| Diff | -0.5% | 0.1% | -0.5% | 0.1% | -2.7% | -0.1% | -0.3% | 0.0% |
| Overall (EL) | RA Hybrid 2x | RA Hybrid 1.5x | LD-P Hybrid 2x | LD-P Hybrid1.5x | RA Hybrid 2x | RA Hybrid 1.5x | LD-P Hybrid 2x | LD-P Hybrid1.5x |
| AVC Merge | -4.0% | -4.8% | -2.5% | -2.7% | -3.6% | -4.8% | -2.0% | -2.3% |
| HEVC Merge | -2.6% | -4.5% | -1.3% | -2.3% | -2.3% | -4.4% | -1.1% | -2.0% |
| Diff | -1.4% | -0.4% | -1.2% | -0.4% | -1.3% | -0.4% | -0.9% | -0.4% |

The approach is to extend the merge and AMVP lists from 5->6 and 2->3 (0377 uses third position for BL MV in both merge and AMVP, 0378 uses the last position in both); some additional differences on how the reference picture lists in EL are constructed.

One explanation for the better performance of AVC could be that it uses uncompressed MV fields (4x4), whereas HEVC uses compressed (16x16).

What was studied in the CE is not a “HLS-only” approach, makes modifications in the EL decoder.

Some difference in output cropping of the base layer between AVC and HEVC

Further study – AHG (Jill Boyce)

### TE6 primary contributions

JCTVC-L0377 TE6: Inter-layer motion-vector prediction by the base-layer MV up-scaling and refinement from AVC base layer [K. Kawamura, T. Yoshino, S. Naito (KDDI)] [late]

- JCTVC-L0425 TE6: Cross-check of JCTVC-L0377 test 6.3 on inter-layer motion vector prediction [T. Hinz, H. Lakshman, H. Schwarz (Fraunhofer HHI)] [late]

JCTVC-L0385 TE6 [W. Jang, J. Boyce, A. Abbas (Vidyo), E. Alshina, C. Kim (Samsung)] [late]

### TE6 cross checks

# Non-CE Technical Contributions

## Deblocking filter in version 1 (3)

See also JCTVC-L0363 regarding in-loop filtering across tile and slice boundaries.

JCTVC-L0232 AHG6: On deblocking filter and parameters signaling [A. Norkin	(Ericsson)]

JCTVC-L0404 AhG6: Cross-check for deblocking filter process and parameter modifications suggested in JCTVC-L0232 [E.Alshina (Samsung)] [late]

JCTVC-L0386 AHG6: On HEVC block artifact reduction [G. Van der Auwera, M. Karczewicz (Qualcomm)] [late]

JCTVC-L0397 Cross check of JCTVC-L0386: On HEVC block artifact reduction [S. Lu (Sony)] [late]

## High-level syntax in version 1 (38)

### General high-level syntax cleanups (9)

JCTVC-L0043 AHG9: General HEVC high-level syntax cleanups [Y.-K. Wang, Y. Chen, A. K. Ramasubramonian (Qualcomm)]

Includes several topics:

- Unspecified NAL unit types: For the 16 unspecified NAL unit types, half of them are proposed to be specified as "prefixes" (i.e. may start a new access unit) and the rest are proposed to be specified as "suffixes" (i.e. shall not precede the first VCL NAL unit in the same access unit). Currently they are all specified as "suffixes". Decision: Adopted.

- For the reserved non-RAP VCL NAL unit types 24..31, half of them (24, 26, 28, 30) are proposed to be specified as reserved non-RAP non-reference VCL NAL unit types, and the rest are proposed to be specified as reserved non-RAP reference VCL NAL unit types. Currently they are all (implicitly) specified as reserved non-RAP reference VCL NAL unit types. No action.

- It is proposed to add the signalling of spatial resolution, color format and bit depth into the VPS. Such video format information is even more important in session negotiations than bit rate and picture rate information, which is currently included in the VPS. The same syntax structure, video_format( ), is shared by VPS and SPS, similarly as profile_tier_level( ), and the syntax elements are now all fixed-length coded.

This aspect of the proposal was discussed extensively. A related issue is the definition of the bit rate and picture rate information (adopted from K0125 of the preceding meeting). It was noted that the VPS syntax is extensible. It was remarked that it is desirable to limit the scope of the VPS to avoid putting things in the VPS now that may not be well thought out. It was noted that most of what is currently in the VPS is scalability/subset (and associated HRD) related information. bit_rate_pic_rate_info( ) was added at the preceding meeting and was suggested to potentially not be mature – e.g. with regard to whether an encoder could be expected to populate that information exactly correctly and whether the provided values should be targets, maxima, or exactly correct values. It was remarked that if a picture rate is to be defined, it would be desirable to be able to use a numerator-denominator representation so that such rates as 30000/1001 can be represented exactly. Decision: Remove bit rate / pic rate from VPS of version 1 and consider it and other information such as colour space, bit depth, spatial resolution, etc. for VPS version 1+n or SEI in version 2.

- It is proposed to add a note at end of the semantics of nuh_temporal_id_plus1, to warn encoders to be cautious in inserting parameter sets NAL units with lower TemporalId than the containing access unit when there are access unit delimiter (AUD) NAL units in the bitstream, which may produce a non-conforming sub-bitstream wherein an AUD NAL unit is not the first NAL unit in the the access unit. "NOTE 10 – When access unit delimiter NAL units are present in a bitstream, encoders should be cautious in inserting a video parameter set, sequence parameter set or picture parameter set NAL unit (e.g. for error resiliene purposes) with TemporalId less than the TemporalId of the access unit containing the inserted NAL unit. This is because the bitstream would be non-conforming in case an access unit delimiter NAL unit is not the first NAL unit in the access unit containing the access unit delimiter NAL unit in an extracted sub-bitstream as the output of the bitstream extraction process as specified in subclause 10.1." Revisit.

- It is proposed to signal time scale and the number of units in a clock tick to be outside of the hrd_parameters( ), to solve the issue of the unclear condition for the presence of the num_ticks_poc_diff_one_minus1 syntax element in the VUI syntax, and also due to the reason that typically all layers in a scalable bitstream would share the same top-level timing information. It is also proposed to signal the syntax element num_ticks_poc_diff_one_minus1 in the VPS, as it is asserted that usually the information applies to all layers of a scalable bitstream. Decision: Adopted (nesting HRD parameters within the timing presence if statements, and adjusting semantics such that when POC is indicated to be proportional to timing in the VPS, this shall also be indicated in the SPS).

- It is proposed, as suggested in an editing note, change the coding of the syntax element min_spatial_segmentation_idc from u(8) to ue(v), and to specify the value range as 0..4095. Decision: Adopted.

JCTVC-L0152 On random access at CRA access units [Y. J. Cho, B. Choi, Y. Park, C. Kim (Samsung)]

The discussion in Track B was chaired by M. M. Hannuksela.

This document introduces methods to avoid reported decoder misbehaviour at CRA access units. Reported decoder misbehaviours at CRA access units are caused by 1) parameter set mismatch and 2) failure to random access signalling and/or detection. In this document, informative notes to avoid the reported decoder misbehaviour at CRA access units are proposed, and also, a condition to set the value of HandleCraAsBlaFlag is proposed for clarification.

Remark: This sounds like a systems issue.

It was noted that issue 1 is related to contribution JCTVC-L0047. No action taken on issue 1.

It was commented by some participants that the proposed NOTE 4 in clause 7.4.1.4.1 seemed unneccesary. No action taken regarding the proposed NOTE 4.

Proposal 2 is phrased by the proponent as follows: "If seq_parameter_set_id is different from that of active sequence parameter set, HandleCraAsBlaFlag is set to 1." No action taken.

JCTVC-L0170 HEVC v1 scalability hook: long-term pictures with layer_id values [M. M. Hannuksela, A. Hallapuro (Nokia)]

The discussion in Track B was initially chaired by Y.-K. Wang.

It is proposed in this contribution to enable SHVC to use a long-term reference picture having nuh_layer_id equal to A as reference for inter prediction for a picture having nuh_layer_id greater than A. This functionality would, for example, enable storing a long-term reference picture at a low resolution and hence consume a relatively moderate amount of decoded picture buffer (DPB) memory rather than storing long-term reference pictures separately at each layer they are intended to be used as reference for inter prediction.

In order to realize the proposed functionality, the following changes are proposed for HEVC version 1:

- Association of nuh_reserved_zero_6bits (which is expected to be called nuh_layer_id in SHVC) with each long-term reference picture in the reference picture set (RPS).

- Syntax changes to indicate the nuh_reserved_zero_6bits for long-term reference pictures in RPS related syntax, the presence of which is controlled by a sequence parameter set flag.

Remark: There are some issues on long-term reference picture in both text and software (at least in software), and also there are no conformance bitstreams. We need to make sure to have these fixed.

Remark: It seems there is an assumption on how DPB management is done.

Remark: It is also assumed that all short-term reference pictures are only from the same layer, i.e. inter-layer reference pictures cannot be LTRPs.

Question: It is OK to not specify this in Version 1? The propoent answered no.

It was requested to have some more time to study further the details of the proposal.

Revisit.

JCTVC-L0179 Output flag location [J. Boyce, W. Jang, D. Hong, S. Wenger (Vidyo)]

The discussion in Track B was initially chaired by Y.-K. Wang.

In SVC, output_flag was included in the NAL unit header extension, to indicate that a coded picture not be output.  In the current HEVC draft specification, pic_output_flag is located in the slice segment header, and made optional according to output_flag_present_flag in the PPS.   The location of the pic_output_flag in the slice header following variable length coded syntax elements is burdensome to a middle box that sometimes changes the value of that flag.  Two options for alternate solutions to indicate that a coded picture not be displayed are proposed which simplify middle box operation and improve robustness.

In the first proposed option, pic_output_flag is placed early in the slice segment header, before any variable length coded elements, and is always present.

In the second proposed option, a “no display” SEI message is introduced, and the pic_output_flag syntax element is removed from the slice segment header, and the output_flag_present_flag is removed from the PPS.

It was asked why a middle box would like to change the value of the pic_output_flag? It was answered that a middle box may decide that it is better not to output certain pictures that originall had the flag equal to 1.

The following use cases were also mentioned for the output flag, including: a "golden picture" that is coded only for inter prediciton reference but not for output; no-output of low-quality base layer; mandating the output/no-output of some logos; to realize the functionality that could be realized by the full frame freeze and full frame freeze release SEI messages included in AVC but not in HEVC.

Suggestion: Try to make the flag accessible without the need of entropy decoding and also not to mandate sending the flag for dependent slice segments.

Revisit.

JCTVC-L0202 Sign data hiding flag for chroma [J. Sole, M. Karczewicz (Qualcomm)]

The discussion in Track B was chaired by B. Bross.

The PPS sign_data_hiding_flag specifies whether sign data hiding is enabled. Sign data hiding provides coding efficiency improvements for luma, while losses have been observed for chroma. Therefore, it is proposed to have two flags to enable sign data hiding: one for luma and one for chroma components. This gives encoders more flexibility to select a better complexity/efficiency trade-off.

One expert expressed support to have this flexibility.

The question was raised whether it is beneficial to do SDH for chroma at all instead of making it switchable.

Another concern was raised with regard to adding an additional syntax element at this stage which would result in more conformance streams to be produced and increase the amount of code.

No action taken.

JCTVC-L0421 Crosscheck of JCTVC-L0202 on Sign Data Hiding for Chroma [Felix Henry, Gordon Clare] [late]

JCTVC-L0249 Revisit of JCTVC-K0154 on simplification of PicOrderCntMsb calculation and specification [C. Auyeung, J. Xu, A. Tabatabai (Sony)]

The discussion in Track B was chaired by Y.-K. Wang.

It is a purely editorial issue. It was delegated to the editors. The group usually follows the practice that purely editorial issues can be resolved by the editors.

No action taken.

JCTVC-L0254 AHG9: On RPS derivation and marking process for long-term reference pictures [Hendry, B. Jeon (LG)]

The discussion in Track B was initially chaired by Y.-K. Wang.

This proposal claims that there is a case for which the current HEVC specification for RPS derivation and reference picture marking is broken. The case is when there is a long-term reference picture is supposed to be removed from the DPB but is not and another reference picture with the same POC LSB is marked as LTRP in the next picture. If the newly marked LTRP is signalled with delta_poc_msb_present_flag equal to 0, according to current marking process, a decoder will keep the old LTRP, which should have been removed, and discard the supposed to be new LTRP. This document proposed to avoid such case by changing the reference picture marking process long-term reference picture.

The problem is valid. Mandating to always sending the POC MSBs is considered too bits-wasting in many cases. Changing the marking process is not the right approach, e.g. the the capablity of loss of such LTRPs through checking of the RPS become impossible. Discuss offline to find out a proper bitstream restriction that would avoid such problems.

Revisit.

JCTVC-L0323 Specification of active reference indices and decoded picture buffer [Z. Yang, B. Heng, W. Wan (Broadcom)]

JCTVC-L0363 Miscellaneous cleanup remarks for HEVC version 1 [G. J. Sullivan (Microsoft)]

The discussion in Track B was initially chaired by Y.-K. Wang, then by M. M. Hannuksela. Not all of the topics have been reviewed.

A list of miscellaneous cleanup remarks is hereby provided as follows:

- Profile/tier/level syntax should be reviewed, including the following aspects:

  - It is noted that there is no clear ability to define a higher tier for a level, that the headroom for higher level numbers is limited, that the range of profile indicator values seems limited (32 maximum, 3 used), that the nesting relationship between the drafted Main 10 and Main profiles and possibly the nesting relationship between the drafted Main or Main 10 and Main Still Picture profiles could perhaps alternatively be expressed as a constraint flag indication rather than as a different value of general_profile_idc.

Comments:

On a possible higher tier: Whether such a capability is needed depends on whether there is potentially such a need.

On possibly more profiles: It was noted that the profile_space could potentially be used for enabling the indication of more than 32 profiles. However, that would disallow the 32 compatibility flags from fully functioning.

It was noted that currently the general_profile_idc field does not do more than recommending a best usage profile, as which profiles the bitstream conforms to are accurately specified by the 32 flags.

Revisit both sub-topics. Better with David Singer being present.

  - It was suggested to consider requiring, rather than recommending, that Main 10 profile compatibility be expressed when Main profile compatibility is expressed.

It was generally considered as making sense. However, it was noted that right now it is not possible to spcify the same for a future 12-bit profile, if any.

Revisit.

  - It has been suggested to enable some way to identify a bitstream as having the currently-specified syntax and decoding process without necessarily conforming to all constraints of a currently-specified profile/tier/level combination. One possibility would be to use general_profile_idc equal to 0 for this. Another would be to define an "unconstraint" flag within the context of an existing general_profile_idc value. This should be discussed.

One suggestion was to just set the general_level_idc to an unspecified value, such 0 or 255. It was remarked there are some constraints are profile specific, not level specific.

Another suggestion was to add an informative note to say something like when certain patterns of the profile and level related fields indicate such bitstream property.

Revisit.

  - It was suggested to increase the length of general_reserved_zero_16bits (which are anticipated to become "constraint set" flags) to 48 bits. (See the prior meeting notes for JCTVC-K0119.)

(Chaired by YKW.) Decision: Adopted.

  - (Editorial) The semantics of general_profile_idc should be modified to say something like this: "When the coded video sequence conforms to multiple profiles, general_profile_idc should indicate the profile that provides the preferred decoded result or the preferred bitstream identification, as determined by the encoder (in a manner not specified in this Specification)."

- [Get notes from MMH] This seems to be essentially what Note 2 already says in 7.4.3. Decision (Ed.): Delegated to editor for consideration (pending closing the topics above).

- HRD aspects:

  - It should be discussed whether or not modulo wrapping of the CPB removal delay is actually intended (or desirable) to be specified.

- [Get notes from MMH] L0030 specifies modulo wrapping, but needs fixing w.r.t. the value not being allowed to decrease.

  - Regarding sub-picture-level HRD operation, we note that contribution JCTVC-L0044 requests consideration of decoupling the DU and AU HRD operation. This may be advisable, especially since the AU HRD operation is more well proven. We also suggest consideration of a further decoupling – such that the bit rates for DU and AU operation may not need to be the same – since ultra-low-delay decoding may use very-high-speed local data links. [Get notes from MMH] [revisit]

  - (Editorial) We should discuss the possibility of renaming the low_delay_flag (e.g. as "underflow_allowance_flag"), as its name may be confusing (e.g. with respect to DU-level HRD operation). However, we should be aware that a similar flag is similarly named in prior standards. [Get notes from MMH] [no action]

- SEI messages:

  - The various SEI messages should be reviewed in light of the fact that we now have the ability to define SEI messages of both prefix and suffix types. In particular, we suggest that the post-filter hint, user data registered, and user data unregistered (and possibly progressive refinement segment end) SEI message types should be allowed to be either prefix or suffix SEI messages. [Get notes from MMH] Agreed (all four plus filler payload)

  - The scope of the scene information SEI message seems to extend beyond the scope of the coded video sequence in which it appears. This seems ill-advised. Revisit.

  - It should be discussed whether the SEI message type ID should conceptually have a different "numbering space" for prefix and suffix messages. Currently, the message type definition does not depend on whether the message is a prefix or a suffix. [Get notes from MMH] Add a note explaining.

- On the contouring artefact issue of the previous meeting: In prior consideration of JCTVC-K0139, it was suggested that it might be worthwhile to apply the modification to 16x16 TUs in cases where max TU size is 16. This topic was suggested for further study at the time, and should be further discussed.

- Not yet discussed.

- The text relating to slice_temporal_mvp_enable_flag does not seem entirely clear. This flag was recently discussed in JCTVC-K0251, JCTVC-K0341, and item 1.3 of JCTVC-K0120 of the previous JCT-VC meeting and some changes were included in the resulting JCTVC-K1003 text. Some attempt to deal with it has been included in JCTVC-L0030. This should be reviewed.

- [Get notes from MMH] [Add a note pointing out that it has no effect on the decoding process for I slices, but may be useful to inform the decoder of the applied constraint.]

- On CABAC: Our understanding, based on a verbal remark, is that the initialization of the CABAC context probability estimate for cu_skip_flag may be a bit strange – specifically, that the probability estimate for skipping is lower when both the top and left neighbours are skipped than when only one of them is skipped.

- Not yet discussed. [Actually intended to comment on split_coding_unit_flag, suggestion 157  143.]

- POC vs. Reference index: Throughout the HEVC specification, picture order count (POC) is used to determine whether two picture references are considered equal (as opposed to using indices into a reference picture list). One exception is in the motion vector prediction process. It is proposed to consistently use POC information in the motion vector derivation process. Changes to the specification are minor and have a normative effect only in cases where a same reference picture appears multiple times in a reference picture list. [Get notes from MMH] Discussion deferred (not HLS).

- In-loop filtering: When in-loop filtering is enabled across tile boundaries but not across slice boundaries, some tile boundaries may not be slice boundaries, so some tile boundaries would seem to be specified to be filtered while others are not. This may require a decoder to track slice IDs across tiles to figure out whether or not to filter the tile boundary. It should be discussed whether this is really intended or advisable.

- [Get notes from MMH] Explicitly state that this is allowed to occur, and plan to have a conformance bitstream that tests this. (The text is asserted to actually be correct.)

- The contribution suggested general study of the places in JCTVC-K1003 (and JCTVC-L0030) at which the string "[Ed" appears. To be reviewed.

JCTVC-L0372 Clean-up of hrd_parameters( ) high-level syntax [D. Hoang (Zenverge)] [late]

### NAL unit header (2)

JCTVC-L0048 AHG9: A minor NAL unit header change in HEVC for multi-standard extensions [Y. Chen, Y.-K. Wang (Qualcomm)]

The discussion in Track B was chaired by M. M. Hannuksela.

Question: If the 4th bit of nuh_layer_id is set to 1 (as suggested in the contribution), how does an AVC decoder perceive those NAL units? Answer: they are NAL units with type 16 to 31. If the unspecified values (24 to 31) are to be avoided, then more restrictions on nuh_layer_id values are needed. It was noted that AVC legacy decoder should consider NAL units with nal_unit_type equal to 19 (auxiliary coded picture) as "unrecognized" and ignore them.

An asserted benefit of the proposal is that a multi-standard scalable HEVC bitstream could be given to an AVC decoder as such without a need of an extractor preceding AVC and HEVC decoders.

There was a comment that with the proposal the first byte of the NAL unit header can be 0. However, this is not a problem for start code emulation, because the second byte of the NAL unit header is always non-zero.

There was a comment that in the systems layer a more general solution would be needed in order to support e.g. MPEG-2 video. It was commented that in some systems only AVC (or HEVC) base layer could be relevant.

There was a comment that in the proposed syntax nal_unit_type crosses a byte boundary, which could be burdensome for some parsers.

Question: Would the AVC decoder choke when it gets the HEVC EL NAL units? Answer: AVC HRD parameters should take the HEVC EL NAL units into account.

No action taken.

JCTVC-L0131 Multi-standard extension design [B. Choi, Y.J. Cho, M.W. Park, J. Yoon, J. Park (Samsung)]

The discussion in Track B was chaired by M. M. Hannuksela.

Proposal #1: AVC NAL unit encapsulation in HEVC NAL units with changes in the NAL unit header.

Question: Are BASE_NUTs VCL or VCL NAL units from HRD point of view? That aspect was not considered in the contribution. Maybe to use two NAL unit types, one for VCL and another for non-VCL NAL units.

There was a comment that in the JCTVC-K meeting there was a design choice not to change the NAL unit header syntax and parsing based on nal_unit_type.

It was commented that an alternative design could be to use a two-byte HEVC NAL unit header always (without conditional fields) and add a third byte when nal_unit_type is equal to BASE_NUT.

Question: How to specify, in the HEVC specification, the semantics of the syntax elements when nal_unit_type is equal to BASE_NUT?

Comment: JCTVC-K1007 included text on how to utilize the temporal_id_plus1 of the HEVC NAL unit header for AVC base layer.

It was commented that there needs to be an extractor process preceding the AVC and HEVC decoders.

No action taken on proposal #1.

Proposal #2: Profile and level information for AVC base layer into VPS extension. Revisit proposal #2 in the VPS extension category of proposals.

Proposal #3: Move avc_base_codec_flag from VPS extension to the VPS itself.

It was commented that the single flag is not capable of supporting e.g. MPEG-2 video base layer.

It was commented that avc_base_codec_flag might be better suited for systems layer design.

No action taken on proposal #2.

### Parameter sets (8)

Also see the following:

- JCTVC-L0007

- JCTVC-L0225

JCTVC-L0047 AHG9: Indication of parameter sets properties in HEVC [Y.-K. Wang, A. K. Ramasubramonian (Qualcomm)]

The discussion in Track B was chaired by M. M. Hannuksela.

Proposal 1: indication of "full random accessibility" in VPS.

Question: why to have the flag in VPS, not in SPS? Answer: the flag is helpful for systems, not needed for decoding. Can be used e.g. for content negotiation through VPS.

Question: is there an error resilience impact? Probably not, at least not a negative impact.

It was remarked that proposal 1 sounds like a systems issue.

It was noted that the proposed flag is helpful e.g. when encapsulating a bitstream into a container file (as it can enable the file encapsulator not to parse through the bitstream).

Question: why to have the flag in VPS, not in SEI? Answer: the assumption is that VPS is present in session negotiation, content announcement, etc. (whereas SEI messages might not).

There was some support expressed in having the ability to convey the information in proposal 1 through some means.

It was noted that potentially the information of the flag could be carried in an SEI message, but having one SEI message for just one flag might not be reasonable. Consider combining with other pieces of information into a single SEI message.

Revisit (an SEI message design combined with other pieces of information).

Proposal 2: no parameter set update present flag in VPS.

The flag applies to VPS, SPS, and PPS. The proposed semantics of the flag have an impact that crosses the boundary of the coded video sequence. It was noted that a splicer may have to change the value of the proposed flag from 1 to 0.

There was some support expressed in having the ability to convey the information in proposal 2 through some means.

It was noted that potentially the information of the flag could be carried in an SEI message, but having one SEI message for just one flag might not be reasonable. Consider combining with other pieces of information into a single SEI message.

Revisit (an SEI message design combined with other pieces of information).

JCTVC-L0097 Sample scale factor in VUI [Arturo A Rodriguez (Cisco)]

JCTVC-L0132 Video parameter set design [B. Choi, Y.J. Cho, M.W. Park, J. Yoon, J. Park (Samsung)]

JCTVC-L0155 AHG9: On column_width_minus1 syntax [O. Nakagami, T. Suzuki (Sony)]

JCTVC-L0225 Video Sequence Characteristics Signaling in VPS and VPS Extension [M. Haque, A. Tabatabai, S. Deshpande]

JCTVC-L0227 VPS extension with updates of profile-tier-level syntax structure [M. Haque, A. Tabatabai (Sony)

JCTVC-L0247 Improved Bitstream Characteristics in VPS and SEI message [T. C. Thang (UoA), J. W. Kang, H. Lee, J. Lee, J. S. Choi (ETRI)]

JCTVC-L0276 On hrd_parameters( ) [K. Sato (Sony)]

JCTVC-L0280 On profile_tier_level( ) [K. Sato (Sony)]

### Slices and slice header (4)

JCTVC-L0126 Slice header clean-up [B. Choi, Y.J. Cho, M.W. Park, J. Yoon, J. Park (Samsung)]

JCTVC-L0192 Semantics of no_output_of_prior_pics_flag [Arturo A Rodriguez (Cisco)]

JCTVC-L0255 AHG 9: On dependent slice segment signalling [Hendry, B. Jeon (LG)]

JCTVC-L0324 Generic Slice NAL Unit Types [W. Wan, B. Heng, P. Chen (Broadcom)]

### High-level parallelism (2)

JCTVC-L0116 High-level parallelism clean-ups [T. Lee, B. Choi (Samsung)]

### Hypothetical reference decoder (HRD) (3)

JCTVC-L0044 AHG9: HEVC HRD cleanups [Y.-K. Wang, A. K. Ramasubramonian, Y. Chen (Qualcomm)]

JCTVC-L0219 On bumping and sps_max_num_reorder_pics [R. Sjöberg, J. Samuelsson (Ericsson)]

JCTVC-L0328 HRD issue for bitstream splicing [G. J. Sullivan, L. Zhu, S. Sankuratri (Microsoft)]

### Frame packing arrangement (4)

JCTVC-L0046 AHG9: Indication of interlaced and frame-packed video in HEVC [Y.-K. Wang (Qualcomm)]

JCTVC-L0223 Frame packing with 2D compatible output [J. Samuelsson, R. Sjöberg] (Ericsson)]

JCTVC-L0293 On frame packing arrangement SEI message [B. Choi, C. Kim, J. Park (Samsung)]

Not version 1.

JCTVC-L0316 Updated proposal for frame packing arrangement SEI for 4:4:4 content in 4:2:0 bitstreams [Y. Zhang, Y. Wu, S. Kanumuri, S. Sadhwani, G. J. Sullivan, H. S. Malvar (Microsoft)]

Not version 1.

### SEI messages (5)

JCTVC-L0045 AHG9: HEVC SEI messages cleanups [Y.-K. Wang, Y. Chen, A. K. Ramasubramonian (Qualcomm)]

JCTVC-L0049 SEI message: independently decodable regions based on tiles [Y. Ye, Y. He, Y. He (InterDigital), X. Yang, P. Yue, Y. Zhang (Huawei), M. Horowitz (eBrisk Video)]

Prior relevant proposals K0248 and K0116. Not version 1.

JCTVC-L0077 Additional VUI and SEI for chroma sampling filter [T. Chujoh (Toshiba), K. Kazui (Fujitsu Lab.), P. Topiwala, D. Liu, M. Krihnan (FastVDO LLC), M. Mark, A. Gabriellini (BBC)]

Prior relevant proposal K0152. Not version 1.

JCTVC-L0208 AHG9 / HEVC v1: Updated text of the SOP description SEI message [M. M. Hannuksela (Nokia)]

JCTVC-L0325 Bounding redundant SEI messages [W. Wan, W. Ahmad (Broadcom)]

## Motion hook in version 1 (3)

JCTVC-L0177 Hook on temporal motion vector prediction for MV-HEVC [Y. Lin, X. Zheng, J. Zheng (HiSilicon)]

JCTVC-L0400 Cross-check of JCTVC-L0177: Hook on temporal motion vector prediction for M-HEVC [L. Zhang (Qualcomm)] [late] [miss]

JCTVC-L0257 Temporal motion vector prediction hook for efficient merge mode in MV-HEVC [Y. Chen, Y.-K. Wang, V. Seregin, L. Zhang, M. Karczewicz (Qualcomm), K. Ugur, M. M. Hannuksela (Nokia)]

## Other topics for version 1

See JCTVC-L0363 regarding:

- Contouring artefact

- CABAC initialization

- POC versus reference index usage.

## Range extensions

### Transforms (CE1 related)

JCTVC-L0149 Non-CE1: Test 3 - Transform skip for chroma in 4:2:2 Range Extension [C. Rosewarne, V. Kolesnikov, M. Maeda (Canon)]

JCTVC-L0150 Non-CE1: Test 3 - Square transform deblocking for 4:2:2 [C. Rosewarne, V. Kolesnikov, M. Maeda (Canon)]

JCTVC-L0424 Cross-check for JCTVC-L0150 [Marta Mrak, Andrea Gabriellini (BBC)] [late]

JCTVC-L0151 Non-CE1: Test 3 - 32x32 transform in chroma for 4:2:2 [V. Kolesnikov, C. Rosewarne, M. Maeda (Canon)]

- JCTVC-L0417 Crosscheck of non-CE1 test 3 with 32x32 transform in chroma for 4:2:2 Range Extension (JCTVC-L0151) [J. Sole (Qualcomm)] [late]

JCTVC-L0351 Non-CE1: Square transform blocks for 4:2:2 [J. Sole, R. Joshi, M. Karczewicz (Qualcomm), A. Gabriellini, M. Mrak (BBC)]

JCTVC-L0407 Cross-check of Non-CE1: Square transform blocks for 4:2:2 (L0351) by Qualcomm [C. Rosewarne, M. Maeda (Canon)] [late]

### Intra prediction

JCTVC-L0112 AHG7: Reference sample filtering in intra prediction for chroma extended format [J. Min, C. Kim (Samsung)]

JCTVC-L0391 Crosscheck report of reference sample filtering in intra prediction for chroma extended format (JCTVC-L0112) [S. Matsuo, S. Takamura, H. Fujii, A. Shimizu (NTT)] [late]

JCTVC-L0128 AHG7: Modification of intra angular prediction blending [S. Matsuo, S. Takamura, H. Fujii, A. Shimizu (NTT)]

JCTVC-L0374 Cross-check of modified intra angular blending (JCTVC-L0128) [K. Kawamura, S. Naito (KDDI)] [late]

JCTVC-L0129 AHG7: Modification filter for intra angular prediction [S. Matsuo, S. Takamura, H. Fujii, A. Shimizu (NTT)]

JCTVC-L0375 Cross-check of Modification filter for intra angular prediction (JCTVC-L0129) [K. Kawamura, S. Naito (KDDI)] [late]

JCTVC-L0399 AHG7: Cross-check results of Sample-based angular intra prediction for HEVC range extension (JCTVC-L0176)  [W. Gao, J. Ye, H. Yu (Huawei)] [late]

### Mixed format coding

JCTVC-L0162 AHG7: Coding 4:2:2 chroma format with 4:2:0 and 4:4:4 format codecs [A. Gabriellini, M. Mrak (BBC)]

JCTVC-L0175 AHG7: A Consumer 4:4:4 Coding Method Based On A 4:2:0 Codec [P. Topiwala, W. Dai, M. Krishnan (FastVDO)]

JCTVC-L0321 AHG7: Cross-verification of JCTVC-L0175, A Consumer 4:4:4 Coding Method Based On A 4:2:0 Codec [R. Cohen (MERL)] [late]

JCTVC-L0240 AHG7: The performance of extended chroma mode for non 4:2:0 format [J.Kim (LG)]

JCTVC-L0355 Crosscheck of LG's proposal JCTVC-L0240 [Xingyu Zhang (HKUST), Wenjing Zhu, Oscar C. Au] [late]

JCTVC-L0241 rext: Mixed 4:4:4 content [D. Flynn (RIM)] [miss]

JCTVC-L0250 Mixed chroma format coding [M. Budagavi, M. Zhou (TI)]

### Other

JCTVC-L0127 AHG7: LCU-based non-local means filter for chroma components in HEVC range extensions [M. Matsumura, S. Takamura, H. Fujii, A. Shimizu (NTT)]

JCTVC-L0373 Cross-check of non-local means filter for chroma components (JCTVC-L0127) [K. Kawamura, S. Naito (KDDI)] [late]

JCTVC-L0370 AHG7: Inter-plane intra coding of residual signals for range extensions [K. Kawamura, T. Yoshino, S. Naito (KDDI)] [late]

JCTVC-L0123 Cross-check report of inter-plane intra coding of residual signals (JCTVC-L0370) [S. Matsuo, M. Matsumura, S. Takamura, H. Fujii, A. Shimizu (NTT)] [late]

JCTVC-L0371 AHG7: In-loop color-space transformation of residual signals for range extensions [K. Kawamura, T. Yoshino, S. Naito (KDDI)] [late]

JCTVC-L0124 Cross-check report of in-loop color-space transformation of residual signals (JCTVC-L0371) [S. Matsuo, M. Matsumura, S. Takamura, H. Fujii, A. Shimizu (NTT)] [late]

JCTVC-L0189 Analysis on high internal bit depth processing [R. Weerakkody, A. Gabriellini, M. Naccari, M. Mrak (BBC)]

## Scalable video coding

### TE1 related (upsampling filter)

JCTVC-L0199 Non-TE1: Fixed Upsampling Filters [Zhenzhong Chen, Shan Liu, Shawmin Lei (MediaTek)]

(include abstract)

No need to be presented, as the difference compared to current filters is marginal.

JCTVC-L0144 Non-TE1 : crosscheck of fixed upsampling filter from MediaTek [J. Park, C. Kim, B. Jeon (LG)] [late]

    - TE2 related (inter-layer texture prediction)

JCTVC-L0134 Non-TE 2: Constrained intra prediction at enhancement layer [C. Kim, J. Kim, B. Jeon (LG)]

This contribution presents constrained intra prediction (CIP) at enhancement layer (EL). In the SMuC-0.1.1, when constrained intra prediction (CIP) is enabled, Intra BL cannot be used in intra prediction. In the proposed technique, when CIP is enabled, if slice type of base layer is Intra, an intra BL CU at the enhancement layer should be used in intra prediction. Simulation results reportedly show 1.5%, 0.4%, 0.4%, 0.1%, 0.0%, 0.1%, 0.0% 0.0% BD-rate savings on average for AI-2x, AI-1.5x, RA-2x, RA-1.5x, , RA-SNR, LDP-2x, LDP-1.5x and LDP-SNR, respectively, compared with SMuC-0.1.1 anchors(CIP On).

JCTVC-L0345 Crosscheck of constrained intra prediction at enhancement layer in JCTVC-L0134 proposed by LG [T.-D. Chuang, Y.-W. Huang (MediaTek)] [late]

Runtime reported in proposal is not accurate, according to cross-checker the runtime was lower than in SMuC.

Cross-checkers support the idea.

From the subsequent discussion, several experts expressed the opinion that the suggested solution (enabling Intra_BL in combination with EL CIP) may not be generic enough; another possibility to avoid the error propagation might be to enable CIP in both base and enhancement layers.

The suggested solution may also not be generic enough in case of multiple slices, and in case of non-dyadic scalability (where an enhancement layer PU may cover inter and intra PUs from base layer).

In an error-prone environment, an encoder could take action to use intra_BL only for regions where the base layer is also intra coded, there would be no need to forbid the combination of CIP and Intra_BL in a normative.

Further study needed, no action on the current proposal.

JCTVC-L0174 Non-TE2: Inter-layer reference picture placement [P. Yin, T. Lu, T. Chen (Dolby), X.Xiu, Y.Ye (InterDigital)]

This contribution provides some additional data based on TE A2 3.2.1: interlayer reference picture placement.  The objective is to reduce the encoder run-time in the AI case. All the changes are simple non-normative encoder modification. Two sets of test results are presented, and compared to the best case in TE A2 3.2.1, RefIdx setting 2 zeroMV. The first set of test results reports a reduction of about 50% in encoder run-time with a BDrate of {0.3% and 0.8%} for Y for {AI 2x, AI 1.5x} in the enhancement layer. The second set of test results reports a reduction of about 25% in encoder run-time with a BDrate of {-0.1% and 0.0%} for Y for {AI 2x, AI 1.5x} in the enhancement layer.  The simulation results also report an improvement on chroma coding performance in all the test cases, AI, RA and LD-P.

Powerpoint presentation to be uploaded.

- Add a mode decision module for 2Nx2N ILR (Inter-layer reference prediction) with uni-prediction and motion vectors forced to be zero.

- When BL is I_SLICE, two test methods have been configured. In Test Method 1, only intra mode and 2Nx2N ILR mode are tested, and all the other inter modes are not tested. In Test Method 2, only 2Nx2N merge mode, intra mode and 2Nx2N ILR mode are tested.

- When BL is not I_SLICE, two changes are applied. First, in the 2Nx2N inter mode, ME excludes the testing of inter-layer reference picture. Second, the 2Nx2N ILR mode is tested after intra mode.

Decision(SW): Adopt method 1 as default for ref_idx, method 2 as option

JCTVC-L0261 Non-TE2: Cross-check report of JCTVC-L0174 Inter-layer reference picture placement [L. Guo (Qualcomm)] [late]

JCTVC-L0230 Non-TE2: Improved inter-layer texture prediction signaling [D.-K. Kwon, M. Budagavi, M. Zhou (TI)]

In this contribution, the inter-layer texture prediction (IntraBL) signaling in the current SMuC software is modified to reportedly improve coding efficiency. First, it is proposed to signal whether IntraBL CU has coded coefficients using two existing flags for Intra (I) and Inter (P and B) slices, respectively. In Intra slice, it is allowed to signal no_residual_syntax_flag for IntraBL CU when it has no coded coefficients. In Inter slice, it is allowed to signal skip_flag for IntraBL CU, which means that skip_flag can be set to 1 either by the merge process or the inter-layer texture prediction. Second, in Inter slices, it is proposed to modify the merge process so as to support inter-layer texture prediction for PU smaller than 2Nx2N. IntraBL mode is added in the merge candidate list if the number of spatial neighboring candidates is less than 4. Inter-layer texture prediction gets enabled when the signaled merge_index specifies IntraBL mode. Experimental results with common test condition reportedly show that, when compared to the SMuC software (SMuC_0.1.1bf), the proposed methods result in luma BL+EL BD-rate gain of 0.2%, 0.3%, 0.4%, 0.4%, 0.2%, 0.7%, 0.9% and 0.5% for AI 2x, AI 1.5x, RA 2x, RA 1.5x, RA SNR, LD-P 2x, LD-P 1.5x and LD-P SNR, respectively.

The proposal shows gain by putting the Intra_BL to PU level via the merge flag, while keeping signalling at the CU level. Would it be more consistent to put the signalling (e.g. via ref_idx) to the PU level anyway?

It is also mentioned that the approach comes with losses in chroma, but overall the gain looks interesting and further study in the context of PU-level Intra_BL is recommended.

JCTVC-L0236 Non-TE2: Cross-check Results of Improved inter-layer texture prediction signaling (JCTVC-L0230) [Zhan Ma, Felix Fernandes (Samsung)] [late]

    - TE3 related (combined prediction)

JCTVC-L0074 Non-TE3: Adaptive predictor compensation with generalized residual prediction [T.-D. Chuang, S. Liu, M. Guo, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-L0196 Non-TE3: Crosschecking results of Adaptive predictor compensation with generalized residual prediction [X. Li (Qualcomm)] [late]

JCTVC-L0084 Non TEB3: Bi-directional optical flow for inter-layer texture prediction [A.Alshin, E.Alshina, J.H.Park (Samsung)]

JCTVC-L0243 Crosscheck report of Samsung's proposal JCTVC-L0084 [J. Kim (LG)]

JCTVC-L0104 Non-TE3: Simplification of Generalized Residual Inter-Layer Prediction (GRILP) in SHVC  [E. François, J. Taquet, C. Gisquet, G. Laroche, P. Onno (Canon) ]

JCTVC-L0419 Cross-check of JCTVC-L0104 (simplification of generalized residual inter-layer prediction by Canon) [J. Lainema (Nokia)] [late]

JCTVC-L0383 Non-TE3: Crosschecking of Simplification of Generalized Residual Inter-Layer Prediction (GRILP) in SHVC [X. Li (Qualcomm)] [late]

JCTVC-L0140 Non-TE3 : Modifications on inter-layer texture, DC, planar predictions [J. Park, B. Jeon (LG)]

JCTVC-L0312 Non-TE B3: Crosscheck for modifications on inter-layer texture, DC, planar predictions (JCTVC-L0140) [W. Zhang, L. Xu, Y. Han, Z. Deng, X. Cai, Y. Chiu (Intel)] [late]

JCTVC-L0190 Non-TE3: Extension of Test 4.6.2.1 on Generalized Residual Prediction [X. Li, J. Chen, K. Rapaka, M. Karczewicz (Qualcomm)]

JCTVC-L0327 Non-TE3: Cross-check of JCTVC-L0190 on extension of generalized residual prediction [C. Yeo (I2R)] [late]

JCTVC-L0193 Base Layer residue upsampling and skip mode [P. Lopez, P. Andrivon, P. Bordes, P. Salmon]

JCTVC-L0215 On Intra Prediction for SHVC [C. Auyeung, A. Tabatabai (Sony)]

JCTVC-L0265 On interpolation filter for Generalized Residual Prediction [T. Tsukuba, T. Yamamoto, T. Ikai (Sharp)]

JCTVC-L0420 Cross-check of JCTVC-L0265 (on interpolation filter for generalized residual prediction by Sharp) [J. Lainema (Nokia)] [late]

JCTVC-L0267 Hybrid Intra and Inter-layer Prediction [T. Yamamoto (Sharp)]

JCTVC-L0273 Cross-check results of Hybrid Intra and Inter-layer Prediction (JCTVC-L0267) [Zhan Ma, Felix Fernandes (Samsung)] [late]

JCTVC-L0278 SHVC: On Inter-layer Prediction [K. Sato (Sony)]

JCTVC-L0326 Cross check for restriction on Inter-layer prediction (JCTVC-L0278) [T. Yamamoto (Sharp)] [late]

JCTVC-L0337 SHVC: Cross-verification of restriction on inter-layer texture prediction at high temporal hierarchy (JCTVC-L0278) [H. Nakamura (JVC Kenwood)] [late]

JCTVC-L0294 Non-TE3: Simplification of Difference Intra Prediction In SHVC [K. Rapaka, J. Chen, X. Li, M. Karczewicz (Qualcomm)]

JCTVC-L0329 Non-TE3: crosscheck of simplification on difference intra prediction in SHVC (JCTVC-L0294) from QualComm [J. Park, B. Jeon (LG)] [late]

JCTVC-L0384 Non-TE3: Crosschecking of simplification of intra residual planar prediction in TE3-4.2.2 [K. Rapaka (Qualcomm)] [late]

JCTVC-L0412 Non-TE 3: On estimation theoretic prediction for enhancement layer residual in scalable video coding [A. Saxena, F. Fernandes (Samsung)] [late]

### TE4 related (inter-layer filtering)

JCTVC-L0059 Chroma enhancement for ILR picture [J. Dong, Y. He, Y. Ye (InterDigital)]

JCTVC-L0410 Cross-check results of Chroma enhancement for ILR picture (JCTVC-L0059) [W. Pu, J. Chen (Qualcomm)] [late] [miss]

JCTVC-L0076 Non-TE4.4: Inter-layer adaptive filter on upsampled BL with CU on/off flags [C.-Y. Chen, C.-Y. Tsai, M. Guo, S. Liu, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-L0315 Non-TE B4: Crosscheck for inter-layer adaptive filter with CU on/off flags (JCTVC-L0076) [W. Zhang, L. Xu, Y. Han, Z. Deng, X. Cai, Y. Chiu (Intel)] [late]

JCTVC-L0086 Non-TEB4: Band Offset correction for inter-layer texture prediction [E.Alshina, A.Alshin, J.H.Park (Samsung)]

JCTVC-L0245 Cross-check results of Band Offset correction for inter-layer texture prediction (non-TEB4) [W. Pu, J. Chen (Qualcomm)]

JCTVC-L0401 non-TEB4: Cross-check results of Inter Layer SAO (JCTVC-L0107) [W. Pu (Qualcomm)] [late] [miss]

JCTVC-L0087 On integer samples filtering during up-sampling [E.Alshina, A.Alshin, J.H.Park (Samsung)]

JCTVC-L0272 Cross-check for JCTVC-L0087 on integer sample filtering during up-sampling [J. Dong, Y. Ye (InterDigital)] [late]

JCTVC-L0107 Inter Layer SAO [G. Laroche, T. Poirier, C. Gisquet, E. François, P. Onno (Canon)]

JCTVC-L0195 Non-TE4: Switchable De-ringing Filter for Inter-layer Prediction [Zhan Ma, Felix Fernadnes]

JCTVC-L0271 Cross check of Adaptive filter (JCTVC-L0195) [T. Yamamoto (Sharp)] [late]

JCTVC-L0357 Crosscheck of JCTVC-L0195 on Non-TE4: Switchable De-ringing Filter for Inter-layer Prediction [D.-K. Kwon (TI)] [late]

JCTVC-L0233 Multi-Type Inter Layer Sample Adaptive Offset Filter [W. Pu, J. Chen, K. Rapaka, M. Karczewicz (Qualcomm)]

JCTVC-L0348 non TEB4: Cross-check of Multi-Type Inter Layer Sample Adaptive Offset Filter (JCTVC-L0233) [E.Alshina, A.Alshin, J.H.Park (Samsung)] [late]

JCTVC-L0234 High Frequency Pass Inter Layer Sample Adaptive Offset Filter [W. Pu, J. Chen, K. Rapaka, M. Karczewicz (Qualcomm)]

JCTVC-L0349 non TEB4: Cross-check of High Frequency Pass Inter Layer Sample Adaptive Offset Filter (JCTVC-L0234) [E.Alshina, A.Alshin, J.H.Park (Samsung)] [late]

JCTVC-L0252 Non-TE4: Adaptive up-sampling of base layer picture using bilateral filters [J. Zhao, K. Misra, A. Segall (Sharp)]

JCTVC-L0361 Cross-check Results for Non-TE4: Adaptive up-sampling of base layer picture using bilateral filters (JCTVC-L0252) [Z. Ma, F. Fernades (Samsung)] [late]

JCTVC-L0256 Non-TE4: Inter-layer Adaptive Filters [M. Guo, P. Lai, S. Liu, C.-Y. Tsai, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-L0319 Non-TE B4: Crosscheck for inter-layer Wiener filter (JCTVC-L0256) [W. Zhang, L. Xu, Y. Han, Z. Deng, X. Cai, Y. Chiu (Intel)] [late]

JCTVC-L0352 Non-TE4: Crosscheck report of Mediatek's proposal JCTVC-L0256 [J. Kim (LG)] [late]

JCTVC-L0283 Non-TE4: Inter-layer SAO [Z. Chen, S. Liu, X. Zhang, S.-T. Hsiang, C.-M. Fu, S. Lei (MediaTek)]

JCTVC-L0369 Non-TE4: Cross-check report of Inter-layer SAO (JCTVC-L0283) [L. Guo (Qualcomm)] [late] [late]

JCTVC-L0290 Non-TE B4: Inter-layer fixed refining filter [W. Zhang, L. Xu, Y. Han, Z. Deng, X. Cai, Y. Chiu (Intel)]

JCTVC-L0346 Non-TE4.4: Crosscheck of inter-layer filter in JCTVC-L0290 proposed by Intel [C.-Y. Chen, Y.-W. Huang (MediaTek)] [late]

JCTVC-L0087 On integer samples filtering during up-sampling [E.Alshina, A.Alshin, J.H.Park (Samsung)]

JCTVC-L0272 Cross-check for JCTVC-L0087 on integer sample filtering during up-sampling [J. Dong, Y. Ye (InterDigital)] [late]

JCTVC-L0059 Chroma enhancement for ILR picture [J. Dong, Y. He, Y. Ye (InterDigital)]

### TE5 related (Inter-layer syntax prediction with HEVC base)

JCTVC-L0040 Non-TE5: On motion mapping between layers in SHVC [K. Ugur (Nokia)]

JCTVC-L0248 Non-TE5: Cross-verification of motion mapping between layers in SHVC from Nokia [X. Xiu (InterDigital)]

JCTVC-L0068 Fast CU Depth Decision in Enhancement Layer for SHVC [K. Kim, H. Jang, Y. Ahn, D. Sim (KWU)]

JCTVC-L0073 Non-TE5: On the effectiveness of temporal and base layer co-located motion vector prediction candidates [Y. H. Tan, C. Yeo (I2R)]

JCTVC-L0094 Non TEC5: Cross-check of assigning intra prediction mode [E.Alshina, A.Alshin, J.H.Park (Samsung)] [late]

JCTVC-L0103 Non-TE5.3: Inter-layer inferred prediction mode and motion compensation restrictions in SHVC [Christophe Gisquet, Edouard François, Guillaume Laroche, Patrice Onno (Canon)]

JCTVC-L0416 Non-TE5.3: Cross-check report of Inter-layer inferred prediction mode and motion compensation restrictions in SHVC (JCTVC-L0103) [W. Gao, J. Ye, H. Yu (Huawei)] [late] [miss]

JCTVC-L0105 Non-TE5.1: MPM derivation and coding in SHVC [E. François, S. Shi, C. Gisquet, G. Laroche, P. Onno (Canon)]

JCTVC-L0403 Cross-check of Non-TE5.1: MPM derivation and coding in SHVC (JCTVC-L0105) [J. Xu (Sony)] [late]

JCTVC-L0106 Non-TE5.1: simplification of remaining modes coding in SHVC [E. François, S. Shi, C. Gisquet, G. Laroche, P. Onno (Canon)]

JCTVC-L0402 Cross-check of Non-TE5.1: simplification of remaining modes coding in SHVC (JCTVC-L0106) [J. Xu (Sony)] [late]

JCTVC-L0139 Non-TE 5: Motion assignment for Intra/Intra BL at enhancement layer [C. Kim, B. Jeon (LG)]

JCTVC-L0356 Crosscheck of JCTVC-L0139 on Non-TE5: Motion assignment for Intra/Intra BL at enhancement layer [D.-K. Kwon (TI)] [late]

JCTVC-L0156 Simplification of TE5.1.5 on inter-layer intra mode prediction [Z. Zhao, J. Ostermann (Leibniz Uni Hannover)]

JCTVC-L0163 non-TEC5: Cross-verification of simplified TEC5 Test 5.1.5 Inter-layer intra prediction mode coding (ILIPM) [H. Nakamura (JVC Kenwood)]

JCTVC-L0194 Non-TE5: Crosscheck results of On the effectiveness of temporal and base layer co-located motion vector prediction candidates [X. Li (Qualcomm)] [late]

JCTVC-L0220 Inter-layer intra mode prediction [J. Xu, A. Tabatabai (Sony)]

JCTVC-L0408 Non-TE5: cross-check of JCTVC-L0220 on MPM derivation for EL in SHVC [E. François (Canon)] [late]

JCTVC-L0221 Inter-layer MV prediction [J. Xu, A. Tabatabai, K. Sato, S. Lu (Sony)]

JCTVC-L0358 Non-TE5: Cross-check for JCTVC-L0221 Memory reduction for MV data of EL in SHVC [J. Dong, Y. Ye (InterDigital)] [late]

JCTVC-L0239 Assigning intra prediction mode to inter layer intra predicted blocks in enhance layer [J. Kim, S. Liu, M. Gui, S. Lee, J. Park, B. Jeon (LG)]

JCTVC-L0260 Non-TE5.1: Inter-layer Intra mode prediction [M. Guo, S. Liu, S. Lei (MediaTek), J. Park, J. Kim, B. Jeon (LG)]

JCTVC-L0279 SHVC: On Motion Data Compression [K. Sato, S. Lu, J. Xu (Sony)]

JCTVC-L0368 Cross-check of JCTVC-L0279 [J. Lee] [late]

JCTVC-L0291 Non-TE C5: Inter-layer motion data inheritance [Z. Deng, W. Zhang, L. Xu, Y. Han, X. Cai, Y. Chiu (Intel)]

JCTVC-L0411 Non-TE5: Cross-check of Inter-layer motion data inheritance by Intel (JCTVC-L0291) [L. Guo, X. Li (Qualcomm)] [late] [miss]

JCTVC-L0413 Non-TE5: Cross-check of the motion data inheritance proposed in JCTVC-L0291 [H. Yang (Huawei)] [late]

JCTVC-L0336 Non-TE5: on motion mapping in SHVC [J. Chen, V. Seregin, L. Guo, M. Karczewicz (Qualcomm)]

JCTVC-L0365 Non-TE5 Crosscheck of JCTVC-L0336: On motion mapping in SHVC [Y. He, X. Xiu (InterDigital)] [late]

JCTVC-L0398 NonTE5: Partition-dependent inter-layer motion vector prediction [S. Lu, 
K. Sato (Sony)] [late] [initial version rejected as placeholder]

JCTVC-L0418 NonTE5: Crosscheck for partition-dependent inter-layer motion vector prediction (JCTVC-L0398) [W. Zhang, L. Xu, Y. Han, Z. Deng, X. Cai, Y. Chiu (Intel)] [late] [miss]

JCTVC-L0405 Non-TEC5: Cross-check for Inter-layer Intra mode prediction [E.Alshina (Samsung)] [late]

### Performance of tool combinations

JCTVC-L0057 Performance of combined inter-layer adaptive filter and enhancement layer skipped slice [Y. He, Y. He, X. Xiu, Y. Ye (InterDigital), T. Yamamoto, Y. Yasugi (Sharp)]

JCTVC-L0414 Cross-check of the combined skipped slice and adaptive filter proposed in JCTVC-L0057 [H. Yang, D. Jiang (Huawei)] [late]

JCTVC-L0228 Performance of HLS-only tools in SHVC [Y. Ye, J. Dong, X. Xiu, Y. He, Y. He (InterDigital), P. Yin, T. Lu, T. Chen (Dolby)]

JCTVC-L0108 Combination of several interlayer tools [Christophe Gisquet, Edouard François, Guillaume Laroche, Patrice Onno (Canon)]

JCTVC-L0211 NonTE4: Combination of TE-B4 4.2.1 Adaptive Up-sampling Filter and TE3 4.6.2.1 Generalized Residual Prediction [J. Chen, X. Li, M. Karczewicz (Qualcomm)]

JCTVC-L0314 Non-TE B4: Crosscheck for combination of TE B4 4.2.1 and TE3 4.6.2.1 (JCTVC-L0211) [W. Zhang, L. Xu, Y. Han, Z. Deng, X. Cai, Y. Chiu (Intel)] [late]

### Low-complexity Fidelity (SNR) Scalability

JCTVC-L0042 Content Adaptive Complexity Reduction Scheme For Quality/Fidelity Scalable HEVC [H.-R. Tohidypour, M.T. Pourazad, P. Nasiopoulos (UBC & TELUS)]

JCTVC-L0109 An inter layer prediction scheme for SNR scalability of HEVC [Gang Wu, Wenpeng Ding, Yunhui Shi, Baocai Yin (BJUT)]

JCTVC-L0154 Single-loop SNR scalability using Binary Residual Refinement Coding [Christian Feldmann, Fabian Jäger, Juliana Hsu, Mathias Wien (RWTH Aachen)]

### Modifications to ref_idx scheme

JCTVC-L0050 Modified deblocking filter for the ref_idx framework [Y. Ye, Y. He, J. Dong, X. Xiu (InterDigital)]

JCTVC-L0367 Cross-check of Modified deblocking filter for the ref_idx framework (JCTVC-L0050) [H. Lee, J. W. Kang, J. Lee (ETRI)] [late]

JCTVC-L0053 Modified motion vector signaling for the ref_idx framework [X. Xiu, Y. Ye, Y. He, Y. He (InterDigital)]

JCTVC-L0393 Cross-check of Modified motion vector signaling for the ref_idx framework (L0053 from Interdigital) [E. François (Canon)] [late]

JCTVC-L0167 An encoder bug fix for the reference index framework in SHVC reference software [J. Zhang, B. Li, H. Li (USTC), H. Yang (Huawei)]

JCTVC-L0364 Crosscheck of JCTVC-L0167: An encoder bug fix for the reference index framework in SHVC reference software [Y. He (InterDigital)] [late]

JCTVC-L0213 Differential coding for RefIdx based scalability [A. Aminlou, J. Lainema, K. Ugur, M. Hannuksela (Nokia)]

### Transforms in SHVC

JCTVC-L0067 On transform selection for IntraBL mode in SHVC [C. Yeo, Y. H. Tan (I2R)]

JCTVC-L0204 AHG11: DST transform for Intra-BL mode [V. Seregin, T.-D. Chuang, D.-K. Kwon, F. Le Leannec, P. Onno (Canon)]

JCTVC-L0330 Test of Transform Selection for Inter-Layer Texture Prediction on SMuC 0.1.1 [L. Guo, M. Karczewicz, J. Chen (Qualcomm)]

JCTVC-L0366 Cross-check report of Test of Transform Selection for Inter-Layer Texture Prediction on SMuC 0.1.1 (JCTVC-L0330) [Z. Chen, S. Liu (??)] [late]

### Other scalable modalities

JCTVC-L0334 Color Gamut Scalable Video Coding [L. Kerofsky, A. Segall, S.-H. Kim, K. Misra (Sharp)]

JCTVC-L0145 Cross check of JCTVC-L0334 on Color Gamut Scalable Video Coding [S. Lu, T. Suzuki (Sony)] [late]

JCTVC-L0229 Inter-layer texture prediction for bit-depth and chroma format scalability [D.-K Kwon, M. Budagavi, M. Zhou (TI)]

JCTVC-L0282 Crosscheck report of TI's proposal JCTVC-L0229 [C. Kim, B. Jeon (LG)] [late]

## HL syntax in SHVC and 3D extensions (22)

### SHVC high-level syntax (6)

JCTVC-L0039 On SHVC RAP pictures [M. M. Hannuksela (Nokia)]

JCTVC-L0071 AHG9: Inter-layer prediction flag and inter-layer syntax prediction flag [T.-D. Chuang, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-L0111 Lightweight single-loop scalability with SHVC [H. Roodaki, K. Ugur, M. M. Hannuksela (Nokia)]

JCTVC-L0119 Adaptive resolution change with SHVC [K. Ugur, H. Roodaki, M. M. Hannuksela (Nokia)]

JCTVC-L0171 SHVC HLS: support for unequal BL and EL GOP lengths [M. M. Hannuksela, A. Hallapuro, K. Ugur (Nokia)]

JCTVC-L0231 Inter-layer slice header syntax element prediction in SHVC [D.-K. Kwon, M. Budagavi, M. Zhou (TI)]

JCTVC-L0258 Inter layer prediction indication flags and pictures management [Hendry, B. Jeon (LG), D. Kwon, M. Budagavi, M. Zhou (TI)]

### Generic high-level syntax for SHVC and 3D extensions (16)

#### Non-VPS (4)

JCTVC-L0137 Inter-layer SPS prediction for HEVC extensions [Thomas Rusert (Ericsson)]

JCTVC-L0178 Legacy base layer codec support in SHVC [J. Boyce, D. Hong, W. Jang (Vidyo)]

JCTVC-L0188 Unification of scalable and multi-view extensions with HLS only changes [K. Ugur, M. M. Hannuksela, J. Lainema, D. Rusanovskyy (Nokia)]

JCTVC-L0197 Indication of tile boundary alignment [K. Suehring, R. Skupin, T. Schierl (Fraunhofer HHI)]

#### VPS extension (12)

JCTVC-L0138 On VPS extension design [Thomas Rusert (Ericsson)]

JCTVC-L0180 Profile, tier, level and operation points signaling in the VPS extension [J. Boyce (Vidyo)]

JCTVC-L0181 Proposed VPS extension semantics and editorial cleanups to syntax [J. Boyce (Vidyo), Y.-K. Wang (Qualcomm), S. Deshpande (Sharp)]

JCTVC-L0200 On VPS extension [R. Skupin, V. George, K. Suehring, T. Schierl (Fraunhofer HHI)]

JCTVC-L0209 On Implicit Signaling of Scalability Dimension Identifiers [S Deshpande (Sharp)]

JCTVC-L0210 On Layer Dependency Signaling [S Deshpande (Sharp)]

JCTVC-L0226 VPS and vps_extension updates [M. Haque, A. Tabatabai]

JCTVC-L0246 Proposal to the Extension of Video Parameter Set [T. C. Thang (UoA), J. W. Kang, H. Lee, J. Lee, J. S. Choi (ETRI)]

JCTVC-L0262 AHG 9: Signalling of required DPB size in VPS [Hendry, B. Jeon (LG), D. Kwon, M. Budagavi, M. Zhou (TI)]

JCTVC-L0263 AHG 9: On inter-layer dependency signalling in VPS extension [Hendry, B. Jeon (LG)]

JCTVC-L0381 Parallelization indication in HEVC extensions [K. Suehring, R. Skupin, V. George, T. Schierl (Fraunhofer HHI)] [late]

JCTVC-L0390 On layer id and dimension id mapping in VPS extension [Hendry, B. Jeon (LG)] [late]

## Lossless compression (beyond version 1)

JCTVC-L0114 AHG7: Coefficient coding for lossless coding [Y. Piao, J. Min, S. Lee (Samsung)]

JCTVC-L0426 Cross-check of Samsung's proposal on coefficient coding for lossless coding (L0114) [X. Zheng, Y. Lin (Hisilicon)] [late] [miss]

JCTVC-L0117 AHG7: Residual DPCM for HEVC lossless coding [S. Lee, I.-K. Kim, C. Kim (Samsung)]

JCTVC-L0427 Cross-check of Samsung's proposal on residual DPCM for HEVC lossless coding (L0117) [X. Zheng, Y. Lin (Hisilicon)]

JCTVC-L0118 AHG7: Residual quadtree for HEVC lossless coding [T. Lee, S. Lee (Samsung)]

JCTVC-L0161 Intra prediction for lossless coding [Peter Amon, Andreas Hutter (Siemens), Andreas Weinlich, André Kaup (Univ. Erlangen)]

JCTVC-L0176 AHG7: Sample-based angular intra prediction for HEVC range extension [M. Zhou (TI)]

JCTVC-L0275 AHG7: Cross-verification report on JCTVC-L0176 entitled "Sample-based angular intra prediction for HEVC range extension" [K.Tokumitsu, K.Chono (NEC)] [late]

JCTVC-L0302 AHG8: Lossless coding test results for the 4:4:4 screen content sequences [H. Yu (Huawei), W. Gao, J. Ye, Y. Cao, X. Wang]

JCTVC-L0303 AHG8: P2M based dual-coder extension of HEVC [Tao Lin, Shuhui Wang, Peijun Zhang, Kailun Zhou (Tongji U)]

JCTVC-L0313 AHG8: P2M and P2M+SAP as lossless coding tools for screen content coding [Peijun Zhang, Tao Lin, Shuhui Wang, Kailun Zhou (Tongji U)]

JCTVC-L0362 Cross-verification of JCTVC-L0313 [M. Zhou (TI)] [late]

## Interlaced scan and field-based video coding

JCTVC-L0187 HEVC Software modifications for field based coding [Zineb AGYO, Jean-Marc THIESSE, Jérôme VIÉRON]

JCTVC-L0378 Performance evaluation of HEVC on YUV4:2:2 interlace video sources [A. Minezawa, K. Sugimoto, S. Sekiguchi (Mitsubishi)] [late]

## Non-normative: Encoder optimization, decoder speed improvement, post filtering, loss concealment, rate control

### Rate control

JCTVC-L0033 Bug fix for rate control algorithm in HM [B. Li, L. Li, H. Li (USTC)]

JCTVC-L0392 Cross-check of JCTVC-L0033 [J. Xu (Microsoft)] [late]

### Encoder optimization

JCTVC-L0034 Cleanup for motion estimation process in HM [B. Li, H. Li (USTC)]

JCTVC-L0305 Crosscheck for cleanup of ME process in HM (JCTVC-L0034) [W. Zhang, L. Xu, Y. Han, Z. Deng, X. Cai, Y. Chiu (Intel)] [late]

## Unclear allocation

## Withdrawn

JCTVC-L0120 Withdrawn

JCTVC-L0122 Withdrawn

JCTVC-L0143 Withdrawn

JCTVC-L0173 Withdrawn

JCTVC-L0186 Withdrawn

JCTVC-L0298 Withdrawn

JCTVC-L0299 Withdrawn

JCTVC-L0318 Withdrawn

# Plenary Discussions and BoG Reports

## Project development

## List of actions for text of version 1 of HEVC

Note: Purely editorial issues for aspects where the text is actually correct are not listed here.

Communication to and by parent bodies (section 3.1)

Draft text specification of version 1 (section 3.3)

- Updates in L0030?

- "[Ed" comments

Profile, level and constraint definition changes (section 3.5)

- Deblocking filter (section 6.1)

- High-level syntax (section 6.2)

- General high-level syntax cleanups  (section 6.2.1)

  - For the 16 NAL unit types, specify half of them as "prefixes" (i.e. may start a new access unit).

  - Remove bit_rate_pic_rate_info( ) from VPS

  - Change the coding of the syntax element min_spatial_segmentation_idc from u(8) to ue(v), and specify the allowed value range as 0..4095.

  - Increase the length of general_reserved_zero_16bits to 48 bits.

  - Correcting description of (forward increment only) modulo wrapping calculation for CPB removal delay.

  - Allow the post-filter hint, user data registered, user data unregistered, progressive refinement segment end, and filler payload SEI messages to be suffix SEI messages.

- NAL unit header (section 6.2.2)

- Parameter sets (section 6.2.3)

- Slices and slice header (section 6.2.4)

- High-level parallelism (section 6.2.5)

- Hypothetical reference decoder (section 6.2.6)

- Frame packing arrangement (section 6.2.7)

- SEI messages (section 6.2.8)

- Temporal MV prediction "hook" in version 1 (section 6.3)

Other cleanup topics for version 1 (section 6.4)

Impact of extension plans (RExt, SHVC, MHVC)

## BoGs

# Project planning

## WD drafting and software

The following agreement was established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the decision of the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in CEs).

Suggestions for future meetings included the following generally-supported principles:

- No review of normative contributions without WD text

- HM text strongly encouraged for non-normative contributions

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions

The document upload deadline for the next meeting was planned to be 7 Jan. 2013.

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name. Also, core experiment responsibility descriptions should name individuals, not companies. AHG reports and CE descriptions/summaries are considered to be the contributions of individuals, not companies.

## General issues for CEs and TEs

Group coordinated experiments were planned. These fell into two categories:

- "Core experiments" (CEs) are the experiments for which there is a draft design and associated test model software that have been established.

- "Tool experiments" (TEs) are the coordinated experiments on coding tools at a more preliminary stage of work than those of "core experiments".

A preliminary description of each experiment is to be approved at the meeting at which the experiment plan is established.

It is possible to define sub-experiments within particular CEs and TEs, for example designated as CEX.a, CEX.b, etc., for a CEX, where X is the basic CE number.

As a general rule, it was agreed that each CE should be run under the same testing conditions using one software codebase, which should be based on the HM software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

The general agreed common conditions for experiments were described in the output document JCTVC-K1100.

A deadline of three weeks after the meeting was established for organizations to express their interest in participating in a CE to the CE coordinators and for finalization of the CE descriptions by the CE coordinator with the assistance and consensus of the CE participants.

Any change in the scope of what technology will be tested in a CE, beyond what is recorded in the meeting notes, requires discussion on the general JCT-VC reflector.

As a general rule, all CEs are expected to include software available to all participants of the CE, with software to be provided within two (calendar) weeks after the release of the HM 9.0 software basis. Exceptions must be justified, discussed on the general JCT-VC reflector, and recorded in the abstract of the summary report.

Final CEs shall clearly describe specific tests to be performed, not describe vague activities. Activities of a less specific nature are delegated to Ad Hoc Groups rather than designated as CEs.

Experiment descriptions should be written in a way such that it is understood as a JCT-VC output document (written from an objective "third party perspective", not a company proponent perspective – e.g. referring to methods as "improved", "optimized" etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions should not contain verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in referenced documents that are also available in the JCT-VC document archive.

Those who proposed technology in the respective context (by this or the previous meeting) can propose a CE or CE sub-experiment. Harmonizations of multiple such proposals and minor refinements of proposed technology may also be considered. Other subjects would not be designated as CEs.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

It is strongly recommended to plan resources carefully and not waste time on technology that may have little or no apparent benefit – it is also within the responsibility of the CE coordinator to take care of this.

A summary report written by the coordinator (with the assistance of the participants) is expected to be provided to the subsequent meeting. The review of the status of the work on the CE at the meeting is expected to rely heavily on the summary report, so it is important for that report to be well-prepared, thorough, and objective.

A non-final CE plan document was reviewed and given tentative approval during the meeting (with guidance expressed to suggest modifications to be made in a subsequent revision).

The CE description for each planned CE is described in an associated output document JCTVC-K11xx for CExx, where "xx" is the CE number (xx = 01, 02, etc.). Final CE plans are recorded as revisions of these documents.

It must be understood that the JCT-VC is not obliged to consider the test methodology or outcome of a CE as being adequate. Good results from a CE do not impose an obligation on the group to accept the result (e.g., if the expert judgment of the group is that further data is needed or that the test methodology was flawed).

Some agreements relating to CE activities were established as follows:

- Only qualified JCT-VC members can participate in a CE.

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting.

- All software, results, documents produced in the CE should be announced and made available to all CE participants in a timely manner.

- If combinations of proposals are intended to be tested in a CE, the precise description shall be available with the final CE description; otherwise it cannot be claimed to be part of the CE.

## Alternative procedure for handling complicated feature adoptions

The following alternative procedure had been approved at a preceding meeting as a method to be applied for more complicated feature adoptions:

- Run CE + provide software + text, then, if successful,

- Adopt into HM, including refinements of software and text (both normative & non-normative); then, if successful,

- Adopt into WD and common conditions.

Of course, we have the freedom (e.g. for simple things) to skip step 2.

## Common Conditions for HEVC Coding Experiments

No particular changes were noted w.r.t. prior CTC.

Preferred Common Conditions for experiment testing that are intended to be appropriate for both CEs and other experiments were selected by the group and described in output document JCTVC-K1100.

## Software development

The software coordinator had already started integrating changes on top of the prior HM software, and proponents of adopted proposals are required to integrate their changes into the latest version, in coordination with the software coordinator, and test in this environment. All tools were planned to again be thoroughly tested after integration.

Any adopted proposals where software is not delivered by the scheduled date will be rejected.

The planned timeline for software releases was established as follows:

- HM 9.0 should be available within 2 weeks after the meeting.

- Availability of the range extensions software is expected 1 week after HM 9.0.

- HM 9.1 is planned to be available 4 weeks after HM 9.0.

## Subjective verification test plan

Subjective verification is planned to be performed after finalization of standard. Contributions and planning toward the conduct of that testing should begin ASAP.

# Establishment of ad hoc groups (to be updated)

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups will be the main JCT-VC reflector (jct-vc@lists.rwth-aachen.de).

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| JCT-VC project management (AHG1) (jct-vc@lists.rwth-aachen.de) - Coordinate overall JCT-VC interim efforts. - Report on project status to JCT-VC reflector. - Provide report to next meeting on project coordination status. | G. J. Sullivan, J.-R. Ohm (co-chairs) | N |
| HEVC draft and test model editing (AHG2) (jct-vc@lists.rwth-aachen.de) - Produce and finalize JCTVC-L1002 HEVC Test Model 10 (HM 10) Encoder Description. - Produce and finalize JCTVC-L1003 HEVC text specification Draft 10 / FDIS & Consent text. - Gather and address comments for refinement of these documents. - Coordinate with the Software development and HM software technical evaluation AhG to address issues relating to mismatches between software and text. | B. Bross, K. McCann (co-chairs), W.-J. Han, I. K. Kim, J.-R. Ohm, K. Sugimoto, G. J. Sullivan, T. Wiegand (vice-chairs) | N |
| HEVC HM software development and software technical evaluation (AHG3) (jct-vc@lists.rwth-aachen.de) - Coordinate development of the HM software and its distribution to JCT-VC members - Produce documentation of software usage for distribution with the software - Prepare and deliver HM 10.0 software version and the reference configuration encodings according to JCTVC-L1100 based on common conditions suitable for use in most core experiments (expected within 2 weeks after the meeting). - Prepare and deliver HM 10.1 software (and additional "dot" version software releases as appropriate) and appropriate software branches that include additional items not integrated into the 10.0 version (expected within four weeks after the 10.0 software release). - Perform analysis and reconfirmation checks of the behaviour of technical changes adopted into the draft design, and report the results of such analysis. - Suggest configuration files for additional testing of tools. - Coordinate with HEVC Draft and Test Model editing AhG to identify any mismatches between software and text. | F. Bossen (chair), D. Flynn, K. Sühring (vice-chairs) | N |
| HEVC Still Picture profile (AHG4) (jct-vc@lists.rwth-aachen.de) - Consider the editorial and technical maturity of the draft specificaiton of the draft Still Picture profile. - Identify test material and test conditions for study of the capabilities of the draft Still Picture profile. - Conduct tests of the capabilities of the Still Picture profile. - Study the capabilities of the HEVC range extensions for still picture coding, and compare these to the capabilities of the draft Still Picture profile. | K. Ugur (chair), J.-R. Ohm, G. J. Sullivan, A. Tabatabai (vice-chairs) |  |
| HEVC conformance test development (AHG5) (jct-vc@lists.rwth-aachen.de) - Study the requirements of HEVC conformance testing to ensure interoperability. - Discuss the work plan needed to develop HEVC conformance testing. - Study potential testing methodology to fulfil the requirements of HEVC conformance testing. - Establish and coordinate bitstream exchange activities for HEVC. - Study to develop a potential set of HEVC conformance bitstreams. | T. Suzuki (chair), C. Fogg, , W. Wan (vice-chairs) | N |
| HEVC in-loop filtering (AHG6) (jct-vc@lists.rwth-aachen.de) - Clean up and stabilize the HM software, the draft text and the HM encoder description on in-loop filtering. - Study and consider improvements to the common test conditions by signalling deblocking filter control parameters. - Identify test sequences, methodology, and test parameters for testing deblocking filter behaviour. | T. Yamakage and A. Norkin, (co-chairs) | N |
| HEVC range extensions (AHG7) (jct-vc@lists.rwth-aachen.de) - Study aspects of the technical design and develop software relating to the support of non-4:2:0 chroma formats and bit depths beyond 8 bits. - Assist and advise in the work of removing any implicit assumptions of 8-bit depth and 4:2:0 formatting from the current draft and software (where feasible). - Discuss and propose test conditions and test material for the development of the range extensions. - Analyze and consider the implications of the use of square versus non-square transforms for 4:2:2 coding. - Study techniques for color conversion and resampling and their relationship to non-4:2:0 chroma coding. | D. Flynn (chair), P. Andrivon, K. McCann, M. Mrak, C. Rosewarne, K. Sharman, K. Sugimoto, P. Topiwala, H. Yu (vice-chairs) | N |
| Screen content coding (AHG8) (jct-vc@lists.rwth-aachen.de) - Consider needs for lossless coding and screen content coding support in the range extensions. - Study (lossy and lossless) coding tools and performance of HEVC and its range extensions on screen content - Evaluated and identify test material appropriate for screen content coding - Make recommendations for test conditions for screen content coding | W. Gao (chair), M. Budagavi, R. Cohen, A. Duenas, T. Lin, J. Xu (vice-chairs) | N |
| High-level syntax (AHG9) (jct-vc@lists.rwth-aachen.de) - Study NAL unit header, video parameter set, sequence parameter set, picture parameter set, and slice header syntax designs. - Study SEI messages and VUI syntax designs, including checking and fixing texts for SEI messages currently included by referring to the AVC specification. - Study the hypothetical reference decoder (HRD) syntax and operations and the related text for bitstream conformance and decoder conformance. - Work towards simplification and general minor cleanup of the high-level syntax. - Assist in software development and text drafting for the high-level syntax in the HEVC design. | Y.-K. Wang and G. J. Sullivan (co-chairs) | N |
| SHVC tool experiments (AHG10) (jct-vc@lists.rwth-aachen.de) - Study NAL unit header, video parameter set, sequence parameter set, picture parameter set, and slice header syntax designs. Establish configurations for SHVC tool experiments - Generate anchors used for SHVC tool experiments - Create reporting sheets for for tool experiments - Provide configuration data two weeks after software is available - Discuss and identify additional issues related to SHVC tool experiments | X. Li (chair), J. Boyce, P. Onno, Y. Ye. (vice-chairs) | N |
| SHVC software (AHG11) (jct-vc@lists.rwth-aachen.de) - Prepare software (based on HM8.1) for experimentation. - Coordinate as appropriate with software development for TEC6. - Provide software to TEs within two weeks after the meeting - Discuss and identify additional issues related to software | V. Seregin (chair), T. Chuang, Y. He, D. Kwon, F. Le Leannec (vice-chairs) | N |
| SHVC upsampling and downsampling filters (AHG12) (jct-vc@lists.rwth-aachen.de) - Study alternative upsampling and downsampling filters for spatial scalability - Discuss and identify additional issues related to upsampling or downsampling filters | A. Segall (chair), E. Alshina. J. Chen, P. Topiwala, M. Zhou (vice-chairs) | N |

# Output documents (to be updated)

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate those responsible for document production.

JCTVC-L1000 Meeting Report of 12th JCT-VC Meeting [G. J. Sullivan, J.-R. Ohm]

JCTVC-H1001 HEVC software guidelines [K. Suehring, D. Flynn, F. Bossen, (software coordinators)]

(Remains valid, although from a prior meeting.)

JCTVC-L1002 High Efficiency Video Coding (HEVC) Test Model 10 (HM 10) Encoder Description [K. McCann (primary), B. Bross, W.-J. Han, I. K. Kim, K. Sugimoto, G. J. Sullivan] (WG 11 N XXXXX)

JCTVC-L1003 High Efficiency Video Coding (HEVC) text specification draft 10 (for FDIS & Consent) [B. Bross (primary), W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand] (WG 11 N XXXXX)

JCTVC-L1004 HEVC Conformance Draft 2 [T. Suzuki, G. J. Sullivan] (WG 11 N xxxxx)

JCTVC-L1005 HEVC Range Extensions Draft 2 [D. Flynn] (WG 11 N xxxxx)

JCTVC-L1006 Common test conditions and software reference configurations for HEVC range extensions [D. Flynn]

JCTVC-L1007 NAL unit header and parameter set designs for HEVC extensions [J. Boyce, Y.-K. Wang]

JCTVC-L1100 Common HM test conditions and software reference configurations [F. Bossen]

- JCTVC-K1101 Description of Tool Experiment A1: Upsampling filter for SHVC [A. Segall, J. Chen, J. Dong, E. Alshina (TE Coordinators)]

- JCTVC-K1102 Description of Tool Experiment A2: Inter-layer Texture Prediction Signaling in SHVC [L. Guo, Y. He, D. Kown, J. Zan, H. Lakshman, J. Kang (TE Coordinators)]

- JCTVC-K1103 Description of Tool Experiment B3: Combined Prediction in SHVC [X. Li, E. Francois, P. Lai, D. Kwon, A. Saxena (TE coordinators)]

- JCTVC-K1104 Description of Tool Experiment B4: Inter-layer filtering in SHVC [J. Chen, A. Segall, E. Alshina, S. Liu, J. Dong, J. Park (TE coordinators)]

- JCTVC-K1105 Description of Tool Experiment C5: Inter-layer syntax prediction using HEVC base layer [V. Seregin, P. Onno, S. Liu, T. Lee, C. Kim, H. Yang (TE coordinators)]

- JCTVC-K1106 Description of Tool Experiment C6: Inter-layer syntax prediction using AVC base layer [J. Boyce, K. Kawamura, H. Lakshman]

JCTVC-K1101 through JCTVC-K1106 were pre-reviewed only in BoG activity. A two week finalization period was authorized.

JCTVC-K1121 HEVC Range Extensions Core Experiment 1: Square and non-square transforms for 4:2:2 chroma format coding [J. Sole]

JCTVC-K1121 was pre-reviewed in the JCT-VC closing plenary. It will test three variations (all-square, all-non-square, and mixed). A two week finalization period was authorized.

# Future meeting plans, expressions of thanks, and closing of the meeting

Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (starting meetings on the Monday or Tuesday of the first week and closing it on the Tuesday or Wednesday of the second week of the SG 16 meeting), and

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (starting meetings on the Wednesday or Thursday prior to such meetings and closing it on the last day of the WG 11 meeting).

Some specific future meeting plans were established as follows:

- 17–26 Apr. 2013 under WG 11 auspices in Incheon, KR.

- 24 July – 2 Aug. 2013 under WG 11 auspices in Vienna, AT.

- 24 – 30 Oct. 2013 under ITU-T auspices in Geneva, CH.

- 8–17 Jan. 2014 under WG 11 auspices in San Jose, US.

- The agreed document deadline for the April 2013 meeting is April XX.

ITU was thanked for their excellent hosting of the 12th meeting of the JCT-VC. ITU, XXX and XXX were thanked for providing viewing equipment used at the meeting.

The JCT-VC meeting was closed at approximately 1XXX hours on Wed. 23 Jan. 2013.

# Annex A to JCT-VC report:
List of documents

# Annex B to JCT-VC report:
List of meeting participants

The participants of the eleventh meeting of the JCT-VC, according to badge pick-up records produced by the ITU staff (approximately XXX in total), were as follows: