| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 5th Meeting: Geneva, CH, 9-17 October, 2002 | Document: input.doc Filename: input.doc.doc |
| --- | --- |

| Title: | Comments on Reference Picture Buffer |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Louis Kerofsky Sharp Labs of America 5750 NW Pacific Rim Blvd. Camas WA 98607 Shijun Sun Sharp Labs of America 5750 NW Pacific Rim Blvd. Camas WA 98607 Anthony Joch 400-1788 West 5th Avenue Vancouver, B.C., Canada V6J 1P2 | Tel: Email: Tel: Email: Tel: Email: | 360-817-7644 lkerofsky@sharplabs.com 360-817-7531 ssun@sharplabs.com +1 604 737 2426 .33 anthony@ubvideo.com |
| Source: | Sharp Labs of America |  |  |

_____________________________

# Introduction

The current design of the Reference Picture Buffer contains decoded pictures. The term picture is used repeatedly and is a source of confusion.  Two fields may have been decoded individual as field-structured pictures or together in a frame-structured picture.  When referenced, these fields may be grouped to construct a reference frame-structured picture or referenced individually as field-structured pictures.  This dual use of the term picture creates confusion in implementing the reference buffer.  We clarify the description with the goal of enabling implementations using only frame-structured storage.

## Implementation Issues

An implementation typically chooses a single picture structure for storing all reference data.  For many architectures, frames are more efficient for use with frame-prediction encouraging the use of frame stores.  According to the current design, individual fields may be added or removed from the reference buffer.  Removing individual fields can fragment the reference frame memory, requiring de-fragmentation before storing a decoded frame-structured picture even when sufficient memory exists.

## Memory Use

Limits are placed on the amount of memory required for decoding by level parameter in annex A.  We clarify the use of this limit.  A level defines the total memory available, M bytes, for frame storage.  Given a memory size of M bytes and the size of a decoded frame, F bytes, the number of frame stores is limited to N where:

N=MIN(15,M/F)

During operation, this buffer will contain several decoded pictures (fields or frames).  The reference frame buffer contains N frame stores where each frame store contains: memory for storing two fields, a decoded frame number, and a flag for each field indicating the field is free or available for reference.  A frame store is free if both fields are flagged free.  A frame store is available for reference if both fields are available for reference.

frame_store each frame store includes: memory for storing two fields, a decoded frame number, and a flag for each field indicating if the field is free or available for reference.

NOTE: the semantics of num_of_ref_frames need to be changed. Currently, it specifies the total number of “pictures”.  The number of frame stores is meant.

num_of_ref_frames specifies the total number of frames stores for storing both short- and long-term pictures in the reference picture buffer.

## Picture Numbering

Numbers are assigned to decoded pictures for use in reference.  Picture can mean either field-structured picture or frame-structured picture.  In the current design, the distinction between field and frame appears to be made based upon the type of reference being used during decoding.  The type of reference being used depends upon the picture structure of the picture currently being decoded not upon the picture structure in effect when a frame/field was added to the reference buffer.  We clarify this distinction defining both frame-structured and field-structured numbering for referencing elements of the reference buffer.

Frame-structured picture numbering

For each frame store available for reference, we define a frame-structured picture number PN = FN.  Thus each pair of complimentary fields both marked available for reference can be indexed as a frame

Field-structured picture numbering

For each field marked available for reference, we define a field-structured picture number PN=2FN+parity.  Thus each field marked available for reference can be indexed.

MMCO

Each MMCO modifies the state of the reference buffer by marking one or more fields of a frame as (un)available for reference, or moving a picture from short term to long term buffer.  MMCO commands are executed in the order received with the means of indexing determined by the picture structure of the picture containing the MMCO command.

Adding

A frame-structured picture can only be stored in a free frame store, where both fields are marked free.  The frame store is updated with the frame number, FN, and both fields are marked available for reference.  A field-structured picture with frame number FN is stored in a frame store partially occupied by a complementary field, or in a free frame store. The field is marked available for reference.

### Removing

A picture, field or frame, with frame number FN is removed from the frame buffer by marking the specified field or fields free in the frame store having frame number FN.

### Transfer to long-term buffer

Transfer operations apply to entire frame stores rather than individual fields

Sliding Window

The sliding window mode uses the term “capacity” without definition.  Following our remarks, the capacity of the short term buffer is a number of frame stores equal to the total number of frame stores, N,  minus the number of long term frame stores.  When the short term buffer has reached capacity, a field-structured picture can be stored if the field is flagged free in a frame store having the same frame number and opposite parity field marked available for reference.  Otherwise, the oldest frame store is emptied, both fields marked free and the picture (field-structured or frame-structured) is stored in the now free frame store.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name |  |  |  |
| Mailing address |  |  |  |
| Country |  |  |  |
| Contact person |  |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email |  |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  |  |  |
| Title |  |  |  |
| Contribution number |  |  |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: