| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 5th Meeting: Geneva, Switzerland, 9-17 October, 2002 | Document: JVT-E063 Filename: JVT-E063.doc |
| --- | --- |

| Title: | Verification Results on Bi-predictive pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Alexis Michael Tourapis, Feng Wu, and Shipeng Li 3F Sigma Center, No.49 Zhichun Road, Haidian District, Beijing, 100080, China | Tel: Email: | (86-10) 6261-7711 ext 3108 alexismt@ieee.org |
| Source: | Microsoft Corporation |  |  |

_____________________________

In the accompanying document JVT-E063.xls we present the verification results of proposal JVT-E092doc: Proposed Modification on Bi-predictive Pictures.

Our independent results verify those of JVT-E092 and the summary is also included here :

| Ref. | Entropy | Description | Avg.Bit Saving % (excel) | Avg. PSNR gain (excel) | Max .Bit Saving % (excel) | Max. PSNR gain (excel) | Min .Bit Saving % (excel) | Min. PSNR gain (excel) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | UVLC | co-located block in most recently decoded frames. (not ref_idx_l1=0) | 5.34% | 0.248 | 9.85% | 0.434 | 1.47% | 0.067 |
| 5 | UVLC | Proposed New Direct | 11.14% | 0.647 | 27.08% | 1.965 | 1.71% | 0.078 |
| 5 | UVLC | Proposed New Direct + MB Type changes | 11.95% | 0.685 | 27.70% | 1.997 | 2.51% | 0.111 |
| 5 | CABAC | co-located block in most recently decoded frames. (not ref_idx_l1=0) | 2.73% | 0.121 | 4.46% | 0.220 | 0.67% | 0.027 |
| 5 | CABAC | Proposed New Direct | 7.28% | 0.365 | 19.26% | 1.100 | 0.69% | 0.029 |
| 5 | CABAC | Proposed New Direct + MB Type changes | 8.96% | 0.447 | 20.42% | 1.166 | 2.96% | 0.116 |