| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 5th Meeting: Geneva, CH, 9-17 October, 2002 | Document: JVT-E111 Filename: JVT-E111.doc |
| --- | --- |

| Title: | Reducing Pixel Dependencies for Intra-Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Greg Conklin 2601 Elliott Ave Seattle, WA 98101 USA | Tel: Email: | +1 206 674 2337 gregc@real.com |
| Source: | RealNetworks, Inc. |  |  |

_____________________________

1.  Introduction

This document examines the number of predictor pixels used while forming the Intra prediction modes as defined in the CD of the JVT standard.  In the 4x4 block case, it finds that three modes require significantly more predictor pixels.  Therefore, a new definition of mode 3 (“Diagonal Down/Left”), mode 7 (“Vertical-Left”) and mode 8 (“Horizontal-Up”) are proposed that reduces dependency on surrounding pixels.

These intra modes present fewer memory access problems. This is particularly important for pixels to the left of the block since these pixels raise memory caching concerns.  In addition, these new modes reduce the required computational complexity relative to the existing modes, while leaving the compression performance little changed.

2.  Number of Predictor Pels

Fig 1 shows the surrounding pixels used to form each of the 9 current prediction modes.  Only 3 of the prediction modes require use of pixels beyond the 4 directly above and 4 directly to the left.  Therefore, this proposal changes these three modes so that they use less pixels for prediction, as shown below.

Mode 3 – Diagonal Down/Left

a 	= (A + 2*B + C + 2) / 4;

b,e 	= (B + 2*C + D + 2) / 4;

c,f,i	= (C + 2*D + E + 2) / 4;

d,g,j,m	= (D + 2*E + F + 2) / 4;

h,k,n	= (E + 2*F + G + 2) / 4;

l,o	= (F + 2*G + H + 2) / 4;

p	= (G + 3*H + 2) / 4;

Mode 7 – Vertical-Left

a 	= (A + B + 1) / 2;

b,i 	= (B + C + 1) / 2;

c,j 	= (C + D + 1) / 2;

d,k 	= (D + E + 1) / 2;

l 	= (E + F + 1) / 2;

e 	= (A + 2*B + C + 2) / 4;

f,m 	= (B + 2*C + D + 2) / 4;

g,n 	= (C + 2*D + E + 2) / 4;

h,o 	= (D + 2*E + F + 2) / 4;

p 	= (E + 2*F + G + 2) / 4;

Mode 8 – Horizontal-Up

a 	= (I + J + 1) / 2;

b 	= (I + 2*J + K + 2) / 4;

c,e 	= (J + K + 1) / 2;

d,f 	= (J + 2*K + L + 2) / 4;

g,i 	= (K + L + 1) / 2;

h,j 	= (K + 2*L + L + 2) / 4;

l,n,k,

m,o,p	= L;

(a) Vertical	(b) Horizontal	(c) DC

(e) Diagonal Down/Left	(f) Diagonal Down/Right	(g) Vertical-Right

(h) Horizontal-Down	(i) Vertical-Left	(j) Horizontal-Up

Figure 1: Intra Prediction Modes and Surrounding Predictor Pixels

(a) New Diagonal Down/Left	(b) New Vertical-Left	(c) New Horizontal-Up

Figure 2: New Prediction Modes

Figure 2 depicts the new modes and the surrounding pixels used for prediction.

3.  Computational Complexity

Table 1 presents the number of operations required to form the diagonal Intra prediction modes (modes 3-8) in the current FCD.  The number of operations under the “naïve” column were found merely by counting the adds and shifts shown in the mode definition in the FCD.  However, by reusing intermediate calculations, the total number of operations can be reduced, and is calculated under the “smart” column.  Table 2 shows the reduced number of operations using the proposed modes.  The worst case number of operations drops from 42 to 33 – a 21% reduction.

TABLE 1: Number of operations to form Intra prediction modes

| Mode | Direction | Naive |  |  | Smart |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Adds | Shifts | Total | Adds | Shifts | Total |
| 3 | NE | 40 | 13 | 53 | 35 | 7 | 42 |
| 4 | SE | 21 | 14 | 35 | 22 | 7 | 29 |
| 5 | SSE | 24 | 16 | 40 | 23 | 10 | 33 |
| 6 | ESE | 26 | 16 | 42 | 23 | 10 | 33 |
| 7 | SSW | 31 | 16 | 47 | 28 | 10 | 38 |
| 8 | ENE | 31 | 16 | 47 | 28 | 10 | 38 |

TABLE 2: Number of operations to form Intra prediction modes

| Mode | Direction | Naive |  |  | Smart |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Adds | Shifts | Total | Adds | Shifts | Total |
| 3 | NE | 20 | 14 | 34 | 22 | 7 | 29 |
| 4 | SE | 21 | 14 | 35 | 22 | 7 | 29 |
| 5 | SSE | 24 | 16 | 40 | 23 | 10 | 33 |
| 6 | ESE | 26 | 16 | 42 | 23 | 10 | 33 |
| 7 | SSW | 25 | 15 | 40 | 17 | 10 | 28 |
| 8 | ENE | 15 | 9 | 24 | 10 | 6 | 16 |

4.  Coding Performance

Table 3 shows the relative PSNR and Bitrate improvement that is achieved using the proposed modes for high QPs (28-40) and low QPs (12-24).  These results were obtained by encoding each frame of the sequence as an I-Frame.  Sequences that are included in the common conditions used a FrameSkip given in those common conditions.  Otherwise, every frame in the sequence was coded (FrameSkip = 0).  CAVLC was used for entropy coding.

For high QPs, results show a range between a reduction in bitrate of 0.6% and increase in bitrate of 0.9%.  Typically, the results indicate little change in performance.  For low QPs, the result show an even less significant change in performance.

TABLE 3: Bitrate and PSNR improvement using new diagonal modes

| Sequence | Image Size | High QPs (28-40) |  | Low QPs (12-24) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | BD-PSNR | BD-Bits | BD-PSNR | BD-Bits |
| Akiyo | QCIF | 0.01 dB | -0.11% | 0.01 | -0.08% |
| Container* | QCIF | -0.00 dB | 0.04% | 0.00 | 0.00% |
| Foreman* | QCIF | -0.06 dB | 0.86% | -0.02 | 0.15% |
| Mother | QCIF | 0.03 dB | -0.57% | -0.00 | 0.01% |
| News* | QCIF | -0.00 dB | 0.03% | 0.01 | -0.08% |
| Silent* | QCIF | -0.03 dB | 0.54% | -0.01 | 0.10% |
| Akiyo | CIF | 0.00 dB | -0.07% | 0.01 | -0.22% |
| Container | CIF | 0.01 dB | -0.15% | 0.01 | -0.10% |
| Foreman | CIF | -0.04 dB | 0.65% | -0.01 | 0.09% |
| Hall | CIF | 0.00 dB | -0.02% | 0.01 | -0.14% |
| Mother | CIF | 0.01 dB | -0.17% | 0.00 | -0.03% |
| Mobile* | CIF | -0.00 dB | 0.02% | na | na |
| News | CIF | -0.00 dB | 0.05% | 0.02 | -0.22% |
| Paris* | CIF | 0.02 dB | -0.22% | 0.02 | -0.12% |
| Silent | CIF | -0.03 dB | 0.51% | -0.02 | 0.21% |
| Table | CIF | -0.01 dB | 0.14% | 0.01 | -0.06% |
| Tempete* | CIF | -0.01 dB | 0.18% | 0.00 | -0.01% |

*common conditions

5.  No Predictor Pixel Extension

There are additional performance benefits that can be realized if all of the modes used only the 4 pixels directly above and the 4 pixels directly to the left of the block for prediction (e.g. less required decoder logic, facilitation of parallel processing).  Thus, additional experiments have been conducted using the follow definitions for modes 3 and 7.

Mode 3 – Diagonal Down/Left

a 	= (A + C + I + K + 2*(B + J) + 4) / 8;

b,e 	= (B + D + J + L + 2*(C + K) + 4) / 8;

c,f,i	= (C + D + K + L + 2*(D + L) + 4) / 8;

d,g,h,j,

k,l,m,n,

o,p	= (D + L + 1) / 2;

Mode 7 – Vertical-Left

a 	= (A + B + 1) / 2;

b,i 	= (B + C + 1) / 2;

c,j 	= (C + D + 1) / 2;

e 	= (A + 2*B + C + 2) / 4;

f,m 	= (B + 2*C + D + 2) / 4;

g,n 	= (C + 2*D + D + 2) / 4;

d,h,k,

l,o,p	= D;

Table 4 shows that these modes provide a loss in visual performance of up to 3.15%.  Since this loss in quality is significant, especially relative to the potential complexity benefits, it is not recommended that these modes be used in the JVT standard.

TABLE 4: Bitrate and PSNR improvement using only the 4 pixels above

and 4 pixels to the left of the block for prediction

| Sequence | Image Size | High QPs (28-40) |  | Low QPs (12-24) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | BD-PSNR | BD-Bits | BD-PSNR | BD-Bits |
| Akiyo | QCIF | -0.12 dB | 1.60% | -0.03 dB | 0.32% |
| Container* | QCIF | -0.00 dB | 0.00% | 0.01 dB | -0.05% |
| Foreman* | QCIF | -0.10 dB | 1.51% | -0.02 dB | 0.21% |
| Mother | QCIF | -0.16 dB | 2.86% | -0.09 dB | 0.94% |
| News* | QCIF | -0.05 dB | 0.60% | 0.01 dB | -0.06% |
| Silent* | QCIF | -0.12 dB | 2.03% | -0.05 dB | 0.48% |
| Akiyo | CIF | -0.19 dB | 2.73% | -0.03 dB | 0.48% |
| Container | CIF | 0.01 dB | -0.12% | 0.01 dB | -0.07% |
| Foreman | CIF | -0.07 dB | 1.30% | -0.02 dB | 0.25% |
| Hall | CIF | -0.04 dB | 0.59% | -0.01 dB | 0.16% |
| Mother | CIF | -0.17 dB | 3.15% | -0.09 dB | 1.37% |
| Mobile* | CIF | -0.03 dB | 0.36% | na | na |
| News | CIF | -0.10 dB | 1.28% | -0.00 dB | 0.03% |
| Paris* | CIF | 0.01 dB | -0.10% | 0.01 dB | -0.06% |
| Silent | CIF | -0.12 dB | 2.59% | -0.07 dB | 0.62% |
| Table | CIF | -0.02 dB | 0.46% | 0.00 dB | -0.02% |
| Tempete* | CIF | -0.04 dB | 0.49% | 0.01 dB | -0.06% |

*common conditions

6.  Conclusion

It is proposed that Modes 4, 7 and 8 be redefined in order to reduce the number of surrounding pixels used for prediction.  This is particularly important for pixels to the left of the block since these pixels raise memory caching concerns.  By changing the definition of these three modes the worst-case computational complexity required to for the intra prediction is reduced by 30% and has a negligible impact of compression performance.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | RealNetworks, Inc. |  |  |
| Mailing address | P.O. Box 91123 Seattle, WA 98111-9223 |  |  |
| Country | USA |  |  |
| Contact person | Greg Conklin |  |  |
| Telephone | +1 206 674 2337 |  |  |
| Fax | +1 206 674 3580 |  |  |
| Email | gregc@real.com |  |  |
| Place and date of submission | Geneva, Switzerland, October 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Reducing Pixel Dependencies for Intra-Prediction |  |
| Contribution number |  | JVT-E111 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  | Gregory Conklin |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: For purposes of this contribution, the terms of 2.2.1 above [and as detailed in document VCEG-O54 submitted by Gary Sullivan] are agreed to only as necessary to comply with the required portions of a “baseline” JVT video profile.