| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 5th Meeting: Geneva, Switzerland, 9-17 October, 2002 | Document: JVT-E124 Filename: JVT-E124.doc |
| --- | --- |

| Title: | Issues on High-Level Syntax and Semantics |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Mobile Software P.O. Box 88 33721 Tampere Finland | Tel: Email: | +358 7180 73151 miska.hannuksela@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Summary

We propose a few fixes and clarifications regarding high-level syntax and semantics.

Section 2 contains an overview of the proposals. An appendix of the contribution includes the proposed changes relative to JVT-E022d7.

# Proposals

## frame_coding_only_flag

The pic_structure field of the slice header is redundant in coded sequences that are entirely frame-coded. Thus, we propose adding a frame_coding_only_flag into the sequence parameter set. If the flag is 1, only frame coding is used and the pic_structure field is absent. Consequently, 1 bit per slice header is saved. If the flag is 0, field or frame coding can be used (and switched on picture basis and possibly on macroblock pair basis). The flag also helps decoders to allocate the buffers for the decoded pictures buffer appropriately.

## POC Equal to frame_num

The FCD includes two methods of signaling picture order counts. However, in the Klagenfurt output document JVT-D145 and in early drafts of the FCD, a third method was also present. Due to some misunderstandings and communication problems, the method was dropped from the published FCD, though. The method is targeted for applications where pictures are always decoded in the order they are displayed. Consequently, the frame number of a picture can be used as its picture order count. We propose adding the method to the standard text in order to save bits in slice headers.

## Restrictions on Slice Header Parameter Values

The FCD does not specify which slice header parameter shall remain unchanged in all the slices of a coded primary or redundant picture. We propose that it is clearly stated that pic_parameter_set_id, frame_num, pic_structure, pic_order_cnt, delta_pic_order_cnt, sp_switch_flag, and slice_group_change_cycle shall remain unchanged in all the slices of a picture. Moreover, the contents of ref_pic_buf_management( ) shall be unchanged. The rest of the parameters, i.e., first_mb_in_slice, slice_type_idc, redundant_pic_cnt, direct_spatial_mv_flag, num_ref_idx_override_flag, num_ref_idx_l0_active_minus1, num_ref_idx_l1_active_minus1, slice_qp_delta, slice_qp_s_delta, disable_deblocking_filter_flag, slice_alpha_c0_offset_div2, and slice_beta_offset_div2 may change in the slice headers of a picture. The contents of ref_idx_reordering( ) and pred_weight_table( ) may also change.

The slices of a redundant picture shall have the same value of pic_structure as the corresponding primary picture. The frame_num of a redundant picture shall be the same as the frame_num in the primary picture, or both the redundant picture and the primary picture shall issue a reset memory management control operation. The decoded PicOrderCnt of a redundant picture shall be the same as for the corresponding primary picture.

## IDR Picture Is Always Stored

The FCD does not specify clearly if an IDR picture shall always be a stored (reference) picture. We propose stating this clearly. An IDR picture cannot be a non-stored (non-referenced) picture, because it contains an implicit reset operation for the decoded pictures buffer. As one of the characteristics of non-stored pictures is that they can be disposed without decoding, a coded non-stored picture cannot include memory management control operations, as stated in section 7.4.3.2 of the FCD. (Otherwise, the disposal of a non-stored picture would cause a buffer content mismatch between the encoder and the decoder.)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Nokia Corporation |  |  |
| Mailing address |  |  |  |
| Country |  |  |  |
| Contact person |  |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email |  |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Issues on High-Level Syntax and Semantics |  |
| Contribution number |  | JVT-E124 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| X | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  | The submitter is not aware of having any granted, pending, or planned patents associated with the proposed changes into the draft Recommendation ¦ Standard. |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. X |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: