| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 5th Meeting: Geneva, Switzerland, 9-17 October, 2002 | Document: JVT-E125 Filename: JVT-E125.doc |
| --- | --- |

| Title: | Detection of Coded Picture Boundaries |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Mobile Software P.O. Box 88 33721 Tampere Finland | Tel: Email: | +358 7180 73151 miska.hannuksela@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Summary

The contribution proposes solutions to get rid of the shortcomings in detection of coded picture boundaries.

The contribution is categorized as follows: Section 2 describes the problems and gives an overview of the proposed solutions. Section 3 contains the proposed change-marked additions relative to JVT-E022d7.

# Overview of the Proposal

Section 8.3.1 of the FCD specifies the detection of coded picture boundaries as follows:

Decoding of a new picture is started from the slice to be decoded, herein called the current slice, if the slice is not a redundant slice and if one of the following conditions is true:

- The frame number of the current slice is different from the frame number of the previously decoded slice.

- The frame number of the current slice is the same as frame number of the previously decoded slice, the nal_storage_idc of the previously decoded slice is zero, and the nal_storage_idc of the current slice is non-zero.

- The frame number of the current slice is the same as frame number of the previously decoded slice, pic_order_cnt_type is 0, and pic_order_cnt is different from the pic_order_cnt in the previously decoded slice.

- The frame number of the current slice is the same as the previously decoded slice, pic_order_cnt_type is 1 and delta_pic_order_cnt is different from the delta_pic_order_cnt in the previously decoded slice.

We found that there are two bugs in section 8.3.1:

- The rules in section 8.3.1 do not detect the start of a field picture which is the latter field of a frame. (The frame number, pic_order_cnt, and delta_pic_order_cnt of both the fields of a frame are the same.)

- The rules in section 8.3.1 do not detect the coded picture boundary between two IDR pictures that occur consecutively in the NAL unit stream. (The frame number, pic_order_cnt, and delta_pic_order_cnt of an IDR picture are always 0.)

We propose the following bug fixes to overcome the shortcomings:

- Add a rule in section 8.3.1 that detects a coded picture boundary if the pic_structure field of the slice header changes.

- Add a sub-sequence identifier (subseq_id) into the slice header. The subseq_id is present only in IDR slices. The subseq_id in consecutive IDR frames shall differ from each other. Furthermore, add another rule in section 8.3.1 that detects a coded picture boundary between two consecutive IDR pictures based on subseq_id.

# Changes relative to jvt-e022D7

The additions relative to JVT-E022D7 are highlighted with yellow background.

## Section 7.3.3 (Slice header syntax)

| slice_header( ) { | Category | Descriptor |
| --- | --- | --- |
| pic_parameter_set_id | 4 | ue(v) |
| frame_num | 4 | u(v) |
| pic_structure | 4 | ue(v) |
| first_mb_in_slice | 4 | u(v) |
| slice_type_idc | 4 | ue(v) |
| if( nal_unit_type = = 0x5) /* Coded slice of an IDR picture */ |  |  |
| subseq_id | 4 | ue(v) |
| … |  |  |

## Section 7.4.3 (Slice header semantics)

…

subseq_id labels an IDR picture. The values of subseq_id in all the slices of an IDR picture shall remain unchanged. The values of subseq_id in consecutive IDR pictures in decoding order shall differ from each other. The value of subseq_id shall be in the range of 0 to 65534, inclusive.

…

## Section 8.3.1 (Detection of coded picture boundaries)

Decoding of a new picture is started from the slice to be decoded, herein called the current slice, if the slice is not a redundant slice and if one of the following conditions is true:

- The frame number of the current slice is different from the frame number of the previously decoded slice.

- The frame number of the current slice is the same as frame number of the previously decoded slice, the nal_storage_idc of the previously decoded slice is zero, and the nal_storage_idc of the current slice is non-zero.

- The frame number of the current slice is the same as frame number of the previously decoded slice, pic_order_cnt_type is 0, and pic_order_cnt is different from the pic_order_cnt in the previously decoded slice.

- The frame number of the current slice is the same as the previously decoded slice, pic_order_cnt_type is 1 and delta_pic_order_cnt is different from the delta_pic_order_cnt in the previously decoded slice.

- The pic_structure of the current slice differs from the pic_structure of the previously decoded slice.

- Both the current slice and the previously decoded slice belong to an IDR picture, and the subseq_id of the current slice differs from the subseq_id of the previously decoded slice.

## Section D.3.13 (Sub-sequence information SEI message semantics)

…

subseq_id identifies the sub-sequence within a layer. Consecutive sub-sequences within a particular layer in decoding order shall have a different subseq_id from each other. If the target picture resides in sub-sequence layer 0, the value of subseq_id shall be the same as the value of subseq_id in the slice header of the most recent IDR picture in decoding order. The value of subseq_id shall be in the range of 0 to 65534, inclusive.

…

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Nokia Corporation |  |  |
| Mailing address |  |  |  |
| Country |  |  |  |
| Contact person |  |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email |  |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Detection of Coded Picture Boundaries |  |
| Contribution number |  | JVT-E125 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  | The submitter is not aware of having any granted, pending, or planned patents associated with the proposed changes into the draft Recommendation ¦ Standard. |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. X |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: