| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 5th Meeting: Geneva, Switzerland 9-17 October, 2002 | Document: JVT-E126 Filename: JVT-E126.doc |
| --- | --- |

| Title: | Clarification Regarding Sub-Sequence SEI Messages |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dong Tian TICSP Tampere Univ. of Technology P.O. Box 553 FIN-33101 Tampere Finland Miska M. Hannuksela Nokia Mobile Software P.O. Box 68 33721 Tampere Finland | Tel: Email: Tel: Email: | +358 3 3115 3853 tian@cs.tut.fi +358 7180 73151 miska.hannuksela@nokia.com |
| Source: | Tampere University of Technology and Nokia Corporation |  |  |

_____________________________

# summary

We found a couple of ambiguities and shortcomings in the syntax and semantics of the sub-sequence SEI messages and propose solutions for the found problems.

The contribution is organized as follows: Section 2 introduces the sub-sequence concept, gives examples how to use the sub-sequence SEI messages, and comments on the maturity of feature. Section 3 describes the found problems in the FCD and summarizes the proposed solutions. An appendix to this contribution includes the proposed change-marked standard text relative to JVT-E022d7.

# Introduction

## Background

The concept of enhanced GOP introduced in JVT-B042 organizes pictures in sub-sequence layers and sub-sequences. Sub-sequence layers are organized hierarchically based on their dependency on each other so that sub-sequence layer 0 can be independently decoded and sub-sequence layer N can be decoded if layers 0, 1, …, N-1 are also decoded. A sub-sequence represents a number of pictures that can be disposed without affecting the decoding of any other sub-sequence in the same or any lower sub-sequence layer. Each picture belongs to one sub-sequence layer and sub-sequence.

JVT-D098 proposed the signaling of enhanced GOP information as Supplemental Enhancement Information (SEI) messages and was adopted. Three enhanced GOP related SEI messages are included in the FCD: sub-sequence information SEI message, sub-sequence layer characteristics SEI message, and sub-sequence characteristics SEI message.

## Usage Scenarios

The sub-sequence SEI messages can be used for the following purposes:

- Increased decoding and playback rate / fast forward trick mode. Figure 1 shows an example of a GOP where pictures are categorized into three sub-sequence layers. Boxes indicate frames, letters in boxes indicate frame types, numbers in boxes indicate frame numbers, dashed ellipses indicate sub-sequences, vertical alignment of boxes indicate the sub-sequence layer which the picture belongs to, arrows indicate prediction dependencies, and display time runs from left to right. A 7.5 Hz, 15 Hz, or 30 Hz picture rate can be achieved by decoding layers 1, 2, or all 3 layers respectively. If faster-than-real-time decoding is required, the decoder can choose the number of decoded sub-sequence layers. Returning to normal full-picture-rate decoding can happen after any layer 0 picture.

Figure 1. An example of 3-layer sub-sequence structure.

- Computational scalability in decoders. Sometimes there is no file or stream suitable for decoder’s computational capabilities available, and therefore a file that cannot be decoded in real-time has to be used. If the file were encoded using non-stored pictures and/or sub-sequences, the file can be temporally subsampled in coded domain and decoded in real-time. Similarly, in multitasking systems the processing power is shared between multiple processes, and therefore the performance of the video decoder may fluctuate depending on the number and complexity of other tasks executed simultaneously.

- A video communication gateway may sometimes have to tune the bit-rate of a coded video stream. The abilities of bit-rate control methods for coded data are limited. Thus, it is advantageous to carry enhanced GOP information in systems where bit-rate control of coded video is desired. Gateways may decide to forward only certain layers or they may decide to discard one sub-sequence to drop the bit-rate.

## Compatibility with the Rest of the JVT Codec

There is no mandatory decoder operation for any SEI message. Thus, the decoder may also ignore any received sub-sequence SEI message.

The sub-sequence feature has been implemented to the JM software. Its operation has been proved by discarding sub-sequences from coded streams and decoding the manipulated streams after that.

The values of HRD parameters for a coded stream containing sub-sequences are coded according to the full-rate stream. If non-stored pictures and/or sub-sequences are disposed from the stream, the stream still complies with the given HRD parameters, as the arrival and removal times of the remaining pictures remain unchanged.

# Proposal

## Signaling the Start of Sub-Sequence

Each picture in a sub-sequence may be accompanied with the sub-sequence information SEI message. The sub-sequence information SEI message tells the decoder which layer and which sub-sequence the target picture belongs to.

To help the decoder distinguish the two sub-sequences, last_picture_flag is defined in the sub-sequence information SEI message. However, streaming servers may apply rate-shaping techniques that discard pictures starting from the end of the prediction chain of a sub-sequence and discard only of part of the sub-sequence [1]. Consequently, if SEI messages are pre-encoded, the sub-sequence information SEI message containing a last_picture_flag equal to 1 may never be transmitted.

In the JVT FCD, the first stored picture in a sub-sequence has stored_frame_cnt equal to 0. However, stored_frame_cnt equal to 0 may also occur when the stored_frame_cnt wraps over to 0 due to the modulo MAX_FN operation used when incrementing the stored_frame_cnt. Thus stored_frame_cnt cannot be used to detect the beginning of a sub-sequence.

In order to detect the beginning of a sub-sequence robustly, we propose adding two flags first_stored_picture_flag and leading_non_stored_picture_flag to the subseq_information SEI message. If the target picture is the first stored picture in the sub-sequence, first_stored_picture_flag equals to 1. Otherwise, first_stored_picture_flag is 0. If the target picture is a non-stored picture preceding any stored picture in decoding order within the sub-sequence, leading_non_stored_picture_flag equals to 1. Otherwise leading_non_stored_picture_flag is 0.

We also propose that an IDR picture always starts a new sub-sequence in sub-sequence layer 0. An IDR picture cannot be included in higher sub-sequence layers, because it inherently resets the decoded pictures buffer. Furthermore, it makes no sense to include an IDR picture in an earlier sub-sequence, because it does not have any prediction dependencies on other pictures.

## Duration in Bit-Rate Calculations

The sub-sequence layer characteristics SEI message and the sub-sequence characteristics SEI message include the average_bit_rate field specified as follows:

average_bit_rate gives the average bit rate in units of 1000 bits per second. All NAL units in the range of sub-sequence layers specified above are taken into account in the calculation. The average bit rate is calculated according to the decoding time of the NAL units. Value zero means an unspecified bit rate.

JVT-E022d7 contains the following editor’s note regarding the field: “[Ed. Note: It is unclear a) how bit rate is specified (i.e. how is the duration derived in order to compute rate from a number of bits), and b) what is the intended use of this bit rate.]”

We propose the following clarifications:

- Instead of “the decoding time of the NAL units”, we propose referring to the removal time specified according to the HRD description (section C.1.1.2 of the FCD).

- As the average bit-rate may not be known exactly when coding the SEI message, we propose adding the accurate_statistics_flag to the syntax. The flag shall be 1, if the average_bit_rate and the average_frame_rate are exactly correct. The flag shall be 0, if the average_bit_rate and the average_frame_rate may deviate from the actual values.

## Encoding Constraints

As entire sub-sequences may be deleted, memory management control operations on sub-sequence layers 1 and above must be disallowed. (A similar restriction is specified for non-stored pictures in section 7.4.3.2 of the FCD.)

If sub-sequence SEI messages are used, the required_frame_num_update_behaviour_flag shall be 1 to allow gaps in frame numbers without inferring accidental frame losses (see JVT-B042 for details).

# References

- Sang H. Kang and Avideh Zakhor, "Packet Scheduling Algorithm for Wireless Video Streaming", PV2002

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Nokia Corporation |  |  |
| Mailing address |  |  |  |
| Country |  |  |  |
| Contact person |  |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email |  |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Clarification Regarding Sub-Sequence SEI Messages |  |
| Contribution number |  | JVT-E126 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. X |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: