| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 5th Meeting: Geneva, CH, 9-17 October, 2002 | Document: JVT-E127 Filename: JVT-E127.doc |
| --- | --- |

| Title: | Cross Verification Results of Intra Prediction Enhancements |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Dong Tian and Ye-Kui Wang TICSP Tampere Univ. of Technology P.O. Box 553 FIN-33101 Tampere Finland | Tel: Email: | +358 3 3115 3853 tian@cs.tut.fi +358 3 3115 3972 wyk@ieee.org |
| Source: | Tampere University of Technology |  |  |

_____________________________

# summary

This contribution presents the cross verification results of the intra prediction enhancements proposed in JVT-E050 and JVT-E051.

Proposed agenda category: Improvements and enhancements

Keywords for agenda allocation: Intra prediction, chroma intra prediction, luma intra prediction

# Cross Verification of jvt-e050

In this section, the chroma intra prediction method proposed in JVT-E050 is tested.

## Test conditions

The common test condition is used except that all frames are coded as intra frame. Both CABAC and UVLC is tested with QP = 28, 32, 36, 40. The Bjontegaard delta bit-rate is calculated for U and V component separately. In the table below, the negative sign means that the bit-rate is reduced.

## Results

|  |  | Container | Foreman | Mobile | News | Paris | Silent | Tempete | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| U | CABAC | -1.43% | -1.53% | -0.30% | -1.15% | -1.67% | -1.90% | -1.15% | -1.30% |
|  | UVLC | -3.60% | -3.87% | -0.64% | -1.85% | -2.38% | -3.33% | -2.25% | -2.56% |
| V | CABAC | -3.30% | -2.46% | -0.15% | -2.58% | -2.12% | -3.37% | -1.36% | -2.19% |
|  | UVLC | -6.10% | -4.16% | -0.55% | -4.23% | -2.36% | -2.46% | -2.96% | -3.26% |

# Cross Verification of jvt-e051

In this section, the proposed methods in section 2.1 and 2.2 of JVT-E051 are used in the testing at the same time.

## Test conditions

The common test condition is used except that all frames are coded as intra frame. Both CABAC and UVLC is tested with low QP = 12, 16, 20, 24 and high QP = 28, 32, 36, 40. The Bjontegaard delta bit-rate is calculated for Y component. In the table below, the positive sign means that the bit-rate is increased.

## Results

Total intra 4x4 prediction enhancement (section 2.1 and 2.2 in JVT-E051 are combined)

|  | QP | Container | Foreman | Mobile | News | Paris | Silent | Tempete | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CABAC | Low | 0.07% | 0.46% | 0.13% | 0.07% | 0.06% | 0.56% | 0.11% | 0.21% |
|  | High | 0.38% | 1.45% | 0.49% | 0.64% | 0.29% | 1.67% | 0.40% | 0.76% |
| UVLC | Low | 0.12% | 0.31% | 0.05% | 0.06% | 0.04% | 0.52% | 0.04% | 0.16% |
|  | High | 0.22% | 1.82% | 0.49% | 0.81% | 0.35% | 2.86% | 0.64% | 1.03% |

# Conclusions

Our testing results support the corresponding results in JVT-E050 and JVT-E051. The slight difference for Foreman blames to the inconsistency of sequences.