| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 5th Meeting: Geneva, CH, 9-17 October, 2002 | Document: JVT-E142 Filename: JVT-E142.doc |
| --- | --- |

| Title: | Cross-verification of Reduced Sample Dependency Intra (JVT-E111) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Anthony Joch UB Video Inc. 400-1788 West 5th Ave Vancouver, BC, V6J 1P2, Canada | Tel: Email: | +1 604 737 2424 .33 anthony@ubvideo.com |
| Source: | UB Video Inc. |  |  |

_____________________________

This document provides cross-verification of the proposed method presented in JVT-E111, “Reducing Pixel Dependencies for Intra Prediction”. Source code provided by the proponent was verified and run for the common conditions sequences at QP 28-40 with all frames intra-coded. Results for the current intra prediction (“Ref”) and the proposed prediction that eliminates sample dependencies from the bottom and left of the current block (“E111”) are presented in the table below. The results are consistent with those presented in JVT-E111 showing negligible impact on coding efficiency.

| Sequence | frames | QP | Ref PSNR | Ref Bitrate | E111 PSNR | E111 Bitrate | BD-Bitrate | BD-PSNR [db] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| container | 100 | 28 | 37.09 | 252.01 | 37.09 | 252.02 | 0.03% | -0.002 |
|  |  | 32 | 34.25 | 170.96 | 34.26 | 171.10 |  |  |
|  |  | 36 | 31.48 | 112.76 | 31.48 | 112.89 |  |  |
|  |  | 40 | 28.83 | 74.94 | 28.84 | 75.15 |  |  |
| foreman | 100 | 28 | 36.76 | 251.48 | 36.77 | 252.69 | 0.86% | -0.056 |
|  |  | 32 | 33.84 | 166.54 | 33.84 | 167.65 |  |  |
|  |  | 36 | 31.15 | 107.62 | 31.14 | 108.72 |  |  |
|  |  | 40 | 28.61 | 71.13 | 28.61 | 71.96 |  |  |
| news | 100 | 28 | 37.65 | 269.44 | 37.67 | 269.32 | 0.03% | -0.001 |
|  |  | 32 | 34.47 | 190.96 | 34.48 | 191.60 |  |  |
|  |  | 36 | 31.40 | 130.95 | 31.46 | 131.76 |  |  |
|  |  | 40 | 28.62 | 89.32 | 28.63 | 89.68 |  |  |
| silent | 150 | 28 | 36.33 | 409.03 | 36.35 | 411.82 | 0.45% | -0.025 |
|  |  | 32 | 33.37 | 264.33 | 33.38 | 266.34 |  |  |
|  |  | 36 | 30.83 | 166.47 | 30.85 | 167.50 |  |  |
|  |  | 40 | 28.40 | 106.08 | 28.39 | 106.42 |  |  |
| paris | 150 | 28 | 36.58 | 2125.30 | 36.58 | 2121.46 | -0.22% | 0.017 |
|  |  | 32 | 33.40 | 1494.70 | 33.41 | 1491.69 |  |  |
|  |  | 36 | 30.39 | 1006.24 | 30.40 | 1005.93 |  |  |
|  |  | 40 | 27.55 | 674.71 | 27.55 | 674.41 |  |  |
| mobile | 300 | 28 | 35.34 | 7193.22 | 35.33 | 7190.96 | 0.02% | -0.002 |
|  |  | 32 | 31.71 | 5214.18 | 31.71 | 5213.16 |  |  |
|  |  | 36 | 28.39 | 3563.30 | 28.39 | 3564.20 |  |  |
|  |  | 40 | 25.42 | 2383.83 | 25.42 | 2386.54 |  |  |
| tempete | 260 | 28 | 36.11 | 4620.12 | 36.11 | 4623.38 | 0.13% | -0.009 |
|  |  | 32 | 32.74 | 3177.48 | 32.74 | 3180.84 |  |  |
|  |  | 36 | 29.67 | 2069.74 | 29.68 | 2074.96 |  |  |
|  |  | 40 | 26.92 | 1312.78 | 26.92 | 1317.62 |  |  |
| Average |  |  |  |  |  |  | 0.19% | -0.011 |