| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 6th Meeting: Awaji, Island, JP, 5-13 December, 2002 | Document: JVT-F049r1 Filename: JVT-F049r1.doc |
| --- | --- |

| Title: | Corrections on Sub-Sequence SEI Messages |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Mobile Software P.O. Box 88 33721 Tampere Finland Ye-Kui Wang and Dong Tian TICSP Tampere Univ. of Technology P.O. Box 553 FIN-33101 Tampere Finland | Tel: Email: | +358 7180 73151 miska.hannuksela@nokia.com +358 3 3115 3972 wyk@ieee.org +358 3 3115 3853 tian@cs.tut.fi |
| Source: | Nokia Corporation and Tampere University of Technology |  |  |

_____________________________

# Summary

This contribution proposes solutions for three issues related to sub-sequence SEI messages.

Firstly, we present a problem with the non-specified more_sei_payload_data() function that is used in the sub-sequence SEI messages. The proposed solution replaces calls to more_sei_payload_data() with small syntax and semantics changes.

Secondly, the accurate_statistics_flag signals whether the average bitrate and frame rate conveyed in the sub-sequence layer characteristics SEI message are based on measured statistics. The contribution suggests that one accurate_statistics_flag is present for all sub-sequence layers collectively instead of having one accurate_statistics_flag per each sub-sequence layer.

Thirdly, in the sub-sequence characteristics SEI message, prediction depencies are signaled for each sub-sequence. However, the two syntax elements ref_subseq_layer_num and ref_subseq_id are not enough to identify the sub-sequence. We propose to add one more syntax element to solve the problem.

Section 2 describes the problems we are trying to solve, and presents and justifies the chosen solutions. The accompanying file highlights the proposed changes relative to the draft standard (JVT-E146d37).

# Overview of the Proposal

The sub-sequence SEI messages (subclauses D.2.11 to D.2.13 and D.3.11 to D.3.13 of the draft standard JVT-E146d37) use the more_sei_payload_data() function that is unspecified.

If an SEI payload does not naturally end to a byte boundary, padding is added as follows (subclause D.2):

| if( !byte_aligned( ) ) { |  |  |
| --- | --- | --- |
| bit_equal_to_one /* equal to 1 */ | 7 | f(1) |
| while( !byte_aligned( ) ) |  |  |
| bit_equal_to_zero /* equal to 0 */ | 7 | f(1) |
| } |  |  |

more_sei_payload_data() is supposed to return true if there are unread bits left in the SEI payload. Due to the fact that bit_equal_to_one is not always a part of the SEI payload, there is no general way to decide which bits are padding and which are really SEI payload data. Thus, more_sei_payload_data() would return true also if there are only padding bits left in the payload. Note that PayloadSize counts the size in bytes. It can't tell whether there is something shorter than a byte left in the payload or whether it is just padding.

One solution to the problem would be to require usage of bit_equal_to_one always. However, this would be undesirable in SEI payloads that are naturally byte-oriented. Another solution, which we picked, is to modify the SEI payloads so that more_sei_payload_data() is not used. In practice, we propose a subseq_frame_num_flag to be included in the sub-sequence information SEI message. The flag signals whether the subseq_frame_num syntax element is present in the message. Moreover, we propose the addition of num_subseq_layers_minus1 into the sub-sequence layer characteristics SEI message. The parameter indicates the number of layers that the message concerns.

The accurate_statistics_flag is present for each layer in the sub-sequence information SEI message and signals whether the indicated layer characteristics are based on real statistics. We thought that once accurate statistics are obtained, they are obtained for all layers. Vice versa, when accurate statistics are not available for one layer, it is highly likely that the statistics are not present for any of the layers. Thus, we propose moving the accurate_statistics_flag outside the loop so that it concerns all layers collectively.

In the sub-sequence characteristics SEI message, two syntax elements, ref_subseq_layer_num and ref_subseq_id, are used to signal the prediction depencies of a sub-sequence. However, since ref_subseq_id maybe identical for more than one sub-sequence in the same layer, the signaled sub-sequences may not be uniquely identified.

We propose to add one more syntax element, ref_subseq_id_idc, to help uniquely identify a sub-sequence used as reference by the SEI target sub-sequence. ref_subseq_id_idc is a signed integer. If ref_subseq_id_idc is a non-negative interger n, the sub-sequence used as a reference for this sub-sequence is the (n+1)th closest sub-sequence among those whose first picture precedes the first picture of this sub-sequence in decoding order and which resides in the indicated layer and has the indicated sub-sequence identifier. If ref_subseq_id_idc is a negative integer whose absolute value is n, the sub-sequence used as a reference for this sub-sequence is the n th closest sub-sequence among those whose first picture succeeds the first picture of this sub-sequence in decoding order and which resides in the indicated layer and has the indicated sub-sequence identifier.

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Nokia Corporation |  |  |
| Mailing address |  |  |  |
| Country |  |  |  |
| Contact person |  |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email |  |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Corrections on Sub-Sequence SEI Messages |  |
| Contribution number |  | JVT-F049 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| X | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

X

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: