| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 7th Meeting: Pattaya II, Thailand, 7-14 March, 2003 | Document: JVT-G020 Filename: JVT-G020.doc |
| --- | --- |

| Title: | Verification of JVT-G012 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Chong-Soon Lim Block 1022 Tai Seng Avenue #04-3530 Singapore 534415 Sheng-Mei Shen Block 1022 Tai Seng Avenue #04-3530 Singapore 534415 | Tel: Email: Tel: Email: | (65) 6550-5487 cslim@psl.com.sg (65) 6550-5466 shen@psl.com.sg |
| Source: | Panasonic Singapore Laboratories Pte Ltd |  |  |

_____________________________

# Purpose

- In document JVT-G012, an adaptive basic unit layer rate control is proposed.  For details, see JVT-G012.  This document provides a cross-verification of some of the results presented in JVT-G012.

# Test Sequences and Conditions

- A sample of the results presented in JVT-G012 will be verified in this document.  Table 1 shows the test cases used in the verification.  In this document, the Constant bit rate (CBR) results are cross-verified.

| Test sequence | Size | Frame rate | Frames to be encoded | Frame Type | QP range | Search range | GOP length | Number B frame | Skip |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Silent | QCIF | 15 | 50 | IPBBP | 28-40 | 32 | 10 | 2 | 5 |
| Foreman | QCIF | 15 | 50 | IPBBP | 28-40 | 32 | 10 | 2 | 5 |
| News | QCIF | 15 | 50 | IPBBP | 28-40 | 32 | 10 | 2 | 5 |
| Container | QCIF | 15 | 50 | IPBBP | 28-40 | 32 | 10 | 2 | 5 |
| Paris | CIF | 15 | 50 | IPBBP | 28-40 | 32 | 10 | 2 | 5 |
| Tempete | CIF | 30 | 80 | IPBBP | 28-40 | 32 | 10 | 2 | 2 |

Table 1

| MV resolution | 1/4 pel |
| --- | --- |
| Hadamard | ON |
| RD optimization | ON |
| Search Range | 32(QCIF,CIF), ±64(HDTV) |
| Restrict Search Range | 2 |
| Reference Frames | 1 |
| Symbol Mode | CABAC |
| GOP structure | IBBPBB |
| IntraPeriod | 10 |

Table 2	Test Conditions

# Results

|  | JM 6.1 |  |  |  | JVT-G012 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PSNRY (dB) | PSNRU (dB) | PSNRV (dB) | Total bits | PSNRY (dB) | PSNRU (dB) | PSNRV (dB) | Total bits |
| Container (QCIF) | 36.411 | 41.246 | 41.051 | 30359 | 36.85 | 42.253 | 42.038 | 30080 |
| Foreman (QCIF) | 36.051 | 40.393 | 41.2 | 88234 | 36.173 | 40.815 | 41.602 | 88325 |
| News (QCIF) | 37.075 | 40.286 | 40.742 | 60127 | 37.721 | 41.386 | 41.916 | 60037 |
| Paris (CIF) | 35.762 | 39.093 | 39.247 | 339860 | 36.678 | 40.402 | 40.637 | 342305 |
| Silent (QCIF) | 35.963 | 38.837 | 39.856 | 63504 | 36.812 | 40.095 | 41.021 | 63554 |
| Tempete (CIF) | 34.614 | 37.26 | 39.059 | 1048728 | 34.898 | 38.152 | 39.856 | 1045813 |

Table 3 	Results at QP 28

|  | JM 6.1 |  |  |  | JVT-G012 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PSNRY(dB) | PSNRU(dB) | PSNRV(dB) | Total bits | PSNRY(dB) | PSNRU(dB) | PSNRV(dB) | Total bits |
| Container (QCIF) | 33.695 | 39.682 | 39.299 | 17088 | 34.192 | 40.394 | 40.117 | 16872 |
| Foreman (QCIF) | 33.226 | 39.166 | 39.721 | 52606 | 33.309 | 39.203 | 39.827 | 52858 |
| News (QCIF) | 33.995 | 38.599 | 39.131 | 37369 | 34.778 | 39.419 | 39.89 | 37509 |
| Paris (CIF) | 32.701 | 37.242 | 37.477 | 199066 | 33.542 | 38.16 | 38.369 | 200735 |
| Silent (QCIF) | 33.059 | 37.082 | 38.543 | 37244 | 33.568 | 37.728 | 38.972 | 37388 |
| Tempete (CIF) | 31.492 | 35.719 | 37.751 | 492833 | 31.841 | 36.302 | 38.234 | 492638 |

Table 4	Results at QP 32

|  | JM 6.1 |  |  |  | JVT-G012 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PSNRY(dB) | PSNRU(dB) | PSNRV(dB) | Total bits | PSNRY(dB) | PSNRU(dB) | PSNRV(dB) | Total bits |
| Container (QCIF) | 31.062 | 37.736 | 37.605 | 10232 | 31.462 | 38.535 | 38.086 | 10313 |
| Foreman (QCIF) | 30.55 | 37.758 | 38.072 | 31220 | 30.694 | 38.113 | 38.368 | 31917 |
| News (QCIF) | 31.078 | 36.416 | 37.349 | 22712 | 31.781 | 37.55 | 38.366 | 22831 |
| Paris (CIF) | 29.797 | 35.299 | 35.639 | 112958 | 30.447 | 36.038 | 36.441 | 114413 |
| Silent (QCIF) | 30.459 | 35.34 | 37.09 | 21190 | 31.033 | 36.2 | 37.807 | 21269 |
| Tempete (CIF) | 28.828 | 34.134 | 36.346 | 242672 | 29.202 | 34.632 | 36.834 | 243680 |

Table 5	Results at QP 36

|  | JM 6.1 |  |  |  | JVT-G012 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PSNRY(dB) | PSNRU(dB) | PSNRV(dB) | Total bits | PSNRY(dB) | PSNRU(dB) | PSNRV(dB) | Total bits |
| Container (QCIF) | 28.68 | 36.882 | 36.512 | 6641 | 28.781 | 36.868 | 36.666 | 6716 |
| Foreman (QCIF) | 27.956 | 36.789 | 36.832 | 19225 | 27.744 | 36.773 | 36.598 | 19112 |
| News (QCIF) | 28.347 | 35.055 | 36.163 | 13894 | 28.881 | 35.721 | 36.72 | 13856 |
| Paris (CIF) | 27.107 | 33.792 | 34.345 | 64368 | 27.638 | 34.299 | 34.847 | 64859 |
| Silent (QCIF) | 28.025 | 34.471 | 36.278 | 11960 | 28.48 | 34.813 | 36.571 | 12102 |
| Tempete (CIF) | 26.364 | 33.028 | 35.325 | 127355 | 26.78 | 33.32 | 35.706 | 129104 |

Table 6	Results at QP 40

# Conclusion

Some of the results in the document JVT-G012 have been verified in this document.  The PSNR results are not exactly the same as compared to JVT-G012 but they are very close to the results shown in JVT-G012.  The difference in the PSNR values may be due to rounding of the decimal values.

(Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name |  |  |  |
| Mailing address |  |  |  |
| Country |  |  |  |
| Contact person |  |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email |  |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  |  |  |
| Title |  |  |  |
| Contribution number |  |  |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: