INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE NORMALISATION

ISO/IEC JTC 1/SC 29/WG 11

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC 1/SC 29/WG 11/M9583

March 2003

| Title: | Finnish NB Comments on JVT-F100 |
| --- | --- |
| Sub group: | Video / JVT |
| Source: | Finnish NB |
| Status | Input |

_____________________________

| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 7th Meeting: Pattaya II, Thailand, 7-14 March, 2003 | Document: JVT-G030 Filename: JVT-G030.doc |
| --- | --- |

| Title: | Finnish NB comments on JVT-F100 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Contact(s): | Ye-Kui Wang Miska M. Hannuksela Nokia Mobile Software P.O. Box 88 33721 Tampere Finland Dong Tian TICSP Tampere Univ. of Technology P.O. Box 553 FIN-33101 Tampere Finland | Tel: Email: | +358 50 486 7004 ye-kui.wang@nokia.com +358 7180 73151 miska.hannuksela@nokia.com +358 3 3115 3853 tian@cs.tut.fi |
| Source: | Finnish NB |  |  |

_____________________________

This document contains the Finnish NB comments on JVT Awaji output draft JVT-F100.

Note: The only change relative to the published draft JVT-G030d1 is a revised text proposal in comment FINB.D.4.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| All | Finland | E | FINB.0.1 |  |

Context

Entire specification, about the description of the value range of a variable.

Question

Mostly “x shall be in the range of y to z, inclusive” is used, however, ”shall not exceed x” and “shall be less than x”, which may not be clear enough, are also used in a few places.

Proposal

Replace all the uses of ”shall not exceed x” and “shall be less than x” to “x shall be in the range of y to z, inclusive”.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 3.79 | Finland | E | FINB.3.1 |  |

Context

Definition of “parameter”.

Question

- The definition is not accurate, since “parameter” is used in many other places such as “quantisation parameter”, “HRD parameter”.

- Is the definition really necessary?

Proposal

Remove the definition.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 6.4 | Finland | E | FINB.6.1 |  |

Context

Inverse scanning process and neighboring relation

Question

6.4.4-6.4.8 describe neighboring availabilities and relations, not “inverse scanning process”, so they should not be included in 6.4 titled “inverse scanning processing”.

Proposal

Add a new subclause 6.5 with a proper title.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.3.2.1 7.4.2.1 Annex A | Finland | T | FINB.7.1 |  |

Context

Sequence parameter set, profile interoperability

Question

more_than_one_slice_group_allowed_flag, arbitrary_slice_order_allowed_flag, and redundant_pictures_allowed_flag provide means to detect the decodability of a Baseline bitstream with a Main Profile decoder. Similar flags could be specified for coding tools specific to Main Profile only and to Extended Profile only to allow similar decodability detection between Main and Extended profiles.

Proposal

See JVT-G018 for details.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.3.2.2 | Finland | T | FINB.7.2 |  |

Context

Picture parameter RBSP syntax

Question

For mb_slice_group_map_type equal to 2, both the top_left_mb and the bottom_right_mb are macroblock addresses relative to the top-left corner of the picture. This is inefficient in terms of bitrate.

Proposal

Indicate top and left separately as macroblock/macroblock pair coordinates relative to the top-left corner of the picture. Indicate bottom relative to the value of top, and right relative to the value of left.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.3.2.2 | Finland | T | FINB.7.3 |  |

Context

Picture parameter set RBSP syntax

Question

Indicating the picture size in macroblocks for mb_slice_group_map_type equal to 6 (the num_mbs_in_frame_or_field_minus1 syntax element) can be done more efficiently in terms of bitrate and more consistently compared to the sequence parameter set (which contains pic_width_in_mbs_minus1 and height_in_mbs_minus1).

Proposal

Replace num_mbs_in_frame_or_field_minus1 with picture width and height similarly to the corresponding parameters in the sequence parameter set.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.3.3.3, 7.4.3.3, C.4.1 | Finland | E | FINB.7.4 |  |

Context

no_output_of_prior_pics_flag

Question

The flag has nothing to do with decoded reference picture marking, but it is conveyed in the decoded reference picture marking syntax dec_ref_pic_marking().

Proposal

Move the flag to slice header.

Move the semantics of the lfag from 7.4.3.3 to 7.4.3.

Modify the second paragraph in 7.4.3.3 to make it not mention to the flag.

Remove the last sentence in the last paragraph in C.4.1.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.4.1.1 | Finland | T | FINB.7.5 |  |

Context

NAL unit order of redundant coded slices

Question

Redundant coded slices of a primary coded picture shall not be interleaved with any slice of or associated to another primary coded picture. However, this is not specified in the constraints of NAL unit order in 7.4.1.1.

Proposal

Specify it in 7.4.1.1.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.4.2.1, 8.2.2.3 | Finland | T | FINB.7.6 |  |

Context

Picture order count type 2

Question

In the semantics of pic_order_cnt_type, the constraint for using pic_order_cnt_type equal to 2 should be modified.

An informative note should be added in 8.2.2.3 to specify the condition when pic_order_cnt_type equal to 2 can be applied.

Proposal

Modify the constraint in the semantics of pic_order_cnt_type as follows, and remove from the normative text the same constraint in 8.2.2.3:

pic_order_cnt_type shall not be equal to 2 in a sequence that contains two or more consecutive non-reference frames, complementary non-reference field pairs, or non-paired non-reference fields in decoding order.

Add a note in 8.2.2.3 as follows:

NOTE - Picture order count type 2 cannot be used when the output order of pictures is different from the decoding order.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.4.2.1, 8.2.7.2 | Finland | E | FINB.7.7 |  |

Context

required_frame_num_update_behaviour_flag

Question

In the semantics of required_frame_num_update_behaviour_flag, it says “specifies … the decoding process in case of an inferred gap between values of frame_num as specified in subclause 8.2.7.2.” However, in 8.2.7.2, required_frame_num_update_behaviour_flag appears only in an informative note, and therefore the process specified in subclause 8.2.7.2 is performed regardless of the value of required_frame_num_behaviour_flag. The intent is that the normative part of subclause 8.2.7.2 is performed only if required_frame_num_behaviour_flag equals to 1.

Proposal

Modify the first paragraph and the following note as follows:

This process is invoked when required_frame_num_update_behaviour_flag is equal to 1 and frame_num is not equal to PrecedingRefFrameNum and is not equal to ( PrecedingRefFrameNum + 1 ) % MaxFrameNum.

NOTE – If required_frame_num_update_behaviour_flag is equal to 0 and frame_num is not equal to PrecedingRefFrameNum and is not equal to ( PrecedingRefFrameNum + 1 ) % MaxFrameNum, the decoding process should infer an unintentional loss of pictures.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.4.3 All | Finland | E | FINB.7.8 |  |

Context

Semantics of frame_num. Definition of “current picture”.

Question

The subclause states:

…

When the current picture is not an IDR picture, it shall not have frame_num equal to PrecedingRefFrameNum unless at least one of the following conditions are fulfilled:

- redundant_pic_cnt is not equal to 0 for the current picture

…

Term “current picture” has not been defined. Does it mean the primary coded picture whose slice or slice data partition is currently being decoded or is next in decoding order? Are redundant slices considered to be within the same “current picture” as the corresponding primary coded picture? Are all other NAL units associated with a primary coded picture according to subclause 7.4.1.2 of JVT-F100 considered to be within the same “current picture” as the primary coded picture?

According to subclause 8.2.1 of JVT-F100, coded picture boundaries are not detected between a primary coded picture and any redundant coded picture associated with it. A reason not to detect a picture boundary between a primary coded picture and redundant coded pictures associated to it is that their slices may be interleaved in decoding order.

Consequently, the term “current picture” in subclause 7.4.3 as printed above is confusing.

Proposal

Define term “current picture” in clause 3: A NAL unit, being decoded or next in decoding order if no NAL unit is being decoded, and all other NAL units that are associated to the same primary coded picture according subclause 7.4.1.2 as the NAL unit being decoded or next in decoding order.

Remove the bullet point in subclause 7.4.3: “redundant_pic_cnt is not equal to 0 for the current picture”.

Check the draft standard for the term “current picture”. If “current picture” is used to indicate either “the current primary coded picture” or “the current redundant coded picture” but not both collectively, replace the term accordingly.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.4.3 | Finland | T | FINB.7.9 |  |

Context

Semantics of frame_num

Question

The subclause states:

…

When the current picture is not an IDR picture, it shall not have frame_num equal to PrecedingRefFrameNum unless at least one of the following conditions are fulfilled:

…

- the current picture and the preceding primary reference picture are both reference fields and all of the following conditions are true.

- the current field and the preceding field have opposite parity

- the preceding primary picture is the preceding primary reference picture

- if there is a primary picture that precedes the preceding primary reference picture, the primary picture that precedes the preceding primary picture fulfills at least one of the following conditions:

  - it does not have frame_num equal to PrecedingRefFrameNum

  - it is an IDR picture

  - it is a field that is part of a complementary reference field pair that includes an IDR picture

…

The purpose of the last two items “it is an IDR picture” and “it is a field that is part of a complementary reference field pair that includes an IDR picture” is unclear to us. Moreover, the last item “it is a field that is part of a complementary reference field pair that includes an IDR picture” seems to allow an illegal case: frame_num is equal to zero for three consecutive coded fields in decoder order where the first one is an IDR picture.

Proposal

Consider if the last two items are correctly specified and needed. If yes, add an informative note regarding them.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.4.3.3 | Finland | T | FINB.7.10 |  |

Context

Memory management control operation

Question

It is stated in the subclause:

When decoding a field, if a memory_management_control_operation command equal to 3 assigns a long-term frame index to a field that is part of a short-term reference frame or a short-term complementary reference field pair, another memory_management_control_operation command to assign the same long-term frame index to the other field of the same frame or complementary reference field pair shall be present in the same decoded reference picture marking syntax structure.

Why send the other memory_management_control_operation rather than inferring it?

Proposal

Specify the semantics as follows:

When decoding a field, if a memory_management_control_operation command equal to 3 assigns a long-term frame index to a field that is part of a short-term reference frame or a short-term complementary reference field pair, the same long-term frame index is assigned to the other field of the same frame or complementary reference field pair.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.1 | Finland | E | FINB.8.1 |  |

Context

Decoding process introductions

Question

It says in 8.1 that

Subclause Error: Reference source not found describes the decoding process for NAL units with nal_unit_type equal to 1 through 5.

Subclause Error: Reference source not found describes the decoding process for slice headers and slice data in NAL units with nal_unit_type equal to 1, 2, and 5.

Subclause Error: Reference source not found describes the decoding process for slice headers and slice data in NAL units with nal_unit_type equal to 1 and 2.

Subclause Error: Reference source not found describes the decoding process for slice headers and slice data in NAL units with nal_unit_type equal to 1, 3, 4 and 5.

Subclause Error: Reference source not found describes the decoding process for slice headers and slice data in NAL units with nal_unit_type equal to 1, 3, 4 and 5.

Subclause Error: Reference source not found describes the decoding process for NAL units with nal_unit_type equal to 1 through 5.

First, the introductions of subclause 8.2 and 8.7 are exactly the same, and introductions of . subclause 8.3 to 8.6 are also exactly the same or very similar. Can these kinds of description give any helpful information to the reader?

Second, 8.3 describes intra prediction process, we cannot see the decoding process of slice headers in 8.3, which is mentioned in the text reprinted above. The problem is similar for subclauses 8.4 to 8.6.

Proposal

Rephrasing the 8.1, possibly as a whole.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.2.1 | Finland | E | FINB.8.2 |  |

Context

Picture boundary detection

Question

The 5th and 6th conditions for the process of picture boundary detection are reprinted below:

- The frame_num of the current slice is the same as the frame_num in the previous slice, and either delta_pic_order_cnt[ 0 ] is different from the delta_pic_order_cnt[ 0 ] in the previous slice, or delta_pic_order_cnt[ 1 ] is different from the delta_pic_order_cnt[ 1 ] in the previous slice.

- The frame_num of the current slice is the same as in the previous slice, and pic_order_cnt_type is less than 2, and the derived picture order counts TopPicOrderCnt and BottomPicOrderCnt are different for the two slices. [Ed.(AG) something wrong in here]

It is easy to notice that the 5th condition is a special case of the 6th condition.

Proposal

The 5th condition should be removed. (Since the inputs to this process include already TopPicOrderCnt and/or BottomPicOrderCnt, syntax elements delta_pic_order_cnt[0] and delta_pic_order_cnt[1] need not to be mentioned.)

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.2.7.4.3 | Finland | E | FINB.8.3 |  |

Context

Assignment process of a LongTermFrameIdx to a short-term reference picture

Question

The sentence in the second paragraph (copied below) is unclear and confusing.

picNumX shall refer to a frame or complementary reference field pair or non-paired reference field marked as "used for short-term reference" and not marked as "non-existing".

In addition, the sentence is contradictory to the last paragraph (partly copied below):

If the current picture is a field, the marking of the short-term reference field specified by picNumX …

Proposal

Resolve the issue.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| Annexes A, C, E | Finland | T | FINB.A.1 |  |

Context

max_dec_frame_buffering and DPB size

Question

Item g in subclause A.3.1 discusses about DPB size and max_dec_frame_buffering. max_dec_frame_buffering is specified in Annex E. DPB size is used in the Decoded Picture Buffer operation specified in Annex C.

DPB size and max_dec_frame_buffering and their use in Annex C have not been clearly specified. Does the value of max_dec_frame_buffering signaled in VUI “overrule” the DPB size in the formula in item g of A.3.1? Should max_dec_frame_buffering be referred to instead of “DPB size” in Annex C?

Proposal

This should be clarified such that if max_dec_frame_buffering is not present, the value here is the default.  If max_dec_frame_buffering is present it must obey the constraint stated in item g of A.3.1.  And if max_dec_frame_buffering is present, it must be obeyed rather than the the constraint in item g of A.3.1.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| Annex C | Finland | E | FINB.C.1 |  |

Context

Operation of DPB

Question

The text of the operation of DPB targets at two purposes: 1) bitstream and output timing conformance, and 2) output order conformance. The former description is in subclause C.4 whereas the latter description is in subclause C.6.1. The structure of the standard would be more consistent, if these two DPB operation descriptions would reside in the same level in the subclause structure and consecutive to each other. The description of operation of the output order DPB (subclause C.6.1) continues unlogically in subclause C.6.2 (and onwards).

Proposal

Perform the editorial corrections suggested above.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| C.4 C.6.4.2 | Finland | T | FINB.C.2 |  |

Context

Operation of DPB

Question

When required_frame_num_behaviour_flag equals to 1, the decoding process for gaps in frame_num (subclause 8.2.7.2) is performed and may result into creation of frames marked as “non-existing”. The “non-existing” frames play a role which reference frames are marked as “unused for reference” according to the sliding window decoded reference picture marking process (subclause 8.2.7.3). Consequently, the process of emptying/removing frame buffers from the DPB depends on subclause 8.2.7.2. Furthermore, the DPB overflow prevention condition for bitstream conformance (subclause C.5) must apply for bitstreams that cause insertion of frames marked as “non-existing” in addition to “conventional” bitstreams.

Proposal

Include a step of performing the decoding process for gaps in frame_num before the picture decoding step (C.4.1) in the DPB operation.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| Annex C | Finland | T | FINB.C.3 |  |

Context

Picture storage in DPB

Question

In case of coded fields, Annex C currently specifies that the first reference field of a complementary reference field pair is stored into the DPB right after it is decoded (unless it is a non-reference field that is supposed to be output immediately). This storage operation typically causes one frame buffer to be emptied in the bumping decoder process as indicated on the left-hand side in the figure below. However, if a field that is not the second field of a complementary field pair remained in the temporary frame buffer, no bumping would take place and the oldest frame marked as “used for reference” could still be used as prediction reference for the second field of a complementary field pair. This option is illustrated on the right-hand side in the figure below.

See also the next comment for another problem that could be solved with the proposal herein.

Proposal

Modify decoding process for reference picture lists construction (subclause 8.2.6), decoded reference picture marking process (subclause 8.2.7) and decoded picture buffer operation (Annex C) according to the idea described above.

Benefits from the proposal:

- In the proposed storage process, it is likely to have one more frame to be used as reference and thus to be benefical to the coding efficiency from the motion compensation point of view.

- If the first decoded field remains in the temporal frame store when decoding the second field, the status of DPB will be identical for the field pair. No DPB operations, including marking for reference pictures, storage of pictures into DPB, will follow the decoding of the first field if its opposite parity field is to be decoded. dec_ref_pic_marking() in the two fields, if present, will be identical also, which can be utilized to verify the received MMCOs. As a alternative, dec_ref_pic_marking() can be absent in the first field to save bits consumption.

- If the second proposed resolution in the next comment is adopted, the first decoded field will stay in the temproal frame store at first no matter it is used for reference or not.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| C.6 | Finland | T | FINB.C.4 |  |

Context

Decoder conformance, coded non-reference fields

Question

Assume that the DPB is fully occupied and two non-reference fields composing a complementary field pair are next in decoding order. The output order of the two fields is opposite to the decoding order, i.e., the first field in decoding order is output after the second field in decoding order. Both of the fields precede any picture in the DPB in output order. Let us then consider the operation of the bumping decoder right after the first field in decoding order is decoded: As the newly decoded field is a non-reference field and as it has the lowest picture order count among all pictures in the DPB, it is output immediately. After that the second field in output order is decoded and output based on the same reasons. However, the output order of these two fields according to the bumping decoder operation is different from the intended output order.

Proposal

One of the proposed solutions below:

- Disallow such a non-reference complementary field pair whose both fields precede any picture in the DPB in output order but whose decoding order is opposite to their output order.

- Specify the DPB for output order conformance so that a non-reference field remains in the temporary frame buffer until the second field of the complementary field pair is decoded (provided that the next picture in decoding order is the second field of the complementary field pair).

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| C.6.4 | Finland | T | FINB.C.5 |  |

Context

Current decoded picture marking and storage

Question

It is stated that “For a decoded frame, its FramePicOrderCnt is stored.” However, the top field and the bottom field of a coded frame may have a different picture order count from each other, and the description of the output order conformant DPB should take this into account so that the fields of a frame are output in their relative POC order.

Proposal

For a decoded frame, both its TopPicOrderCnt and BottomPicOrderCnt are stored. If TopPicOrderCnt is equal to BottomPicOrderCnt, a decoded frame is output at a time. If TopPicOrderCnt differs from BottomPicOrderCnt, two decoded fields are output in ascending order of picture order count.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| C.6.4.3 | Finland | T | FINB.C.6 |  |

Context

“Bumping process”

Question

The handling of frames marked as “non-existing” is not specified.

Proposal

Add the following as the first item in the "bumping" operation, when an empty frame buffer is needed for a (non IDR) picture:

a) If the DPB includes a frame marked as “non-existing” and “unused for reference”, the corresponding frame buffer is emptied and the bumping operation is terminated.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| D | Finland | E | FINB.D.1 |  |

Context

SEI introduction

Question

The first three sentences in Annex D say as follows:

“This annex specifies syntax and semantic for supplemental enhancement information that provides data constructs that are synchronous with the video data content.  Each sei_payload( ) specifies payloadType and payloadSize parameters. This annex specifies supplemental enhancement information (SEI) that provides a data delivery mechanism construct that is delivered synchronous with the video data content.”

The second sentence is incorrect, because it is “each SEI message” not “each sei_payload()” that specifies payloadType and payloadSize parameters, and this statement is useless for SEI introduction.

The third sentence is redundant with the first sentence.

Proposal

Modify the first three sentences as follows:

This annex specifies syntax and semantic for supplemental enhancement information (SEI) that provides data constructs that are synchronous with the video data content.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| D.1.8, D.2.8 | Finland | T | FINB.D.2 |  |

Context

Syntax and semantics of decoded reference picture marking repetition SEI message

Question

Syntax element original_frame_num should be conditional to orginal_idr_flag, since if orginal_idr_flag is equal to 1, then original_frame_num can be inferred as zero.

Proposal

Modify the corresponding part of the syntax as follows, and changing the semantics accordingly:

original_idr_flag

if(original_idr_flag)

original_frame_num

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| D.1.9, D.2.9 | Finland | T | FINB.D.3 |  |

Context

Spare picture SEI message syntax and semantics

Question

There is a bug for syntax and semantics of spare picture SEI message and some unclear texts needing clarification. The bug is that the picture size in macroblocks, which is dependent on field_pic_flag in slice header, is required to parse the SEI message syntax. However, parsing of the SEI message syntax shall not be dependent on any data from slice header. The unclear place is the description of the compression method for spare picture unit map if compressed spare map is coded.

Proposal

Following syntax changes are proposed:

| spare_pic( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| target_frame_num | 5 | ue(v) |
| num_spare_pics_minus1 | 5 | ue(v) |
| for( i = 0; i < num_spare_pics_minus1+1; i++ ) { |  |  |
| delta_spare_frame_num[ i ] | 5 | ue(v) |
| spare_area_idc[ i ] | 5 | ue(v) |
| if( spare_area_idc[ i ] = = 1 ) |  |  |
| for( j = 0; j < PicSizeInMapUnits; j++ ) |  |  |
| spare_unit_flag[ i ][ j ] | 5 | u(1) |
| else if( spare_area_idc[ i ] = = 2 ) { |  |  |
| mapUnitCnt = 0, j = 0 |  |  |
| do { |  |  |
| zero_run_length[ i ][ j ] | 5 | ue(v) |
| mapUnitCnt += zero_run_length[ i ][ j++ ] + 1 |  |  |
| } while(mapUnitCnt< PicSizeInMapUnits ) |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

Following semantics changes are proposed:

The processing of spare picture is in units of slice group map unit, i.e., each processing unit is a macroblock if frame_mbs_only_flag is equal to 0 and a macroblock pair otherwise.

The spare picture SEI message indicates that certain units, called spare  units, in one or more decoded reference pictures resemble the co-located units in a certain decoded picture, called the target picture, so much that any of these spare decoded units can be used to replace a co-located incorrectly decoded unit in the target picture in the decoded picture buffer and in the decoder output.  Decoded pictures that contain spare units are called spare pictures.

target_frame_num identifies the target picture whose spare pictures and units are specified later in the message. The target picture is the latest reference picture having frame_num equal to target_frame_num.

NOTE – The target picture should precede, in decoding order, the associated picture of the spare picture SEI message.

num_spare_pics_minus1 specifies the number of pictures that contain spare picture or units for the target picture. num_spare_pics_minus1 shall be in the range of  0 to 16 x (2 - frame_mbs_only_flag) - 1, inclusive.

delta_spare_frame_num[ i ] specifies to which spare picture the following spare picture information in the i-th loop count belongs. When the loop count i is equal to zero, candidateSpareFrameNum[ 0 ] is derived as follows:

if( target_frame_num[ 0 ] > 0 )
	candidateSpareFrameNum[ 0 ] = target_frame_num[ 0 ] – 1
else
	candidateSpareFrameNum[ 0 ] = MaxFrameNum – 1

When i is equal to other values than zero, candidateSpareFrameNum[ i ] is derived as:

if( spareFrameNum [ i - 1 ] > 0 )
	candidateSpareFrameNum[ i ] = spareFrameNum [ i - 1 ] – 1
else
	candidateSpareFrameNum[ 0 ] = MaxFrameNum – 1

For each loop count i, spareFrameNum is derived as follows:

spareFrameNum[ i ] = candidateSpareFrameNum[ i ] – delta_spare_frame_num[ i ]
if( spareFrameNum[ i ] < 0 )
	spareFrameNum[ i ] = MaxFrameNum + spareFrameNum[ i ]	(D-1)

spare_area_idc[ i ] indicates how the locations of spare units of the spare picture specified in the i-th loop count are coded. spare_area_idc[ i ] equal to 0 indicates that all units of the i-th spare picture are spare units hence no further signalling than the indicator is needed. spare_area_idc[ i ] equal to 1 indicates an uncompressed spare unit map. spare_area_idc[ i ] equal to 2 indicates a compressed spare unit map. A spare unit map consists of flags for each unit location of a picture.  spare_area_idc[ i ] shall be in the range of 0 to 2, inclusive.

If spare_area_idc[ i ] is 1, there is a spare_unit_flag[ i ][ j ] for each unit address of the spare unit map in raster scan order for each spare picture. spare_unit_flag[ i ][ j ]  equal to 0 indicates that the unit is a spare unit, and spare_unit_flag[ i ][ j ] equal to 1 indicates that the unit is not a spare unit.

If spare_area_idc[ i ] is equal to 2, a spare unit map of the i-th target picture is compressed as follows. The coded spare unit map is scanned in counter-clockwise box-out order as specified in subclause Error: Reference source not found. Each coded zero_run_length[ i ][ j ] represents zero_run_length[ i ][ j ] consecutive zeros followed by a one. If i is equal to zero, one means that the unit is a spare unit while zero means the unit is not a spare unit. That is, each element of the spare unit map of the 0th spare picture, spare_unit_flag[ 0 ][ j ], is derived as:

for( j = 0, loop = 0; j < PicSizeInMapUnits; loop++) { 
	for( k = 0; k < zero_run_length[ 0 ][ loop ]; k++)
	    spare_unit_flag[ 0 ][ j++ ] = 0
	spare_unit_flag[ 0 ][ j++ ] = 1
}

If i is greater than zero, each element of the spare unit map of the ith spare picture, spare_unit_flag[ i ][ j ], is derived as:

for( j = 0, loop = 0; j < PicSizeInMapUnits; loop++) { 
	for( k = 0; k < zero_run_length[ 0 ][ loop ]; k++)
	    spare_unit_flag[ i ][ j ] = 0 XOR spare_unit_flag[ i - 1 ][ j++ ]
	spare_unit_flag[ i ][ j ] = 1 XOR spare_unit_flag[ i - 1 ][ j++ ]
}

Where XOR denotes exclusive or operation.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| D.1.10 D.2.10 | Finland | T | FINB.D.4 |  |

Context

Scene information SEI message

Question

It is unclear when the scene information SEI message becomes effective and how long it persists: [Ed Note: For USNB comment D-11, we need some text on 1) When a scene information SEI is received, does it take effect at the next coded picture? 2) Does this scene information persist until the next scene information SEI is received?]

Editorial changes to describe scene transitions more clearly have also been requested.

Proposal

In contrast to some other SEI messages, such as the full-frame freeze SEI message, specifying a default timeout for the scene information SEI message does not make sense, as decoders need to know correct scene information accurately to choose the correct error concealment algorithm, for example. Specifying that the scene information remains unchanged between every pair of scene information SEI messages that contain the same values of scene_id, scene_transition_type, and second_scene_id is not a good design choice either, since when a transmission error occurs, the decoder cannot wait until the next scene information SEI message is received in order to conclude the error concealment algorithm correctly.

We propose that the scene information is valid until the next scene information SEI message is received. A new flag indicating if the scene information known is proposed. Proposed changes relative to JVT-F100 are included in the attached document:

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| D.2.19 | Finland | T | FINB.D.5 |  |

Context

Motion-constrained slice group set SEI message syntax and semantics

Question

The syntax, mainly num_slice_groups_minus1, and semantics, mainly exact_match_flag, of the motion-constrained slice group set SEI message are buggy and not clear, as can be seen from the follow proposal.

Proposal

- Replace the syntax element num_slice_groups_minus1 with num_slice_groups_in_set_minus1, as follows:

| motion_constrained_slice_group_set( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| num_slice_groups_in_set_minus1 | 5 | ue(v) |
| for( i = 0; i <= num_slice_groups_minus1; i++) |  |  |
| slice_group_id[ i ] | 5 | u(v) |
| exact_match_flag | 5 | u(1) |
| pan_scan_rect_flag | 5 | u(1) |
| if( pan_scan_rect_flag = = 1 ) |  |  |
| pan_scan_rect_id | 5 | ue(v) |
| } |  |  |

- Modify the semantics as follows:

This SEI message signals that inter prediction over slice group boundaries in the sequence is constrained as specified below.

The picture that the next slice or slice data partition belongs to is herein referred to as the current picture. The current picture for the motion-constrained slice group set SEI message shall be an IDR picture. The message concerns all consecutive pictures in decoding order until the next IDR picture (exclusive). These pictures are referred to as the target picture set.

A slice group set containing at least one slice group is specified in the message. The specified slice group IDs identify the slice groups, if present, belonging to the slice group set. The message concerns all slice groups, if present, in the slice group set in each picture within the target picture set.

No sample value outside the slice group set in any picture in the target picture set shall be used in inter prediction of any element within the slice group set in any picture in the target picture set.

num_slice_groups_in_set_minus1 + 1 specifies the number of slice groups in the slice group set. num_slice_groups_in_set_minus1 is in the range of 0 to num_slice_groups_ minus1, inclusive.

slice_group_id[ i ] identifies the slice group that belongs to the slice group set. The allowed range is from 0 to num_slice_groups_in_set_minus1, inclusive. The size of the slice_group_id parameter is Ceil( Log2( num_slice_groups_in_set_minus1 + 1 ) ) bits.

exact_match_flag equal to 0 indicates that, within the target picture set, when the macroblocks that do not belong to the slice group set are not decoded, the value of each sample in the slice group set needs not be exactly the same as the value of the same sample when all the macroblocks are decoded. exact_match_flag equal to 1 indicates that, within the target picture set, when the macroblocks that do not belong to the slice group set are not decoded, the value of each sample in the slice group set shall be exactly the same as the value of the same sample  when all the macroblocks in the target picture set are decoded.

Note - if disable_deblocking_filter_idc equals to 2 in all slices in the target picture set, exact_match_flag should be 1.

pan_scan_rect_flag equal to 0 specifies that pan_scan_rect_id is not present. pan_scan_rect_flag equal to 1 specifies that pan_scan_rect_id is present.

pan_scan_rect_id indicates that the specified slice group set covers at least the pan-scan rectangle identified by pan_scan_rect_id within the target picture set.

Note - The size, shape, and location of the slice groups in the slice group set may change within the target picture set.