| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 12th Meeting: Redmond, WA, USA, 17-23 July, 2004 | Document: JVT-L028 Filename: JVT-L028.doc |
| --- | --- |

| Title: | Additional Experimental Results on Residue Sampling (JVT-K018) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Woo-Shik Kim, Dmitry Birinov, Hyun Mun Kim Multimedia Lab. Samsung AIT P.O.Box 111, Suwon 440-600 Korea | Tel: Email: | 82-31-280-9204 hyunmun27.kim@samsung.com |
| Source: | Samsung AIT |  |  |

_____________________________

- Introduction

In the last meeting in Munich, we proposed the residue sampling method [1] for the JVT FRExt [2]. We showed 1~2 dB PSNR gain over conventional H.264/MPEG-4 AVC main profile.

Some people commented that the comparison was not fair since the proposed method is based on the 4:4:4 coding and H.264/MPEG-4 AVC is based on the 4:2:0 coding. At that time we couldn’t show the experimental results of residue sampling compared with 4:4:4 coding without residue sampling.

In this document, we show the experimental results of residue sampling compared to 4:4:4 coding without residue sampling. The target bit-rate is set to 10 Mbps, which would be good for many applications such as HD-DVD.

- What Is Residue Sampling?

The 4:2:0 and 4:2:2 formats are the mostly well-known sample formats. In these formats, only chrominance components are sub-sampled in the ratio of 1/4 and 1/2, respectively, compared to the luminance sampling rate.

In the professional applications, it would be better not to use the chrominance sub-sampling, which would result in the color distortion. However, the chrominance sampling would be required for the low bit rate applications.

To adapt to the variety of applications, we proposed to use the adaptive sampling method, where the sampling is applied to the intra/inter-predicted residue data. Since the sampling is applied to the residue data that has less information than the original data, the loss from the sampling process is reduced.

We proposed to apply this residue sampling adaptively on macroblock basis, by adding two flags, “residue_sampling_flag” to the sequence parameter set, and “mb_residue_sampling_flag” to the macroblock layer after macroblock type. The sampling is performed depending on encoder’s discretion by setting these flags.

Details can be found in [1].

- Experimental Results

We compared the coding efficiency of the residue sampling method with that of YCoCg-R [3] 4:4:4 coding. In this experiment, the residue sampling is always applied to all MBs. Film sequences are used which were defined as common test materials for the JVT FRExt. The test conditions are summarized as follows:

- Search range: 32

- Number of reference frame: 3

- Entropy coding: CABAC

- RD-optimized mode selection: On

- GOP structure: IBBP…

- Number of slice groups: 1

- No rate control (fixed QP value of 18, 24, 30 and 36)

PSNR differences between Residue Sampling in all MBs and YCoCg-R 4:4:4 at 10 Mbps:

| Sequences | PSNR differences (dB) of each color components |  |  |  |
| --- | --- | --- | --- | --- |
|  | RGB | G | R | B |
| AnalogTV | 0.8 | 1.6 | 0.8 | 0.0 |
| Bicycle | 0.2 | 0.5 | 0.3 | -0.1 |
| CardToss | 0.7 | 1.4 | 1.0 | -0.2 |
| Dinner | 0.8 | 1.5 | 1.0 | -0.1 |
| Restaurant | 0.4 | 0.4 | 0.4 | 0.0 |
| Tomatoes | 0.0 | 0.4 | 0.1 | -0.3 |
| AVE | 0.5 | 1.0 | 0.6 | -0.1 |

Results of residue sampling over YCoCg-R 4:4:4 in RD-curves of RGB average and G component only:

< PSNR of RGB average >                  < PSNR of G component only>

< PSNR of RGB average >                  < PSNR of G component only>

More test resluts are available in the accompanying Excel file.

The RD-curves show that the coding efficiency of the residue sampling is superior to the conventional 4:4:4 chroma format coding througout the low to high bit-rate range.

The table shows the PSNR gain at 10 Mbps. Not only the RGB average gain, but also that of G coponent show the effectiveness of the residue sampling method.

In this experiments, the residue sampling is always applied to all MB. If the residue sampling is adaptively applied on the MB basis by setting the mb_residue_sampling_flag, the coding efficiency will be increased further.

- Conclusions

The use of residue sampling in RGB space achieves great coding efficiency for various kinds of RGB data such as film material. The advantage is clearly shown in the simulation results. We can select residue sampling options depending on image characteristics and requirements of the given application at encoder’s discretion. It helps to achieve the best coding efficiency for any type of input data.

We hope that this information would be benificial for the development of the future video coding technologies.

References

- W.-S. Kim, D. Birinov and H. M. Kim, “Adaptive residue transform and sampling,” ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6, Document JVT-K018, Mar. 2004.

- “Draft text of ITU-T Rec. H.264 | ISO/IEC 14496-10 AVC Fidelity Range Extensions Amendment,” ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6, Document JVT-K047, Jul. 2004.

- H. Malvar and G. Sullivan, “YCoCg-R: A color space with RGB reversibility and low dynamic range,” ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6, Document JVT-I014, Sep. 2003.