| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) Meeting: Redmond, U.S.A., 17-23rd July, 2004 | Document: JVT-L035 Filename: input.doc |
| --- | --- |

| Title: | DPB capacity indicator for deblocking filter display preference SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name Jiuhuai Lu 100 Universal City Plaza, 3153PHL Universal City, CA 91608 U.S.A. Shinya Kadono 1006 Kadoma, Kadoma Osaka 573-8501, Japan | Tel: Email: | +1(818)777-8147 jiuhuai@research.panasonic.com kadono.shinya@jp.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# 1. Background

The deblocking filter display preference SEI adopted into Annex D provides flexibility to allow pictures to be filtered for motion compensation while using the unfiltered copy for output. In using this SEI message, the deblocking filter is used to reduce picture noise rather than blocking errors. By using the SEI, it allows output pictures to preserve picture details, yet allowing motion compensation to use less noisy reference pictures, which improves coding efficiency when pictures contain wealth of details, including film grains.

The use of the deblocking filter display preference SEI message, however, requires that a decoder stores the unfiltered copy of reference pictures in addition to storing filtered reference pictures into DPB. The decoder has to have sufficient storage space for keeping the unfiltered pictures.

Depending on the application domains or products that implement this SEI, encoders may assume that the DPB is the only storage available for unfiltered picture and therefore impose limits on DPB capacity for normative decoding in order to reserve for unfiltered copies. (After reserving sufficient amount of DPB capacity for this process, the remaining DPB capacity for normative decoding process will still be at least half of the total capacity specified in Annex A.) In other applications, encoders may choose not to reserve any part in DPB for the storage by the SEI and assume that additional memory is prepared in decoders.  Because of these implementation possibilities, there is no way for a decoder to know ahead of time what DPB capacity usage scheme the encoder has followed. It is therefore difficult to for those decoders to decide whether to use the SEI or not to use SEI. In this proposal, we specify one bit flag dpbb_capacity_reserved so that a decoder can use the information to make a decision whether to utilize the SEI or not as in the following table.

Table 1 Different decoder to use the deblocking filter display preference SEI

|  | Encoder restricts the use of DPB capacity for normative decoding process (dpb_capactiy_reserved = 1) | No restriction imposed on use of DPB capacity for normative decoding process (dpb_capactiy_reserved = 0) |
| --- | --- | --- |
| Decoder with no additional memory | Can use the SEI | Cannot use the SEI |
| Decoder with additional memory | Can use the SEI | Can use the SEI |

# 2. Syntax addition to the deblocking filter display preference SEI

| deblocking_filter_display_preference( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| deblocking_display_preference_cancel_flag | 5 | u(1) |
| if( !deblocking_display_preference_cancel_flag ) { |  |  |
| dpb_capacity_reserved | 5 | u(1) |
| display_prior_to_deblocking_preferred_flag | 5 | u(1) |
| deblocking_display_preference_repetition_period | 5 | ue(v) |
| } |  |  |
| } |  |  |

3. Semantics addition to the deblocking filter display preference SEI

`

dpb_capacity_reserved equal to 1 indicates that the use of DPB capacity is restricted relative to DPB amount specified in Annex A, so that the remaining DPB capacity can be used for maximal storage required by the deblocking filter display preference SEI to store unfiltered pictures.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Panasonic Technologies Company |  |  |
| Mailing address | 100 Universal City Plaza Universal City, CA 91608 |  |  |
| Country | U.S.A. |  |  |
| Contact person | Jiuhuai Lu |  |  |
| Telephone | +1(818)777-8147 |  |  |
| Fax |  |  |  |
| Email | jiuhuai@research.panasonic.com |  |  |
| Place and date of submission | Redmond, WA, U.S.A., July 20, 2004 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | ITU-T Rec. H.264 ¦ ISO/IEC 14496-10 |  |
| Title |  | DPB usage indicator for deblocking filter display preference SEI message |  |
| Contribution number |  | JVT-L035 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| X | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: