| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 14th Meeting: Hong Kong, PRC China 17-21 Jan., 2005 | Document: JVT-N012 Filename: input.doc |
| --- | --- |

| Title: | The Fast Mode Decision for High Profile |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Inchoon Choi School of ECE, SungKyunKwan Univ. 300 Chunchun-dong, Suwon, Korea Woong Il Choi School of ECE, SungKyunKwan Univ. 300 Chunchun-dong, Suwon, Korea Jeyun Lee School of ECE, SungKyunKwan Univ. 300 Chunchun-dong, Suwon, Korea Byeungwoo Jeon School of ECE, SungKyunKwan Univ. 300 Chunchun-dong, Suwon, Korea | Tel: Email: | +82-31-290-7186 sonne@ece.skku.ac.kr +82-31-290-7186 creata@ece.skku.ac.kr +82-31-290-7186 jeyun22@ece.skku.ac.kr +82-31-290-7144 bjeon@yurim.skku.ac.kr |
| Source: | Sung Kyun Kwan University |  |  |

_____________________________

## Abstract

The fast algorithms, JVT-J033 [1] and JVT-K021 [2] are shown to significantly reduce computational complexity for RD optimization-based mode decision without outstanding loss in Baseline and Main Profiles. However, there has been no report on the performance when those methods are applied to high profile. This contribution provides the results. The simulation results show that these algorithms also provide considerable reduction in computational complexity and processing time in High Profile.

## Early skip detection and Selective intra mode decision

The proposed algorithms in JVT-J033 and JVT-K021 provided two algorithms for fast high-complexity mode decision. One of them is ‘early skip detection for P and B slice’. It checks whether the skip mode can be chosen for the current macroblock mode or not. To do so, first of all, one needs to estimate the motion vector and reference frame for the given macroblock in order to check the skip condition in P slice. After the motion vector and the reference frame are found, then we calculate the RD cost of 16x16 macroblock and check the P-SKIP condition. The procedure in case of B-slice is similar to that of P-slice. To check whether the current macroblock mode can be decided as SKIP in B-slice, macroblock is encoded as the direct mode in advance. Another class of fast algorithm is ‘Selective Intra mode decision’: it compares degree of spatial correlation with that of temporal correlation for deciding whether proceeding to intra coding is worthwhile. Since proper decision between intra and inter modes need some objective measure of spatial and temporal correlation, we proposed to utilize boundary errors for spatial correlation and number of bits for encoded motion-compensated residual data for the temporal correlation.

Experimental Results

The video sequences and experimental conditions are shown in Table 1. To evaluate the overall performance, all proposed methods are implemented in JM9.2 encoder and tested under the High profile.

Table 1 Simulation condition

| Profile |  | High |
| --- | --- | --- |
| Codec |  | JM 9.2 |
| MV search range |  | 32 |
| QP |  | 28, 34, 36, 40 |
| Number of Reference |  | 5 (QCIF, CIF) / 3 (HD) |
| Common coding option |  | Rate distortion optimization, Hadamard transform CABAC Intra period = 4 IBBP coding ‘encoder_highquality_HD.cfg’ config file in JM9.2 was used |
| PC |  | Xeon 2.4GHz(dual CPU), 1GB RAM |
| Test sequence | QCIF (100 frames, 10Hz) | Container, Foreman, News |
|  | QCIF (150 frames, 15Hz) | Silent |
|  | CIF (150 frames, 15Hz) | Paris |
|  | CIF (300 frames, 30Hz) | Mobile, Tempete |
|  | 720p (33 frames, 30Hz) | Bigships, Crew, Harbour, City |

TS indicate the average time saving in encoding process, which is given as below.

Where Time[reference] and Time[proposed] are respectively the entire encoding times of JM 9.2 reference software and proposed method.

Table 2 Simulation result for M1 (Early skip detection)

|  | TS [%] | BDBR [%] | BDPSNR [dB] |
| --- | --- | --- | --- |
| CONTAINER | 77.78 | 0.25 | -0.02 |
| FOREMAN | 46.23 | 0.54 | -0.03 |
| NEWS | 69.41 | 0.59 | -0.04 |
| MOBILE | 38.68 | 1.17 | -0.06 |
| SILENT | 55.93 | 0.29 | -0.01 |
| PARIS | 57.92 | 0.27 | -0.02 |
| TEMPETE | 37.29 | 0.52 | -0.02 |
| Average | 54.75 | 0.52 | -0.03 |

Table 3 Simulation result for method M2 (Selective Intra coding)

|  | TS [%] | BDBR [%] | BDPSNR [dB] |
| --- | --- | --- | --- |
| CONTAINER | 11.46 | 0.33 | -0.02 |
| FOREMAN | 9.67 | 0.84 | -0.04 |
| NEWS | 11.57 | 0.69 | -0.05 |
| MOBILE | 16.22 | 0.51 | -0.03 |
| SILENT | 10.92 | 0.53 | -0.03 |
| PARIS | 18.27 | 0.28 | -0.02 |
| TEMPETE | 16.44 | 0.50 | -0.02 |
| Average | 13.51 | 0.53 | -0.03 |

Table 4 Experimental result of M1+M2 for low resolution

|  | TS [%] | BDBR [%] | BDPSNR [dB] |
| --- | --- | --- | --- |
| CONTAINER | 79.68 | 0.36 | -0.02 |
| FOREMAN | 70.01 | 1.52 | -0.08 |
| NEWS | 61.87 | 1.16 | -0.08 |
| MOBILE | 55.56 | 1.50 | -0.07 |
| SILENT | 50.65 | 0.88 | -0.05 |
| PARIS | 58.23 | 0.57 | -0.03 |
| TEMPETE | 64.39 | 0.91 | -0.04 |
| Average | 62.91 | 0.99 | -0.05 |

Table 5 Experimental result of M1+M2 for HD

|  | TS [%] | BDBR [%] | BDPSNR [dB] |
| --- | --- | --- | --- |
| CREW | 63.68 | 11.38 | -0.32 |
| BIGSHIPS | 75.22 | 2.07 | -0.07 |
| HARBOUR | 46.41 | 1.19 | -0.04 |
| CITY | 66.92 | 2.32 | -0.10 |
| Average | 63.06 | 4.24 | -0.13 |

In case of High profile, the early skip detection scheme (M1) and the selective intra coding (M2) turn out providing similar complexity reduction with minor quality loss with compare to other profile, as shown in Table 2~4. In Table 5, we can see that overall time saving of given methods is about 63% in case of HD sequence. In this case, BDBR and BDPSNR loss are 4.24% and -0.13dB, respectively. Besides, we don’t see any visual artifact in the decoded sequences. The result shows that the proposed fast mode decision methods significantly reduce overall encoding time with only negligible degradation in case of High profile as well.

## Conclusion

To reduce computational complexity and to speed-up the decision process under RD optimization=ON in high profile, we reported performance of the two fast mode decision schemes under the high profile. This information contribution has shown that they considerably reduce encoding time only with negligible coding loss.

## Reference

[1] B. Jeon and J.Lee, “Fast mode decision for H.264,” ITU-T, Doc. #JVT-J033, 2003.

[2] J.Lee, I.Choi, W.Choi and B. Jeon, “ Fast Mode Decision for B slice,” ITU-T, Doc. #JVT-K021

(Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name |  |  |  |
| Mailing address |  |  |  |
| Country |  |  |  |
| Contact person |  |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email |  |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  |  |  |
| Title |  |  |  |
| Contribution number |  |  |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: